ISTP DATA MANAGEMENT PLAN

Version 4.3

Table of Contents

1.0  Introduction	1

2.0  Mission Science Elements	1

2.1  Global Geospace Science Initiative	1

2.2  Collaborative Solar Terrestrial Research Initiative	2

3.0  Mission Science Objectives and Success Criteria	3

4.0  Mission Operation Elements	4

4.1  Central Data Handling Facility	5

4.2  Data Distribution Facility	6

4.3  Special Data Distribution Activities	7

4.4  The Science Planning and Operations Facility	7

4.4.1  SPOF Science Planning Functions	7

4.4.2  SPOF Operations Functions	8

4.4.3  SPOF Science Advancement Functions	8

4.5  SOHO Experimenters Operations Facility	9

4.6  Cluster Joint Science Operations Center	9

5.0  Data Products	10

5.1  SIRIUS Data	10

5.2  Level 0 Data	10

5.3  Key Parameters	10

5.4  Citable Key Parameters	12

5.5  Event Data	12

5.6  Other Data Products	12

5.7  Archival Data Volume	12

5.8  SOHO Data Products	13

5.9  Cluster Data Products	13

5.10  Solar Wind Catalog	14

6.0  Correlative Science Data Management	14

6.1  Campaign Coordination Functions of the SPOF	14

6.2  Inter-Agency Correlative Science Activities	14

6.3  SOHO Solar Data Analysis Center	16

6.4  Cluster Science Data System	16

7.0  Data Archiving and Distribution	16

7.1  Data Archive Products Contents and Disposition	16

7.2  Archive Data Access	16

8.0  Project Data Management Documentation	17

9.0  References	17

Acknowledgments	19

Table of Contents (Continued)

Figure Captions	19

Table 1 - Data Services at the CDHF	23

Table 2 - Key Parameter Standards and Conventions	24

Table 3 - Wind KP Data Product Summary	25

Table 4 - Polar KP Data Product Summary	26

Table 4 - Polar KP Data Product Summary	27

Table 4 - Polar KP Data Product Summary	28

Table 5 - Ground-based Location & KP Data Product Summary	29

Table 5 - Ground-based Location & KP Data Product Summary	30

Table 6 - Supplementary Missions KP Data Product Summary	31

Table 7 - Geotail KP Data Product Summary	33

Table 7 - Geotail KP Data Product Summary	34

Table 8 - SOHO KP Data Product Summary	35

Table 9 - SOHO Science Data Product Summary	36

Table 10 - ClusterSummary Parameters Data Products	37

Table 11 - Cluster Science Data Centers	38

Table 12 - Interball/Aurora Key Parameters	39

Table 13 - Interball/Tail Key Parameters	40

Table 14 - ISTP Data Products and Disposition Overview	41

Appendix A - Overview of the Wind Mission Instruments	42

Appendix B - Overview of the Polar Mission Instruments	47

Appendix C - Rules for Data Rights

ISTP/NASA funded Missions	53

Appendix D - Rules for Data Rights

Geotail Mission	54

Appendix E - Rules for Data Rights

SOHO Mission	55

Table of Contents (Continued)

Appendix F - Rules for Data Rights

Cluster Mission	57

Appendix G - Rules for Data Rights

Interball Mission	59

Appendix H - IACG Rules for Data Exchange	60

Appendix I-Bibliography of ISTP Data Handling Documents	64

Appendix J - Acronym List	69

.c.1.0  Introduction

Solar-terrestrial physics concerns the study of physical processes that control the flow, distribution and dissipation of mass, momentum, and energy from the solar wind to the Earth’s atmosphere.  When the charged particles in the solar wind reach the Earth, some solar wind particles enter the magnetosphere directly along open magnetic field lines or through dynamical coupling processes.  These solar wind entry processes lead to significant influence of the solar wind on the configuration, dynamics and energetics of the Earth’s upper atmosphere.

An international effort that utilizes multi-space agency spacecraft to achieve a new understanding of solar-terrestrial physics is currently underway.  This effort is called the International Solar Terrestrial Physics (ISTP) program.  The goal of the ISTP is to attain correlative, inter-comparison of simultaneous measurements taken by the ISTP spacecraft in different geospace regions and ground observing stations, and the interpretation of the observations by theoretical modeling studies to understand the underlying physics of the entire solar-terrestrial system.

For this effort, the National Aeronautics and Space Administration (NASA) is providing the Wind and Polar satellites.  The Institute of Space and Astronautical Science (ISAS) in Japan is providing the Geotail satellite.  NASA is also collaborating with the European Space Agency (ESA) in two additional solar-terrestrial missions, SOHO and Cluster.

Supplementary observations will be made by several existing equatorial and solar wind spacecraft.  The equatorial spacecraft are the Geosynchronous Operational Environmental Satellite (GOES) of the National Oceanic and Atmospheric Administration (NOAA) and the Los Alamos National Laboratory (LANL) spacecraft from the Department of Energy (DOE).  The equatorial satellites will monitor magnetic-field and particle changes that occur when particles are energized during auroral events.  Additional data from other existing satellites such as NASA’s Interplanetary Monitoring Platform satellite (IMP-8) will also be used.

The combined data sets from the above spacecraft and from complementary ground-based observations will be used to construct quantitative models to describe how mass, momentum, and energy carried by the solar wind are transported across magnetospheric boundaries, stored and energized in the magnetosphere, and subsequently dissipated into the Earth’s atmosphere.  The purpose of this document is to provide the overview data management plan for the above missions and to set the scene for the data archiving and distribution.

.c.2.0  Mission Science Elements

The ISTP Program will provide internationally-coordinated scientific studies of major areas of Geospace by means of remote and in situ spacecraft measurements, ground-based observations, and theoretical investigations.  These studies will use data products produced by the “core missions,” as well as those obtained  from other sources, for correlative purposes.  The ISTP core missions are Geotail, Wind, Polar, SOHO, and Cluster.  NASA’s participation in the ISTP Program is through two programs:  the Global Geospace Science Initiative, and the Collaborative Solar Terrestrial Research Initiative.

.c2.2.1  Global Geospace Science Initiative

The Global Geospace Science (GGS) Initiative consists of the Wind, Polar, and Geotail missions; selected ground-based observing stations; selected existing supplementary NASA, NOAA and LANL missions; and the theoretical global modeling effort.

The Wind spacecraft is placed initially in a dayside apogee, double lunar swingby orbit of 80 to 250 RE at apogee.  An overview of the Wind mission instruments is given in Appendix A.  After approximately a year in orbit, Wind will be re-positioned into a halo orbit about the sunward (L1) libration point.  The Polar spacecraft is placed in an elliptical orbit of approximately 2 x 9 RE from the Earth’s center with an initial apogee near the north geomagnetic pole.  An overview of the Polar mission instruments is given in Appendix B.

The Wind spacecraft provides essential measurements pertaining to the solar wind (particle fluxes, composition, density, flow velocity, electric and magnetic fields).  The Polar spacecraft provides essential observations of the auroral zone (through remote imaging) and the polar magnetosphere (particle fluxes, composition, density, electric and magnetic fields).  The eight scientific instruments on Wind and the eleven instruments on Polar all produce measurements that are processed in the Central Data Handling Facility (CDHF) to key parameter data products (see below).  The rules for the data rights for the Wind and Polar missions are given in Appendix C.

The Japanese Geotail spacecraft will initially be operated in a double-lunar swingby orbit having a nightside variable apogee radius of 80 to 220 Earth radii (RE).  At a future date, it is to be placed into an 8 x 30 RE near-tail orbit.  Geotail observations in the Earth magnetotail provide essential science data products (hot tail plasmas, high and low energy particles, magnetic and electric field properties).  Seven flight instruments are used to produce these data products, four of which include a significant participation by U.S. scientists.  Measurements from Geotail are processed in the ISTP CDHF to key parameter data products (see below).  The rules for the data rights for the Geotail mission are given in Appendix D.

The data from the three correlative science missions, Wind, Polar, and Geotail, will be supplemented by data products (in the form of key parameters, see below) derived from ancillary observations taken by IMP-8, GOES 6 & 7, and the LANL spacecraft.  This data complement will provide magnetic field and plasma measurements from IMP-8, averaged magnetic field and energetic particle measurements from GOES 6 & 7, and particle data from the LANL spacecraft, covering different regions in the Geospace.

Ground-based investigations will provide scientific measurements of spatial and temporal variations of ionospheric parameters (such as electron density, electric fields, magnetospheric energy deposition, latitude profiles, Joule heating, and derived geophysical parameters).

Theoretical investigations will focus upon 3-D magnetosphere modeling, kinetic simulations, magnetosphere-ionosphere coupling, MHD (single fluid), and a thermosphere global circulation model.

.c2.2.2  Collaborative Solar Terrestrial Research Initiative

The missions that are part of the Collaborative Solar Terrestrial Research (COSTR) initiative are SOHO and Cluster.

SOHO is to be placed into a halo orbit about the sunward (L1) libration point and maintained in this orbit throughout its operational lifetime.  SOHO will produce science data products pertaining to solar wind plasma flow characteristics, coronal temperature, density, and velocity and with solar oscillations to deduce interior structure.  Data from the eleven flight instruments on SOHO will be accessible in the Experiment Operations Facility (EOF).  The scientific and technical aspects of the instruments on SOHO are extensively discussed in the ESA Special Publication compiled by Domingo and Guyenne [1989]. An overview of these products is given in Table 9.  The rules for the data rights for the SOHO mission are given in Appendix E.

Cluster has been baselined by ESA to consist of four spin-stabilized spacecraft, placed into nearly identical elliptical polar orbits approximately 4 x 22 RE with periods of about 66 hours.  The four spacecraft will be re-positioned to form vertices of a tetrahedron with spatial separations that can be adjusted during the mission.  Cluster will produce science data pertaining to electric and magnetic field properties such as currents, plasma waves, plasma densities, high energy electrons, and energetic ion distributions and composition.  The scientific and technical aspects of the instruments on Cluster are extensively discussed in the ESA Special Publication compiled by Schmidt and Goldstein [1988].

Each Cluster spacecraft will provide up to eleven flight instrument systems, including five component parts of the Wave Experiment Consortium.  The Cluster science data is to be captured by ESA and distributed to the various data centers of the Cluster Science Data System (CSDS) for processing into scientific data products (referred to as prime and summary parameters). An overview of these products is given in Table 10.  The rules for the data rights for the Cluster mission are given in Appendix F.

.c.3.0  Mission Science Objectives and Success Criteria

The science objectives for the Global Geospace Science Initiative (Wind, Polar, and Geotail spacecraft) are:

•	To evaluate globally the effects of solar activity on the Sun-Earth system;

•	To understand how individual parts of the closely coupled, highly time-dependent geospace system work together;

•	TO determine the control exercised by plasma processes in geospace of the energy input to the Earth’s atmosphere;

•	To understand fundamental physical processes that may occur in other astrophysical contexts; and

•	To provide a comprehensive data set for determining the accuracy and prediction capability of geospace models.

The minimum criteria for mission success are:

•	Acquisition and analysis of coordinated data sets from multi-spacecraft located in the four critical regions of geospace (solar wind, polar auroral magnetosphere, ionosphere, and the geomagnetic tail);

•	Acquisition and analysis of coordinated geophysical data from ground-based facilities to complement the spacecraft measurements; and

•	Development of the global theoretical and empirical models to assist in the interpretation of measurements.

These minimum success criteria will be met by continuing theory effort, ground-based measurements, and the successful launch and operation of the Wind and Polar spacecraft.  For the Wind spacecraft, specific minimum criteria require the instruments to provide a complete set of measurements that characterize the solar wind input (IMF, flow velocity, density, and composition) to the magnetosphere at about 1 minute time resolution.  For the Polar spacecraft, specific minimum criteria require the instruments to make remote imaging observations of the structure and morphology of the entire aurora zone and in situ particle, and electric and magnetic field measurements at about 1 (or a few) minute time resolution.

.c.4.0  Mission Operation Elements

The ISTP science initiative encompasses use of Level 0 (telemetry) data obtained from six active mission elements, correlative data from other mission sources, as well as ground-based and theory investigations, to produce key parameters, event data products, PI archive data products and models.

Figure 1 illustrates telemetry and Level 0 data flow for the ISTP core missions (Wind, Polar, Geotail, SOHO, and Cluster) and IMP-8.  Three space networks capture science telemetry and provide radiometric tracking of the spacecraft for orbit determination purposes.  The ad hoc NASA ground tracking network acquires IMP-8 telemetry data.  The NASA Deep Space Network (DSN) acquires Wind, Polar, and SOHO spacecraft telemetry (real-time and playback), and captures the Geotail tape recorder playback telemetry.  The Japanese ISAS space network acquires the real time Geotail science telemetry.  ESA will use its space network to acquire Cluster spacecraft telemetry.  Telemetry and radiometric data, initially captured at the network ground stations, are relayed through communications networks to data capture/orbit/attitude facilities for conversion to Level 0 data products that include the orbit and attitude ephemeris files.  Command history logs are maintained by the respective control centers and are incorporated as ancillary files into the Level 0 data products.

Telemetry streams, acquired by the NASA networks, are routed to the data capture facilities at Goddard Space Flight Center (GSFC) for Level 0 data processing.  These telemetry streams from Geotail (playback), Wind, and Polar, as well as Level 0 quicklook data products from the IMP-8 spacecraft, are electronically transferred to the NASA CDHF at GSFC, where they are maintained in short duration storage (approximately 2 years after acquisition).  Level 0 data for SOHO field and particle instruments are electronically transferred from the Packet Processing System (PACOR) to the CDHF and maintained in short duration storage (approximately 2 years after acquisition).  PACOR electronically transmits SOHO Level 0 data to the Data Distribution Facility (DDF) at GSFC.  The DDF accesses CDHF databases for transfer of other Level 0 data, all of which are processed onto media for mailing distribution (see below).  PACOR also routes pre-processed real-time telemetry, as well as “quick look” playback telemetry, to the Experiment Operations Facility (EOF).  The EOF electronically accesses SOHO orbit and attitude ephemeris databases in the CDHF to process summary data products, which are transferred to CDHF databases.  CDHF databases are maintained as short-term archives and are accessible to authorized ISTP investigators.

As shown in Figure 1, ISAS will acquire the high rate real-time telemetry data from Geotail and place this data into their Scientific Information Retrieval Integrated Utilization System (SIRIUS).  NASA will acquire the low rate playback telemetry data from Geotail which is then written to compact disk at the GSFC data capture facility.

ESA facilities at the European Space Operations Center (ESOC) are used to produce the Cluster Level 0, orbit/attitude, and command history ancillary data.  The Cluster data products will be distributed on Compact Disk-Write Once (CD-WO) media and electronically via the ESA SPAN network to the Cluster Science Data Centers and to the Cluster Principal Investigators (PIs).

The GSFC Science Planning and Operations Facility (SPOF) provides the focus for planning of instrument operations on Wind and Polar for NASA.  The Joint Science Operations Center (JSOC) at the Rutherford Appleton Laboratory (RAL) will be providing similar services for the Cluster spacecraft.  ISAS provides the focus for planning of instrument operations on Geotail.  ESA utilizes the EOF at GSFC for planning of instrument operations on SOHO.  Summary data products will be produced in the EOF and transferred to the CDHF as key parameter image data.

Figure 2 provides a high level overview of the ISTP science data flow.  A major ingredient of the processed data consists of key parameters (KP) produced from the Level 0 information.  KP data products are generated in the CDHF for Geotail, Wind, and Polar missions as well as for magnetic field data from IMP-8, and the particle instruments on SOHO. The KP data generated in the CDHF are based upon algorithms provided by the principal investigator teams.  These KP data are supplemented by IMP-8 KP plasma data which are generated by the PI and remotely networked to the CDHF for inclusion in the database.

Supplementary science data products will be used to augment those obtained from the active missions.  GOES 6 & 7 will provide key parameter data pertaining to magnetic fields and energetic particle measurements.  In addition, key parameter data from three to five LANL spacecraft will provide one- to three-minute averages for solar wind plasma and low energy particles, respectively.  The supplementary data will be generated at the investigator’s remote facility, networked to the CDHF, and maintained in CDHF databases for use by all the ISTP investigators.  In addition, Cluster summary parameter data will be produced in the Cluster Science Data Centers. NASA will receive this data via RAL placing it within the CDHF ISTP data base.

Ground-based investigations will produce key parameter datasets based upon Earth-based observations of the ionosphere, particularly in the high-latitude regions.  These datasets will be electronically transmitted to the CDHF and maintained in databases for use by the ISTP investigators.  The ISTP science initiative encompasses theory investigations to produce improved models of the 3-D magnetosphere, coupling with the ionosphere, thermosphere global circulation, and MHD simulations.  Datasets produced by these models will be maintained in the CDHF.

On a regular basis, the key parameter datasets will be transferred from the CDHF and copied, in the GSFC Data Distribution Facility (DDF), onto storage media (optical disks) that are mailed to the designated recipients.  Media are also used to provide processed data to the National Space Science Data Center (NSSDC).  The NSSDC also receives key parameter datasets via the local electronic network at GSFC from the CDHF 2 to 5 days after they are generated.  Cluster Science Data System (CSDS) data products would be received by the NSSDC on suitable media by agreements with ESA and the respective PIs.  Also, for certain data products, the NSSDC will have electronic access to datasets via the ESA SPAN and the NSI networks.

The SPOF will provide a focus for quality control monitoring of key parameter data products, using electronic access to the CDHF databases.  Remote Investigators may interface directly with SPOF or EOF for planning of instrument command sequences.

Event data products will be produced as part of an ISTP science initiative campaign activity or as the result of identifying scientifically interesting “events” from the key parameter datasets.  Campaigns involve multi-mission international collaboration, coordinated with the Inter-Agency Consultative Group (IACG) for space science, addressing specific scientific topics.

Generally, identification of “events” from key parameter datasets will result in further analysis at Remote Investigator facilities of the associated higher resolution instrument data contained in the Level 0 products.  Event data products resulting from these analyses are to be archived at the NSSDC where they will be made electronically available.

.c2.4.1  Central Data Handling Facility

The Central Data Handling Facility (CDHF) is the centralized computer system where measurements from the wide variety of geospace instrumentation, ground-based instrumentation, and theoretical studies are brought together and subsequently distributed, along with ancillary data, in a unified fashion, to the ISTP PI and Co-I teams for further analysis.  The combined data sets from the participating spacecraft, complementary ground-based observations, and theory

investigations will be used to obtain a more complete understanding of how mass, momentum, and energy from the solar wind are transported across geospace boundaries, stored and energized in the magnetosphere, and subsequently dissipated into the Earth’s atmosphere.

The CDHF (both hardware and software) has been designed to provide for flexibility and substantial evolution as the program proceeds, in order to accommodate the inevitably unforeseen developments.  The reader is referred to Lehtonen, [1993] or Mish, et al., [1993] for details on the architecture of the CDHF.

The major data services of the CDHF are summarized in Table 1.  Essentially, the facility ingests the telemetry streams (Level 0 data and SIRIUS), orbit, attitude, and command history from the Wind, Polar, Geotail, SOHO, and IMP-8 spacecraft, and KPs from a variety of other correlative sources, and outputs KP data sets, Level 0, SIRIUS and ancillary data.  In addition, network transfers of most data products can be scheduled on a recurring basis or done on an ad-hoc basis through the CDHF data services menu.  Section 5.2 discusses key parameter data content in more detail.

.c2.4.2  Data Distribution Facility

The Data Distribution Facility (DDF) provides the data distribution and delivery functions for the GSFC’s Information Processing Division (IPD).  In this way, IPD saves the expense of building, operating, and maintaining unique and distinct facilities and procedures for each mission, while gaining the ability to better serve its customers.  The DDF can be easily upgraded to new distribution technologies as they mature and are adopted as standard.

The ISTP data are transferred from the CDHF to the DDF over one of GSFC’s local area networks called InfoLAN.  From there, it is reproduced and distributed to the PIs and to the NSSDC.

The DDF is being developed in three phases.  Phase I started operations in 1992, supporting magnetic tape distributions for the Geotail mission.  This work served as a prototype for Phase II and III development.  Phase II allows ISTP distributions to be made on Compact Disk-Write Once (CD-WO).

Phase III incorporates the latest technologies and standards to provide an integrated and flexible system for supporting multiple missions with a range of physical and electronic distribution capabilities, while also greatly reducing the level of manual operations required.  It is intended that Phase III will be operational before the SOHO launch.

Phase II capabilities reside in three subsystems:  a Host Computer Subsystem (HCS), a Data Recording Subsystem (DRS), and the InfoLAN Mass Storage System (IMSS).  The HCS hardware consists of a VAX-11/785 and a VAX 6000/410 and their peripherals.  The DRS currently consists of three personal computers connected by a Novell network, magnetic disk storage, two Phillips CD-Write Drives, other peripherals (printers, diskette drives, etc.), and software.  The DRS and the HCS communicate using DEC’s Pathworks (subset of DECnet).  The DRS is designed to be easily expandable, not only with similar equipment, but with additional types of recording devices and additional types of computers.

For Geotail, Wind, and Polar, DDF will distribute at least ten distinct products.  Some of these products will be distributed on a nearly daily basis, while others may be distributed on a quarterly basis, based on both the capacity of the media (DDF’s current CD-WOs have a capacity of about 650 Mbytes) and the turn-around time needed.  Some of these products will be distributed to only a few investigators, while others may be distributed to as many as 100 investigators.

The DDF is currently producing approximately 8 CD-WOs per day for ISTP.  After Wind is launched in FY95, this will increase to 35 per day; and after Polar is launched in FY95/96, this will increase to 53 per day.  These numbers do not include the copies being duplicated at an external facility, or products being created for other projects.

.c2.4.3  Special Data Distribution Activities

Special data distribution activities have been established to meet the needs of other Federal Agency programs for near real-time data.  For instance, the magnetic field and plasma KP data from the SWIM experiment on the Wind spacecraft are computed by the CDHF within several minutes of real-time acquisition and sent electronically to the Air Force Space Weather Forecast Center via the Space Environment Laboratory (SEL) and to the Air Force Phillips Laboratory over the NASA Science Internet.

.c2.4.4  The Science Planning and Operations Facility

The Science Planning and Operations Facility (SPOF) is the component of the GGS (Wind and Polar) ground system responsible for the development and coordination of ISTP science planning operations, and operates under the direction of the ISTP Project Scientist.  The SPOF computer system consists of four UNIX-based workstations, one Macintosh and one personal computer interfaced via the GSFC Local Area Network with the CDHF and the GSFC Command Management System (CMS).  Figure 3 provides a schematic view of the SPOF functional interfaces.  SPOF is responsible for the development and coordination of the science plans for ISTP/GGS spacecraft.  In addition, the SPOF provides a communication link between the PIs and the CMS for the transfer of Wind and Polar instrument commands.  SPOF responsibilities fall logically into three main functions:  (1) science planning activities, (2) operations activities, and (3) science advancement functions; each of these functions is explained below.  For a more detailed explanation of the SPOF, the reader is referred to Mish et. al, [1993].

.c3.4.4.1  SPOF Science Planning Functions

SPOF science planning functions include:  (1) coordination of science planning for Geotail, Wind, and Polar;  coordination and planning efforts also involve ancillary spacecraft such as IMP-8, GOES, LANL and others as available; in addition, ground-based and theory elements are included as necessary through the ISTP Deputy Project Scientist for Theory and Ground-Based Experiments; (2) coordination of science operations for Wind and Polar; (3) use of ephemeris and model tools to generate science plans, including predicted “event” times; (4) generation of the Long Term Science Plan (LTSP) based on inputs from the GGS Science Working Group; (5) development of the BiWeekly Science Plan (BWSP) in coordination with the PIs and the CMS; (6) review and quality control checks on KP data; and (7) coordination with planning facilities from other projects on a regular basis and specifically during science campaigns.

Development of the LTSP is the responsibility of the GGS Science Working Group (SWG), but SPOF interacts with the SWG to facilitate its development.  During the periodic meetings of the SWG, SPOF presents results from its planning activities, including predicted crossings for key boundaries, magnetospheric region occupation reports, and anticipated magnetic conjunctions between spacecraft and/or ground-based stations.  Duration of the LTSP is approximately six months.  The plan is initially generated prior to launch, and subsequently updated after periodic meetings of the SWG and the PIs.  Records of accomplished activities are fed back into the plan for accounting purposes.  An up-to-date copy of the LTSP is placed in a SPOF directory on the CDHF to allow individual investigators to review the LTSP via their network connection.

SPOF is responsible for the development of the BWSP in coordination with the PIs and the CMS.  Specifically, SPOF insures that the BWSP is free of science-related conflicts and complies with the

LTSP.  Validation with respect to engineering constraints is carried out at the CMS, and folded into the BWSP by the SPOF.  The BWSP is updated by SPOF on a weekly basis and includes:  (1) requests from the Wind and Polar PIs for instrument activities and/or instrument reconfigurations; (2) inputs from the Project Scientist and SWG; (3) instrument coordination procedures; (4) procedures for coordination with other missions when appropriate; and (5) inputs from GGS-related activities.  As with the LTSP, PIs will have access to a copy of the BWSP via the CDHF.

.c3.4.4.2  SPOF Operations Functions

SPOF is responsible for the following operations functions:  (1) maintaining a communication link with the RDAFs via the NSI network for receiving instrument commands; (2) ensuring security of this interface; (3) receiving all Wind and Polar instrument command requests, microprocessor loads, and any special scheduling event definitions from the PIs via the RDAFs; (4) checking commands for compliance with the science plans, time tagging if required, and resolving any science conflicts; (5) forwarding all Wind and Polar instrument command requests and microprocessor loads to the CMS for engineering constraint validation; (6) obtaining command history, daily mission schedule, and DSN schedules from the CDHF; (7) accepting validation reports from the CMS; (8) developing and maintaining the SPOF databases and the associated reports placed in a SPOF directory on the CDHF, e.g., LTSP, BWSP.

Instrument command requests (ICRs) transmitted by the PIs to the SPOF are checked for science conflicts and compliance with the overall science plan, and subsequently are transmitted to the CMS for validation with respect to engineering constraints.  Upon science and engineering validation, requests are incorporated into the BWSP and the detailed operations schedule.

The Interface Control Document (ICD) between the CMS-SPOF-PIs describes in detail the file formats used for ICRs; command mnemonics are interpreted using the mnemonic dictionary contained in the project data base resident on the SPOF.

Microprocessor data, generated by the PIs for their instruments, are composed of instructions and/or data to be stored in the memory of their onboard instrument microprocessor.  It is the PI’s responsibility to validate the microprocessor data and to segment it into loadable blocks of bytes.  The SPOF performs no value-added functions for microprocessor loads, but just passes them on to CMS.

.c3.4.4.3  SPOF Science Advancement Functions

SPOF Science Advancement Functions involve active participation of SPOF scientists in ISTP related research.  SPOF researchers are currently involved with the ISTP/GGS Theory Program, where they are collaborating with ISTP Theory Investigators, under the direction of the ISTP Deputy Project Scientist for Theory and Ground-Based Experiments, on the assessment and comparison of various global magnetospheric models based on both data as well as MHD simulations, in order to evaluate their value as quantitative guides in gluing together the ISTP spacecraft observations.  Furthermore, research collaborations with various ISTP scientists are underway, and it is anticipated that more will emerge as the project unfolds.

SPOF personnel are active members of the ISTP/GGS Theory Group working under the direction of the ISTP Deputy Project Scientist for Theory and Ground Based Experiments.  The Theory Group is concentrating on the comparison and evaluation of global magnetospheric models.  Initial tests involved inter-calibrations between magnetohydrodynamic (MHD) simulations, and comparisons between empirical magnetic field models and MHD simulations.  The most significant improvement to the MHD simulations has been the extension of the modeled domain to 300-400 RE in the tailward direction.  Several key tests have been devised to validate these simulations by the GGS Theory Group.  Results from the MHD simulations will be made available to the ISTP

community via the CDHF; in addition, SPOF will integrate such MHD results into its scientific visualization mission planning tools.

Global magnetic field models are among the essential tools required for SPOF operations.  The widely used Tsyganenko models [Tsyganenko 1987, 1989 & 1990] have been incorporated into the SPOF, and enhancements to these models are under development.  As new models become available and are validated, they will be incorporated into the suite of tools available to the SPOF for science planning and data analysis.

Effective mission planning tools are essential to SPOF operations.  The Satellite Situation Center (SSC) software [Peredo, et al., 1992a; Parthasarthy, et al., 1992; Aist-Sagara, et al., 1993] provides a key baseline capability.  SSC applications represent the core set of ephemeris tools, capable of generating reports of magnetic conjunctions, region occupancy, boundary crossings, coordinate conversions and limited graphical displays.  The SSC tools have been updated to include the most recent magnetic field models, up-to-date definitions for key magnetospheric regions, and to create a graphical user interface. The SPOF has developed additional visualization analysis tools to compliment the SSC.  These tools are available from the SPOF.

.c2.4.5  SOHO Experimenters Operations Facility

The SOHO EOF functions are described in detail in the document by Poland and Domingo [1993].  The EOF is to be located at NASA/GSFC and will serve as the focal point for mission science planning and instrument operations.  Most EOF elements will be physically located at the SOHO Solar Data Analysis Center (see below).  From the EOF, experiment PI representatives will receive real-time and playback flight telemetry data, process these data to determine instrument commands, and send commands to their instruments, both in near-real-time and on a delayed execution basis.  They will be able to perform data reduction and analysis, and have capabilities for data storage.  Workstations connected to an EOF Local Area Network (LAN) will support the Project Scientists and planning and operations staff in the EOF, and there will be connections to external facilities to transfer data from GSFC support elements, remote investigator institutions, other science observatories, and ESA facilities.  The EOF will also have connections to the SOHO Mission Operations Control Center (SMOCC) and other required elements at GSFC for scheduling and commanding the SOHO flight experiments.

The functions of the EOF relevant to this Data Management Plan are detailed in the document by Domingo et al., [1993].  Chapter 3 of this document describes the data sets available in the EOF and at the different SOHO facilities.  NASA will forward real-time Level 0 (= depacketized) data to the EOF over the LAN to the PI workstations with no additional front-end processing for quality assessment.  Playback data will be sent to the EOF in the same manner with transmission delays from DSN (approximately 3 hours) and processing delays to turn the data around (approximately 2 hours).  Level 0 housekeeping data are to be treated in the same way, and will be available in both raw counts and engineering units.  A data set called ancillary data also will be collected, consisting of various parameters relating to spacecraft condition.  Some of these ancillary data parameters will be transmitted to the Data Distribution Facility (DDF) for distribution on hard media to the PIs.

Summary data will be prepared to be used both to plan observations at the EOF and to provide an overview of the observations that have been obtained from SOHO.  The EOF also will handle some processed science data products, but it has not been determined if a central archiving facility for the processed data products will be developed (at the EOF or NSSDC, for instance).

.c2.4.6  Cluster Joint Science Operations Center

The Joint Science Operations Centre (JSOC) is located at the Rutherford Appleton Laboratory in England.  The purpose of the JSOC is to optimize the scheduling of all Cluster instrument plans to

maximize the scientific return.  In essence, the JSOC has similar responsibilities for the Cluster mission as the SPOF does for GGS.  (see section 4.4).

.c.5.0  Data Products

Section 5 discusses the details of the generation and composition of ISTP data products.  Section 7 specifically addresses the accessibility of the ISTP archival data products. Table 14 provides an overview of the ISTP archival data products and associated software along with a summary of their public accessiblity.  It is important to note that not all the data products from all the ISTP instruments will follow the accessibility overview given in Table 14 due to a variety of funding and instrument specific calibration processes.  Therefore, Table 14 should be used as a general guide to the ISTP data products.

.c2.5.1  SIRIUS Data

At this time there are no plans for archiving the SIRIUS data at NASA.

.c2.5.2  Level 0 Data

The CD-WOs produced by the DDF conform to the ISO 9660, which defines both the physical and logical format of the CD-WO.  This ensures that most CD-WO drives on most platforms will be able to read the disk.  In addition, the CD-WOs produced by DDF follow other emerging NASA standards, guidelines, and practices, especially use of SFDUs, and a file naming and directory structure which will be compatible with most platforms.

.c2.5.3  Key Parameters

In order to accomplish the ISTP scientific goals of correlative analysis, the ability to quickly survey a vast array of scientific data being gathered by the many spacecraft and ground-based data sources is essential.  The key parameter (KP) data product was designed to accomplish the survey task.  In general, KPs are low resolution time series (on the order of 1 minute) computed from each instrument.

Both internally- and externally-generated KP data sets adhere to mutually agreed upon standards and conventions to permit rapid analysis and comparisons of the over 100 KP data sets from a variety of instrumentation.  The more important of these standards and conventions are listed in Table 2.  All KP data sets are created as Common Data Format (CDF) files with ISTP Guidelines (see Kessel et al., 1994).

KPs computed internally to the CDHF are based on Level 0 (decommutated telemetry) or SIRIUS data (full Geotail telemetry stream) as input.  There are also KPs generated externally to the CDHF which are ingested from a variety of ground-based instruments and non-ISTP spacecraft (see next sections).  In general, each KP data set covers one complete day.  Internally-generated KPs are available within a few days of real-time acquisition.  The delay from real-time for externally-generated KPs can range from near-real-time to several weeks, depending on their source.   For rapid access, KPs residing on the CDHF are also accessible via standard computer networks (both DECnet and TCP/IP protocols are supported) approximately 1 hour after generation or ingestion into the CDHF.

Software for internally-generated KP production is provided by the PI team for each instrument and is integrated into the CDHF production environment prior to the launch (in most cases) of each spacecraft (Wind, Polar, Geotail, SOHO, and IMP-8).  The software and associated documentation will be archived at the NSSDC.

The KP data product summaries for Wind, Polar, Geotail, and SOHO are given in Tables 3, 4, 7, and 8, respectively.

The first 6 months after instrument turn-on is a period of time set aside to allow these PI teams to come to a better understanding of the on-orbit operation of their instrumentation and thus refine the KP software appropriately.  It is also anticipated that additional refinement of the KP software will take place over the life of the ISTP mission, so the CDHF environment has been designed to allow for rapid update of KP software, and the ability to handle multiple versions of the same KP data set, which can exist because of reprocessing.

Quality information is available for each KP data set in the CDHF catalog.  Each KP data set has quality flages that are generated and embedded in the KP data records as part of the routine processing.  Once the PI groups have had a chance to review their KP data, they then put the additional quality information into the CDHF data set catalog.  This latter quality information may be days to weeks after the KP data files have been generated, and contains the information on the KP software version.

The residency time for KP data sets on the CDHF storage is ~ 1 year.  CD-WO disks will be generated by the DDF for distribution to the ISTP scientific community and NSSDC typically every 90 days.  The long time period is due to the likelihood of reprocessing the KP data after the PI teams have done their quality evaluations.  The DDF will write onto the CDs the most recent version of the KP data.

The ISTP CD-WOs will use the ISO 9660 Level 1 standard to provide compatibility across major computing platforms (for more details on CD-WO formats, see Mish, et al., 1993).  In addition to the data files, each CD contains an ASCII file listing the contents of the CD and a cumulative index of all files distributed to date on CD.

Level 0, SIRIUS, orbit, attitude and spin phase data are used as input into the PI-provided software that produces the CDHF internally-generated KPs.  The KPG software, which can be written in “C” and/or FORTRAN (ANSI 77), does not directly interface with the VAX/VMS operating system, but resides in a shell that provides some isolation.  Thus, input and output to the KP software is accomplished using various “ISTP CDHF Software System” (ICSS) “calls” which, in turn, interact with the operating system.  For example, the Level 0, orbit, attitude, and spin phase data required by a particular set of KPG software would be obtained via these ICSS calls.  Coordinate transformations are also available via ICSS calls.  Output to CDF files is through calls to the CDF software.

Also ingested by the CDHF are externally-generated KPs from the Ground-Based Investigations (GBIs), SOHO spacecraft (some SOHO KPG software execute in the CDHF), Cluster , GOES 6 & 7 spacecraft, LANL89, LANL90, and LANL91 spacecraft, the IMP-8 spacecraft, and the Interball mission (only during IACG Campaign times, see section 6.2).  These KPs are sent to the CDHF via NSI.

The KP data product summary for ground-based data and supplementary missions data are given in Tables 4 and 5, respectively.

Key Parameter data that reside in the CDHF are reviewed by the SPOF both for monitoring the quality of the KP products, and to identify events of scientific interest.  Identification of unusual values in a KP data set and/or irregularities will result in electronic mail from SPOF to the appropriate PI.  While the SPOF routinely spot-checks KP quality and alerts investigators of anomalies, in all instances, the PIs are responsible for evaluating if individual or global reprocessing of the KP data is necessary.

Future funding for the U.S. instrument investigators will depend on their ability to provide open access to KP data from Geotail, Wind, and Polar. Investigators can choose to provide open access to the science community at large to their KP data through the World Wide Web (WWW).  These data should be made available within 3 days after they have been generated in the ISTP/CDHF or SPOF homepages. The non-citability of this data will be prominently noted in the interface.

.c2.5.4  Citable Key Parameters

It is expected that the quality of the KP data will improve as the investigators learn more about their instruments from in-orbit calibrations and therefore improve the algorithms in the CDHF which generate these data.  The creation of additional citable KP data may also result from reprocessing in the CDHF.  Future funding to U.S. principal investigators will depend on their ability to provide open access and production of KP data of publishable quality (called citable KP data) from ground-based instruments, the theory group, Geotail, Wind, and Polar.  All citable KP data sets are also created as ISTP Common Data Format (CDF) files.

Accessibility to citable KP data by the science community at large will be at the instrument investigators’ discretion and could be made available over the WWW as the non-citable KP data discussed in the previous section.  Because pre-launch funding for key parameters has been omitted for most investigators (for Wind), it is recognized that there will be a significant delay, perhaps up to 18 months, after launch before citable KP data will be produced for some instruments.

.c2.5.5  Event Data

As mentioned above, the KP data sets will be used to identify interesting scientific “events.”  In general, this identification will result in analysis of the associated higher resolution instrument data by the PI teams at their remote data analysis facilities (RDAF) using Level 0 or SIRIUS data.  The higher resolution data products generated by this analysis are called event data.  Event data products will also be created as CDF files and submitted to the CDHF for subsequent joint analysis and distribution on CD-ROM.

.c2.5.6  Other Data Products

Each of the investigators has facilities at his or her institution that are used in processing, analyzing, and correlating ISTP data.  In some cases, it is expected that some investigators will routinely generate additional instrument data products.  These products along with associated documentation and the generation software will be delivered to the NSSDC for long-term archiving and community-wide distribution.  What these products are and how they will be managed is TBD but decisions should be made approximately 18 months after launch of Wind and Polar.

.c2.5.7  Archival Data Volume

Level 0 telemetry data is archived at the NSSDC.  The basic science data rates are estimated at:

Spacecraft	Science Data Rate	Approximate archive size

Wind	4.8 kb/s	19 GBytes/year

Polar	51.5 kb/s	202 GBytes/year

Geotail	16 kb/s	63 GBytes/year

When the Wind spacecraft is inside a radial distance of  ~60 Earth Radii (RE) the science data rate will probably be twice the above stated rate.  In addition, the VLF wide-band data rates will be:

Wind	64 kb/s @ 1-4 hours/day (currently planned)

Polar	250 kb/s @ 3.6 hours/day (12 hours/day for 1st month of mission)

It is estimated that once Geotail, Wind, and Polar are fully operational, including the ground-based data, that approximately 20 megabytes/day of KP or citable KP data will be generated.  This will amount to approximately 10 gigabytes/year to be kept online in the NSSDC archive (see section 7).

The proprietary data rights for all Wind and Polar data are given in Appendix C and for Geotail in Appendix D.

.c2.5.8  SOHO Data Products

The SOHO data products are described in more detailed in Domingo et al., [1993].  In brief, the SOHO summary data will consist of three classes of data.  The first two classes will consist of representative images from each of the imaging experiments (in SFDU/FITS), and key parameter data from the non-imaging experiments (in SFDU/CDF).  Both these data types are shown in Table 8.  The third class will be lists of observing programs and start/stop times of data sequences. Serving as a synopsis of solar conditions and the science programs that have been carried out by SOHO, these summary data will be available on-line in the EOF for 28 days and will then be transmitted to the DDF for distribution on hard media (CD-WO) to the PIs.  Due to their quick-look, unverified status, the summary data will not be citable.  An event log will be maintained in the summary data file and be distributed by the DDF, but the contents of the log will be allowed to change, reflecting revisions to include events identified by observatories other than SOHO.

The summary data also are to include planned and actual observing programs, describing the instrument operation modes and heliographic areas covered.  The summary data are to be archived in the permanent SOHO archive at NSSDC.

Each PI team will transform the Level 0 data and related data into processed data files, or they will maintain software which is used to perform direct scientific analysis with Level 0 data.  The details of the archived science data files may be found in the individual experiment Operations Manuals.

The processed science data files that are expected to be generated are given in Table 9.  In addition to the science data products, various catalogs are to be compiled in database tables that are to be linked together using standard relational database techniques.  The information in these database tables includes general information about observing programs, specific information about the actual observing sequences used to carry out those observing programs, and information about events that may be relevant to more than one instrument.  A standard query language (e.g., SQL) will be used to access the databases.  These catalogs may be accessed electronically over the EOF LAN.

All of the data products described in this section are to be accessible during the lifetime of the SOHO project at the SOHO Solar Data Analysis Center (see section 6.3 below) and permanently archived at NSSDC.

.c2.5.9  Cluster Data Products

It is expected that a wide scientific community from over 20 nations will be using Cluster data.  The Cluster mission will produce several types of data products that will be processed and distributed from the data centers which are part of the Cluster Science Data System (CSDS) [see for example, Schmidt, R., and L. J. C. Woolliscroft, 1993].

The Summary Parameter Database (SPD) is comparable to the KP data generated in the GGS and Geotail missions.  These data are approximately 60 scientific and ancillary parameters at 1 minute resolution from a single Cluster spacecraft and will be widely accessible for selection of events. The SPD parameters are listed in Table 10.  In addition to the digital SPD, Summary Data Plots (SDP) will be generated at the Cluster Science Data Centers and also used for event selection.  The SDP are mainly line plots of the SPDB parameters with temporal resolutions of between 30 and 60 seconds.

The Prime Parameter Database (PPDB) will be about 60 scientific parameters at a temporal resolution of about 4 seconds (approximately one spin period).  These data are generated from each instrument on all four spacecraft and will contain ancillary spacecraft information.  The PPDB will be available to all Cluster Investigators through the CSDS.

The highest resolution data from Cluster will not be part of the ISTP CDHF (which is also serving as the U.S. Cluster Science Data Center), but will reside at the PI institutions.  The PIs are to respond to requests from the science community for individual high resolution event data.

.c2.5.10  Solar Wind Catalog

Detailed knowledge of the solar wind conditions upstream of the magnetosphere is essential to the understanding of the transfer of energy across the solar wind-magnetosphere-ionosphere system.  As such, widespread interest in solar wind information is anticipated across the ISTP and science communities.  A collaboration has been established between SPOF and Dr. R. Lepping to jointly develop a catalog of solar wind events observed by the IMP-8 and WIND spacecraft.  This catalog will be made available to ISTP scientists through the CDHF to aid in the correlative analysis and will be archived at the NSSDC.

.c.6.0  Correlative Science Data Management

.c2.6.1  Campaign Coordination Functions of the SPOF

The SPOF is responsible for the science planning and coordination of GGS campaigns; in addition, SPOF works closely with the science planning facilities from non-ISTP programs such as those from the Inter-Agency Consultative Group (IACG) participants (see below) in order to promote optimal use of resources and effective coordination across these programs.  In the campaign mode, SPOF operates according to its nominal operations schedule, but with special emphasis on verification that all instruments operate in modes compatible with campaign goals, unless there are instrument health/safety issues.

.c2.6.2  Inter-Agency Correlative Science Activities

The IACG for space science was formed in 1981 by the scientific heads from four space agencies:  ESA (European Space Agency) of Europe, IKI (Institut Kosmicheskikh Issledoranii, i.e., Space Research Institute) of Russia, ISAS (Institute of Space and Astronautical Science) of Japan, and NASA of the United States.  The main goal of the IACG was to increase the overall scientific return from their missions through cooperation among the agencies.  In the early 1980s, the IACG coordinated efforts to study Halley’s Comet.  The success of the coordinated effort of observations on Halley’s Comet provided the basis for further cooperation.  In 1986, IACG adopted solar-terrestrial science as their next science focus.  In the 1990s, the four member agencies have up to 25 projects involving over 35 spacecraft that are either fully or partially dedicated to studies of the solar-terrestrial environment.  A core set of spacecraft has been defined to play the major role in coordination effort.  The primary spacecraft to be included in the core group are the Wind and Polar (from NASA), Geotail (from ISAS), the four Interball (from IKI), and the SOHO and the

four Cluster spacecraft (from ESA).  It is important to note that the four Interball spacecraft have been added to the ISTP core missions to make up the IACG core missions.

Implementation plans for the first and the second IACG campaigns have been completed.  The first IACG campaign will focus on the study of magnetotail energy flow and non-linear dynamics [see Whipple and Lancaster, 1992].  The second campaign plan is on boundaries in collisionless plasmas [see Whipple and Lancaster, 1993].  The third IACG campaign has been defined as “solar events and their manifestations in interplanetary space and in the Earth’s magnetosphere,” but an implementation plan has not yet been completed.  The fourth IACG campaign is on “solar-heliospheric structures out-of-the ecliptic.”  An implementation plan for this last campaign should be completed by April 1994.

For the campaigns to be successful, it is recognized that:

•	The data exchange between mission communities be facilitated by harmonized data systems.  The exchange of higher-resolution data than key or summary parameters for the periods of coordinated measurements should be envisaged and is currently being planned.

•	The mission-oriented theory and numerical simulation communities be involved in the planning and execution of campaigns.

•	The individual mission-related ground-based observation programs be coordinated and integrated into the ISTP data management activities.

•	The agencies’ science mission planning facilities be available for predictions of the proper spacecraft constellations.

Specific science topics identified for the first campaign included:  definition of structure and energy flow through the boundary layers of the Earth’s magnetosphere, definition of the quiescent extended configuration of the magnetotail and its dependence on ambient solar wind plasma and fields, the origin of plasma in the magnetospheric plasma sheet, mapping the response of the magnetosphere to externally-driven disturbances, and understanding the physics of internal magnetospheric instabilities and energy release.  The key spacecraft for this campaign will be Geotail (in the deep tail), Wind (monitoring the solar wind), Polar and Interball/Aurora (measuring auroral forms and particle precipitation), and Interball/Tail (in the near-Earth tail region). Key parameter data from both Interball/Tail (see Table 12) and Interball/Aurora (see Table 13) will be generated in Russia and networked to the ISTP/CDHF for data management and accesses.

The SPOF will help coordinate campaign planning by combining spacecraft ephemeris and model software to predict advantageous times for campaign intervals.  As discussed above, the SPOF uniquely combines spacecraft ephemeris and model software to predict intervals when the spacecraft are in advantageous positions for scientific studies, and presents the results to the IACG Science Working Group Teams for campaign planning.  The SPOF plays an essential coordination role of working closely with the science planning facilities from the other agencies.

Once the planning is complete and the data have been acquired, the ISTP key parameter data will be reviewed to identify specific intervals for detailed investigation.  Additional data sets, such as those resident at NSSDC or at other investigator facilities, will be incorporated into the event identification and detailed analysis processes in accordance with agreements between members of the IACG science community.  Detailed event data will be analyzed and inter-compared, obtaining a scientific understanding that could not be accomplished by one instrument or spacecraft data alone.  The science analysis will be accomplished through a series of Coordinated Data Analysis Workshops (or CDAWs).  It is envisioned that the CDAWs will be both centralized and

distributed, being hosted by more than one agency.  These CDAWs will make use of existing data centers and facilities necessary to manage and manipulate the large volumes of data expected.  The rules for data rights for the IACG 1st Campaign are given in Appendix G.

.c2.6.3  SOHO Solar Data Analysis Center

The Solar Data Analysis Center (SDAC), currently located at NASA/GSFC, is intended to become the SOHO mission data analysis center and main archive site.  Originally created to serve the EOF of the Solar Maximum Mission in 1979, the SDAC currently serves as main distribution center for CGRO/BATSE solar hard X-ray data, Yohkoh data, and SMM data.  Current plans call for the SDAC to move from its present location in building 21 to building 26 (by January 1995), where it will be co-located with the ISTP facility there.  The SDAC will continue supporting access and guidance in using solar archive data the SDAC already holds, while supporting new SOHO observing plans and archiving.  That is, the SDAC will be the site of most SOHO science planning EOF functions, whereas daily/hourly/near-real-time SOHO operations are to be controlled from the EOF at GSFC building 3.  The SDAC includes workstations, data storage facilities and personnel to carry out these functions.

.c2.6.4  Cluster Science Data System

There will be eight Cluster data centers which make up the CSDS. Nearly all of the Cluster data centers are responsible for generating and distributing a number of data products from specific Cluster instruments.  Each PI is associated with one of these data centers.  The data centers and the associated instruments are listed in Table 11 (see Daly et al., [1993] for more details).  Not all the Cluster data centers offer the same services.  For instance, the U.K. data center at RAL will provide an on-line archive of the summary and prime parameter datasets, while the German data center at MPE Garching plans to hold little reduced data on-line, but to hold access software to the Level 0 data.  The rules for data rights for the Cluster mission are given in Appendix F.

.c.7.0  Data Archiving and Distribution

The raw and processed data generated in the CDHF from the NASA/GGS spacecraft are being routinely archived at the NSSDC for long-term access.  These data products come to the NSSDC in two distinct ways.  The NSSDC receives the CD-WOs generated from the DDF.  As discussed earlier, these CDs contain KPs, decommutated telemetry, and associated ancillary data.  The CD-WO data from the DDF is managed and held in an off-line archive. The NSSDC also receives, on a daily basis, KP data that exceeds a few days old from the time they are generated on the CDHF.  These KP data are placed in an on-line archive system and are accessible to registered ISTP users and IACG campaign participants.  These data are being managed as proprietary data products and will not be released to the general scientific community until the proprietary period has expired.  The length of the proprietary period is TBD.  In addition, the event data will also be stored in the NSSDC on-line archive facility when it becomes available.

.c2.7.1  Data Archive Products Contents and Disposition

An overview of the data products generated from Wind, Polar, Geotail and their disposition is shown in Table 14.

.c2.7.2  Archive Data Access

The NSSDC’s on-line archive facility that is being used for the rapid access to all the archived KP data is called the NASA/NSSDC Data Archive and Distribution Service, or NDADS.  The purpose of the NDADS system is to manage public data and data that will eventually be made public.  Since

the CDHF’s primary purpose is to process and distribute the most recent ISTP data, it is not designed to manage all the processed and archive ISTP data.  ISTP investigators wanting retrospective ISTP data will then have to access the NSSDC’s archive.  NDADS provides archive data services not only to ISTP scientists, but also to a worldwide science user community.  Access to ISTP data (after the proprietary period) by the general science community will be through the NDADS system and not though the CDHF.  For more details on the NDADS system architecture, the reader is referred to Mish, et al., [1993].

One of the most important features of the NDADS system is that it provides the capability for scientists to retrieve data from the archive by several methods.  The public data archive can easily be accessed through the automated retrieval mail system, or ARMS [Van Steenberg, 1993].  Users initiate a data request from their remote machines using a specifically formatted mail message.  Once a request has been initiated, the data are quickly retrieved from the optical storage system and placed in a staging disk on NDADS, or sent automatically to the remote user’s location via networks (such as the NSI-DECnet and the TCP/IP Internet).

For the proprietary ISTP data, interactive accounts are being established on NDADS to authenticate users.  Authorized accounts for accessing ISTP proprietary data are approved by the NASA/ISTP Science Office.  The interactive NDADS interface for accessing the ISTP data provides a full help facility, data selection, electronic data ordering, data browse, and is compatible with VT100-type terminals up to X-terminals.  An interactive interface for accessing and visualizing various aspects of the public data on NDADS is currently under development and will be heavily based on the existing ISTP interface.  The NDADS system is operational nearly 24 hours/day, 7 days/week and is a major archive and data distribution facility on the Space Physics Data System.

.c.8.0  Project Data Management Documentation

An extensive amount of data management documentation for the ISTP mission is currently available.  Appendix H provides the current list of ISTP data management documentation.

.c.9.0  References

Daly, P. W., W. Baumjohann, J. Dandonras, K-H. Glabmeier, C. Harvey, S. Schwartz, K. Tokar, L. J. C. Woolliscroft, “Report of the data products task group for the Cluster Science Data System,” DS-MPA-TN-001, February, 1993.

Domingo, V., A. Poland, B. Fleck, and T. Appourchaux, “SOHO Science Operations Plan,” Issue 1.1, June 1993.

Domingo, V., and T. D. Guyenne, editors, “The SOHO Mission,” ESA SP-1104, 1989.

Kessel, R. L., R. E. McGuire, H. K. Hills, and N. J. Love,  “CDF Implementation Guidelines for ISTP”, Version 2.3,  February 7, 1994.

Lehtonen, K., “Architecting the International Solar-Terrestrial Physics (ISTP) Central Data Handling Facility (CDHF),” GSFC Technical Document, June 1993.

Mish, W. H., M. Peredo, J. L. Green, and M. G. Reph, “ISTP Science Data Systems and Products,” submitted for publication, 1993.

Poland, A., and V. Domingo, “SOHO Experiment Operations Facility Functional Requirement Document,” Draft Version 1.0, 1993.

Schmidt, R. and M. L. Goldstein, editors, “The Cluster Mission,” ESA SP-1103, 1988.

Schmidt, R., and L. J. C. Woolliscroft, “Cluster Science Data System - A distributed system to serve a wide international community,” 1993.

Whipple, E. C., and H. Lancaster, editors, “Toward A New Era of Global Solar-Terrestrial Research - The First IACG Campaign:  Magnetotail Energy Flow and Non-Linear Dynamics,” Science Applications International Corporation (SAIC), October 1992.

Whipple, E. C., and H. Lancaster, editors, “Toward A New Era of Global Solar-Terrestrial Research - The Second IACG Campaign:  Boundaries in Collisionless Plasmas,” Science Applications International Corporation (SAIC), October 1993.

.c.Acknowledgments

Comments by David Batchelor, Evan Eller, Shing Fung, Joseph Gurman, Mona Kessel, Joseph King, Robert McGuire, William Mish, and Arthur Poland are gratefully acknowledged. In addition, I would like to gratefully acknowledge Mauricio Peredo for developing and maintaining the GGS Key Parameter summary tables used in this report.  I would also like to acknowledge the members on the SAIC staff for their assistance in compiling the information in the brief instrument descriptions for Wind and Polar, given in Appendices A and B, respectively.

.c.Figure Captions

Figure 1 - The level 0 data capture facilities for the ISTP mission.  The ESA network is used to track and capture the Cluster spacecraft and instrument data.  The NASA DSN is used to track and capture the Geotail, Wind, Polar, and SOHO spacecraft and their instrument data.  The NASA STDN system is used to track and capture the IMP-8 spacecraft and instrument data.  The ISAS network tracks and captures the Geotail spacecraft and high rate data.

Figure 2 - The process data flow for ISTP.  KP data from SOHO, ground-based instruments, IMP-8, GOES 6 & 7, and the LANL equatorial spacecraft are generated on computer systems external to the CDHF.  SOHO KP data will be processed in the EOF and in the CDHF (three instruments).  The SOHO KP data processes in the EOF will be networked to the CDHF for data management and distribution.  The KP data from the CDHF is then sent to the DDF and NSSDC facilities.  The CSDS will provide Cluster data to the NSSDC.

Figure 3 -  SPOF Functional Interfaces include transmission of instrument command requests and science coordination plans with the GGS Science Working Group and PIs at their RDAFs.  Wind and Polar commands, command validation and schedule submissions are exchanged with the CMS.  Key Parameter, ephemeris and ancillary data are extracted from the CDHF, while SPOF reports are placed on the CDHF for access by the PI teams.  Coordination of science planning activities takes place between SPOF and the SSC.

.c.Table 1 - Data Services at the CDHF

Service for accessing and transferring data

•	Provides an electronic interface with the RDAFs for support of remote login, file transfer, and electronic mail.

•	Data can be marked proprietary.

•	Transfers of data can be scheduled to occur at specified times.

•	Queries for data can be scheduled to run at regular intervals with the data automatically sent to a specified destination.

•	On-line, limited plotting of orbit and KP data.

•	Quick-look instrument telemetry available in “near real time” over network.

On-line retention of data

•	KPs on-line for at least 1 year.

•	Orbit, attitude, and command history on-line for at least 1 year.

•	Telemetry data on-line for 8 days.

Database queries and reports

•	Users can query the CDHF databases for information about ISTP data sets.

•	Users can generate reports from these databases.

•	Authorized users can update database entries to indicate quality of KP data.

Infrastructure support

•	Provides a means of monitoring/accounting for the status of data import/export, data processing tasks, and network activity.

•	Provides a data store for the SPOF.

•	Provides both a KPGS development environment and a KPGS production environment.

•	Provides a scheduler to handle recurring tasks and recurring network activity.

•	Provide a computational platform for parameterized theoretical models.

.c1.Table 2 - Key Parameter Standards and Conventions;

•	KP time resolution is on the order of 1 min (actually, many KPs are time tagged using multiples of 1/2 the Major Frame rate of the spacecraft.  Major Frame rates are:  Geotail - 16 sec; Wind - 92 sec or 46 sec; Polar - 9.2  sec; IMP 8 - 1.28 sec) and is generally time tagged to the middle of the measurement.

•	KP time tag is presented in two  separate representations in each KP record.

- ISTP Standard Time (Time_PB5) - An intuitive representation (Year, Day, & elapsed ms of day).

- NSSDC Standard Time (Epoch) - A representation easy to use in computing and display elapsed ms since AD.

•	Missing or bad KPs are represented with standard “fill” values.

•	Coordinate systems for KPs are left to the discretion of the PI teams - GSE & GSM are common systems used by many of the KPs.

•	Externally generated KPs always have spacecraft position and attitude embedded in each KP record, if appropriate.  Again, the particular coordinate system is left to the discretion of the PI team.

•	Each KP record contains quality information; also the CDHF catalog stores quality information for each individual KP data set.

•	Each KP record contains a Post Gap flag that explains, if appropriate, the reason a gap in data coverage occurred.

•	Generally KP data sets span one day.

•	All KP data sets (both external and internal) are presented as CDFs with detached SFDUs.

•	KP software is written in FORTRAN and C.

.c1.  Table 3 - Wind KP Data Product Summary;

| Investigation | Key Parameters | Time resolution |
| --- | --- | --- |

| WAVES | Ne, Peak, avg. for E,B per freq decade | 2 min |
| --- | --- | --- |

| SWE | Bulk velocity: (Vx, Vy, Vz)GSE, (Vx, Vy, Vz)GSM, (V,Vlong, Vlat)GSE, proton density, 100 * (alpha part. density/proton density) Most probable proton thermal speed Delta time (1/2 length of spectra) (X,Y,Z)GSE, (X,Y,Z)GSM, R | At least every 2 min |
| --- | --- | --- |

| 3-D PLASMA | e flux: 7 energies (0.1-225 keV), ion flux: 7 energies (0.07 - 400 keV) Ne, Ni, Ve, Vi, Te, Ti, e heat flux along B, Vs/c (GSE), (X,Y,Z)GSE | 46 or 92 sec |
| --- | --- | --- |

| MFI | ¦<B>¦, <¦B¦>, RMS, Num. pts. in avg., (Bx, By, Bz, B, f, l)GSM; (Bx, By, Bz, B, f, l)GSE, (X,Y,Z)GSE, (X,Y,Z)GSM, R | 46 or 92 sec1 |
| --- | --- | --- |

| SMS | PARTICLE FLUXES Differential Intensity (10-6 - 10-5 part/cm2-sec-sr-keV/e) SOLAR WIND PARAMETERS Solar wind speed: H+ (150 - 2370 km/sec) Fe+10 (60 - 1000 km/sec) Solar wind density (10-6 - 102 cm-3) Solar wind kinetic temperature (104 - 108 K) | 2.0 min 3.0 min 3.0 min 3.0 min 3.0 min |
| --- | --- | --- |

| TGRS | Major Frame Number, Instrument Mode, Fast Rate Counter Major Frame Accumulation, Average Fast Rate | 5 min |
| --- | --- | --- |

| EPACT | STEP fluxes: He3 (80-160 keV/n), He5 (320-640 keV/n), CNO3 (80-160 keV/n), CNO5 (320-640 keV/n), Fe3 (80-160 keV/n), Fe6 (320 to 640 keV/n) LEMT fluxes: He (3.2-6.2 keV/n), O (3.2-6.2 keV/n), Fe (3.2-6.2 keV/n) APE-A fluxes: Electrons (0.2-2 MeV/n), Protons (4.6-6.6 MeV/n) Protons (6.6-25 MeV/n), He (4.6-6.6 MeV/n), He (6.6-25 MeV/n) APE-B: Electrons (1-10 MeV), Protons (19-28 MeV/n) Protons (28-72 MeV/n), He (19-28 MeV/n) He (28-72 MeV/n) | 46 or 92 sec1 |
| --- | --- | --- |

| KONUS | n/a |  |
| --- | --- | --- |

.c1.  Table 4 - Polar KP Data Product Summary;

(page 1 of 3)

| Investigation | Key Parameters | Time resolution |
| --- | --- | --- |

| TIMAS | Partial density for H+ (low, med, high energy range), O+ (low, med, high energy range), He+ (full energy range) and He++ (full energy range); Three dimensional velocity for H+ (low, full energy range), O+ (low, full energy range), Temperature (parallel, perpendicular to B) for H+ (low, full energy range), O+ (low, full energy range) NOTE: Energy ranges not final until after instrument calibration, but best guess about ranges is: density_low_E_range (15 eV to 100 eV), density_medium_E_range (100 eV to 1 keV), density_high_E_range (1 keV to 30 keV), vel_temp_low_E_range (15 eV to 1 keV) and vel_temp_full_E_range (15 eV to 30 keV) | 1 min |
| --- | --- | --- |

| MFE | POS-GSE, POS-GSM, RAD-DIST, (T,H)=tilt angle of dipole axis using dipole moment of date (IGRF with secular correction), B-Total (every 9.2 sec), BSC (B in s/c coords., every 9.2 sec), Comp_Delta (compressional standard deviation of the magnetic field in nT calculated over 6 vectors), Trans_Delta (Square root of the sum of the variance in the x, y and z direction less the compressional variance, BTAV (average magnitude of the magnetic field, averaged over 0.92 min from magnitudes taken every 9.2 s, B–GSE, B-GSM, BTIGRF (IGRF magnetic field magnitude calculated at the epoch of the measurements using the IGRF and secular terms), BCIGRF_GSE (Components of the IGRF magnetic field calculated at the epoch of measurement in GSE coords.), BCIGRF_GSM (Components of the IGRF magnetic field calculated at the epoch of measurement in GSM coords.), BT_Model (Magnitude of the Tsyganenko magnetic field model using the tilt of the dipole and the T89c model for the Kp=3-, 3, 3+ range, calculated every 9.2 sec and averaged over the 0.92 min of the observed field), BC_MODEL_GSE (the 3 components of the Tsyganenko magnetic field model using the tilt of the dipole and the T89c model for the Kp=3-, 3, 3+ range, calculated every 9.2 sec and averaged over the 0.92 min of the observed field; rotated into GSE coordinates; epoch of the measurements is used to calculate model), BC_MODEL_GSM (the 3 components of the Tsyganenko magnetic field model using the tilt of the dipole and the T89c model for the Kp=3-, 3, 3+ range, calculated every 9.2 sec and averaged over the 0.92 min of the observed field; rotated into GSM coordinates; epoch of the measurements is used to calculate model), FOOT_PRINT (the northern geographic longitude and latitude and the southern geographic longitude and latitude respectively of the foot print of the field line that passes through the spacecraft in the model field) | 0.92 min |
| --- | --- | --- |

| HYDRA | Mean electron energy, Mean ion energy, Index indicating detection of statistically significant beam or void in the source/loss cone region, Energy at which the structure in the source/loss cone is inferred to be present | ~ 1 min |
| --- | --- | --- |

| EFI | E-spin plane (V12), Erms (V12); Ex, Ey (V12), avg. sigmas (V12); Ex’, Ey’ (V34), avg. sigmas (V34); s/c potential, band pass filters 1-3, V34 potential; probe bias currents (V12, V34, V56) | 9.2 sec |
| --- | --- | --- |

.c1. Table 4 - Polar KP Data Product Summary;

(page 2 of 3)

| Investigation | Key Parameters | Time resolution |
| --- | --- | --- |

| PWI | Dt (half a time interval), fce,p, MLAT, MLT, R (geocentric radial distance), E peak, E average, B peak, B average, Freq., SFR_Mode | 1 min |
| --- | --- | --- |

| CAMMICE | Integral Spin Averaged Fluxes: He+ (~40 keV - ~400 keV), He++ (~30 keV - ~400 keV), O+ (~80 keV - ~400 keV), O≥2+ (~50 keV - ~400 keV), He (~300 keV - ~10 MeV), CNO (~300 keV - ~15 MeV), >CNO (2 MeV - ~15 MeV) Angular Distributions (Anisotropy parameters): He+, He++ (~50 - ~ 100/keV/amu), O+ (~50 - ~ 200/keV/amu), He, O (> 100/keV/amu) Spectral Fits (Cubic in Log J vs. Log E space): He+, He++, O+ | 96 sec |
| --- | --- | --- |

| VIS | 1. Pixels (image pixel counts range from 0 to 255); 2,3. Center time (the time assigned to an image is the center time of the integration period within a resolution of 50 msec); 4. Image source (sensor number, 1=low res, 2=med res, 3=Earth sensor); 5. Half integration time; 6. Filter wheel position; 7. Field-stop wheel position; 8. Platform pitch angle; 9,10. Mirror pointing angles - elevation and azimuth in s/c coordinates; 11,12. Geographic coordinates every 15th row and every 15th column; 13. Spacecraft position vector in GCI coords. for the image center; 14. Presumed altitude of emissions; 15,16,17. Image-to-GCI rotation matrix; 18. Zenith angle of center line-of-sight at presumed altitude; 19. Sun vector in GCI coords. for the image center time given in KPs 2 and 3; 20. Solar zenith angle at observed point of center line-of-sight; 21,22. Low and high color mapping limits; 23. Display orientation flag | 5 min |
| --- | --- | --- |

| UVI | Poleward/Equatorward extent of auroral oval, Energy flux in 4 quadrants, sector, Characteristic energy, LBH_long Auroral Image (228 ¥ 200), row number, column number | 10 min |
| --- | --- | --- |

| PIXIE | Accumulation half-interval, energy bin endpoints, energy bin upper limits, energy bin lower limit, energy range specifiers, energy range upper limit, energy range lower limit, X-ray source array, vertical pixel coord., horizontal pixel coord., X-ray source array - high & low energy, total X-ray flux - high & low energy, location of terminator - markers 1 & 2, pixel coords., location of Greenwich meridian, location of geographic pole, location of geomagnetic pole, location of sub-satellite point, MLT - markers 0, 6, 12 & 18 hrs, geographic long/lat map, MLT map, magnetic lat map, number of sub-images, aperture size | 10 min/image |
| --- | --- | --- |

.c1. Table 4 - Polar KP Data Product Summary;

(page 3 of 3)

| Investigation | Key Parameters | Time resolution |
| --- | --- | --- |

| TIDE | First 13 moments of the ion velocity distribution function for each mass species sampled within the period of integration: N1, V1, T1, H1, N2, V2, T2, H2, N3, V3, T3, H3, N4, V4, T4, H4, N5, V5, T5, H5 (N=density, V=velocity, T=temperature, H=heat flux) | 1 min |
| --- | --- | --- |

| CEPPAD | Angular Distribution: Proton anisotropy parameters (>25 keV), error of fit (rms) Electron anisotropy parameters (>25 keV), error of fit (rms) HIST anisotropy parameters (>300 keV), error of fit (rms) Pitch Angle Flux Ratios (spin averaged, >25 keV: j(0)/j(90), j(45)/j(90), j(180)/j(90) for protons and electrons Energy Spectra: Cubic Fit IPS Protons, error of fit (rms), Cubic Fit IES Electrons, error of fit (rms) Integral Energetic Particle Fluxes: Proton flux > 25 keV, Electron flux > 30 keV, Electron flux > 300 keV Key Parameter Quality Flags: IPS (protons), IES (electrons), HIST (electrons) | 96 sec |
| --- | --- | --- |

| CEPPAD/ SEPS | Angular distribution (flux) Electrons (zenith) E>50keV background,3deg,9deg,15deg,21deg Electrons (nadir) E>50keV background,3deg,9deg,15deg,21deg Protons (zenith) E>50keV background,3deg,9deg Protons (nadir) E>50keV background,3deg,9deg Energy spectra (flux) Electrons (zenith) loss cone 50keV, 150keV,300keV,1MeV Electrons (nadir) source cone 50keV, 150keV,300keV,1MeV Protons (zenith) loss cone 50keV, 150keV,300keV,1MeV Protons (nadir) source cone 50keV, 150keV,300keV,1MeV | 184 sec |
| --- | --- | --- |

.c1.  Table 5 - Ground-based Location & KP Data Product Summary;

(page 1 of 2)

| Investigation | Key Parameters | Time resolution |
| --- | --- | --- |

| CANOPUS/BARS | North & East components of drift velocity, number of measured vectors, eccentric dipole field line latitude | 1 min |
| --- | --- | --- |

| CANOPUS/MARIA | Riometer absorption corrected for instrument characteristics, CU & CL indices, CU lat/lon (geodetic) CL lat/lon (geodetic), site availability, sites, sites lat/lon | 1 min |
| --- | --- | --- |

| CANOPUS/MPA | l5577 intensity (latitude scan for each photometer, 17 bins each scan) | 5 min |
| --- | --- | --- |

| CANOPUS/ASI | l5577 image (One 16 x 25 pixel image) | 5 min |
| --- | --- | --- |

| DARN/PACE | GEO position, half scan time, half range in GEO coords., velocity components | ~ 100 sec |
| --- | --- | --- |

| DARN Goose Bay | GEO position, half scan time, half range in GEO coords., velocity components | ~ 100 sec |
| --- | --- | --- |

| DARN Saskatoon | GEO position, half scan time, half range in GEO coords., velocity components | ~ 100 sec |
| --- | --- | --- |

| DARN Iceland West | GEO position, half scan time, half range in GEO coords., velocity components | ~ 100 sec |
| --- | --- | --- |

| DARN Kapuskasing | GEO position, half scan time, half range in GEO coords., velocity components | ~ 100 sec |
| --- | --- | --- |

| SONDRESTROM | Begin/end/half integration time, GEO lat/lon, altitude, normalized power, Ne, Ti, Te, Ne error, Ti error, Te error, Vions, Vions error | 10 sec to 5 min |
| --- | --- | --- |

| SESAME/Advanced Ionospheric Sounder | fmin: lowest plasma frequency recorded (MHz) fEmax: highest plasma frequency recorded from E-region (MHz), fFmax: highest plasma frequency recorded from F-region (MHz) | 15 min |
| --- | --- | --- |

| SESAME/ELF-VLF Logger Investigation | VLF-1: mean, omni-directional logarithmic intensity of 1 kHz signal with 1 kHz bandwidth (dB) VLF-3: As above, but for 3 kHz signal (dB), 1/2-duration of observation | 1 min |
| --- | --- | --- |

| SESAME/Fabry Perot Interferometer | US: Horizontal Doppler velocity in geographic South direction (m/s), UE: As above, but for geographic East direction, indicator to show emission wavelength | 30 min (Apr - Sep only) |
| --- | --- | --- |

| SESAME/Fluxgate Magnetometer | HDZ_system (components in Cartesian coords.), B_HDZ (Magnetic field components H, D, Z in nT; H=Horizontal (+) North, (-) South; D=Horizontal (+) East, (-) West; Z=Vertical (+) Down) | 1 min |
| --- | --- | --- |

| SESAME/Riometer | Absorption (dB): AN (North), AE (East), AS (South), AW (West). Note: in each case direction shifted onto an L-shell-aligned coordinate system | 1 min |
| --- | --- | --- |

.c1.  Table 5 - Ground-based Location & KP Data Product Summary;

(page 2 of 2)

| Name | Latitude (degrees) | Longitude (degrees) |
| --- | --- | --- |

| Sondrestrom | 67 | 309.2 |
| --- | --- | --- |

| SESAME | -75.6 | 333.23 |
| --- | --- | --- |

| Canopus/Back | 57.7 | 265.8 |
| --- | --- | --- |

| Canopus/Contwoyto Lk | 65.8 | 248.8 |
| --- | --- | --- |

| Canopus/Dawson | 64.1 | 220.9 |
| --- | --- | --- |

| Canopus/Eskimo Point | 61.1 | 266 |
| --- | --- | --- |

| Canopus/Ft Churchill | 58.8 | 265.9 |
| --- | --- | --- |

| Canopus/Ft McMurray | 56.7 | 248.8 |
| --- | --- | --- |

| Canopus/Ft Simpson | 61.7 | 238.8 |
| --- | --- | --- |

| Canopus/Ft Smith | 60 | 248.1 |
| --- | --- | --- |

| Canopus/Gillam | 56.4 | 265.4 |
| --- | --- | --- |

| Canopus/Island Lake | 53.9 | 265.3 |
| --- | --- | --- |

| Canopus/Nipawin | 53.5 | 256.3 |
| --- | --- | --- |

| Canopus/Pinawa | 50.2 | 264 |
| --- | --- | --- |

| Canopus/Rabbit Lake | 58.2 | 256.3 |
| --- | --- | --- |

| Canopus/Rankin Inlet | 62.8 | 267.9 |
| --- | --- | --- |

| Canopus/Red Lake | 50.9 | 266.5 |
| --- | --- | --- |

| DARN/Goose Bay | 53.32 | 299.54 |
| --- | --- | --- |

| DARN/PACE | -75.6 | 333.23 |
| --- | --- | --- |

| DARN/Saskatoon | 52.16 | 253.47 |
| --- | --- | --- |

| DARN/Iceland West | 64.43 | 337.99 |
| --- | --- | --- |

| DARN/Kapuskasing | 49.41 | 277.53 |
| --- | --- | --- |

.c1. Table 6 - Supplementary Missions KP Data Product Summary;

IMP-8

| Investigation | Key Parameters | Time resolution |
| --- | --- | --- |

| MAG | ¦<B>¦, <¦B¦>, magnitude/components RMS (GSE), Bx, By, Bz, ¦B¦, B-Lat/Long (GSE & GSM), (x,y,z)GSE&GSM, R, N | 61.4 sec |
| --- | --- | --- |

| PLA | V(RTN), V(GSE), V(GSM), Vpolar(GSE) proton number density, most probable thermal speed, (x,y,z)GSE&GSM, R | 1 min |
| --- | --- | --- |

LANL

| Investigation | Key Parameters | Time resolution |
| --- | --- | --- |

| LANL/SOPA | Electrons: 50-225 keV & 315-1500 keV flux, density, temperature; Protons: 50-400 keV & 1.2-5 MeV flux, density, temperature; He (0.9-1.3 MeV) flux, Heavy ions (C, N, O): 5-15 MeV flux, (lat, lon,R)GEO | 61.44 sec |
| --- | --- | --- |

| LANL/MPA | Low_Ions (1-130 ev): partial density, flow velocity, temperature (T^, T¦¦), temperature eigenvalue ratio (T^/Tmid); High_Ions (130-43,000 eV): partial density, flow velocity, temperature (T^, T¦¦), polar angle of symm. axis (rel. to s/c-Earth direction), azimuth of symm. axis (rel. to North), temperature eigenvalue ratio (T^/Tmid); Electrons (30-45,000 eV): partial density, temperature (T^, T¦¦), polar angle of symm. axis (rel. to s/c-Earth direction), azimuth of symm. axis (rel. to North), temperature eigenvalue ratio (T^/Tmid), s/c potential, background (lat,lon,altitude)GEO, (mlat, mlt,altitude)MAG | 172 sec |
| --- | --- | --- |

Table 6 - Supplementary Missions KP Data Product Summary

(Continued)

GOES-6, 7, (8, 9)

| Investigation | Key Parameters | Time resolution |
| --- | --- | --- |

| GOES_6/EPS | electron & proton counts, (R,lat,lon)GEO, (x,y,z)GEO,(x,y,z)GSE, (x,y,z)GSM, EPS-status | 5 min |
| --- | --- | --- |

| GOES_6/MAG | B (GSE, GSM and SEL coords.), (R,lat,lon)GEO, (x,y,z)GEO,(x,y,z)GSE, (x,y,z)GSM, MAG-status | 1 min |
| --- | --- | --- |

| GOES_7/EPS | electron & proton counts, (R,lat,lon)GEO, (x,y,z)GEO,(x,y,z)GSE, (x,y,z)GSM, EPS-status | 5 min |
| --- | --- | --- |

| GOES_7/MAG | B (GSE, GSM and SEL coords.), (R,lat,lon)GEO, (x,y,z)GEO,(x,y,z)GSE, (x,y,z)GSM, MAG-status | 1 min |
| --- | --- | --- |

| GOES_8/EP8 | TBD | TBD |
| --- | --- | --- |

| GOES_8/MAG | TBD | TBD |
| --- | --- | --- |

| GOES_9/EP8 | TBD | TBD |
| --- | --- | --- |

| GOES_9/MAG | TBD | TBD |
| --- | --- | --- |

.c1.  Table 7 - Geotail KP Data Product Summary;

(page 1 of 2)

| Investigation | Key Parameters | Time resolution |
| --- | --- | --- |

| CPI | SOLAR WIND ANALYZER Ion number density, avg. energy & bulk flow vel. HOT PLASMA ANALYZER Ion number density, avg. energy & bulk flow vel Electron avg. energy, plasma pressure COMPOSITION ANALYZER H+, He+, He++, O+ flags | 64 sec |
| --- | --- | --- |

| EPIC | Spin-averaged ion differential intensities, ICS; Uncertainties ion differential intensities, ICS; Ion chan energy band, ICS; Ion Channels, ICS; Ion chan energy plus, ICS; Ion chan energy minus, ICS; Spin-averaged proton differential intensities, STICS; Uncertainties proton differential intensities, STICS; H chan energy band, STICS; H channels, STICS; H chan energy plus, STICS; H chan energy minus, STICS; Spin-averaged electron integral intensity (>34 keV), ICS; Uncertainty electron integral intensity, ICS; e chan energy band, ICS; e channels, ICS; e chan energy plus, ICS; e chan energy minus, ICS;Spin-averaged He/H diff intensity ratios (82 & 233 keV/67 keV), ICS; Uncertainties He/H diff intensity ratios (82 & 233 keV/67 keV), ICS; He/H channels, ICS; Spin-averaged O/H diff intensity ratios (203 & 1244 keV/67 keV, ICS; Uncertainties O/H diff intensity ratios (203 & 1244 keV/67 keV), ICS; O/H channels, ICS; Spin-averaged He++/H+ diff intensity ratio (9.4 - 212 keV/67 keV), STICS; Uncertainty He++/H+ diff intensity ratio (92 & 233 keV/67 keV), STICS; He++/H+ channels, STICS; Spin-averaged O+/H+ diff intensity ratio (9.4 -212 keV/67 keV), STICS; O+/H+ diff intensity ratio (82 & 233 keV/67 keV), STICS; O+/H+ channels, STICS; >34 keV electron anisotropy parameters, ICS; uncertainties electron anisotropy parameters, ICS; 9.4-212.1 keV/e proton anisotropy parameters, STICS; Uncertainties proton anisotropy parameters, STICS67.0 keV proton anisotropy, ICS; Uncertainties proton anisotropy parameters, ICS; 128.7 keV proton anisotropy parameters, ICS; Uncertainties proton anisotropy parameters, ICS | 96 sec |
| --- | --- | --- |

| HEP LD-spectrometers HEP BD-spectrometers | Electrons J (E > 20 keV, az = 0˚, 90˚, 180˚, 270˚) Protons J (E > 20 keV, az = 0˚, 90˚, 180˚, 270˚) Helium J (E > 20 keV, az = 0˚), Oxygen J (E > 50 keV) Electrons J (E > 0.12 MeV, az = 0˚, 90˚, 180˚, 270˚) Protons J (E > 0.4 MeV, az = 0˚, 90˚, 180˚, 270˚) | 180 sec 180 sec 180 sec 180 sec 180 sec |
| --- | --- | --- |

| MGF | ¦B¦, Bx, By, Bz, s (GSE), XGSE, YGSE, ZGSE | 64 sec |
| --- | --- | --- |

| EFD | Sphere Probe: Ex, Ey, Residue, s/c potential, bias value Wire antenna: Ex, Ey, Residue, s/c potential, bias value Half-time resolution, XGSE, YGSE, ZGSE | 64 sec |
| --- | --- | --- |

.c1. Table 7 - Geotail KP Data Product Summary;

(page 2 of 2)

| Investigation | Key Parameters | Time resolution |
| --- | --- | --- |

| LEP | e, ion energy spectra per sector, n, T, vel, q, f avg. H+, O+, counts per sector | 5.6 min 2.8 min 5.6 min |
| --- | --- | --- |

| PWI | Peak and average of spectral intensity for the E of the Multi Channel Analyzer (E at 10 Hz, 100 Hz, 1 kHz, 10 kHz and 100 kHz), Peak and average of spectral intensity for the B of the Multi Channel Analyzer (B at 10 Hz, 100 Hz, 1 kHz, and 10 kHz), Peak and average of the spectral intensities for the Sweep Frequency Analyzer, Frequency corresponding to the peak of the spectral intensities, XGSE, YGSE, ZGSE | 64 sec |
| --- | --- | --- |

.c1.  Table 8 - SOHO KP Data Product Summary;

| Investigation | Key Parameters | Time resolution |
| --- | --- | --- |

| CELIAS | Solar wind speed, Solar EUV rate (2 values 0., 1. order), heavy ion rate | 5 min |
| --- | --- | --- |

| COSTEP | electron flux: 0.2-0.8, 0.8-3, 3-6.4, 6.4-9.6 MeV; proton flux: 0.3-0.8, 0.8-2, 2-5, 5-9, 9-28, 28-44, 44-56 MeV; Helium flux: 4.5-8.3, 8.3-26, 26-42, 42-55 MeV/n | 5 min |
| --- | --- | --- |

| ERNE | electron flux: 4-16, >16 MeV; proton flux: 2-4, 4-8, 8-16, 16-32, 32-64, 64-128 MeV; 4He flux: 2-4, 4-8, 8-16, 16-32, 32-64, 64-128 MeV/n; abundance ratios: deuteron-to-proton (4-8, 32-64 MeV/n), 3He-to-4He (4-8, 32-64 MeV/n) | 1 min |
| --- | --- | --- |

| Experiment | Summary Parameters (Imaging) | Time Resolution |
| --- | --- | --- |

| MDI | Full disk magnetogram, 1024 x 1024 Full disk continuum, 1024 x 1024 | TBD |
| --- | --- | --- |

| EIT | Full disk images in lines Fe IX, Fe XII, FE XV, He II, 1024 x 1024 | TBD |
| --- | --- | --- |

| UVCS | 1.2 - 10 RS Corona Ly alpha, 256 x 256 1.2 - 10 RS Corona Temperature, 256 x 256 | TBD |
| --- | --- | --- |

| LASCO | 1.1 - 30 RS Corona White Light, 1024 x 1024 1.1 - 3 RS Corona in lines Fe XIV, Fe X, Ca XV, 1024 x 1024 | TBD |
| --- | --- | --- |

| GOLF | Global Magnetic Field | 6-hour mean 1 value per day |
| --- | --- | --- |

| VIRGO | Solar Constant, SPM, LOI for each pixel | 1 value per day |
| --- | --- | --- |

| SWAN | Observation sensors 1 and 2 | TBD |
| --- | --- | --- |

.c1.  Table 9 - SOHO Science Data Product Summary;

| Experiment | Summary Parameters | Time resolution |
| --- | --- | --- |

| GOLF | Time series of global velocity field | TBD |
| --- | --- | --- |

| VIRGO | Time series of irradiance | TBD |
| --- | --- | --- |

| MDI | Time series of: full disk velocity, intensity, and magnetogram images; high-resolution velocity, intensity and magnetogram images; mode amplitudes, low resolution velocity, intensity images | TBD |
| --- | --- | --- |

| SUMER | Time series of images (small and full Sun), whole solar spectra, solar events | TBD |
| --- | --- | --- |

| CDS | Time series of images and spectra | TBD |
| --- | --- | --- |

| EIT | Full disk, portion of solar image taken in conjunction with CDS and SUMER observations | TBD |
| --- | --- | --- |

| UVCS | Level 1: Uncalibrated solar UV images, blocks of visible light data. Level 2: Same as level 1, but calibrated (gain table and intensity calibrations) | TBD |
| --- | --- | --- |

| LASCO | Time series of coronal images (brightness, electron densities, % polarization, magnetic field orientation, bulk velocity and line ratios | TBD |
| --- | --- | --- |

| SWAN | Antisolar Ly-alpha intensity, Ly-alpha maps of the sky | TBD |
| --- | --- | --- |

| CELIAS | Particle fluxes and energies for Solar Wind and Suprathermal particles; densities, mean velocities, velocity distributions for Solar Wind | TBD |
| --- | --- | --- |

| CEPAC | Counting rates and spectra of various elements or groups of elements, abundance of isotopes | TBD |
| --- | --- | --- |

.c1.  Table 10 - ClusterSummary Parameters Data Products;

| Investigation | Summary Parameters | Time resolution |
| --- | --- | --- |

| FGM | B, dB/B, FGM status word | 1 min |
| --- | --- | --- |

| STAFF | B power: 0.3-10, 10-180, 180-4000 Hz E power: 10-180, 180-4000 Hz, STAFF status word, WEC status word | 1 min |
| --- | --- | --- |

| EFW2 | Edusk, sE, Vps, Iprobe, E power: 0.3-10, 10-180 Hz, EFW status word, WEC status word [Dawn-dusk electric field, variations in electric field, probe-s/c voltage, and probe current] | 1 min |
| --- | --- | --- |

| WHISPER4 | n(e), quality, E power: 4-80 kHz, WHISPER status word, WEC status word | 1 min |
| --- | --- | --- |

| WBD4 | WBD status word, WEC status word | 1 min |
| --- | --- | --- |

| DWP4 | Significance level, DWP status word, WEC status word | 1 min |
| --- | --- | --- |

| EDI | E, EDI status word | 1 min |
| --- | --- | --- |

| CIS | n(p), n(O+), n(He+), n(He++), n(HIA), v(p), v(HIA), v(O+), T(p), T(HIA), CIS status word [HIA = Hot Ion Analyzer, measures most dominant ion species] | 1 min |
| --- | --- | --- |

| PEACE | n(e), v(e), T¦¦(e), T^(e), Q¦¦(e), PEACE status word [electron density, velocity, temperature, heat flux] | 1 min |
| --- | --- | --- |

| RAPID | Jlow(s), Jhi(s), s= e, p, He, mass > 4 A¦¦(e), A¦¦(p), RAPID status word [J = particle flux, 2 energy ranges, 4 species A = field aligned flux anisotropy, 2 species] | 1 min |
| --- | --- | --- |

| ASPOC | Ion current, ASPOC status word | 1 min |
| --- | --- | --- |

| Auxiliary information | R1, position of s/c 1, DRn; n = 2,3,4, position relative to s/c 1, V1, velocity of s/c 1, QG, QR, min ¦DRij¦, max ¦DRij¦, [Tetrahedron quality parameters of Glassmeir and Roux], qSCn, fSCn; n = 1-4, spin axes in GSE, GSEÆGSM conversion angle, dipole tilt angle, spacecraft status word | 1 min |
| --- | --- | --- |

.c1.Table 11 - Cluster Science Data Centers;

| DC Name | Location | Experiments |
| --- | --- | --- |

| UKCDC | Rutherford Appleton Laboratory, U.K | DWP, FGM, PEACE |
| --- | --- | --- |

| CFC | Centre National de Etudes Spatiales, Toulouse, France | CIS, STAFF, WHISPER, EFW |
| --- | --- | --- |

| MPEDC | Max-Planck-Institut fur Extraterrestrische Physik Garching, Germany | EDI |
| --- | --- | --- |

| LINDC | Max-Planck-Institut fur Aeronomie Katlenburg-Lindau, Germany | RAPID |
| --- | --- | --- |

| ACDC | Institut fur Weltraumforschung Graz, Austria | ASPOC |
| --- | --- | --- |

| SCD | Kunliga Tekniska Hogskolan Stockholm, Sweden | EFW |
| --- | --- | --- |

| USCSDC | Goddard Space Flight Center Greenbelt, MD | WBD |
| --- | --- | --- |

| HDC | Central Research Institute for Physics Budapest, Hungary | -- |
| --- | --- | --- |

| ? | China? | -- |
| --- | --- | --- |

| ? | India? | -- |
| --- | --- | --- |

.c1.Table 12 - Interball/Aurora Key Parameters;

| Key Parameters | Contributing Instrument(s) | Time resolution |
| --- | --- | --- |

| High-temperature plasma: F(e-), <Ee>, F(H+), <E H+> | ION, SKA-3, PROMICS-3A | TBD |
| --- | --- | --- |

| High-temperature plasma: F(O+), <E O+>, azimuth angle O+ and polar angle O+ | PROMICS-3A | TBD |
| --- | --- | --- |

| High-temperature plasma: Flux of ions 27-32 keV (1 detector directed from Sun, 2 directed at angle 45˚ to Sun or in scanning mode) Flow of electrons 27-32 keV (1 detector directed from Sun, 2 directed at angle 45˚ to Sun or in scanning mode) Pitch angle curve | DOK-2A | TBD |
| --- | --- | --- |

| Cold plasma: Density of O+ ions, Density of H+ ions, Temperature of O+ ions, Temperature of H+ ions, Velocity of O+ ions (Vx, Vy, Vz), Velocity of H+ ions (Vx, Vy, Vz) | HYERBOLOID | TBD |
| --- | --- | --- |

| Cold plasma: Temperature of electrons | KM-7 | TBD |
| --- | --- | --- |

| Cold plasma: Plasma density | AP-3A | TBD |
| --- | --- | --- |

| Cold plasma: Beam current, probe potential | RON | TBD |
| --- | --- | --- |

| Wave investigations: Spectrogram of HF E, Spectrogram of HF B (cyclotron gyrofrequency Fce added) | POLRAD | TBD |
| --- | --- | --- |

| Wave investigations: Spectrogram of HF E, Spectrogram of HF B (cyclotron gyrofrequency Fce added) | MEMO | 30 sec |
| --- | --- | --- |

| VLF E, VLF B, ELF/VLF SFA E, ELF/VLF SFA B, Service parameter (SFA) [SFA = Sweep Frequency Analyzer] | MEMO | 30 sec |
| --- | --- | --- |

| Electric and Magnetic fields: Spectrogram of ULF E (orthogonal), Spectrogram of ULF B (orthogonal), Spectrogram of ULF E (parallel), Spectrogram of ULF B (parallel) | IESP-2 | TBD |
| --- | --- | --- |

| 3 components of magnetic field, total magnetic field | IMAP-3 | TBD |
| --- | --- | --- |

| 2 components of electric field, electric field DC | IESP-2 | TBD |
| --- | --- | --- |

| Auroral image | UVAI | 30 min |
| --- | --- | --- |

| Auxiliary information on s/c position (GSE & GSM) |  | every 1-3 hrs |
| --- | --- | --- |

.c1.Table 13 - Interball/Tail Key Parameters;

| Descriptors | Key Parameters | Contributing Instrument(s) | Time resolution |
| --- | --- | --- | --- |

| EMFT | Magnetic field: Bx, By, Bz, ¦B¦ | IMAP, MIF-M | ~ 2 min |
| --- | --- | --- | --- |

| EMFT | Fluctuations: Electric field (1-8 Hz, 530-750 Hz, 20 kHz) | OPERA | ~ 2 min |
| --- | --- | --- | --- |

| EMFT | Fluctuations: Magnetic field (1-4 Hz, 600-800 Hz) | MIF-M/PRAM | ~ 2 min |
| --- | --- | --- | --- |

| EMFT | Fluctuations: Electron flux (1-4 Hz) | IFPE/PRAM |  |
| --- | --- | --- | --- |

| GPPT | Thermal plasma: ne, Te, | ELECTRON | ~ 2 min |
| --- | --- | --- | --- |

| GPPT | Thermal plasma: ni, Vi, Ti | MONITOR (solar wind) AP (plasmasphere) EU-1 or CORALL (other regions) | ~ 2 min |
| --- | --- | --- | --- |

| EPTP | Oxygen: EO+ FO+, | PROMICS | ~ 2 min |
| --- | --- | --- | --- |

| EPTP | Energetic particles: fluxes of ions and electrons (two directions, energy ranges TBD) | DOK-2 | ~ 2 min |
| --- | --- | --- | --- |

| EPTP | Energetic particles: fluxes of ions (1-5 meV) and electrons 150-500 keV | SKA-2 | ~ 2 min |
| --- | --- | --- | --- |

|  | Tail probe coordinates |  | 1 hr |
| --- | --- | --- | --- |

|  | PC (polar cap) geomagnetic index (from Thule and Vostok stations) |  | 15 min |
| --- | --- | --- | --- |

.c1.Table 14 - ISTP Data Products and Disposition Overview;

| ISTP/GGS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

| Data Product | Purpose/Use | When Created? | Where Created? | When Archive? | When Publicly Accessible? | Appropriate Usage once non-proprietary? |
| --- | --- | --- | --- | --- | --- | --- |

| Level-0 | Telemetry-level product; | <2 days from data acquisition | CDHF | <30 days from data acquisition | 1 year from data acquisition | Requires access to archived PI-supplied s/w; very often may require PI-assistance in interpretation |
| --- | --- | --- | --- | --- | --- | --- |

| KPs | Low resolution; full coverage for all instruments; non-citable; browse | <3 days from data acquisition | CDHF | within 5 days after generation | once delivered to the NSSDC | non-citable |
| --- | --- | --- | --- | --- | --- | --- |

| “Citable” KPs | Low resolution, full coverage for given instrument in the time period released, citable browse and synoptic | TBD | CDHF/ RDAF | TBD | TBD; released by specific instrument for specific time periods | Citable;correctly and independently usable |
| --- | --- | --- | --- | --- | --- | --- |

| Event Data | High resolution; select times and for all instruments; citable analysis product | TBD | RDAF | TBD | TBD | Citable;correctly and independently usable, approximately 10% of the ISTP level-0 data |
| --- | --- | --- | --- | --- | --- | --- |

| PI Archive Products | Various resolutions; products vary by instrument; citable analysis products | <2 years from data acquisition | RDAF | <2 years from data acquisition | 2 years from data acquisition | Citable;correctly and independently usable |
| --- | --- | --- | --- | --- | --- | --- |

| Level-0 Processing s/w | Used at the CDHF for generation of KP data and at the RDAF for generation of event data | At or shortly after launch | RDAF | TBD | TBD | TBD |
| --- | --- | --- | --- | --- | --- | --- |

| KPGS s/w | Used at the CDHF | At or shortly after launch | CDHF | TBD | TBD | TBD |
| --- | --- | --- | --- | --- | --- | --- |

| Citable KP s/w | Used at the CDHF | Within 18 months after launch | CDHF/ RDAF | TBD | TBD | TBD |
| --- | --- | --- | --- | --- | --- | --- |

| Geotail |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

| Sirius Data (playback) | Used at the CDHF | <2 days from data acquisition | CDHF | <30 days from data acquisition | TBD if to be released | TBD if to be released |
| --- | --- | --- | --- | --- | --- | --- |

.c1.Appendix A - Overview of the Wind Mission Instruments;

Energetic Particle Acceleration, Composition & Transport (EPACT)

The principal Investigator of EPACT is Dr. T. von Rosenvinge, NASA/Goddard Space Flight Center, Greenbelt, MD.  The EPACT investigation will provide a comprehensive study of energetic particle acceleration and transport processes in solar flares, the interplanetary medium, and planetary magnetospheres, as well as the galactic cosmic rays and the anomalous cosmic ray component.  EPACT measurements will determine elemental and isotopic abundances for the minor ions making up the solar wind, with energies in excess of 20 keV.  This direct sampling of solar matter is a way to study events on the solar surface and the incorporation of solar material into the solar wind.  EPACT will also provide information on shocks in the interplanetary medium, which accelerate particles from solar-wind energies to several hundred keV.

Measurement Objectives:

•	Energy spectra of electrons and atomic nuclei of different charge and isotopic composition, from hydrogen to iron, over an energy range extending from 0.1 to 500 MeV/nucleon.

•	Isotopic composition of medium energy particles (2 - 50 MeV/nucleon) in solar flares, in the anomalous component and in galactic cosmic rays, extending up to Z = 90.

•	Determination of angular distributions of these fluxes.

The EPACT instrument consists of three integrated telescope/electronics boxes mounted on the body of the spacecraft.  The extensive dynamic range of particles to be measured is divided between three Low Energy Matrix Telescopes (LEMT), two Alpha-Proton-Electron Telescopes (APE), an Isotope Telescope (IT), and a Supra Thermal Energetic Particle Telescope (STEP).  The APE and IT instruments are contained in a single package known as the Electron Isotope Telescope (ELITE).  These solid state detector telescopes all use the dE/dx by E method of particle identification, except STEP, which obtains particle mass by measuring time-of-flight and energy.  An onboard recorder allows continuous observations to be made.

Russian Gamma-Ray Spectrometer (KONUS)

The Principal Investigators for the Russian Gamma-Ray Spectrometer, KONUS, are Dr. T. L. Cline, NASA/Goddard Space Flight Center, Greenbelt, Maryland and Dr. E. Mazets, IOFFE Physical Technical Institute, St. Petersburg, Russia.  The objective of KONUS is to perform gamma-ray burst studies similar to the TGRS studies.  It will perform event detection and will measure time history and energy spectra.  Although KONUS has lower resolution than TGRS, it has broader area coverage to complement that of TGRS so that, when their data are combined, they provide coverage of the full sky.  KONUS is the first Russian instrument to fly on an American satellite since civil space cooperation between the U.S. and Russia was resumed in 1987.  KONUS will continuously monitor cosmic gamma-ray bursts and solar flares in the energy range 10 keV to 10 MeV.

Measurement Objectives:

•	Measurement of gamma-ray burst energy spectra over the energy range 10 keV to 10 MeV, with energy resolution E/∆E = 15 @ 200 keV.

•	Measurement of burst time histories in three energy ranges covering 10 to 770 keV.

•	High-time-resolution measurement (2 ms resolution) for the high-intensity sections of a burst time history.

•	Continuous measurement of the gamma- and cosmic-ray background, interrupted only to read out bursts.

The KONUS instrument consists of two Russian sensors mounted on the top and bottom of the spacecraft aligned with the spin axis, a U.S. interface box, and a Russian electronics package mounted in the spacecraft body.  The sensors, copies of ones successfully flown on the Soviet COSMOS, VENERA and MIR missions, are identical and interchangeable NaI scintillation crystal detectors of 200 cm2 area, shielded by Pb/Sn.  The design and location of the two sensors ensure practically isotropic angular sensitivity.  The relative count rates recorded by the two detectors can provide a burst source locus to within a few degrees relative to the spin axis.  On-board analysis of background and burst events is performed by four pulse height analyzers, four time history analyzers, two high resolution time history analyzers, and a background measurement system.

Magnetic Field Investigation (MFI)

The principal investigator for the Magnetic Field Investigation (MFI) is Dr. R. Lepping of NASA/Goddard Space Flight Center, Greenbelt, Maryland.  MFI will investigate the large-scale structure and fluctuation characteristics of the interplanetary magnetic field, which influence the transport of energy and the acceleration of particles in the solar wind and dynamic processes in the Earth’s magnetosphere.  The fundamental observations of solar wind magnetic fields are important to the study of the solar wind and magnetosphere coupling process and also to the interpretation of other observational data from WIND.

MFI will measure the large-scale structure and fluctuation characteristics of the interplanetary magnetic field as functions of time, and through correlative studies to relate them to the dynamics of the magnetosphere.

Measurement Objectives:

•	Accurate, high-resolution vector magnetic field measurements in near real time on a continuous basis.

•	A wide dynamic measuring range, from ±0.004 nT to ±65,536 nT, in eight discrete range steps.

•	Measurement rates up to 44 vector samples per second for analysis of fluctuations.

The MFI instrument consists of dual triaxial fluxgate magnetometers mounted on a 12-meter radial boom, and a data processing and control unit within the body of the spacecraft.  The magnetometer sensors each produce analog signals proportional to the strength of the magnetic field component aligned with the sensor.  These signals are then digitized and processed by a microprocessor-controlled data system.  Mounting the magnetometers at the outboard end and at an inboard location on the boom helps substantially to reduce contamination of the measurements by spacecraft-generated magnetic fields.  MFI has a very wide field measurement capability, from ±0.004 nT to ±65,536 nT, with both automatic and commandable range-change capability.  Measurement rates are:  one vector per 92 seconds for key parameter data, 10.9 vectors per second for rapid data for standard analysis, and 44 vectors per second for Fast Fourier Transform data, and data from the snap-shot memory which can be triggered automatically for pre-defined field changes.

3-D Plasma and Energetic Particle Analyzer (3-D Plasma)

The Principal Investigator for the 3-D Plasma and Energetic Particle Analyzer (3-D Plasma) is Dr. R. Lin, Space Sciences Laboratory, University of California , Berkeley, CA.  The 3-D Plasma investigation will measure ions and electrons in the interplanetary medium with energies including that of the solar wind and into the energetic particle range.  This experiment will cover the gap

between the energy ranges covered by SWE and EPACT.  It will study particles in the thermal and suprathermal energy range, upstream of the bow shock in the foreshock region and the transient particles emitted by the Sun during solar particle events following solar flares.  The 3-D Plasma instrument was designed to study transport of particles and basic plasma acceleration processes in the interplanetary medium, and to measure the particle and plasma input to and output from the Earth’s magnetosphere.

Measurement Objectives:

•	The three-dimensional distribution of plasma and energetic electrons and ions over the particle energy range from solar wind to cosmic ray energies, a few eV to several MeV.

•	Energy resolution of 0.20 (∆E/E) and angular resolution of 5.6° by 5.6°, for particles from 3 eV to 30 keV; and energy resolution of 0.3 (∆E/E) and angular resolution of 22.5° by 36°, for particles from 20 keV to 11 MeV.

•	Perturbations to the electron distribution function, in wave-particle interactions.

The 3-D PLASMA instrument consists of two sensor packages mounted on small radial booms, and an electronics package mounted inside the spacecraft.  One boom-mounted sensor package contains an array of 6 double-ended semiconductor telescopes, each with two or three closely sandwiched silicon detectors to measure electrons and ions above 20 keV.  One side of each telescope is covered with a thin foil that absorbs ions below 400 keV.  On the other side, the incoming electrons below 400 keV are swept away by a magnet so that electrons and ions are cleanly separated.  Higher energy electrons (up to ~1 MeV) and ions (up to 11 MeV) are identified by the two double-ended telescopes which have a third detector.  The first sensor package also contains a pair of ion electrostatic analyzers (PESA-L and -H) for measuring ion fluxes from  ~3 eV to 40 keV.  The second sensor package contains a pair of electron electrostatic analyzers (EESA-L and -H) for measuring electron fluxes from ~3 eV to 30 keV, and for making input (from EESA-H) to a fast particle correlator (FPC).  The FPC, also using plasma wave data from WAVES as input, measures perturbations to the electron distribution function and studies other wave-particle interactions.

Solar Wind and Suprathermal Ion Composition Experiment (SMS)

The principal investigator for the solar wind and suprathermal ion composition experiment (SMS) is Dr. G. Gloeckler from the Institute of Physical Sciences and Technology and from the Department of Physics at the University of Maryland.  The SMS experiment comprises three major instruments:  Solar Wind Ion Composition Spectrometer (SWICS), High Mass Resolution Spectrometer (MASS), and Suprathermal Ion Composition Spectrometer (STICS). This experiment will determine the abundance, composition and differential energy spectra of solar wind ions, and the composition, charge state and 3-D distribution functions of suprathermal ions.  These ions and their abundance fluctuations provide information about events on the solar surface and the formation of the solar wind, complementing the EPACT and 3D-PLASMA investigations.

Measurement Objectives:

•	Energy, mass and charge composition of major solar wind ions from H to Fe, over the energy range from 0.5 to 30 keV/e.  (SWICS)

•	High mass-resolution elemental and isotopic composition of solar wind ions from He to Ni, having energies from 0.5 to 12 keV/e.  (MASS)

•	Composition, charge state and 3-D distribution functions of suprathermal ions (H to Fe) over the energy range from 8 to 230 keV/e.  (STICS)

The SMS experiment consists of five separate packages mounted on the spacecraft body.  SWICS uses electrostatic deflection, post-acceleration, and a time-of-flight vs. energy measurement to determine the energy and elemental charge state composition of solar wind ions.  MASS uses

energy/charge analysis followed by a time-of-flight measurement, to determine solar-wind ion composition with high mass-resolution (M/∆M > 100), for the first time.  STICS, similar to SWICS but not using post-acceleration, has a large geometric factor and wide-angle viewing for studies of suprathermal ions.  The SMS data processing unit and STICS analog electronics unit are mounted separately.

Solar Wind Experiment (SWE)

The principal investigator for the solar wind experiment  (SWE) is Dr. K. Ogilvie from NASA/Goddard Space Flight Center.  SWE will measure ions and electrons in the solar wind and the foreshock regions (particles whose energies are in the kiloelectronvolt range).  From these measurements, the solar wind velocity, density, temperature and heat flux can be deduced.  Electron and ion velocity distributions should reveal properties of the flowing solar wind plasmas and their pivotal role in the transfer of mass, momentum, and energy from the Sun to the Earth.  Measurements made in the foreshock region will contribute to understanding the structure of the bow shock.

Measurement Objectives:

•	High time-resolution 3-D velocity distributions of the ion component of the solar wind, for ions with energies ranging from 200 eV to 8.0 keV.

•	High time-resolution 3-D velocity distributions of subsonic plasma flows including electrons in the solar wind and diffuse reflected ions in the foreshock region, with energies ranging from 7 eV to 22 keV.

•	High angular-resolution measurements of the “strahl” (beam) of electrons in the solar wind, along and opposite the direction of the interplanetary magnetic field, with energies ranging from 5 eV to 5 keV.

The SWE instrument consists of five integrated sensor/electronics boxes and a data processing unit (DPU).  The sensor units are mounted on the top and bottom shelves of the spacecraft, extending through the top and bottom surfaces.  3-D velocity distribution measurements of the ion component in the solar wind are made by a pair of Faraday Cup analyzers, which provide a wide field-of-view and the capability for flow characterization within one spin revolution (3 seconds).  3-D velocity distribution measurements of ions and electrons in plasmas having Mach numbers <1 are made using six cylindrical electrostatic deflection analyzers, called Vector Electron and Ion Spectrometers (VEIS), arranged in two triaxial sets.  A toroidal electrostatic analyzer with channelplate detectors functions as the strahl sensor, measuring electron velocity distribution and pitch angle.  An ultra-violet calibration unit for the VEIS is mounted inside the spacecraft with the DPU.

Transient Gamma-Ray Spectrometer (TGRS)

The principal investigator for the Transient Gamma-Ray Spectrometer is Dr. B. J. Teegarden from NASA/Goddard Space Flight Center.  TGRS will detect transient gamma-ray burst events and will make the first high-resolution spectroscopic survey of cosmic gamma-ray bursts, and will also make measurements of gamma-ray lines in solar flares.  Cosmic gamma-ray bursts are among the most violent and energetic processes known to exist in nature, characteristically emitting most of their luminosity at gamma-ray wavelengths.  The high resolution spectroscopy of solar flares will contribute to the study of solar flare activities and help in the understanding of the coupling between the active corona and photosphere.

Measurement Objectives:

•	Spectroscopic measurements of transient gamma-ray events, in the energy range from 15 keV to 10 MeV.

•	Energy resolution of 2.0 keV @ 1.0 MeV (E/∆E = 500).

•	Monitoring of the time variability of the 511 keV line emission from the galactic center, on time scales from ~2 days to >1 year.

The TGRS instrument consists of four assemblies:  detector-cooler assembly, pre-amp, and analog processing unit, all mounted on a tower on the +Z end of the spacecraft, and a digital processing unit mounted in the body of the spacecraft.  The detector is a 215 cm3 high purity n-type germanium crystal of dimensions:  6.7 cm (diameter) by 6.1 cm (length), radiatively cooled to 85 degrees K.  The germanium serves as a reaction medium for incoming gamma rays, which, depending on their energy, are either stopped by or passed through the detector crystal.  Particle energy and angle of incidence are calculated based on a number of primary and secondary interaction processes, including photoelectric, Compton, pair and bremsstrahlung radiation as well as the ionization energy losses of secondary electrons.  A two-stage cooler surrounds the detector, providing a field of view of 170˚.  Gamma-ray bursts and solar flares are expected to be detected at a frequency of several per week, with typical durations between 1 second and several minutes.  Between bursts the instrument is maintained in a waiting mode, measuring background counting rates and energy spectra.  When a burst or flare occurs, the instrument switches to a burst mode, where each event in the detector is pulse-height analyzed and time tagged in a burst memory.  Then the instrument switches to a dump mode for reading out the burst memory.

Radio and Plasma Wave Experiment  (WAVES)

The principal investigator for the Radio and Plasma Wave experiment (WAVES) is Dr. J.-L. Bougeret from the Laboratoire de Recherche Spatiale Observatoire de Paris-Meudon, France.  WAVES will provide comprehensive measurements of the radio and plasma wave phenomena that occur in the solar wind upstream of the Earth’s magnetosphere and in key regions of the magnetosphere.  Analyses of these measurements, in coordination with the other onboard plasma, energetic particles, and field measurements, will further the understanding of solar wind and interplanetary plasma processes.

Measurement Objectives

•	Low-frequency electric waves and low-frequency magnetic fields, from DC to 10 kHz.

•	Electron thermal noise, from 4 kHz to 256 kHz.

•	Radio waves, from 20 kHz to 14 MHz.

•	Time-domain waveform sampling, to capture short duration events that meet quality criteria set into the WAVES data processing unit (DPU).

The sensor system of the WAVES experiment consists of three electric antenna systems (two coplanar, orthogonal wire antennas in the spin-plane and a rigid spin-axis dipole) and a triaxial magnetic search coil.  The longer and shorter spin-plane dipoles have lengths of 100 and 15 m tip-to-tip, respectively, while the spin-axis dipole is 10.56 m long tip-to-tip.  The triaxial magnetic search coil for measuring low-frequency magnetic fields is mounted at the outboard end of a 12 m radial boom. There are five main receiver systems:  a low frequency (DC to 10 kHz) Fast Fourier Transform receiver, a broadband (4 kHz to 256 kHz) electron thermal noise receiver, two swept-frequency radio receivers (20 kHz to 1 MHz, and 1 MHz to 14 MHz), and a time-domain waveform sampler (up to 120,000 samples per second).  The DPU controls and acquires data from all operations of the experiment, and can be reprogrammed from the ground.  The receiver systems and DPU are housed within the spacecraft body.  WAVES has onboard interconnects with 3-D PLASMA and with SWE.

.c1.Appendix B - Overview of the Polar Mission Instruments;

Charge and Mass Magnetospheric Ion Composition Experiment (CAMMICE)

The principal investigator for the Charge and Mass Magnetospheric Ion Composition Experiment (CAMMICE) is Dr. T. Fritz from Boston College, Boston.  CAMMICE will determine the composition of major ion constituents in the near-Earth plasma sheet and in the ring current.  This investigation will add to our understanding of the mechanisms that control the energization, storage, and precipitation of particles in the high-latitude magnetosphere.

Measurement Objectives:

•	Energy, time-of-flight, and the energy per charge of the incident ion flux, over the energy range from 6 keV/Q to 400 keV/Q, to make a unique determination of the ion charge state, mass, and incident energy.

•	Rate of energy loss and incident ion energy for ions, over the energy range from 100 keV per ion to 60 MeV per ion, to make a unique determination of the ion mass, elemental identification, and incident energy.

CAMMICE consists of two sensor systems:  the Magnetospheric Ion Composition Sensor (MICS), and the Heavy Ion Telescope (HIT).  The MICS sensor uses a conically-shaped electrostatic analyzer, a secondary-electron generation/detection system, and a solid-state detector, covering the energy range from 6 keV/Q to 400 keV/Q.  The HIT sensor uses a three-element solid-state detector telescope, covering the energy range from 100 keV per ion to 60 MeV per ion.  MICS and HIT are mounted on the radial surface of the spacecraft, each with a separately mounted data processing unit.

Comprehensive Energetic-Particle Pitch-Angle Distribution (CEPPAD)

The principal investigator for the Comprehensive Energetic-Particle Pitch-Angle Distribution (CEPPAD) instrument is Dr. B. Blake of the Space Sciences Laboratory at The Aerospace Corporation, Los Angeles, California.  The CEPPAD investigation will use a variety of techniques to provide detailed energy spectra and angular distributions of energetic particles, which in turn will provide quantitative information on the sources, energization, transport, and losses of energetic particles in the magnetosphere.  The CEPPAD will also measure the rate of particle precipitation into the Earth’s upper atmosphere.

Measurement Objectives:

•	Pitch angle measurements of electrons with energies from 25 keV to 3 MeV and protons with energies from 10 keV to 17 MeV, with angular resolution of 3° in the source/loss cone, and ~12° in other directions.

•	Identification and pitch angle measurements of alpha particles from 300 to 1100 keV/n and CNO nuclei with energies above 300 keV/n.

The CEPPAD consists of three packages:  two body-mounted, and one on the despun platform.  One body-mounted package consists of the Imaging Electron Sensor (IES) and the High Sensitivity Telescope (HIST).  IES measures electrons with energies from 25 to 400 keV using a spectrometer incorporating a Microstrip Solid Plate Detector (MSSD) with five elements.  HIST, a solid-state detector telescope, measures electrons with energies to 3 MeV and protons to 17 MeV, also alpha particles and CNO ions.  The second body-mounted package consists of the Imaging Proton Sensor (IPS) and the digital processing unit (DPU) serving IPS, IES and HIST.  The IPS, functioning similarly to the IES, measures protons from 10 kev to 1 MeV.  The despun platform

package is the Source/Loss Cone Energetic Particle Spectrometer (SEPS), which contains its own DPU.  SEPS exploits the pointing of the platform along the ambient magnetic field to greatly enhance its angular resolution of energetic particles within the source/loss cone.

Electric Field Instrument (EFI)

The principal investigator for the Electric Field Instrument (EFI) is Dr. F. Mozer of the Space Sciences Laboratory at the University of California, Berkeley, California.  EFI will measure the electric field strength and thermal electron density in the vicinity of the spacecraft. These measurements will be used to infer the electric-field structure of the high-latitude magnetosphere, the cusp, and the plasma mantle; and will also provide direct evidence for auroral field-aligned electrical potential drops, which have been previously inferred to occur.

Measurement Objectives:

•	Triaxial components of the ambient vector electric field and the thermal electron density, over a frequency range of DC to 20 kHz.

•	Electric field measurements over a dynamic range of 0.02 to 1000 mV/m, and plasma densities from 0.1 to 100 particles per cubic centimeter.

•	A sampling rate of 40 samples per second in the normal mode, and 1000 samples per second in the burst mode.

EFI utilizes six spherical potential sensors, mounted in pairs at the ends of two coplanar, orthogonal wire booms in the spin plane and on a rigid boom aligned along the spin axis.  An axial boom stabilizer is provided.  The spin-plane sensor pairs have separations of 130 and 100 m, respectively, while the spin-axis boom sensors are separated by 14 m.  An electronics box is mounted within the spacecraft body.  A two megabyte burst memory allows storage of high-time-resolution field and plasma density measurements.  EFI sensors provide input to PWI, and EFI interfaces with MFE, TIDE, and Hydra instruments for coordination of burst-mode measurements.

Fast Plasma Analyzer (Hydra)

The principal investigator for the Fast Plasma Analyzer (Hydra) is Dr. J. Scudder from the Department of Physics and Astronomy at the University of Iowa, Iowa City, Iowa.  Hydra will provide rapid measurements of the three-dimensional low-energy ion and electron distribution functions. The low-energy electrons sampled by Hydra will be especially good tracers of the global magnetic topology when compared with simultaneous measurements acquired by SWE in the solar wind.  Hydra will also document the electron and ion signatures accompanying geomagnetic substorms, auroral arcs, field-aligned currents, and particle precipitation.

Measurement Objectives:

•	Mean energy of electrons and ions between 2 eV and 35 keV, with time resolution of 0.5 seconds, for dense coverage over 4π steradians.

•	Angular resolution of 1.5° and time resolution of 3 seconds, for electrons between 100 eV and 10 keV along and opposed to the magnetic field.

Hydra utilizes two types of instruments:  a deployment of cylindrical electrostatic analyzers (ESA) in six pairs, providing 12 simultaneous samples, and a pair of parallel plate analyzers (PPA) functioning as a loss/source cone sensor; mounted in four boxes (2- ESA, 2-PPA) on the spacecraft body, with the sensors protruding through the solar arrays.  Each ESA has a Field of View (FOV) of 10° by 10°; and the PPA covers a conical FOV of 30° by 30° simultaneously along and opposed to the magnetic field, providing angular resolution in 1.5° by 1.5° pixels.  The data processing unit is mounted inside the spacecraft body.  Hydra receives magnetic field raw data

from MFE, and is linked to EFI and TIDE to ensure coordinated high-time resolution “burst mode” measurements.

Magnetic Fields Experiment (MFE)

The principal investigator for the Magnetic Fields Experiment (MFE) is Dr. C. Russell from the Institute of Geophysics at the University of California Los Angeles, Los Angeles, California.  MFE will measure accurately the magnetic fields in the high and low altitude polar magnetosphere.  These measurements will furnish data both to other instruments on board the spacecraft and to the ground investigator team, while at the same time, providing a state-of-the-art research tool for furthering the understanding of magnetospheric processes.

Measurement Objectives:

•	Accurate vector magnetic field measurements over the frequency range from 0 to 50 Hz, at field strengths from 10¯6 to 0.6 gauss.

•	Measurement rates up to 100 vector samples per second with onboard processing, allowing burst-mode telemetry at that rate, or economical data compression and distribution.

The MFE instrument consists of two triaxial fluxgate magnetometers mounted on a 6-meter radial boom, and an electronics package mounted inside the spacecraft.  The magnetometer sensors each produce analog signals proportional to the strength of the magnetic field component aligned with the sensor.  One magnetometer is mounted at the outboard end of the radial boom and the other at one-third the length of the boom, helping to significantly reduce contamination of the measurements by spacecraft-generated magnetic fields.   Onboard processing allows direct broadcast to other instruments on the spacecraft, and telemetry data transmission at 10 samples per second in the standard mode, or 100 samples per second in the burst mode.  The design provides a fully redundant instrument.

Polar Ionospheric X-Ray Imaging Experiment (PIXIE)

The principal investigator for the Polar Ionospheric X-Ray Imaging Experiment (PIXIE) is Dr. W. Imhoff from the Lockheed Palo Alto Research Laboratory in Palo Alto, California.  Using a pin-hole camera concept, PIXIE will measure the X-ray emissions from the Earth’s atmosphere.  The morphology and spectra of energetic electron precipitation and the effects upon the atmosphere will be derived from these measurements.  These measurements can be used to estimate the total electron energy deposition rate and the energy distribution of the precipitating electrons, and thus to compute the altitude profile of ionization and electrical conductivity.

Measurement Objectives:

•	Images of atmospheric bremsstrahlung X-rays, with ±21° by ? field-of-view for global coverage, over the energy range from 3 to 60 keV, and with sufficient spatial and time resolution (a few minutes) to generate movies of dynamic variations in auroral luminosities and associated effects.

•	Up to 64 channels energy resolution, and spatial resolution of ?, over a detector area of 18 by 18 cm, for image construction.

The PIXIE instrument is a multiple pinhole X-ray camera mounted on the despun platform, with its electronics unit.  The number and size of the pinholes are selectable, using two aperture plates, thus enabling field-of-view and sensitivity to change as POLAR moves around its highly eccentric orbit.  The focal plane detector uses a multiwire position-sensing gas proportional counter to measure the energy and position of each X-ray event, with shielding from cosmic X-rays and

active rejection of other charged particle backgrounds.  The electronics unit includes low and high voltage power supplies, an analog signal processor for the detector, a digital section that controls event processing, and a data processing unit that routes data to telemetry and controls the apertures.  Data on each valid X-ray event is telemetered to the ground, where it is processed into images.

Plasma Wave Investigation (PWI)

The principal investigator for the Plasma Wave Investigation (PWI) is Dr. D. Gurnett from the Department of Physics and Astronomy, University of Iowa in Iowa City, Iowa.  PWI will provide comprehensive measurements of plasma wave phenomena in the high latitude auroral zones, in the dayside cusp regions, and in the plasmasphere and plasmasheet.  PWI will sample the electric-field noise above the highest EFI frequencies, and will sample the magnetic fluctuations above the highest frequencies detectable by MFE.  These measurements will help in the study of plasma behavior and wave-particle processes.  These wave-particle processes mediated by the electromagnetic turbulence are thought to play a central role in momentum transfer in the geospace system.

Measurement Objectives:

•	Comprehensive measurements of spectral power and wave vector characteristics of electromagnetic and electrostatic plasma waves over the frequency range from 0.1 Hz to 800 kHz for wave electric and magnetic fields.

•	Measurement of spectral and wave vector characteristics of electromagnetic and electrostatic plasma waves generated by ground-based active experiments.

PWI uses the sensor system of EFI for electric field measurements, consisting of spherical sensors on two coplanar, orthogonal wire booms in the spin-plane and on a rigid spin-axis boom.  The longer and shorter spin-plane antennas have lengths of 130 and 100 m tip-to-tip, respectively, while the spin-axis boom is 14 m tip-to-tip.  For magnetic field measurements, PWI uses a triaxial magnetic search coil and a loop antenna, both mounted at the outboard end of a 6-m radial boom.  There are five main receiver systems mounted on the spacecraft body:  a low frequency AC receiver (0.1 Hz to 25 Hz); a wideband receiver with selectable 11, 22, or 90 kHz bandwidths, with lower band edge at  0, 125, 250 or 500 kHz; a narrow-band swept-frequency receiver covering the range 26 Hz to 808 kHz; a high-time-resolution multi-channel analyzer with ranges of 5.6 Hz to 311 kHz for electric fields and 5.6 Hz to 10 kHz for magnetic fields; and a waveform receiver covering the range from 20 Hz to 16 kHz.

Thermal Ion Dynamics Experiment (TIDE)

The principal investigator for the Thermal Ion Dynamics Experiment (TIDE) is Dr. T. Moore at Marshall Space Flight Center, Huntsville Alabama.  TIDE will sample ions escaping from the ionosphere.  Low-energy ions from 0.1 to 100 eV per charge will be the focus of TIDE; such ions can be remotely diagnosed by their relatively low ionization state as compared with ions of solar-wind origin.  These measurements will be used to track the outflow of ionospheric plasma throughout the magnetosphere.  To make possible the measurement of such low-energy ions, TIDE will use a Plasma Source Instrument (PSI) which, on command, emits xenon plasma to neutralize spacecraft charge build-up.

Measurement Objectives:

•	3-D low energy (0 to 300 eV) measurements differential in energy and angle as well as mass.

•	96% unit sphere coverage per spin, with angular coverage of 10° by 157.5° for each of 7 simultaneous sectors, and angular resolution of 10° by 22.5°.

•	Time resolution of 0.188 sec. for 2-D measurements, and 6 sec. for 3-D.

TIDE consists of an integrated sensors-electronics unit mounted on the spacecraft body, with the sensor head exposed to space, having a wide field-of-view with one edge aligned with the spin axis.  TIDE utilizes 7 separate channels (angular sectors), each comprising a collimator, electrostatic mirrors and retarding potential analyzer (RPA), coupled with a time-of-flight mass analyzer system.  The PSI, separately body-mounted with an external exit aperture, consists of a 20,000 hour xenon plasma source, a gas feed system and an electronics system for powering and monitoring.  PSI is operated on command to regulate the spacecraft at a few tenths of a volt positive relative to the space plasma.  The high time-resolution mode is triggered by data from EFI and Hydra.

Toroidal Imaging Mass-Angle Spectrograph (TIMAS)

The principal investigator for the Toroidal Imaging Mass-Angle Spectrograph (TIMAS) is Dr. E. Shelley of the Lockheed Palo Alto Research Laboratory in Palo Alto, California.  TIMAS will sample ions of resolved mass that are either stored particles of solar-wind origin or energized ions of ionospheric origin.  TIMAS will characterize the principal source regions for entry of solar wind plasma and the hot ionospheric plasma into the magnetosphere.  TIMAS data will be combined with data from TIDE and from the SMS/SWICS instrument on WIND to determine the ultimate origin of mass in the high-latitude magnetosphere.

Measurement Objectives:

•	Full 3-D velocity distribution functions of all major magnetospheric ion species, with time resolution of 3 seconds, and 11° by ? angular resolution,  over the energy per charge range of 15 eV/e to 32 keV/e.

•	Simultaneous measurements of all mass/charge components from 1 AMU/e to greater than 32 AMU/e over nearly 360° by 10° instantaneous field-of-view, in 20 milliseconds.

The TIMAS instrument is a double-focusing (angle and energy) mass spectrograph, utilizing two toroidal electrostatic analyzers in tandem.  A toroidal magnetic field then disperses the particles by momentum, leading to an image pattern of rings on a microchannel plate detector, representing mass spectrum dispersed radially and incident direction dispersed in azimuth.  The instrument is mounted on the radial surface of the spacecraft, sweeping out a 4π solid angle image in a half spin-period.  Fast-settling internal power supplies provide controlled high voltages to the electrostatic analyzers for various energy levels, and sequential processing through two SA-3300 microprocessors corrects, prioritizes, and compresses data for telemetry transmission.

Ultraviolet Imager (UVI)

The principal investigator for the Ultraviolet Imager (UVI) is Dr. G. Parks from the University of Washington, Seattle, Washington.  UVI will image both dayside and nightside aurorae in the far ultraviolet (FUV) range.  Improvements over previous imagers will allow the instrument to provide simultaneous global imaging from orbital positions greater than 6 Earth radii.  UVI images will provide spatial and temporal descriptions of the aurorae, total particle energy flux, characteristic energy, thermospheric neutral composition, and ionospheric conductances.  These images will be used to map the temporal and spatial evolution of electric fields, needed for global modeling of the thermosphere at the lower end of the solar-terrestrial chain.

Measurement Objectives:

•	2-D images of the global auroral oval, spectrally resolved for four key FUV wavelengths between 1300 Å and 1900 Å.

•	Field-of-view of 8°, providing angular resolution of about ?, with ground resolution from 5 to 50 km, dependent upon altitude.

The UVI is an intensified charge coupled device (CCD) used in conjunction with a fast reflective optical system.  The instrument is mounted on the despun platform, permitting simultaneous imaging of the entire auroral oval throughout 9 hours of each 18 hour orbit.  Five wheel-mounted interference filters, each combining both transmission and reflective elements, serve the purpose of passing the FUV wavelengths of interest, while blocking long wavelength (primarily visible) scattered sunlight.  The CCD detector has an active area of 244 by 550 pixels, formatted to produce an oval image frame of 200 by 228 effective pixels.  A data processing unit, separately mounted on the despun platform, integrates the images over a standard frame time of 37 seconds, for telemetry to the ground.

Visible Imaging System (VIS)

The principal investigator for the Visible Imaging System (VIS) is  Dr. Louis Frank from the Department of Physics and Astronomy at the University of Iowa, Iowa City, Iowa.  VIS will image the global aurora at visible wavelengths, with sufficient spatial and temporal resolution to resolve the principal dynamical features of auroral phenomena.  Data from VIS and from theory investigations will be used to assess the dissipation of magnetospheric energy into the auroral ionosphere, and to develop a model of the energy flow within the magnetosphere.

Measurement Objectives:

•	Auroral images with 12 filters for narrow passbands including  N2+ at 391.4 nm, OI at 557.7 and 630.0 nm, HI at 656.3 nm, and OII at 732.0 nm.

•	Low-resolution images covering 5.6° by 6.3°, and moderate-resolution images covering 2.8° by 3.3°, with images integrated over 256 by 256 pixels, and with corresponding resolution at Earth’s surface of 21 km and 11 km, respectively, from 8 RE.

VIS is a set of three low-light-level cameras, all mounted in a single package on the despun platform.  Two of these cameras share primary and some secondary optics and are designed to provide low- and moderate-resolution images of the nighttime aurora oval, from the POLAR orbit at 1 to 8 RE distance.  These cameras produce auroral images with 12 narrow passband filters at visible wavelengths, at a frame rate of 12 seconds.  A third camera (the Earth camera) is used to monitor the directions of the fields-of-view of the auroral cameras to ensure that they are correctly pointed and protected from extremely bright sources such as the Sun and sunlit Earth.  All three cameras use charge coupled device (CCD) detectors, with optically-coupled image intensifiers.

.c.Appendix C - Rules for Data Rights:  ISTP/NASA funded Missions

and Instruments

Approved: March 15, 1996

Revised: November 14, 1996 (Level 0 Policy added)

Preamble

The ISTP program is predicated on the concept of placing a constellation of satellites at key positions within the Sun-Earth connected system to obtain simultaneous, comprehensive and closely coordinated data.  The accomplishment of the scientific objectives of the program is dependent upon the availability of these coordinated data sets for analysis by members of the mission science teams, their co-investigators, associates, and students, and as quickly as possible, by scientists selected as guest investigators and by the general scientific community.  The purpose of this document is to set forth the rights and obligations of the Principal Investigators with the various missions and the various associates and collaborators who wish access to the data.

Missions, Instruments and Investigations Covered

The ISTP/NASA funded missions, instruments and investigations consist of the spacecraft in the Global Geospace Science program (GGS): Wind, Polar, and collaborating spacecraft, plus the associated ground-based investigations and theory investigations; the instruments on Geotail: CPI and EPIC; the WBD (NASA portion) instrument on Cluster; the MDI, LASCO and UVCS instruments on SOHO, and ground-based investigations.

Data Rights and Rules for Data Use

1.  The NASA funded missions and instruments have an "open" data policy.

2.  Key Parameter digital data will be non-proprietary and publicly available to identify possible scientifically interesting events or intervals.  The Key Parameters are not intended for analysis or publication purposes, and should not be used in that manner.

3.  All GGS PIs and/or PI teams will contribute relevant digital processed data for any special events or intervals that are selected for study by the GGS SWG and for the IACG Campaigns in a timely and responsive fashion. All NASA Campaign data will be archived and publically available.

4.  During a validation period of up to three months after data acquisition, all use of the data for scientific investigations must be approved by the PI whose data are being used. PIs can release their own validated data at any time but may not release data that are in their possession from other instruments.

5. Higher level processed instrument and theory data products, along with their associated documentation and all relevant software (in source code form), will be publically available immediately after validation. These data products, along with Level 0 data will be archived at the National Space Science Data Center (NSSDC) within a year of generation.

The scientific community is encouraged to exercise and demonstrate professional courtesy, appreciation, and prudence in utilizing the openness of the data policy.  This would be accomplished, along with greater scientific certainty in the use and interpretation of the data, by engaging the PI and members of the investigative team to collaborate on the planned studies leading to co-authored abstracts and/or publications.

Policy with respect to Level 0 Data Distribution

NSSDC has been designated as the deep archive for the ISTP Level 0 data. In general they only receive and archive the "All Product" CD-ROMs. Because of its nature, Level 0 data is proprietary to the PI team for one year - after one year it is open to the public.  Thus Level 0 data less than one year old requested by someone other than a member of the PI team requires written permission from the PI.  Level 0 data over and above the initial distribution will be provided only on a charge back basis to requesters.

.c.Appendix D - Rules for Data Rights:  Geotail Mission

As of June 1989

1.  Key parameters and Overview data should not be used for actual analyses or publication by investigators of any category.  The only possible exception may be the PI’s usage of these data derived from his/her own instrument.

2.  At ISAS, the data in the SIRIUS file (namely, BPB, BRT, SRT data) are to be regarded as raw data.

3.  Scientific analysis of the Japanese experiment data in Japan is to be based on CPB and CRT.  Until the agreed-on time after the acquisition of data, the PI has the right to the data and governs the analysis and publication programs.  When more than one PI is involved in a study, the priority is to be determined by negotiation between the PIs concerned.  The PI’s right to the data is accompanied by his responsibility to produce best scientific outcome from the data.

Where:

BPB - Basic data file (level 0 uncalibrated) derived from the play-back data

BRT - Basic data file derived from the real-time data

SRT - SIRIUS data file derived from the real-time data

CPB - Calibrated and sorted data derived from the play-back data

CRT - Calibrated and sorted data derived from the real-time data

.c1.Appendix E - Rules for Data Rights:  SOHO Mission;

From the SOHO Science Operations Plan

Issue 1.1

June 1993

1.3 Use of SOHO data

1.3.1 Data rights

The objective of a coherent policy on these aspects should be to ensure the maximum exploitation of the SOHO data in order to extract the best scientific output from the mission.  For this purpose, it is necessary to find a just equilibrium between two conflicting strategies.  These are:

a) to open up free access to the widest possible community, thereby making available special capabilities and expertise from outside the SOHO teams, and

b) to protect the interests of the PI teams who have invested so much personal effort, and through rewards for this effort to motivate them to continue to work for the collective scientific interest.

Leadership in applying such a policy rests with the PIs, who have a moral responsibility for maximizing the science from the mission, as well as the structure and authority within their teams for applying a well-defined strategy.  However, since the joint exploitation of combined data sets is a key objective of SOHO, it is important to seek joint PI or SWT agreements on this policy.

1.3.2 Data access

The term Data Access defined in this section refers to the right to have access to the data, to survey them in evaluating possible use, but NOT to carry out analysis with a view to publication.

It is proposed that data access rights be accorded by all SOHO PIs to all other SOHO PIs for the purpose of operations planning during the mission.  Access rights defined in the above manner may serve many other useful functions, e.g., to allow potential investigators to verify the availability of required data sets before finalizing a proposition.

1.3.3 SOHO Science projects

SOHO differs greatly from a classical ground-based telescope or observatory satellites such as IUE, in may respects.  For SOHO, it is inappropriate (with a few exceptions) to identify a scientific proposal with a specific observing slot.  It is necessary to define science projects using SOHO in a more flexible form.  A proposed project should consist of a clear scientific objective, together with the proposed means for its accomplishment.  It might or might not include the need for new observations, the definition of new observing sequences, cooperation between SOHO instruments or with non-SOHO space or ground-based observations.  It could be based upon analysis of synoptic data over long periods, recorded primarily for completely different objectives.  It could involve the use of a new analysis or theoretical techniques to analyze existing data.  In spite of flexibility in the form, science projects should be closely defined.  Otherwise, approval to follow one research aspect could be interpreted to cover a very wide range of activities.

1.3.4 Responsibilities

It is suggested that PI individual responsibilities should include:

a) To manage, in the broadest sense the attribution of science projects amongst their team scientist and CoIs.

b) To advise on the selection of Guest Investigator (GI) proposals which concern their instrument.

c) To consider how wider access with appropriate controls could help in stimulating outside (GI) participation.

d) To define, and announce publicly a Publication Policy.  This should include a policy for Initial Publications; i.e., a declared number of early papers, formally authored, which can also give rewards for those involved in technical or engineering effort.  An On-going Publications Policy should indicate the approval or vetting procedure (if any), authorship policy, etc.

Collective PI or SWT agreements and responsibilities include:

a) Data access policy, as suggested above.

b) The establishment and managing of joint science projects.

c) Advice on the selection of GI joint science proposals.

d) To encourage a common strategy for the PI (Individual) policy.

1.3.5 Guest Investigators

A SOHO Guest Investigator program is foreseen for the EOF.  The Guest Investigators program must start one year before launch.  It is thought that ESA and NASA will issue an Announcement of Opportunity in due time.  The SOHO SWT will recommend the conditions under which the guest investigators will be recruited and will participate in the SOHO research program.

.c1.Appendix F - Rules for Data Rights:  Cluster Mission;

DRAFT

September 9, 1993

•	Access to raw data requires authorization by the Principal Investigator (PI) or his/her delegate

•	Access to Summary Parameters is, in principle, unrestricted

•	Access to Prime Parameters is restricted to the Cluster community

•	Neither Summary Parameters nor Prime Parameters are certified for publication

•	Publications ensuing from the usage of Cluster instrument data require the consent by the PI or his/her delegate (expiration time:  TBD)

These rules are subject to endorsement by the PIs (both in their wording and completeness).

RULES GOVERNING ACCESS AND USE OF THE

CLUSTER/GROUND-BASED DATABASE

DRAFT

April 5, 1995

Preamble:  Certain projects operate under a set of rules (e.g., the CDAW, DE and ISEE Rules of the Road) which identify obligations of investigators and researchers with regard to data provided by other investigators. Because the activities of and principal publications by members of the Cluster/Ground-Based (CLGB) Coordination are, by design, collaborative efforts which greatly exploit interchange of data within the research community, the following rules are adopted.

[If a conflict is identified between these rules and the rules governing a data set provided to the Cluster/Ground-based data center the project rule is adopted and an amended copy of the CLGB rules will be forwarded to each member of the CLGB coordination project.]

1. Access to the CLGB database will be granted to established, participating members of the CLGB Coordination. Members who withdraw from participation in the project are obligated to continue to respect the rules established here.

CLGB members are those scientists belonging to institution- specific research teams providing data to the CLGB database, and/or belonging to modeling/theory teams whose participation is invited by the CLGB Working Group. CLGB members are also those other individuals who, through attendance at regularly scheduled workshops, make significant contributions to organizing, reformatting, and interpreting data in the database.

2. Members of the CLGB Coordination may share data with members of their research team, but are not entitled to further distribute data provided by other investigators.  The CLGB member is

responsible to CLGB for ensuring that these rules are followed by members of his/her research team.

3. The CLGB database is established primarily for identification of suitable correlative studies by members of the CLGB. Access to the database and support software provides individual members of CLGB the means of correlating the summary data sets for the purpose of event identification.  Preparation of summary data sets is the responsibility of the investigator who supplies the data, and is not to be undertaken by other members of the CLGB, unless agreed upon by the principal investigator in question.

4. If an investigator’s summary data can be used in the more detailed analysis of an event, the investigator responsible for providing these data should be kept informed of what it is being used for, and should be given the opportunity to update the preliminary dataset with more refined data. He should be invited at an early and appropriate time to participate in the correlative analysis, and must be invited to participate as a co-author of any resulting publication or presentation, including abstracts.  All co-authors of a research paper or abstract must be given a reasonable opportunity to review that work prior to submission for publication.  If an investigator chooses to decline co-authorship after reviewing the work, that investigator is still encouraged to provide a critical review of the principal author if a disagreement exists concerning the interpretation of that investigator’s data.

5.   If an investigator declines co-authorship of a paper, an acknowledgment of that investigator’s participation shall be provided.  Authors are also requested to give acknowledgment to the joint efforts of the CLGB working group and the WDC at RAL, as this will help in making these efforts more visible to the research community and supporting agencies, and may assist in establishing and enhancing future joint activities. A short, standard wording of an acknowledgment will be offered as a suggestion.

6.   New CLGB members will be welcome at any time, and in particular after the first major (series of) publications of CLGB results. These new members will abide by these rules of the road. Access of the database according to these rules of the road will be granted until, in the first instance, December 1999. Extension of this period would be desirable and will be discussed by CLGB members in the light of gained experience. (After this period the database may be opened without restrictions, however, individual investigators will be given the opportunity to withdraw their data from the CLGB database prior to a public release.)

7.   Access to this database will be restricted by computer name to a list of institutes, approved by the CLGB working group. A log of all retrievals of a subset of the database will be sent to the relevant data provider.

8.   Submission of data to RAL for CLGB Coordination by an investigator does not constitute submission of data to the CLGB as part of any contractual obligation.

9.   Unless declared otherwise, the summary data in the CLGB database at RAL are provided to identify scientifically interesting periods. They are designed to support data analysis efforts only, and are not to be used for any other purpose without the specific authorization of the appropriate Principal Investigators. Any of the summary datasets can contain unverified data.

.c1.Appendix G - Rules for Data Rights:  Interball Mission;

INTERBALL KEY PARAMETERS

RULES OF THE ROAD

Key Parameters (KP) are fast and reliable data set on main plasma, wave, energetic particle and geomagnetic characteristics. Documentation on KP and sample KP are open, full KP are available according to these Rules of the Road.

Access Rights

All INTERBALL participants will have free access to all KP from all mission experiments. The password to the archive will be given to PIs and/or their Co-PIs in IKI RAN after the launch. It is the responsibility of PI to distribute the password among his/her Co-Is.

For the selected time intervals (e.g. during international campaigns) KP will be available to the part of the scientific community, involved in the campaign. INTERBALL community for these periods of time will have an access to appropriate KP or their analog of partner missions. This procedure is supposed to facilitate background for further more extended and detailed data analysis on PI-to-PI basis.

Other interested researches must apply to the INTERBALL Scientific Committee.

After three years all KP will have unrestricted access.

Publication Rights

KP are intended mainly for planning, event selection and preliminary studies. Explicit use of the KP data in the publication must be agreed with the PI at an early and appropriate time.

.c1.Appendix H - IACG Rules for Data Exchange;

RULES GOVERNING ACCESS AND USE OF THE

IACG 1st CAMPAIGN DATABASE

DRAFT 1/26/94

5/13/95

Preamble

The overall scientific objectives of the IACG 1st Campaign are discussed in detail in the publication edited by Whipple and Lancaster [Toward a New Era of Global Solar-Terrestrial Research: 1st IACG Campaign on Magnetotail Energy Flow and Non-linear Dynamics, SAIC, October 1992].  It is the responsibility of the IACG Working Group-3 to prepare the appropriate orbit analysis and for IACG Working Group-1 to select the times when the IACG spacecraft are in the most favorable positions for the correlative analysis thereby defining the time interval of the 1st IACG Campaign.  The IACG Working Group-2 provides support for facilitating data exchange including networks, formats, and data visualization tools.

Nearly all spacecraft and ground-base projects operate under a set of data exchange rules that identify obligations of researchers about data provided by other science research investigators.  Because the scientific aim of the IACG 1st Campaign is, by design, multi-mission collaborative efforts, which greatly extends this interchange of data within the international research community, the following rules are adopted for use in the IACG 1st Campaign.  For the 1st Campaign, the IACG core missions are Geotail, Wind, Polar, and the four Interball spacecraft.  Ancillary data are also being used in the IACG 1st Campaign and include selected instruments on (but are not limited to) IMP-8, GOES 6 & 7, the Los Alamos National Laboratory equatorial spacecraft, and a variety of ground based radar and other instruments.

These "Rules of the Road" apply only to the first IACG Campaign and do not impose any limitation on any principal investigator to distribute his/her data, or to exchange and to cooperate with other scientists and scientific groups as he/she sees fit.

Mission Rules and KP distribution

Each of the core IACG missions and relevant ancillary data sources are generating various levels of data products.  In general, the key parameter data (referred to as KP or key physical parameter plots (KPPP), or Summary Parameters) are being generated by all the IACG core missions and other ancillary data sources in a common format.

During Campaign periods the key parameter data are freely exchanged and accessible to all principal investigators (PI) and co-investigators (Co-I) on the IACG "core" missions and ancillary data instruments in accordance with the IACG rules.  IACG rules state that the key parameter data will be used for multi-spacecraft event identification and are not citable or publishable unless explicit certification is given by the appropriate instrument PI.  This rule will also apply to the IACG 1st Campaign.

Campaign Rules

These "Rules of the Road" govern access to and use of data contributed to the IACG 1st Campaign database and data analysis effort.  The IACG 1st Campaign databases may include any database which may be created for the correlative analysis of data from intervals corresponding to the IACG 1st Campaign and may be assembled to contribute to any of the 1st Campaign science objectives.  It is expected that these databases will contain a number of different data products such as: PI certified KP data (data which can be published), event data, high time resolution data, etc.

Strict adherence to these rules by all participants, and those applying to participate, is expected.  It is the responsibility of each participant to ensure that data taken by them from the IACG 1st Campaign database in whatever form (plots, listings, electronic files, etc.) are held and used according to these Rules.  Any member who withdraws from participation in the IACG 1st Campaign is obligated to continue to respect the rules established here.

Membership  (IACG Mission Scientists)

All PIs and Co-Is from IACG core missions and ancillary instruments who wish to participate in multi-mission campaign database must provide a "statement of research intent" to the 1st Campaign Coordination Committee.  This statement should include the scientific approach, data required (including his/her own), expected analysis techniques, and a list of all investigators involved.  This statement is required by the Campaign Workshop Organization Committee for the better preparation of the campaign science workshops.  The statement of intent should not exceed one or two pages.

Membership  (non- IACG Mission Scientists)

Access to the IACG 1st Campaign database will be granted to established, participating members of the IACG 1st Campaign.  Membership for other scientists wishing to contribute, but who are not PI or Co-Is from the IACG core missions or ancillary instruments, will be approved through a peer reviewed proposal process.  Campaign members are those scientists who are named in the approved proposals.  Proposal instructions are given in the attachment to these rules.

All Campaign members are responsible to the IACG 1st Campaign Coordination Committee for ensuring that these rules are followed. The IACG 1st Campaign Coordination Committee consists of A. Nishida (ISAS), M. Acuna (NASA), R. Schmidt (ESA), and L. Zelenyi (IKI) with J. Green (NASA) as lead coordinator.

Sharing of Data

For the IACG 1st Campaign, key parameter data are routinely exchanged between campaign members and are used to support the identification of events. Members of the IACG 1st Campaign may share higher level campaign data products with members of their research team, but are not entitled to further distribute the Campaign data provided by other investigators.  Distribution of detailed instrument data is the responsibility of the instrument Principal Investigator.

Data Set Preparations

The IACG 1st Campaign database will be established solely for correlative studies by members of the Campaign.  Access to the database and support software will provide individual members of

the Campaign the means of correlating the established data sets.  Preparation of an established data set is the responsibility of the investigator who supplies the data.  It is envisioned that several 1st Campaign databases will be constructed.  It is desirable that the campaign databases be located at several central organizations such as NASA, ISAS, etc.  However, the database location is a decision that is collectively made by the teams of scientists working on the 1st Campaign science objectives.

Authorship

When an investigator's data (such as PI certified key parameter data or any other processed data product) is used in the analysis of an event, the investigator who provided these data should be kept informed of what they are being used for, should be invited at an early and appropriate time to participate in the correlative analysis, and would normally have the option of being a co-author of any resulting publication or presentation, including abstracts.  Authors are reminded that acknowledgments of the joint efforts of the Campaign participants and the IACG assist in making these efforts more visible to the research community and supporting agencies, and assist in establishing and enhancing future joint activities.

Public Release of Data

Unrestricted access of the database will be granted at the conclusion of the Campaign. Non-NASA Investigators will be given the opportunity to withdraw their data from the IACG 1st Campaign database before this public release.

Approved by:

A. Nishida, ISAS		R. Schmidt, ESA

M. Acuna, NASA		L. Zelenyi, IKI

ATTACHMENT TO THE IACG 1ST CAMPAIGN

PROPOSAL INSTRUCTIONS FOR NON-IACG MISSION SCIENTISTS

Evaluation and Content

All IACG 1st Campaign proposals will be sent to the Science Working Groups (SWG) of the respective missions for evaluation and prioritization by the lead Campaign Coordinator.  In addition, a copy of the proposal must be sent to the Campaign Coordinator.  This is necessary for determining compatibility with instrument capabilities and resource limitations.

The SWG(s) will review the proposals for:

-	Potential contribution to the Scientific Objectives of the 1st Campaign

-	IACG missions to be used and how

-	Impact on instrument operations

-	Proposer's ability to complete the investigation

-	Additional data (ground-based, etc.) brought to the campaign by the proposer or other type of contribution.

-	Prior approval from the PIs whose data are required

These prioritized proposals will be forwarded from the SWG to the IACG Campaign Coordination Committee for final approval.  The IACG 1st Campaign Coordination Committee consists of A. Nishida (ISAS), M. Acuna (NASA), R. Schmidt (ESA), and L. Zelenyi (IKI) with J. Green (NASA) as lead coordinator.

The proposals are not to exceed 4 typed pages and have the following structure:

-	Title

-	Lead Investigator and other Co-Investigator(s) names and addresses

-	Scientific objectives

-	Precise definition of the data products needed

-	Research Plan and analysis methods

-	Special Requirements on instrument modes or other operations

-	Experience of the Team and their contribution.

Prior endorsements from the IACG mission instrument investigators whose data would be involved are required.

Funding

The role of the IACG is not to fund scientists for participation within the campaign process but to facilitate the coordination among the IACG missions, to acquire the data necessary to meet the campaign science objectives, and to promote the correlative data analysis.

Scientists who seek to be members of the 1st Campaign and who submit proposals must provide their own resources for their proposed research.  Therefore, no budget information needs to be part of the science proposal.

Contact

All questions concerning any aspect of the proposal or status of the review should be directed to the lead Campaign Coordinator.

James L. Green

Space Science Data Operations Office, Code 630

NASA/Goddard Space Flight Center

Greenbelt, MD  20771

.c1. Appendix I-Bibliography of ISTP Data Handling Documents;

(In time order within topic)

CLUSTER SCIENCE DATA SYSTEM (CSDS) - (abbreviated list)

Schmidt, R. and M. L. Goldstein, editors, “The Cluster Mission,” ESA SP-1103, 1988.

Cluster Science Data System:  Final Report of the CSDS Working Group, ESA-CL-EST-RP-001, February 1990.

Study of Requirements for a Cluster Science Data Analysis Support Centre, RAL and IC Consultants Ltd., December 1991.

Cluster Science Data System Requirements Specifications, ESA-DS-EST-RS-001, January 31, 1993.

Daly, P. W., W. Baumjohann, J. Dandonras, K-H. Glabmeier, C. Harvey, S. Schwartz, K. Tokar, L. J. C. Woolliscroft, “Report of the data products task group for the Cluster Science Data System,” DS-MPA-TN-001, February 1993.

Daly, P. W., et al., “Report of the data products task group for the Cluster Science Data System,” Issue 2, Revision 1, June 21, 1994.

U.S. Cluster Science Data Center Architectural Design Document, CSC, September 1993.

Schmidt, R., and L. J. C. Woolliscroft, “Cluster Science Data System - A distributed system to serve a wide international community,” submitted for publication, 1994.

CENTRAL DATA HANDLING FACILITY (CDHF)

Hardware Performance Specification for the ISTP Program Central Data Handling Facility (CDHF), Preliminary Draft, May 1988.

ISTP Program Central Data Handling Facility (CDHF) System Requirements Review (SRR) Material, July 14, 1988.

ADPE Acquisition Plan 1649 for the ISTP CDHF, Computer Sciences Corp., Contract NAS5-31500, November 1988.

ISTP Program Central Data Handling Facility (CDHF) Software Specification Review (SSR) Material, February 21, 1989.

ISTP Program Central Data Handling Facility (CDHF) Preliminary Design, Prepared under Contract NAS5-31500, Computer Sciences Corp., CSC/SD-89/6206, October 1989.

ISTP Program Central Data Handling Facility (CDHF) Operations Concept, Revision 1, May 1990.

Detailed Design Specification for the ISTP Program Central Data Handling Facility (CDHF), Computer Sciences Corp., Contract NAS5-31500, CSC/SD-90/6153, August 1990.

ISTP Central Data Handling Facility (CDHF) Critical Design Review (CDR) Material, August 30 and 31, 1990.

Data Format Control Document (DFCD) between the ISTP Program Information Processing Division (IPD) Ground Data Processing System and the ISTP Mission Investigators.  This document describes Level Zero (LZ) & SIRIUS telemetry files, orbit files, attitude files, command history files, and spin phase files which comprise the input to the KP software, along with information on SFDUs and ISTP/CDFs.

ISTP CDHF Software System Programming Guide to Key Parameter Generation Software.  This document describes the CDHF KP support routines (the so-called ICSS routines) used to interface KP software to the CDHF environment.  This document also discusses ISTP/CDFs.

ISTP CDHF Simulated Instrument Data Set (SIDS).  This document describes how KP test data is to be provided into the CDHF environment so that this test data can be mapped into LZ files and thus be used for testing of the KP software.

ISTP CDHF Software System User’s Guide.  This document describes the CDHF user services, such as query to the CDHF catalog; the transfer of data sets from the CDHF to the RDAFs; and updating quality information concerning KPs.

ADPE Acquisition Plan 1649 for the ISTP CDHF, Computer Sciences Corp., Contract NAS5-31500, updated by W. H. Mish, July 1991.

Whitman, R., ISTP Project Briefing Material, March 26, 1992.

ISTP Program Central Data Handling Facility (CDHF) System Requirements Specification, Revision 3, October 1992.

Lehtonen, K., “Architecting the International-Solar Terrestrial Physics (ISTP) Central Data Handling Facility (CDHF) 1992-2001,” Technical Reference Library, Information Processing Division, Goddard Space Flight Center, Code 562.7, Greenbelt, MD 20771, June, 1993.

COMMON DATA FORMAT (CDF)

CDF User's Guide, Version 2.5, NSSDC/WDC-A-R&S 94-11, NASA/Goddard Space Flight Center, December 1994.

CDF C Reference Manual, Version 2.5, NSSDC/WDC-A-R&S 94-12, NASA/Goddard Space Flight Center, December 1994.

CDF Fortran Reference Manual, Version 2.5, NSSDC/WDC-A-R&S 94-13, NASA/Goddard Space Flight Center, December 1994.

Goucher, G. W., and G. J. Mathews, "A Comprehensive Look at CDF", NSSDC/WDC-A-R&S 94-07, NASA/Goddard Space Flight Center, August 1994.

Kessel, R. L., R. E. McGuire, H. K. Hills, and N. J. Love,  “CDF Implementation Guidelines for ISTP”, Version 2.3,  February 7, 1994.

DATA CAPTURE FACILITY (DCF)

ISTP Program Data Capture Facility (DCF) System Requirements Specification, August 1989.

Software Requirements Specification for the ISTP Program Data Capture Facility, September 1989.

DATA DISTRIBUTION ON CD-ROM

Kulkarni, R., R. Whitman, “ISTP/CDHF Data Distribution System Design Concept,” May 11, 1992.

Rykowski, T., “Minutes of recent file naming convention/CD-ROM distribution meeting of July 29, 1992,” August 7, 1992.

Sawyer, D. and L. Reich, “Proposal for changes to the VOLDESC.SFD file on the test DDF CD-ROM, and other matters,” August 11, 1992.

Reich, L. and D. Sawyer, “Directory/Filename Mapping Requirements and Objects.  NOST/NSSDC,” Greenbelt, MD, June 1992.

DATA DISTRIBUTION FACILITY (DDF)

Data Distribution Facility Support of the ISTP Program System Requirements Review Material, Mission Support Systems Branch, Information Processing Division, May 5, 1989.

Preliminary Design Specification for Data Distribution Facility (DDF) Support of ISTP Program, Contract NAS5-31500, Computer Sciences Corp., CSC/TM-89/6145, November 1989.

Data Distribution Facility Support of the ISTP Program, Preliminary Design Review Material, Mission Support Systems Branch, Information Processing Division, December 11, 1989.

Workload Analysis for DDF Support of ISTP, Computer Sciences Corp., September 1989.

Media Analysis Report for DDF Support of ISTP, Computer Sciences Corp., November 1989.

Preliminary Design Specification (PDR) for DDF Support of ISTP, Computer Sciences Corp., November 1989.

Operations Concept for Data Distribution Facility (DDF) Support of the International Solar-Terrestrial Physics (ISTP) Program, Information Processing Division, January 1990.

System Requirements Specification for DDF Support of the International Solar Terrestrial Physics (ISTP) Program, Information Processing Division, January 1990.

Mission Operations and Data Systems Directorate (MO&DSD) Operational Development Network (MODNET) Configuration Handbook, LORAL AEROSYS/541, April 1990.

Detailed Design Specification for Data Distribution Facility (DDF) Support of the ISTP Program (Geotail Prototype), Contract NAS5-31500, Computer Sciences Corp., CSC/SD- 90/6124, June 1990.

Data Distribution Facility Support of the ISTP Program (Geotail Prototype) Material, Mission Support System Branch, Information Processing Division, July 19, 1990.

Detailed Design Specification for DDF Support of ISTP Program (GEOTAIL Prototype), Computer Sciences Corp., July 1990.

Distribution Media Analysis for ISTP, Information Processing Division, August 1990.

McCusker, T.,  “CD-ROM Production Power,” Datamation, February 1991.

Phase III DDF Concept Feasibility Report, Computer Technology Assoc., August 1991.

Phase III DDF Functional and Performance Requirements, Computer Technology Assoc., August 1991.

Data Recording Subsystem Reference and Operations Manual, Version 1.1, IHS, June 1992.

Interface Control Document (ICD) Between the Institute of Space and Astronautical Science (Japan) and Goddard Space Flight Center (U.S.) for the Transfer of ISTP/GEOTAIL Data,  Computer Sciences Corp., April 1992.

DDF Phase III Operations Concept, Computer Sciences Corp., July 1992.

DDF Phase III System Requirements Document, Computer Technology Assoc., July 1992.

Statement of Work and Specification for a Data Recording Subsystem, Information Processing Division, July 1992.

DDF Phase III System Design Specification, Computer Technology Assoc., August 1992..

Data Distribution Facility Phase II CDR Material, Mission Support System Branch, Information Processing Division, October 15, 1992.

DDF Phase II Detailed Design Specification, Computer Sciences Corp., October 1992.

DDF Phase III Preliminary Design Specification, Computer Sciences Corp., February 1993.

DDF Phase III Database Design Specification, Computer Sciences Corp., February 1993.

END-TO-END DATA SYSTEM

ISTP Project Collaborative Solar-terrestrial Research (COSTR) Initiative:  Systems & Operation Requirements Document for Geotail (SORD), 501-212/Geotail, Draft - March 1990.

Wind Mission Operations Review Material, October 21-22, 1991.

Polar Mission Operations Review Material, March 10-11, 1993.

Mish, W. H., M. Peredo, J. L. Green, and M. G. Reph, “ISTP Science Data Systems and Products,” accepted for publication, 1995.

INTER-AGENCY CONSULTATIVE GROUP (IACG)

Toward A New Era of Global Solar-Terrestrial Research - The First IACG Campaign:  Magnetotail Energy Flow and Non-Linear Dynamics, E. C. Whipple and H. Lancaster, editors, Science Applications International Corporation (SAIC), October 1992.

Toward A New Era of Global Solar-Terrestrial Research - The Second IACG Campaign:  Boundaries in Collisionless Plasmas, Science Applications International Corporation (SAIC), October 1993.

IMP-8

Mish, W. H., R. P. Lepping, “Magnetic Field Experiment Data Processing Systems:  Explorers 47 and 50,” NASA/GSFC X-694-76-158, August 1976.

Whitaker, M. P., Minutes of IMP-8 Meeting, January 7, 1992.

Presentation to the Information Processing Division CCB, “IMP-8 Quicklook Processing in the ISTP-Era,” January 16, 1992.

Memorandum of Understanding between the ISTP Projects, the IMP-8 Project and the Information Processing Division (IPD) for Expedited Quicklook Processing of Magnetometer and Solar Plasma Telemetry Data, May 5, 1992.

KEY PARAMETERS (KPS)

Mish, W. H. (Editor), “ISTP Key Parameter Generation Software (KPGS) Standards & Conventions,” NASA/GSFC, Version 1.3, March 1994.

LANL S/C

Mccomas, D. J., R. D. Belian, “Proposal to the National Aeronautics and Space Administration for Los Alamos Geosynchronous Orbit Data for ISTP to be Performed by the Los Alamos National Laboratory,” Los Alamos, NM, November 1991.

Briefing Material from ISTP Meeting at Los Alamos, February 25, 1992.

Memorandum of Agreement between the NASA Office of Space Science and Applications Space Physics Division and the LANL for MPA and SOPA Data Exchange in Support of the NASA GGS and the ISTP Program, June 1992.

NSSDC

Satellite Situation Center:  Comprehensive User’s Guide, Version 2.1, December 1993.

Van Steenberg, C., “NDADS Automated Retrieval Mail System Users’ Guide,” NSSDC/WDC-A-R&S, 93-19, May 15, 1993.

Memorandum of Understanding between the ISTP Science Initiative Office and the Space Science Data Operations Office, April 1993.

SEL/GOES S/C

Cruickshank, C. M., editor, “Space Environment Laboratory Data Acquisition and Display System II,” Space Environment Laboratory, Boulder, Colorado, March 1988.

Memorandum of Agreement Between the National Aeronautics and Space Administration (NASA) Office of Space Science and Applications Space Physics Division and the National Oceanic and Atmospheric Administration (NOAA) Space Environmental Laboratory (SEL) and the National Geophysical Data Center (NGDC) for Geostationary Operational Environmental Satellite (GOES) Data Exchange in Support of the NASA Global Geospace Science (GGS) Program, June 1992.

SCIENCE PLANNING AND OPERATION FACILITY (SPOF)

Peredo, M., “Science Planning and Operations Facility (SPOF),” March 1992.

Peredo, M., “Requirements Document for the ISTP WIND/POLAR Science Planning and Operations Facility (SPOF),” February 1992.

Peredo, M., C. C. Goodrich, D. McNabb, R. Kulkarni, C. Mesztenyi, “Interactive Tools for ISTP:  Science Planning and Data Analysis,” SPOF Movie, Version 1.0, October 1992.

Interface Control Document Between the International Solar-Terrestrial Physics (ISTP) WIND/POLAR Command Management System (CMS) and the Science Planning Operations

Facility (SPOF) and the Principal Investigators (PIs), Prepared under Contract NAS5-31500, Computer Sciences Corp., CSC/TR-90/6126, November 1992.

Peredo, M., C. C. Goodrich, D. McNabb, R. Kulkarni, C. Mesztenyi, “Interactive Tools for ISTP:  Science Planning and Data Analysis,” SPOF Movie, Version 2.0, January 1993.

Peredo, M., C. C. Goodrich, D. McNabb, R. Kulkarni, C. Mesztenyi, “Interactive Tools for ISTP:  Science Planning and Data Analysis,” SPOF Movie, Version 2.1, February 1993.

SOLAR HELIOSPHERIC OBSERVATORY (SOHO) - (abbreviated list)

Domingo, V., and T. D. Guyenne, editors, “The SOHO Mission,” ESA SP-1104, 1989.

Solar Oscillations Investigation- Michelson Doppler Imager (SOI-MDI) Instrument Data Management Plan, Draft V1.4, April 15, 1991.

SOHO Science Operations Plan, Issue 1.1, June 1993.

Poland, A., and V. Domingo, “SOHO Experiment Operations Facility Functional Requirement Document,” Draft Version 1.0, 1993.

Domingo, V., A. Poland, B. Fleck, and T. Appourchaux, “SOHO Science Operations Plan,” Issue 1.1, June 1993.

SOHO Data Archive Requirements, V2.0, January 25, 1994.

SOLAR WIND INTERPLANETARY MEASUREMENTS (SWIM)

Whitman, R., “ISTP CDHF Near Real Time Subsystem,” Computer Sciences Corp., February 19, 1991.

Memorandum of Agreement between the DOD Space Test Program (SSD/CLP) and NASA space Physics Division (NASA) for the Support of Geophysics Laboratory (GL) Experiment GL-804 (SWIM), April 12, 1991.

SWIM Preliminary and Critical  Design Review Material, Computer Sciences Corp., September 17, 1991 & February 27, 1992.

Whitaker, M. P., Minutes of Design Review for NRT Generation of SWIM KPGS, May 27, 1992.

STANDARD FORMATTED DATA UNITS (SFDUs)

Space Data Systems Operations with SFDU:  System and Implementation Aspects, CCSDS 610.0-G-5, CCSDS Panel 2 Green Book, Issue 5, February 1987.

SFDU - Structure and Construction Rules, CCSDS 620.0-B-1, CCSDS Panel 2 Blue Book, Issue 1, February 1988.

Drexler, M., D. Sawyer and G. Smith, “CCSDS Standard Formatted Data Units - Standards for Data Understanding,” AIAA:  Proceedings of the AIAA Second International Symposium on Space Information Systems, AIAA-90-5045, 1990.

McGuire, R. E., D. Sawyer, “A Simplified Overview of Standard Formatted Data Units (SFDUs) and their Proposed Application in the GGS/ISTP Program.”

SFDU - Structure and Construction Rules, CCSDS 620.0-R-1.4, CCSDS Panel 2 Red Book, Issue 1.4 - Draft Recommendation, March 1991.  [This draft, upon final agency approvals, will become an Issue 2 Blue Book that incorporates and replaces the February 1988 document].

Standard Formatted Data Units - A Tutorial, CCSDS 621.0-G-1, Green Book, May 1992.

Parameter Value Language Specification (CCSD0006), CCSDS 641.0- B-1, Blue Book, May 1992.

Parameter Value Language - A Tutorial, CCSDS 641.0-G-1, Green Book, May 1992.

.c1. Appendix J - Acronym List;

AF	- Air Force

ASCII	- American Standard Code for Information Exchange -

CA	- Control Authority

CAMMICE	-	Charge & Mass Magnetospheric Ion Composition Experiment (Polar S/C)

CCB	- Configuration Control Board

CCSDS	- Consultative Committee for Space Data Systems

CDAWs	- Coordinated Data Analysis Workshops run by NSSDC

CDF	- Common Data Format - developed by NSSDC

CDHF	- Central Data Handling Facility

CDR	- Critical Design Review

CD-WO	- Compact Disk - Write Once

CEPPAD	- Comprehensive Energetic Particle Pitch Angle Distribution Experiment (Polar S/C)

CMS	- Command Management System

CoI	-	Co-investigator

CANOPUS	-	Ground Based Investigation of high latitude ionosphere

COSTR	-	Collaborative Solar Terrestrial Research (initiative)

CPI	-	Comprehensive Plasma Composition Instrument on Geotail

CPU 	-	Central Processing Unit

CSDS	-	Cluster Science Data System

DARN	-	Dual Auroral Radar Network (a GBI)

DCF	-	Data Capture Facility

DDF	-	Data Distribution Facility

DEC	-	Digital Equipment Corp.

DECnet	-	Networking Protocol developed by DEC

DFCD	-	Data Formats Control Document

DOE	-	Department of Energy

DOS	-	Disk Operating System for IBM or IBM compatible PCs.

DSN	-	Deep Space Network

EFD	-	Electric Field Instrument (Geotail S/C)

EFI	-	Electric Field Instrument (Polar S/C)

EPACT	-	Energetic Particle:  Acceleration, Composition & Transport Instrument (Wind S/C)

EPIC	-	Energetic Particles & Ion Composition Instrument (Geotail S/C)

EOF	-	Experiment Operations Facility for SOHO

EPS	-	Energetic Particle Sensor (GOES S/C)

ESA	-	European Space Agency

ESIS	-	European Space Information System

FDF	-	Flight Dynamics Facility

GBI	-	Ground Based Investigations

GCI	-	Geocentric Celestial Inertial Coordinate System

GSE	-	Geocentric Solar Ecliptic Coordinate System

GSM	-	Geocentric Solar Magnetospheric Coordinate System

GGS	-	Global Geospace Science (Program)

GOES	-	Geostationary Operational Environmental Satellite

GSFC	-	Goddard Space Flight Center

HEP	-	High-Energy Particles Instrument (Geotail S/C)

HSC	-	DEC Hierarchical Storage Controller

HYDRA	-	Fast Plasma Analyzer (Polar S/C)

IACG	-	Inter-Agency Consultative Group

I/O	-	Input/Output

ICSS	-	ISTP CDHF Software System

IMP-8	-	Interplanetary Monitoring Platform-8 (a series of these were flown in the 1960’s and 70’s.)

IMSL	-	International Mathematical and Statistics Library

IPD	-	Information Processing Division

ISAS	-	The Japanese Institute of Space and Astronautical Sciences

ISTP	-	International Solar-Terrestrial Physics (Program)

JSOC	-	Joint Science Operations Center

KPs	-	Key Parameters

KPGS	-	Key Parameter Generation Software

LACN	-	Local Area Computer Network

LANL	-	Los Alamos National Laboratory

LEP	-	Low-Energy Particles Instrument (Geotail S/C)

MAG	-	Magnetic Field Instrument

MGF	-	Magnetic Field Instrument (Geotail S/C)

MIT	-	Massachusetts Institute of Technology

MF	-	Major Frame of telemetry

MFE	-	Magnetic Field Experiment (Polar S/C)

MFI	-	Magnetic Field Instrument (Wind S/C)

MO&DA	-	Mission Operations and Data Analysis

MPA	-	Magnetospheric Plasma Analyzer (LANL S/C)

NAG	-	Numerical Algorithms Group

NASA	-	National Aeronautics and Space Administration

Nascom	-	NASA Communication Network that links DSN to GSFC

NGDC	-	NOAA National Geophysical Data Center

NOAA	-	National Oceanic and Atmospheric Administration

NRT	-	Near Real-Time System used in SWIM

NSI	-	NASA Science Internet

NSSDC	-	National Space Science Data Center

PACOR	-	Packet Processor

PC	-	Personal Computer (usually an IBM or IBM-compatible running the DOS or OS/2 operating system)

PDR	-	Preliminary Design Review

PI	-	Principal Investigator

PIXIE	-	Polar Ionospheric X-Ray Imaging Experiment (Polar S/C)

PLA	-	Plasma Instrument

POCC	-	Payload Operations and Control Center

PPDB	-	Prime Parameter Database (Cluster)

PWI	-	Plasma Wave Instrument

QL	-	Quicklook - Telemetry that is used to monitor health and safety of S/C and instruments.

RAL	-	Rutherford Appleton Laboratory

RDAF	-	Remote Data Analysis Facility

RE	-	Earth Radii

Rs	-	Sun Radii

SDP	-	Summary Data Plots (Cluster)

SEL	-	NOAA Space Environment Laboratory

SESAME	-	Satellite Experiments Simultaneous with Antarctic MEasurements (a GBI)

SFDU	-	Standard Formatted Data Units

SIDS	-	Simulated Instrument Data Set

SIRD	-	Support Instrumentation Requirements Document

SIRIUS	-	Scientific Information Retrieval Integrated Utilization System

SOHO	-	Solar & Heliospheric Observatory

SOPA	-	Synchronous Orbit Particle Analyzer (LANL S/C)

SORD	-	Systems & Operations Requirements Document

SPDB	-	Summary Parameter Data Base (Cluster)

SPOF	-	ISTP Science Planning and Operations Facility

SRR	-	Software Requirements Review

SWE	-	Solar Wind Experiment (Wind S/C)

SWICS/STICS	-	Solar Wind & Suprathermal Ion Composition Experiment (Wind S/C)

SWIM	-	Solar Wind Interplanetary Measurements

TAE	-	Transportable Applications Executive

TCP/IP	-	Transmission Control Protocol/Internet Protocol

TGRS	-	Transient Gamma-Ray Spectrometer (Wind S/C)

TDS	-	Time Domain Sampler (part of Wind WAVES inst)

TIDE	-	Thermal Ion Dynamics Experiment (Polar S/C)

TIMAS	-	Toroidal Imaging Mass-Angle Spectrograph (Wind S/C)

UVI	-	Ultraviolet Imager (Polar S/C)

VAX/VMS	-	DEC VAX computer operating system

VIS	-	Visible Imaging System (Polar S/C)

WAVES	-	Radio/Plasma Waves Instrument (Wind S/C)

3D-P	-	3-D Plasma Analyzer (Wind S/C)

Time resolution changes from 46 sec (when the spacecraft is within 60 RE) to 92 sec (when the radial distance exceeds 60 RE) because of the change in bit rate.

Information extracted from:  “Report of the Data Products Task Group for the Cluster Science Data System,” Issue 2, Revision 1, June 21, 1994.

All vector quantities are given in GSE coordinates, except the electric field Edusk which is given in the spacecraft spin plane and is nearly parallel to the Y-GSE axis.

These experiments are collectively referred to as WEC, the Wave Experiment Consortium

Note that individual KPs for the Interball/Aurora spacecraft are computed with contributions from one or more instruments

Note that individual KPs for the Interball/Tail spacecraft are computed with contributions from one or more instruments.  EMFT = Electro Magnetic Fields Tail, GPPT = General Plasma Parameters Tail, EPTP = Energetic Particles Tail Probe.