Unit 5:  Additional Materials

# Leveraging/Cribbing Operation

- Conduct a sizeup of the scene: Gather facts, identify hazards, and establish priorities. - Have one person in charge and formulate a plan of action based upon the information you have received. Identify how and where to lift and crib. - Gather necessary materials for lifting/cribbing operations: Lever, fulcrum, cribbing blocks, spacers/wedges. - Use cribbing materials to stabilize the object prior to lifting. (Set the foundation of the box crib.) - Distribute crib materials as necessary to be readily accessible during the lifting operation. - Prepare to lift the object: Assemble the lever and fulcrum at the previously identified location. - Have someone available to handle the victim. - Initiate the lift, using the lever and fulcrum for mechanical advantage. - As the object is lifted, add cribbing as needed; build on the foundation of the box crib. - When the object is adequately supported, remove the lever and fulcrum. The victim may then be removed. - Reinitiate the lift and begin removing cribbing materials, reversing the process by which the crib was built. - Progressively lower the object to the ground. - Reassemble the lifting/cribbing supplies to be available for additional operations.

# Arrangement for Leveraging/Cribbing Operation

Team Organization for Leveraging/Cribbing Operation, showing the victim underneath a collapsed wall and the CERT members at the following locations: - Group Leader: In front of collapse, positioned so that he or she can view the entire operation while remaining out of the rescuers’ way. - Lever Person: At the front edge of the collapsed wall and positioned so that he or she can position a fulcrum and lever under the wall. - Crib Persons: On either side of the collapsed wall and positioned to enable the placement of cribbing as the wall is raised with the lever. - Medical Care/Victim Removal Person: Next to the Crib Person who is closest to the victim’s head.

# Box Cribbing

Four steps for building box cribbing: Step 1: Position two pieces of wood parallel to each other on either side of the collapse. Step 2: Place two pieces of wood perpendicularly across the base pieces. Steps 3 and 4: Add additional layers of wood, with each perpendicular to the previous level.

| # Probable Severity and Type of Earthquake Damage Based on Construction Type |  |  |  |
| --- | --- | --- | --- |
| Construction Type | Description | Probable Damage Areas | Severity |
| Single-Family Dwelling | - Wood frame - Pre-1933 - Hillside | - Masonry chimney - Utilities - Foundation movement - Utilities - Porches - Unique hazards - Ground failure | Light Moderate Heavy |
| Multiple-Family Dwelling | - Up-and-down and/or side-by-side living units | - Soft first floor - Utilities | Moderate |
| Unreinforced Brick | - Pre-1933 construction - Lime or sand mortar - “King Row” or “Soldier Row” (bricks turned on edge every 5-7 rows) - Reinforcing plates | - Arched/recessed windows and doors - Walls collapse, then roof | Heavy |
| Tilt-Up | - Large warehouses and plants - Concrete slabs lifted into place - Walls inset approximately 6-8 inches - Lightweight roof construction | - Roof collapses, then walls | Heavy |
| High-Rise | - Steel reinforced | - Broken glass - Content movement - Exterior trim/fascia | Light |