SECTION SF 30 BLOCK 14 CONTINUATION PAGE

SUMMARY OF CHANGES

SECTION SF 1449 - CONTINUATION SHEET

SOLICITATION/CONTRACT FORM

The required response date/time has changed from 19-Sep-2008 11:00 AM to 24-Sep-2008 11:30 AM.

The following have been added by full text:

DETAILED DESCRIPTION

|  | ITEM 0001 | QUANTITY | UNIT | UNIT PRICE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  |  | 2 | Each |  |  |
|  | REFRIGERATED MERCHANDISER, DROP-IN FFP Randell Model No. 44250DIMA - OR EQUAL *** Display Case, refrigerated, drop-in, reach-in, 50"L, 25-1/4"D, 44"H, mirror-back, 4 adjustable shelves, sliding self-closing glass doors, s/s exterior & interior, self-contained refrigeration system, 1/3 HP, 230V/50Hz/1Ph. *** Price shall include set-up and the cut-out of the serving line counter top. *** |  |  |  |  |
|  |  |  |  | NET AMT |  |

|  | ITEM 0002 | QUANTITY | UNIT | UNIT PRICE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | Each |  |  |
|  | HOT/COLD DROP-IN UNIT FFP Beck E.B. Großküchen Model No. H/C WELLS 2 - OR EQUAL *** Drop-In Hot/Cold Food Unit, electric, (2) 12"x20" pan size, switch for hot or cold operation, dry and wet heat, thermostatic controls, s/s top corrision resistant steel exterior, drain & gate valve, 1/4 HP, 230V/50Hz/1Ph 2.2 KW Sneezeguard with stainless steel sturgeons, top part made of curved safety glass, front side made of straight safety glass, with lights. *** Including - 1(one) each Adapter Bar, 20" - 2 (two) each Adapter Bar, 12" *** Alternative: Same unit with individually controlled wells. *** Price shall include set-up and the cut-out. *** |  |  |  |  |
|  |  |  |  | NET AMT |  |

|  | ITEM 0003 | QUANTITY | UNIT | UNIT PRICE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | Each |  |  |
|  | HOT/COLD DROP IN UNIT FFP Beck E.B. Großküchen Model No. H/C WELLS 3 - OR EQUAL *** Drop-In Hot/Cold Food Unit, electric, (3) 12"x20" pan size, switch for hot or cold operation, dry and wet heat, thermostatic controls, s/s top corrosion resistant steel exterior, drain & gate valve, 1/4 HP, 240v/50/1, 22kw, Heating elements 2200 Watts Sneezeguard with stainless steel sturgeons, top part made of curved safety glass, front side made of straight safety glass, with lights. *** Including: - 2 (two) each Adapter Bar, 20" - 6 (six) each Adapter Bar, 12" *** Alternative: Same unit with individually controlled wells. *** The price shall include the installation and the cut-out. |  |  |  |  |
|  |  |  |  | NET AMT |  |

|  | ITEM 0004 | QUANTITY | UNIT | UNIT PRICE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  |  | 2 | Each |  |  |
|  | FLIP/FLAP DOORS FFP Beck E.B. Großküchen Model No. FLIP/FLAP DOORS - OR EQUAL *** Flip/Flap doors mounted between the kitchen and the serving lines. Made of stainless steel with see thru safety glass window in the upper part. The door is consisting of the right side and left side door. Approx. 4 feet (1219mm) from the bottom the doors shall be covered with black rubber. The doors are attached to the wall with extra heavy duty hinges. Dimensions: 1400w x40d x 2010h mm *** The price shall include the installation of the doors. *** |  |  |  |  |
|  |  |  |  | NET AMT |  |

|  | ITEM 0005 | QUANTITY | UNIT | UNIT PRICE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | Each |  |  |
|  | FLIP/FLAP DOORS FFP Beck E.B. Großküchen Model No. FLIP/FLAP DOORS - OR EQUAL *** Flip/Flap door mounted between the kitchen and the serving lines. Made of stainless steel with see thru safety glass window in the upper part. The door is attached to the wall with extra heavy duty hinges. Approx. 4 feet (1219mm) from the bottom the door shall be covered with black rubber. Dimensions: 875w x 40d x 2010 h mm *** The price shall include the installation of the doors. *** |  |  |  |  |
|  |  |  |  | NET AMT |  |

|  | ITEM 0006 | QUANTITY | UNIT | UNIT PRICE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | Each |  |  |
|  | SHORT-ORDER FFP Beck E.B. Großküchen Model No. SHORT ORDER - OR EQUAL *** Installation Wall (back side) made of stainless steel, top 14 gauge. Trayslide stainless steel with tray runners , tray slide 13" depth, backside edged on, front side with stainless steel frame covered with the existing wood panels. On the backside with stainless steel back with integrated installaltion channel which is holding the electrical outlets (7 x Phase 1 and 3 x Phase 3) for the roll/push on equipment. Left side with 1640mm (64”) stainless steel side wall. Sneezeguard with stainless steel sturgeons, top part made of curved heat resistant safety glass, front side made of straight heat resistant safety glass. Over the hot food 6 well serving table with heat lamp. The other part with light. Dimensions: wxdxh 7735 x 500 x 850/1350 mm 25' 1" x 20" x 33 1/2"/53 1/8" *** Alternative: - Front and back side made of stainless steel (instead of wooden panels) - Front side with stainless steel frame covered with laminated pressed wood panels in a shade which matches the existing wood panels. - Trayslide made of 20mm thick granite – charcoal colored with stainless steel tray runners *** The price shall include demounting of the old short order line and installation of the new line. *** |  |  |  |  |
|  |  |  |  | NET AMT |  |

|  | ITEM 0007 | QUANTITY | UNIT | UNIT PRICE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | Each |  |  |
|  | SHORT-ORDER FFP Beck E.B. Großküchen Model No. MAIN LINE - OR EQUAL *** Installation Wall (back side) made of stainless steel, top 14 gauge. Trayslide stainless steel with tray runners , tray slide 13" depth, backside edged on front side with stainless steel frame covered with the existing wood panels. On the backside with stainless steel back with integrated installaltion channel which is holding the electrical outlets (8 x Phase 1 and 2 x Phase 3) for the roll/push on equipment. Right side with 1640mm (64”) stainless steel side wall. Sneezeguard with stainless steel sturgeons, top part made of curved heat resistant safety glass, front side made of straight heat resistant safety glass. Over the hot food 6 well serving table with heat lamp. The other part with light. Dimensions: wxdxh 6744 x 500 x 850/1350 mm 22' 1 1/2" x 20" x 33 1/2"/53 1/8" *** Alternative: - Front and back side made of stainless steel (instead of wooden panels) - Front side with stainless steel frame covered with laminated pressed wood panels in a shade which matches the existing wood panels. - Trayslide made of 20mm thick granite – charcoal colored with stainless steel tray runners *** The price shall include demounting of the old short order line and installation of the new line. *** |  |  |  |  |
|  |  |  |  | NET AMT |  |
|  | ITEM 0008 | QUANTITY | UNIT | UNIT PRICE | AMOUNT |
|  |  | 2 | Each |  |  |
|  | HOT FOOD TABLE FFP Beck E.B. Großküchen Model No. HFW6 - OR EQUAL *** Hot Food Table on wheels, with individual controlled wells, Top 14 gauge stainless steel with 6 x GN 1/1 individual welded-in pans max. 200mm (8") deep, dry and wet heat, separate temperature control for each well (+30°C to +110°C), drainage for each well manifold to one drain with valve. Cook side with plate shelf. Underneath with bottom shelf. Electrical cap.: 6.0 KW - 400V/50Hz/3Ph Dimensions: wxdxh 2400 x 800 x 850mm, 94 1/2"" x 30 1/2" x 33 1/2" *** |  |  |  |  |
|  |  |  |  | NET AMT |  |

|  | ITEM 0009 | QUANTITY | UNIT | UNIT PRICE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | Each |  |  |
|  | KICK BASE STAINLESS STEEL FFP Beck E.B. Großküchen Model No. KICK BASE - OR EQUAL *** Kick base stainless steel, for the existing Serving Line, seales the counters to the floor against water, protects the food against moisture. Water can not flow under the counters. Length around the counters (approx. 64 meters (210') Heigh: 6" *** Alternative: KICK BASE STAINLESS STEEL (approx. 49.5 meters (163’) *** The price shall include the installation of the kick-base. *** |  |  |  |  |
|  |  |  |  | NET AMT |  |

The following have been deleted:

DETAILED DESCRIPTION

(End of Summary of Changes)