# TEITAC Hardware Subcommittee
Report

## September 4, 2007

# 2.1-A – Contrast Ratio for Display (Background)

## Objective:

### Identify a contrast ratio measure for backlit and reflective display technologies that supports access

## Areas of Controversy:

### Conflict between guidance provided in non-accessibility based ISO standards for displays (9241-3)

### Concerns about technical measurement requirements.  Light measurement in theory versus practice.

## Process

### A LOT of discussion at the Hardware SC and TEITAC plenary

### Draft language submitted to EWG & listserve

## Status

### Final proposed language submitted

# 2.1-A - Contrast Ratio for Display

## For displays on products, at least one mode must exist where the contrast ratio between the luminance at light state compared to luminance at dark state shall be X:1.

### Note:  Light (or high) state typically refers to the lighter condition for the display. For example, white content/background on an LCD. Dark (or low) state typically refers to the darker condition for the display. For example, black content/background on an LCD.

# 2.1-B - Contrast Ratio for Legends and Instructions (Background)

## Objective:

### Identify a contrast ratio measure for fixed “printed” legends and/instructions that are a fixed element to a product that supports access

## Areas of Controversy:

### Conflict between guidance provided in non-accessibility based ISO standards for legends (9241-4)

### Concerns about technical measurement requirements.  Measurement protocols for determining contrast ratio.

## Process

### A lot of discussion at the Hardware SC and TEITAC plenary

### Draft language submitted to EWG & listserve

## Status

### Final proposed language submitted

# 2.1-B - Contrast Ratio for Legends and Instructions

## If the legend or instructions on the device are the only means of conveying information, then the contrast ratio shall be X:1 and the relative luminance difference must be at least 15% of XXX.

### Note:  XXX is a placeholder for the final scale on which the 15% is based.

### Note:  If other means of conveying the information conveyed by the label or instructions, is uniquely tactilely discernible though shape and/or location, exist then this contrast ratio requirement does not apply.

### Note:  This requirement excludes system labels, such as the regulatory labels, where information can be found in other sources associated with the product either in hard or soft copy format.

# 2.1-D - Touch Operated Controls (Background)

## Objective:

### ensure there is a way for people with disabilities to use a device with touch-operated controls

## Areas of Controversy:

### Conflict between accommodation techniques between types of disabilities (ie. what is good for someone who is blind is not necessarily good for someone with physical disabilities, and visa versa)

### Concerns about using speech input as the only alternate means to touching the screen (not always possible or appropriate)

### Technical and economical feasibility concerns (eg. The iPhone, capacitive touch control panels, etc)

## Process

### A lot of discussion at the Hardware SC and TEITAC plenary

### Draft language submitted to EWG & listserve

## Status

### Final proposed language submitted

# 2.1-D - Touch Operated Controls (Language)

## If a product utilizes touch screens or touch-operated controls then the product must provide, directly or through AT, and without requiring user-speech:

### A functionally equivalent, alternate means of operation that uses Mechanical Controls and does not require vision.

### A functionally equivalent, alternate means of operation that does not require fine motor control.

## Note:  This provision should not discourage the provision of control via user speech in addition to the above methods.

# 2.1-E - Connector or Connection Language (Background)

## Objective:

### Ensure there is a connection point for AT (or other customized input methods) to IT, to accommodate user input provided via “connections”

## Areas of Controversy:

### IT devices with proprietary connections (connector and/or protocol)

### Sometimes needed for reasons of security, market advantage, reliability, etc.

### Proprietary connections make it economically unfeasible for some AT to accommodate

### Typical devices where this is an issue:  telecom (cell phones), portable media players (iPods), Closed Systems

## Process

### Collaboration with the Telecom SC on draft language

### Working group struck under Telecom & HW SC (Mary, Randy, Paul)

### Draft language submitted to listserve and sub-committees

## Status

### Language still under development

# 2.1-E - Connector or Connection Language

## If users can access and control the user interface of a product through a non-standard user connection, they must also be able to control that functionality through a standard user connection using standard protocols for that type of input or output. If an adapter is required to convert a non-standard user connection on an E&IT device into a standard user connection, it shall be the responsibility of the E&IT vendor to provide such adapter.

### Note:  “Standard Connection” shall be deemed to be a connection that has been readily adopted by industry and is in common and current use. For example, while a serial port with a DB25 connector used for mouse input was a standard connection many years ago, it is no longer in common and current use on modern computers, and would no longer be considered a standard connection.

# 2.2-C - Audio Connection (Background)

## Objective:

### Ensure consistent access for addressing audio connections

## Areas of Controversy:

### Addressing public versus private impact to design and access

## Process

### A LOT of discussion at the Hardware SC , telecom, and TEITAC plenary

### Draft language submitted to EWG & listserve

## Status

### Final proposed language submitted

# 2.2-C - Audio Connection

## When products provide auditory output beyond simple tonal feedback or signaling, the audio signal must be provided at a standard signal level through an industry standard connection that will allow for private listening.

### If the product is a public-shared product then the connection must be via a standard 2.5mm or 3.5mm phone jack.

### If the product is personal-private product then the connection can be wired or wireless as long as a 2.5mm or 3.5mm phone jack adapter is available from the manufacturer.