SECTION SF 30 BLOCK 14 CONTINUATION PAGE

SUMMARY OF CHANGES

SECTION SF 1449 - CONTINUATION SHEET

SOLICITATION/CONTRACT FORM

The required response date/time has changed from 02-Sep-2008 02:00 PM to 09-Sep-2008 02:00 PM.

The following have been modified:

SPECIFICATIONS

Statement of Work

F-22 Maintenance Stands

Requirement

1 sets of F-22 wing stands [a set is comprised of one left wing and one right wing stand.]

Specifications

Material

- The Stands should be constructed of aluminum

  - No painting required.

  - Stand will be of sufficient strength to accommodate 4 personnel and a large tool box.

Dimensions

- Must have a work deck spanning the entire length of the wing leading edge, and be no more than 8’ wide, with a tread grip type surface incorporated into the work deck.

- Stands should be height adjustable and should be able to be placed at various heights to accommodate different heights of the aircraft.

- The work deck should have  a four inch high toe kick, plus an additional 18 inch high expanded metal fence and also include a handrail at least 42” tall, around all sides, except the area mating to the wing leading edge.

- See F-22 dimensions for wing length and height and area where work will occur.

  - Height of stand needs to be within of few inches if tge tio if the wing so the maintainers will only have to take a small step up to access top of jet.

Safety

- Stands must comply with OSHA and Air Force safety standards.

- The stand must have an extended guard rail along the front portion of the intake, precluding falls from that area.

- The Wing Stands should have full swivel lock, heavy duty casters, with brakes and be easy to roll.

Additional

- Load capacity should be able to support a large tool box and at least one technician [about 1,000 lbs].

- All areas that might contact the Jet will be padded with non marking chemical resistant padding.

- There should be a 50’ retractable grounding wire incorporated for grounding stand during work in explosive environment

- A tow bar will be provided on each stand

  - The tow bar should be able to be stowed out of the way. If the stand needs to be towed to a location outside its normal work area the tow bar can be unlatched and employed.

- There shall be four heavy duty reels on the upper deck and four in the lower area with two each electrical cords (explosion proof) and two each air hoses measuring 35’ in length. When no longer needed, a simple tug would release them back into their stowed position.

- Lighting (explosion proof to meet OSHA and AFOSH standards for paint facilities) should be provided to “Flood” the area under wing.

(End of Summary of Changes)