- Overview Field Activity Collection System (FACS) Log Example

| Topic | Information |
| --- | --- |
| USGS Activity ID i.e. YYPRJ## * | 08LCA02 |
| Other ID (if any) |  |
| Organization(s)/Program | St. Johns River Water Management District, Palatka, FL - United States Geological Survey, Saint Petersburg, Florida, Center for Coastal and Watershed Studies, Coastal and Marine Geology Program |
| Project/Theme | Florida Aquifer / sinkhole / cave study |
| Area of Operation | Central Florida Lakes |
| Principal Investigator(s) | Jeff Davis (SJRWMD) and Christopher Reich (USGS) |
| Information Specialist(s) | Jordan Sanford |
| Activity Type | High Resolution Seismic Reflection; DC Continuous Resistivity Profiles (CRP) and Stationary, High Resolution DC Resistivity |
| Scientific Purpose/Goals | The objective of this contract is to obtain HRMSRP and DC Resistivity profiles as applicable to the sites designated as Sentinel Lakes to characterize the subsurface of each water body. Of special concern is identifying discontinuities in the confining units between surface water and groundwater systems. |
| Platform | John boat/whaler/land-based surveys |
| Starting Date | 04/07/2008 (CRP) |
| Starting Port/Location | Johns Lake, FL |
| Ending Date | 04/18/2008 (land-based Resistivity) |
| Ending Port/Location | Lake Colby, FL |
| Equipment Used | Advanced Geosciences, Inc. (AGI) Supersting R8/IP,100-m CRP cable (10-m electrode spacing) and Lowrance DGPS WAAS navigation for CPR system; AGI Supersting R8/IP and 112-m-long cable (2-m electrode spacing) for stationary DC Resistivity; Yellow Springs Instrument (YSI) 556MPS. |
| Information to be Derived (for example, Grain Size, Depth to Basement) | Resistivity techniques will be utilized to obtain information on geologic framework and composition in lakes identified in the project. |
| Summary of Activity and Data Gathered | CRP surveys were conducted in 8 lakes: Avalon, Cherry, Helen, Hiawassee, Johns, Louisa, Saunders and Sylvan. Land-based resistivity surveys were conducted in 3 lakes: Colby, Crooked, and Prevatt. |
| Notes (include staff, shop time etc) | Jordan Sanford |

*The St. Petersburg naming convention is as follows: YYPRJ##, where YY is a 2 digit abbreviation for the calendar year the data were collected, PRJ is a 3 letter acronym for the project, task or theme the data were collected under and ## is a sequential number for each field activity under that PRJ for that calendar year.  If a project already has adopted a standardized unique and meaningful field activity naming convention, there are ways to incorporate it into the system. Please caution against using strictly geographic specific PRJs.  We strongly suggest using topical ones instead, since locations may be revisited for various purposes during a field season.  Arnell Harrison (aharrison@usgs.gov x3111) will coordinate assigning these Field Activity Ids with input from the PIs or project data managers.