Preservation Note 2

Reproducing Historic Paint Colors

Finishes

Revised Feb-95

Painting-09900

--------------------------------------------------------------------------------

Include the following clause in the special Conditions Section of the painting specifications for General Administration historic properties:

--------------------------------------------------------------------------------

09900 Painting - Reproducing Historic Paint Colors:

"Custom tint paints by computer matching. Color selection must follow the codes indicated on the finish schedule. Accurate color reproduction is accomplished by matching modern paints to the sample color chips corresponding to each code. There are two coding systems for describing color. They are the Munsell and Plochere color systems.

The contractor must obtain the coded chips for each color indicated in the finish schedule. Purchase chips directly from the following firm(s):

Munsell Color System/MacBeth

405 Middle Britain Road

New Winsor, NY 12553-6148

(800) 622-2384

FAX (914) 565-2511

(914) 565-7660

(8 x 10 cards only)*

Plochere Color System

1818 Hyperion Avenue

Los Angeles, CA 90027

(213) 661-0700

Chips range from $2 (Plochere 3" X 5") to $35 (Munsell 8" X 10"). All color chips must be computer matched. Major paint manufacturers known to be capable of computer color matching include Sherwin William's, Benjamin Moore, and Duron. Visual matching is not acceptable.

Submit written verification of having color chips computer matched to paints to the Contracting Officer 48 hours prior to painting. These verifications must be written on paint manufacturer's stationary and hand signed by the individual conducting the computer color matching. No substitution for coded colors will be accepted."

* Smaller Munsell Cards are available from:

Frank Welsh

PO Box 767

Bryn Mawr, PA 19010

(215) 525-3564

(215) 525-1333