ENERGY STAR Commercial Food Service Equipment:

Sample Procurement Language

Commercial Dishwashers

The Vendor Must:

Provide commercial dishwashers that earn the Energy Star and meet the Energy Star specifications for energy and water efficiency as outlined below. The vendor is encouraged to visit energystar.gov for complete product specifications and an updated list of qualifying products.

| Efficiency Requirements for Commercial Dishwashers—Effective October 11, 2007 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Machine Type | High Temp Efficiency Requirements |  | Low Temp Efficiency Requirements |  |
|  | Idle Energy Rate* | Water Consumption | Idle Energy Rate* | Water Consumption |
| Under Counter | < 0.90 kW | < 1.00 gal/rack | < 0.5 kW | < 1.70 gal/rack |
| Stationary Single Tank Door** | < 1.0 kW | < 0.950 gal/rack | < 0.6 kW | < 1.18 gal/rack |
| Single Tank Conveyor | < 2.0 kW | < 0.700 gal/rack | < 1.6 kW | < 0.790 gal/rack |
| Multiple Tank Conveyor | < 2.6 kW | < 0.540 gal/rack | < 2.0 kW | < 0.540 gal/rack |

* Idle energy rate as measured with door closed and rounded to 2 significant digits.

** Includes pot, pan, and utensil machines.

Commercial Solid Door Refrigerators and Freezers

The Vendor Must:

Provide commercial solid door refrigerators and freezers that earn the Energy Star and meet the Energy Star specifications for energy efficiency as outlined below. The vendor is encouraged to visit energystar.gov for complete product specifications and an updated list of qualifying products.

| Product Type | Current Criteria (Energy Consumption Under Test Conditions) |
| --- | --- |
| Refrigerators | ≤ 0.10 V + 2.04 kW-hours/day |
| Freezers | ≤ 0.40 V + 1.38 kW-hours/day |
| Refrigerator-Freezers | ≤ 0.27 AV - 0.71 kW-hours/day |
| Ice Cream Freezers | ≤ 0.39 V + 0.82 kW-hours/day |

V = Internal volume in ft3

AV = Adjusted volume = (1.63 x freezer volume in ft3) + refrigerator volume in ft3

Commercial Fryers

The Vendor Must:

Provide commercial fryers that earn the Energy Star and meet the Energy Star specifications for energy efficiency as outlined below. The vendor is encouraged to visit energystar.gov for complete product specifications and an updated list of qualifying products.

| Energy Efficiency Requirements for Open Deep-Fat Gas Fryers—Effective August 15, 2003 |  |
| --- | --- |
| Heavy Load (French fry) Cooking Energy Efficiency | > 50% |
| Idle Energy Rate | < 9,000 Btu/hr* |

*Based on 15-inch fryer

| Energy Efficiency Requirements for Open Deep-Fat Electric Fryers—Effective August 15, 2003 |  |
| --- | --- |
| Heavy Load (French fry) Cooking Energy Efficiency | > 80% |
| Idle Energy Rate | < 1000 watts* |

*Based on 15-inch fryer

Commercial Hot Food Holding Cabinets

The Vendor Must:

Provide commercial hot food holding cabinets that earn the Energy Star and meet the Energy Star specifications for energy efficiency as outlined below.  Dual function equipment, such as cook-and-hold models, cannot qualify as ENERGY STAR. The vendor is encouraged to visit energystar.gov for complete product specifications and an updated list of qualifying products.

Current Criteria—Effective August 15, 2003

Maximum Idle Energy Rate = 40 watts/ft3

The maximum idle energy rate is based on the “idle energy rate—dry test” in ASTM F2140-01. Interior volume (ft3) of each qualifying model must be measured according to the protocol provided below.

Measuring Interior Volume: Commercial hot food holding cabinet interior volume shall be calculated using straight-line segments following the gross interior dimensions of the appliance and using the following equation: interior height x interior width x interior depth. Interior volume shall not account for racks, air plenums or other interior parts.

Commercial Ice Machines

The Vendor Must:

Provide commercial ice machines that earn the Energy Star and meet the Energy Star specifications for energy and water efficiency as outlined below.  Ice machines that use water-cooled technology as well as flake and nugget ice machines are not eligible for ENERGY STAR under Version 1.0.  EPA intends to include flake and nugget ice machines once a test standard is made available and a robust database is established that may be used to derive performance requirements. The vendor is encouraged to visit energystar.gov for complete product specifications and an updated list of qualifying products.

| Efficiency Requirements for Commercial Ice Machines—Effective January 1, 2008 |  |  |  |
| --- | --- | --- | --- |
| Equipment Type | Harvest Rate, H (lbs ice/day) | Energy Use Limit (kWh/100 lbs ice) | Potable Water Use Limit (gal/100 lbs ice) |
| Air-Cooled |  |  |  |
| IMH | < 450 | 9.23 – 0.0077H | < 25 |
|  | > 450 | 6.20 – 0.0010H | < 25 |
| RCU (without remote compressor) | < 1000 | 8.05 – 0.0035H | < 25 |
|  | > 1000 | 4.64 | < 25 |
| RCU (with remote compressor) | < 934 | 8.05 – 0.0035H | < 25 |
|  | > 934 | 4.82 | < 25 |
| SCU | < 175 | 16.7 – 0.0436H | < 35 |
|  | > 175 | 9.11 | < 35 |

Commercial Steam Cookers

The Vendor Must:

Provide commercial steam cookers that earn the Energy Star and meet the Energy Star specifications for energy efficiency as outlined below. Only 3-, 4-, 5-, and 6-pan capacity units can qualify for ENERGY STAR. Models may include countertop models, wall-mounted models and floor-models mounted. The vendor is encouraged to visit energystar.gov for complete product specifications and an updated list of qualifying products.

| Energy Efficiency Requirements for Electric Steam Cookers—Effective August 1, 2003 |  |  |
| --- | --- | --- |
| Pan Capacity | Cooking Energy Efficiency* | Idle Rate** (watts) |
| 3-pan | 50% | 400 |
| 4-pan | 50% | 530 |
| 5-pan | 50% | 670 |
| 6-pan | 50% | 800 |

*Cooking Energy Efficiency is based on heavy load (potato) cooking capacity.

| Energy Efficiency Requirements for Gas Steam Cookers—Effective August 1, 2003 |  |  |
| --- | --- | --- |
| Pan Capacity | Cooking Energy Efficiency* | Idle Rate** (Btu/h) |
| 3-pan | 38% | 6,250 |
| 4-pan | 38% | 8,350 |
| 5-pan | 38% | 10,400 |
| 6-pan | 38% | 12,500 |

*Cooking Energy Efficiency is based on heavy load (potato) cooking capacity.**Idle Energy Rate: The rate of appliance energy consumption while it is maintaining or holding at a stabilized operating condition or temperature.

Vending Machines

The Vendor Must:

Provide vending machines that earn the Energy Star and meet the Energy Star specifications for energy efficiency as outlined below. The vendor is encouraged to visit energystar.gov for complete product specifications and an updated list of qualifying products.

| # Energy Consumption |  |
| --- | --- |
| Current Criteria—Effective April 1, 2004 | New Criteria—Effective January 1, 2007 |
| Y = 0.55 [8.66 + (0.009 x C)] | Y = 0.45 [8.66 + (0.009 x C)] |

Y = 24 hr energy consumption (kWh/day) after the machine has stabilized

C = vendible capacity

Low Power Mode: In addition to meeting the 24-hour energy consumption requirements listed above, qualifying models shall come equipped with hard wired controls and/or software capable of automatically placing the machine into a low power mode during periods of extended inactivity while still connected to its power source to facilitate the saving of additional energy, where appropriate. The machine shall be capable of operating in each of the low power mode states described below:

1. Lighting low power state – lights off for an extended period of time.

2. Refrigeration low power state – the average beverage temperature is allowed to rise above 40°F for an extended period of time.

3. Whole machine low power state – the lights are off and the refrigeration operates in its low power state.

In addition, the machine shall be capable of automatically returning itself back to its normal operating conditions at the conclusion of the inactivity period. The low power mode-related controls/software shall be capable of on-site adjustments by the vending operator or machine owner.

Note: EPA’s goal in including these low power mode requirements is to ensure that existing machine software capabilities are available and may be used to their fullest potential based on the individual requirements of the host site. However, machines that are vending temperature sensitive product, such as milk, must not have the refrigeration low power state enabled on site by the vending operator or machine owner due to the risk of product spoilage.