CECOM Material Safety Data Sheet

1. Product and Manufacturer:

Item Identification: BA5374/U                          Hazardous Characteristics Code:

National Stock Number:   6135-01-455-9646                 (For Agency use ONLY)

Type Number: BA5374/U w 2 DL123 CELLS                   Each Battery shall weigh no more

Common Name: BATTERY NON-RECHARG,DRY       than 2.5 ounces. Each battery contains 1.98          Contract Number:                                                                        grams of Lithium maximum

Manufactures Identification: SANYO DL123                   Each battery is cylindrical in shape with the                                                                                                                               following dimensions:

Overall Length= 2.307" (+-) .03

Manufacturer’s Federal Supply Code (CAGE): OGVJ8

Name and Address: UNICOR-Federal Prison Ind., Inc, Hwy 698 & Fairton-Millville Rds., P.O. Box 99, Fairton, N.J. 08094

Emergency telephone number: 856-453-4175/4179

Information telephone number: Same as above

Effective Date: 7/5/00

Date Submitted:

Name, Title and Signature:                               /s/

of Preparer                            Michael J. Killmer, UNICOR Factory Manager

Section II. Hazardous Ingredients:

Hazardous

Components                                    Exposure     Limits

(Chemical Name                                             OSHA PEL       ACGIH          OTHER                %

And                               CAS                      (Ppm or             (Ppm or          Recommended     Item

Symbol)                            #                           mg/m3)             mg/m3)         Limits                   Wgt

Lithium                              7439-93-2                   ------                 -------                                         2.8

Maganese Dioxide             1313-13-4                   5 mg/m3         5 mg/m3                                   35

Graphite (synthetic Gr)                                         10 mg/m3      10 mg/m3                                 3.5

Propylene Carbonate (PC)  108-32-7                                                                                             6

Dimethoxyethane(DME)    110-71-4                                                               3 mg/m3             1.5

Lithium Perchlorate (LiCIO4)                                                                                                       0.3

AMSEL Form 1164-E (Rev. Dec 97)    ( Previous Edition Obsolete )

Page 1 of 4

Section III. Physical Properties:

Boiling Point( C):  PC: 242   DME: 85            Melting Point: Not Determined

Vapor Pressure(@ 20 ): PC: 0.03, DME: 61     Vapor Density(Air=1): DME 3.1

Evaporation Rate (butyl acetate=1: DME: 4.99

Solubility in Water: PC: moderately soluble DME: completely soluble

DO NOT EXPOSE DAMAGED BATTERIES TO WATER

Specific Gravity (H2O=1):   MnO2: 4.9     Gr: 2.4    Li; 0.53

PC: 1.2         DME: 0.87    LiCIO4: 2.4

Odor And Appearance:   PC is odorless and a colorless liquids. Li is a soft, silvery metal.

MnO2 and Gr are black powders. DME is a colorless liquid with a sweet odor.

Section IV. Fire & Explosion Hazards

Flash Point: N/A , If not abused physically or mechanically

LEL: N/A            PEL: N/A

Extinguishing Media: Lith-x,Dry Sand, Dry Graphite Powder, Cover Completely

Special Fire Fighting Procedures: Allow battery to cool while covered with extiguishing media, remove adjacent combustible materials if safe to do so, SCBA required.

Unusual Fire & Explosion Hazards: DO NOT USE WATER TO EXTINGUISH FIRES, CELLS CAN REACT WITH WATER VIOLENTLY. Cells can vent electrolyte if subjected to extreme high temperatures. Heating above 354 F will lead to melting of lithium and represents a severe fire and explosion hazard.

Section V. Reactivity Data:

Stability:    Stable

Conditions to avoid: Exposure to extreme high temperatures, disassemble, recharge or shorting.   Incompatibility:    Do not expose damaged batteries to water.

Hazardous Decomposition Products: May include Carbon Monoxide and caustic Lithium Oxide  Hazardous Polymerization: Will Not Occur

AMSEL Form 1164(Rev. E Dec 97) (Previous Obsolete)

Page 2 of 4

Section VI. Health Hazard Data

Routes of Entry: (Electrolyte)              Inhalation

Dermal

Ingestion

Health Hazards: Risk occurs if battery is misused, cell vents or is opened.

Acute: Electrolyte is an irritant. Intact lithium is reactive with moisture or water when batteries are damaged do to misuse.

Chronic: May cause Dermatitis. Some components have been shown to be teratogenic to animals. Avoid direct contact.

Carcinogenicity: NTP: no

IARC: no

OSHA: no

Signs of exposure: Exposure to skin or eyes may cause irritation. Inhalation may result in difficulty breathing.

Medical conditions aggravated by exposure:    Unknown

First Aid Procedures:

Eye:             Flush with water for 15 minutes. Seek medical attention.

Skin:            Flush with water for 15 minutes. Seek medical attention.

Inhalation:    Remove to fresh air. Seek medical attention.

Section VII.  Safe Handling & Use:

Steps to be taken if material is released or spilled:

Allow battery to cool and vapor to dissipate. Avoid contact. Avoid inhalation of vapors. Remove spilled electrolyte with absorbent material. Wear full face organic respirator.

Waste Disposal Method:  Dispose of in accordance with local, state and federal regulations. This battery is hazardous waste in AK, CA, MN, RI and WA states, which biossay waste characterization techniques.

Precautions to be taken in handling and storage: Batteries may explode, ignite or vent electrolyte if crushed ,opened ,punctured , or exposed to extreme temperatures. Do not short circuit, install with incorrect polarity,or attempt to recharge. Do not incinerate.

AMSEL Form 1164-E(Rev. Dec 97)         (Previous Edition Obsolete)

Page 3 of 4

Section VIII. Personal Protection

Respiratory Protection:        Use full-face organic vapor respirator if cell vents.

Ventilation:                            Provide if cells vent.

Hand Protection:                   Natural or Butyl Rubber Gloves.

Eye Protection:                      Chemical Goggles.

Protective Clothing:               Use encapsulating suit if fire occurs.

Hygienic Practices:                 Avoid exposure. Wash hands after handling.

Section IX. Transportation Data

Application Regulation:      49CFR 172.101,173.185,IATA

Proper Shipping Name:          Lithium Battery, Solid Cathode

Class:                                       N/A

UN ID No.:                              N/A

Packing Group:                       N/A

Label:                                       N/A

Transportation: This battery contains less than 1 gram/cell and less than 2 grams/battery of lithium. It is not subject to Hazardous Material shipping requirements.

AMSEL Form 1164-E(Rev.        Dec 97)                (Previous Edition Obsolete)

Page 4 of 4