Updated : May 1988

QUALITY CONTROL CRITERIA - CIMIS WEATHER DATA

All hourly and daily weather data will be flagged according

to the following quality control criteria as it is received

from the weather stations.

All quality control checks are based on metric units.

There are two types of quality control flags :

1) Severe

where :

C - data value is not collected by this station

M - data value is not available

S - sensor is not in service

I - data value has no meaning/ignore

R - out of range (severe)

2) Informative

where :

F - data has been calculated.  See flag description for

Net Radiation and Evapotranspiration sensor

calculations.

H - daily data value flagged when corresponding hourly

data is severe.  (Note: this flag is not set when

the corresponding hourly data is N/C.)

Y - out of range

Q - all quality control could not be performed because

a comparison sensor is severe.  (Note: this flag is

not set when the comparison sensor is N/C.)

Hourly and daily values will be flagged N/C and N/A when appropriate.

CIMIS QUALITY CONTROL CRITERIA             2

HOURLY DATA

Sensor

#        Name             Flag       Description

------     ----             ----       -----------

1    Solar Radiation (Rs)  S  when Rs <= -50 or Rs >= 4000 W/m2

R  when Rs/I >= 1 and COS(Z) >= .1736

R  when Rs <= 0 and COS(Z) >= .1736

R  when (Rs <= -10 or Rs > 10)

and COS(Z) <= -.1736

Y  when Rs/I > .85 + 20

and COS(Z) >= .1736

Y  when Rs <= -6 or Rs >= 6

and COS(Z) <= -.1736

where :

I = an estimate of hourly solar incident radiation

2

in W/m  received on a horizontal surface assuming

no atmosphere.  The portion in brackets is a

correction for the change in distance between

Earth and Sun over a year.

I = S * {1.00011 + .034221 COS(X) + .00128 SIN(X)

+ .000719 COS(2X) + .000077 SIN(2X)} COS(Z)

2

S = solar constant = 1367 W/m

X = 2 * 3.14 (d-1)/D

d = day of year

D = # of days in the year

COS(Z) = the cosine correction for the angle of incidence

COS(Z) = SIN(Y) SIN(DEC) + COS(Y) COS(DEC) COS(A)

Y = latitude in radians (+ for N. latitude)

DEC = declination

DEC =  .006918 - .399912 COS(X) + .070257 SIN(X)

- .006758 COS(2X) + .000907 SIN(2X)

- .002697 COS(3X) + .001480 SIN(3X)

A = hour angle = 15(L - 12)(2 * 3.14 / 360)

L = local apparent time = (t - .5) - 1 + F + E

t = local standard time (1 to 24)

F = time difference (hours) from the standard

meridian west of the location

= 4 * (ST - LO) / 60

(note: If ST-LO is less than 7.5 then F=F+1.0)

ST = (FIX(LO / 15) + 1) * 15

where FIX truncates the fraction leaving

an integer

LO = longitude

E = equation of time (hours) to correct for

day length < > 24 hours

= (.000075 + .001868 COS(X) - .032077 SIN(X)

- .014615 COS(2X) - .04089 SIN(2X)) * 229.18 / 60

Note : when COS(Z) = .1736 the sun is 10o  above the horizon.

CIMIS QUALITY CONTROL CRITERIA             3

HOURLY DATA

Sensor

#        Name             Flag       Description

------     ----             ----       -----------

2    Net Radiation (Rn)    S  when Rn >= 4000 W/m2

R  when Rn < -100 or Rn >= 800

R  when Rn > 10 and COS(Z) <= -.1736

Q  when T is severe

Q  when Rs is severe

R  when Rn >= 1.15*ERmax + 30

and COS(Z) >= .1736

R  when Rn >= 1.3*ERmax

and COS(Z) >= .35

R  when Rn <= .6*ERdmin - 30 and

COS(Z) >= .35 and ERdmin > 0

Y  when Rn <= .7*ERdmin - 20 and

ERdmin >= 0 and COS(Z) >= .1736

Y  when Rn <= 1.25 * ERdmin and

ERdmin < 0 and COS(Z) >= .1736

Y  when Rn >= 1.1*ERmax + 20

and COS(Z) >= .1736

Y  when Rn < 1.15*ERnmin

and COS(Z) < -.1736

Y  when Rn > -.1 and COS(Z) <= -.1736

F  when Rn is severe and has been

calculated.  The user must

specify ' -Q' when requesting

station #'s to view this flag.

(i.e. Enter station #'s > 6 -Q)

where :

a = .25 = albedo of short grass

Rld = downward flux of long-wave radiation (W/m2 )

Rld = Sb * T4 (1 - .261 EXP(-7.77 * 10-4  (273 - T)2 )

Rlu = upward flux of long-wave radiation (W/m2 )

Rlu = Sb * T4

T = air temperature in Kelvin

Sb = 5.64E-8 = Steffen Boltzman constant

ERdmin = estimate of the minimum daytime net radiation

for a given value of solar radiation.  During

lapse conditions, it has been suggested that

the Rld equation is over estimated and should

be reduced by 20 W/m2.

ERdmin = (1 - a)Rs + (Rld - 20) - Rlu

ERnmin = estimate of the minimum nighttime net radiation

for a given value of solar radiation.

ERnmin = Rld - Rlu

ERmax = estimate of the maximum net radiation for a

given value of solar radiation.

ERmax = (1 - a) Rs

CIMIS QUALITY CONTROL CRITERIA             4

HOURLY DATA

Sensor

#        Name             Flag       Description

------     ----             ----       -----------

o

3    Soil Temperature      S  when Ts <= -50 or Ts >= 100 C

at 15 cm (Ts)         R  when Ts < -10 or Ts > 45

R  when > 3 between this and

previous hour

Y  when Ts < 0 or Ts > 38

Y  when > 2 between this and

previous hour

o

4    Air Temperature (T)   R  when T < -15 or T > 60 C

Y  when T < -10 or T > 55

5    Vapor Pressure (e)    R  when e <= 0 kPa

Q  when T is severe

R  when e > 1.05 * Es

Q  when P is severe

Y  when P >= 1 and RH <= 50%

Y  when e >= 1.01 * Es

where :

Es = saturated vapor pressure

= .6108 EXP(17.27 T / (T + 237.3))

RH = relative humidity = (e / Es) * 100

T = Air Temperature

P = Precipitation

6    Experimental #1          no further quality control

for this sensor

7    Experimental #2          no further quality control

for this sensor

8    Wind Speed (U)        S  when U < .447 or U > 60 m/s

R  see table A for site specific

requirements

R  when U <= .447 for 3 consecutive

hours and COS(Z) >= .34

Y  when U <= .447 for 2 consecutive

hours

Y  see Table A for site specific

requirements

CIMIS QUALITY CONTROL CRITERIA             5

Table A

-------

Site specific maximum hourly wind speed.

Flag      Flag      Station

'Y'       'R'        Group

--------   ------     -------

8 m/s   14 m/s        1

10       16            2

12       18            3

14       20            4

16       22            5

Group   Station #'s

-----   ---------------------------------

1  -  4, 33

2  -  5, 9, 13, 14, 16, 25, 31, 35, 42, 47,

61, 69, 70, 71

3  -  1, 11, 17, 22, 23, 24, 27, 28, 32,

34, 36, 39, 50, 53, 60, 62, 72

4  -  2, 3, 6, 7, 8, 10, 12, 18, 19, 20, 21, 30,

37, 38, 41, 43, 44, 45, 46, 48, 49, 51, 52,

55, 57, 58, 59, 63, 64, 65, 67, 68, 73, 74,

75

5  -  15, 26, 29, 40, 54

CIMIS QUALITY CONTROL CRITERIA             6

HOURLY DATA

Sensor

#        Name             Flag       Description

------     ----             ----       -----------

9    Resultant Wind (M)    Q  when U is severe

R  when M >= U

where :

U = Wind Speed

o

10   Wind Direction (D)    R  when D < 0 or D > 360

Q  when U is severe

I  when U <= .45

where :

U = Wind Speed

11   Standard Deviation

of Wind Direction (s) S  when s > 188 and s < 189

R  when s < 0 or s >= 180

12   Precipitation (P)     R  when P < 0 or P > 100 mm

or P is not a whole number

Q  when Rs is severe

R  when P > 0 and Rs > .75 * I

and COS(Z) >= .1735

Y  when P > 0 and Rs > .65 * I

and COS(Z) >= .1735

13   Soil Temperature                                 o

at 2 cm (t)           S  when t < -50 or t >= 100 C

R  when t < -15 or t > 50

Y  when t < -5 or t > 43

Y  when > 4 between this and

previous hour

14   Wind Gust (g)         R  when g > 50 m/s

Y  when g > 20

CIMIS QUALITY CONTROL CRITERIA HOURLY DATA

Sensor

#        Name             Flag       Description

------     ----             ----       -----------

15   Evapotranspiration

(ET)                 R  when Rn is severe

R  when T is severe

R  when e is severe

R  when U is severe

F  when daytime hourly ET value is

averaged because ET is not

available or sensors used

to calculate ET are flagged

severe

where :

Rn = Net Radiation

T = Air Temperature

e = Vapor Pressure

U = Wind Speed

note: up to 2 consecutive values will be averaged.  Nighttime

hourly ET value is set to 0 if the ET was not available

or the sensors used to calculate ET are flagged severe.

16   Relative Humidity

(RH)                 R  when T is severe

R  when e is severe

where :

T = Air Temperature

e = Vapor Pressure

25   Precipitation (P)     R  when P < 0, P >= 300 mm,

or P not a whole number

Y  when P not = total hourly

Precipitation

H  when one or more hourly

Precipitation is severe

26   Maximum Soil Temperature

at 2 cm (Tsx)         S  when Tsx = Tsn = Tsa or Tsx < Tsn

or Tsa < Tsn or Tsx < Tsa

o

S  when Tsx <= -50 or Tsx >= 100 C

R  when Tsx < -15 or Tsx > 50

Y  when Tsx < -5 or Tsx > 43

where :

Tsn = Minimum Soil Temperature at 2 cm

Tsa = Average Soil Temperature at 2 cm

27   Minimum Soil Temperature

at 2 cm (Tsn)         S  when Tsx = Tsn = Tsa or Tsx < Tsn

or Tsa < Tsn or Tsx < Tsa

o

S  when Tsn <= -50 or Tsn >= 100 C

R  when Tsn < -15 or Tsn > 50

Y  when Tsn < -5 or Tsn > 43

where :

Tsx = Maximum Soil Temperature at 2 cm

Tsa = Average Soil Temperature at 2 cm