|  | I. Project Monitoring |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PM10 | Base Sites (Calexico and Mexicali) | 24-Hour | Every Sixth Day between 03/18/92 to 08/29/92 | DRI Sequential Filter Sampler (SFS) with SA 254I PM10 Medium-Volume Inlet and Battery-Powered Portable PM10 Survey Sampler |
|  |  |  |  | 24-Hour | Daily between 12/21/92 and 01/07/93 | DRI Sequential Filter Sampler (SFS) with SA 254I PM10 Medium-Volume Inlet |
|  |  |  |  | 6-Hour | 4 Times/Day for the Periods: Summer (08/12/92 to 08/27/92), Winter (12/11/92 to 12/20/92), and Spring (05/13/93 to 05/19/93) | DRI Sequential Filter Sampler (SFS) with SA 254I PM10 Medium-Volume Inlet |
|  |  | PM10 | 20 to 30 Satellite Sitesa | 24-Hour | Daily for the Periods: Summer (08/12/92 to 08/27/92), Winter (12/11/92 to 12/20/92), and Spring (05/13/93 to 05/19/93) | Battery-Powered Portable PM10 Sampler |
|  |  | PM10 Mass | Base and Satellite Sites | 24-Hour and 6-Hour | All Periods | Gravimetric Analysis on Teflon-membrane Filters (Cahn 31 Electro- microbalance) |
|  |  | PM10 Particle Light Absorption | Base and Satellite Sites | 24-Hour and 6-Hour | All Periods | Light Transmission on Teflon-membrane Filters (Tobias TBX-10 Densitometer) |
|  | I. Project Monitoring (continued) |  |  |  |  |  |
|  |  | PM10 Elements | Base and Satellite Sites | 24-Hour and 6-Hour | All Periods | X-ray Fluorescence Analysis on Teflon-membrane Filters (Kevex 0700/8000 XRF Analyzer) |
|  |  | PM10 Water Soluble Chloride (Cl- ), Nitrate (NO3- ), and Sulfate (SO4=) ions | Base Sites (SFS Samples Only) | 24-Hour and 6-Hour | All Periods | Ion Chromatographic Analysis on Quartz-fiber Filter Extracts (Dionex 2020i Ion Chromatograph) |
|  |  | PM10 Water Soluble Sodium (Na+) and Potassium (K+) Ions | Base Sites (SFS Samples Only) | 24-Hour and 6-Hour | All Periods | Atomic Absorption Spectrophotometry on Quartz-fiber Filter Extracts (Perkin-Elmer Model 2380 System) |
|  |  | PM10 Water Soluble Ammonium (NH4+) | Base Sites (SFS Samples Only) | 24-Hour and 6-Hour | All Periods | Technicon Automatic Colorimetry on Quartz-fiber Filter Extracts (TRAACS 800 System) |
|  |  | PM10 Organic and Elemental Carbon (OC, EC) | Base Sites (SFS Samples Only) | 24-Hour and 6-Hour | All Periods | Thermal/Optical Reflectance Carbon Analysis on Pre-fired Quartz-fiber Filters (DRI/OGC Thermal/Optical Reflectance Carbon Analyzer) |
| II. Supplemental PM10 Monitoring |  |  |  |  |  |  |
|  |  | Total Suspended Particulate (TSP) | Laidlaw Environmental Services | 24-Hour | Daily between 01/06/91 and 06/29/91 | Sierra-Andersen High-volume sampler |
| II. Supplemental PM10 Monitoring (continued) |  |  |  |  |  |  |
|  |  | TSP Mass | Laidlaw Environmental Services | 24-Hour | Daily between 01/06/91 and 06/29/91 | Gravimetric Analysis on Teflon-membrane filters |
|  |  | PM10 | Brawley, El Centro, Calexico Police and Fire Station, and Calexico | 24-Hour | Daily between 01/01/92 and 08/29/93 | General Metal Works IP10-70 High-volume Sampler with General Metal Works G-1200 Size selective Inlet |
|  |  | PM10 Mass | Brawley, El Centro, Calexico Police and Fire Station, and Calexico | 24-Hour | Daily between 01/01/92 and 08/29/93 | Gravimetric Analysis on quartz-fiber filters |
|  |  | Fine (PM2.5), coarse (PM2.5 -PM10), and PM10 | El Centro and Calexico | 24-Hour | Daily between 01/01/92 and 08/29/93 | Sierra-Andersen SA241 Dichotomous Sampler with Model 3000 Virtual Impactor |
|  |  | Fine, coarse, and PM10 Mass | El Centro and Calexico | 24-Hour | Daily between 01/01/92 and 08/29/93 | Gravimetric Analysis on Teflon-membrane filters |
|  |  | Fine, coarse, and PM10 Elements | El Centro and Calexico | 24-Hour | Daily between 01/01/92 and 08/29/93 | ARB X-ray Fluorescence Analysis on Teflon-membrane filters (Kevex 0700/8000 XRF Analyzer) |
|  |  | PM10 Mass | Calexico | Hourly | Daily between 03/07/92 to 08/29/93 | Beta Attenuation Monitor (Sierra-Andersen FH62I-N) |

a  See Table 2-1a for site identification.