Validation 	Sub

Flag   	Flag	Description

a			(reserved)

b			blanks.

b0	unspecified field blank.

b1	field/dynamic blank.

b2	laboratory control blank.

b3	distilled-deionized water blank.

b4	method blank.

b5	extract/solution blank.

b6	transport blank.

c			sample spectrum being reprocessed or recalculated.

c1	carbon run recalculated.

c2	ion run reprocessed due to integration failure.

c3	ion run recalculated with high concentration range calibration curve.

c4	average of several replicate runs.

c5	ion analyses were performed with high concentration range calibration curve.

c6	XRF spectrum reprocessed using manually adjusted background.

d			data point deleted from main data base.

e			use of IC filter in ion analyses.

e1	extract analyzed with IC filter.

e2	extract analyzed without IC filter.

f			filter damaged.

f1	filter damaged, outside of analysis area.

f2	filter damaged, within analysis area.

f3	Teflon membrane substrate separated from the ring.

f4	filter deposit side facing down in Petri slides.

f5	filter dropped during handling.

f6	filter stuck to Petri slide.

g			(reserved)

h			(reserved)

i			abnormal sample deposit.

i1	inhomogeneous sample deposit.

i2	deposit smeared or scraped after sampling.

i3	deposit appears to have fallen off.

i4	foreign particles on filter

i5	non-white carbon punch after carbon analysis.

i6	larger particles on filter than size cut device allows.

i7	particles on filter holder.

i8	visible deposition on the back of the filter.

i9	deposit trapped between layers of filters; layers separated prior to chemical analysis.

j			abnormal deposit area, possible leakage during sampling.

k			(reserved)

l			analysis value lower than minimal detection limit.

m			analysis results affected by matrix effect.

m1	organic/elemental carbon split is undeterminable due to an apparent sample matrix effect; all measured carbon is arbitrarily reported as organic.

n			various dilution factors used in ion analyses.

n0	1:10 dilution was made on sample extract.

n1	1:20 dilution was made on sample extract.

n2	1:2 dilution was made on sample extract.

n3	1:3 dilution was made on sample extract.

n4	1:4 dilution was made on sample extract.

n5	1:5 dilution was made on sample extract.

n6	1:100 dilution was made on sample extract.

n7	1:50 dilution was made on sample extract.

n8	1:110 dilution was made on sample extract.

n9	1:21 dilution was made on sample extract.

n10	1:11 dilution was made on sample extract.

o			(reserved)

p			(reserved)

q			quality control standards.

q1	routine quality control standard.

q2	Environmental Resources Associates (ERA) quality control standard.

q3	(NBS) simulated rainwater standard.

q4	calibration standard.

r			replicate analysis

r1	first replicate analysis on the same analyer.

r2	second replicate analysis on the same analyzer.

r3	third replicate analysis on the same analyzer.

r4	sample rerun, replaced with new value.

r5	replicate on different analyzer.

r6	re-extraction was made for sample rerun.

r7	Sample rerun, no improvement, used old value.

r8	Sample rerun, no change, replaced with the averaged value.

s			suspect of sample validity.

t			(reserved)

u			(reserved)

v			void sample.

v1	sample void due to quality control standard check exceeded ± 10% of specified concentration range.

v2	sample void due to replicate analysis exceeded ± 10% of specified concentration range.

v3	sample void due to potential contamination.

v4	fine concentration statistically greater than the total.

v5	concentration out of expected range.

w

w1	Pre-weight is questionable.

w2	Post-weight is questionable.

w3	Reweighed at Level I validation without corrections (within ± 15 mg/filter).

w4	Reweighed at Level I validation with corrections (exceeded ± 15 mg/filter).

w5	Water spots on deposit.

x			(reserved)

y			(reserved)

z			(reserved)