# Product Name:

# 1m Raster Files

Overview:

Geo-TIFF File Format

Based On Hydro And Last Return Topo data

Datum

Horizontal – NAD83

Decimal Degrees

Vertical – NAVD88

Meters

~1m Pixel Resolution

Pixel Value Is Average Elevation

5km of Shoreline per File

Metadata Are Included

General Description:

This data set contains Digital Elevation Model (DEM) grids created from the ASCII Hydrographic and Topographic Last Return XYZ product files for each data tile and provided in a geo-rectified TIFF format.  The coordinates shall be in NAD83 geographic coordinates with elevations in meters and referred to NAVD88.  A triangulated irregular network (TIN) surface shall be created from using the hydrographic and topographic data for each tile.  Elevations shall be interpolated from the TIN at 1m meter intervals.    No elevations are to be interpolated from triangles having sides longer than 2.5 x hydrographic data spot density.  This logic is intended to avoid the generation of elevation data in areas where insufficient data points exist to support the interpolation.  The resulting elevation shall be provided with precision to two decimal places, or centimeter level.  Cells containing no data are assigned an elevation value of -9999.  Each DEM file shall cover approximately 5 km of shoreline having the same limits as the point data tile boxes.