***************************************************************************

Mission Statement:  Determine what can be accomplished to provide the most accurate and timely information to flightcrews and operators.

Step 1 – Define Information Required:

Runway Condition Reports (airport operators) – FAA Airports for action

- Time of report

- Type of contaminant for each 1/3 of the runway

  - Characterize type:

    - snow, wet or dry

    - ice, patchy

    - water, standing

    - slush

- Depth of contaminant for each 1/3 of the runway

- Time of last runway treatment (anti / deice treatments, plowing, brooming, etc)

- Length and width treated

- Anticipated time of next treatment

- Runway Mu for each 1/3 of the runway

Braking Action Reports (pilots)

- Time stamp

- Type & model of aircraft making report

  - Weight / loading of aircraft(?)

  - Carrier(?)

  - Where exit runway(?)

- Affected runway

- Braking Action Reports (using newly defined terminology)

Step 2 - How to achieve timely provision of above information:

Short Term:

- Issue a SAFO

  - Provide listing of information required above

  - Harlan agreed to compile SAFO information from WG members

- Include abbreviated information set on ATIS

  - Runway Mu on each 1/3 of the runway

  - Latest Braking Action Report (using newly defined terminology)

  - Runway contaminant type & depth on each 1/3 of the runway

- Trigger ATIS “Braking Action Advisories in Effect” at a report of less than GOOD braking action – FAA Air Traffic for action

- Flightcrews to provide more Braking Action PIREPS

- Dispatchers and Air traffic to solicit and disseminate more Braking Action PIREPS through ADDS or NAIMES

- Amend the NOTAM system

  - More responsive

  - L-NOTAM available

- Amend the AIM to include newly defined Braking Action Report terminology

Longer Term:

- Information uploadable to select NWS / NOAA websites for immediate and universal access

- Standardized electronic formatting of information

  - Standard reporting of information

  - Rapid capturing of information

  - Easy access to captured information

- Review and amend the QICP (Quality Internet Communication Protocol) approval process

  - Advisory Circular 00-62

  - Review approval criteria

  - Accelerate approval process