| U.S. Department of Transportation Federal Aviation Administration | Advisory Circular |
| --- | --- |

| Subject: Change 11 to AIRPORT DESIGN | Date: 3/28/2007 Initiated by: AAS-100 | AC No: 150/5300-13 Change: 11 |
| --- | --- | --- |

- PURPOSE.  This Change clarifies the standard for the Runway Protection Zone (RPZ).  Recently, there has been some confusion about the permissibility of vehicle parking in RPZs.  The FAA adopted a prohibition against parking in the extended Runway Object Free Area (ROFA) in 1989.  While the intent of the RPZ standard has always been to expand this prohibition to the central portion of the RPZ along its entire length, this intent was not entirely clear in the previous standard.

This Change also does the following—

  - Updates the Table of Contents.

  - Updates Figure 2-3.

  - Makes an editorial correction to Table 3-3.

2.	CHANGED TEXT.  Changed text is indicated by vertical bars in the margins.

PAGE CONTROL CHART

| Remove Pages | Dated | Insert Pages | Dated |
| --- | --- | --- | --- |
| ii-v vi-ix x-xii 13-14 19-20 26-1 26-2 | 2/14/97 9/30/00 2/14/97 9/29/06 11/10/94 9/29/06 9/29/06 | ii-xii 13 14 19 20 26-1 26-2 | 3/28/07 3/28/07 9/29/06 11/10/94 3/28/07 3/28/07 9/29/06 |

David L. Bennett

Director of Airport Safety and Standards

This page intentionally left blank.

# CONTENTS

Paragraph	Page

Chapter 1.  REGULATORY REQUIREMENTS AND DEFINITION OF TERMS

1.	GENERAL	  1

2.	DEFINITIONS	  1

3.	RELATED/REFERENCED READING MATERIAL	  3

4.	AIRPORT REFERENCE CODE (ARC)	  5

5.	AIRPORT LAYOUT PLAN	  5

6.	MODIFICATION OF AIRPORT DESIGN STANDARDS TO MEET LOCAL CONDITIONS	  5

7.	NOTICE TO THE FAA OF AIRPORT DEVELOPMENT	  5

8.	NOTICE TO THE FAA OF PROPOSED CONSTRUCTION	  6

9.	FAA STUDIES	  6

10.	FEDERAL ASSISTANCE	  6

11.	ENVIRONMENTAL ASSESSMENTS	  6

12.	STATE ROLE	  6

13.	LOCAL ROLE	  6

14. to 199.  RESERVED	  6

### Chapter 2.  AIRPORT GEOMETRY

200.	INTRODUCTION	  9

201.	PRINCIPLES OF APPLICATION	  9

202.	RUNWAY LOCATION AND ORIENTATION	  9

203.	ADDITIONAL RUNWAYS	 10

204.	TAXIWAY SYSTEM	 10

205.	AIRPORT APRONS	 10

206.	SEPARATION STANDARDS	 10

207.	PARALLEL RUNWAY SEPARATION--SIMULTANEOUS VFR OPERATIONS	 11

208.	PARALLEL RUNWAY SEPARATION--SIMULTANEOUS IFR OPERATIONS	 11

209.	RUNWAY TO PARALLEL TAXIWAY AND TAXILANE SEPARATION	 12

210.	BUILDING RESTRICTION LINE (BRL)	 12

211.	OBJECT CLEARING CRITERIA	 12

212.	RUNWAY PROTECTION ZONE (RPZ)	 13

213. to 299.  RESERVED	 13

### Chapter 3.  RUNWAY DESIGN

300.	INTRODUCTION	 21

301.	RUNWAY LENGTH	 21

302.	RUNWAY WIDTH	 21

303.	RUNWAY SHOULDERS	 21

304.	RUNWAY BLAST PAD	 21

305.	RUNWAY SAFETY AREA (RSA)	 21

306.	OBSTACLE FREE ZONE (OFZ)	 22

307.	RUNWAY OBJECT FREE AREA	 23

308.	CLEARWAY STANDARDS	 23

309.	STOPWAY STANDARDS	 23

310.	RESCUE AND FIREFIGHTING ACCESS	 24

311. to 399.  RESERVED	 24

### Chapter 4.  TAXIWAY AND TAXILANE DESIGN

400.	INTRODUCTION	 33

401.	DIMENSIONAL STANDARDS	 33

402.	TAXIWAY SHOULDERS	 33

403.	TAXIWAY SAFETY AREA (TSA)	 33

404.	TAXIWAY AND TAXILANE OBJECT FREE AREA (OFA)	 33

405.	PARALLEL TAXIWAY	 33

406.	TAXIWAY INTERSECTIONS	 34

407.	ENTRANCE TAXIWAYS	 34

408.	BYPASS TAXIWAYS	 34

409.	HOLDING BAYS	 34

410.	TURNAROUNDS	 34

411.	DUAL PARALLEL TAXIWAYS	 34

412.	TAXIWAY BETWEEN PARALLEL RUNWAYS	 35

413.	EXIT TAXIWAYS	 35

414.	APRON TAXIWAYS AND TAXILANES	 35

415.	END-AROUND TAXIWAYS	35

416. to 499.  RESERVED	 38

### Chapter 5.  SURFACE GRADIENT AND LINE OF SIGHT

500.	INTRODUCTION	 49

501.	BACKGROUND	 49

502.	SURFACE GRADIENT STANDARDS	 49

503.	LINE OF SIGHT STANDARDS	 56

504. to 599.  RESERVED	 56

### Chapter 6.  SITE REQUIREMENTS FOR NAVAID AND ATC FACILITIES

600.	GENERAL	 59

601.	MICROWAVE LANDING SYSTEM	 59

602.	INSTRUMENT LANDING SYSTEM	 61

603.	NONDIRECTIONAL BEACON	 63

604.	VERY HIGH FREQUENCY OMNIRANGE	 64

605.	APPROACH LIGHTING SYSTEMS	 64

606.	OMNIDIRECTIONAL APPROACH LIGHTING SYSTEMS	 65

607.	LEAD-IN LIGHTING SYSTEMS	 65

608.	AIRPORT ROTATING BEACONS	 65

609.	AIRPORT TRAFFIC CONTROL TOWERS	 65

610.	AIRPORT SURVEILLANCE RADAR	 66

611.	AIRPORT SURFACE DETECTION EQUIPMENT	 66

612.	RUNWAY VISUAL RANGE FACILITIES	 66

613.	AUTOMATIC WEATHER OBSERVATION STATIONS (AWOS)	 66

614.	PHYSICAL SECURITY	 67

615.	CABLE PROTECTION	 67

616. to 699.  RESERVED	 67

### Chapter 7.  RUNWAY AND TAXIWAY BRIDGES

700.	INTRODUCTION	 69

701.	SITING PRECEPTS	 69

702.	DIMENSIONS	 69

703.	LOAD CONSIDERATIONS	 69

704.	DECK DESIGN	 69

705.	MARKING AND LIGHTING	 69

706.	OTHER CONSIDERATIONS	 69

707.	PASSENGER AND BAGGAGE TUNNELS	 70

708. to 799.  RESERVED	 70

# Chapter 8.  THE EFFECTS AND TREATMENT OF JET BLAST

800.	INTRODUCTION	 77

801.	JET BLAST EFFECTS	 77

802.	BLAST FENCES	 77

803.	SHOULDERS AND BLAST PADS	 78

Appendix 1.  WIND ANALYSIS

1.	OBJECTIVE	 87

2.	CROSSWINDS	 87

3.	COVERAGE AND ORIENTATION OF RUNWAYS	 87

4.	ASSEMBLING WIND DATA	 87

5.	ANALYZING WIND DATA	 88

6.	CONCLUSIONS	 88

7.	ASSUMPTIONS	 88

8.	COMPUTER WIND ANALYSIS	 88

### Appendix 2.  RUNWAY END SITING REQUIREMENTS

1.	PURPOSE	100

2.	APPLICATION	100

3.	LIMITATIONS	100

4.	EVALUATION CONSIDERATIONS	100

5.	CLEARANCE REQUIREMENTS	101

### Appendix 3.  AIRPORT REFERENCE POINT

1.	DISCUSSION	109

2.	SAMPLE COMPUTATION	109

3.	ACCURACY	109

#### Appendix 4.  COMPASS CALIBRATION PAD

1.	PURPOSE	111

2.	BACKGROUND	111

3.	APPLICATION	111

4.	DESIGN OF COMPASS CALIBRATION PAD	111

5.	LOCATION OF COMPASS CALIBRATION PAD	112

6.	CONSTRUCTION OF COMPASS CALIBRATION PAD	112

7.	VOR CHECKPOINT	113

Appendix 5.  SMALL AIRPORT BUILDINGS, AIRPLANE PARKING, AND TIEDOWNS

1.	GENERAL	117

2.	TRANSIENT APRON	117

3.	APRON FOR BASED AIRPLANES	117

4.	TIEDOWNS	118

5.	OTHER CONSIDERATIONS	118

6.	HANGARS	118

7.	ADMINISTRATION BUILDING	118

8.	AIRPORT SURVEY	122

9.	BUILDING PLAN	122

10.	EXPANSION	122

11.	CIRCULATION	122

12.	WAITING ROOM	122

13.	MANAGER'S OFFICE	123

14.	EATING FACILITIES	123

15.	PUBLIC RESTROOMS	123

16.	ROADS AND AUTO PARKING	123

### Appendix 6.  METRIC CONVERSION AND TYPICAL AIRPORT LAYOUT PLAN

Appendix cancelled (pp. 125-130)

### Appendix 7.  AIRPORT LAYOUT PLAN COMPONENTS AND PREPARATION

Appendix cancelled (pp. 131-138)

### Appendix 8.  RUNWAY DESIGN RATIONALE

1.	SEPARATIONS	139

2.	OBSTACLE FREE ZONE (OFZ)	139

3.	RUNWAY SAFETY AREA	139

4.	RUNWAY OBJECT FREE AREA (ROFA)	139

5.	RUNWAY SHOULDERS AND BLAST PADS	140

6.	CLEARWAY	140

7.	STOPWAY	140

8.	RUNWAY PROTECTION ZONE (RPZ)	140

### Appendix 9.  TAXIWAY AND TAXILANE DESIGN RATIONALE

1.	INTRODUCTION	141

2.	BACKGROUND AND RATIONALE	141

3.	EXIT TAXIWAY LOCATION	142

4.	WINGTIP TRACE	146

### Appendix 10.  TAXIWAY FILLET DESIGN

1.	INTRODUCTION	149

2.	EXAMPLE NO. 1, JUDGMENTAL OVERSTEERING	150

3.	EXAMPLE NO. 2, MAINTAINING COCKPIT OVER CENTERLINE	150

#### Appendix 11.  COMPUTER PROGRAM

1.	AIRPORT DESIGN (FOR MICROCOMPUTERS) VERSION 4.2	153

2.	HOW TO OBTAIN A COPY OF AIRPORT DESIGN (FOR MICROCOMPUTERS) VERSION 4.2	153

3.	REQUIREMENTS	153

4.	SETUP ON A MICROCOMPUTER	153

5.	RUN AIRPORT DESIGN PROGRAM	153

6.	HOT KEYS	153

7.	RUNWAY AND TAXIWAY WIDTH AND CLEARANCE STANDARD DIMENSIONS	154

8.	RECOMMENDED RUNWAY LENGTHS	154

9.	STANDARD WIND ANALYSIS	154

10.	TAXIWAY DESIGN	155

11.	AIRPORT CAPACITY AND DELAY FOR LONG RANGE PLANNING	156

12.	DECLARED DISTANCE LENGTHS	156

13.	INPUT AIRPLANE DATA AVAILABILITY	156

### Appendix 12.  AIRPLANE DATA

1.	BACKGROUND	165

2.	EXPLANATORY INFORMATION	166

Appendix 13.  AIRPLANES ARRANGED BY AIRPLANEMANUFACTURER AND

AIRPORT REFERENCE CODE

Section 1.  Alphabetical Listing (U.S. customary units)	251

Section 2.  Alphabetical Listing (SI units)	257

Section 3.  Listing Small Airplanes by Airport Reference Code (U.S. customary units)	263

Section 4.  Listing Large Airplanes by Airport Reference Code (U.S. customary units)	264

Section 5.  Listing Small Airplanes by Airport Reference Code (SI units)	269

Section 6.  Listing Large Airplanes by Airport Reference Code (SI units)	270

#### Appendix 14.  DECLARED DISTANCES

1.	APPLICATION	275

2.	BACKGROUND	275

3.	FAA APPROVAL FOR APPLYING DECLARED DISTANCES IN AIRPORT DESIGN	275

4.	RUNWAY SAFETY AREA (RSA) AND RUNWAY OBJECT FREE AREA (ROFA) LENGTHS	276

5.	RUNWAY PROTECTION ZONE (RPZ) LOCATION AND SIZE	276

6.	CLEARWAY LOCATION	276

7.	NOTIFICATION	276

### Appendix 15. TRANSFER OF ELECTRONIC DATA

1.	INTRODUCTION	283

2.	BACKGROUND	283

3.	DEFINITIONS	283

4.	APPLICATION	283

5.	CADD FILE DELIVERABLES	284

6.	DATABASES DELIVERABLES	286

7.	PHOTOGRAMMETRY DELIVERABLES	286

8.	FEATURES AND OBJECTS CODE	287

9.	MEDIA	289

10.	FAA POINT OF CONTACT	289

## Appendix 16.  NEW INSTRUMENT APPROACH PROCEDURES

1.	BACKGROUND	291

2.	INTRODUCTION	291

3.	ACTION	291

4.	DEFINITIONS	291

Appendix 17.  MINIMUM DISTANCES BETWEEN CERTAIN AIRPORT FEATURES AND

ANY ON-AIRPORT AGRICULTURE CROPS (1 page).

Appendix 18.  ACRONYMS (1 page).

Appendix 19.  INDEX (4 pages

).

Table	Page

1-1.	Increases in airport design standards associated with an upgrade in the first component (aircraft approach category) of the airport reference code	  7

1-2.	Increases in airport design standards to provide for lower approach visibility minimums	  8

2-1.	Runway separation standards for aircraft approach categories A & B	 14

2-2.	Runway separation standards for aircraft approach categories C & D	 15

2-3.	Taxiway and taxilane separation standards	 16

2-4.	Runway protection zone (RPZ) dimensions	 19

3-1.	Runway design standards for aircraft approach category A & B visual runways and runways with not lower than 3/4-statute mile (1 200 m) approach visibility minimums	25

3-2.	Runway design standards for aircraft approach categories A & B runways with lower than 3/4-statute mile (1 200 m) approach visibility minimums	 26

3-3.	Runway design standards for aircraft approach categories C & D	 26-1

4-1.	Taxiway dimensional standards	 38

4-2.	Taxiway fillet dimensions	 40

4-3.	Wingtip clearance standards	 40

4-4.	Visual screen height calculation formula (same elevation as runway)	48-5

4-5.	Visual screen height calculation formula (EAT below DER elevation) for Design Group III	48-6

4-6.	Visual screen height calculation formula (EAT below DER elevation) for Design Group IV	48-7

4-7.	Visual screen height calculation formula (EAT below DER elevation) for Design Groups V and VI	48-8

4-8.	Visual screen vertical height calculation tables	48-9

4-9.	Visual screen panel wind-loading deflection allowance	48-12

4-10.	CIE chromaticity coordinate limits	48-12

4-11.	Minimum reflection levels	48-13

A2-1.	Approach/Departure Requirements Table	103

A9-1.	Exit taxiway cumulative utilization percentages	142

A16-1A.	Precision instrument approach requirements	292

A16-1B	Approach procedure with vertical guidance (APV-RNP) approach requirements	293

A16-1C	Nonprecision approach requirements	294

A16-2	Survey requirements for instrument approach procedures	295

A17-1.	Minimum Distances Between Certain Airport Features and Any On-Airport Agriculture Crops	296

##### Figure		Page

2-1.	Typical airport layout	 17

2-2.	Parallel runway separation	 18

2-3.	Runway protection zone	 20

3-1.	Runway safety area	 27

3-2.	Obstacle free zone (OFZ) for visual runways and runways with not lower than 3/4 statute mile (1 200 m) approach visibility minimums	28

3-3.	Obstacle free zone (OFZ) for runways serving small airplanes exclusively with lower than 3/4-statute mile (1 200 m) approach visibility minimums	 29

3-4.	Obstacle free zone (OFZ) for runways serving large airplanes with lower than 3/4-statute mile (1 200 m) approach visibility minimums	 30

3-5.	Obstacle free zone (OFZ) for runways serving large airplanes with lower than 3/4-statute mile (1 200 m) approach visibility minimums and displaced threshold	 31

3-6.	Precision object free zone	32

3-7.	Clearway	 32-1

3-8.	Stopway	 32-2

4-1.	Taxiway intersection details	 39

4-2.	Maintaining cockpit over centerline	 41

4-3.	Judgmental oversteering	 42

4-4.	Example of pavement fillet computer program printout	 43

4-5.	Entrance taxiway	 44

4-6.	Bypass taxiway	 44

4-7.	Dual parallel taxiway entrance	 45

4-8.	Typical holding bay configurations	 46

4-9.	Taxiway turnaround	 46

4-10.	Crossover taxiway	 47

4-11.	Right-angled exit taxiway	 47

4-12.	Acute-angled exit taxiway	 48

4-13.	Example of acute-angled exit taxiway computer layout data page 1	 48-1

4-14.	Example of acute-angled exit taxiway computer layout data page 2	 48-2

4-15.	Typical end-around taxiway layout	48-3

4-16.	End-around taxiway visual screen width calculations	48-4

4-17.	Visual screen width calculation formula	48-5

4-18.	Examples of mounting screen to vertical column	48-10

4-19.	Examples of panel layout for 13-foot-high screen	48-11

4-20.	Diagonal stripe orientation	48-12

4-21.	Examples of frangibility connections	48-14

5-1.	Longitudinal grade limitations for aircraft approach categories A & B	 50

5-2.	Transverse grade limitations for aircraft approach categories A & B	 51

5-3.	Longitudinal grade limitations for aircraft approach categories C & D	 52

5-4.	Transverse grade limitations for aircraft approach categories C & D	 53

5-5.	Runway safety area grade limitations beyond 200 feet (60 m) from the runway end	 55

5-6.	Runway visibility zone	 57

6-1.	AZ antenna siting	 59

6-2.	Typical NAVAID placement	 60

6-3.	AZ antenna critical area	 61

6-4.	EL antenna siting	 61

6-5.	EL antenna critical area	 61

6-6.	ILS LOC siting and critical area	 62

6-7.	GS siting and critical area	 62

6-8.	Marker beacon site	 63

6-9.	NDB site	 63

6-10.	A TVOR installation	 64

7-1.	Full width runway-taxiway bridge	 71

7-2.	Cross-section full width runway-taxiway bridge	 72

7-3.	Minimum width taxiway bridge with positive edge protection,  O'Hare Airport, Chicago, IL	 73

7-4.	Example structural deck and depressed roadway, O'Hare Airport, Chicago, IL	 74

7-5.	Suggested shoulder marking of minimum width taxiway bridge	 75

7-6.	Controlled use service road, Los Angeles International Airport,  Los Angeles, CA	 76

8-1.	Velocity distance curves, DC-8	 79

8-2.	Velocity distance curves, B-727	 80

8-3.	Velocity distance curves, B-747	 81

8-4.	Velocity distance curves, DC-10	 82

8-5.	Blast velocities of business jet airplanes	 83

8-6.	Typical blast deflector fences, metal	 84

8-7.	Typical blast deflector fences, concrete	 85

A1-1.	Wind vector diagram	 89

A1-2.	Typical environmental data service wind summary	 90

A1-3.	Windrose blank showing direction and divisions	 91

A1-4.	Completed windrose using figure A1-2 data	 92

A1-5.	Windrose analysis	 93

A1-6.	Windrose analysis--estimating area not included	 94

A1-7.	Computer printout page 1	 95

A1-8.	Computer printout page 2	 96

A1-9.	Computer printout page 3	 97

A1-10.	Lotus cell-formulas page 1	 98

A1-11.	Lotus cell-formulas page 2	 99

A2-1.	Approach slopes	105

A2-2.	Approach slopes—with offset approach course	106

A2-3.	Departure surface for Instrument Runways TERPS (40:1)	107

A2-4.	One-Engine Inoperative (OEI) Obstacle Identification Surface (62.5:1)	108

A3-1.	Sample layout	109

A3-2.	Sample computation - airport reference point	110

A4-1.	Marking layout and details of wheel block	114

A4-2.	Type I. compass calibration pad	115

A4-3.	Type II. compass calibration pad	116

A5-1.	Parking apron area	119

A5-2.	Tiedown layouts	120

A5-3.	T-hanger layout	121

A8-1.	Approximate distance airplanes undershoot and overrun the runway end	140

A9-1.	Wingtip clearance - parallel taxiways	143

A9-2.	Wingtip clearance from taxiway	144

A9-3.	Wingtip clearance from apron taxiway	144

A9-4.	Wingtip clearance from taxilane	145

A9-5.	Pavement edge clearance on tangent	146

A9-6.	McDonnell-Douglas MD-88 wingtip clearance trace for a 100-foot (30.5 m) radius centerline	147

A9-7.	McDonnell-Douglas MD-88 wingtip clearance trace for a 120-foot (36.5 m) radius offset centerline	147

A9-8.	Boeing 727-200 wingtip clearance trace for a 120-foot (36.5 m) radius offset centerline	148

A9-9.	Boeing 727-100 wingtip clearance trace for a 120-foot (36.5 m) radius offset centerline	148

A10-1.	Taxiway intersection details	151

A10-2.	Depiction of symbols	152

A11-1.	THIS FIGURE INTENTIONALLY LEFT BLANK	156

A11-2.	Estimated airplane data elements for input in the computer program	157

A11-3.	Example of the airport design airplane and airport data window	158

A11-4.	Example printout of width and clearance standard dimensions page 1	158

A11-5.	Example printout of width and clearance standard dimensions page 2	159

A11-6.	Example printout of wind analysis (two bi-directional runways)	160

A11-7.	Example printout of windrose (two bi-directional runways)	161

A11-8.	Example printout of wind analysis (one uni-directional runway)	162

A11-9.	Example printout of windrose (one uni-directional runway)	163

A11-10.	Nomenclature used in the taxiway design task	164

A11-11.	Nomenclature used in the declared distance task	164

A12-1.	Single engine, high wing, tailwheel airplanes 8,000 lb. (3,628 Kg) or less	167

A12-2.	Single engine, high wing, tailwheel airplanes 8,000 lb. (3,628 Kg) or less (cont'd)	168

A12-3.	Single engine, high wing, tricycle gear airplanes 8,000 lb. (3,628 Kg) or less	169

A12-4.	Single engine, low wing, tricycle gear airplanes 8,000 lb. (3,628 Kg) or less	170

A12-5.	Single engine, low wing, tricycle gear airplanes 8,000 lb. (3,628 Kg) or less (cont'd)	171

A12-6.	Twin engine, low or mid wing, tricycle gear airplanes 8,000 lb. (3,628 Kg) or less	172

A12-7.	Twin engine, low or mid wing, tricycle gear airplanes 8,000 lb. (3,628 Kg) or less (cont'd)	173

A12-8.	Twin engine, high or mid wing, tricycle gear airplanes 8,000 lb. (3,628 Kg) or less	174

A12-9.	Aérospatiale Nord 262	175

A12-10.	Aérospatiale/Sud SE-210 Caravelle	176

A12-11.	Airbus Industries A300, 310, and 320	177

A12-12.	Avions de Transport Regional ATR-42 & -72	178

A12-13.	Avions Marcel Dassault Mystère 20 (Fan Jet Falcon)	179

A12-14.	BAe 1-11	180

A12-15.	B.A.C./SNIAS Concorde	181

A12-16.	B.A.C./Vickers VC-10	182

A12-17.	B.A.C./Vickers Viscount	183

A12-18.	Beech Starship	184

A12-19.	Beechcraft Airliner	185

A12-20.	Beechcraft King Air	186

A12-21.	Beechcraft Model 18 and Conversions	187

A12-22.	Beechcraft Queen Air	188

A12-23.	Boeing B-52 Stratofortress	189

A12-24.	Boeing KC-97L	190

A12-25.	Boeing KC-135A	191

A12-26.	Boeing 707-720	192

A12-27.	Boeing 727	193

A12-28.	Boeing 737	194

A12-29.	Boeing 747	195

A12-30.	Boeing 757	196

A12-31.	Boeing 767	197

A12-32.	British Aerospace 146	198

A12-33.	Canadiar CL-44	199

A12-34.	Canadiar CL-66	200

A12-35.	Cessna Citation	201

A12-36.	Construcciones Aeronauticas CASA C-212 and 235	202

A12-37.	Convair-liner and Turboprop Conversions	203

A12-38.	De Havilland Canada C-7 Caribou	204

A12-39.	De Havilland Canada DASH 7 & DASH 8	205

A12-40.	Douglas C-124 Globemaster	206

A12-41.	De Havilland Canada DHC-6 Twin Otter	207

A12-42.	Dornier Gmb H	208

A12-43.	Douglas DC-3	209

A12-44.	Douglas DC-4/6/7	210

A12-45.	Embraer EmB 110	211

A12-46.	Embraer EmB 120	212

A12-47.	Fairchild C-119K Flying Boxcar	213

A12-48.	Fairchild C-123K Provider	214

A12-49.	Fairchild F-27	215

A12-50.	Fokker F-27	216

A12-51.	Fokker F-28	217

A12-52.	Gates Learjet	218

A12-53.	General Dynamics/Convair 880/990	219

A12-54.	Grumman Gulfstream I	220

A12-55.	Grumman Gulfstream II	221

A12-56.	Grumman G-64/G-III	222

A12-57.	Grumman G-73	223

A12-58.	Hamburger-Flugzeubau HFB-320 Hansa	224

A12-59.	Hawker Siddeley DH. 104 Dove	225

A12-60.	Hawker Siddeley DH. 114 Heron	226

A12-61.	Hawker Siddeley HS-125	227

A12-62.	Hawker Siddeley HS-748	228

A12-63.	Ilyushin IL-62	229

A12-64.	Israel Aircraft Industries Westwind	230

A12-65.	Lockheed Constellation and Super Constellation	231

A12-66.	Lockheed C-5B Galaxy	232

A12-67.	Lockheed C-141 Starlifter	233

A12-68.	Lockheed L-188 Electra II	234

A12-69.	Lockheed L-100 Hercules	235

A12-70.	Lockheed L-1011 Tristar	236

A12-71.	Lockheed L-1329 Jetstar	237

A12-72.	Martin 404	238

A12-73.	McDonnell-Douglas DC-8	239

A12-74.	McDonnell-Douglas DC-9 and MD-80	240

A12-75.	McDonnell-Douglas DC-10	241

A12-76.	McDonnell-Douglas MD-11	242

A12-77.	Mitsubishi MU-2	243

A12-78.	Nihon/N.A.M.C. YS-11A	244

A12-79.	Rockwell International NA-265 Sabreliner	245

A12-80.	SAAB SF 340	246

A12-81.	Short Brothers	247

A12-82.	Shorts SC. 5/10 Belfast	248

A12-83.	Swearingen Merlin	249

A12-84.	Swearingen Metro	250

A14-1.	Takeoff run available (TORA)	277

A14-2.	Takeoff distance available (TODA)	278

A14-3.	Accelerate-stop distance available (ASDA)	279

A14-4.	Landing distance available (LDA)	280

A14-5.	Example of a runway extended to 7000 feet	281

A14-6.	Example of a runway with threshold displaced for runway safety area	282

b.	Recommendations.  Other objects that are desirable to clear, if practicable, are objects that do not have a substantial adverse effect on the airport but, if removed, will enhance operations.  These include objects in the controlled activity area and obstructions to air navigation that are not covered in paragraph 211.a, especially those penetrating an approach surface.  On a paved runway, the approach surface starts 200 feet (61 m) beyond the area usable for takeoff or landing, whichever is more demanding.  On an unpaved runway, the approach surface starts at the end of the area usable for takeoff or landing.

212.  RUNWAY PROTECTION ZONE (RPZ).  The RPZ's function is to enhance the protection of people and property on the ground.  This is achieved through airport owner control over RPZs.  Such control includes clearing RPZ areas (and maintaining them clear) of incompatible objects and activities.  Control is preferably exercised through the acquisition of sufficient property interest in the RPZ.

a.	Standards.

(1)	RPZ Configuration/Location.  The RPZ is trapezoidal in shape and centered about the extended runway centerline. The central portion and controlled activity area the two components of the RPZ (see Figure 2-3).  The RPZ dimension for a particular runway end is a function of the type of aircraft and approach visibility minimum associated with that runway end.  Table 2-4 provides standard dimensions for RPZs. Other than with a special application of declared distances, the RPZ begins 200 feet (60 m) beyond the end of the area usable for takeoff or landing.  With a special application of declared distances, see Appendix 14, separate approach and departure RPZs are required for each runway end.

(a)	The Central Portion of the RPZ.  The central portion of the RPZ extends from the beginning to the end of the RPZ, centered on the runway centerline.  Its width is equal to the width of the runway OFA (see Figure 2-3).  Paragraph 307 contains the dimensional standards for the OFA.

(b)	The Controlled Activity Area.  The controlled activity area is the portion of the RPZ to the sides of the central portion of the RPZ.

(2)	Land Use.  In addition to the criteria specified in paragraph 211, the following land use criteria apply within the RPZ:

(a)	While it is desirable to clear all objects from the RPZ, some uses are permitted, provided they do not attract wildlife (see paragraph 202.g., Wildlife Hazards, and Appendix 17 for dimensional standards), are outside of the Runway OFA, and do not interfere with navigational aids.  Automobile parking facilities, although discouraged, may be permitted, provided the parking facilities and any associated appurtenances, in addition to meeting all of the preceding conditions, are located outside of the central portion of the RPZ.  Fuel storage facilities may not be located in the RPZ.

(b)	Land uses prohibited from the RPZ are residences and places of public assembly. (Churches, schools, hospitals, office buildings, shopping centers, and other uses with similar concentrations of persons typify places of public assembly.)  Fuel storage facilities may not be located in the RPZ.

b.	Recommendations.  Where it is determined to be impracticable for the airport owner to acquire and plan the land uses within the entire RPZ, the RPZ land use standards have recommendation status for that portion of the RPZ not controlled by the airport owner.

c.	FAA Studies of Objects and Activities in the Vicinity of Airports.  The FAA policy is to protect the public investment in the national airport system.  To implement this policy, the FAA studies existing and proposed objects and activities, both off and on public-use airports, with respect to their effect upon the safe and efficient use of the airports and safety of persons and property on the ground.  These objects need not be obstructions to air navigation, as defined in 14 CFR Part 77.  As the result of a study, the FAA may issue an advisory recommendation in opposition to the presence of any off-airport object or activity in the vicinity of a public-use airport that conflicts with an airport planning or design standard or recommendation.

213. to 299.  RESERVED

Table 2-1.  Runway Separation Standards for aircraft approach categories A & B

| ITEM | DIM 1/ | AIRPLANE DESIGN GROUP |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I 2/ | I |  | II |  | III |  | IV |
| Visual runways and runways with not lower than ¾-statute mile (1200 m) approach visibility minimums |  |  |  |  |  |  |  |  |  |
| Runway Centerline to: |  |  |  |  |  |  |  |  |  |
| Parallel Runway Centerline | H | - Refer to paragraphs 207 and 208 - |  |  |  |  |  |  |  |
| Holdline |  | - Refer to Advisory Circular 150/5340-1 - |  |  |  |  |  |  |  |
| Taxiway/Taxilane Centerline 3/ | D | 150 ft 45 m | 225 ft 67.5 m |  | 240 ft 72 m |  | 300 ft 90 m |  | 400 ft 120 m |
| Aircraft Parking Area | G | 125 ft 37.5 m | 200 ft 60 m |  | 250 ft 75 m |  | 400 ft 120 m |  | 500 ft 150 m |
| Helicopter Touchdown Pad |  | - Refer to Advisory Circular 150/5390-2 - |  |  |  |  |  |  |  |
| Runways with lower than ¾-statute mile (1200 m) approach visibility minimums 4/ |  |  |  |  |  |  |  |  |  |
| Runway Centerline to: |  |  |  |  |  |  |  |  |  |
| Parallel Runway Centerline | H | - Refer to paragraphs 207 and 208 - |  |  |  |  |  |  |  |
| Holdline |  | - Refer to Advisory Circular 150/5340-1 - |  |  |  |  |  |  |  |
| Taxiway/Taxilane Centerline 3/ | D | 200 ft 60 m | 250 ft 75 m |  | 300 ft 90 m |  | 350 ft 105 m |  | 400 ft 120 m |
| Aircraft Parking Area | G | 400 ft 120 m | 400 ft 120 m |  | 400 ft 120 m |  | 400 ft 120 m |  | 500 ft 150 m |
| Helicopter Touchdown Pad |  | - Refer to Advisory Circular 150/5390-2 - |  |  |  |  |  |  |  |

1/	Letters correspond to the dimensions on Figure 2-1.

2/	These dimensional 213standards pertain to facilities for small airplanes exclusively.

3/	The taxiway/taxilane centerline separation standards are for sea level.  At higher elevations, an increase to these separation distances may be required to keep taxiing and holding airplanes clear of the OFZ (refer to paragraph 206).

4/	For approaches with visibility less than ½-statute miles, runway centerline to taxiway/taxilane centerline separation increases to 400 feet (120 m).

Table 2-4. Runway protection zone (RPZ) dimensions

| Approach Visibility Minimums 1/ | Facilities Expected To Serve | Dimensions |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Length L Feet (meters) | Inner Width W1 feet (meters) | Outer Width W2 feet (meters) | RPZ acres |
| Visual And Not lower than 1-Mile (1 600 m) | Small Aircraft Exclusively | 1,000 (300) | 250 (75) | 450 (135) | 8.035 |
|  | Aircraft Approach Categories A & B | 1,000 (300) | 500 (150) | 700 (210) | 13.770 |
|  | Aircraft Approach Categories C & D | 1,700 (510) | 500 (150) | 1,010 (303) | 29.465 |
| Not lower than ¾-Mile (1 200 m) | All Aircraft | 1,700 (510) | 1,000 (300) | 1,510 (453) | 48.978 |
| Lower than ¾-Mile (1 200 m) | All Aircraft | 2,500 (750) | 1,000 (300) | 1,750 (525) | 78.914 |

1/ The RPZ dimensional standards are for the runway end with the specified approach visibility minimums. The departure RPZ dimensional standards are equal to or less than the approach RPZ dimensional standards. When a RPZ begins other than 200 feet (60 m) beyond the runway end, separate approach and departure RPZs should be provided. Refer to Appendix 14 for approach and departure RPZs.

Figure 2-3. Runway protection zone

Table 3-3. Runway design standards for aircraft approach categories C & D

| ITEM | DIM 1/ | AIRPLANE DESIGN GROUP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI |
| Runway Length | A | - Refer to paragraph 301 - |  |  |  |  |  |
| Runway Width | B | 100 ft | 100 ft | 100 ft 2/ | 150 ft | 150 ft | 200 ft |
|  |  | 30 m | 30 m | 30 m 2/ | 45 m | 45 m | 60 m |
| Runway Shoulder Width 3/ |  | 10 ft | 10 ft | 20 ft 2/ | 25 ft | 35 ft | 40 ft |
|  |  | 3 m | 3 m | 6 m 2/ | 7.5 m | 10.5 m | 12 m |
| Runway Blast Pad Width |  | 120 ft | 120 ft | 140 ft 2/ | 200 ft | 220 ft | 280 ft |
|  |  | 36 m | 36 m | 42 m 2/ | 60 m | 66 m | 84 m |
| Runway Blast Pad Length |  | 100 ft | 150 ft | 200 ft | 200 ft | 400 ft | 400 ft |
|  |  | 30 m | 45 m | 60 m | 60 m | 120 m | 120 m |
| Runway Safety Area Width 4/ | C | 500 ft | 500 ft | 500 ft | 500 ft | 500 ft | 500 ft |
|  |  | 150 m | 150 m | 150 m | 150 m | 150 m | 150 m |
| Runway Safety Area Length Prior to Landing Threshold 5/, 6/ |  | 600 ft | 600 ft | 600 ft | 600 ft | 600 ft | 600 ft |
|  |  | 180 m | 180 m | 180 m | 180 m | 180 m | 180 m |
| Runway Safety Area Length Beyond RW End 5/, 6/ | P | 1,000 ft | 1,000 ft | 1,000 ft | 1,000 ft | 1,000 ft | 1,000 ft |
|  |  | 300 m | 300 m | 300 m | 300 m | 300 m | 300 m |
| Obstacle Free Zone Width and Length |  | - Refer to paragraph 306 - |  |  |  |  |  |
| Runway Object Free Area Width | Q | 800 ft | 800 ft | 800 ft | 800 ft | 800 ft | 800 ft |
|  |  | 240 m | 240 m | 240 m | 240 m | 240 m | 240 m |
| Runway Object Free Area Length Beyond RW End 7/ | R | 1,000 ft | 1,000 ft | 1,000 ft | 1,000 ft | 1,000 ft | 1,000 ft |
|  |  | 300 m | 300 m | 300 m | 300 m | 300 m | 300 m |

1/	Letters correspond to the dimensions on figures 2-1 and 2-3.

2/	For Airplane Design Group III serving airplanes with maximum certificated takeoff weight greater than 150,000 pounds (68,100 kg), the standard runway width is 150 feet (45 m), the shoulder width is 25 feet (7.5 m), and the runway blast pad width is 200 feet (60 m).

3/	Design Groups V and VI normally require stabilized or paved shoulder surfaces.

4/	For Airport Reference Code C-I and C-II, a runway safety area width of 400 feet (120 m) is permissible.

5/	The runway safety area (RSA) length begins at each runway end when a stopway is not provided.  When a stopway is provided, the length begins at the stopway end.

6/ 	The standard RSA length beyond the runway end may be reduced to the standard RSA length prior to landing threshold if a standard Engineered Materials Arresting System (EMAS) is provided.  To qualify for this reduction, the EMAS installation must provide the ability to stop the critical aircraft exiting the end of the runway at 70 knots, and the runway must provide either instrument or visual vertical guidance for approaches in the opposite direction.  See AC 150/5220-22.

7/ 	The runway object free area length beyond the end of the runway never exceeds the standard RSA length beyond the runway end as provided by note 6 above.

This page intentionally left blank