POLAR Spacecraft Status As of: Thurs 31 July 2008, 16:00 EST

(Changes from last week in bold border)

| Subsystem/ Component | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- |
| LAUNCHED | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU-2: TOAR472 | B-BUS XSU-2: TOAR 471 1/15/08 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO |
| ADACS | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB N2 Low PSI invalid | ###### RGYRO Mode Estimate Y-ROLL Rgmode=PASSIVE Gyros-1 & 3 Inop/Off Gyro chans XA-ZB Pitch wheel bias 2400 *Z gas unload = 145 Gyro2 safe macro | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB GAS Enabled Dec 05 *Z Gas unload =45 | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch TOAR 447 Z-momentum transition complete Pitch Torquer again active |
| RXO | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 2 (since 4/3/03) 3 channels inop | 2 since 4/18/05 (analog drifting?) | 2 Swapped 9/20/06 | 1 Analog telem drifting |
| Recorders (DTR, SSR) | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B 4A Pressure leak | SSRs 1,2,3,4,5 Safestate=5B Current spikes |
| VTX (APT data) | #1 ON As of 19 June 07 | OFF / INOP since 11/15/00 | #2 ON 137.62 MHZ | #1 ON 137.1 MHZ As of 16 Aug 07 |
| BTX | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 2-Omni HRPT 2 Omni=rcp Power TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 Standby 3 low power INOP -10 Aug 2007 | 1 – HRPT TOAR424 low power 2 & 4 Playback STX-1 Power drop 2.29W 3/2/08 STX-4 nominal STX-3 Failed 2/14/08 | 3 - HRPT 1 - Playback 2 - Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, eclipse states and sun angle) | Array Off -45 Since 1/25/01 Full Sun Sun Angle 13 Uptrend to 18 early Jan 07 SA CANT=37 degs | Array Off -20 Since 9/13/07 Eclipse all year Sun Angle 34 Steady trend SA CANT=22 degs Full sun S/C in Sept 07 BVR Phase Control auton swap 7/19/07 | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 66 Steady trend SA CANT=37 degs BVR Phase Control auton swap 9/17/07 Swap back 9/18/07 SADPOS Enabled 2/12/08 | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 63 uptrend to 65 on 1 Jan 07 SA CANT=22 BVR Phase Control auton swap 3/29/06 & 1/4/07 |
| AVHRR | Nominal Channel 3B only Occasional minor scan motor instabilities | Nominal Recent SM current increases TCE24 OFF 4/14/04 Channel 3B only | ### Nominal 3A-B switch enabled Previous scan motor current mini surges | Nominal Channel 3B only Scan motor current drop 29 Aug 06 |
| AMSU-A1 | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise Channel 4 severely degraded 4 Mar 08 | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| `AMSU-A2 | Nominal | Nominal | Nominal | Non-nominal DIG-A Temps erratic 2/7/08 Cold Cal Pos 2 2/21/08 |
| AMSU-B | Operational Bias in All Chans. Degraded Chan-3 | Operational Space Position-3 Chan-8 degradation | Nominal | Replaced by MHS |
| ### HIRS | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | NOMINAL New all time high filter motor current reached 9/17/06. Imagery occasionally exhibits high levels of radiometric noise | NOMINAL 1 Pixel cross-track misalignment. | Operational Filter Heat OFF. LW channel 1-12 noise TOAR 448 Loose LWIR lens. LW imagery erratic. CH 1 Period Monitor event 13 May 08 |
| ### MHS/MIU |  |  |  | Nominal Fly Wheel Motor Current Spiking but not alarming since 27 Dec 06 Entered Motor Spin State on 9 May 08 H4 gain increased 9 May 08 H3 gain increased 9 May 08 |
| DCS RFI investigation | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 inop/off |
| SARR | Operational B-Side, 13 Dec 07 AGC Mode Intermittent failures for 243MHz A&B | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 124 & 243 MHz = INOP & OFF as of 29 Jul 08 | Nominal A-side All AGC Mode | Nominal A-Side All AGC Mode Since 15 Aug 05 |
| SARP | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=7 P-CDEM-HVPS=7 11 Jan 0714:47z | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=4 27 June 06 16:00z | Nominal Elec-CH-HVPS=1 27 June 06 17:00z Prot-CH-HVPS=1 11 Jan 07 14:15z |
| SBUV |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops TOAR 463 sun glint IBSL test performed 12 Sept 06 Grating motor investigation | Partially-Operational Anode mode since 6 July 05 13:30z Electronics turnoff taken out of STESM safestate table TOAR 462 sun glint Chop Motor stalled 22 May 08. Currently all motors turned on; not fully operational |
| SADPOS |  | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | TCE26 & 15H Telem valid TOAR 432 closed TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Spacecraft in thermal uptrend | Nominal TCE24 OFF 4/14/05 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 AMSU-A1 survival heaters on 10/29/03 | All TCE’s active HIRS filter heater turned OFF 9/28/06 |

COMMENTS/CHANGES:

MISC:	On 30 July 2008 (08/212) a minor FSW change to correct a flaw in the CTCES table was executed.  The set point for the HIRS TCE was not using the correct temperature point.  This was done for

There is a solar eclipse on 1 August 2008 (08/214) for spacecrafts.

Since Jason-2’s launch data gaps has been noticed on NOAA-16.

NOAA-15:	There were relatively no changes to the NOAA-15 bus or payload health situation during the past week.

The #2 gyro is still relatively quiet. Daily monitoring continues.

Summer eclipse season will be ending in mid August.

NOAA-16: 	There was relatively no change in the performance of the NOAA-16 bus & payload subsystems during the past week.

Summer eclipse season will be ending in mid August.

The NOAA-16 121 and 243 MHz SARR were turned off on 29 July 2008.

NOAA-17: 	There was relatively no change in the performance of the NOAA-17 payload subsystem during the past week.

NOAA-18:	There were relatively no changes to the NOAA-18 health situation during the past week.

The NOAA-18 SBUV is still functioning properly.  The plan is to return the instrument to normal operations in the next few weeks.

The NOAA-18 HIRS Scan Mirror Temperature telemetry point erratic readings from 2008/205 (July 23) have not returned.

On 2008/211 at 13:03:03 UTC, the SOCC AET observed the following telemetry bi-level points flagging in and out of their yellow limits: MHKDCOFE = 1 - (HK5:5) DC Offset Error Flag MSPDCOFE = 1 - (SD5:5) DC Offset Error Flag MSPDCOFV = 0 - (SD9:1) DC Offset Valid Flag MSPH5VAL = 0 - (SD10:10) H5 Data Validity Flag.  Additionally, the two composite ground software points that monitor the above points (QHKPRFLT and QSPPRFLT) were flagging in and out of yellow limits.

This signature of telemetry indicates that the Channel H5 data is triggering warning flags from the internal MHS Software. Upon investigation of the MHS Science Data, it appears that the H5 signal is dipping below the 5% of signal lower threshold for a few orbits each day. The minimum signal level, seen during the Space View pixels, has been near the 5% level for a while. Figure 2 depicts the lowest level observed (Space View) over the last 17 days for H5. There have been occasional crossings of the 5% boundary in recent days with a very slow downward trend of approximately 1% of the signal range in the time period observed.

Figure 3 is a plot of the 15 telemetered pixels for Channel H5 on DOY 2008/211. Channel H5 is using approximately 69% of its range. Performing a DC Offset Initialization (MHS FOM Procedure P330) would shift it back to the center of its range as it drifts below the 5% threshold. This is the recommended procedure (TOAR 482) to deal with the anticipated drift/shift expected in the more problematic Channels H3 and H4. It is an appropriate response in this situation as well.

At the current drift rate, the data is still usable and should remain so for several weeks.  This will give us a little time to continue monitoring the Channel H5 signal levels and to prepare to implement the corrective actions recommended in the related Channel H3 and H4 TOAR 482.

MetOp-A:       There were relatively no changes to the health and safety of the NOAA instruments on MetOp-A this past week.

Based on the CVF Digital A daily, weekly and long term trend reports, all parameters are nominal and stable.  All performance parameters are well within specification (e.g. NEdT, NEdN, NEdA and SNR).   Note:  AMSU-A1 Channel 7 NEdT is still increasing and following the same curve as mentioned in previous weekly reports.

Figure 2 NOAA-18 MHS (2008/193-212)

Figure 3 NOAA-18 MHS (2008/211-212)