POLAR Spacecraft Status As of: Thurs 21 Aug 2008, 12:00 EST

(Changes from last week in bold border)

| Subsystem/ Component | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- |
| LAUNCHED | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU-2: TOAR472 | B-BUS XSU-2: TOAR 471 1/15/08 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO |
| ADACS | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB N2 Low PSI invalid | ###### RGYRO Mode Estimate Y-ROLL Rgmode=PASSIVE Gyros-1 & 3 Inop/Off Gyro chans XA-ZB Pitch wheel bias 2400 *Z gas unload = 145 Gyro2 safe macro | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB GAS Enabled Dec 05 *Z Gas unload =45 | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch TOAR 447 Z-momentum transition complete Pitch Torquer again active Gas auto unloading enabled 8/18/08 |
| RXO | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 2 (since 4/3/03) 3 channels inop | 2 since 4/18/05 (analog drifting?) | 2 Swapped 9/20/06 | 1 Analog telem drifting |
| Recorders (DTR, SSR) | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B 4A Pressure leak | SSRs 1,2,3,4,5 Safestate=5B Current spikes |
| VTX (APT data) | #1 ON As of 19 June 07 | OFF / INOP since 11/15/00 | #2 ON 137.62 MHZ | #1 ON 137.1 MHZ As of 16 Aug 07 |
| BTX | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 2-Omni HRPT 2 Omni=rcp Power TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 Standby 3 low power INOP -10 Aug 2007 | 1 – HRPT TOAR424 low power 2 & 4 Playback STX-1 Power drop 2.29W 3/2/08 STX-4 nominal STX-3 Failed 2/14/08 | 3 - HRPT 1 - Playback 2 - Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, eclipse states and sun angle) | Array Off -45 Since 1/25/01 Full Sun Sun Angle 13 Uptrend to 18 early Jan 07 SA CANT=37 degs | Array Off -20 Since 9/13/07 Eclipse all year Sun Angle 34 Steady trend SA CANT=22 degs Full sun S/C in Sept 07 BVR Phase Control auton swap 7/19/07 | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 66 Steady trend SA CANT=37 degs BVR Phase Control auton swap 9/17/07 Swap back 9/18/07 SADPOS Enabled 2/12/08 | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 63 uptrend to 65 on 1 Jan 07 SA CANT=22 BVR Phase Control auton swap 3/29/06 & 1/4/07 |
| AVHRR | Nominal Channel 3B only Occasional minor scan motor instabilities | Nominal Recent SM current increases TCE24 OFF 4/14/04 Channel 3B only | ### Nominal 3A-B switch enabled Previous scan motor current mini surges | Nominal Channel 3B only Scan motor current drop 29 Aug 06 |
| AMSU-A1 | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise Channel 4 severely degraded 4 Mar 08 | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| `AMSU-A2 | Nominal | Nominal | Nominal | Non-nominal DIG-A Temps erratic 2/7/08 Cold Cal Pos 2 2/21/08 |
| AMSU-B | Operational Bias in All Chans. Degraded Chan-3 | Operational Space Position-3 Chan-8 degradation | Nominal | Replaced by MHS |
| ### HIRS | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | NOMINAL New all time high filter motor current reached 9/17/06. Imagery occasionally exhibits high levels of radiometric noise | NOMINAL 1 Pixel cross-track misalignment. | Operational Filter Heat OFF. LW channel 1-12 noise TOAR 448 Loose LWIR lens. LW imagery erratic. CH 1 Period Monitor event 13 May 08 |
| ### MHS/MIU |  |  |  | Nominal Fly Wheel Motor Current Spiking but not alarming since 27 Dec 06 Entered Motor Spin State on 9 May 08 H4 gain increased 9 May 08 H3 gain increased 9 May 08 |
| DCS RFI investigation | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 inop/off |
| SARR | Operational B-Side, 13 Dec 07 AGC Mode Intermittent failures for 243MHz A&B | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 124 & 243 MHz = INOP & OFF as of 29 Jul 08 406 RCVR ATT set to 5dB, PDS ATT set to 0dB | Nominal A-side All AGC Mode | Nominal A-Side All AGC Mode Since 15 Aug 05 |
| SARP | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=7 P-CDEM-HVPS=7 11 Jan 0714:47z | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=4 27 June 06 16:00z | Nominal Elec-CH-HVPS=1 27 June 06 17:00z Prot-CH-HVPS=1 11 Jan 07 14:15z |
| SBUV |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops TOAR 463 sun glint IBSL test performed 12 Sept 06 Grating motor investigation | Partially-Operational Anode mode since 6 July 05 13:30z Electronics turnoff taken out of STESM safestate table TOAR 462 sun glint Chop Motor stalled 22 May 08. Currently all motors turned on; not fully operational |
| SADPOS |  | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | TCE26 & 15H Telem valid TOAR 432 closed TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Spacecraft in thermal uptrend | Nominal TCE24 OFF 4/14/05 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 AMSU-A1 survival heaters on 10/29/03 | All TCE’s active HIRS filter heater turned OFF 9/28/06 |

COMMENTS/CHANGES:

MISC:

NOAA-15:	There were relatively no changes to the NOAA-15 bus or payload health situation during the past week.

The #2 gyro is still relatively quiet. Daily monitoring continues.

Summer eclipse season ended on 17 August 08 as expected.  Slight battery temperature increase and patch current of about 250 mamps (about pre-summer value).  Bus voltage min and max variation have settled into average bus voltage.  The load current has decreased due to reduced TCS demand and no charging of batteries required.

NOAA-16: 	There was relatively no change in the performance of the NOAA-16 bus & payload subsystems during the past week.

Summer eclipse season ended on 16 August 08 as expected.  Batteries 1 and 2 have had typical temperature increases that have settled in at 12 deg C and 9.5 deg C, battery 3 had a very slight increase in temperature.  The patch current is about 210 mamps which is about pre-summer value.   Bus voltage min and max variation have settled into average bus voltage.  The load current has decreased due to reduced TCS demand and no charging of batteries required.

Per request by USMCC/SARSAT, NOAA-16's SARR 406 MHz receiver attenuators will be set to 5 DB and its PDS channel attenuators to 0 DB on 20 August 08 (08/233) at 18:10z.  CTEC confirmed that they saw the change in the attenuator settings.

NOAA-17: 	There was relatively no change in the performance of the NOAA-17 payload subsystem during the past week.

NOAA-17 continues to have occasional bus voltage spikes.  It appears the shunts are responding to a small change in the load current that causes a large swing in the bus voltage and then a large swing in the shunt drive voltage.  This could be a result of the solar array current versus voltage curve operating near the knee. See Figures 1 & 2.

NOAA-18:	There were relatively no changes to the NOAA-18 health situation during the past week.

The Air Force informed NOAA of a possible close approach with a piece of debris from the FENGUYN Chinese satellite that was destroyed January 2007 on 18 August at 10:16z.  Auto Thruster Firing (Gas) was enabled on NOAA-18, and no other configuration change was done to the spacecraft based on the analysis of the size of the object.  The spacecraft was over the South Pole at the time of the predicted close approach.  No effects were seen on the spacecraft.

As a result of the Automatic Thruster Unloading Enable commanding for the close approach, the automatic momentum unloading system (ATMUL) was turned on for this encounter and left enabled to guard against future close encounters. The pitch thruster firing that occurred at 18:14 UT was determined to be due to nominal operational conditions and not the result of outside forces i.e. impact from debris or foreign object.  During normal spacecraft operations the momentum carried by the Z (pitch) RWA ranges from -25 to about -40 inch pound seconds (see Figure 3). The automatic thruster unloading threshold for the pitch axis is set at ±40 in. lb sec, which is too close to operational levels to avoid inadvertent thruster firings when automatic thruster momentum unloading is enabled. ATMUL was disabled to avoid additional inadvertent thruster firings. The threshold for the Z axis should be increased to 45 in. lb sec if ATMUL is to be enabled for future close encounters.  Once the investigation is complete and the thresholds are increased, auto thruster unloading will be re-enabled and remain enabled.  During the + pitch thruster firing the ROLL reasonableness test failed on the NON control side, which caused the MIMU to switch from MIMU 2 to MIMU 1, and set MIMU switching to INHIBIT.

The N18 MHS Channel 5 signal has been slowly drifting downward. This is an anticipated condition resulting from seasonal effects (cyclical temperatures) and aging hardware. The signal levels in the space view are near the lower 5% level, 3276 counts. When the signal is below 5%, the MHS raises flags to indicate the sensor may need adjustment to keep the signal from saturating. Figure 4 depicts the daily and orbital cycles in which the signal dips below the 5% level. The plan is to perform a DC Offset correction that will move the signal level back towards the center of its range. There are no current concerns of data loss/degradation or instrument health.

MetOp-A:       There were relatively no changes to the health and safety of the NOAA instruments on MetOp-A this past week.

The slightly increasing trend identified last week in the SARR 121.5 MHz Rx-A AGC Voltage (mean values) did not continue this week and can therefore be considered insignificant.

Figure 1 NOAA-17 Shunts (2008/230-231)

Figure 2 NOAA-15 RWA Speeds (2008/136-232)

Figure 3 NOAA-18 X,Y,Z Momentum (2008/231)

Figure 3 NOAA-18 MHS Channel 5 (2008/228-234)