POLAR Spacecraft Status As of: Tues 5 Dec 06, 14:00 EST

(changes from last week in bold border)

| Subsystem/ Component | NOAA-12D AM 16:59=AN STIP data only No IPD processing | NOAA-14J AM: 21:10=AN STIP data only No IPD processing | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC&LAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- | --- | --- |
| LAUNCHED | 05/14/91 | 12/30/94 | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 Recursive SCT | OBP2 OBP1 inoperative Recursive SCT | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU2 (XSU1 inop) | B-BUS XSU-1 | B-BUS XSU-1: TOAR444 | B-BUS XSU-1 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO | NO | NO |
| ADACS | Nominal Mode Autonomous swap to YGC 30 Aug, commanded back to nominal 31 Aug RGmode=Monitor Gyro chans=XYZ | YGC Mode Autonomous swap 1 Dec Very low N2 gas | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB N2 Low PSI invalid | ###### RGYRO Mode Estimate Y-ROLL Rgmode=PASSIVE Gyros-1 & 3 Inop/Off Gyro chans XA-ZB Gyro2 safe macro | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB GAS Enabled Dec 05 | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch TOAR 447 Z-momentum transition complete Pitch Torquer again active |
| RXO | PRI BU random enables daily bias+6.5 mS | PRI BU random enables daily bias–7.0 mS | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase Enabled | External Synch Macro rephased once per orbit | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 1 | 2 Since 19 Jul 06 | 2 (since 4/3/03) 3 channels inop | 2 since 4/18/05 (analog drifting?) | 2 Swapped 9/20/06 | 1 Analog telem drifting |
| Recorders (DTR, SSR) | DTRs 3A, 4, 5B Safestate 5B 4b low pressure | DTRs 1, 2B, 5 Safestate- 1B | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B 4A Pressure leak | SSRs 1,2,3,4,5 Safestate=5B Current spikes |
| VTX (APT data) | #1 ON 137.50 MHZ | OFF (nominal) Since 8/2/05 | #1 ON As of 13 Nov | OFF / INOP since 11/15/00 | #2 ON 137.62 MHZ | #2 ON 137.9125 MHZ |
| BTX | #1 136.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 1 – HRPT; 2,3 - Playback | 3 – HRPT 2, 1 – Playback | 2-Omni HRPT 2 Omni=rcp Power TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 – Playback & MCM usage 3 low power unused | 3 – HRPT TOAR424 low power 2 & 4 Playback 1-Standby STX-1 Power drop | 1 - HRPT 3 - Playback 2-Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, charge & V/T rates, eclipse states and sun angle) | ArrayOff +60 since 8/31/06 Full Sun Sun Angle 16 uptrend to 20 in early Jan 07 Shunt degraded SA CANT=37 degs PSSS trigger =18.5 | Array Off -50 Since 22Nov05 Eclipse all year Sun angle 55 steady Batts 1-3 LRC Degraded shunts SA CANT=22 degs PSS macro to safe SAD Motor switching enabled SAD side =A | Array Off -45 Since 1/25/01 Full Sun Sun Angle 13 Uptrend to 18 early Jan 07 SA CANT=37 degs | Array Off -40 Since 7/16/02 Eclipse all year Sun Angle 34 Steady trend SA CANT=22 degs Transitioning to full sun S/C in Sept 07 | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 66 Steady trend SA CANT=37 degs | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 63 uptrend to 65 on 1 Jan 07 SA CANT=22 BVR Phase Control auton swap 3/29/06 |
| AVHRR | Nominal HRPT and APT data only | Nominal HRPT data only Autonomous T/off 9/28 back on 9/30 Occasional SM current spikes. | Nominal Channel 3B only Occasional minor scan motor instabilities | Nominal Recent SM current increases TCE24 OFF 4/14/04 Channel 3B only | ### Nominal 3A-B switch enabled Previous scan motor current mini surges | Nominal Channel 3B only Scan motor current drop 29 Aug 06 |
| AMSU-A1 |  |  | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| AMSU-A2 |  |  | Nominal | Nominal | Nominal | Nominal |
| AMSU-B |  |  | Operational Bias in All Chans. Degraded Chan-3 | Operational Space Position-3 Chan-8 degradation | Nominal | Replaced by MHS |
| ### HIRS | ON/Inop Turned On 4/30/05 for power balance Filter Wheel OFF | NOMINAL IPD no longer processing data Outgas heater turned ON 13 Nov 14:00z turned OFF 14 Nov 14:00z | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | NOMINAL New all time high filter motor current reached 9/17/06. Imagery occasionally exhibits high levels of radiometric noise | NOMINAL 1 Pixel cross track misalignment. | Operational Filter Heat OFF. LW channel 1-12 noise TOAR 448 Loose LWIR lens. LW imagery erratic. |
| ### MHS/MIU |  |  |  |  |  | Nominal |
| MSU | OFF/Inop Turned off for power balance 6/23/05 Turnon considered for power balance | Operational AMDAHL Process shutdown permanently 10 Oct |  |  |  |  |
| SSU |  | OFF/Nominal Turned off 5 May for power balance |  |  |  |  |
| DCS RFI investigation | Nominal | Nominal Autonomous turnoff 28 Sept Autonomous turnon on 30 Sept | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 inop/off |
| SARR |  | Nominal Autonomous turnoff 28 Sept Commanded back on 29 Sept | Operational A-Side, AGC Mode Intermittent failures for 243MHz A&B | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 243 MHz = INOP. | Nominal A-side All AGC Mode | Nominal A-Side All AGC Mode Since 15 Aug 05 |
| SARP |  | OFF Power Failure | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | ON/Nominal Autonomous reset 8 Jun, back to nominal 10 June Turned on 8/30/04 Elec-CH-HVPS=4 Prot-CH-HVPS=0 TED level=3 | Operational Elec-CH-HVPS=0 Prot-CH-HVPS=0 1 of 4 Tscopes inop Autonomous Recursive SCT=no IFC capability | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=6 P-CDEM-HVPS=5 27 Jun 15:30z | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=4 27 June 16:00z | Nominal Elec-CH-HVPS=1 27 June 17:00z Prot-CH-HVPS=0 |
| SBUV |  | ON No longer operational. Used for power balance only since AMDAHL Process shutdown 10 Oct 06 |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops TOAR 463 sun glint IBSL test performed 12 Sept Grating motor investigation | Nominal Anode mode since 6 July 13:30z Electronics turnoff taken out of STESM safestate table TOAR 462 sun glint |
| SADPOS |  |  |  | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | Nominal TCE’s back to nominal configuration following 7/25/04 UVtrip. | Nominal Power survival safestate macro Implemented 28 Sept. Most TCE’s off. All TCE’s re-enabled 30 Sept 15:30z HIRS outgas heater turned on 13Nov, off 14Nov | TCE26 & 15H Telem invalid TOAR 432 TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Spacecraft in thermal uptrend | Nominal TCE24 OFF 4/14/05 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 AMSU-A1 survival heaters on 10/29/03 | All TCE’s active HIRS filter heater turned OFF 9/28/06 TCE3H displaying invalid readings TOAR 458 submitted |
|  | N12 | N14 | N15 | N16 | N17 | ### N18 |

COMMENTS/CHANGES:

NOAA-12:	EMOSS engineer Carl Gliniak is investigating some recent bus voltage instabilities on N12.  Nothing is yet considered

anomalous but some change in performance was detected in recent days and possible reconfigurations may be necessary to compensate for these changes (illustration 1).  Presently, a turnon of the MSU is being considered the best way to approach this situation with approval from NOAA management still pending the date for such an operation is TBD.

NOAA-14:	N14 was commanded back to nominal mode on 30 Nov at 18:35z with yaw precision improving markedly as a result.

However, almost exactly 24 hours later, as STX-1 was being turned on, the spacecraft again autonomously switched into YGC mode (where it remains today) with yet another degradation of yaw precision resulting.  At the time the switch occurred the NESAAI1 value was being dwelled upon (1 second dwell) so additional data is now available for the continuing investigation into this phenomenon.  With the main SOCC operational equipment move currently underway, and no complaints from direct readout users being received by NOAA, it was determined that N14 will remain in YGC mode at least for the next couple of weeks.  Jim Shepherd is leading the EMOSS side of this investigation.  Plans are still in effect to place the ESA in high speed dwell mode for 10 seconds before and 10 seconds after a STX-1 turnon (out of the ground schedule) but the date for this operation is presently TBD.

Other than the YGC mode operations the remainder of the N14 spacecraft performed nominally during the past week with the power subsystem remaining nominal and the AVHRR experiencing no scan motor current spikes during this entire time period (illustration 2).

NOAA-15:	There were relatively no changes to the N15 health situation during the past week.  Illustrations 3 and 4 depicts primary payloads and ADACS component performance during this time frame.

NOAA-16: 	There were relatively no changes to the N16 health situation during the past week (except a minor improvement in the

AVHRR scan motor current and a reversal of the AVHRR thermal uptrend, Illustration 5).  In general, N16 attitude control was nominal during the past week (illustration 6) although there was a short lived surge in the gyro-2 spin motor current on 2 Dec which was accompanied by a change in IMU block temp and TCA voltage (illustration 7).  Given the significant degradation of the N16 ADACS even small performance changes are a matter of concern.  Fortunately, these values quickly returned back to their more typical performance ranges.

Starting mid-September 2007, N16 will transition from what had always been a permanently eclipsing spacecraft to a full sun orbit (with eclipse seasons each summer).  This orbit-sun profile has been predicted to last at least until 2011 but may last even longer.  Carl Gliniak is investigating the implications of this change in order to be proactive about any reconfigurations that might be required when this even begins (see illustrations 17a and B at the end of this report).

NOAA-17: 	The N17 payloads generally performed nominally during the past week (illustrations 8) and no new concerns or

information has been forthcoming regarding a possible problem with the N17 SBUV (see last weeks report).  Otherwise, the N17 ADACS was generally nominal and stable during the past week although the IMU TCA voltage did experience a short lived surge on 29 Nov that was not accompanied by any other changes in ADACS component performance characteristics (illustrations 9 and 10) except perhaps a one count increase in gyro-1 spin motor current.  Such surges in TCA voltage may be indicative of a problem within the gyro and concerns with KLM gyro performance are continual.  However, everything returned back to nominal values and ranges within a short period of time so at the moment there does not seem to be cause for concern.

NOAA-18:	The N18 HIRS remained nominal from an engineering standpoint but degraded from a long wave imagery user perspective during the past week (illustration 11).  On the other hand, the N18 AVHRR and AMSU’s performed nominally and stably during this same time period (illustration 12).  The MHS was also relatively nominal during the past week except that the flywheel motor abruptly showed an increase of more than 100 mAmps (illustration 13) starting on 29 Nov (within previously observed norms).  Analysis of this payload has been difficult in the past week  because of the excessive number of noise spikes introduced by the ABE trending system into the engineering data stream.

Otherwise the N18 ADACS performed nominally during the past week (illustration 14 and 15) while the momentum transition (from negative to positive values) addressed in previous Weekly Reports appears to have reached its new equilibrium (illustration 16).  Since 29 Nov, the torquer coils have again begun to activate to unload the Rwheels and preliminary analysis has determined that they are all performing nominally.

NOTES: Illustrations 1, 2, 4, 6, 7, 9, 10, 14, 15 and 16 have been omitted for web hosting.

AVHRR							HIRS

Illustration 3:  N15 AVHRR & HIRS Health Analysis

(Nominal, even improved performance [AVHRR scan motor upper left plot] during this continued thermal uptrend.  AMSU’s [not depicted this week] were all nominal although still trending upward thermally.)

AVHRR							HIRS

Illustration 5:  N16 AVHRR & HIRS Health Analysis

(AVHRR scan motor current [upper left plot] improved again during the past week.  HIRS remained stable during this time.)

AVHRR							HIRS

Illustration 8:  N17 AVHRR & HIRS Health Analysis

(Nominal if not perfectly stable [AVHRR scan motor on Jday 333; upper left plot] last week.  AMSU’s A2 and B [not depicted this week] were nominal and stable during this same time period.)

Illustration 11:  N18 HIRS Health Analysis

(Generally nominal and stable last week from an engineering perspective but degraded from a long wave products point of view [lower right plot].)

Illustration 12:  N18 AMSU’s and AVHRR Health Analysis

(Nominal and stable last week.)

Illustration 13:  N18 MHS Motor Health Analysis

(Perhaps the worst noise interjection by ABE since N18 launch significantly reduces the fidelity of these two plots.  However, on Jday 333 [again 333] it is noticeable that there was an abrupt increase in the flywheel motor current although the range remains within acceptable norms.)