POLAR Spacecraft Status As of: Wed 13 Dec 06, 16:00 EST

(changes from last week in bold border)

| Subsystem/ Component | NOAA-12D AM 16:59=AN STIP data only No IPD processing | NOAA-14J AM: 21:10=AN STIP data only No IPD processing | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC&LAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- | --- | --- |
| LAUNCHED | 05/14/91 | 12/30/94 | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 Recursive SCT | OBP2 OBP1 inoperative Recursive SCT | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU2 (XSU1 inop) | B-BUS XSU-1 | B-BUS XSU-1: TOAR444 | B-BUS XSU-1 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO | NO | NO |
| ADACS | Nominal Mode Autonomous swap to YGC 30 Aug, commanded back to nominal 31 Aug RGmode=Monitor Gyro chans=XYZ | YGC Mode Autonomous swap 1 Dec Return to nominal TDB Very low N2 gas | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB N2 Low PSI invalid | ###### RGYRO Mode Estimate Y-ROLL Rgmode=PASSIVE Gyros-1 & 3 Inop/Off Gyro chans XA-ZB Gyro2 safe macro | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB GAS Enabled Dec 05 | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch TOAR 447 Z-momentum transition complete Pitch Torquer again active |
| RXO | PRI BU random enables daily bias+6.5 mS | PRI BU random enables daily bias–7.0 mS | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase Enabled | External Synch Macro rephased once per orbit | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 1 | 2 Since 19 Jul 06 | 2 (since 4/3/03) 3 channels inop | 2 since 4/18/05 (analog drifting?) | 2 Swapped 9/20/06 | 1 Analog telem drifting |
| Recorders (DTR, SSR) | DTRs 3A, 4, 5B Safestate 5B 4b low pressure | DTRs 1, 2B, 5 Safestate- 1B | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B 4A Pressure leak | SSRs 1,2,3,4,5 Safestate=5B Current spikes |
| VTX (APT data) | #1 ON 137.50 MHZ | OFF (nominal) Since 8/2/05 | #1 ON 137.50 MHz | OFF / INOP since 11/15/00 | #2 ON 137.62 MHZ | #2 ON 137.9125 MHZ |
| BTX | #1 136.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 1 – HRPT; 2,3 - Playback | 3 – HRPT 2, 1 – Playback | 2-Omni HRPT 2 Omni=rcp Power TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 – Playback & MCM usage 3 low power unused | 3 – HRPT TOAR424 low power 2 & 4 Playback 1-Standby STX-1 Power drop | 1 - HRPT 3 - Playback 2-Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, charge & V/T rates, eclipse states and sun angle) | ArrayOff +60 since 8/31/06 Full Sun Sun Angle 16 uptrend to 20 in early Jan 07 Shunt degraded SA CANT=37 degs PSSS trigger =18.5 | Array Off -50 Since 22Nov05 Eclipse all year Sun angle 55 steady Degraded shunts SA CANT=22 degs PSS macro to safe SAD Motor switching enabled SAD side =A | Array Off -45 Since 1/25/01 Full Sun Sun Angle 13 Uptrend to 18 early Jan 07 SA CANT=37 degs | Array Off -40 Since 7/16/02 Eclipse all year Sun Angle 34 Steady trend SA CANT=22 degs Transitioning to full sun S/C in Sept 07 | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 66 Steady trend SA CANT=37 degs | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 63 uptrend to 65 on 1 Jan 07 SA CANT=22 BVR Phase Control auton swap 3/29/06 |
| AVHRR | Nominal HRPT and APT data only | Nominal HRPT data only Autonomous T/off 9/28 back on 9/30 Occasional SM current spikes. | Nominal Channel 3B only Occasional minor scan motor instabilities | Nominal Recent SM current increases TCE24 OFF 4/14/04 Channel 3B only | ### Nominal 3A-B switch enabled Previous scan motor current mini surges | Nominal Channel 3B only Scan motor current drop 29 Aug 06 |
| AMSU-A1 |  |  | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| AMSU-A2 |  |  | Nominal | Nominal | Nominal | Nominal |
| AMSU-B |  |  | Operational Bias in All Chans. Degraded Chan-3 | Operational Space Position-3 Chan-8 degradation | Nominal | Replaced by MHS |
| ### HIRS | ON/Inop Turned On 4/30/05 for power balance Filter Wheel OFF | NOMINAL IPD no longer processing data Outgas heater turned ON 13 Nov 14:00z turned OFF 14 Nov 14:00z | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | NOMINAL New all time high filter motor current reached 9/17/06. Imagery occasionally exhibits high levels of radiometric noise | NOMINAL 1 Pixel cross track misalignment. | Operational Filter Heat OFF. LW channel 1-12 noise TOAR 448 Loose LWIR lens. LW imagery erratic. |
| ### MHS/MIU |  |  |  |  |  | Nominal Channel H5 space counts low |
| MSU | ON/Inop Turnon 12/7 @ 15:52 for power balance | Operational AMDAHL Process shutdown permanently 10 Oct |  |  |  |  |
| SSU |  | OFF/Nominal Turned off 5 May for power balance |  |  |  |  |
| DCS RFI investigation | Nominal | Nominal Autonomous turnoff 28 Sept Autonomous turnon on 30 Sept | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 inop/off |
| SARR |  | Nominal Autonomous turnoff 28 Sept Commanded back on 29 Sept | Operational A-Side, AGC Mode Intermittent failures for 243MHz A&B | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 243 MHz = INOP. | Nominal A-side All AGC Mode | Nominal A-Side All AGC Mode Since 15 Aug 05 |
| SARP |  | OFF Power Failure | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | ON/Nominal Autonomous reset 8 Jun, back to nominal 10 June Turned on 8/30/04 Elec-CH-HVPS=4 Prot-CH-HVPS=0 TED level=3 | Operational Elec-CH-HVPS=0 Prot-CH-HVPS=0 1 of 4 Tscopes inop Autonomous Recursive SCT=no IFC capability | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=6 P-CDEM-HVPS=5 27 Jun 15:30z | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=4 27 June 16:00z | Nominal Elec-CH-HVPS=1 27 June 17:00z Prot-CH-HVPS=0 |
| SBUV |  | ON No longer operational. Used for power balance only since AMDAHL Process shutdown 10 Oct 06 |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops TOAR 463 sun glint IBSL test performed 12 Sept Grating motor investigation | Nominal Anode mode since 6 July 13:30z Electronics turnoff taken out of STESM safestate table TOAR 462 sun glint |
| SADPOS |  |  |  | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | Nominal TCE’s back to nominal configuration following 7/25/04 UVtrip. | Nominal Power survival safestate macro Implemented 28 Sept. Most TCE’s off. All TCE’s re-enabled 30 Sept 15:30z HIRS outgas heater turned on 13Nov, off 14Nov | TCE26 & 15H Telem invalid TOAR 432 TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Spacecraft in thermal uptrend | Nominal TCE24 OFF 4/14/05 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 AMSU-A1 survival heaters on 10/29/03 | All TCE’s active HIRS filter heater turned OFF 9/28/06 TCE3H displaying invalid readings TOAR 458 submitted |
|  | N12 | N14 | N15 | N16 | N17 | ### N18 |

COMMENTS/CHANGES:

NOAA-12:	EMOSS engineer Carl Gliniak is investigating some recent bus voltage instabilities on N12.  Nothing is yet considered anomalous but some change in performance was detected.  The MSU was turned on 7 Dec at 16:52 to improve the power balance.

NOAA-14:	N14 remains in YGC mode.  Jim Shepherd is leading the EMOSS side of this investigation.  Plans are still in effect to place the ESA in high speed dwell mode for 10 seconds before and 10 seconds after a STX-1 turnon (out of the ground schedule) but the date for this operation is presently TBD.

Other than the YGC mode operations the remainder of the N14 spacecraft performed nominally during the past week with the power subsystem remaining nominal and the AVHRR experiencing no scan motor current spikes during this entire time period.

NOAA-15:	There were relatively no changes to the N15 health situation during the past week.

NOAA-16: 	There were relatively no changes to the N16 health situation during the past week.  In general, N16 attitude control was nominal during the past week.

Starting mid-September 2007, N16 will transition from what had always been a permanently eclipsing spacecraft to a full sun orbit (with eclipse seasons each summer).  This orbit-sun profile has been predicted to last at least until 2011 but may last even longer.  Carl Gliniak is investigating the implications of this change in order to be proactive about any reconfigurations that might be required when this even begins.

NOAA-17: 	The N17 payloads generally performed nominally during the past week and no new concerns or information has been forthcoming regarding a possible problem with the N17 SBUV.  Otherwise, the N17 ADACS was generally nominal and stable during the past week.

NOAA-18:	The N18 HIRS remained nominal from an engineering standpoint but degraded from a long wave imagery user perspective during the past week.  On the other hand, the N18 AVHRR and AMSU’s performed nominally and stably during this same time period.  The MHS was also relatively nominal during the past week except that on 8 Dec at 10:55z, several NOAA-18 MHS mnemonics flagged yellow, including the performance fault code, and the Channel H5 DAC offset showed an error.  A preliminary analysis by Eumetsat indicates that there is no risk to the instrument and no immediate action is required.  The DAC offset flag is designed to highlight when one of the five MHS channels nears the edge of its dynamic range and therefore risks saturation. The flag is raised when any channel enters this 5% band top or bottom of the dynamic range (5% = 3276, 95% = 62244).  In this case it appears that the Channel H5 SpaceView (COLD) counts are low. Engineering anticipates updating the DAC settings for Channel H5 and reconfiguring the MHS instrument within the following week.  However, the performance fault code (QSPPRFLT) will flag if an unrelated fault occurs.

Otherwise the N18 ADACS performed nominally during the past week.