THE TWENTY-NINTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/29)

(Fukuoka, Japan, 27-31 October 2008)

Agenda Item 5: Air Traffic Management (ATM) Issues

User Preferred Routes (UPRs) between RJAA and YSSY/YBBN/YBCS

(Joint Paper Presented by Civil Aviation Bureau, Japan (JCAB) and Federal Aviation Administration (FAA)

SUMMARY

This Information Paper provides information on the efforts to analyze the impacts and benefits of UPRs between RJAA and YSSY/YBBN/YBCS.

- Introduction

  - At the IPACG 26 meeting, Japan Airlines gave a presentation on a paper trial they had conducted to analyze the benefits of User Preferred Routes (UPRs) between Japan and Australia.  Their paper trial had shown a potential savings of over 1800 kilograms (kg) of fuel for a single flight.  They expressed their desire to flight plan UPRs between Japan and Australia.

  - JCAB presented a paper on flexible tracks between RJAA and YSSY/YBBN at the IPACG 27 meeting.  The study detailed the seasonal tendencies of the flexible track placement.  Southbound routes from RJAA to YSSY are east of B586 in most cases.  The northbound routes from YSSY to RJAA usually remain east of B586 in all seasons except summer.  During August the flexible routes would have a tendency to move west of B586.  The flexible routes would create more traffic conflicts than the current ATS Routes.  This was especially true in August when the flexible routes moved west of B586 where the traffic density is greater.

1.3 	UPRs between RJAA and YSSY/YBBN were discussed at the IPACG 27 Providers Meeting.  Both the FAA and JCAB had some concerns regarding the impact of these UPRs on air traffic control (ATC) Operations.  The FAA and JCAB agreed that a paper study would be beneficial in helping determine what the impact to ATC operations would be.  More discussion was needed to address the details of how this paper study would be conducted.  It was generally agreed between the JCAB and FAA that the implementation of UPRs should be the primary focus of the paper study, and that flexible routes based on PACOTS would be investigated as a secondary priority if UPRs could not be implemented.  It was agreed that the FAA would undertake efforts to coordinate a paper trial with the other affected ANSPs.

1.4	At the ISPACG 22 meeting JCAB and the FAA met with the other air navigation service providers (ANSPs) and airlines to reach an agreement on the details for the RJAA-YSSY/YBBN/YBCS UPR paper trial.  For the purposes of the initial paper trial, the ANSPs agreed to keep the restrictions very limited to determine the full potential savings and ATC impacts.

  - The initial paper trial indicated that significant annual savings of 1.89 million kg of fuel could be obtained by the implementation of UPRs between RJAA and YSSY/YBBN/YBCS.  There would be an additional environmental savings of 5.97 million kg of carbon dioxide (CO2) emissions.  One operator speculated that the actual annual savings would be even higher.

  - Partial data from the initial paper trial had been obtained at the IPACG/28 meeting.  The data and operational impacts were discussed.  It was determined that an additional paper trial would be required to determine the operational impacts during different seasonal conditions.

  - An additional UPR paper trial was coordinated to begin on 6 October 2008.  The ANSPs were asked to provide any operational requirements that would be necessary for the actual implementation of RJAA – YSSY/YBBN/YBCS UPRs for the second paper trial.  The details for the second paper trial are as follows:

A Paper Trial of UPRs between RJAA and YSSY/YBBN/YBCS began on 6 October 2008.  The paper trial runs for an eight week period with participating operators generating UPRs for their flights departing on Mondays (UTC Date) to compare against the current ATS routes in use today.  The comparison analyzes ATC impacts, operational advantages, time and fuel savings.

Trial Dates:

Comparison Paper UPRs will be generated for flights departing on the following UTC dates:

October 6, 2008

October 13, 2008

October 20, 2008

October 27, 2008

November 3, 2008

November 10, 2008

November 17, 2008

November 24, 2008

UPR Generation Paper Trial Guidelines:

1.  The North and Southbound UPRs will remain within the Fukuoka, Oakland, Guam, Port Moresby and Brisbane CTAs.

2.  The UPR paper routes will start or end at the Waypoints along latitude 30 North and follow existing conventional ATS, departure and arrival routings north of 30N.

3.  The flying time between filed reporting points must not exceed 80 minutes flying time.

4.  Aircraft routes must include filed reporting points on the CTA boundary crossings in the FPL.

5.  Aircraft must file around NOTAMed and charted active military airspace.

6.  To segregate UPR traffic with domestic traffic operating along the eastern coast of Australia (e.g. Brisbane – Cairns), the following gates will be used for Japan/Australia UPRs:

“150NM Cairns” (i.e. BENKI, MEMIG, PUPEB, MUNEL), HM, KELPI, or SETER

Aircraft departing Australia are to plan ATS routes (or CS DCT BENKI) to the gate then UPR

Aircraft arriving in Australia may plan UPR to the gate then via ATS route (or BENKI DCT CS)

7.  When generating the data and routings for this UPR paper trial, volcanic activity or ash clouds will not be considered for this effort.

Data Collection:

As a minimum we are looking to collect the following information.  Operators may use the attached Excel spreadsheet for the data collection if they find it helpful.

1. Aircraft Callsign

2. Type Aircraft

3. Current Structured Routing in use today for the flight including the departure and destination.

4. Enroute time for Current Fixed Routing

5. Fuel Burn for the Current Fixed Routing note kilos or pounds.

6. Corresponding UPR Route of flight

7. Enroute time for UPR Routing

8. Fuel Burn for the UPR Routing note kilos or pounds.

9. Note any other operational advantages if any.  i.e.  The UPR route allowed us to avoid an area of forecast severe turbulence.

After all the data has been compiled for the Trial day please forward it to the following email addresses, dennis.addison@faa.gov, kosugi-s07vf@atmc.mlit.go.jp, Adam.Watkin@AirservicesAustralia.com, Geoffrey.k.Ross@airservicesaustralia.com, ibe-n2z7@mlit.go.jp, Tim.Cornelison@faa.gov, Richard.Storaci@faa.gov, tbola@caa.com.pg

2.	Discussion

  - Initial data from the second paper trial will have been received at the beginning of the IPACG/29 meeting.  User Preferred Routes between RJAA and YSSY/YBBN/YBCS will be discussed at the IPACG/29 Providers Meeting.

- Action by the meeting

  - The meeting is invited to:

    - Note the information in this paper and:

    - Provide input on the UPR Paper Trials between RJAA and YSSY/YBBN/YBCS.