Common Terms and Definitions Group

| Braking Action Term | Definition | Aircraft Reaction (Pilot Reporting Guide) | ICAO Code |  | Runway Surface Condition | MU (runway MU)* |
| --- | --- | --- | --- | --- | --- | --- |
| Good | More braking capability is available than is used in typical deceleration on a non-limiting runway (i.e., a runway with additional stopping distance available). Directional control good. |  | 5 | Wet runway with a water depth of 1/8” or less Compacted snow with OAT below -15ºC ¾” or less of dry snow |  | 0.40 & above |
| Medium to Good |  |  | 4 |  |  | 0.36 – 0.39 |
| Medium | Noticeably degraded braking condition. Expect and plan for a longer stopping distance such as might be expected on a packed or compacted snow–covered runway. Effective directional control. |  | 3 | Compacted and sanded snow Compacted snow with an OAT above -15ºC Sanded ice |  | 0.30 – 0.35 |
| Medium to Poor |  |  | 2 |  |  | 0.29 – 0.26 |
| Poor | Very degraded braking condition (a potential for hydroplaning). Expect and plan for a significantly longer stopping distance such as might be expected on an ice-covered runway. Directional control minimally effective. |  | 1 | Wet snow Slush Wet runway with a water depth more than 1/8” Heavy rain Ice |  | 0.25 & below |
| Nil | Braking action is minimal to non-existent and/or directional control uncertain. | (Max) Thrust reverser utilized until stop. Directional control dependant on aerodynamic forces only (modulated TR use). Poor directional control even while taxiing. Full brake application with little or no effect. (Full anti-skid cycling throughout rollout.) |  | Wet Ice |  |  |

* Little confidence currently on any correlation.

ATC must require braking action reports from pilots whenever braking action is less than GOOD.

Wet means