| Firm Name: FEI Number: Inspection Date(s): FCE Number: Investigators: |  |  |  |  |
| --- | --- | --- | --- | --- |
| DEPARTMENT OF HEALTH AND HUMAN SERVICES FOOD AND DRUG ADMINISTRATION FDA LACF INSPECTION REPORT |  |  |  |  |
| This inspection report is available in PDF and M.S. Word on the forms site: http://www.fda.gov/opacommore choices/fdaforms/ora.html (only the M.S. Word version has expandable fields). Narrative responses to each item can be entered in the item’s “comments” area or where otherwise prompted. Complete documentation of deficiencies including deviations from Part 113 should be narrated with reference to photos, exhibits, etc, in the Turbo EIR under “Objectionable Conditions and Management’s Response”. When necessary, refer the reader to the appropriate section of the Turbo EIR for a full explanation of details. This form should be downloaded from the forms site to a computer drive prior to completion and copying. The finished report should be submitted as an attachment to the Turbo EIR. |  |  |  |  |
| PROCESS ESTABLISHMENT, FILING AND SCHEDULES |  |  |  |  |
| - HAS THE FIRM REGISTERED WITH FDA AND FILED A PROCESS FOR ALL LACFS PROCESSED AT THIS FACILITY, AND FOR FOREIGN FIRMS, ALL PRODUCTS PROCESSED AND SHIPPED TO THE U.S.? Yes No COMMENTS: |  |  |  |  |
| - HAVE PROCESSES BEEN ESTABLISHED FOR ALL LACFS PROCESSED AT THIS FACILITY? – 113.83 Yes No COMMENTS: |  |  |  |  |
| - LIST THE FIRM’S PROCESS AUTHORITIES: |  |  |  |  |
| - ARE THE PROCESS AUTHORITIES THE SAME AS THAT FILED WITH FDA? Yes No COMMENTS: |  |  |  |  |
| - DOES THE FIRM HAVE A PROCESS LETTER OR OTHER PROCESS SOURCE DOCUMENTATION LISTING CRITICAL FACTORS NECESSARY TO CONTROL IN THE ATTAINMENT OF COMMERCIAL STERILITY? Yes No COMMENTS: |  |  |  |  |
| 6. DO CRITICAL FACTORS/LIMITS LISTED IN SOURCE DOCUMENTS MATCH CRITICAL FACTORS/LIMITS FOR SELECTED PRODUCTS AND PROCESSES FILED WITH FDA? Yes No NOTE - CRITICAL FACTORS MAY EXIST WHICH THE FIRM CONTROLS BUT HAVE NOT BEEN IDENTIFIED IN THE PROCESS FILING AND/OR HAS FAILED TO IDENTIFY AND DOES NOT CONTROL) COMMENTS: |  |  |  |  |
| 7. HAVE FILED, SCHEDULED PROCESSES BEEN CHANGED IN SUCH A WAY THAT COULD AFFECT THE ATTAINMENT OF COMMERCIAL STERILITY? Yes No (THERE ARE MANY FACTORS WHICH MAY AFFECT HEAT PENETRATION AND THE ATTAINMENT OF COMMERCIAL STERILITY; THESE FACTORS INCLUDE CONTAINER TYPE AND POSITION, TYPE OF HEATING MEDIUM, PRODUCT FACTORS SUCH AS FILL WEIGHT, VISCOSITY, PARTICLE SIZE, PERCENT SOLIDS, AND EQUIPMENT FACTORS SUCH AS FILLING METHOD, HEAD SPACING AND ROTATIONAL SPEED—SEE PP. 8,9& 22 OF LACF FIELD GUIDE- PART 2. A CHANGE IN ANY OF THESE FACTORS COULD RESULT IN AN UNDER PROCESS.) COMMENTS: |  |  |  |  |
| 8. IF PROCESS CHANGE(S) HAVE BEEN MADE THAT COULD AFFECT THE ATTAINMENT OF COMMERCIAL STERILITY, HAVE THE CHANGE(S) BEEN REVIEWED AND SUBSTANTIATED BY A QUALIFIED SCIENTIFIC AUTHORITY AND FILED WITH FDA? (108.35(c)(2)(ii)) Yes No COMMENTS: |  |  |  |  |
| - THE FOLLOWING PRODUCTS WERE COVERED DURING THIS INSPECTION: |  |  |  |  |
| PRODUCT | STYLE OF PACK | CONTAINER TYPE/SIZE |  |  |
| COMMENTS: |  |  |  |  |
| - LIST ALL CRITICAL FACTORS TO THE ATTAINMENT OF COMMERCIAL STERILITY PER PROCESS AUTHORITY LETTER AND FILING FORM(S) FOR PRODUCTS COVERED DURING THIS INSPECTION: (INCLUDE VENT TIME/TEMP., INITIAL TEMPERATURE, MIN. PROCESS TIME/TEMP. AND ALL OTHER CRITICAL FACTOR TARGET VALUES—LIST OTHER CRITICAL FACTORS AND OPERATING PROCESSES IN COMMENTS.) RETORT VENT SCHEDULE: MINUTES AND TO °F. |  |  |  |  |
|  |  | MIN. CRITICAL FACTORS |  |  |
|  |  | Initial Temp. | Process Time | Process Temp. |
| PRODUCT | CONTAINER TYPE/SIZE |  |  |  |
| COMMENTS, INCLUDING OTHER CRITICAL FACTORS: |  |  |  |  |
| RAW MATERIALS – 113.81 |  |  |  |  |
| 11. DOES THE FIRM TAKE ADEQUATE MEASURES TO PREVENT THE BUILD-UP OF MICROORGANISMS IN UNPROCESSED PRODUCT BEFORE THERMAL PROCESSING? Yes No FOR EXAMPLE, RAW VEGETABLES SHOULD BE ADEQUATELY CLEANED BEFORE FILLING. FILLED/SEAMED AND UNPROCESSED CANS SHOULD BE RETORTED WITHIN A REASONABLE TIME LIMIT TO PREVENT INCIPIENT SPOILAGE. HOT WATER BLANCHERS SHOULD BE MAINTAINED AT TEMPERATURES ABOVE THAT WHICH WILL SUPPORT THE GROWTH OF THERMOPHILES AND BE EMPTIED, CLEANED AND SANITIZED ON A REGULAR BASIS TO PREVENT THE GROWTH OF |  |  |  |  |
| THERMOPHILES. RAW MATERIALS SUSCEPTIBLE TO CONTAMINATION BY THERMOPHILES (SUGAR, SALT, ETC.) SHOULD BE RECEIVED WITH A SUPPLIER’S GUARANTEE OR CERTIFICATE OF ANALYSIS) COMMENTS: |  |  |  |  |
| - WHAT IS THE SOURCE OF WATER USED FOR PROCESSING AND CLEAN-UP IN THE PLANT? COMMENTS: |  |  |  |  |
| 13. IS THE PLANT WATER ADEQUATELY TREATED WITH CHLORINE OR OTHER APPROVED CHEMICALS TO RENDER IT POTABLE? Yes No COMMENTS: |  |  |  |  |
| 14. ARE ALL FOOD AND COLOR ADDITIVES FDA APPROVED? Yes No N/A COMMENTS: |  |  |  |  |
| 15. ARE ADDITIVES USED TO TREAT BOILER WATER AND ARE THEY APPROVED FOR SUCH USE? (LIST ADDITIVES THAT ARE USED INCLUDING CHEMICAL NAME) Yes No COMMENTS: |  |  |  |  |
| PRODUCT PREPARATION – 113.81 |  |  |  |  |
| - ARE PRODUCTS PREPARED ACCORDING TO THE METHOD (HYDRATING, DRYING, ACIDIFYING, BLANCHING, ETC.)? AND/OR FORMULATION SPECIFIED IN THE RECOMMENDED SCHEDULED PROCESS? Yes No N/A COMMENTS: |  |  |  |  |
| 17. WHEN MAINTENANCE OF pH (ABOVE 4.6) OF A NORMALLY LOW-ACID FOOD IS A BASIS FOR A SCHEDULED PROCESS, DOES THE FIRM ENSURE THAT THE EQUILIBRIUM pH OF THE FINISHED PRODUCT MEETS THE VALUE SPECIFIED IN THE SCHEDULED PROCESS? – 113.81(e) Yes No N/A IN THIS CASE, THE FIRM MUST MONITOR pH AS A CRITICAL FACTOR AT INTERVALS OF SUFFICIENT FREQUENCY AND PREPARE/MAINTAIN RECORDS; THE pH METER SHOULD BE CALIBRATED TO ENSURE IT’S ACCURACY. COMMENTS: |  |  |  |  |
| - FOR WATER ACTIVITY CONTROLLED PROCESSES, IS THE WATER ACTIVITY (Aw) CAREFULLY CONTROLLED TO ENSURE THAT THE Aw OF THE FINISHED PRODUCT MEETS THAT OF THE SCHEDULED PROCESS? – 113.81(f) Yes No N/A (WHEN NORMALLY LOW-ACID FOODS REQUIRE SUFFICIENT SOLUTE TO PERMIT SAFE PROCESSING AT LOW TEMPERATURES, SUCH AS IN BOILING WATER, THERE SHALL BE CAREFUL SUPERVISION TO ENSURE THAT THE EQUILIBRIUM WATER ACTIVITY OF THE FINISHED PRODUCT MEETS THAT OF THE SCHEDULED PROCESS 113.81(F)). IN THIS CASE, THE FIRM MUST MONITOR WATER ACTIVITY AT INTERVALS OF SUFFICIENT FREQUENCY AND PREPARE/MAINTAIN RECORDS; THE WATER ACTIVITY METER SHOULD BE CALIBRATED TO ENSURE ITS ACCURACY. COMMENTS: |  |  |  |  |
| 19. ARE INGREDIENTS WEIGHED PROPERLY USING ACCURATE SCALES? Yes No COMMENTS: |  |  |  |  |
| EMPTY CONTAINER INTEGRITY |  |  |  |  |
| 20. DESCRIBE THE CONTAINERS BEING USED DURING THIS INSPECTION (SIZE, MATERIAL COMPOSITION, ETC.): |  |  |  |  |
| - SOURCE FOR THE FIRM’S CONTAINERS: INTEGRITY TESTS PERFORMED BY THE FIRM OR THE SUPPLIER ON INCOMING CONTAINERS: DESCRIBE HOW THE FIRM ASSURES THAT INCOMING CONTAINERS MEET THE SUPPLIER’S SPECIFICATION (FOR EXAMPLE, DO INCOMING CANS HAVE THE PROPER BASE WEIGHT, ENAMEL COATING, SEAMING COMPOUND, ETC?): |  |  |  |  |
| - DOES THE FIRM HAVE WRITTEN CRITERIA TO ACCEPT OR REJECT INCOMING EMPTY CONTAINER STOCK? Yes No ARE RECORDS KEPT OF ACCEPTED/REJECTED CONTAINER STOCK? Yes No COMMENTS: |  |  |  |  |
| 23. DOES THE FIRM CORRELATE INCOMING CONTAINERS (NAME OF SUPPLIER, CODES, ETC) WITH CONTAINER USAGE IN PRODUCTION? Yes No COMMENTS: |  |  |  |  |
| 24. ARE EMPTY CONTAINER HANDLING PROCEDURES ADEQUATE TO PREVENT DAMAGE? Yes No COMMENTS: |  |  |  |  |
| 25. ARE CONTAINERS AND LIDS CLEAN BEFORE FILLING? Yes No COMMENTS: |  |  |  |  |
| FILLING |  |  |  |  |
| 26. DESCRIBE THE METHOD OF FILLING CONTAINERS (HAND, VIBRATION, POCKET, ETC.) IS THIS METHOD THE SAME AS USED DURING PROCESS ESTABLISHMENT TESTS? Yes No COMMENTS: |  |  |  |  |
| - WERE ALL CRITICAL FACTORS (FILL WT, HEAD SPACE, ETC) BEING ADEQUATELY CONTROLLED? Yes No (CRITICAL FACTORS SPECIFIED IN THE SCHEDULED PROCESS SHALL BE MEASURED AND RECORDED ON THE PROCESSING RECORD AT INTERVALS OF SUFFICIENT FREQUENCY TO ENSURE THAT THE FACTORS ARE WITHIN THE LIMITS SPECIFIED IN THE SCHEDULED PROCESS 113.40(A)(14)) THERE ARE NUMEROUS CRITICAL FACTORS TO CONTROL DEPENDING ON THE PRODUCT, CONTAINER AND PROCESSING SYSTEM, ETC. -113.81(C) (SEE LACF INSPECTION GUIDE-PART 2). COMMENTS: |  |  |  |  |
| 28. DOES PRODUCT OVERLAY THE EDGES OF FILLED CONTAINERS? Yes No COMMENTS: |  |  |  |  |
| 29. ARE CAN FLANGES FREE OF DAMAGE AFTER FILLING? Yes No COMMENTS: |  |  |  |  |
| CLOSING |  |  |  |  |
| - LIST THE MANUFACTURER, MODEL NO. AND TYPE OF CLOSING MACHINES IN USE BY THE FIRM: COMMENTS: |  |  |  |  |
| - IS CONTAINER CLOSURE EQUIPMENT MAINTAINED SANITARY AND IN A GOOD STATE OF REPAIR? Yes No (FOR EXAMPLE, CHECK TO SEE IF THE FIRM HAS A MAINTENANCE LOG FOR THE DOUBLE SEAMERS THAT DOCUMENTS ROUTINE MAINTENANCE SUCH AS ADJUSTING OR CHANGING CHUCKS & ROLLS, ETC.; VISUAL OBSERVATION OF THE SEAMER LOG AND REVIEW OF SANITATION MONITORING RECORDS CAN INDICATE HOW THE FIRM CLEANS AND MAINTAINS THEIR DOUBLE SEAMING EQUIPMENT) COMMENTS: |  |  |  |  |
| 32. DURING PRODUCTION RUNS, DOES THE FIRM PERFORM VISUAL AND DESTRUCTIVE TESTS ON CONTAINER SEAMS/ SEALS IN ACCORDANCE WITH PART 113.60(A)? Yes No DESCRIBE ALL VISUAL AND DESTRUCTIVE TESTS PERFORMED INCLUDING TESTING FREQUENCY AND ALL MEASURED PARAMETERS (SEE LACF FIELD GUIDE-PART 3 FOR A DESCRIPTION OF METAL, GLASS AND FLEXIBLE PACKAGE CLOSURES, SEALING PARAMETERS, CONTAINER DEFECTS AND INTEGRITY TESTS) COMMENTS: |  |  |  |  |
| - ARE FILLED/SEALED CONTAINERS ADEQUATELY HANDLED IN A MANNER THAT REDUCES THE POTENTIAL FOR LEAKS AND ASSURES GOOD INTEGRITY? Yes No (FOR EXAMPLE, RETORT CRATES SHOULD NOT HAVE SHARP OR POINTED SURFACES THAT COULD PUNCTURE CONTAINERS; CONTAINERS SHOULD BE LOADED INTO CRATES AND RETORTS AND UNLOADED WITHOUT CAUSING CONTAINER DAMAGE) COMMENTS: |  |  |  |  |
| 34. DO PRODUCT CODES COMPLY WITH PART 113.60(C)? Yes No (THE CODE SHALL BE PERMANENTLY VISIBLE TO THE NAKED EYE AND SHALL IDENTIFY THE PACKER, PRODUCT, YEAR, DAY AND PERIOD OF PACK) COMMENTS: DESCRIBE THE CODING SYSTEM INCLUDING A CODE BREAKDOWN FOR PRODUCTS PRODUCED DURING THIS INSPECTION: |  |  |  |  |
| THERMAL PROCESSING EQUIPMENT & PROCEDURES – 113.40 |  |  |  |  |
| 35. WHAT TYPE OF THERMAL PROCESSING EQUIPMENT DOES THE FIRM USE? (LIST THE NUMBER AND TYPE OF RETORTS; SPECIFY WHICH RETORTS WERE BEING USED DURING THIS INSPECTION) COMMENTS: |  |  |  |  |
| 36. DOES THE THERMAL PROCESSING EQUIPMENT COMPLY WITH PART 113.40? Yes No (FOR A DETAILED DESCRIPTION OF DIFFERENT THERMAL PROCESSING EQUIPMENT AND SYSTEMS AND THE REGULATION REQUIREMENTS, SEE PP. 23-40 OF LACF FIELD GUIDE-PART 2 AND 21CFR PART 113.40; REFER TO ATTACHMENTS A-I FOR REPORTING FORMATS COVERING THE DIFFERENT THERMAL PROCESSING SYSTEMS) COMMENTS: |  |  |  |  |
| 37. WHERE VENTING ARRANGEMENTS VARY FROM THE EXAMPLES IN 113.40(a)(12), HAVE TEMPERATURE DISTRIBUTION STUDIES BEEN CONDUCTED ON THE RETORTS TO ESTABLISH EQUAL TEMPERATURE DISTRIBUTION AND, WHERE APPLICABLE, A VENT CYCLE? Yes No IF SO, WHO CONDUCTED THE STUDY, WHEN WAS IT CONDUCTED AND IS THERE DOCUMENTATION AVAILABLE THAT DOCUMENTS THE STUDY? (FOR AN EXPLANATION OF TEMPERATURE DISTRIBUTION, SEE P. 21 OF LACF FIELD GUIDE – PART 2) OTHER COMMENTS: |  |  |  |  |
| 38. HAVE THERE BEEN ANY CHANGES TO THE RETORTS OR THERMAL PROCESSING SYSTEM SINCE THE LAST TEMPERATURE DISTRIBUTION STUDY THAT COULD AFFECT TEMPERATURE DISTRIBUTION? Yes No (THE RETORT DESIGN, LOADING CONFIGURATION, SMALLEST CONTAINER SIZE AND MANY OTHER FACTORS CAN AFFECT THE ATTAINMENT OF TEMPERATURE DISTRIBUTION IN THE RETORT – SEE PP 21-22 OF LACF FIELD GUIDE-PART 2. A CHANGE IN ANY OF THESE FACTORS COULD NECESSITATE A NEW TEMPERATURE DISTRIBUTION STUDY AND POSSIBLY A NEW VENT SCHEDULE. IF A CHANGE HAS BEEN MADE IN THE THERMAL PROCESSING SYSTEM THAT COULD AFFECT TEMPERATURE DISTRIBUTION, THE FIRM SHOULD HAVE ON FILE DOCUMENTATION OF THE CHANGE, INCLUDING THE REVIEW AND APPROVAL BY A QUALIFIED PROCESS AUTHORITY.) COMMENTS: |  |  |  |  |
| 39. DOES THE FIRM OPERATE THE RETORTS USING THE PROCEDURE DEVELOPED DURING THE TEMPERATURE DISTRIBUTRION STUDY OR AS OUTLINED IN OTHER SUPPORTING DOCUMENTATION? Yes No COMMENTS: |  |  |  |  |
| 40. DO CRATES, TRAYS, GONDOLAS, ETC FOR HOLDING CONTAINERS FOR PROCESSING IN STEAM IN STILL RETORTS MEET THE REQUIREMENTS OF 113.40(a)(9)? Yes No COMMENTS: |  |  |  |  |
| THERMAL PROCESSING ROOM OPERATIONS – 113.87 |  |  |  |  |
| 41. ARE SCHEDULED PROCESSES AND VENTING PROCEDURES (IF APPLICABLE) POSTED IN THE RETORT ROOM OR READILY AVAILABLE TO THE RETORT OPERATOR? – 113.87(a) Yes No COMMENTS: |  |  |  |  |
| 42. DO POSTED (OPERATING) SCHEDULED PROCESSES MEET OR EXCEED THE RECOMMENDATIONS OF THE PROCESS AUTHORITY AND PROCESS SCHEDULES FILED WITH FDA? Yes No COMMENTS: |  |  |  |  |
| 43. HAS THE FIRM ESTABLISHED AN ADEQUATE SYSTEM FOR PRODUCT TRAFFIC CONTROL IN THE RETORT ROOM TO PREVENT UNRETORTED PRODUCT FROM BYPASSING THE RETORT PROCESS? Yes No |  |  |  |  |
| (EACH RETORT BASKET OR ONE OR MORE CANS WITHIN SHALL BE PLAINLY MARKED WITH HEAT-SENSITIVE INDICATOR TAPE, DYE OR PAINT OR BY OTHER EFFECTIVE MEANS THAT WILL INDICATE VISUALLY TO THERMAL PROCESSING PERSONNEL THOSE UNITS THAT HAVE BEEN RETORTED; A VISUAL CHECK SHALL BE PERFORMED TO DETERMINE WHETHER OR NOT THE APPROPRIATE CHANGE HAS OCCURRED IN THE HEAT-SENSITIVE INDICATOR AS A RESULT OF RETORTING FOR ALL RETORT BASKETS TO ENSURE THAT EACH UNIT OF PRODUCT HAS BEEN RETORTED; A WRITTEN RECORD OF THESE CHECKS SHOULD BE MADE – 113.87(b)). COMMENTS: |  |  |  |  |
| 44. IS THE INITIAL TEMPERATURE,“IT”, OF THE CONTENTS OF CONTAINERS TO BE PROCESSED DETERMINED AND RECORDED WITH SUFFICIENT FREQUENCY? – 113.87(c) Yes No MEASURE THE “IT” OF AT LEAST 1 RETORT LOAD WITH A CALIBRATED THERMOMETER AND REPORT THE RESULTS IN “COMMENTS”. COMMENTS: (THE INITIAL TEMPERATURE IS A CRITICAL FACTOR IN THE ATTAINMENT OF COMMERCIAL STERILITY—EQUALLY IMPORTANT AS PROCESS TIME, RETORT TEMPERATURE AND ANY OTHER CRITICAL FACTOR) DOES THE “IT” MEASURED BY THE INVESTIGATOR AGREE WITH THE FIRM’S MEASURED “IT” AND DOES THIS “IT” AT LEAST MEET OR EXCEED THE MINIMUM “IT” FILED WITH FDA? Yes No IF NO, EXPLAIN: |  |  |  |  |
| 45. ARE PROCEDURES FOR MEASURING INITIAL TEMPERATURE PROPERLY MADE? Yes No (INITIAL TEMPERATURE IS DETERMINED BY SELECTING A CONTAINER REPRESENTING THE COLDEST CONTAINER IN THE RETORT LOAD; JUST PRIOR TO THE START OF THE PROCESS, THE CONTENTS OF THE CONTAINER ARE THOROUGHLY MIXED AND THE TEMPERATURE IS DETERMINED USING A CALIBRATED THERMOMETER. FOR THOSE RETORT SYSTEMS THAT USE WATER PRIOR TO OR DURING PROCESSING, PROVISIONS SHALL BE MADE TO ENSURE THAT THE INITIAL TEMPERATURE IS REPRESENTATIVE OF EITHER THE COLDEST CONTAINER (COLDEST COMPONENT IN CONTAINER) OR THE WATER IN THE RETORT, WHICHEVER IS COLDER) – 113.87(c) IF QUESTIONABLE, DESCRIBE THE FIRM’S PROCEDURE AND FREQUENCY FOR CHECKING PRODUCT “IT”: |  |  |  |  |
| 46. ARE THERMAL PROCESS TIMING DEVICES ACCURATE? Yes No (POCKET OR WRISTWATCHES ARE NOT CONSIDERED SATISFACTORY; DIGITAL CLOCKS THAT DO NOT DISPLAY SECONDS MAY BE USED IF THE OPERATING PROCESS AND THE VENTING SCHEDULE HAVE A 1-MINUTE OR GREATER SAFETY FACTOR OVER THE SCHEDULED PROCESS) – 113.87(d). COMMENTS: |  |  |  |  |
| 47. DOES THE RETORT OPERATOR ADEQUATELY CONTROL AND MONITOR THE RETORT DURING PROCESSING? Yes No (THE OPERATOR SHOULD VISUALLY MONITOR THE MIG THERMOMETER AT THE END OF THE COME-UP TIME (START OF THERMAL PROCESS) AND DURING THE PROCESS) (THE RECORDING THERMOMETER CHART SHALL BE ADJUSTED TO AGREE AS NEARLY AS POSSIBLE WITH BUT NOT BE HIGHER THAN THE MIG THERMOMETER DURING THE PROCESSING PERIOD (113.40(a)(2)) COMMENTS: |  |  |  |  |
| 48. IS THE STEAM SUPPLY (PRESSURE) TO THE RETORTS SUFFICIENT TO ASSURE AN ADEQUATE COME-UP AND THERMAL PROCESS? Yes No COMMENTS: |  |  |  |  |
| 49. OBSERVE A FULL RETORT CYCLE USING A CALIBRATED STOPWATCH. COMPARE YOUR OBSERVATIONS WITH THE FILED AND POSTED PROCESSES. DOES YOUR OBSERVATIONS OF THE VENT TIME/COME-UP TIME AND TEMPERATURE AND THE PROCESS TIME AND TEMPERATURE AGREE WITH OR EXCEED THE VENT AND SCHEDULED PROCESSES ESTABLISHED BY THE PROCESS AUTHORITY AND FILED WITH FDA? Yes No COMMENTS: |  |  |  |  |
| POST PROCESS HANDLING |  |  |  |  |
| 50. ARE POST PROCESS CAN CONVEYOR TRACKS MAINTAINED SANITARY? Yes No (CONVEYOR TRACKS SHOULD BE CLEAN , SANITIZED AND DRY) COMMENTS: |  |  |  |  |
| 51. IS RETORT COOLING WATER RECIRCULATED OR HELD IN A COOLING CANAL? Yes No COMMENTS: |  |  |  |  |
| 52. IS RETORT COOLING WATER TREATED WITH CHLORINE OR OTHER SANITIZER (S)? Yes No LIST THE SANITIZER (S) USED IN THE FIRM’S COOLING WATER AND THE CONCENTRATION LEVEL (MEASURED DURING THIS INSPECTION) AT THE WATER DISCHARGE END. (CONTAINER COOLING WATER SHALL BE CHLORINATED OR OTHERWISE SANITIZED AS NECESSARY FOR COOLING CANALS AND FOR RECIRCULATED WATER SUPPLIES; THERE SHOULD BE A MEASURABLE RESIDUAL OF THE SANITIZER AT THE WATER DISCHARGE POINT OF THE CONTAINER COOLER – 113.60(b)). COMMENTS: |  |  |  |  |
| 53. ARE CONTAINER HANDLING PROCEDURES AND CONVEYANCE EQUIPMENT ADEQUATE TO PROTECT THE CONTAINER BODY AND SEALS FROM DAMAGE THAT COULD RESULT IN LEAKAGE AND POST PROCESS CONTAMINATION? Yes No COMMENTS: |  |  |  |  |
| 54. WHAT IS THE FIRM’S PROCEDURE IF CONTAINER DEFECTS ARE FOUND AFTER THERMAL PROCESSING TO ASSURE THAT THE LOT IS SAFE FOR DISTRIBUTION? |  |  |  |  |
| 55. EXAMINE ANY SUSPECT PRODUCT CODES IDENTIFIED THROUGH RECORD REVIEW. IF NO LOTS ARE SUSPECT, RANDOMLY SELECT SEVERAL CODES FOR VISUAL EXAMINATION. REPORT THE RESULTS OF THE FIELD EXAMINATION IN THE COMMENTS FIELD BELOW. IF ABNORMAL CONTAINERS ARE IDENTIFIED THROUGH WAREHOUSE EXAMINATION, REVIEW THE CORRESPONDING PROCESSING AND CONTAINER INSPECTION RECORDS. SAMPLE ABNORMAL LOTS FOLLOWING IOM SAMPLE SCHEDULE CHART 2. COMMENTS: |  |  |  |  |
| RECORDS – 113.100 |  |  |  |  |
| 56. IS PROCESSING AND PRODUCTION INFORMATION RECORDED AT THE TIME IT IS OBSERVED BY THE RETORT OPERATOR? 113.100(b) Yes No COMMENTS: |  |  |  |  |
| 57. DO PROCESSING AND PRODUCTION RECORDS INCLUDE THE PRODUCT, PRODUCT CODE, DATE, RETORT NO., APPROX. NUMBER OF CONTAINERS PER CODING INTERVAL, INITIAL TEMPERATURE, ACTUAL PROCESSING TIME, MIG AND RECORDING THERMOMETER READINGS AND OTHER APPROPRIATE PROCESSING DATA PER PART 113.100(a)? Yes No COMMENTS: |  |  |  |  |
| 58. ARE RECORDING THERMOMETER CHARTS IDENTIFIED BY DATE, RETORT NUMBER, AND OTHER DATA AS NECESSARY SO THEY CAN BE CORRELATED WITH THE WRITTEN RECORD OF LOTS PROCESSED? – 113.100(b) Yes No COMMENTS: |  |  |  |  |
| 59. ARE PROCESSING AND PRODUCTION RECORDS SIGNED OR INITIALED BY THE RETORT OPERATOR AND REVIEWED FOR COMPLETENESS & SIGNED OR INITIALED AND DATED BY PLANT MANAGEMENT WITHIN 1 WORKING DAY AFTER THE ACTUAL PROCESS TO ASSURE THAT THE PRODUCT RECEIVED THE SCHEDULED PROCESS? – 113.100(b) Yes No COMMENTS: |  |  |  |  |
| 60. ARE THE RESULTS OF VISUAL AND DESTRUCTIVE CONTAINER INTEGRITY TESTS DOCUMENTED PER PART 113.60(a)? ) Yes No (WRITTEN RECORDS SHALL SPECIFY THE PRODUCT CODE, THE DATE AND TIME OF CONTAINER CLOSURE INSPECTIONS, THE MEASUREMENTS OBTAINED, AND ALL CORRECTIVE ACTIONS TAKEN; THE RECORDS SHALL BE SIGNED OR INITIALED BY THE CONTAINER CLOSURE INSPECTOR AND REVIEWED BY MANAGEMENT WITH SUFFICIENT FREQUENCY TO ASSURE THE CONTAINERS ARE HERMETICALLY SEALED – 113.100(c)). COMMENTS: |  |  |  |  |
| 61. REVIEW A SELECT NUMBER OF PROCESSING RECORDS (RETORT LOGS, RECORDING THERMOMETER CHARTS, RECORDS OF OTHER CRITICAL FACTOR MONITORING), AND CONTAINER INTEGRITY TEST RECORDS (BOTH VISUAL AND TEARDOWN INSPECTION RECORDS) REPRESENTATIVE OF UP TO 7 PRODUCTION DAYS DURING A 3 MONTH PERIOD, IF AVAILABLE, IMMEDIATELY PRIOR TO THIS INSPECTION. FOLLOW THE PROCEDURES FOR SELECTING RECORDS OUTLINED ON PAGE 83 (ATTACHMENT 12) OF LACF INSPECTION GUIDE-PART 2. DID THE REVIEW OF THESE RECORDS DISCLOSE ANY DEVIATIONS FROM PART 113 OR ANY DEFICIENCIES OR INFORMATION INDICATING THAT ANY LOT OF LACF PRODUCED AT THIS ESTABLISHMENT MAY HAVE THERMAL PROCESS DEVIATIONS OR CONTAINER INTEGRITY DEFICIENCIES? Yes No IF YES, EXPLAIN IN “COMMENTS” BELOW. REPORT THE TYPE AND DATES OF RECORDS REVIEWED. COMMENTS: |  |  |  |  |
| 62. ARE COPIES OF ALL RECORDS PROVIDED FOR IN PART 113 EXCEPT THOSE PERTAINING TO THE ESTABLISHMENT OF SCHEDULED PROCESSES RETAINED AT THE PROCESSING PLANT FOR AT LEAST 1 YEAR FROM THE DATE OF MANUFACTURE AND AT THE PROCESSING PLANT OR OTHER REASONABLY ACCESSIBLE LOCATION FOR AN ADDITIONAL 2 YEARS? – 113.100(e) Yes No COMMENTS: |  |  |  |  |
| PROCESS DEVIATIONS – 113.89 |  |  |  |  |
| 63. DOES THE FIRM HAVE A WRITTEN PROCEDURE FOR HANDLING PROCESS DEVIATIONS? Yes No COMMENTS: |  |  |  |  |
| 64. DOES THE FIRM MAINTAIN A SEPARATE FILE OR LOG FOR DOCUMENTING PROCESS DEVIATIONS? Yes No COMMENTS: |  |  |  |  |
| 65. WERE ANY PROCESS DEVIATIONS NOTED DURING THE INSPECTION? Yes No IF SO, WERE THE DEVIATIONS PROPERLY HANDLED? Yes No COMMENTS: |  |  |  |  |
| 66. WERE ANY PROCESS DEVIANT LOTS (NOT PROPERLY HANDLED) SHIPPED INTERSTATE? Yes No IF SO, WAS FDA NOTIFIED? – 108.35(d&e) Yes No COMMENTS: |  |  |  |  |
| 67. DOES MANAGEMENT FULLY UNDERSTAND THE DEFINITION AND MEANING OF THE TERM “PROCESS DEVIATION” AND PROCEDURES FOR HANDLING THEM AS DEFINED AND STATED IN 113.89? Yes No COMMENTS: |  |  |  |  |
| INCUBATION – 113.40(g)(3) |  |  |  |  |
| 68. ARE THE RESULTS OF INCUBATION TESTS RECORDED? Yes No DESCRIBE ANY INCUBATION TESTS PERFORMED ON FINISHED PRODUCTION LOTS (INCLUDE SAMPLING, INCUBATION AND TEST PROCEDURES): IF POSITIVE RESULTS ARE FOUND, WHAT FOLLOW-UP ACTION DOES THE FIRM TAKE TO ASSURE THAT THE AFFECTED LOT IS SAFE FOR DISTRIBUTION? NOTE – INCUBATION TESTING IS RECOMMENDED BUT NOT REQUIRED FOR ASEPTICALLY PROCESSED PRODUCTS – 113.40(g)(3). |  |  |  |  |
| PERSONNEL – 108.35/113.10 |  |  |  |  |
| 69. ARE ALL OPERATORS OF THERMAL PROCESSING SYSTEMS AND CONTAINER CLOSURE INSPECTIONS UNDER THE OPERATING SUPERVISION OF A PERSON WHO HAS ATTENDED A SCHOOL APPROVED BY FDA? Yes No COMMENTS: |  |  |  |  |
| PLANT AND EQUIPMENT SANITATION – 110.35/40 |  |  |  |  |
| 70. IS PLANT AND EQUIPMENT SANITATION ADEQUATE TO PREVENT THE ADULTERATION OF FOOD WITH PHYSICAL, CHEMICAL OR MICROBIOLOGICAL CONTAMINANTS? Yes No (SUMMARIZE THE FIRM’S PROCEDURES FOR CLEANING AND SANITIZING FOOD CONTACT EQUIPMENT BOTH PRE AND POST PROCESS) COMMENTS: |  |  |  |  |
| RECALL PROCEDURES |  |  |  |  |
| - DOES THE FIRM HAVE PREPARED RECALL PROCEDURES ON FILE THAT COMPLY WITH 108.35(f)? Yes No |  |  |  |  |
| COMMENTS: |  |  |  |  |
| 72. DOES THE FIRM MAINTAIN INITIAL DISTRIBUTION RECORDS PER 113.100(d)? Yes No COMMENTS: |  |  |  |  |