NASA GSFC Airborne Science Flight Operations Standards

- Minimum Operating Standards For All Aircraft Operations

Operations

- PIC – Airline Transport Pilot (ATP) Certificate

- SIC – Commercial Pilots Certificate with an instrument rating

- All pilots in previous year - 100 hrs TT & a satisfactory proficiency flight check

- All pilots in previous 90 days – 3 Takeoffs & Landings (Night if night ops)

- Continuous radio capability with a ground station for flight following

- Life raft which holds all for extended overwater ops (50NM from land)

- Aircraft performance – positive climb gradient with engine loss

- Alcohol use policy – 10 hours prior & not under the influence

- Flight plans will be filed for all flights with appropriate ATC

Maintenance

- Aircraft History - date of manufacture, registration #, & total time

- Copy of Aircraft Registration & Airworthiness Certificate

- Copy of Current Weight & Balance

- Copy of last Airframe and Powerplant Inspection

- FAA Form 337 or other documentation on any modifications

- FAA Form 337 or other documentation on any equipment installations

- Documentation showing compliance with all Airworthiness Directives

- List personnel authorized to perform aircraft maintenance & their qualifications

- List personnel authorized to return aircraft to service & their qualifications

- Other standards requiring justification/management approval for deviation from

Operations

- Non-turbine or Single pilot aircraft operations

- PIC without 200 hours in make/model

- PIC without – ATP, 500ME, 2500TT, 200Inst, 50 Night, 40(10 make/model) last 90 days

- SIC without – Comm w/inst,   700TT, 100Inst, 50 Night, 30(10 make/model) last 90 days

- Pilots without – 6 T/Os & Ldgs (Night if night ops), 3 instr appr/3 instr hrs in last 90 days

- No GPWS or radar altimeter for flight below 2000 feet (except for t/o & land)

- No ELT for remote (outside of VHF range from ground station) operations

- Crew Duty over 12 hours (14hrs if none midnight-4AM) & crew rest less than 12 hours

- Crew flight time limits greater than 8 hours a day or 40 hours a week

- Flights in close proximity to known or forecast severe weather

- No filed alternate landing site with destination weather below 5000/5

- No operating weather radar for other than day VFR flights

Maintenance

- Any deviations from manufacturers’ overhaul & replacement schedule

- Documentation to be reviewed for all flight operations

- Description of company that owns/operates the aircraft with contact info

- Description of company personnel by position & organization chart

- Training & medical records - Initial & recurrent for crewmembers & maintenance personnel

- List of aircraft radios, navigation equipment, & survival gear

- Company operations rules – include Min runway length, Min fuel, Min Wx for T/O, Ldg, & Alt, and drug & alcohol testing policy along with aviation safety program/policy.

- Copy of Contract or Letter of Agreement

- Project Documentation– Description of project including NASA project manager contact info, project objectives, payload description, list of personnel & equipment onboard during flight and:

- Flight plan – include airports, route of flight, op area, other aircraft involved

- Safety plan – list of possible ground & flight hazards and risks including safety controls

- Flight Readiness Review Briefing – brief to mgmnt on readiness of maint, ops, & project