# Carbon Cycle Investigation

Sample Site Field Guide

Task

Locate, set up and photograph a Carbon Cycle Sample Site.

Pre-requisites

Before setting up a field site you should know:

The number of your paces that equal 21.2 m and 30 m

How to read a compass and walk on a bearing

What a GPS does and how to use it

Materials

GPS

Compass

Camera

50 meter tape measure or a 30 m length of rope

Flagging tape or 5 flags

## Carbon Cycle Sample Site Data Sheet

### Student Field Guide for GPS Protocol and GPS Data Sheet

Clipboard

Pencil

Calculator

Procedure

- Locate an approximately 30 m x 30 m area that is representative of the vegetation in your schoolyard, town and region. Notes: This may be a site previously or currently used for Land Cover measurements.  If a 30 m x 30 m area is not available find the largest area of any size that is available (this may be circular, rectangular or a smaller square).

- Complete the top of your Carbon Cycle Sample Site Data Sheet.

- Flag the center of your Carbon Cycle Site.

- Identify the latitude, longitude and elevation of the center following the Field Guide for GPS Protocol.  Record the average (may need a calculator) latitude, longitude and elevation from the GPS Data Sheet on the Carbon Cycle Sample Site Data Sheet. *This may be completed back in the classroom.

- Using the camera, take a photo in each cardinal direction – north, south, east, west.  Use your compass to determine the directions.  Record each photo number in the correct arrow on your Data Sheet.

- From the center, use your compass to pace out 21.2 m in each diagonal direction – NE, SE, NW, SW and place a flag in each of the four corners.  To check if you have paced correctly and your site is square pace or measure 30 m from corner to corner using the cardinal directions as you move around the sample site.

- Note and record any unusual or helpful metadata on your Carbon Cycle Sample Site Data Sheet.