POLAR Spacecraft Status As of: Thurs 24 Jan 08, 12:00 EST

(Changes from last week in bold border)

| Subsystem/ Component | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- |
| LAUNCHED | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU-2: TOAR472 | B-BUS XSU-2: TOAR 471 1/15/08 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO |
| ADACS | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB N2 Low PSI invalid | ###### RGYRO Mode Estimate Y-ROLL Rgmode=PASSIVE Gyros-1 & 3 Inop/Off Gyro chans XA-ZB Pitch wheel bias 2400 *Z gas unload = 145 Gyro2 safe macro | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB GAS Enabled Dec 05 *Z Gas unload =45 | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch TOAR 447 Z-momentum transition complete Pitch Torquer again active |
| RXO | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 2 (since 4/3/03) 3 channels inop | 2 since 4/18/05 (analog drifting?) | 2 Swapped 9/20/06 | 1 Analog telem drifting |
| Recorders (DTR, SSR) | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B 4A Pressure leak | SSRs 1,2,3,4,5 Safestate=5B Current spikes |
| VTX (APT data) | #1 ON As of 19 June 07 | OFF / INOP since 11/15/00 | #2 ON 137.62 MHZ | #1 ON 137.1 MHZ As of 16 Aug 07 |
| BTX | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 2-Omni HRPT 2 Omni=rcp Power TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 Restored -11 Oct 2007 3 low power INOP -10 Aug 2007 | 3 – HRPT TOAR424 low power 2 & 1 Playback STX-1 Power drop STX-4 no modulation | 1 - HRPT 3 - Playback 2-Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, eclipse states and sun angle) | Array Off -45 Since 1/25/01 Full Sun Sun Angle 13 Uptrend to 18 early Jan 07 SA CANT=37 degs | Array Off -20 Since 9/13/07 Eclipse all year Sun Angle 34 Steady trend SA CANT=22 degs Full sun S/C in Sept 07 BVR Phase Control auton swap 7/19/07 | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 66 Steady trend SA CANT=37 degs BVR Phase Control auton swap 9/17/07 Swap back 9/18/07 | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 63 uptrend to 65 on 1 Jan 07 SA CANT=22 BVR Phase Control auton swap 3/29/06 & 1/4/07 |
| AVHRR | Nominal Channel 3B only Occasional minor scan motor instabilities | Nominal Recent SM current increases TCE24 OFF 4/14/04 Channel 3B only | ### Nominal 3A-B switch enabled Previous scan motor current mini surges | Nominal Channel 3B only Scan motor current drop 29 Aug 06 |
| AMSU-A1 | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| `AMSU-A2 | Nominal | Nominal | Nominal | Nominal |
| AMSU-B | Operational Bias in All Chans. Degraded Chan-3 | Operational Space Position-3 Chan-8 degradation | Nominal | Replaced by MHS |
| ### HIRS | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | NOMINAL New all time high filter motor current reached 9/17/06. Imagery occasionally exhibits high levels of radiometric noise | NOMINAL 1 Pixel cross-track misalignment. | Operational Filter Heat OFF. LW channel 1-12 noise TOAR 448 Loose LWIR lens. LW imagery erratic. |
| ### MHS/MIU |  |  |  | Nominal Fly Wheel Motor Current Spiking but not alarming since 27 Dec 06 Entered Motor Spin State on 17 Jan 08 H4 gain increased 2 Oct 07 H3 gain increased 31Oct 07 |
| DCS RFI investigation | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 inop/off |
| SARR | Operational B-Side, 13 Dec 07 AGC Mode Intermittent failures for 243MHz A&B | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 243 MHz = INOP. | Nominal A-side All AGC Mode | Nominal A-Side All AGC Mode Since 15 Aug 05 |
| SARP | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=7 P-CDEM-HVPS=7 11 Jan 0714:47z | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=4 27 June 06 16:00z | Nominal Elec-CH-HVPS=1 27 June 06 17:00z Prot-CH-HVPS=1 11 Jan 07 14:15z |
| SBUV |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops TOAR 463 sun glint IBSL test performed 12 Sept 06 Grating motor investigation | Nominal Anode mode since 6 July 05 13:30z Electronics turnoff taken out of STESM safestate table TOAR 462 sun glint |
| SADPOS |  | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | TCE26 & 15H Telem valid TOAR 432 closed TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Spacecraft in thermal uptrend | Nominal TCE24 OFF 4/14/05 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 AMSU-A1 survival heaters on 10/29/03 | All TCE’s active HIRS filter heater turned OFF 9/28/06 |

COMMENTS/CHANGES:

MISC:	Analysis and planning for the upcoming Eclipse on 7 February 2008 is ongoing.

NOAA-15:	There were relatively no changes to the NOAA-15 payload health situation during the past week.

NOAA-15 yaw gyro is getting noisier again after the first two weeks of 2008 were relatively quiet. Daily monitoring continues.

NOAA-16: 	There was relatively no change in the performance of the NOAA-16 payload or bus subsystems during the past week.

NOAA-17: 	There was relatively no change in the performance of the NOAA-17 payload or bus subsystems during the past week.

Additional testing of the STX-4/XSU is being performed on 24-25 Jan 2008 to verify STX-4 is completely operational.

NOAA-18:	There were relatively no changes to the NOAA-18 health situation during the past week.

SARSAT requested 24 hours of 1 sec dwell on the SAR 406 AGC level (Ch 457) which was performed from 13:51:24 on 23 Jan to 13:51:24 on 24 Jan 2008.

MetOp-A:      On 16 Jan 2008 (08/016) at 13:42z MetOp-A entered a PLSOL, Payload Switch Off-line.  All payload including NOAA instruments were shut off at this time.  All NOAA instruments were commanded back on by 18 Jan 2008 and have returned to nominal levels.  The following table shows the times each instrument was commanded back to operational mode:

| Instrument | Date/Time |
| --- | --- |
| SEM | 2008-017-11:40:29.961 |
| AVHRR | 2008-017-21:16:01.835 |
| ADCS | 2008-017-16:31:30.866 |
| AMSU-A2 | 2008-017-18:22:41.925 |
| AMSU-A1 | 2008-017-19:52:45.843 |
| HIRS | 2008-017-21:17:37.933 |
| SARR | 2008-018-09:27:31.754 |
| SARP | 2008-018-09:30:29.789 |
| ADCS | 2008-018-09:33:24.786 |
| SEM | 2008-018-10:42:29.089 |

AR9125, “CFS (CCU) Scrubbing Counter”:  This anomaly eventually triggered the PLSOL that occurred on Jan. 16 at 13:42 UTC.  EUMETSAT assumes a hardware failure occurred on the nominal side I/O board of the CCU computer, causing an autonomous reconfiguration of the CCU subsystems to the redundant sides.  When this occurred, the PLM PMC temporarily lost communication with the CCU and consequently requested the CCU to issue a PLSOL.  When recovering the satellite in the evening of January 16, EUMETSAT reconfigured all other CCU subsystems back to their nominal sides, but left the I/O board selection as the redundant side.  The follow-on investigations will be determined by ARB, but it is likely that EUMETSAT will use some opportunity in the future (e.g. perhaps during the OOP-out of plane- maneuver or after the next PLSOL) to return to the nominal side I/O board in order to determine whether the nominal side indeed has a hardware failure or whether a software reset would have cleared the problem.

There is an in-plane maneuver scheduled for 31 Jan 2008.