POLAR Spacecraft Status As of: Tues, 14 Feb, 16:00 EST

(changes from last week in bold border)

| Subsystem/ Component | NOAA-12D AM 16:59=AN STIP data only | NOAA-14J AM: 21:10=AN STIP data only | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC&LAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- | --- | --- |
| LAUNCHED | 05/14/91 | 12/30/94 | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 Recursive SCT | OBP2 OBP1 inoperative | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU2 (XSU1 inop) | B-BUS XSU-1 | B-BUS XSU-1 XSU-1 TOAR 444 | B-BUS XSU-1 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO | NO | NO |
| ADACS X=Yaw Y=Roll Z=Pitch | Nominal Mode RGmode=Monitor Gyro chans=XYZ | Nominal Mode Rgmode=Disabled Gyro chans=XYZ | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB | ###### RGYRO Mode Estimate ROLL Rgmode=PASSIVE Gyro-1 Off 10/24/05 Gyro-3 Off 6/10/04 Gyro chans XA-ZB TOAR 454 | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB Gyro-2 TOAR 451 | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch TOAR 447 NEW Z-Momentum shift transition started 1 Feb 06 |
| RXO | PRI BU random enables daily bias+6.5 mS | PRI BU random enables daily bias–7.0 mS | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase Enabled | External Synch Rephase each orbit | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 1 | 1 | 2 (since 4/3/03) #2 has 3 chans inop | 2 since 4/18/05 | 1 Analog telem drifting | 1 Analog telem drift? |
| Recorders (DTR, SSR) | DTRs 3A, 4, 5B Safestate 5B | DTRs 1, 2B, 5 Safestate- 1B | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B | SSRs 1,2,3,4,5 Safestate=5B Current spikes TOAR 450 |
| VTX (APT data) | #1 ON 137.50 MHZ | OFF (nominal) Since 8/2/05 | #1 ON 137.50mhz | INOP/Off OFF since 11/15/00 | #2 ON 137.62 MHZ Turnoff planned 3/1 | #1 ON 137.10 MHZ Swapped 1/4/06 |
| BTX | #1 136.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 1 – HRPT; 2,3 - Playback | 3 – HRPT 2, 1 – Playback | 2-Omni HRPT 2 omni=rcp Power Drop TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 – Playback & MCM usage 3 low power unused | 3 – HRPT TOAR424 low power 2 & 4 Playback 1-Standby STX-1 Power drop TOAR 443. | 1 - HRPT 3 - Playback 2-Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, charge & V/T rates, eclipse states and sun angle) | ArrayOff +60 since 6/23/05 Full Sun Sun Angle 20 steady trend Batts 1&2=LRC Batt 1&2 V/T=4 Shunt degraded SA CANT=37 degs | Array Off -50 Batts 1&3V/T=6/9 Batt2 VT=7/9 Since 11/22/05 Eclipse all year Sun angle 43 Steady trend Batts 1-3= LRC Degraded shunts SA CANT=22 degs | Array Off -45 Since 1/25/01 Full Sun Sun Angle 10 Downtrend to 7 1 Mar Batts 1-3= LRC V/T=7/9 SA CANT=37 degs | Array Off -40 Since 7/16/02 Eclipse all year Sun Angle 57 Steady Trend Batts 1-3= LRC V/T=4/8 SA CANT=22 degs | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 61 Steady trend Batts 1-3= LRC V/T= 4/8 SA CANT=37 degs | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 65 up trend to 72 mid-Jul TOAR 453 on Batt C/D ratio Batts 1-3=LRC Batts 1-3 V/T=4/8 SA CANT=22 |
| AVHRR | Nominal | Variable Status upgraded to Yellow 20 Dec Occasional scan motor current surge and bar codes Channels 2&4 APT | Nominal Rare major scan motor surges. Last Incident 11/18/04 Channel 3B only Recent scan motor current rise | Variable Nominal scan motor currents & imagery since mid Oct 05 TCE24 OFF 4/14/04 Channel 3B only. | ### Nominal 3A-B switch enabled | Nominal 3A-B switch Disabled 8/05/05 Channel 3B only |
| AMSU-A1 |  |  | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| AMSU-A2 |  |  | Nominal | Nominal Space Position-2 | Nominal | Nominal Full scan mode |
| AMSU-B |  |  | Operational Bias in All Chans. Motor surges most recent 11/14/04 degraded data on Chan-3 since 4/30 | Operational Space Position-3 IPD reports channel 8 degradation | Nominal | Replaced by MHS |
| ### HIRS | OFF/Inop Turned off 6/23/05 For power balance Filter Wheel turned OFF 10/17/02. | NOMINAL Operational Occasional loss of Fwheel synch | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | Operational Imagery high levels radiometric noise TOAR 441 CLOSED Recent FW synch dropouts Occasional major FM surges, most recent 12/9/04 | NOMINAL 1 Pixel cross track misalignment. | Operational LW channel 1-12 noise TOAR 448 Problem isolated to loose LWIR lens. Increased longwave noise since 5 Feb Status UPGRADED to Green 4 Jan 05 |
| ### MHS/MIU |  |  |  |  |  | Nominal Space View=0 Turbo anomalies 16 Dec & 6 Jan TOAR 456 Investigating dropouts on 20 Jan |
| MSU | OFF/Inop Turned off for power balance 6/23/05 | Operational Scan motor and antenna problems EOL test planned |  |  |  |  |
| SSU |  | Nominal |  |  |  |  |
| DCS RFI under investigation | Nominal | Nominal | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 off TOAR 449 Closed 6 no fix possible |
| SARR |  | Nominal | Operational A-Side, AGC Mode Intermittent failures for 243MHz A&B | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 243 MHz = INOP. | Nominal A-side All AGC Mode | Nominal A-Side All AGC Mode Since 15 Aug 05 |
| SARP |  | OFF Power Failure | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | ON/Nominal Turned on 8/30/04 Elec-CH-HVPS=4 Prot-CH-HVPS=0 TED level=3 as of 27 Sept | Operational Elec-CH-HVPS=0 Prot-CH-HVPS=0 1 of 4 Tscopes inop Autonomous TED shutdown 24 Sept | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=5 P-CDEM-HVPS=4 | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=3 | Nominal Elec-CH-HVPS=0 Prot-CH-HVPS=0 |
| SBUV |  | Degraded Grate motor sticks Flex memory 3 (giant step) ops suspended 12/2/03 Reduced operations Transient anomaly 1 Sept 05 |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops Procedure changes forthcoming | Nominal Anode mode since 6 July 13:30z Electronics turnoff taken out of STESM safestate table |
| SADPOS |  |  | N/A | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | Nominal TCE’s back to nominal configuration following 7/25/04 UVtrip. | Nominal | TCE26 & 15H Telem invalid TOAR 432 TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Thermal downtrend | Nominal TCE24 OFF 4/14 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 TOAR 428 CLOSED 9/14/04 no corrective action possible. AMSU-A1 survival heaters on 10/29/03 | All TCE’s active |
|  | N12 | N14 | N15 | N16 | N17 | ### N18 |

COMMENTS/CHANGES:

NOAA-12:	There were no changes to N12 during the past reporting period.  N12 and N15 are coming into RF conflict starting 19

Feb with N15 VTX-1 (APT data) will be off beginning 1 Mar through 2 April.

NOAA-14:	The AVHRR scan motor current spikes have ceased in recent days resulting in nominal HRPT imagery.  The N14

Power subsystem was also relatively nominal during the past week (illustration 1).

NOAA-15:	There were no changes of note during the past week.  The N15 payload suite was generally nominal with the AMSU’s

thermal trend currently down, slowly, as the sun angle on the spacecraft falls to 7 degrees (illustration 2).  The AVHRR scan motor exhibited some improvements last week after some instabilities were evident the week prior (illustration 3) while the N15 HIRS filter motor current remained rock solid.

N12 and N15 are coming into RF conflict starting 19 Feb with N15 VTX-1 (APT data) will be off beginning 1 Mar through 2 April.

NOAA-16:	The N16 AMSU’s were all nominal and stable during the past week (illustration 5) and the AVHRR returned to more

nominal values after experiencing a significant but not anomalous scan motor current surge the week prior (illustration 6).  On the other hand, despite this instability on scan motor current (and synch delta) N16 AVHRR imagery remained completely nominal throughout with no wave patterns, MIRP rephase frame losses or bar codes evident at any time (illustration 7).  The HIRS also remained relatively stable at slightly below historical high filter motor current values last week (illustration 8) and there was no additional instances where filter wheel synch was momentarily lost (since 4 Feb).

There was no change to the DCS configuration on N16 during the past week with pseudo messaging remaining on for N16 but off for all other POES spacecraft.

NOAA-17:	The two operational N17 AMSU’s were stable and nominal during the past week (illustration 12) as were the AVHRR

and HIRS (illustration 13).

NOAA-18:	The N18 HIRS long wave channels remained somewhat unstable and less than nominal during much of the past week

with impacts on products still occurring even though all HIRS engineering values appear nominal and stable (illustration 16 & 17) nor were any spacecraft reconfigurations implemented that could produce LWIR lens vibrations.   Other than these HIRS problems, the remainder of the N18 payload suite was nominal and stable during the past week (illustration 18-20).

Engineers continue to investigate the “dual problem” of MHS dropouts on 20 Jan and the minor frame counter “problem” that has since been determined to be caused an as yet isolated ground system processing problem.  No additional information has yet been determined.

POWER  SUBSYSTEM                                            AVHRR

Illustration 1:  NOAA-14 Power & AVHRR Health Analysis

Illustration 2:  N15 AMSU’s Health Analysis

(Nominal with thermal profile trending downward as sun angle cycle approaches its bottom of 7 degrees in the next several days.)

Illustration 3:  N15 AVHRR Health Analysis

(A return to more auspicious values for the scan motor last week. Not yet all the way back but more than halfway.)

Illustration 4 omitted for web publication

Illustration 5:  N16 AMSU’s Health Analysis

(Nominal and stable last week.)

Illustration 6:  N16 AVHRR Health Analysis

(After a significant rise, stability was reached than a return to more nominal values.  No imagery impacts were apparent.)

Illustration 7:  N16 AVHRR Imagery Analysis

(During the “worst” of last weeks scan motor current surge, no waves, no bars, and no frame losses.)

Illustration 8:  N16 HIRS Health Analysis

(generally nominal and stable last week.)

Illustrations 9-11 omitted for web publication

ILLUSTRATION 12:  N17 AMSU’s Health Analysis

(Everything appears nominal and stable during the past week.)

HIRS						     AVHRR

ILLUSTRATION 13:  N17 AVHRR & HIRS Health Analysis

(Nominal and stable last week.)

Illustration 17 omitted for web publication

ILLUSTRATION 16:  N18 HIRS Health History Analysis

(Engineering aspect of the HIRS remained nominal and stable throughout the channel-1 noise situation nor were any spacecraft reconfigurations implemented that might have induced vibration into the loose lens cover.)

ILLUSTRATION 17:  N18 HIRS Imagery Comparison

(Significantly less stable than desired. Especially channel-1)

ILLUSTRATION 18:  N18 MHS Health Analysis

(Nominal and stable last week, spikes all caused by ground processing problems.)

ILLUSTRATION 19:  N18 AMSU’s Health Analysis

(Stable and nominal across the board.)

ILLUSTRATION 20:  N18 AVHRR Analysis

(Stable and nominal)

Illustration 21 and 22 omitted for web publication

# POLAR SPACECRAFT TIP CLOCK ERROR

## WEEK ENDING February 10, 2006

| DAY/DATE | NOAA-12 | NOAA-14 | NOAA-15 | NOAA-16 | NOAA-17 | NOAA-18 |
| --- | --- | --- | --- | --- | --- | --- |
| FRI. 02/03 | +502 msec | -179 msec | +097 msec | -088 msec | -242 msec | -279 msec |
| SAT. 02/04 | +503 msec | -179 msec | +096 msec | -088 msec | -240 msec | -275 msec |
| SUN. 02/05 | +502 msec | -181 msec | +094 msec | -087 msec | -238 msec | -273 msec |
| MON. 02/06 | +502 msec | -178 msec | +093 msec | -085 msec | -237 msec | -272 msec |
| TUE. 02/07 | +500 msec | -180 msec | +090 msec | -086 msec | -236 msec | -270 msec |
| WED. 02/08 | +498 msec | -177 msec | +092 msec | -084 msec | -235 msec | -269 msec |
| THU. 02/09 | +497 msec | -171 msec | +092 msec | -085 msec | -234 msec | -270 msec |

NOAA-14:  As of 12/23/03 the TIP clock is corrected daily by –7.0 msec to compensate for drift.

NOAA-15:  As of 04/22/03 the TIP clock is corrected daily by –8.0 msec to compensate for drift.

NOAA-16:  As of 12/23/03 the TIP clock is corrected daily by -5.5 msec to compensate for drift.

NOAA-17:  As of 12/23/03 the TIP clock is corrected daily by –3.0 msec to compensate for drift.

NOAA-12:  As of 12/10/03 the TIP clock is corrected daily by +6.5 msec to compensate for drift.

*    A clock jump of -500 milliseconds on 09/15/05 due to fractional second daily ETCUP. A clock

jump of +500 milliseconds on 11/30/05 due to fractional second daily ETCUP. Alternating +/-

500 millisecond clock jumps will ccur at 76-day intervals as long as fractional second daily

ETCUP is utilized to manage clock drift. The next -500 millisecond jump is expected on 02/14/06.

**  TIP Clock corrections were made to compensate for the positive leap second introduced on 12/31/05.