POLAR Spacecraft Status As of: Thus 19 Feb 2009, 12:00 EST

(Changes from last week in bold border)

| Subsystem/ Component | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- |
| LAUNCHED | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU-2: TOAR472 | B-BUS XSU-2:TOAR 471 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO |
| ADACS | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB N2 Low PSI invalid | ###### RGYRO Mode Forced Nominal Mode Estimate Y-ROLL Rgmode=PASSIVE Gyros-1 & 3 Inop/Off Gyro chans XA-ZB Pitch wheel bias 2400 *Z gas unload = 145 Gyro2 safe macro | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB GAS Enabled Dec 05 *Z Gas unload =45 | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch Z-momentum transition complete Pitch Torquer again active Gas auto unloading enabled &*Z Gas unload =45 12/9/08 |
| RXO | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 2 since 4/3/03 3 channels inop analog telem drifting | 2 since 4/18/05 analog telem drifting | 2 Swapped 9/20/06 analog telem drifting | 1 Analog telem drifting |
| Recorders (DTR, SSR) | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B 4A Pressure leak | SSRs 1,2,3,4,5 Safestate=5B Current spikes |
| VTX (APT data) | #1 ON As of 19 June 07 | OFF / INOP since 11/15/00 | #2 ON 137.62 MHZ | #1 ON 137.1 MHZ As of 16 Aug 07 |
| BTX | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 2-Omni HRPT 2 Omni=rcp Power TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 Standby 3 low power INOP -10 Aug 2007 | 1 – HRPT TOAR424 low power 2 & 4 Playback STX-1 Power drop 2.29W 3/2/08 STX-4 nominal STX-3 Failed 2/14/08 | 3 - HRPT 1 - Playback 2 - Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, eclipse states and sun angle) | Array Off -45 Since 1/25/01 Full Sun Sun Angle 13 Uptrend to 18 early Jan 07 SA CANT=37 degs | Array Off -20 Since 9/13/07 Eclipse all year Sun Angle 34 Steady trend SA CANT=22 degs Full sun S/C in Sept 07 BVR Phase Control auton swap 7/19/07 | Array Off. –55 Since 12/18/08 Eclipse all year Sun Angle at 66 Steady trend SA CANT=37 degs BVR Phase Control auton swap 9/17/07 Swap back 9/18/07 SADPOS Enabled 2/12/08 PSE swap to back-up 11/3/08 | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 63 uptrend to 65 on 1 Jan 07 SA CANT=22 BVR Phase Control auton swap 3/29/06 & 1/4/07 |
| AVHRR | Nominal Channel 3B only Occasional minor scan motor instabilities | Nominal Recent SM current increases TCE24 OFF 4/14/04 Channel 3B only | ### Nominal 3A-B switch enabled Previous scan motor current mini surges | Nominal Channel 3B only Scan motor current drop 29 Aug 06 |
| AMSU-A1 | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise Channel 4 severely degraded 4 Mar 08 | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| `AMSU-A2 | Nominal | Nominal | Nominal | Nominal DIG-A Temps erratic 2/7/08 Cold Cal Pos 2 2/21/08 DIG-A Temps return to nominal 8/5/08 |
| AMSU-B | Operational Bias in All Chans. Degraded Chan-3 | Operational Space Position-3 Chan-8 degradation | Nominal | Replaced by MHS |
| ### HIRS | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | NOMINAL New all time high filter motor current reached 9/17/06. Imagery occasionally exhibits high levels of radiometric noise | NOMINAL 1 Pixel cross-track misalignment. | Operational Downgraded to red for products unusable Filter Heat OFF. LW channel 1-12 noise TOAR 448 Loose LWIR lens. LW imagery erratic. CH 1 Period Monitor event 13 May 08 |
| ### MHS/MIU |  |  |  | Nominal Fly Wheel Motor Current occasionally spiking since 27 Dec 06 Entered Motor Spin State on 9 May 08 H4 gain increased to 2-9 May 08 H3 gain increased to 3-9 May 08 H3/H4 QSB Instability latest 23 Dec 08 Automatic DC Offset Adjustment enabled 5 Sep 08 H3 DCO changed to 0x91-4 Dec 08, H4 DCO changed to 0x91-23 Dec 08 |
| DCS RFI investigation | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 inop/off |
| SARR | Operational B-Side Receivers & Converters Activated, 13 Dec 07 AGC Mode Intermittent failures for 243MHz A&B 1.21.5 & 243 MHz Off1 Feb 09 | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 121.5 & 243 MHz = INOP & OFF as of 29 Jul 08 PDS ATT set to 0dB, 406 MHz RCVR ATT set to 2dB on 5 Sept. 08 | Nominal A-side All AGC Mode 1.21.5 & 243 MHz Off1 Feb 09 | Nominal A-Side All AGC Mode Since 15 Aug 05 1.21.5 & 243 MHz Off 1 Feb 09 |
| SARP | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=7 P-CDEM-HVPS=7 11 Jan 0714:47z | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=4 27 June 06 16:00z | Nominal Elec-CH-HVPS=1 27 June 06 17:00z Prot-CH-HVPS=1 11 Jan 07 14:15z |
| SBUV |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops TOAR 463 sun glint IBSL test performed 12 Sept 06 Grating motor investigation | Operational Anode mode since 6 July 05 13:30z Electronics turnoff taken out of STESM safestate table TOAR 462 sun glint Chop Motor stalled 22 May 08 & 9 Nov 08. Thermal increase 1 to 1.5 C, 31 Dec 08, under investigation |
| SADPOS |  | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | TCE26 & 15H Telem valid TOAR 432 closed TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Spacecraft in thermal uptrend | Nominal TCE24 OFF 4/14/05 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 AMSU-A1 survival heaters on 10/29/03 | All TCE’s active HIRS filter heater turned OFF 9/28/06 |

COMMENTS/CHANGES:

MISC:

NOAA-15:	There were relatively no changes to the NOAA-15 bus or payload health situation during the past week.

The #2 gyro is still relatively quiet. Daily monitoring continues.

NDCS5V limit updated for TIP Drift.

NOAA-16: 	There was relatively no change in the performance of the NOAA-16 bus & payload subsystems during the past week.

NAIPSD1 limit updated for TIP Drift.

NOAA-17: 	There was relatively no change in the performance of the NOAA-17 bus & payload subsystems during the past week.

NXSU2PL1 limit updated for TIP Drift.

NOAA-18:	There was relatively no change in the performance of the NOAA-18 bus & payload subsystems during the past week.

MetOp-A:       There was relatively no change in the performance of the NOAA Instruments on MetOp-A spacecraft during the past week.

Trend Observations (NOAA Instruments)

- Based on the CVF Digital A daily, weekly and long term trend reports, all AMSU, AVHRR and HIRS parameters are nominal and stable.  All performance parameters are well within specification (e.g. NEdT, NEdN, NEdA and SNR), with the exception of AMSU-A1 Channel 7 NEdT which has exceeded the 0.25 K specifications and is currently at 0.43 K and steeply increasing (see Figures 2 & 3).  The AMSU HK and science trends are summarized each week in an Excel file available on the MetOp Data server under the following directory: /2009_CVF_data/CVF_Reports/AMSA_trend/AMSA_DATA_FILE

- The AVHRR Scan Motor Current continues it’s decreasing trend, as previously addressed as part of the Dec. 1-2 Annual Performance Review at the SOCC.  See Figure 4 for the daily averages, currently at 182 mAmps and still decreasing.

- The HIRS Filter Wheel re-synch period during this reporting period averaged between 97-98 seconds between resynchs.

Anomalies

AR10693, “AMSU-A1 Channel 7 NEdT”:  EUMETSAT has raised an anomaly report to document the increased noise in Channel 7.  Since mid December 2008, the increasing trend has become steeper and is currently at 0.43 K and climbing.  An ARB telecon was held on January 15.  The NOAA SOCC raised a SOAR on Jan. 23 (SOAR-METOP-A-0014), which should now allow NASA and Northrop Grumman to support with root cause investigations.

Operations Performed

- The SEM IFCs were run on their regular Tuesday schedule, with the TED IFC starting on Feb. 17 at 09:07:37 UTC and the MEPED IFC starting at 09:08:57 UTC.

- HRPT operations continued in toggling mode, operating on the HRPT B-side and only operating in northern hemisphere low risk zones during descending passes (i.e. over northern Europe and the northern Atlantic).

Operations Planned

The next SEM IFCs are planned for Tue., Feb.24.

Figure 2: MetOp-A AMSU-A1 Channel 7 NEdT

Figure 3: MetOp-A AMSU-A1 Channel 7 Gain

Figure 4: MetOp-A AVHRR Average Scan Motor Current