POLAR Spacecraft Status As of: Thurs 28 Feb 2008, 12:00 EST

(Changes from last week in bold border)

| Subsystem/ Component | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- |
| LAUNCHED | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU-2: TOAR472 | B-BUS XSU-2: TOAR 471 1/15/08 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO |
| ADACS | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB N2 Low PSI invalid | ###### RGYRO Mode Estimate Y-ROLL Rgmode=PASSIVE Gyros-1 & 3 Inop/Off Gyro chans XA-ZB Pitch wheel bias 2400 Gyro2 safe macro | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB GAS Enabled Dec 05 | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch TOAR 447 Z-momentum transition complete Pitch Torquer again active |
| RXO | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 2 (since 4/3/03) 3 channels inop | 2 since 4/18/05 (analog drifting?) | 2 Swapped 9/20/06 | 1 Analog telem drifting |
| Recorders (DTR, SSR) | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B 4A Pressure leak | SSRs 1,2,3,4,5 Safestate=5B Current spikes |
| VTX (APT data) | #1 ON As of 19 June 07 | OFF / INOP since 11/15/00 | #2 ON 137.62 MHZ | #1 ON 137.1 MHZ As of 16 Aug 07 |
| BTX | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 2-Omni HRPT 2 Omni=rcp Power TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 Restored -11 Oct 2007 3 low power INOP -10 Aug 2007 | 1 – HRPT TOAR424 low power 2 & 4 Playback STX-1 Power drop STX-4 nominal STX-3 Failed 2/14/08 | 3 - HRPT 1 - Playback 2 - Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, eclipse states and sun angle) | Array Off -45 Since 1/25/01 Full Sun Sun Angle 13 Uptrend to 18 early Jan 07 SA CANT=37 degs | Array Off -20 Since 9/13/07 Eclipse all year Sun Angle 34 Steady trend SA CANT=22 degs Full sun S/C in Sept 07 BVR Phase Control auton swap 7/19/07 | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 66 Steady trend SA CANT=37 degs BVR Phase Control auton swap 9/17/07 Swap back 9/18/07 SADPOS Enabled 2/12/08 | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 63 uptrend to 65 on 1 Jan 07 SA CANT=22 BVR Phase Control auton swap 3/29/06 & 1/4/07 |
| AVHRR | Nominal Channel 3B only Occasional minor scan motor instabilities | Nominal Recent SM current increases TCE24 OFF 4/14/04 Channel 3B only | ### Nominal 3A-B switch enabled Previous scan motor current mini surges | Nominal Channel 3B only Scan motor current drop 29 Aug 06 |
| AMSU-A1 | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| `AMSU-A2 | Nominal | Nominal | Nominal | Non-nominal DIG-A Temps erratic 2/7/08 Cold Cal Pos 2 2/21/08 |
| AMSU-B | Operational Bias in All Chans. Degraded Chan-3 | Operational Space Position-3 Chan-8 degradation | Nominal | Replaced by MHS |
| ### HIRS | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | NOMINAL New all time high filter motor current reached 9/17/06. Imagery occasionally exhibits high levels of radiometric noise | NOMINAL 1 Pixel cross-track misalignment. | Operational Filter Heat OFF. LW channel 1-12 noise TOAR 448 Loose LWIR lens. LW imagery erratic. |
| ### MHS/MIU |  |  |  | Nominal Fly Wheel Motor Current Spiking but not alarming since 27 Dec 06 Entered Motor Spin State on 17 Jan 08 H4 gain increased 2 Oct 07 H3 gain increased 31Oct 07 |
| DCS RFI investigation | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 inop/off |
| SARR | Operational B-Side, 13 Dec 07 AGC Mode Intermittent failures for 243MHz A&B | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 243 MHz = INOP. | Nominal A-side All AGC Mode | Nominal A-Side All AGC Mode Since 15 Aug 05 |
| SARP | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=7 P-CDEM-HVPS=7 11 Jan 0714:47z | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=4 27 June 06 16:00z | Nominal Elec-CH-HVPS=1 27 June 06 17:00z Prot-CH-HVPS=1 11 Jan 07 14:15z |
| SBUV |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops TOAR 463 sun glint IBSL test performed 12 Sept 06 Grating motor investigation | Nominal Anode mode since 6 July 05 13:30z Electronics turnoff taken out of STESM safestate table TOAR 462 sun glint |
| SADPOS |  | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | TCE26 & 15H Telem valid TOAR 432 closed TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Spacecraft in thermal uptrend | Nominal TCE24 OFF 4/14/05 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 AMSU-A1 survival heaters on 10/29/03 | All TCE’s active HIRS filter heater turned OFF 9/28/06 |

COMMENTS/CHANGES:

MISC:	Due to NOAA-17 STX-3 failure, NOAA-18 & 17 STX configurations will be changed per the table below on 26 Feb 2008:

| Spacecraft | HRPT Broadcast | Playback transmitters | Time of change |
| --- | --- | --- | --- |
| NOAA-17* | STX-1 1698.0 Mhz | STX-2 STX-4 | Rev:29485 Time:01:01:15z |
| NOAA-18* | STX-3 1707.0 Mhz | STX-1 STX-2 | Rev:14265 Time:00:35:30z |
| NOAA-15 | STX-2 (OMNI) 1702.5 Mhz | STX-4 | No change |
| NOAA-16 | STX-2 1702.5 Mhz/LCP | STX-1 | No change |

NOAA-15:	There were relatively no changes to the NOAA-15 payload health situation during the past week.

NOAA-15 IMU status is unchanged. Daily monitoring continues.

NOAA-16: 	There was relatively no change in the performance of the NOAA-16 payload or bus subsystems during the past week.

NOAA-17: 	There was relatively no change in the performance of the NOAA-17 payload or bus subsystems during the past week.

NOAA-18:	There were relatively no changes to the NOAA-18 health situation during the past week.

MetOp-A:      There were relatively no changes to the MetOp-A NOAA instruments health situation during the past week.

The AVHRR spikes in the space view and back scan counts occurred again on Feb. 24,

but these have been confirmed to be caused by the moon contaminating the space view.

This has been occurring on 29.5 day centers since launch (generally 2 days after full moon),

but was first documented by AR90