POLAR Spacecraft Status As of: Tues, 16 May 06, 16:00 EST

(changes from last week in bold border)

| Subsystem/ Component | NOAA-12D AM 16:59=AN STIP data only | NOAA-14J AM: 21:10=AN STIP data only | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC&LAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- | --- | --- |
| LAUNCHED | 05/14/91 | 12/30/94 | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 Recursive SCT | OBP2 OBP1 inoperative | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU2 (XSU1 inop) | B-BUS XSU-1 | B-BUS XSU-1: TOAR444 | B-BUS XSU-1 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO | NO | NO |
| ADACS | Nominal Mode RGmode=Monitor Gyro chans=XYZ | Nominal Mode Rgmode=Disabled Gyro chans=XYZ | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB | ###### RGYRO Mode Estimate Y-ROLL Rgmode=PASSIVE Gyros-1 & 3 Inop/Off Gyro chans XA-ZB | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch |
| RXO | PRI BU random enables daily bias+6.5 mS | PRI BU random enables daily bias–7.0 mS | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase Enabled | External Synch Rephase each orbit | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 1 | 1 | 2 (since 4/3/03) #2 has 3 chans inop | 2 since 4/18/05 | 1 Analog telem drifting | 1 Analog telem drift? |
| Recorders (DTR, SSR) | DTRs 3A, 4, 5B Safestate 5B | DTRs 1, 2B, 5 Safestate- 1B | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B | SSRs 1,2,3,4,5 Safestate=5B Current spikes TOAR 450 |
| VTX (APT data) | #1 ON 137.50 MHZ | OFF (nominal) Since 8/2/05 | #1 ON 137.50 MHZ | OFF / INOP since 11/15/00 | #2 ON 137.62 MHZ | #1 ON 137.10 MHZ |
| BTX | #1 136.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 1 – HRPT; 2,3 - Playback | 3 – HRPT 2, 1 – Playback | 2-Omni HRPT 2 omni=rcp Power Drop TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 – Playback & MCM usage 3 low power unused | 3 – HRPT TOAR424 low power 2 & 4 Playback 1-Standby STX-1 Power drop TOAR 443. | 1 - HRPT 3 - Playback 2-Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, charge & V/T rates, eclipse states and sun angle) | ArrayOff +60 since 6/23/05 Eclipse season started 29 Apr SADBIAS activated 29Apr Sun Angle 26 uptrend to 35 in mid-June Batts 1&2=LRC Batt 1&2 V/T=4 Shunt degraded SA CANT=37 degs | Array Off -50 Batts 1&3V/T=6/9 Batt2 VT=7/9 Since 11/22/05 Reduced load 5 May with SSU turnoff Eclipse all year Sun angle 55 up trend to 60 in June Batts 1-3= LRC Degraded shunts SA CANT=22 degs PSSS change plan | Array Off -45 Since 1/25/01 Eclipse season started 15 May Peaks mid-Jun Ends 1 Aug Sun Angle 21 New uptrend to 32 in mid-June Batts 1-3= LRC V/T=7/9 SA CANT=37 degs | Array Off -40 Since 7/16/02 Eclipse all year Sun Angle 47 Up Trend to 52 in mid-June Batts 1-3= LRC V/T=4/8 SA CANT=22 degs | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 69 uptrend to 71 at end of May Batts 1-3= LRC V/T= 4/8 SA CANT=37 degs | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 67 up trend to 72 mid-Jul Batts 1-3=LRC Batts 1-3 V/T=4/8 SA CANT=22 BVR Phase Control autonomous swap 3/29=TOAR 457 |
| AVHRR | Nominal | Nominal Since 18 Feb 06 Occasional scan motor current surge Channels 2&4 APT | Nominal Channel 3B only Recent minor scan motor instabilities | Nominal since mid Oct 05 TCE24 OFF 4/14/04 Channel 3B only | ### Nominal 3A-B switch enabled Scan motor stability degradation? | Nominal Channel 3B only |
| AMSU-A1 |  |  | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| AMSU-A2 |  |  | Nominal | Nominal Space Position-2 | Nominal | Nominal Full scan mode |
| AMSU-B |  |  | Operational Bias in All Chans. Motor surges most recent 11/14/04 Degraded Chan-3 since 4/30/05 | Operational Space Position-3 IPD reports channel 8 degradation | Nominal | Replaced by MHS |
| ### HIRS | ON/Inop Turned On 4/30/05 for power balance Filter Wheel OFF | NOMINAL Operational Occasional loss of Fwheel synch | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | Operational Imagery high levels radiometric noise TOAR 441 CLOSED Recent momentary FW synch dropouts Occasional major FM surges, most recent 12/9/04 | NOMINAL 1 Pixel cross track misalignment. | Operational LW channel 1-12 noise TOAR 448 Loose LWIR lens. Inconsistent, LW often erratic. |
| ### MHS/MIU |  |  |  |  |  | Nominal Space View=0 Two Turbo anomalies Caused by proton hits over SAA |
| MSU | OFF/Inop Turned off for power balance 6/23/05 | Operational Scan motor and antenna problems EOL test planned |  |  |  |  |
| SSU |  | OFF/Nominal Turned off 5 May for power balance |  |  |  |  |
| DCS RFI investigation | Nominal | Nominal | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 inop/off |
| SARR |  | Nominal All time high component temps | Operational A-Side, AGC Mode Intermittent failures for 243MHz A&B | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 243 MHz = INOP. | Nominal A-side All AGC Mode | Nominal A-Side All AGC Mode Since 15 Aug 05 |
| SARP |  | OFF Power Failure | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | ON/Nominal Turned on 8/30/04 Elec-CH-HVPS=4 Prot-CH-HVPS=0 TED level=3 | Operational Elec-CH-HVPS=0 Prot-CH-HVPS=0 1 of 4 Tscopes inop | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=5 P-CDEM-HVPS=4 | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=3 | Nominal Elec-CH-HVPS=0 Prot-CH-HVPS=0 |
| SBUV |  | Degraded Grate motor sticks Flex memory 3 (giant step) ops suspended 12/2/03 Reduced operations Transient anomaly 1 Sept 05 |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops Procedure changes forthcoming | Nominal Anode mode since 6 July 13:30z Electronics turnoff taken out of STESM safestate table |
| SADPOS |  |  | N/A | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | Nominal TCE’s back to nominal configuration following 7/25/04 UVtrip. | Nominal | TCE26 & 15H Telem invalid TOAR 432 TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Spacecraft thermal uptrend ended | Nominal TCE24 OFF 4/14/05 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 AMSU-A1 survival heaters on 10/29/03 | All TCE’s active TCE3H displaying invalid readings TOAR 458 submitted |
|  | N12 | N14 | N15 | N16 | N17 | ### N18 |

COMMENTS/CHANGES:

NOAA-12:	Eclipse season is progressing nominally and there has been no repetition of the autonomous SADBIAS implementation

that occurred on 29 Apr.  On 10 May at 20:10z the SADBIAS value was commanded back to 80 deg offset (where it should be, vs. 54 deg where it actually was) after which SADBIAS was re-enabled for the first time since the 29 Apr event.

NOAA-14:	N14, batteries continue to trend lower (illustration 1) during the dark part of their orbit, again falling slightly below 20

volts, despite Engineers turned off the nominal and operational SSU payload on 5 May, in order to conserve power and prevent a potential power survival safestate situation.

The N14 SARR continued to operate at or near all time high temperatures during the past week (illustration 2).  There is presently no health related concerns that have been identified

NOAA-15:	The N15 AMSU’s A1 and A2 were generally nominal during the last week while continuing their thermal uptrends.

On the other hand AMSU-B continued to exhibit slightly less stable scan motor current performance (and trends) during the past week although it remains within historically acceptable norms (illustration 3).  The N15 AVHRR also showed slightly less than optimum scan motor stability during the past week although it too remains within historically acceptable norms (illustration 4).  There have been no reports received of degraded products from either of these situations.

N15 began its eclipse season on 15 May as the first battery discharge currents were registered.  At the present time no reconfigurations are planned and the eclipse is proceeding as predicted (illustrations 5 and 6).

The N15 Attitude Control Subsystem was also generally nominal and stable last week (illustrations 7 and 8).

NOAA-16:	The N16 payload suite was also generally nominal and stable during the past week (illustration 9) with the

AVHRR sustaining its historical low scan motor current and all other performing in a nominal and stable manner.  Thermally N16 remains stable with no discernable trend.  The N16 ADACS subsystem was also nominal during the past week (illustrations 10 and 11) relative to its severely degraded status although there were some minor perturbations on 13 and 14 May due to a full moon and sun-moon warning events.

NOAA-17:	The N17 AVHRR scan motor continues to exhibit less stability than engineers prefer but also continues to produce nominal products.  The scan motor current “mini surges” continue to increase in frequency but the value remains within an acceptable norm.  Changes to the synch delta value generally mirrors the scan motor current changes  Engineers continues to monitor this payload closely.  In addition, the N17 HIRS showed slightly less stability on the past several days as the filter motor current increased slight and the period monitor value range followed suit.  Otherwise the N17 payload suite was nominal during the past week (illustration 12).

NOAA-18:	Most of the N18 payload suite was nominal and stable during the past week (illustration 15) except for the HIRS which

started to degrade on 8 May and has remained that way since that time.  Furthermore, for the first time, the filter housing temperatures appeared to show slightly less stability by suddenly rising slightly in conjunctions with a slight rise in filter motor current (illustration 16).  HIRS imagery remains degraded in numerous channels at the present time (illustration 17).  The payload remains Yellow in status with no new tests or reconfigurations under consideration at this time

The N18 ADACS was nominal during the past week (illustrations 18 and 19) and the TCE3H heater continued its abnormal telemetry reporting which resulted in TOAR 458 being submitted two weeks previous.  Ultimately, there were no changes to N18 during the past week.

Illustration 1:  NOAA-14 Power Analysis

Battery voltages continue dipping below 20 Volts in darkness despite turning off SSU.  Otherwise power subsystem was relatively nominal during the past week. )

Illustration 2: N14 SARR Temps

(Running at or near all time highs on numerous components. Not considered a serious concern at these levels.)

Illustration 3:  N15 AMSU Health Analysis

(AMSU-A1 and A2 were both nominal and stable last week although their thermal uptrend’s continued.  AMSU-B continued to run at slightly higher than usual scan motor currents.  Note the jump in AMSU-B motor current coinciding with that of its component temps on JDAY 130 [10 May, red x.])

Illustration 4:  N15 AVHRR Health Analysis.

(Scan motor continues to trend in an inauspicious direction, as does synch delta although both remain within historically acceptable ranges.)

Illustration 5:  N15 Power Analysis

(Eclipse season is underway.)

Illustration 6:  N15 Battery Analysis

(Starting to show the effects of the new eclipse season.)

Illustration 7,8:  omitted for web release

Illustration 9:  N16 Primary Payloads Health Analysis

(Nominal and stable last week.)

ILLUSTRATION 10, 11:  omitted for web release.

ILLUSTRATION 12:  N17 Primary Payload Health Analysis

(AVHRR instabilities continue, although it has not gotten any worse [middle plots].  HIRS instability seems to have recently started anew [top plots] with no reports of degraded imagery.  AMSU’s look nominal.)

ILLUSTRATION 13, 14: omitted for web release.

ILLUSTRATION 15:  N18 Primary Payload Motor Current Analysis

(Nominal and stable last week.)

ILLUSTRATION 16:  N18 HIRS Health Analysis

(Hardware was nominal and stable last week with the possible exception o the filter housing temps which appeared to have trended up [middle right plots] in conjunction with the slight rise in filter motor current [bottom right plot]. Long wave channels [both top plots] were degraded all week.

ILLUSTRATION 17:  N18 HIRS Imagery Analysis

(Imagery again degraded red X shows channels with obvious streak degradation.)

ILLUSTRATION 18, 19:  omitted for web release.

# POLAR SPACECRAFT TIP CLOCK ERROR

## WEEK ENDING May 12, 2006

| DAY/DATE | NOAA-12 | NOAA-14 | NOAA-15 | NOAA-16 | NOAA-17 | NOAA-18 |
| --- | --- | --- | --- | --- | --- | --- |
| FRI. 05/05 | +375 msec | -135 msec | +016 msec | -010 msec | -130 msec | -118 msec |
| SAT. 05/06 | +370 msec | -134 msec | +017 msec | -009 msec | -130 msec | -116 msec |
| SUN. 05/07 | +372 msec | -134 msec | +015 msec | -011 msec | -127 msec | -115 msec |
| MON. 05/08 | +369 msec | -130 msec | +011 msec | -008 msec | -126 msec | -113 msec |
| TUE. 05/09 | +368 msec | -129 msec | +010 msec | -008 msec | -126 msec | -113 msec |
| WED. 05/10 | +368 msec | -132 msec | +011 msec | -008 msec | -125 msec | -111 msec |
| THU. 05/11 | +367 msec | -128 msec | +012 msec | -007 msec | -123 msec | -109 msec |

NOAA-14:  As of 12/23/03 the TIP clock is corrected daily by –7.0 msec to compensate for drift.

NOAA-15:  As of 04/22/03 the TIP clock is corrected daily by –8.0 msec to compensate for drift.

NOAA-16:  As of 12/23/03 the TIP clock is corrected daily by -5.5 msec to compensate for drift.

NOAA-17:  As of 12/23/03 the TIP clock is corrected daily by –3.0 msec to compensate for drift.

NOAA-12:  As of 12/10/03 the TIP clock is corrected daily by +6.5 msec to compensate for drift.

*    A clock jump of -500 milliseconds on 02/14/06 due to fractional second daily ETCUP. A clock

jump of +500 milliseconds on 05/01/06 due to fractional second daily ETCUP. Alternating +/-

500 millisecond clock jumps will ccur at 76-day intervals as long as fractional second daily

ETCUP is utilized to manage clock drift. The next -500 millisecond jump is expected on 07/16/06.

**  TIP Clock corrections were made to compensate for the positive leap second introduced on 12/31/05.