POLAR Spacecraft Status As of: Tues 31 July 07, 17:00 EST

(Changes from last week in bold border)

| Subsystem/ Component | NOAA-12D AM 16:59=AN STIP data only No IPD processing | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC&LAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- | --- |
| LAUNCHED | 05/14/91 | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 Recursive SCT | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU-1 | B-BUS XSU-1: TOAR444 | B-BUS XSU-1 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO | NO |
| ADACS | Nominal Mode Autonomous swap to YGC 30 Aug 06, commanded back to nominal 31 Aug RGmode=Monitor Gyro chans=XYZ | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB N2 Low PSI invalid | ###### RGYRO Mode Estimate Y-ROLL Rgmode=PASSIVE Gyros-1 & 3 Inop/Off Gyro chans XA-ZB Pitch wheel bias 2400 Gyro2 safe macro | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB GAS Enabled Dec 05 | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch TOAR 447 Z-momentum transition complete Pitch Torquer again active |
| RXO | PRI BU random enables daily bias+6.5 mS | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase Enabled | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 1 | 2 (since 4/3/03) 3 channels inop | 2 since 4/18/05 (analog drifting?) | 2 Swapped 9/20/06 | 1 Analog telem drifting |
| Recorders (DTR, SSR) | DTRs 3A, 4, 5B Safestate 5B 4b low pressure | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B 4A Pressure leak | SSRs 1,2,3,4,5 Safestate=5B Current spikes |
| VTX (APT data) | #1 ON 137.50 MHZ | #1 ON As of 19 June 07 | OFF / INOP since 11/15/00 | #2 ON 137.62 MHZ | #2 ON 137.9125 MHZ |
| BTX | #1 136.77 MHZ | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 1 – HRPT; 2,3 - Playback | 2-Omni HRPT 2 Omni=rcp Power TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 – Playback & MCM usage 3 low power unused | 3 – HRPT TOAR424 low power 2 & 1 Playback STX-1 Power drop STX-4 no modulation | 1 - HRPT 3 - Playback 2-Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, eclipse states and sun angle) | ArrayOff -60 since 5/23/07 Full Sun Sun Angle 16 uptrend to 20 in early Jan 07 Shunt degraded SA CANT=37 degs PSSS trigger =18.5 | Array Off -45 Since 1/25/01 Full Sun Sun Angle 13 Uptrend to 18 early Jan 07 SA CANT=37 degs | Array Off -40 Since 7/16/02 Eclipse all year Sun Angle 34 Steady trend SA CANT=22 degs Transitioning to full sun S/C in Sept 07 BVR Phase Control auton swap 7/19/07 | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 66 Steady trend SA CANT=37 degs | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 63 uptrend to 65 on 1 Jan 07 SA CANT=22 BVR Phase Control auton swap 3/29/06 & 1/4/07 |
| AVHRR | Nominal HRPT and APT data only | Nominal Channel 3B only Occasional minor scan motor instabilities | Nominal Recent SM current increases TCE24 OFF 4/14/04 Channel 3B only | ### Nominal 3A-B switch enabled Previous scan motor current mini surges | Nominal Channel 3B only Scan motor current drop 29 Aug 06 |
| AMSU-A1 |  | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| `AMSU-A2 |  | Nominal | Nominal | Nominal | Nominal |
| AMSU-B |  | Operational Bias in All Chans. Degraded Chan-3 | Operational Space Position-3 Chan-8 degradation | Nominal | Replaced by MHS |
| ### HIRS | ON/Inop Turned On 4/30/05 for power balance Filter Wheel OFF | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | NOMINAL New all time high filter motor current reached 9/17/06. Imagery occasionally exhibits high levels of radiometric noise | NOMINAL 1 Pixel cross-track misalignment. | Operational Filter Heat OFF. LW channel 1-12 noise TOAR 448 Loose LWIR lens. LW imagery erratic. |
| ### MHS/MIU |  |  |  |  | Nominal Fly Wheel Motor Current Spiking but not alarming since 27 Dec 06 Entered Motor Spin State on 27 March 07 |
| MSU | ON/Inop Turn on 7/9/07 for power balance |  |  |  |  |
| SSU |  |  |  |  |  |
| DCS RFI investigation | Nominal | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 inop/off |
| SARR |  | Operational A-Side, AGC Mode Intermittent failures for 243MHz A&B | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 243 MHz = INOP. | Nominal A-side All AGC Mode | Nominal A-Side All AGC Mode Since 15 Aug 05 |
| SARP |  | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | ON/Nominal Autonomous reset 8 Jun 06, back to nominal 10 June Turned on 8/30/04 Elec-CH-HVPS=4 Prot-CH-HVPS=0 TED level=3 | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=6 P-CDEM-HVPS=5 27 Jun 0615:30z | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=4 27 June 06 16:00z | Nominal Elec-CH-HVPS=1 27 June 06 17:00z Prot-CH-HVPS=0 |
| SBUV |  |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops TOAR 463 sun glint IBSL test performed 12 Sept 06 Grating motor investigation | Nominal Anode mode since 6 July 05 13:30z Electronics turnoff taken out of STESM safestate table TOAR 462 sun glint |
| SADPOS |  |  | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | Nominal TCE’s back to nominal configuration following 7/25/04 UVtrip. | TCE26 & 15H Telem valid TOAR 432 closed TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Spacecraft in thermal uptrend | Nominal TCE24 OFF 4/14/05 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 AMSU-A1 survival heaters on 10/29/03 | All TCE’s active HIRS filter heater turned OFF 9/28/06 |

COMMENTS/CHANGES:

NOAA-12:	There were relatively no changes to the rest of the NOAA-12 health situation during the past week.  MSU power balance appears to continue to hold stable at the motor's turn on temperature of 70 C.

SADBIAS was triggered due to the regulated filtered bus voltage exceeding 28.5 volts.  NOAA-12’s delicate power balance is being investigated.

NOAA-15:	There were relatively no changes to the NOAA-15 payload health situation during the past week.

The Gyro-2 has been showing signs of instability.  It has shown this type of behavior in the past.

NOAA-16: 	There was relatively no change in the performance of the NOAA-16 payload subsystems during the past week and the rest of the spacecraft subsystems were also nominal during this time.  NOAA-16’s ETCUP biases were changed on 31 July 2007.  It will take a few days to notice the TIP clock drift rate changes.

The BVR Phase Controller switched is still being investigated.

NOAA-16 will be in full sunlight beginning in mid-September.  This is the first time ever it will not be eclipsing.

NOAA-17: 	There was relatively no change in the performance of the NOAA-17 payload subsystems during the past week.  NOAA-17’s ETCUP biases were changed on 31 July 2007.  It will take a few days to notice the TIP clock drift rate changes.

No updates or conclusion were reached about the Attitude disturbance on 17 July 2007.  The investigation continues.

NOAA-18:	There were relatively no changes to the NOAA-18 health situation during the past week.  DDR4 Current spikes YH violations were noted; its YH limit was raised to within 2 counts of the RH.  This happened on DDR3 last month with the same limit adjustment.  All DDRs are seeing the current increases.

The near future VTX will be switched from side 2 to 1.  We are waiting for EUMETSAT feedback to coordinate the date of this event.

MetOp A:	No significant changes reported on the NOAA instruments on MetOp.  The HRPT transmitter failure investigation continues.

NOTES:	All figures for N-15 Gyro-2 instability and N-18 DDRs current increases have been omitted for web listing.