POLAR Spacecraft Status As of: Tues, 12 Sept 06, 16:00 EST

(changes from last week in bold border)

| Subsystem/ Component | NOAA-12D AM 16:59=AN STIP data only | NOAA-14J AM: 21:10=AN STIP data only | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC&LAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- | --- | --- |
| LAUNCHED | 05/14/91 | 12/30/94 | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 Recursive SCT | OBP2 OBP1 inoperative | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU2 (XSU1 inop) | B-BUS XSU-1 | B-BUS XSU-1: TOAR444 | B-BUS XSU-1 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO | NO | NO |
| ADACS | Nominal Mode Autonomous swap to YGC 30 Aug, | Nominal Mode Rgmode=Monitor Gyro chans=XYZ | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB | ###### RGYRO Mode Estimate Y-ROLL Rgmode=PASSIVE Gyros-1 & 3 Inop/Off Gyro chans XA-ZB | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch TOAR 447 |
| RXO | PRI BU random enables daily bias+6.5 mS | PRI BU random enables daily bias–7.0 mS | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase Enabled | External Synch Rephase each orbit | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 1 | 2 Since 19 Jul 06 | 2 (since 4/3/03) 3 channels inop | 2 (drifting?) since 4/18/05 | 1 Analog telem drifting Swap planned 9/20/06 | 1 Analog telem drifting |
| Recorders (DTR, SSR) | DTRs 3A, 4, 5B Safestate 5B | DTRs 1, 2B, 5 Safestate- 1B | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B | SSRs 1,2,3,4,5 Safestate=5B Current spikes TOAR 450 |
| VTX (APT data) | #1 ON 137.50 MHZ | OFF (nominal) Since 8/2/05 | #1 ON 137.50 MHZ | OFF / INOP since 11/15/00 | #2 ON 137.62 MHZ | #2 ON 137.9125 MHZ Investigating output power instability |
| BTX | #1 136.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 1 – HRPT; 2,3 - Playback | 3 – HRPT 2, 1 – Playback | 2-Omni HRPT 2 Omni=rcp Power Drop TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 – Playback & MCM usage 3 low power unused | 3 – HRPT TOAR424 low power 2 & 4 Playback 1-Standby STX-1 Power drop TOAR 443. | 1 - HRPT 3 - Playback 2-Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, charge & V/T rates, eclipse states and sun angle) SUN-MOON ECLIPSE 22 SEPT BETWEEN 08:00 AND 16:00Z | ArrayOff +60 since 8/31/06 Full Sun Sun Angle 17 trend down to 9 in Oct Batts 1&2=LRC Batt 1&2 V/T=4 Shunt degraded SA CANT=37 degs PSSS trigger =18.5 | Array Off -50 Since 22Nov05 Batts 1&3V/T=6/9 Batt2 VT=7/9 Eclipse all year Sun angle at 57 steady trend Batts 1-3= LRC Degraded shunts SA CANT=22 degs PSS Macro loaded | Array Off -45 Since 1/25/01 Full Sun Sun Angle 17 downtrend to 5 in mid-Oct Batts 1-3= LRC V/T=7/9 SA CANT=37 degs | Array Off -40 Since 7/16/02 Eclipse all year Sun Angle 40 Down trend to 33 in late Oct Batts 1-3= LRC V/T=4/8 SA CANT=22 degs | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 67 Steady trend Batts 1-3= LRC V/T= 4/8 SA CANT=37 degs | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 65 downtrend to 60 on 1 Nov 06 Batts 1-3=LRC Batts 1-3 V/T=4/8 SA CANT=22 BVR Phase Control auton swap 3/29/06 |
| AVHRR | Nominal | Nominal Since 18 Feb 06 Occasional scan motor current surge Channels 2&4 IR channels re-enabled 14-Aug-06 | Nominal Channel 3B only Recent minor scan motor instabilities | Nominal since mid Oct 05 TCE24 OFF 4/14/04 Channel 3B only | ### Nominal 3A-B switch enabled Frequent scan motor current mini surges | Nominal Channel 3B only Scan motor current drop 29 Aug 06 |
| AMSU-A1 |  |  | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| AMSU-A2 |  |  | Nominal | Nominal Space Position-2 | Nominal | Nominal Full scan mode |
| AMSU-B |  |  | Operational Bias in All Chans. Motor surges most recent 11/14/04 Degraded Chan-3 since 4/30/05 | Operational Space Position-3 IPD reports channel 8 degradation | Nominal | Replaced by MHS |
| ### HIRS | ON/Inop Turned On 4/30/05 for power balance Filter Wheel OFF | NOMINAL Operational Occasional loss of Fwheel synch Electronics turned back on 14-Aug-06 after turnoff for 10 Aug POM test | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | NOMINAL Imagery occasionally exhibits high levels of radiometric noise Recent momentary FW synch dropouts Occasional major FM surges, most recent 12/9/04 | NOMINAL 1 Pixel cross track misalignment. | IWT Test Mode Lens stabilization test #1 implemented 9/11/06. Mirror placed at IWT LW channel 1-12 noise TOAR 448 Loose LWIR lens. Inconsistent, LW often erratic. |
| ### MHS/MIU |  |  |  |  |  | Nominal Space View=0 Occasional Flywheel motor current surges most recent 17Jul 06 Rare Turbo anomalies over SAA Most recent 12/16/05 |
| MSU | OFF/Inop Turned off for power balance 6/23/05 | Operational Scan motor and antenna problems |  |  |  |  |
| SSU |  | OFF/Nominal Turned off 5 May for power balance |  |  |  |  |
| DCS RFI investigation | Nominal | Nominal | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 inop/off |
| SARR |  | Nominal All time high component temps | Operational A-Side, AGC Mode Intermittent failures for 243MHz A&B | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 243 MHz = INOP. | Nominal A-side All AGC Mode | Nominal A-Side All AGC Mode Since 15 Aug 05 |
| SARP |  | OFF Power Failure | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | ON/Nominal Autonomous reset 8 Jun, back to nominal 10 June Turned on 8/30/04 Elec-CH-HVPS=4 Prot-CH-HVPS=0 TED level=3 | Operational Elec-CH-HVPS=0 Prot-CH-HVPS=0 1 of 4 Tscopes inop | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=6 P-CDEM-HVPS=5 27 Jun 15:30z | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=4 27 June 16:00z | Nominal Elec-CH-HVPS=1 27 June 17:00z Prot-CH-HVPS=0 |
| SBUV |  | Degraded Grate motor sticks Flex memory 3 (giant step) ops suspended 12/2/03 Reduced operations Transient anomaly 1 Sept 05 |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops TOAR 463 sun glint IBSL test planned for 12 Sept | Nominal Anode mode since 6 July 13:30z Electronics turnoff taken out of STESM safestate table TOAR 462 sun glint IBSL Testing implemented 9/12/06 |
| SADPOS |  |  | N/A | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | Nominal TCE’s back to nominal configuration following 7/25/04 UVtrip. | Nominal | TCE26 & 15H Telem invalid TOAR 432 TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Spacecraft thermal in downtrend | Nominal TCE24 OFF 4/14/05 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 AMSU-A1 survival heaters on 10/29/03 | All TCE’s active HIRS filter heater turned ON 9/11/06 TCE3H displaying invalid readings TOAR 458 submitted |
|  | N12 | N14 | N15 | N16 | N17 | ### N18 |

COMMENTS/CHANGES:

NOAA-12:	No changes since last weeks report.  Spacecraft is operating in nominal attitude mode with nominal power balance and

no residual effects from the autonomous swap to YGC mode and the solar array offset move two weeks ago.

NOAA-14:	The N14 AVHRR has begun to exhibit some scan motor current instabilities in recent days with two, significant though short-lived surges occurring in the past week (illustration 1).  These surges were not accompanied by any changes in other AVHRR telemetry (temperatures, synch deltas, etc) and there has been no reports of degraded imagery received by EMOSS engineers.  Otherwise, there was no change to N14 during the past week with all other subsystems performing in a nominal and stable manner and the power system remaining well within balance (illustration 2).

NOAA-15:	There were no changes to N15 during the past week as all payloads, power and attitude control subsystems performed

nominally while in the middle of a strong thermal downtrend that is expected to last until mid October (illustrations 3 through 5).

NOAA-16:	There were no significant changes to N16 during the past week as all payloads, power and attitude control subsystems

performed nominally with a steady thermal trend (illustrations 6).  The attitude control system did experience some minor instabilities on 8 and 9 Sept when a full moon introduced light into ESA quads 2&4 which could not be knocked out because of the Rgyro attitude control mode situation.  However, these instabilities only last two days and the situation has since resolved itself (illustrations 7 & 8).

NOAA-17:	The N17 AMSU’s (A2 and B) were generally nominal and stable last week while the AVHRR experienced almost

continual scan motor current “mini surges” (all within the “normal” range for such events, see illustration 9).  Otherwise the remainder of the spacecraft was relatively nominal, stable and unchanged during the past week (illustration 10).

EMOSS engineers continue planning the N17 TIP swap because of its analog drift problem.  The current planning date for this change has been selected as 20 Sept.  Will Chadwick and Jon Woodward are EMOSS POC’s for this reconfiguration.

EMOSS engineers implemented N17 SBUV IBSL tests on 12 Sept starting at 15:09z which were partially executed out of stored commands and partially out of realtime commanding.  Details of this test were specified in the 29 Aug Weekly Report.  Sally House is EMOSS POC.

NOAA-18:	N18 HIRS lens stabilization testing began on 11 Sept when the mirror was placed in IWT position at 16:00z and the

filter heater was turned on one orbit later (illustrations 11 and 12).  This configuration is expected to remain in place at least through 13 Sept after which additional testing may be implemented pending analysis by NASA and factory engineers.  While in this configuration, HIRS imagery products are not available to EMOSS engineers nor is the channel noise data that typically appears in this report.  Sally House is EMOSS POC for this testing which was also outlined in the 29 Aug Weekly Report.  Preliminary reports from Changyong Cao indicate that the noise dropped significantly during the IWT Dwell (see illustration 13).

The N18 AVHRR is performing nominally following a noticeable drop in its scan motor current last week.  No imagery

degradation or peripheral AVHRR affects have been noted since that occurrence (illustration 14).  Otherwise, all other

payloads and subsystems were nominal for N18 during the last week (illustrations 15 and 16).  One additional item of

note was the N18 VTX transmitter power.  Engineer John Vollmer, replacing Will Chadwick as POES DHS-Comm

engineers noticed a significant difference in the stability of the N18 VTX-2 transmitter output power (illustration 17) relative to that of N18 VTX-1 or the VTX transmitters on N15 and 17 (N16 VTX is inoperative and N14 has been off for years.)  This stability has actually improved somewhat since the VTX—2 was turned back on in late July 06 after having been off for 7 months prior.  Subsequent coordination of this finding with NASA engineers has led to the conclusion that this is not presently a cause for concern but is worthy of closer scrutiny.

NOTES: Illustrations 2, 5, 7-10, 16 and 17 have been omitted for web hosting.

Illustration 1:  NOAA-14 AVHRR Health Analysis

(The scan motor [lower right plot] is showing some signs of instability in recent days but no other item is coinciding with this instability. )

HIRS							AVHRR

Illustration 3:  NOAA-15 HIRS and AVHRR Health Analysis

(Nominal last week, as thermal trend continues downward.)

Illustration 4:  N15 AMSU Analysis

(Nominal and stable last week while continuing its thermal downtrend.  The minor instabilities on the AMSU-B scan motor current value [upper right plot] appear to have stabilized.)

Illustration 6:  N16 HIRS and AVHRR Health Analysis

(Nominal and stable last week with the HIRS filter motor current [lower right plot] actually showing some minor improvements.  All three AMSU’s, not depicted in this report, were also nominal and stable last week.)

HIRS								AVHRR

ILLUSTRATION 9:  N17 HIRS & AVHRR Health Analysis

(AVHRR scan motor mini surges are increasing in frequency.  No impacts have been detected as yet.  AMSU’s-2 and B, not depicted this week were both nominal and stable last week.)

ILLUSTRATION 11:  N18 HIRS Analysis

(Lens stabilization test #1 implemented late on Jday 254 [11 Sept].  Positioned mirror at IWT at 16:00z then one orbit later turned on filter heater.  The effects are obvious.  Imagery no longer being telemetered.)

ILLUSTRATION 12:  N18 HIRS DIG-A Data Analysis

(Period monitor and filter wheel synch remained nominal and stable after lens test was implemented.  All channels now no longer being telemetered [see middle plot] into a form that can be reviewed directly by EMOSS engineers.)

ILLUSTRATION 13:  N18 HIRS Test Data Analysis

(Changyong Cao slide depicting improvement in HIRS channel-2 noise situation with the implementation of IWT test.)

ILLUSTRATION 14:  N18 AVHRR and AMSU Health Analysis

(Nominal and stable last week.)

ILLUSTRATION 15:  N18 MHS Analysis

(Ignoring all the ABE system induced spikes, MHS was nominal and stable last week.)