POLAR Spacecraft Status As of: Thurs 23 Oct 2008, 12:00 EST

(Changes from last week in bold border)

| Subsystem/ Component | NOAA-15K AM: 17:49=AN GAC data only | NOAA-16L PM: 15:04=AN GAC data | ##### NOAA-17M AM: 22:23=AN GAC&LAC data | ##### NOAA-18N PM 13:49=AN GAC & LAC data |
| --- | --- | --- | --- | --- |
| LAUNCHED | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU-2: TOAR472 | B-BUS XSU-2: TOAR 471 1/15/08 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO |
| ADACS | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB N2 Low PSI invalid | ###### RGYRO Mode Estimate Y-ROLL Rgmode=PASSIVE Gyros-1 & 3 Inop/Off Gyro chans XA-ZB Pitch wheel bias 2400 *Z gas unload = 145 Gyro2 safe macro | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB GAS Enabled Dec 05 *Z Gas unload =45 | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch TOAR 447 Z-momentum transition complete Pitch Torquer again active Gas auto unloading disabled 8/18/08 |
| RXO | PRI daily bias -8 mS -500 mS 12/31/05 | PRI Daily Bias –5.5 mS -1500 mS 31Dec05 | PRI Daily Bias –3 mS -1500 mS 31Dec05 | PRI No daily bias -1500 mS 31Dec05 |
| CLOCK DIV | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 2 (since 4/3/03) 3 channels inop | 2 since 4/18/05 (analog drifting?) | 2 Swapped 9/20/06 | 1 Analog telem drifting |
| Recorders (DTR, SSR) | DTRs 1, 2, 3, 4 Safestate 1B EOT | DTRs 1, 2, 3, 4A Safestate 3A 4B STIP only | DTR 1,3,4 / SSR 2 Safestate=SSR 2B 4A Pressure leak | SSRs 1,2,3,4,5 Safestate=5B Current spikes |
| VTX (APT data) | #1 ON As of 19 June 07 | OFF / INOP since 11/15/00 | #2 ON 137.62 MHZ | #1 ON 137.1 MHZ As of 16 Aug 07 |
| BTX | #1 137.35 MHZ | #2 137.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ |
| STX 1 = rcp 1698.0 mhz 2 = lcp 1702.5 mhz 3= rcp 1707.0 mhz 4= rcp 2247.5 mhz | 2-Omni HRPT 2 Omni=rcp Power TOAR 446 4-Playback 1,2,3-Degraded | 2 – HRPT 1– Playback 4 Standby 3 low power INOP -10 Aug 2007 | 1 – HRPT TOAR424 low power 2 & 4 Playback STX-1 Power drop 2.29W 3/2/08 STX-4 nominal STX-3 Failed 2/14/08 | 3 - HRPT 1 - Playback 2 - Playback 4-Test & Standby |
| ### POWER (Array offset and CANT angle, eclipse states and sun angle) | Array Off -45 Since 1/25/01 Full Sun Sun Angle 13 Uptrend to 18 early Jan 07 Batts 1-3= LRC V/T=7/9 SA CANT=37 degs | Array Off -20 Since 9/13/07 Eclipse all year Sun Angle 34 Steady trend Batts 1-3= LRC V/T=4/8 SA CANT=22 degs Full sun S/C in Sept 07 BVR Phase Control auton swap 7/19/07 | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 66 Steady trend Batts 1-3= LRC V/T= 4/8 SA CANT=37 degs BVR Phase Control auton swap 9/17/07 Swap back 9/18/07 SADPOS Enabled 2/12/08 | Array Off. -40 Since 6/6/05 Eclipse all year Sun Angle 63 uptrend to 65 on 1 Jan 07 Batts 1-3=LRC Batts 1-3 V/T=4/8 SA CANT=22 BVR Phase Control auton swap 3/29/06 & 1/4/07 |
| AVHRR | Nominal Channel 3B only Occasional minor scan motor instabilities | Nominal Recent SM current increases TCE24 OFF 4/14/04 Channel 3B only | ### Nominal 3A-B switch enabled Previous scan motor current mini surges | Nominal Channel 3B only Scan motor current drop 29 Aug 06 |
| AMSU-A1 | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise Channel 4 severely degraded 4 Mar 08 | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Full Scan mode |
| `AMSU-A2 | Nominal | Nominal | Nominal | Non-nominal DIG-A Temps erratic 2/7/08 Cold Cal Pos 2 2/21/08 |
| AMSU-B | Operational Bias in All Chans. Degraded Chan-3 | Operational Space Position-3 Chan-8 degradation | Nominal | Replaced by MHS |
| ### HIRS | NOMINAL FW Abrupt return to nominal 7/12/05 After years of minor and major current surges FW mode=Hi Powr | NOMINAL New all time high filter motor current reached 9/17/06. Imagery occasionally exhibits high levels of radiometric noise | NOMINAL 1 Pixel cross-track misalignment. | Operational Filter Heat OFF. LW channel 1-12 noise TOAR 448 Loose LWIR lens. LW imagery erratic. CH 1 Period Monitor event 13 May 08 |
| ### MHS/MIU |  |  |  | Nominal Fly Wheel Motor Current Spiking but not alarming since 27 Dec 06 Entered Motor Spin State on 9 May 08 H4 gain increased 9 May 08 H3 gain increased 9 May 08 Automatic Gain Adjustment enabled 5 Sept. 08 |
| DCS RFI investigation | Nominal | Nominal Pseudo message turned on 1/5/06 | Nominal | Nominal DRU’s 1-7 On DRU8 inop/off |
| SARR | Operational B-Side, 13 Dec 07 AGC Mode Intermittent failures for 243MHz A&B | Operational Mode =AA A-side, AGC Mode (B-side in FG mode) 124 & 243 MHz = INOP & OFF as of 29 Jul 08 PDS ATT set to 0dB, 406 MHz RCVR ATT set to 2dB on 5 Sept. 08 | Nominal A-side All AGC Mode | Nominal A-Side All AGC Mode Since 15 Aug 05 |
| SARP | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | Nominal A-side 23 May05 |
| SEM | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=7 P-CDEM-HVPS=7 11 Jan 0714:47z | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=4 27 June 06 16:00z | Nominal Elec-CH-HVPS=1 27 June 06 17:00z Prot-CH-HVPS=1 11 Jan 07 14:15z |
| SBUV |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops TOAR 463 sun glint IBSL test performed 12 Sept 06 Grating motor investigation | Partially-Operational Anode mode since 6 July 05 13:30z Electronics turnoff taken out of STESM safestate table TOAR 462 sun glint Chop Motor stalled 22 May 08. Currently all motors turned on; not fully operational |
| SADPOS |  | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled Timer =12 mins |
| THERMAL Heaters, louvers TCE’s | TCE26 & 15H Telem valid TOAR 432 closed TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 Spacecraft in thermal uptrend | Nominal TCE24 OFF 4/14/05 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 AMSU-A1 survival heaters on 10/29/03 | All TCE’s active HIRS filter heater turned OFF 9/28/06 |

COMMENTS/CHANGES:

MISC:

NOAA-15:	There were relatively no changes to the NOAA-15 bus or payload health situation during the past week.

The #2 gyro is still relatively quiet. Daily monitoring continues.

NOAA-15 NTIP2_3 began flagging YH on Jday 294 (10/20), changed the YH from3.88 V to 3.90 V due to TIP Drift.

NOAA-16: 	There was relatively no change in the performance of the NOAA-16 bus & payload subsystems during the past week.

NOAA-17: 	There was relatively no change in the performance of the NOAA-17 payload subsystem during the past week.

NOAA-18:	There were relatively no changes to the NOAA-18 health situation during the past week.

MetOp-A:       There were relatively no changes to the health and safety of the NOAA instruments on MetOp-A this past week.

An Out-of-plane (OOP) maneuver was successfully performed 23 October 2008.

- Burn1: 14:30:03.021 Duration 4m05.334s

- AMSU-A1 deactivation 2008-10-23 09:23. Activation 2008-10-23 15:30

  - AMSU-A1:  2 deg C drop

- AMSU-A2 deactivation 2008-10-23 09:28. Activation 2008-10-23 15:30

  - AMSU-A2:  3 deg C drop

- SEM deactivation 2008-10-23 09:18. Activation 2008-10-23 15:35

  - SEM DPU: 2-3 deg C drop

  - SEM TED: 1-2 deg C drop

  - SEM MEPED: 1 deg C drop

- HIRS deactivation 2008-10-23 09:21. Activation 2008-10-23 16:00

  - HIRS: 0 deg C drop

- IASI deactivation 2008-10-23 09:10. Activation 2008-10-23 16:00

- MHS deactivation 2008-10-23 09:25. Activation 2008-10-23 16:00

- ASCAT deactivation 2008-10-23 09:33. Activation 2008-10-23 15:30

- GOME the mirror will be placed in launch position for the period 13:08.42 to 14:50.30 on 2008-10-23

- GRAS deactivation 2008-10-23 09:38. Activation 2008-10-23 15:30