Job Aid-RNP 10 Authorization.doc												Rev 1:  13 August 2007

RNP 10 JOB AID WITH FAA DOCUMENT REFERENCES

OPERATOR APPLICATION TO CONDUCT RNP 10 OPERATIONS IN WATRS PLUS CONTROL AREAS (CTA)

Introduction.   This RNP 10 Job Aid was developed by the FAA Flight Technologies & Procedures Division at FAA Hq to provide guidance to operators and inspectors on the process for operators to obtain RNP 10 for operation on oceanic routes/airspace in WATRS Plus CTAs.

RNP 10/50 NM lateral separation is planned to be introduced in WATRS Plus CTAs on 5 June 2008.  (See Webpage references on page 4).   The WATRS Plus CTAs are:  the West Atlantic Route System (WATRS), the Atlantic portion of Miami Oceanic Airspace and the San Juan FIR.  (RNP 10 is currently applied in the Pacific, the South Atlantic and other global areas).

Note:  a General Aviation operator holding a current Letter of Authorization (LOA) showing RNP 10 authority does not have to re-apply for RNP 10 authority for WATRS Plus CTAs.

Purposes of this Job Aid.  The Job Aid:

1.  Provides core RNP 10 reference documents for operators and inspectors.

2.  Provides a series of tables that show:  the content of an application, related reference paragraphs, location in operator documents/exhibits where an RNP 10 element is addressed and columns for the inspector to comment on and track the status of various RNP 10 program elements.

Explanation of RNP 10 vs. RNP 4.   RNP 10 is the minimum “Navigation Specification” required for the application of 50 NM lateral separation.  RNP 4 is an operator option.  A separate RNP 4 Job Aid will be posted on the FAA WATRS Plus Webpage.

Important Explanatory Notes.   See page 5 “Notes on Issuing Operations Specifications and General Aviation Letters of Authorization”.

Job Aid Organization

|  | Subject | Page |
| --- | --- | --- |
| Part 1 | General Information | 3 |
| Part 2 | Operator/Aircraft Identification Information | 6 |
| Part 3 | Operator Exhibits/Document Submissions | 7 |
| Part 4 | Content of Operator Application for RNP 10 or better Authorization | 9 |
| Part 5 | Guide For Determining Aircraft Eligibility For RNP 10 | 11 |
| Part 6 | Basic Pilot Procedures for RNP 10 Operations | 13 |
| Part 7 | In-flight Contingency, Weather Deviation & Strategic Lateral Offset Procedures | 15 |
| Part 8 | FAA Notice to be posted by 31 Aug 07: WATRS Plus Operational Policy and Procedures | 16 |
| Part 9 | Contacts | 16 |

Recommended Inspector And Operator Actions.   The following are suggestions on how the job aid can be used:

|  | Lead | Action |
| --- | --- | --- |
| 1 | Inspector | Reviews the “Basic Events in the RNP 10 Approval Process” on page 3 with the operator in the pre-application meeting to provide an overview of approval process events. |
| 2 | Inspector | Reviews this Job Aid with the operator to establish the form and content of the operator application for RNP 10 authority. |
| 3 | Operator | Uses the Job Aid as a guide to assemble documents/exhibits for its RNP 10 application. See suggested list in Part 3. |
| 4 | Operator | Annotates Job Aid to show location of RNP 10 program elements in the operator exhibits/documents. |
| 5 | Operator | Submits Job Aid and RNP 10 exhibits/documents to inspector. |
| 6 | Inspector | Annotates Job Aid to show task or document “complete/satisfactory” or “open/further operator action required”. |
| 7 | Inspector | Informs the operator as soon as possible, when further operator action is required. |
| 8 | Operator | Provides inspector, when requested, with revised material. |
| 9 | Inspector | Issues LOA or OpSpecs, as applicable, to operator when required tasks and documents are completed. |

PART 1 – GENERAL INFORMATION

Basic Events In RNP 10 Authorization Process

|  | Operator Actions | Inspector Actions |
| --- | --- | --- |
| 1 | Establishes need to obtain RNP 10 authority. |  |
| 2 | Reviews Airplane Flight Manual (AFM), AFM Supplement or Type Certificate Data Sheet or other appropriate documents (e.g., Service Bulletins, Service Letters) to determine aircraft eligibility for RNP 10. Operator contacts airplane or avionics manufacturer, if necessary, to confirm airplane RNP 10 or better eligibility. |  |
| 3 | Contacts a Flight Standards District Office (FSDO) or Certificate Management Office (CMO), as applicable, to arrange a pre-application meeting to discuss requirements for operational approval. |  |
| 4 |  | Establishes, during pre-application meeting: - Form and content of operator exhibits/documents supporting RNP 10 authorization. - Whether or not a review of operator program documents will suffice for RNP 10 authorization or if an FAA observed validation flight will also be required. Note: an FAA observed validation flight may not be necessary. Inspectors should contact one of the navigation specialists listed in Job Aid Part 9 for guidance and recommendation. |
| 5 | Submits exhibits/documents to FAA Flight Standards office, at least 60 days in advance of the planned start of RNP 10 operations, for operational approval. |  |
| 6 |  | Reviews operator submissions. |
| 7 |  | Only if required, FAA observed validation flight is completed. See note above. |
| 8 |  | Issues operational approval in the form of OpSpecs (Parts 121, 125 or 135) or OPSS generated LOA (part 91), as appropriate, when airworthiness and operational requirements are fulfilled. |

WATRS PLUS WEBPAGE:  Core Documents, Contacts, etc.    The FAA “WATRS Plus Webpage” can be accessed from the FAA Oceanic Operations Standards Homepage at:

www.faa.gov/ats/ato/130.htm or www.faa.gov/about/office_org/headquarters_offices/ato/service_units/enroute/oceanic/

| Core FAA Reference Documents | Subject or Title | WATRS Plus Webpage Location |
| --- | --- | --- |
| 1. Advance FAA Notice (15 Mar 07) WATRS Plus Airspace Redesign & Separation Reduction | Project description, planned operational policies/procedures. | Section 2 – Project Description |
| 2. FAA Order 8400.12 (as amended) | RNP 10 Operational Approval | Section 2 - FAA RNP Authorization Policy & Procedures: RNP 10 and Operator-Option RNP 4 |
| 3. Acceptable Navigation System Configurations For Operations In WATRS Plus Control Areas (CTAs) | Explanation of RNP 10 dual equipage and operation on Non-RNP 10 routes within WATRS Plus CTAs | Section 2 - FAA RNP Authorization Policy & Procedures, Acceptable Navigation System Configurations For Operations in WATRS Plus Control Areas |
| 4. Pilot Procedures: In-flight Contingencies, Weather Deviation and SLOP (Strategic Lateral Offset Procedures) | Extract from FAA Notice posted on WATRS Plus Webpage. See Job Aid Part 7. | Section 2 - Operating Policy, Contingency, Weather Deviation & Strategic Lateral Offset Procedures |
| -5. FAA Notice: WATRS Plus Operational Policy & Procedures (to be posted by 31 Aug 07 | Detailed RNP 10/50-lateral Operating Policies & Procedures For WATRS Plus CTAs | To be posted by 31 Aug 07 in Section 2 – Operating Policy |
| 6. FAA Inspector Guidance: Notice 8000.340, Figure 1.5.5 | Inspector guidance on RNP 10 Operational Approval | Section 2 - FAA Flight Standards Inspector Guidance, Operational Approval to Operators to Conduct Operations in Oceanic Airspace Where RNP 10 is Required |

Note:    A separate RNP 4 Job Aid will be posted on the WATRS Plus Webpage for operators that elect to obtain RNP 4 authorization for WATRS  Plus operations.  It will provide references to FAA Order 8400.33 (Procedures For Obtaining Authorization For RNP 4 Oceanic and Remote Area Operations).  8400.33 is posted on the WATRS Plus Webpage.

Applicable Part 91 (GA) Letters of Authorization (LOA):   Operations Safety System (OPSS) Generated

1.  OPSS LOA B036 (Operation in Required Navigation Performance Airspace)

2.  OPSS LOA D098 (Flight in Special Areas of Operations For Short-term Operations)

Applicable Operations Specifications (OpSpecs) Paragraphs

1.  B036 (Class II Navigation Using Multiple Long-Range Navigation Systems (LRNS)

2.  B054 (Class II Navigation Using Single Long-Range Navigation system):  currently applicable for WATRS and other specific oceanic areas.  See FAA Notice: WATRS Plus Operational Policy & Procedures on WATRS Plus Webpage for policy for operation of Non-RNP 10 aircraft (including single LRNS aircraft) in WATRS Plus airspace after the June 2008 project implementation.

Notes On Issuing Operations Specifications and General Aviation LOAs (Reference FAA Notice 8000.340, Figure 1.5.5)

1.  Responsible State Authority.   The following is ICAO and FAA guidance on State (country) responsibility for issuing authorization for operations such as RNP 10, RNP 4 and RVSM.

a.  International Commercial Operators:   the State of Registry makes the determination that the aircraft meets the applicable RNP requirements.  The State of Operator issues operating authority (e.g., Operations Specifications (OpSpecs)).

b.   International General Aviation (IGA) Operators:  The State of Registry makes determination that aircraft meets the applicable RNP requirements and issues operating authority (e.g., Letter of Authorization (LOA).

2.  Separate Part 91 (GA) LOAs are not required to be issued for individual areas of operations.   LOA B036 contains a statement of operator responsibility for policies/procedures applicable to individual areas of operation.

3.  A Part 91 (GA) operator holding a current LOA showing RNP 10 authority does not have to re-apply for RNP 10 authority in WATRS Plus Control Areas.

4.   Inspector action for Part 91 (GA) operators is to “accept” RNP 10 related operations and airworthiness programs.   Formal “approval” is not required.   (See FAA Order 8700.1, Chapter 3, paragraph 11 for discussion of these terms).

5.  An operator planning to obtain initial RNP 10 authority for WATRS Plus CTAs may apply for and be issued authority based on operational policies published and referenced in this Job Aid.

6.  OpSpecs.  Part 121, 125 and  135 operators showing RNP authority in OpSpec paragraph B036 should list the authority applicable to individual areas of operations in OpSpec paragraph B050 (Authorized Areas of En Route Operations, Limitations, and Provisions).

Related ICAO Reference Documents  (The FAA RNP 10 authorization process is in accordance with ICAO RNP documents.  The ICAO documents shown below are for reference only).

1.  ICAO Annex 2 (Rules of the Air).  See paragraphs 2.3 (Responsibility for compliance with the rules of the air) and 5.1.1 (Aircraft equipment).

2.  ICAO Annex 6 (Operation of Aircraft), Part 1 (International Commercial Air Transport) and Part 2 (International General Aviation).  See paragraph 7.2 (Navigation Equipment).

3.  New ICAO Doc 9613 (new Performance Based Navigation (PBN) Manual.  The ICAO PBN Manual was distributed in April 2007.  It is posted on the WATRS Plus Webpage.

4.  ICAO Doc 4444 (Procedures For Air Navigation Services – Air Traffic Management), paragraph 15.2 (Special Procedures For In-flight

Contingencies In Oceanic Airspace)

PART 2:   OPERATOR/AIRCRAFT  IDENTIFICATION INFORMATION

OPERATOR NAME:

| AIRCRAFT MAKE, MODEL, SERIES | # REGISTRATION NUMBER(s) | # SERIAL NUMBERS(s) | LONG-RANGE NAVIGATION SYSTEMS (LRNS): NUMBER, MANUFACTURER & MODEL | RNP TYPE REQUESTED (E.G., RNP 10, RNP 4) |
| --- | --- | --- | --- | --- |

#Note:  may be provided separately in a form and manner acceptable to the inspector.

DATE OF PRE-APPLICATION MEETING ________

DATE APPLICATION RECEIVED: __________ DATE OPERATOR PLANS TO START RNP 10 OPERATIONS __________

NOTIFCATION TIME TO FLIGHT STANDARDS FIELD OFFICE ADEQUATE?  	  ___YES  	 ___NO

Note:  Separate Part 91 (GA) LOAs are not required to be issued for individual areas of operations.

PART 3:    OPERATOR EXHIBITS/DOCUMENTS SUBMITTED

| Exhibit |  | Exhibit/Document Title | Operator Indication of Inclusion | Inspector Comments |
| --- | --- | --- | --- | --- |
| A |  | (Recommended) Operator Letter Requesting RNP 10 Authority |  |  |
| B |  | Aircraft Eligibility Group. Operator statement of RNP 10 Eligibility Group or Groups into which its aircraft/Long Range Navigation System (LRNS) combinations fall. See Job Aid Part 5. |  |  |
| C |  | Aircraft airworthiness documents…..showing RNP 10 eligibility… in accordance with FAA Order 8400.12, paragraphs 11, 12, 15b: e.g., AFM, AFM Revision, AFM Supplement or Type Certificate Data Sheet (TCDS) showing that aircraft LRNS are RNP 10 eligible. (See Job Aid Parts 4 and 5). |  |  |
| D |  | Aircraft Modified To Meet RNP 10 Standards……documentation of aircraft inspection and/or modification. If applicable, maintenance records documenting installation or modification of aircraft/LRNS (e.g., Major Repair and Alteration (FAA Form 337)) |  |  |
| E |  | For INS or IRU Only Equipped Aircraft: RNP 10 Time Limit and Area of Operations. Documentation establishing the RNP 10 time limit and area of operations or routes for which the specific aircraft/navigation system is eligible. (Not applicable to GPS equipped aircraft.) |  |  |
| F |  | Maintenance Program: for aircraft with established LRNS maintenance practices, provide list of document or program references. For newly installed LRNS, provide LRNS maintenance practices for review. |  |  |
| G |  | Minimum Equipment List (MEL) (only for operators operating under an MEL): MEL or MMEL showing provisions for LRNS |  |  |
| H |  | 1. Part 91 (GA) Operator Method of Pilot Training/Knowledge: operator in-house training, part 142 training center or other course of instruction. 2. Part 121, 125 or 135 Pilot &, if applicable, Dispatcher Training documents |  |  |
| I | 1. Part 91 Operator Operations Manuals or Documents: Operations Manual or section of operator’s application documenting RNP 10 operational policy/procedures. 2. Part 121, 125 or 135 Operations Manuals material |  |  |  |

PART 3:    OPERATOR EXHIBITS/DOCUMENTS SUBMITTED (cont.)

| Exhibit |  | Exhibit/Document Title | Operator Indication of Inclusion | Inspector Comments |
| --- | --- | --- | --- | --- |
| J | Operating History. (If any, past problems, incidents, track keeping errors, corrective actions.) |  |  |  |
| K | Removal of RNP 10 Operating Authority. Awareness of necessity for timely, effective follow-up to navigation errors and potential for removal of RNP 10 authority. |  |  |  |
| L | Only if required, plan for FAA Observed Validation Flight. See Job Aid Part 4, item 11. |  |  |  |

*SUBMISSION INCLUDES:  ____ AIRCRAFT/LRNS COMPLIANCE DOCUMENTION

____ OPERATIONAL POLICY/PROCEDURES

____ MAINTENANCE MANUAL SECTIONS RELATED TO LRNS (if not previously reviewed)

Note:  Exhibits/documents may be included in a binder or submitted as a stand-alone document(s).

PART 4:  CONTENT OF OPERATOR APPLICATION FOR RNP 10 OR BETTER

| # | Content of Operator Application For RNP 10 Reference 8400.12, Paragraph 9 | 8400.12 and Other Reference Paragraphs: | Where Found in Operator Exhibits/Documents Note: operator should update this column to reflect the content of its application. | FAA Inspector Recommendation and/or Comments | Inspector Tracking: Item Status and Date |
| --- | --- | --- | --- | --- | --- |
| 1. | Operator Request Letter: statement of intent to obtain RNP 10 or better authority. (Recommended) |  | Exhibit A |  |  |
| 2. | Aircraft/Navigation System RNP 10 Eligibility Group. Airworthiness documents that establish the proposed aircraft/navigation system group, its RNP 10 approval status and, in a form acceptable to the inspector, a list of airframes in that group. | Paragraphs 11 and 12 See Job Aid, Part 5. | Exhibit B, C, D, as applicable |  |  |
| 2a. | Dual Equipage Requirement: at least two Long Range Navigation Systems (LRNS) with adequate display and functionality for oceanic operations | 15b See Job Aid Part 5 | Exhibit B, C, D, as applicable |  |  |
| 3. | Time Limit For INS or IRU Only Equipped Aircraft. Approved or requested RNP 10 time limit for aircraft for which Inertial Navigation Systems (INS) or Inertial Reference Units (IRU) are the only source of Long Range Navigation (LRN). (Not applicable to GPS equipped aircraft) | Paragraphs 12b(2), (3); 12d; | Exhibit E |  |  |
| 4. | RNP-10 Area of Operations For INS or IRU Only Equipped Aircraft. Documentation establishing the RNP-10 area of operations or routes for which the specific aircraft/navigation system is eligible. (Not applicable to GPS equipped aircraft) | 12e; position updating (12f and 12g, as applicable); 15e | Exhibit E |  |  |
| 5. | a. Part 91 (General Aviation) Pilot Training or Knowledge. (e.g., operator in-house training, part 142 training center or other course of instruction). b. Part 121, 125 or 135 Pilot and, if applicable, Dispatcher Training documents. | Paragraphs 9a(4); position updating (12f and 12g, as applicable); paragraph 15; Appendix 4; Appendix 7 (if applicable); Contingency, Weather, Offset Procedures (See Job Aid Parts 6 & 7). | Exhibit H |  |  |

| # | Content of Operator Application For RNP 10 Reference 8400.12, Paragraph 9 | 8400.12 and Other Reference Paragraphs: | Where Found in Operator Exhibits/Documents Note: operator should update this column to reflect the content of its application. | FAA Inspector Recommendations and/or Comments | Inspector Tracking: Item Status and Date |
| --- | --- | --- | --- | --- | --- |
| 6. | a. Part 91 (GA) Operations Manuals or Documents. Operations Manual or section of operator’s application documenting RNP 10 operational policies and procedures. b. Part 121, 125 or 135 Operations Manuals and Checklists. | Paragraphs 9a(5), 12f and 12g (as applicable), paragraph 15; Appendix 4; Appendix 7 (if applicable); Contingency, Weather, Offset Procedures (See Job Aid Parts 6 & 7). | Exhibit I |  |  |
| 7. | Maintenance Practices. For aircraft with established LRNS maintenance practices, provide document references. For newly installed LRNS, provide maintenance practices for review. | Paragraph 14 | Exhibit F |  |  |
| 8. | Minimum Equipment List (MEL) updates, if applicable. (Only applicable if operator conducts operations under an MEL). | Paragraph 13 | Exhibit G |  |  |
| 9. | Operating History. Operating history that identifies past problems, incidents, track keeping errors, if any, and actions taken to correct the situation. | Paragraph 9a(6) | Exhibit J |  |  |
| 10. | Removal of RNP 10 Operating Authority. Awareness of the necessity to follow up action after navigation error reports, and the potential for removal of RNP 10 operating authority. | Paragraph 9b(3) | Exhibit K |  |  |
| 11. | Only if required, plan for FAA Observed Validation Flight. An FAA observed validation flight may not be necessary. Inspectors should contact one of the navigation specialists listed in Job Aid Part 9 for guidance and recommendation. | FAA Inspector Guidance Notice 8000.340, Fig 1.5.5 paragraph 3K | Exhibit L |  |  |

PART 5   GUIDE FOR DETERMINING AIRCRAFT ELIGIBILITY

| # | Subject | 8400.12 & Other Reference Paragraphs: | Location in Operator Exhibits | FAA Recommendations & Comments | Inspector Tracking Item Status & Date |
| --- | --- | --- | --- | --- | --- |
| 1 | Defining Group Aircraft | 11a | B, C, as applicable |  |  |
| 2 | Dual Long Range Navigation System Equipage (minimum) | 15b | B,C, D, as applicable |  |  |
| 3 | Eligibility Group 1 – Aircraft Eligibility Through RNP Certification (RNP compliance documented in Airplane Flight Manual (AFM) | 12a | B, C, D, as applicable |  |  |
| 4 | Eligibility Group 2 – Aircraft Eligibility Through Prior Navigation System Certification | 12b 12b(1) - no longer pplicable. | B, C, D, as applicable |  |  |
| 4a | INSs or IRUs Approved In Accordance With 14 CFR Part 121, Appendix G (Time limit 6.2 hours) | 12b(2) | B, C, D, as applicable |  |  |
| 4b | INSs or IRUs Approved For North Atlantic MNPS or Australian RNAV Operations (Time limit 6.2 hours) | 12b(3) | B, C, D, as applicable |  |  |
| 4c | Obtaining Approval For Extended INS or IRU Time Limit | 12d Appendix 2 | B, C, D, as applicable |  |  |
| 4d | GPS Approved to Primary Means of Navigation Standards (reference AC 20-138, as amended) | 12b(4) | B, C, D, as applicable |  |  |
| 4e | Multisensor Systems Integrating GPS (reference AC 20-130, as amended) | 12b(5) | B, C, D, as applicable |  |  |
| 4f | Equipage With One GPS and One Other Approved LRNS (e.g., INS or IRU). Note: this category is being added to 8400.12 to harmonize with ICAO guidance. | Ref. Inspector Guidance: Notice 8000.340, Figure 1.5.5, paragraph 3C(5). | B, C, D, as applicable |  |  |

PART 5   GUIDE FOR DETERMINING AIRCRAFT ELIGIBILITY (continued)

| 5 | Eligibility Group 3 – Eligibility Through Data Collection | 12c | B, C, as applicable |  |  |
| --- | --- | --- | --- | --- | --- |
| 5a | Sequential Method | 12c(1), Appendix 1 | B, C, as applicable |  |  |
| 5b | Periodic Method | 12c(2), Appendix 6 | B, C, as applicable |  |  |

PART 6:  BASIC PILOT PROCEDURES FOR RNP 10 OPERATIONS

| # | Subject | Reference Paragraphs 8400.12, Appendix 4 & Others | Location In Operator Exhibits | FAA Recommendations/Comments | Inspector Tracking Item Status & Date |
| --- | --- | --- | --- | --- | --- |
| 1 | Flight Planning. | Paragraph 2 | Exhibits H and I |  |  |
|  | Verify aircraft authorized RNP 10 and two long-range navigation systems (LRNS) operational. | 2a | Exhibits H and I |  |  |
|  | When equipped with INS or IRU only, verify that RNP 10 time limit (area of operations) is accounted for. | 2b | Exhibits H and I |  |  |
|  | **Annotate ICAO Flight Plan block 10 (Equipment) with “R” and “Z” and annotate Item18 with “NAV/RNP10”. See Job Aid Part 8. -If two approved LRNS not operational, delete above annotations & operate in accordance with applicable policy. | 2c **Block 10 “Z” and Item 18 annotations are WATRS Plus area policy. | Exhibits H and I |  |  |
|  | As applicable, verify that GPS requirements such as Fault Detection & Exclusion (FDE) are accounted for. | 2d | Exhibits H and I |  |  |
|  | Account for navigation system operating restriction, if any. | 2e | Exhibits H and I |  |  |
| 2 | Pre-flight Procedures at the Aircraft. | 3 | Exhibits H and I |  |  |
|  | Review maintenance logs and forms for LRNS status; confirm navigation database current. | 3a | Exhibits H and I |  |  |
|  | During external inspection of aircraft, check condition of navigation-related items such as navigation antennas. | 3b | Exhibits H and I |  |  |

| # | Subject | Reference Paragraphs 8400.12, Appendix 4 | Location In Operator Exhibits | FAA Recommendations/Comments | Inspector Tracking Item Status & Date |
| --- | --- | --- | --- | --- | --- |
| 3 | En route Procedures | 4 | Exhibits H and I |  |  |
|  | Before oceanic entry point, verify two LRNS operating. If not, notify ATC and operate in accordance with policy applicable to the airspace. | 4a | Exhibits H and I |  |  |
|  | Before entering oceanic airspace, perform navigation accuracy check and position update (if necessary) using accepted method. See item 4 below. | 4b | Exhibits H and I |  |  |
|  | Follow in-flight operating drills to to prevent inadvertent deviation from cleared routes. | 4c | Exhibits H and I |  |  |
|  | Advise ATC of loss of long-range navigation capability and operate in accordance with policy applicable to the airspace. | 4d | Exhibits H and I |  |  |
| 4 | LRNS Position Updating |  | Exhibits H and I |  |  |
|  | Automatic Position Updating (as applicable) | 12f | Exhibits H and I |  |  |
|  | Manual Position Updating (as applicable) | 12g, Appendix 7 | Exhibits H and I |  |  |

PART 7:  IN-FLIGHT CONTINGENCY, WEATHER DEVIATION & SLOP (STRATEGIC LATERAL OFFSET PROCEDURES)

NOTE:  The guidance cited below is not applicable to a specific lateral separation standard (i.e, it is not specific to RNP 10 and 50 nm

Lateral separation).  It is applicable to general oceanic operations.  An operator may have previously adopted the guidance for oceanic

operations.   If so, the operator only needs to confirm that that is the case.

# WEBPAGE REFERENCE DOCUMENT:  see WATRS Plus Webpage, Section 2 - Operating Policy:  “Contingency, Weather

Deviation and Strategic Lateral Offset Procedures”.

Note:  FAA guidance reflects ICAO Document 4444, section 15.2 (Special Procedures For In-flight Contingencies In Oceanic Airspace).

| # | Subject | # Webpage Reference Document Paragraphs | Location In Operator Exhibits | FAA Recommendations/Comments | Inspector Tracking Item Status & Date |
| --- | --- | --- | --- | --- | --- |
| 1 | Procedures For In-flight Contingencies In Oceanic Airspace |  | Exhibits H and I |  |  |
|  | Introduction (purpose of in-flight contingency procedures) | Introduction: paragraphs 1, 2 | Exhibits H and I |  |  |
|  | General Procedures | General Procedures: Paragraphs 1-4 | Exhibits H and I |  |  |
|  | Special Procedures For ETOPS Aircraft | ETOPS paragraph | Exhibits H and I |  |  |
| 2 | Weather Deviation Procedures For Oceanic Operations |  | Exhibits H and I |  |  |
|  | General Procedures | Paragraphs 1-3 | Exhibits H and I |  |  |
|  | Obtaining Priority From ATC When Weather Deviation Required | Paragraph 4 | Exhibits H and I |  |  |
|  | Actions To Be Taken When Controller-Pilot Communications Are Established | Paragraph 5 | Exhibits H and I |  |  |
|  | Actions To Be Taken If A Revised ATC Clearance Cannot Be Obtained | Paragraph 6 | Exhibits H and I |  |  |

| # | Subject | # Webpage Reference Document Paragraphs | Location In Operator Exhibits | FAA Recommendations/Comments | Inspector Tracking Item Status & Date |
| --- | --- | --- | --- | --- | --- |
| 3 | Strategic Lateral Offsets Procedures (SLOP) In Oceanic Airspace |  |  |  |  |
|  | Introduction (objective of SLOP) | Paragraph 1 | Exhibits H and I |  |  |
|  | Guidelines For Offset Positions | Paragraph 2 | Exhibits H and I |  |  |
|  | Guidance For Using SLOP | Paragraph 3 | Exhibits H and I |  |  |
|  | SLOP Application In The San Juan FIR And Bermuda Airspace | Paragraphs 3(f) and 3(g) | Exhibits H and I |  |  |

PART 8 – WATRS PLUS OPERATIONAL POLICY & PROCEDURES  (to be posted by 31 Aug 2007)

PART 9 – CONTACTS

| Name | Position/Organization | Phone/FAX | Email |
| --- | --- | --- | --- |
| David Maloy | Navigation Specialist, Flight Technologies & Procedures Division (AFS-400). Office in Windsor Locks, Conn. | Ph. 860-654-1006; Fax 860-654-1009 | David.Maloy@faa.gov; |
| Anderson Davie | Navigation Specialist, San Francisco Intl. Field Office (IFO) | Ph. 650-876-2771; Fax 650-697-7231 | Anderson.Davie@faa.gov; |
| James D. Brown | Navigation Specialist, San Francisco IFO | Ph. 650-876-2771; Fax 650-697-7231 | James.D.Brown@faa.gov; |
| Madison Walton | Flight Standards Lead for WATRS Plus Initiative; AFS-400, Washington, D.C. | Ph. 202-385-4596; Fax 202-385-4653 | Madison.Walton@faa.gov; |
| Roy Grimes | Flight Standards Specialist, FAA Program Support; CSSI, Inc., Washington, D.C. | Ph. 202-863-3692; Fax 202-863-2398 | RGrimes@cssiinc.com; |