OFFICIAL FAA HOLDOVER TIME TABLES

WINTER 2008-2009

SUMMARY OF CHANGES FROM 2007-2008

### TYPE I FLUIDS. The Type I fluid holdover time (HOT) table values are unchanged.

### TYPE II FLUIDS. One new Type II fluid, Kilfrost ABC-K Plus, has been added to the list of qualified Type II fluids. The addition of this fluid did not cause any of the values to change in the Type II generic HOT table, Table 2. A brand specific table has been added for this new fluid (Table 2F). Aviation Xi’an High-Tech KHF-II, which was added to the list of qualified fluids in 2007 was removed due to fluid specification concerns. Specifically, there may still be batches of the fluid on the market that were shipped/delivered with viscosities lower than the published Lowest On-Wing Viscoisty for use with the HOT guidelines. More recently, Aviation Xi’an High Tech indicated to the FAA that they were going to requalify the fluid at a higher viscosity; the FAA requested the new aerodynamic testing results, however these have not been provided by Aviation Xi’an.

### TYPE III FLUIDS. The Type III fluid HOT table values are unchanged.

### TYPE IV FLUIDS. In the Type IV generic HOT tables, the values were increased in the Rain on Cold Soaked Wing cells due to the removal of obsolete data.

TABLE OF CONTENTS FOR HOT GUIDELINES

FOR WINTER 2008-2009

TABLE												PAGE

Table 1.	FAA Guidelines for Holdover Times SAE Type I Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature…………………………….............................................A-13

Table 1A.         FAA Guidelines for the Application of SAE Type I Fluid Mixture Minimum Concentrations as

a Function of Outside Air Temperature……………………………………………………........…A-14

Table 1B.         Snowfall Intensities as a Function of Prevailing Visibility……………………………………......A-15

Table 2.	FAA Guidelines for Holdover Times SAE Type II Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature…………………………………………………….........A-16

Table 2A.         FAA Guidelines for Holdover Times ABAX (SPCA) ECOWING 26 Type II Fluid Mixtures

as a Function of Weather Conditions and Outside Air Temperature……………………….…. ….A-17

Table 2B.         FAA Guidelines for Holdover Times CLARIANT SAFEWING MP II 2025 ECO Type II

Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature……………...A-18

Table 2C.         FAA Guidelines for Holdover Times CLARIANT SAFEWING MP II FLIGHT Type II

Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature.......................A-19

Table 2D.	FAA Guidelines for Holdover Times KILFROST ABC-2000 Type II Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature. …………………..…................................. A-20

Table 2E.	FAA Guidelines for Holdover Times KILFROST ABC-II PLUS Type II Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature…..……………………................ A-21

Table 2F.	FAA Guidelines for Holdover Times KILFROST ABC-K PLUS Type II Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature.…….………………………….... A-22

Table 2G.	FAA Guidelines for Holdover Times NEWAVE AEROCHEMICAL FCY-2 Type II Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature. ….……………… A-23

Table 2H.        FAA Guidelines for Holdover Times OCTAGON E-MAX Type-II Fluid Mixtures as a

Function of Weather Conditions and Outside Air Temperature…………………….……………. A-24

Table 3.	FAA Guidelines for Holdover Times SAE Type III Fluid Mixture as a Function of Weather Conditions and Outside Air Temperature……………………………............................................ A-25

Table 4.	FAA Guidelines for Holdover Times SAE Type IV Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature…………………………….............................................A-26

Table 4A.	FAA Guidelines for Holdover Times ABAX (SPCA) AD-480 Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature……………………………..........................…A-27

Table 4B.	FAA Guidelines for Holdover Times CLARIANT SAFEWING MP IV 2001 Type IV Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature……………….. ….A-28

Table 4C.	FAA Guidelines for Holdover Times CLARIANT SAFEWING MP IV 2012 PROTECT

Type IV Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature……A-29

TABLE												PAGE

Table 4C.	FAA Guidelines for Holdover Times CLARIANT SAFEWING MP IV 2012 PROTECT

Type IV Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature……A-29

Table 4D.	FAA Guidelines for Holdover Times CLARIANT SAFEWING MP IV LAUNCH Type IV Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature………….….……..A-30

Table 4E.	FAA Guidelines for Holdover Times DOW UCAR™ ULTRA+ ADF/AAF Type IV

Fluid as a Function of Weather Conditions and Outside Air Temperature………….…................ A-31

Table 4F.	FAA Guidelines for Holdover Times DOW UCAR™ ENDURANCE EG106 Type IV Fluid as a Function of Weather Conditions and Outside Air Temperature…………………………….......... A-32

Table 4G.	FAA Guidelines for Holdover Times DOW UCAR™ FLIGHTGUARD AD-480 Type IV Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature…………………... A-33

Table 4H.        FAA Guidelines for Holdover Times KILFROST ABC-S Type IV Fluid Mixtures as a

Function of Weather Conditions and Outside Air Temperature………………………..……........ A-34

Table 4I.	FAA Guidelines for Holdover Times KILFROST ABC-S PLUS Type IV Fluid Mixtures

as a Function of Weather Conditions and Outside Air Temperature……………………………... A-35

Table 4J.          FAA Guidelines for Holdover Times LYONDELL ARCTIC SHIELD™ Type IV Fluid

Mixtures as a Function of Weather Conditions and Outside Air Temperature………………....... A-36

Table 4K.	FAA Guidelines for Holdover Times OCTAGON MAX-FLIGHT Type IV Fluid Mixtures

as a Function of Weather Conditions and Outside Air Temperature…………………………...….A-37

Table 4L.	FAA Guidelines for Holdover Times OCTAGON MAX-FLIGHT 04 Type IV Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature………………………….............. A-38

Table 4M.	FAA Guidelines for Holdover Times OCTAGON MAXFLO Type IV Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature…………………………………………..... A-39

Table 5.	FAA Guidelines for the Application of SAE Type II, Type III, and Type IV Fluid MixturesA-.A-40

Table 6.	Lowest On-wing Viscosity Values for Neat and Diluted SAE Type II and Type IV Fluids….....A-41

Table 7.	List of Qualified Deicing/Anti-Icing Fluids – Winter 2008-2009……………………….……….. A-42

- FAA TYPE I Holdover Time Guideline

- TABLE 1.  FAA GUIDELINES FOR HOLDOVER TIMES SAE TYPE I FLUID MIXTURES AS A FUNCTION OF WEATHER CONDITIONS AND OUTSIDE AIR TEMPERATURE

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit | - Active Frost | Freezing Fog | Snow/Snow Grains |  |  | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** |  | - Other‡ |
|  |  |  |  | - Very Light | - Light  | - Moderate |  |  |  |  |  |
| -3 and above | 27 and above | 0:45 | 0:11-0:17 | 0:18-0:22 | 0:11-0:18 | 0:06-0:11 | 0:09-0:13 | 0:02-0:05 | 0:02-0:05 |  |  |
| below -3 to -6 | below 27 to 21 | 0:45 | 0:08-0:13 | 0:14-0:17 | 0:08-0:14 | 0:05-0:08 | 0:05-0:09 | 0:02-0:05 | CAUTION: No holdover time guidelines exist |  |  |
| below -6 to -10 | below 21 to 14 | 0:45 | 0:06-0:10 | 0:11-0:13 | 0:06-0:11 | 0:04-0:06 | 0:04-0:07 | 0:02-0:05 |  |  |  |
| below -10 | below 14 | 0:45 | 0:05-0:09 | 0:07-0:08 | 0:04-0:07 | 0:02-0:04 |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 degrees Celsius (32 degrees Fahrenheit) only

‡ Heavy snow, snow pellets, ice pellets, moderate and heavy freezing rain, hail

 TO USE THESE TIMES, THE FLUID MUST BE HEATED TO A MINIMUM TEMPERATURE OF 60 C (140 F) AT THE NOZZLE AND AT LEAST 1       LITER/M2 ( 2 GALS/100FT2) MUST BE APPLIED TO DEICED SURFACES

SAE Type I fluid/water mixture is selected so that the freezing point of the mixture is at least 10 C (18 F) below OAT.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

SAE TYPE I FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

TABLE  1A.  FAA Guidelines for the Application of SAE Type I Fluid Mixture

Minimum Concentrations as a Function of OUTSIDE AIR TEMPERATURE

Concentrations in % volume

| Outside Air Temperature (OAT) | One-step Procedure Deicing/Anti-icing1 | Two-step Procedure |  |
| --- | --- | --- | --- |
|  |  | First step: Deicing | Second step: Anti-icing 1, 2 |
| -3 C (27 F) and above | Mix of fluid and water heated to 60 C (140 F) minimum at the nozzle, with a freezing point of at least 10 C (18 F) below OAT | Heated water or a mix of fluid and water heated to 60 C (140 F) minimum at the nozzle | Mix of fluid and water heated to 60 C (140 F) minimum at the nozzle, with a freezing point of at least 10 C (18 F) below OAT |
| Below -3 C (27 F) |  | Freezing point of heated fluid mixture shall not be more than 3 C (5 F) above OAT |  |
| - Fluids must only be used at temperatures above their lowest operational use temperature (LOUT). - To be applied before first-step fluid freezes, typically within 3 minutes. |  |  |  |
| Notes: - Upper temperature limit shall not exceed fluid and aircraft manufacturers’ recommendations. - To use Type I holdover time guidelines in snow conditions, at least 1 liter per square meter (2 gal. Per 100 square feet) fluid must be applied to the deiced surfaces. - This table is applicable for the use of Type I Holdover Time Guidelines. If holdover times are not required, a temperature of 60 C (140 F) at the nozzle is desirable. - The lowest operational use temperature (LOUT) for a given fluid is the higher of: - The lowest temperature at which the fluid meets the aerodynamic acceptance test for a given aircraft type, or - The actual freezing point of the fluid plus a freezing point buffer of 10°C (18°F). Caution: Wing skin temperatures may differ and, in some cases, be lower than OAT. A stronger mix (more glycol) can be used under these conditions. |  |  |  |

August 2008

Table 1B.  Snowfall INTENSITIES as a Function of prevailing Visibility

| Time of Day | Temp. |  | Visibility (Statute Mile) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Degrees Celsius | Degrees Fahrenheit |  2 1/2 | 2 | 1 1/2 | 1 | 3/4 | 1/2 |  1/4 |  |
| Day | colder/equal -1 | colder/equal 30 | Very Light | Very Light | Light | Light | Moderate | Moderate | - Heavy | - Snowfall Intensity |
|  | warmer than -1 | warmer than 30 | Very Light | Light | Light | Moderate | Moderate | Heavy | - Heavy |  |
| Night | colder/equal -1 | colder/equal 30 | Very Light | Light | Moderate | Moderate | - Heavy | Heavy | Heavy |  |
|  | warmer than -1 | warmer than 30 | Very Light | Light | Moderate | - Heavy | Heavy | Heavy | Heavy |  |
| NOTE 1: This table is for estimating snowfall intensity. It is based upon the technical report, “The Estimation of Snowfall Rate Using Visibility,” Rasmussen, et al., Journal of Applied Meteorology, October 1999 and additional in situ data. NOTE 2: This table is to be used with Type I fluid guidelines. It may also be used with Type II, III, or IV fluid guidelines. |  |  |  |  |  |  |  |  |  |  |
| HEAVY = Caution—No Holdover Time Guidelines Exist |  |  |  |  |  |  |  |  |  |  |

August 2008

FAA TYPE II Holdover Time Guideline

### TABLE 2.  FAA Guidelines for Holdover Times SAE Type II FLUID MIXTURES 
as a Function of Weather Conditions and OUTSIDE AIR TEMPERATURE

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Type II Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 8:00 | 0:35-1:30 | 0:20-0:45 | 0:30-0:55 | 0:15-0:30 | 0:05-0:40 |  |
|  |  | 75/25 | 5:00 | 0:25-1:00 | 0:15-0:30 | 0:20-0:45 | 0:10-0:25 | 0:05-0:25 |  |
|  |  | 50/50 | 3:00† | 0:15-0:30 | 0:05-0:15 | 0:05-0:15 | 0:05-0:10 |  |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 8:00† | 0:20-1:05 | 0:15-0:30 | ***0:15-0:45 | ***0:10-0:20 | CAUTION: No holdover time guidelines exist |  |
|  |  | 75/25 | 5:00† | 0:20-0:55 | 0:10-0:20 | ***0:15-0:30 | ***0: 05-0: 15 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 8:00† | 0:15-0:20 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | SAE Type II fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when SAE Type II fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air              temperature range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- SAE TYPE II FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 2A.  FAA Guidelines for Holdover Times ABAX (SPCA) ECOWING 26 Type II Fluid Mixtures 
as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type II Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 8:00 | 1:25-2:35 | 0:40-1:00 | 0:50-1:35 | 0:40-0:50 | 0:20-1:25 |  |
|  |  | 75/25 | 5:00 | 1:05-1:55 | 0:25-0:45 | 0:45-1:05 | 0:25-0:35 | 0:10-1:00 |  |
|  |  | 50/50 | 3:00† | 0:30-0:45 | 0:10-0:20 | 0:15-0:25 | 0:05-0:10 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 8:00† | 0:45-2:15 | 0:35-0:55 | ***0:30-1:10 | ***0:15-0:35 |  |  |
|  |  | 75/25 | 5:00† | 0:35-1:15 | 0:25-0:40 | ***0:20-0:50 | ***0:15-0:25 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 8:00† | 0:25-0:45 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | ABAX Ecowing 26 Type II fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when ABAX Ecowing 26 Type II fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature
                   range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- ABAX ECOWING 26 TYPE II FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 2B.  FAA Guidelines for Holdover Times CLARIANT SAFEWING MP II 2025 ECO Type II Fluid Mixtures 
as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type II Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 8:00 | 1:30-2:05 | 0:40-1:10 | 0:40-1:00 | 0:25-0:35 | 0:10-1:15 |  |
|  |  | 75/25 | 5:00 | 0:55-1:45 | 0:25-0:45 | 0:25-0:45 | 0:20-0:25 | 0:05-0:50 |  |
|  |  | 50/50 | 3:00† | 0:20-0:35 | 0:05-0:15 | 0:10-0:15 | 0:05-0:10 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 8:00† | 0:45-1:50 | 0:35-1:00 | ***0:35-1:05 | ***0:20-0:35 |  |  |
|  |  | 75/25 | 5:00† | 0:40-1:20 | 0:25-0:45 | ***0:30-0:40 | ***0:15-0:25 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 8:00† | 0:25-0:45 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | CLARIANT Safewing mpii 2025 ECO Type II fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when CLARIANT Safewing mpii 2025 ECO Type II fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

†  Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature
                   range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- CLARIANT SAFEWING MP II 2025 ECO TYPE II FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 2C.  FAA Guidelines for Holdover Times CLARIANT SAFEWING MP II Flight Type II Fluid Mixtures 
as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type II Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 8:00 | 3:30-4:00 | 1:00-1:35 | 1:20-2:00 | 0:45-1:25 | 0:10-1:30 |  |
|  |  | 75/25 | 5:00 | 2:30-4:00 | 0:40-1:20 | 1:15-2:00 | 0:30-0:55 | 0:05-1:20 |  |
|  |  | 50/50 | 3:00† | 0:55-1:45 | 0:10-0:25 | 0:20-0:30 | 0:10-0:15 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 8:00† | 0:55-1:45 | 0:40-1:05 | ***0:35-1:30 | ***0:25-0:45 |  |  |
|  |  | 75/25 | 5:00† | 0:40-1:10 | 0:20-0:40 | ***0:25-1:10 | ***0:30-0:40 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 8:00† | 0:30-0:50 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | CLARIANT Safewing mp ii Flight Type II fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when CLARIANT Safewing mp ii Flight Type II fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail
            	    †  Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature
                      range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- CLARIANT SAFEWING MP II FLIGHT TYPE II FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

TABLE 2D.  FAA Guidelines for Holdover Times KILFROST ABC-2000 Type II Fluid Mixtures 
as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type II Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 8:00 | 1:30-3:05 | 0:30-1:00 | 0:55-1:35 | 0:40-0:50 | 0:15-1:10 |  |
|  |  | 75/25 | 5:00 | 1:40-3:30 | 0:30-1:05 | 0:45-1:15 | 0:40-0:50 | 0:15-1:40 |  |
|  |  | 50/50 | 3:00† | 1:00-2:10 | 0:15-0:30 | 0:15-0:25 | 0:05-0:15 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 8:00† | 0:35-1:25 | 0:25-0:45 | ***0:25-0:50 | ***0:10-0:30 |  |  |
|  |  | 75/25 | 5:00† | 0:35-1:15 | 0:25-0:50 | ***0:25-0:55 | ***0:15-0:30 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 8:00† | 0:20-0:45 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | KILFROST ABC-2000 Type II fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when KILFROST ABC-2000 Type II fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below  -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature
                     range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT

- KILFROST ABC-2000 TYPE II FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 2E.  FAA Guidelines for Holdover Times KILFROST ABC-II PLUS Type II Fluid Mixtures 
as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type II Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other ‡ |
| -3 and above | 27 and above | 100/0 | 8:00 | 1:10-2:25 | 0:25-0:55 | 0:35-1:10 | 0:30-0:40 | 0:05-1:00 |  |
|  |  | 75/25 | 5:00 | 1:10-2:25 | 0:25-0:50 | 0:30-1:00 | 0:20-0:40 | 0:05-0:50 |  |
|  |  | 50/50 | 3:00† | 0:15-0:45 | 0:15-0:35 | 0:05-0:25 | 0:05-0:15 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 8:00† | 0:30-1:05 | 0:15-0:35 | ***0:15-0:45 | ***0:10-0:30 |  |  |
|  |  | 75/25 | 5:00† | 0:20-0:55 | 0:15-0:35 | ***0:15-0:30 | ***0:10-0:20 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 8:00† | 0:15-0:20 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | KILFROST ABC-II PLUS Type II fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when KILFROST ABC-II PLUS Type II fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 degrees Celsius (32 degrees Fahrenheit) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature
                   range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- KILFROST ABC-II PLUS TYPE II FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 2F.  FAA Guidelines for Holdover Times KILFROST ABC-K PLUS Type II Fluid Mixtures

- as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type II Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other ‡ |
| -3 and above | 27 and above | 100/0 | 8:00 | 2:15-3:45 | 1:00-1:40 | 1:50-2:00 | 1:00-1:25 | 0:20-2:00 |  |
|  |  | 75/25 | 5:00 | 1:40-2:30 | 0:35-1:10 | 1:25-2:00 | 0:50-1:10 | 0:15-2:00 |  |
|  |  | 50/50 | 3:00† | 0:35-1:05 | 0:05-0:15 | 0:20-0:30 | 0:10-0:15 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 8:00† | 0:30-1:05 | 0:50-1:25 | ***0:25-1:00 | ***0:15-0:35 |  |  |
|  |  | 75/25 | 5:00† | 0:25-1:25 | 0:35-1:05 | ***0:25-0:55 | ***0:05-0:30 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 8:00† | 0:30-0:55 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | KILFROST ABC-K PLUS Type II fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when KILFROST ABC-K PLUS Type II fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 degrees Celsius (32 degrees Fahrenheit) only

*** No holdover time guidelines exist for this condition below  -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature
                    range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- KILFROST ABC-K PLUS TYPE II FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 2G.  FAA Guidelines for Holdover Times NEWAVE AEROCHEMICAL FCY-2 TYPE II Mixtures 
as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type II Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 8:00 | 1:15-2:25 | 0:30-0:55 | 0:35-1:05 | 0:25-0:35 | 0:05-0:45 |  |
|  |  | 75/25 | 5:00 | 0:50-1:30 | 0:20-0:40 | 0:25-0:45 | 0:15-0:25 | 0:05-0:25 |  |
|  |  | 50/50 | 3:00† | 0:25-0:35 | 0:15-0:25 | 0:10-0:20 | 0:05-0:10 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 8:00† | 0:45-1:30 | 0:15-0:30 | ***0:20-0:45 | ***0:15-0:20 |  |  |
|  |  | 75/25 | 5:00† | 0:30-1:05 | 0:10-0:20 | ***0:15-0:30 | ***0:05-0:15 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 8:00† | 0:25-0:35 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | NEWAvE AEROCHEMICAL FCY-2 Type II fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when NEWAYE AEROCHEMICAL FCY-2 Type II fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature
                    range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- NEWAVE AEROCHEMICAL FCY-2 TYPE II FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

TABLE 2H.  FAA Guidelines for Holdover Times OCTAGON E-MAX Type II Fluid Mixtures 
as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type II Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 8:00 | 2:05-3:45 | 0:40-1:20 | 0:45-1:35 | 0:30-0:40 | 0:15-1:30 |  |
|  |  | 75/25 | 5:00 | 1:25-2:50 | 0:25-0:55 | 0:40-1:10 | 0:20-0:30 | 0:10-1:05 |  |
|  |  | 50/50 | 3:00† | 0:30-0:55 | 0:10-0:25 | 0:15-0:30 | 0:10-0:15 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 8:00† | 0:50-1:45 | 0:35-1:10 | ***0:35-1:00 | ***0:20-0:30 |  |  |
|  |  | 75/25 | 5:00† | 0:30-1:20 | 0:25-0:50 | ***0:35-1:05 | ***0:15-0:30 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 8:00† | 0:20-0:35 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | OCTAGON E-MAX Type II fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when OCTAGON E-MAX Type II fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below  -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature
                    range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- OCTAGON E-MAX TYPE II FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

FAA TYPE III HOLDOVER TIME GUIDELINE

- Table 3.  FAA Guidelines for Holdover Times SAE Type III Fluid Mixture as a Function of Weather Conditions and OUTSIDE AIR TEMPERATURE.

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit | - Type III Fluid Concentration Neat Fluid/Water (Volume %/Volume %) | - Active Frost | Freezing Fog | Snow/Snow Grains |  |  | Freezing - Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | - Other‡ |
|  |  |  |  |  | Very Light | Light | Moderate |  |  |  |  |
| -3 and above | 27 and above | 100/0 | 2:00 | 0:20 - 0:40 | 0:35 - 0:40 | 0:20 - 0:35 | 0:10 - 0:20 | 0:10 - 0:20 | 0:08 - 0:10 | 0:06 - 0:20 |  |
|  |  | 75/25 | 1:00 | 0:15 - 0:30 | 0:25 - 0:35 | 0:15 - 0:25 | 0:08 - 0:15 | 0:08 - 0:15 | 0:06 - 0:10 | 0:02 - 0:10 |  |
|  |  | 50/50 | 0:30 | 0:10 - 0:20 | 0:15 - 0:20 | 0:08 - 0:15 | 0:04 - 0:08 | 0:05 - 0:09 | 0:04 - 0:06 |  |  |
| below -3 to -10 | below 27 to 14 | 100/0 | 2:00 | 0:20 - 0:40 | 0:30 - 0:35 | 0:15 - 0:30 | 0:09 - 0:15 | 0:10 - 0:20 | 0:08 - 0:10 |  |  |
|  |  | 75/25 | 1:00 | 0:15 - 0:30 | 0:25 - 0:30 | 0:10 - 0:25 | 0:07 - 0:10 | 0:09 - 0:12 | 0:06 - 0:09 |  |  |
| Below -10 | below 14 | 100/0 | 2:00 | 0:20 - 0:40 | 0:30 - 0:35 | 0:15 - 0:30 | 0:08 - 0:15 | CAUTION: No holdover time guidelines exist |  |  |  |
| SAE Type III fluid may be used below -10 C (14 F), provided the freezing point of the fluid is at least 7 C (13 F) below OAT and aerodynamic acceptance criteria are met. Consider the use of SAE Type I when Type III fluid cannot be used. |  |  |  |  |  |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

*Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

**This column is for use at temperatures above 0 °C (32 °F) only

‡Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST WILL REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- SAE TYPE III FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR - AND DOES NOT PROVIDE - PROTECTION DURING FLIGHT.

August 2008

FAA TYPE IV Holdover Time GuidelineS

- TABLE 4.  FAA Guidelines for Holdover Times SAE Type IV Fluid Mixtures as a Function of
Weather Conditions and OUTSIDE AIR TEMPERATURE

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 1:15-2:30 | 0:35-1:15 | 0:40-1:10 | 0:25-0:40 | 0:10-1:05 |  |
|  |  | 75/25 | 5:00 | 1:05-1:45 | 0:20-0:55 | 0:35-0:50 | 0:15-0:30 | 0:05-0:40 |  |
|  |  | 50/50 | 3:00† | 0:15-0:35 | 0:05-0:15 | 0:10-0:20 | 0:05-0:10 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 0:20-1:20 | 0:20-0:40 | ***0:20-0:45 | ***0:10-0:25 |  |  |
|  |  | 75/25 | 5:00† | 0:25-0:50 | 0:15-0:35 | ***0:15-0:30 | ***0:10-0:20 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:15-0:40 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | SAE Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when SAE Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

*  Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature       range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- SAE TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

TABLE 4A.  FAA Guidelines for Holdover Times ABAX (SPCA) AD-480 Type IV Fluid Mixtures as a 
function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 2:00-3:30 | 0:40-1:20 | 0:50-1:30 | 0:35-0:55 | 0:15-1:35 |  |
|  |  | 75/25 | 5:00 | 1:30-2:45 | 0:30-1:05 | 0:50-1:15 | 0:30-0:45 | 0:10-1:15 |  |
|  |  | 50/50 | 3:00† | 0:30-0:45 | 0:10-0:20 | 0:15-0:25 | 0:05-0:15 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 0:20-1:20 | 0:30-0:55 | ***0:25-1:20 | ***0:15-0:30 |  |  |
|  |  | 75/25 | 5:00† | 0:25-0:50 | 0:20-0:45 | ***0:25-1:05 | ***0:15-0:30 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:15-0:40 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | ABAX AD-480 Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when ABAX AD-480 Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 degrees Celsius (32 degrees Fahrenheit) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature   range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- ABAX AD-480 TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 4B.  FAA Guidelines for Holdover Times CLARIANT SAFEWING MP IV 2001 Type IV Fluid 
 Mixtures as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 1:20-3:20 | 1:00-1:55 | 0:55-1:55 | 0:40-1:00 | 0:15-2:00 |  |
|  |  | 75/25 | 5:00 | 1:20-2:00 | 0:35-1:00 | 0:35-1:10 | 0:25-0:35 | 0:10-1:25 |  |
|  |  | 50/50 | 3:00† | 0:15-0:40 | 0:10-0:20 | 0:10-0:20 | 0:05-0:15 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 0:45-1:35 | 0:30-0:50 | ***0:55-1:35 | ***0:30-0:45 |  |  |
|  |  | 75/25 | 5:00† | 0:30-1:00 | 0:20-0:35 | ***0:40-1:10 | ***0:20-0:30 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:20-0:45 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | CLARIANT SAFEWING MP IV 2001 Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when CLARIANT SAFEWING MP IV 2001 Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- CLARIANT SAFEWING MP IV 2001 TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 4C.  FAA Guidelines for Holdover Times CLARIANT SAFEWING MP IV 2012 PROTECT Type IV Fluid Mixtures as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 1:15-2:30 | 0:40-1:15 | 0:40-1:10 | 0:25-0:45 | 0:10-1:05 |  |
|  |  | 75/25 | 5:00 | 1:10-2:05 | 0:25-0:55 | 0:35-0:50 | 0:15-0:30 | 0:05-0:40 |  |
|  |  | 50/50 | 3:00† | 0:25-0:45 | 0:15-0:25 | 0:15-0:20 | 0:05-0:10 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 0:45-1:45 | 0:20-0:40 | ***0:25-0:45 | ***0:15-0:25 |  |  |
|  |  | 75/25 | 5:00† | 0:25-1:05 | 0:20-0:40 | ***0:15-0:30 | ***0:10-0:20 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:20-0:45 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | CLARIANT SAFEWING MP IV 2012 PROTECT Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when CLARIANT SAFEWING MP IV 2012 PROTECT Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- CLARIANT SAFEWING MP IV 2012 PROTECT TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 4D.  FAA Guidelines for Holdover Times CLARIANT SAFEWING MP IV LAUNCH Type IV Fluid 
Mixtures as a function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 4:00-4:00 | 1:05-1:45 | 1:30-2:00 | 1:00-1:40 | 0:15-1:40 |  |
|  |  | 75/25 | 5:00 | 3:40-4:00 | 1:00-1:45 | 1:40-2:00 | 0:45-1:15 | 0:10-1:45 |  |
|  |  | 50/50 | 3:00† | 1:25-2:45 | 0:25-0:45 | 0:30-0:50 | 0:20-0:25 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 1:00-1:55 | 0:50-1:20 | ***0:35-1:40 | ***0:25-0:45 |  |  |
|  |  | 75/25 | 5:00† | 0:40-1:20 | 0:45-1:25 | ***0:25-1:10 | ***0:25-0:45 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:30-0:50 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | CLARIANT SAFEWING MPIV LAUNCH Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when CLARIANT SAFEWING MPIV LAUNCH Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 degrees Celsius (32 degrees Fahrenheit) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- CLARIANT SAFEWING MP IV LAUNCH TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 4E.  FAA Guidelines for Holdover Times DOW UCAR™ ULTRA+ ADF/AAF Type IV Fluid 
Mixtures as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** |  | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 1:35-3:35 | 0:35-1:15 | 0:45-1:35 | 0:25-0:40 | 0:10-1:20 |  |  |
|  |  | 75/25 |  |  |  |  |  | CAUTION: No holdover time guidelines exist |  |  |
|  |  | 50/50 |  |  |  |  |  |  |  |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 1:25-3:00 | 0:25-0:55 | ***0:45-1:25 | ***0:30-0:45 |  |  |  |
|  |  | 75/25 |  |  |  |  |  |  |  |  |
| below -14 to -24 | below 7 to -12 | 100/0 | 12:00† | 0:40-2:10 | 0:20-0:45 |  |  |  |  |  |
| below -24 | below -12 | 100/0 | DOW UCAR ULTRA+ Type IV fluid may be used below -24 °C (-12 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when DOW UCAR ULTRA+ Type IV fluid cannot be used. |  |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

†Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- DOW UCAR ULTRA+ TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 4F.  FAA Guidelines for Holdover Times DOW UCAR™ ENDURANCE EG106 Type IV Fluid 
Mixtures as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** |  | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 2:05-3:10 | 0:40-1:20 | 1:10-2:00 | 0:50-1:15 | 0:20-2:00 |  |  |
|  |  | 75/25 |  |  |  |  |  | CAUTION: No holdover time guidelines exist |  |  |
|  |  | 50/50 |  |  |  |  |  |  |  |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 1:50-3:20 | 0:30-1:05 | ***0:55-1:50 | ***0:45-1:10 |  |  |  |
|  |  | 75/25 |  |  |  |  |  |  |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:30-1:05 | 0:15-0:30 |  |  |  |  |  |
| below -25 | below -13 | 100/0 | DOW UCAR ENDURANCE EG 106 Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when DOW UCAR ENDURANCE EG 106 Type IV fluid cannot be used. |  |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature  range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- DOW UCAR ENDURANCE EG 106 TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

TABLE 4G.  FAA Guidelines for Holdover Times DOW UCARTM FLIGHTGUARD AD-480 Type IV Fluid

Mixtures as a function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 2:00-3:30 | 0:40-1:20 | 0:50-1:30 | 0:35-0:55 | 0:15-1:35 |  |
|  |  | 75/25 | 5:00 | 1:30-2:45 | 0:30-1:05 | 0:50-1:15 | 0:30-0:45 | 0:10-1:15 |  |
|  |  | 50/50 | 3:00† | 0:30-0:45 | 0:10-0:20 | 0:15-0:25 | 0:05-0:15 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 0:20-1:20 | 0:30-0:55 | ***0:25-1:20 | ***0:15-0:30 |  |  |
|  |  | 75/25 | 5:00† | 0:25-0:50 | 0:20-0:45 | ***0:25-1:05 | ***0:15-0:30 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:15-0:40 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | DOW UCAR FLIGHTGUARD AD-480 Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when DOW UCAR FLIGHTGUARD AD-480 Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 degrees Celsius (32 degrees Fahrenheit) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- DOW UCAR FLIGHTGUARD AD-480 TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 4H.  FAA Guidelines for Holdover Times KILFROST ABC-S Type IV Fluid Mixtures 
as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 2:35-4:00 | 1:00-1:40 | 1:20-1:50 | 1:00-1:25 | 0:20-1:15 |  |
|  |  | 75/25 | 5:00 | 1:05-1:45 | 0:30-0:55 | 0:45-1:10 | 0:35-0:50 | 0:10-0:50 |  |
|  |  | 50/50 | 3:00† | 0:20-0:35 | 0:05-0:15 | 0:15-0:20 | 0:05-0:10 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 0:45-2:05 | 0:45-1:20 | ***0:20-1:00 | ***0:10-0:30 |  |  |
|  |  | 75/25 | 5:00† | 0:25-1:00 | 0:25-0:50 | ***0:20-1:10 | ***0:10-0:35 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:20-0:40 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | KILFROST ABC-S Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when KILFROST ABC-S Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 degrees Celsius (32 degrees Fahrenheit) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- KILFROST ABC-S TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 4I.  FAA Guidelines for Holdover Times KILFROST ABC-S PLUS Type IV Fluid Mixtures
 as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 2:10-4:00 | 1:15-2:00 | 1:50-2:00 | 1:05-2:00 | 0:25-2:00 |  |
|  |  | 75/25 | 5:00 | 1:25-2:40 | 0:45-1:15 | 1:00-1:20 | 0:30-0:50 | 0:10-1:20 |  |
|  |  | 50/50 | 3:00† | 0:30-0:55 | 0:15-0:30 | 0:15-0:40 | 0:15-0:20 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 0:55-3:30 | 1:00-1:45 | ***0:25-1:35 | ***0:20-0:30 |  |  |
|  |  | 75/25 | 5:00† | 0:45-1:50 | 0:35-1:00 | ***0:20-1:10 | ***0:15-0:25 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:40-1:00 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | KILFROST ABC-S PLUS Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when KILFROST ABC-S PLUS Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 degrees Celsius (32 degrees Fahrenheit) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

†  Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- KILFROST ABC-S PLUS TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 4J.  FAA Guidelines for Holdover Times LYONDELL ARCTIC SHIELDTM Type IV Fluid 
Mixtures as a function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/ Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 1:55-3:10 | 0:50-1:25 | 0:55-1:40 | 0:45-1:05 | 0:15-1:25 |  |
|  |  | 75/25 | 5:00 | 1:20-2:15 | 0:40-1:05 | 0:55-1:25 | 0:30-0:45 | 0:05-1:20 |  |
|  |  | 50/50 | 3:00† | 0:35-0:45 | 0:20-0:35 | 0:20-0:30 | 0:10-0:15 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 1:00-2:25 | 0:45-1:15 | ***0:25-1:30 | ***0:25-0:30 |  |  |
|  |  | 75/25 | 5:00† | 0:50-1:45 | 0:35-0:55 | ***0:30-1:15 | ***0:25-0:30 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:25-0:45 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | LYONDELL ARCTIC SHIELD Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when LYONDELL ARCTIC SHIELD Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 degrees Celsius (32 degrees Fahrenheit) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- LYONDELL ARCTIC SHIELD TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 4K.  FAA Guidelines for Holdover Times OCTAGON MAX-FLIGHT Type IV Fluid 
Mixtures as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 2:40-4:00 | 0:50-1:35 | 0:55-2:00 | 0:35-1:00 | 0:15-1:15 |  |
|  |  | 75/25 | 5:00 | 2:05-3:15 | 0:45-1:45 | 1:15-2:00 | 0:35-1:10 | 0:10-0:40 |  |
|  |  | 50/50 | 3:00† | 0:55-1:45 | 0:25-1:15 | 0:35-1:00 | 0:15-0:30 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 0:50-2:30 | 0:25-0:50 | ***0:25-1:10 | ***0:20-0:40 |  |  |
|  |  | 75/25 | 5:00† | 0:30-1:05 | 0:20-0:50 | ***0:20-1:00 | ***0:15-0:30 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:20-0:45 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | OCTAGON MAX-FLIGHT Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when OCTAGON MAX-FLIGHT Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- OCTAGON MAX-FLIGHT TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 4L.  FAA Guidelines for Holdover Times OCTAGON MAX-FLIGHT 04 Type IV Fluid Mixtures 
as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Active Frost | Freezing Fog | Snow/Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 2:40-4:00 | 1:25-2:00 | 2:00-2:00 | 1:10-1:30 | 0:20-2:00 |  |
|  |  | 75/25 | 5:00 | 2:05-3:15 | 1:05-2:00 | 1:50-2:00 | 1:00-1:20 | 0:20-2:00 |  |
|  |  | 50/50 | 3:00† | 0:55-1:45 | 0:25-1:15 | 0:35-1:10 | 0:25-0:35 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 0:50-2:30 | 0:35-1:10 | ***0:25-1:30 | ***0:20-0:40 |  |  |
|  |  | 75/25 | 5:00† | 0:30-1:05 | 0:40-1:20 | ***0:20-1:00 | ***0:15-0:30 |  |  |
| below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:20-0:45 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | OCTAGON MAX-FLIGHT 04 Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when OCTAGON MAX-FLIGHT 04 Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature   range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- OCTAGON MAX-FLIGHT 04 TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

- TABLE 4M.  FAA Guidelines for Holdover Times OCTAGON MAXFLO Type IV Fluid Mixtures 
as a Function of Weather Conditions and Outside Air Temperature

CAUTION:  THIS TABLE IS FOR DEPARTURE PLANNING ONLY AND SHOULD BE USED IN CONJUNCTION WITH PRETAKEOFF CHECK PROCEDURES.

| Outside Air Temperature |  | Manufacturer Specific Type IV Fluid Concentration Neat-Fluid/Water (Volume %/Volume %) | Approximate Holdover Times Under Various Weather Conditions (hours: minutes) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Degrees Celsius | Degrees Fahrenheit |  | Frost | Freezing Fog | Snow /Snow Grains | Freezing Drizzle* | Light Freezing Rain | Rain on Cold Soaked Wing** | Other‡ |
| -3 and above | 27 and above | 100/0 | 12:00 | 2:20-3:35 | 0:40-1:30 | 1:20-2:00 | 0:30-1:00 | 0:10-2:00 |  |
|  |  | 75/25 | 5:00 | 1:25-2:00 | 0:20-0:55 | 0:40-1:05 | 0:20-0:35 | 0:05-1:15 |  |
|  |  | 50/50 | 3:00† | 0:20-0:40 | 0:05-0:15 | 0:10-0:20 | 0:05-0:10 | CAUTION: No holdover time guidelines exist |  |
| below -3 to -14 | below 27 to 7 | 100/0 | 12:00† | 1:10-2:20 | 0:25-1:00 | ***0:35-1:45 | ***0:30-0:50 |  |  |
|  |  | 75/25 | 5:00† | 0:40-1:25 | 0:15-0:40 | ***0:35-1:15 | ***0:15-0:30 |  |  |
| Below -14 to -25 | below 7 to -13 | 100/0 | 12:00† | 0:30-1:00 | 0:15-0:30 |  |  |  |  |
| below -25 | below -13 | 100/0 | OCTAGON MAXFLO Type IV fluid may be used below -25 °C (-13 °F) provided the freezing point of the fluid is at least 7 °C (13 °F) below the OAT and the aerodynamic acceptance criteria are met. Consider use of SAE Type I when OCTAGON MAXFLO Type IV fluid cannot be used. |  |  |  |  |  |  |

THE RESPONSIBILITY FOR THE APPLICATION OF THESE DATA REMAINS WITH THE USER.

* Use light freezing rain holdover times if positive identification of freezing drizzle is not possible

** This column is for use at temperatures above 0 °C (32 °F) only

*** No holdover time guidelines exist for this condition below -10 C (14 F)

‡ Snow pellets, ice pellets, heavy snow, moderate and heavy freezing rain, and hail

† Radiational cooling during active frost conditions may reduce holdover times when operating close to the lower end of the outside air temperature   range.

CAUTIONS:

- THE TIME OF PROTECTION WILL BE SHORTENED IN HEAVY WEATHER CONDITIONS. HEAVY PRECIPITATION RATES OR HIGH MOISTURE CONTENT, HIGH WIND VELOCITY, OR JET BLAST MAY REDUCE HOLDOVER TIME BELOW THE LOWEST TIME STATED IN THE RANGE. HOLDOVER TIME MAY BE REDUCED WHEN AIRCRAFT SKIN TEMPERATURE IS LOWER THAN OAT.

- OCTAGON MAXFLO TYPE IV FLUID USED DURING GROUND DEICING/ANTI-ICING IS NOT INTENDED FOR AND DOES NOT PROVIDE PROTECTION DURING FLIGHT.

August 2008

TABLE 5.  FAA Guidelines for the Application of SAE Type II, Type III, and Type IV Fluid Mixtures 
Minimum Concentrations as a Function of Outside Air Temperature

Concentrations in % Volume

| Outside Air Temperature (OAT) | One-step Procedure Deicing/Anti-icing1 | Two-step Procedure |  |
| --- | --- | --- | --- |
|  |  | First step: Deicing | Second step: Anti-icing1, 2, |
| -3 C (27 F) and above | 50/50 Heated3 Types II, III or IV | Heated water or a heated mix of Types I, II, III or IV and water | 50/50 Type II, III, or IV |
| Below -3 C (27 F) to -14 C (7 F) | 75/25 Heated3 Types II, III or IV | Heated suitable mix of Types I, II, III or IV, and water with a freezing point not more than 3 C (5 F) above actual OAT | 75/25 Type II, III, or IV |
| below -14 C (7 F) to -25 C (-13 F) | 100/0 Heated3 Types II, III or IV | Heated suitable mix of Types I, II, III or IV, and water with a freezing point not more than 3 C (5 F) above actual OAT | 100/0 Type II, III, or IV |
| Below -25 C (-13 F) | SAE Type II/IV fluid may be used below -25 C (-13 F) provided that the OAT is at or above the LOUT. SAE Type III fluid may be used below -10°C (14°F) provided that the OAT is at or above the LOUT Consider the use of SAE Type I when Type II, III, or IV fluid cannot be used. |  |  |
| 1) Fluids must only be used at temperatures above their lowest operational use temperature (LOUT). 2) To be applied before first step fluid freezes, typically within 3 minutes. 3) Clean aircraft may be anti-iced with unheated Types II, III, or IV fluid. |  |  |  |
| NOTES: - For heated fluids, a fluid temperature not less than 60 C (140 F) at the nozzle is desirable. Upper temperature limit shall not exceed fluid and aircraft manufacturers’ recommendations. - The lowest operational use temperature (LOUT) for a given fluid is the higher of: - The lowest temperature at which the fluid meets the aerodynamic acceptance test for a given aircraft type, or - The actual freezing point of the fluid plus a freezing point buffer of 7°C (13°F). |  |  |  |
| CAUTIONS: - Wing skin temperatures may differ and in some cases may be lower than OAT. A stronger mix (more glycol) can be used under these conditions. - As fluid freezing may occur, 50/50 Types II, III, or IV fluid shall not be used for the anti-icing step of a cold-soaked wing as indicated by frost or ice on the lower surface of the wing in the area of the fuel tank. - An insufficient amount of anti-icing fluid, especially in the second step of a two-step procedure, may cause a substantial loss of holdover time, particularly when using a Type I fluid mixture for the first step (deicing) of a two-step procedure. |  |  |  |

August 2008

TABLE 6

LOWEST ON-WING VISCOSITY VALUES FOR ANTI-ICING FLUIDS

(See Page 35 for Table 6 Notes)

| Table 6-1: Type II Anti-Icing Fluids |  |  |  |
| --- | --- | --- | --- |
| Fluid Name | Fluid Dilution | Lowest On-Wing Viscosity a (mPa.s) |  |
|  |  | Manufacturer Method | AIR 9968 revision a method |
| ABAX (SPCA) Ecowing 26 | 100/0 | 4,900 e | 4,600 g |
|  | 75/25 | 2,200 g | 2,200 g |
|  | 50/50 | 50 g | 50 g |
| Clariant Safewing MP II 2025 ECO | 100/0 | 5,500 b | 5,750 g |
|  | 75/25 | 10,000 b | 10,000 g |
|  | 50/50 | 3,000 b | 3,250 g |
| Clariant Safewing MP II Flight | 100/0 | 3,340 g | 3,340 g |
|  | 75/25 | 17,500 g | 17,500 g |
|  | 50/50 | 11,500 g | 11,500 g |
| Clariant Safewing MP II 1951 | 100/0 | 2,500 b | 2,750 g |
|  | 75/25 | 2,900 b | 3,000 g |
|  | 50/50 | 50 b | 50 g |
| Kilfrost ABC-3 | 100/0 | 2,500 c | 2,500 j |
|  | 75/25 | 2,000 c | 2,000 j |
|  | 50/50 | 400 c | 400 j |
| Kilfrost ABC-2000 | 100/0 | 2,350 c | 2,350 g |
|  | 75/25 | 3,000 c | 3,000 j |
|  | 50/50 | 1,000 c | 1,000 j |
| Kilfrost ABC-II Plus | 100/0 | 3,600 c | 3,600 g |
|  | 75/25 | 4,000 c | 4,000 j |
|  | 50/50 | 1,000 c | 1,000 j |
| Kilfrost ABC-K Plus | 100/0 | 2,850 c | 2,640 g |
|  | 75/25 | 12,650 c | 12,650 c |
|  | 50/50 | 4,200 c | 5,260 g |
| Newave Aerochemical FCY-2 | 100/0 | 7,000 c | 8,920 g |
|  | 75/25 | 18,550 c | 18,550 c |
|  | 50/50 | 6,750 c | 7,030 g |
| Octagon E Max II | 100/0 | 13,520 d | 13,520 g |
|  | 75/25 | 11,400 g | 11,400 g |
|  | 50/50 | 2,820 g | 2,820 g |

| Table 6-2: Type III Anti-Icing Fluids |  |  |  |
| --- | --- | --- | --- |
| Fluid Name | Fluid Dilution | Lowest On-Wing Viscosity a (mPa.s) |  |
|  |  | Manufacturer Method | AIR 9968 revision a method |
| Clariant Safewing MP III 2031 ECO | 100/0 | 30 h | Not Applicable |
|  | 75/25 | 55 h | Not Applicable |
|  | 50/50 | 10 h | Not Applicable |

| Table 6.3 Type IV Anti-Icing Fluids |  |  |  |
| --- | --- | --- | --- |
| Fluid Name | Fluid Dilution | Lowest On-Wing Viscosity a (mPa.s) |  |
|  |  | Manufacturer Method | SAE AIR 9968 revision a method |
| ABAX (SPCA) AD-480 | 100/0 | 15,200 e | 12,800 c |
|  | 75/25 | 16,000 e | 12,400 c |
|  | 50/50 | 4,000 e | 3,800 g |
| Clariant Safewing MP IV 2001 | 100/0 | 18,000 b | 18,000 c |
|  | 75/25 | 8,000 b | 11,500 g |
|  | 50/50 | 1,200 b | 1,750 g |
| Clariant Safewing MP IV 2012 Protect | 100/0 | 7,800 b | 7,250 g |
|  | 75/25 | 17,800 b | 17,700 c |
|  | 50/50 | 4,500 b | 4,250 g |
| Clariant Safewing MP IV Launch | 100/0 | 7,550 g | 7,550 g |
|  | 75/25 | 18,000 g | 18,000 g |
|  | 50/50 | 17,800 g | 17,800 g |
| Dow UCAR ADF/AAF ULTRA+ | 100/0 | 36,000 f | 28,000 c |
|  | 75/25 | Dilution Not Applicable | Dilution Not Applicable |
|  | 50/50 | Dilution Not Applicable | Dilution Not Applicable |
| Dow UCAR Endurance EG106 | 100/0 | 24,850 f | 2,230 g |
|  | 75/25 | Dilution Not Applicable | Dilution Not Applicable |
|  | 50/50 | Dilution Not Applicable | Dilution Not Applicable |
| Dow UCAR FlightGuard AD-480 | 100/0 | 15,200 e | 12,800 c |
|  | 75/25 | 16,000 e | 12,400 c |
|  | 50/50 | 4,000 e | 3,800 g |
| Kilfrost ABC-S | 100/0 | 17,000 c | 17,000 c |
|  | 75/25 | 12,000 c | 12,000 c |
|  | 50/50 | 2,000 c | 2,000 j |
| Kilfrost ABC-S PLUS | 100/0 | 17,900 c | 17,900 c |
|  | 75/25 | 18,300 c | 18,300 c |
|  | 50/50 | 7,500 c | 7,500 j |
| Lyondell Arctic Shield | 100/0 | 23,150 i | 28,000 c |
|  | 75/25 | 21,700 i | 22,100 c |
|  | 50/50 | 6,400 i | 7,640 g |
| Octagon Max-Flight | 100/0 | 5,540 d | 5,540 g |
|  | 75/25 | 15,000 g | 15,000 g |
|  | 50/50 | 5,200 g | 5,200 g |
| Octagon Max-Flight 04 | 100/0 | 5,540 d | 5,540 g |
|  | 75/25 | 15,000 g | 15,000 g |
|  | 50/50 | 5,200 g | 5,200 g |
| Octagon MaxFlo | 100/0 | 8,670 g | 8,670 g |
|  | 75/25 | 8,200 g | 8,200 g |
|  | 50/50 | 2,200 g | 2,200 g |

NOTES

#### a. The SAE Aerospace Information Report (AIR) 9968 Revision A (December 2004) viscosity method should only be used for field verification and auditing purposes; when in doubt as to which method is appropriate, use the manufacturer method.

#### b. Brookfield Spindle SC4-34/13R, small sample adapter, 10 mL of fluid, at 20°C, 0.3 rpm, for 
15 minutes 0 seconds.

#### c. Brookfield Spindle LV2-disc with guard leg, 150 mL of fluid, at 20°C, 0.3 rpm, for 
10 minutes 0 seconds.

#### d. Brookfield Spindle LV1 with guard leg, 500 mL of fluid, at 20°C, 0.3 rpm, for 
33 minutes 20 seconds.

#### e. Brookfield Spindle SC4-34/13R, small sample adapter, 10 mL of fluid, at 20°C, 0.3 rpm, for 
30 minutes 0 seconds.

#### f. Brookfield Spindle SC4-31/13R, small sample adapter, 10 mL of fluid, at 0°C, 0.3 rpm, for 
10 minutes 0 seconds.

#### g. Brookfield Spindle LV1 with guard leg, 500 mL of fluid, at 20ºC, 0.3 rpm, for 
10 minutes 0 seconds.

#### h. Brookfield Spindle LV0, UL-Adapter, 16 mL of fluid, at 20ºC, 0.3 rpm, for 
10 minutes 0 seconds.

#### i. Brookfield Spindle SC4-31/13R, small sample adapter, 9 mL of fluid, at 20°C, 0.3 rpm, for 
33 minutes 0 seconds.

#### j. Brookfield Spindle LV1 with guard leg, 150 mL of fluid, at 20ºC, 0.3 rpm, for 
10 minutes 0 seconds.

### SIGNIFICANCE OF TABLE 6.  The viscosity values of the fluids in Table 6 are those provided by the fluid manufacturers for holdover time testing. For the holdover time guidelines to be valid, the viscosity of the fluid on the wing shall not be lower than that listed in this table. The user should periodically ensure that the viscosity of a fluid sample taken from the wing is not lower than the value listed here.

Table 7.  List of Qualified (1) Deicing/Anti-Icing Fluids-Winter 2008-2009

- Qualified Type I Deicing/Anti-Icing Fluids

| - Company Name | - Fluid Name |
| --- | --- |
| - ABAX Industries (formerly SPCA) | - ABAX DE-950 |
| - ABAX Industries (formerly SPCA) | - ABAX DE-950 Colorless |
| - Arkton Ltd. | - Artica DG |
| - Aviation Xi’an High-Tech | - KHF-1 |
| - Battelle | - D3: Degradable by Design Deicer™ ADF 1006A |
| - Beijing Wangye Aviation Chem. Prod. Co. | - KLA-1 |
| - Beijing Wangye Aviation Chem. Prod. Co | - YJF-1 |
| - Chemical Specialists and Development | - Prist Wing Deicer |
| - Clariant GmbH | - Safewing MPI 1938 TF |
| - Clariant GmbH | - Safewing MPI 1938 TF Pre-mix |
| - Clariant GmbH | - Safewing MP I 1938 ECO (80) |
| - Clariant GmbH | - Safewing MP I 1938 ECO (80) Pre-mix |
| - Clariant GmbH | - Safewing MP I 1938 ECO |
| - Clariant GmbH | - Safewing EG I 1996 |
| - Dow Chemical Company | - UCAR™ ADF Concentrate |
| - Dow Chemical Company | - UCAR™ ADF XL-54 |
| - Dow Chemical Company | - UCAR™ PG ADF Concentrate |
| - Dow Chemical Company | - UCAR™ PG ADF Dilute 55/45 |
| - HOC Industries | - SafeTemp I ES |
| - HOC Industries | - SafeTemp I ES Plus |
| - Inland Technologies | - Duragly - P ready to use |
| - Inland Technologies | - Duragly - E ready to use |
| - Kilfrost | - Kilfrost DF PLUS |
| - Kilfrost | - Kilfrost DF PLUS (80) |
| - Kilfrost | - Kilfrost DF PLUS (88)® |
| - Kilfrost | - Kilfrost DFSUSTAIN ™ |
| - Lyondell Chemical Company | - ARCOPlus® |
| - Lyondell Chemical Company | - ARCTIC Plus® |
| - Newave Aerochemical Company | - FCY-1A |
| - Octagon Process | - EcoFlo |
| - Octagon Process | - Octaflo EF |
| - Octagon Process | - Octaflo EG |
| - Viterbo S.A. | - Jarkleer SAE Type I |

- Table 7.  List of Qualified (1) Deicing/Anti-Icing Fluids-Winter 2008-2009 (Continued)

- Qualified Type II Deicing/Anti-Icing Fluids

| - Company Name | - Fluid Name |
| --- | --- |
| - ABAX Industries (formerly SPCA) | - ABAX Ecowing 26 |
| - Clariant GmbH | - Safewing MP II 1951 |
| - Clariant GmbH | - Safewing MP II 2025 ECO |
| - Clariant GmbH | - Safewing MP II Flight |
| - Kilfrost | - Kilfrost ABC-II PLUS |
| - Kilfrost | - Kilfrost ABC-3 |
| - Kilfrost | - Kilfrost ABC-2000 |
| - Kilfrost | - Kilfrost ABC-K PLUS |
| - Newave Aerochemical Technology | - FCY-2 |
| - Octagon Process | - E-Max |

- Qualified Type III Deicing/Anti-Icing Fluids

| - Company Name | - Fluid Name |
| --- | --- |
| - Clariant GmbH | - Safewing MP III 2031 ECO |

- Qualified Type IV Deicing/Anti-Icing Fluids

| - Company Name | - Fluid Name |
| --- | --- |
| - ABAX (formerly SPCA) | - ABAX AD-480 |
| - Clariant GmbH | - Safewing MP IV 2001 |
| - Clariant GmbH | - Safewing MP IV 2012 Protect |
| - Clariant GmbH | - Safewing MP IV Launch |
| - Dow Chemical Company | - UCAR™ ADF/AAF ULTRA+ |
| - Dow Chemical Company | - UCAR™ Endurance EG106 |
| - Dow Chemical Company | - UCAR™ FlightGuard AD-480 |
| - Kilfrost | - Kilfrost ABC-S |
| - Kilfrost | - Kilfrost ABC-S Plus |
| - Lyondell Chemical Company | - ARCTIC Shield™ |
| - Octagon Process | - Max-Flight |
| - Octagon Process | - Max-Flight 04 |

¹ Qualified indicates that the fluid has been qualified solely to the requirements of the applicable SAE anti-icing and aerodynamic performance specifications in effect at the time of certification, as conducted by the Anti-Icing Materials International Laboratory at the University of Quebec at Chicoutimi, Canada, Web site: http://www.uqac.ca/amil/index.htm. For other specification requirements for Type I fluids, see SAE AMS 1424 (latest version) and for SAE Types II, III, and IV fluids, see SAE AMS 1428 (latest version). Fluids that qualify after the issuance of this list will appear in a later update.