NRC INSPECTION MANUAL	CQV

INSPECTION PROCEDURE 35025

INSPECTION OF COL HOLDER'S SURVEILLANCE OF

CONTRACTOR QUALITY ASSURANCE ACTIVITIES

PROGRAM APPLICABILITY: 2504

35025-01	INSPECTION OBJECTIVE

Verify that the COL holder's implementation of quality assurance (QA) responsibilities relating to surveillance of contractors is consistent with the Quality Assurance Program Description (QAPD) and other lower tier QA documents.

35025-02	BACKGROUND

This inspection is conducted to ensure that the COL holder is providing appropriate oversight of contracted safety-related activities.  In addition, this inspection will focus on issues/concerns/deficiencies in regard to the adequacy of the COL holder=s execution of contractor surveillance responsibilities that may have been identified during earlier QA inspections.  This inspection may be warranted based on the following:

02.01	Examination of surveillance records that disclosed substantive deficiencies, or issues, relating to the adequacy of COL holder=s execution of contractor surveillance responsibilities.

02.02	The results of surveillance activities that are insufficient to ascertain the general status of the contractor's implemented QA activities for the specific contract.

02.03	The general adequacy of the contractor=s QA program has not been previously established by a routine or generic inspection.

02.04	The COL holder has identified numerous and substantive deficiencies relating to the implementation of the contractor's QA program for ongoing or near-future activities; but neither the COL applicant/holder nor the contractor has initiated timely corrective action in this regard.

02.05	The surveillance records do not provide sufficient evidence of adequate verification of contractor adherence to material, test, inspection, and procurement document requirements.

If the concerns are related to the general, or generic adequacy of COL applicant/holder=s execution of its surveillance responsibilities, then the inspection may be selective in choosing a contractor surveillance activity.

In another case, the justification for such an inspection may relate to a single contractor; such as the need to verify applicant execution of follow up surveillance responsibilities relating to resolution of substantive deficiencies identified in a single contractor's established and/or executed QA program.

The selection of contractors for the inspection that will be conducted to evaluate the execution of surveillance responsibilities shall provide appropriate resolution of previously identified concerns.

The inspector must at all times keep in mind that this is an inspection of the COL applicant/holder and not of the contractor.  The primary purpose of the inspections will be to examine the implementation of the COL applicant/holder QA program surveillance responsibilities.

In the performance of this inspection, the inspector's observations and conclusions, relating to measuring the adequacy of COL applicant/holder=s surveillance activities, shall be based on applicable guidance.

35025-03	INSPECTION REQUIREMENTS & GUIDANCE

Verify the following activities were conducted (as required) per applicable documents (i.e. QAPD, Purchase Orders).

03.01	Procurement documents stated the scope of QA responsibilities delegated to the contractor.

03.02	COL applicant/holder=s plans and activities relating to the attainment of overall (reviews, inspection, audits) contractor surveillance objectives were met.

03.03	Corrective actions on previously identified deficiencies were completed  timely.

03.04	The stated scope and overall surveillance objectives of the plan were adequate for the activities performed.

03.05	Individuals assigned to perform the surveillance activity were qualified.

03.06	Pre-surveillance conferences were held.

03.07	Execution of surveillance by members of the COL applicant/holder's organization was completed.

03.08	Identification and summary of deficiencies were documented.

03.09	Planned surveillance objectives were properly documented.

Specific Guidance:  Review a sample of issues/concerns/deficiencies of COL holder=s execution of contractor surveillance responsibilities identified during earlier QA inspections.  The sample size is dependent on the number of previous issues.

35025-04	RESOURCE ESTIMATE

The resource estimate for this inspection procedure is approximately 80 hours of direct inspection effort.

35025-05	REFERENCES

NUREG 0800, Standard Review Plan, Section 17.5, AQuality Assurance Program Description - Design Certification, Early Site Permit and New License Applicants@

AQuality Assurance Program@

END

Exhibit 1: Revision History

Exhibit 1

Revision History Sheet for IP 35025

| Commitment Tracking Number | Issue Date | Description of Change | Training Needed | Training Completion Date | Comment Resolution Ascension # |
| --- | --- | --- | --- | --- | --- |
| N/A | 06/22/05 | 1. Initial issue to support inspections of operational programs described in IMC 2504, NON-ITAAC INSPECTIONS. | N/A | N/A | N/A |
| N/A | 10/03/07 CN 07-030 | 1. Incorporates SRP 17.5 guidance. 2. Completed 4 year commitment search. | N/A | N/A | ML063120126 |