Guide to Completing the Incident Complexity Analysis.

(Type 1, 2)

- If positive responses exceed, or are equal to, negative responses within any primary factor (A through G), the primary factor should be considered as a positive response.

- If any three of the primary factors (A through G) are positive responses, this indicates the fire situation is or is predicted to be of Type 1 complexity.

- Factor H should be considered after numbers 1–3 are completed. If more than two of the items in factor H are answered yes, and three or more of the other primary factors are positive responses, a Type 1 team should be considered. If the composites of H are negative, and there are fewer than three positive responses in the primary factors (A-G), a Type 2 team should be considered. If the answers to all questions in H are negative, it may be advisable to allow the existing overhead to continue action on the fire.

| Incident Complexity Analysis Type 1 & 2 | YES | NO |
| --- | --- | --- |
| A. Fire Behavior (Observed or Predicted) |  |  |
| - Burning index (from on-site measurement of weather conditions) predicted to be above the 90% level using the major fuel model in which the fire is burning. |  |  |
| - Potential exists for extreme fire behavior (fuel moisture, winds, etc.). |  |  |
| - Crowning, profuse or long-range spotting. |  |  |
| - Weather forecast indicating no significant relief or worsening conditions. |  |  |
| Total |  |  |
| B. Resources Committed |  |  |
| - 200 or more personnel assigned. |  |  |
| - Three or more divisions. |  |  |
| - Wide variety of special support personnel. |  |  |
| - Substantial air operation which is not properly staffed. |  |  |
| - Majority of initial attack resources committed. |  |  |
| Total |  |  |

| C. Resources Threatened |  |  |
| --- | --- | --- |
| - Urban interface. |  |  |
| - Developments and facilities. |  |  |
| - Restricted, threatened, or endangered species habitat. |  |  |
| - Cultural sites. |  |  |
| - Unique natural resources, special-designation areas, wilderness. |  |  |
| - Other special resources. |  |  |
| Total |  |  |
| D. Safety |  |  |
| - Unusually hazardous fireline construction. |  |  |
| - Serious accidents or fatalities. |  |  |
| - Threat to safety of visitors from fire and related operations. |  |  |
| - Restrictions and/or closures in effect or being considered. |  |  |
| - No night operations in place for safety reasons. |  |  |
| Total |  |  |
| E. Ownership |  |  |
| - Fire burning or threatening more than one jurisdiction. |  |  |
| - Potential for claims (damages). |  |  |
| - Different or conflicting management objectives. |  |  |
| - Disputes over suppression responsibility. |  |  |
| - Potential for unified command. |  |  |
| Total |  |  |
| F. External Influences |  |  |
| - Controversial fire policy. |  |  |
| - Pre-existing controversies/relationships. |  |  |
| - Sensitive media relationships. |  |  |
| - Smoke management problems. |  |  |
| - Sensitive political interests. |  |  |
| - Other external influences. |  |  |
| Total |  |  |

| G. Change in Strategy |  |  |
| --- | --- | --- |
| - Change in strategy to control from confine or contain |  |  |
| - Large amounts of unburned fuel within planned perimeter. |  |  |
| - WFSA invalid or requires updating. |  |  |
| Total |  |  |
| H. Existing Overhead |  |  |
| - Worked two operational periods without achieving initial objectives. |  |  |
| - Existing management organization ineffective. |  |  |
| - Overhead overextended mentally and/or physically. |  |  |
| - Incident action plans, briefings, etc. missing or poorly prepared. |  |  |
| Total |  |  |