Roadside Incident Response

Considerations

- Firefighter and public safety will always be the number one priority.

- Utilize L.C.E.S. in all incident activities.

- Personal Protective Equipment will be utilized on all incidents.

Upon Arrival at the Scene

- Size up of the incident- see Incident Response Pocket Guide (IRPG)

- What has happened?

- What is happening?

- What will or could happen?

- Is this a HazMat situation?

Risk Management Process

- Decision Point, Go/No Go. See the IRPG.

Tactical Considerations

- Anytime traffic flow is affected by the incident, contact the jurisdictional law enforcement agency for assistance.

- Conduct all operations as far from traffic lanes as possible to provide for crew and public safety.

- Park units on the same side of the roadway when ever possible to avoid traffic congestion.

- Personnel do not exit the fire apparatus until instructed to do so by the module leader.

- Exit the fire apparatus away from the roadway or where hazard exposure is minimized.

- Exit the fire apparatus with full personal protective equipment.

- Post a lookout to watch for and control oncoming traffic.

- Utilize forward and rear spotters when visibility is impaired or road conditions warrant.

- Utilize and place road flares or other traffic warning signs when ever possible.

- If equipment needs to be removed from the traffic side of the apparatus, one person will retrieve the equipment and a lookout will watch for oncoming traffic.

- Engine operators will operate pumps from the non-traffic side or from the cab of the apparatus when possible.

- Keep all hose, fire tools, and equipment out of traffic lanes when possible.

- Wear high visibility vests and utilize other safety equipment as necessary while operating along roadways.

- During night operations utilize reflective clothing, vests and other safety equipment as necessary.

- All emergency responses on roadways will be concluded as quickly as possible to reduce personnel exposure.

- Cancel or demob unnecessary apparatus as soon as possible.

Each agency emergency vehicle operator will follow their particular state laws and agency policies governing the operations of emergency vehicles.