APPENDIX 10-9

HazMat IC Checklist

### Think Safety

- Assess situation.

- Safe approach, upwind/upgrade/upstream.

- Identify, isolate and deny entry.

- Notify agency dispatcher.

- Exact location, use GPS.

- Request needed assistance and identify a safe route.

Scene Management

- Goal is to protect life, environment and property.

- Attempt to identify substance using DOT North American Emergency Response Guide. Use binoculars, placards/labels, container shapes/colors, Material Safety Data Sheets (MSDS), shipping papers.

- Quantity of material involved.

- Exposures and hazards surrounding the site.

Organizational Responsibilities

- Establish chain of command.

- Develop action plan for area security and evacuation.

- Advise all on scene and responding resources of changes in situation.

- Keep dispatcher advised of changes.

- Document all actions taken:

- Contacts

- Employee exposures

Rules of thumb for isolation distances

- Minor event (1 drum, 1 bag, etc.) = 150 feet

- Major event (1 drum or more, etc.) = 500 feet

- Residential and light commercial = 300 feet

- Open areas = 1000 feet

- BLEVE (Boiling Liquid Expanding Vapor Explosion) potential = 2500 feet (one-half mile)

- Stage arriving units 2500 feet upwind.

- Position vehicles headed out.

1-800-424-9300 - CHEMTREC (Chemical Transportation Emergency Center) For immediate information about a chemical or to seek assistance from a manufacturer.

1-800-424-8802 - National Response Center- To report spills of oil and Hazardous Material.