(Revised)

Atmospheric Products from the Ozone Monitoring Instrument (OMI)

Suraiya P. Ahmada, Pieternel F. Leveltb, Pawan K. Bhartiac, Ernest  Hilsenrathc,

Gilbert W. Leppelmeierd, and James E. Johnsona

- aGoddard Earth Sciences Data and Information Services Center (GES DISC), Code 610.2/RSIS NASA Goddard Space Flight Center, Greenbelt, MD 20771, U.S.A.

bRoyal Netherlands Meteorological Institute (KNMI), De Bilt, The Netherlands

- cAtmospheric Chemistry and Dynamics Branch, Code 613.3, NASA Goddard Space Flight Center, Greenbelt, MD 20771, U.S.A.

dFinnish Meteorological Institute (FMI), Helsinki, Finland

- Abstract

Ozone Monitoring Instrument (OMI) is a Dutch-Finnish ozone monitoring imaging spectrometer that is designed to provide accurate measurements of total column ozone, ozone profile, surface UV irradiance, aerosols and cloud characteristics, and the column amounts of trace gases SO2, NO2, HCHO, BrO, and OClO at high spatial resolution. The OMI along with the three other instruments, the Microwave Limb Sounder (MLS), the High Resolution Dynamics Limb Sounder (HIRDLS), and the Tropospheric Emission Spectrometer (TES), will be flown on the NASA’s Aura mission in early 2004. The standard atmospheric chemistry and dynamics products derived from OMI, MLS, and HIRDLS will be archived at the NASA’s GES DISC (TES data products will be archived at NASA Langley Research Center DISC) and will be freely available to the public. Highlights of OMI data products, as well as their availability, distribution and data support are discussed in this paper.

Keywords: atmospheric chemistry, ozone, trace gases, NO2, SO2, BrO, HCHO, OClO, clouds, aerosols, UV and visible spectral radiance, surface UV-B flux, solar irradiance

- 1. Introduction

To continue the long-term study of the trends in ozone and other trace gases, and to improve the understanding of atmospheric chemical processes, the Ozone Monitoring Instrument (OMI), a contribution of the Netherlands agency for aerospace programs (NIVR) in collaboration with the Finnish Meteorological Institute (FMI) to NASA’s Aura mission, will be flown on the Aura spacecraft. Aura is part of the NASA's long-term Earth Observing System (EOS) mission and is scheduled for launch in early 2004. The Aura spacecraft will also carry three other instruments: the Microwave Limb Sounder (MLS), the High Resolution Dynamics Limb Sounder (HIRDLS), and the Tropospheric Emission Spectrometer (TES). These four instruments will provide atmospheric radiance measurements at high spectral and spatial resolution for studying changes in atmospheric ozone, tropospheric pollutants and other trace gases. The high quality atmospheric chemistry and dynamics products derived from these sensors will provide answers1 to the three main objectives of the Aura mission; is the Earth’s ozone layer recovering, is air quality changing, and how is the Earth’s climate changing? (http://aura.gsfc.nasa.gov/).

The OMI is a nadir sounder and the MLS and HIRDLS are limb sounding instruments. The TES has both limb sounding and nadir sounding modes. The OMI will provide measurements in UV and Visible spectral regions and will

focus on monitoring the ozone layer, ozone depleting trace gases BrO and OClO, tropospheric pollutants (tropospheric ozone, NO2, tropospheric aerosols, SO2, and HCHO), clouds, aerosols, and erythemal surface UV-B flux. Tropospheric ozone and aerosols are also important for climate change. The HIRDLS is an infrared limb-scanning radiometer designed to look through the "edge" of Earth's atmosphere to study aerosols and clouds. The MLS is a passive microwave radiometer/spectrometer that will study ozone depletion and radiation in the Earth's troposphere and stratosphere. The TES is an infrared imaging spectrometer and will measure global distributions of key atmospheric pollutants in the troposphere.

The OMI is built by Dutch Space/TNO-TPD in the Netherlands, in co-operation with Finnish subcontractors VTT and Patria Finavitec. The Principle Investigator (PI) Institute is KNMI (Royal Netherlands Meteorological Institute).         Dr. Pieternel F. Levelt is the principle investigator. The co- investigators are Dr. Gilbert W. Leppelmeier (FMI) and Ernest Hilsenrath (NASA), and US science team leader is Dr. Pawan K. Bhartia (NASA). An international team consisting of Dutch, US and Finnish scientists has been established and are assigned with the responsibilities of developing retrieval algorithms, monitoring in-flight instrument calibration, operational handling of the instrument, and conducting the data validation.

The OMI Level-0 to 1B and Level-2 & 3 data processing will occur at NASA Goddard Space Flight Center and at KNMI in the Netherlands. Overall responsibility2 for the OMI mission lies with the Netherlands Agency for Aerospace Programs (NIVR) with the participation of FMI (Finnish Meteorological Institute).

The standard atmospheric chemistry and dynamics products derived from OMI, MLS, and HIRDLS aboard Aura, and data from future upper atmospheric research missions, will be archived at the NASA GES DISC. Data from TES will be archived at NASA Langley Research Center DAAC. These atmospheric products will provide continuity to the over 30 year long-term ozone data records obtained from the heritage atmospheric data missions including Nimbus-4 BUV and Nimbus-7 Solar Backscatter Ultraviolet Spectrometers (SBUV), a series of Total Ozone Mapping Spectrometer (TOMS) instruments, and will provide continuity to a suite of important ozone depleting trace gases available from the Upper Atmospheric Research Satellite (UARS) instruments, also archived at the NASA GES DISC and freely available to the public.

This paper gives an overview of the OMI instrument, data processing, data products, and the data services provided by the NASA GES DISC Atmospheric Composition Data Support team to the user in the areas of accessing data products, documentation, browse, and data analysis software.

- 2. Characteristics of THE OMI Instrument

The OMI instrument will be flown on the Aura spacecraft into a 705 km, 98.2 degree inclination, sun-synchronous polar orbit with an equatorial crossing at 1:38 pm (in the ascending mode). OMI will observe almost the same air mass within 15 minutes of the Aqua measurements.  OMI has a very large field of view of 114 degrees for the swath registration (± 57 degrees relative to nadir) perpendicular to the flight direction of the satellite. In nominal operation2, the OMI swath width on the ground is 2600 km. This wide swath width enables OMI to achieve global coverage in one day (14 orbits).

2.1 Spectral Sampling

OMI is a non-scanning wide angle nadir pointing hyper-spectral push broom imaging spectrograph, to observe solar backscatter radiation in the visible and ultraviolet spectral region. The Earth will be viewed in 1560 wavelength bands in the spectral region 270 to 500 nm with a spectral resolution of approximately 0.5 nm (Table 1). The spectral samplings are grouped into two optical channels, UV & visible (VIS). The UV channel is split into two sub-channels: UV-1 ranging from 270 to 310 nm, and the UV-2 ranging from 310 to 365 nm for stray-light considerations. The VIS-channel ranges from 365 to 500 nm (Table 1). Two 2-dimensional CCD detectors are used, one for UV-1 & 2 and other for the VIS channel. The nominal integration time of the CCD image is 0.4 sec. To decrease the data rate and increase the signal-noise ratio, onboard binning and co-adding of subsequent CCD images are performed. The number of exposures or images that are averaged is specified by the "binning factor", which is set by the instrument operations team3.

2.2 Spatial Sampling

There are three sampling modes, global, spatial zoom-in, and spectral zoom-in. In the global (default) mode, the earth measurements are taken with the nominal co-addition time of 2 sec (i.e. binning factor 8). Using this co-addition time, for a swath of 2600 km, a nadir pixel size of 13 x 24 km2 is achieved for the VIS and UV-2 channels, and 13 x 48 km2 for the UV-1 channel (Table 1). In the zoom-in measurement or high resolution mode, binning factor 4 is used. The spatial and spectral zoom-in modes offer the possibility of smaller ground pixels at the expense of reduced swath width or spectral range. For example, in the spatial zoom-in mode (13 x 12 km2), the swath width for UV-2 & VIS is reduced to 725 km and in the spectral zoom-in mode the spectral coverage for UV and VIS region is reduced. Spatial zoom-in mode will be used for regional studies. A high resolution data mode with no binning are referred as “small pixel” data and is used for calibration measurements3.

The nadir ground pixel size also increases because of the earth curvature effects at large viewing angles. The pixel-size in the swath-direction increases from 13 x 24 km2 (exact nadir position) to approximately 13 x 128 km2 at the outermost swath-angle (57 degrees). The geolocation2, 4 of the OMI ground pixels is accurate to about one tenth of the CCD pixel or 1 to 2 km.

Table 1. OMI swath width and nadir pixel size (along x across track) for different observation Modes

| Observation mode | Swath width |  | Nadir ground pixel size | Spectral range | Application |
| --- | --- | --- | --- | --- | --- |
| Global mode |  |  |  |  |  |
| UV-1 UV-2 VIS | 2600 km 2600 km 2600 km | 13 x 48 km2 13 x 24 km2 13 x 24 km2 |  | 270 - 310 nm 310 - 365 nm 365 - 500 nm | Global observations of almost all OMI products |
| Spatial zoom-in mode |  |  |  |  |  |
| UV-1 UV-2 VIS | 2600 km 725 km 725 km | 13 x 24 km2 13 x 12 km2 13 x 12 km2 |  | 270 - 310 nm 310 - 365 nm 365 - 500 nm | Regional observations of all OMI products |
| Spectral zoom-in mode |  |  |  |  |  |
| UV VIS | 2600 km 2600 km | 13 x 12 km2 13 x 12 km2 |  | 306 - 364 nm 350 - 432 nm | Global observation of some products |

2.3 On-board Calibrators and Calibration Measurements

The pre-flight spatial, spectral, and radiometric calibration5 has been done in detail by the Dutch Space, the prime instrument contractor, and the TNO-TPD team, who have designed and built the optical systems at TNO-TPD facilities.  In addition to the pre-flight calibration, OMI will also perform the in-flight calibration5 measurements. The on-board calibrators include: three solar diffusers for solar measurements (one for daily use and the others for monitoring the degradation of the daily used diffuser), Internal White Light Source (WLS) observed (on average once per week) via a transmission diffuser for relative radiometric calibration of the instrument optics plus detectors, two Light Emitting Diodes (LEDs) observed once a week on average for monitoring the efficiency of the detectors. Dark current measurements are also performed. OMI also uses a polarization scrambler to depolarize the incoming polarized radiation. Wavelength calibration is performed by using Fraunhofer lines in the solar irradiance and earth radiance spectra. For geolocation verification purpose the earth radiance measurements are made in super spatial zoom-in mode ("small pixel" mode, no binning).

- 3. Science Data Processing

The EOS Data and Operations System (EDOS) is responsible for data capture and initial Level-0 processing of spacecraft data.  Level-0 product consists of raw instrument data at original resolution, time ordered, with duplicate packets removed. Aura data from OMI, MLS, HIRDLS, and TES are further processed using the algorithms provided by the individual instrument science teams to convert counts to geolocated radiances (Level-1B products) and produce geophysical data products (Levels-2 & 3) from Level-1B. This activity takes place on the Science Investigator-led Processing Systems (SIPS) centers established for each instrument. For OMI, two data centers are under development, the OMI SIPS or OSIPS located at NASA GSFC and the ODPS (OMI Dutch Processing System) located at the KNMI, The Netherlands. The surface spectral UV-B flux product will be processed at the FMI, Finland. OSIPS and ODPS will integrate operational science software, process and reprocess standard products, and deliver them to the GES DISC for archive and distribution.

- 4. OMI Data Products

The OMI data products are available at four processing Levels: Level-0, Level-1B, Level-2, and Level-3. Level-0 products are raw sensor counts. Level-0, Level-1B, and Level-2 products contain orbital swath data, whereas Level-3 products contain global data that are averaged over time (daily or monthly) for small equal angle (latitude x longitude) grids covering the whole globe. OMI Level-2 data files will also contain temporal, spatial, solar, and viewing geometry parameters, quality flags, and extensive quality assurance information, in addition to the standard retrieved geophysical parameters.  Brief details of these products are given below.

4.1 OMI Radiometrically Calibrated and Geolocated Radiance Products (Level-1B)

An extensive calibration and data processing algorithm called the “Level-0 to 1B Algorithm” has been developed6 by Dutch Space in co-operation with the Dutch OMI team. The Level-0 to 1B algorithm is used at OSIPS (NASA GSFC) to produce Level-1B OMI products7. It takes the raw sensor measurements (Level-0 data), calibration and spacecraft ephemeris information and produces radiometrically calibrated and geolocated radiances. Six types of Level-1B standard products are produced (Table 2).

Table 2. OMI Calibrated and Geolocated Earth View Radiance, Solar Irradiance, and Calibration Products (Level-1B)

|  | OMI Level-1B Calibrated Radiance & Irradiance |
| --- | --- |
| Product | Product Description |
| OML1BRUG | Geolocated Earth View UV Radiance, Global-mode Product - Geolocated earth view radiances from the UV channel detector in the wavelength range of 270 to 380 nm, using the global measurement mode. It will also contain the data rebinned from the observations taken using zoom-in measurement modes. |
| OML1BRVG | Geolocated Earth View VIS Radiance, Global-mode Product - Geolocated earth view radiances from the VIS channel detector in the wavelength range of 350 to 500 nm, using the global measurement mode. It will also contain the data rebinned from the observations taken using zoom-in measurement modes. |
| OML1BRUZ | Geolocated Earth View UV Radiance, Zoom-in mode Product - Geolocated earth view radiances from the UV channel detector in the wavelength range of 270 to 380 nm, using spectral and spatial zoom-in measurement modes. |
| OML1BRVZ | Geolocated Earth View VIS Radiance, Zoom-in mode Product - Geolocated earth view radiances from the VIS channel detector in the wavelength range of 350 to 500 nm, using spectral and spatial zoom-in measurement modes. |
| OML1BIRR | Solar Irradiance Product - Averaged sun measurements of the solar irradiances from both the UV and VIS channel detectors over a single solar observation in the wavelength range of 270 to 500 nm. |
| OML1BCAL | Calibration Data - In-flight calibration measurement results, including the complete CCD readouts for both the UV and VIS channels from the areas on the CCD that are intended for the calibration purposes (which is outside of the area normally used by the spectrometer). |
| OML1BOPF | Operational Parameter File - Essentially a LUT (look up table) with the information (including pre-flight and in-flight calibration parameters) needed in the Level 0 to 1B processing. |

The lead scientists for Level-1B products are: Marcel Dobber (dobber@knmi.nl), R. H. Voors (voors@knmi.nl), and J. de Vries (JdeVries @tpd.tno.nl).

4.2 OMI Orbital Atmospheric Products (Level-2)

The OMI Level-2 (orbital swaths) products contain the geophysical parameters (at ground-pixel resolution) derived from radiometrically calibrated and geolocated radiances (Level-1B product). The spectral radiances used for the retrieval of OMI products are given in Table 3. Each Level-2 product file consists of parameters retrieved from observations made only in the daytime portion of an orbit (~ 53 minute duration). In addition to standard derived parameters, these files also contain some auxiliary derived parameters, ancillary input parameters, temporal, spatial, solar and viewing geometry, terrain height, ground pixel quality flags, and extensive quality assurance information. In the OMI global operational mode the spatial resolution is 13 x 24 km2 at nadir. Almost all Level-2 products will contain data at 13 x 24 km2 (nadir pixel size) resolution with the exception of few. Level-2 products will be available at higher spatial resolution using the OMI spatial zoom-in mode data 13 x 12 km2. These products will have their short name with “Z” (for the Zoom) at the end.

Table 3. The spectral regions used for the retrieval of OMI products

| Products | Spectral Range (nm) | Products | Spectral Range (nm) |
| --- | --- | --- | --- |
| Surface spectral irradiance & Erythemally weighted UV Flux | 290 - 400 | O3 profile | 270 - 340 |
| Ozone column: DOAS method | 320 - 340 | SO2 | 300 - 330 |
| Ozone column: TOMS Version 8 method | 310 - 380 | HCHO | 335 - 360 |
| Aerosol: near-UV and multi-wavelength method | 330 - 500 | BrO | 344 - 360 |
| Cloud Fraction and Cloud Pressure: O2-O2 absorption method | 320 - 500; 470 - 485 | OClO | 355 - 385 |
| Cloud Fraction and Cloud Pressure: Rotational Raman method | 392 - 398 | NO2 | 405 - 465 |

Ozone Products

There are four Level-2 ozone products (Table 4) that are based on the algorithms that have heritage from Total Ozone Mapping Spectrometer (TOMS), Global Ozone Monitoring Experiment (GOME), and Scanning Imaging Absorption Spectrometer for Atmospheric CHartographY (SCIAMACHY). These ozone products are column ozone, ozone profile, and tropospheric total ozone. Two algorithms are available for the total ozone retrieval, enhanced TOMS Version 8 (V8) algorithm used for more than 25 years for the TOMS, and Differential Optical Absorption Spectroscopy (DOAS) technique used for the GOME and SCIAMACHY ozone retrievals. TOMS V8 algorithm8 retrieves vertical column ozone data essentially using 317.5 and 331.2 nm wavelengths. OMI’s additional hyper-spectral measurements will help in better estimates and corrections of the factors that induce uncertainty in ozone retrieval (e.g., cloud and aerosol, sea-glint effects, profile shape sensitivity, SO2 and other trace gas contamination).  DOAS technique9 is based on the fitting of the satellite measured gases absorption structure at number of absorption lines in the spectral region 331.1 to 336 nm to the one measured in the laboratory. First slant column density is determined then it is translated into a vertical column density, using air mass factor. This air mass factor is computed using radiative transfer models.

Ozone profile retrieval10, is based on the Optimal Estimation method (also referred to as Rodger's maximum-likelihood estimation technique), uses all ozone sensitive wavelengths radiances (in spectral region 270 to 340 nm), a priori ozone profile, radiances, and a Jacobian matrix computed from the radiative transfer model. Hyperspectral capabilities provide better vertical resolution below 20 km compared to SBUV which has a limited number of wavelengths.

The OMI Tropospheric Total Ozone (TTO) product will also be made available from GES DISC. The TTO retrieval11 algorithm  is planned to be an improved version of Tropospheric Ozone Residual (TOR) method. Tropospheric Total Ozone retrieval uses stratospheric ozone column obtained from high vertical resolution ozone profile (HIRDLS or MLS retrievals) and subtracts it from the OMI retrieved total column ozone.

Clouds, Aerosols and Surface UV Irradiance Products

OMI produces cloud fraction and cloud pressure that are used in OMI trace-gas retrievals and by other Aura sensors. Two methods are used for the cloud pressure retrieval, O2-O2 absorption & Rotational Raman Scattering methods.     O2-O2 absorption method12 is based on spectral fitting of O2-O2 absorption band at 477 nm using DOAS technique. Rotational Raman Scattering (RRS) method13 is based on the least square fitting of the Ring spectrum (filling-in of Fraunhofer lines in the range 392 to 398 nm due to rotational Raman scattering.

OMI also provides aerosol information that is needed by the other Aura algorithms. Two algorithms14 (near-UV and multi-wavelength methods) are used for the retrieval of aerosol characteristics over the ocean and land. The availability of the near-UV wavelengths 330 to 490 nm makes the aerosol retrieval over the desert area also possible since the land reflectance is small at these wavelengths. Besides being able to provide information on the aerosol absorption and single scattering albedo, OMI aerosol algorithms can differentiate between sulfate, smoke and dust aerosols, and characterize the type of urban aerosols.

Spectral surface irradiance (at 305, 310, 324, and 380 nm) reaching the ground and the erythemally weighted irradiance (covering 290-400 nm) are produced using the enhanced version of TOMS Surface UV-B flux algorithm15. Measured earth-atmosphere radiances are used in conjunction with radiative transfer model to retrieve UV spectral irradiances reaching the ground. The OMI algorithm uses actual snow thickness information from the ECMWF (European Centre for Medium-Range Weather Forecasts) analysis which improves the values for the high latitudes. The erythemal UV exposure is calculated using a model for the susceptibility of Caucasian skin to the sunburn (erythemal). Some highlights of clouds, aerosols and surface irradiance exposure products are also provided in Table 5.

Table 4. OMI Ozone Products (Level-2)

| OMI Ozone (Total column & Profile) |  |  |
| --- | --- | --- |
| Product | Parameters | Lead Scientist |
| OMTO3 | O3 total column (TOMS V8 method), also other derived and ancillary parameters including N-values, effective Lambertian scene-reflectivity, UV aerosol index, SO2 index, cloud fraction, terrain and cloud pressure, ozone below clouds, geolocation, solar and satellite viewing angles, and quality flags. | P. K. Bhartia Pawan.K.Bhartia@nasa.gov |
| OMDOAO3 | O3 total column (DOAS method), ozone slant column density and its precision (for data assimilation), also other derived and ancillary parameters e.g. ozone ghost column density, air mass factor (all, clear, and cloudy scenes), scene reflectivity, radiance over the DOAS fit window (331.1 to 336.1 nm), root mean square of DOAS fit, cloud fraction, cloud radiance, terrain & cloud pressure, geolocation, viewing angles, and quality flags. | P. Veefkind veefkind@knmi.nl |
| OMO3PR | O3 profile (Optimum estimation method), a priori profile (including uncertainties) for 17 layers, correlation coefficients, averaging kernel, root mean square of fit, total ozone (obtained by integrating the profile), geolocation, solar and satellite viewing angles, and quality flags. -This product will be produced at KNMI and will be archived at the NASA GES DISC as a standard product. | J.F.de Haan haandej@knmi.nl |

Table 5.  OMI Clouds, Aerosols, and Surface UV Exposure Products (Level-2)

| Clouds |  |  |
| --- | --- | --- |
| Product | Parameters | Lead Scientist |
| OMCLDO2 | Cloud fraction and cloud pressure (O2-O2 absorption method), slant column O2-O2 & O3, Ring coefficients, and uncertainties in derived parameters; terrain and geolocation information; solar & satellite viewing angles; and quality flags. | J.F. de Haan haandej@knmi.nl |
| OMCLDRR | Cloud fraction and cloud pressure (Rotational Raman scattering method), other derived and ancillary parameters; terrain and geolocation information; solar & satellite viewing angles; and quality flags. | J. Joiner Joanna.Joiner-1@nasa.gov |
| Aerosols over Ocean and Land |  |  |
| OMAERO | Aerosol optical thickness & single scattering albedo for the clear sky (multi-wavelength method), UV and VIS Aerosol indices; Aerosol types, aerosol uncertainties in the derived parameters; quality flags; solar and satellite viewing angles; and geolocation information. | B.Veihelmann veihelmann @knmi.nl |
| OMAERUV | Aerosol Absorption and extinction optical thickness & single scattering albedo for the clear sky (Near-UV method), near-UV index similar to TOMS aerosol index; dominating aerosol types (carbonaceous, mineral dust, or sulfate); aerosol layer height; quality flags; solar and satellite viewing angles; and geolocation information. The near-UV Aerosol index is going to be reported even in the case of cloud contaminated pixels when aerosols are over the clouds. | O. Torres Omar.Torres@hampton.edu |
| Surface UV-Irradiance |  |  |
| OMUVB | Surface Erythemal UV Exposure (TOMS algorithm): Downward spectral irradiances (W/m2/nm) at the ground for 305, 310, 324, & 380 nm; Erythemally weighted UV irradiance (W/m2) covering 290 to 400 nm spectral region ( noontime values & daily averages). -This product will be produced at FMI (Finland), and will be archived at NASA GES DISC as a standard product. | A. Arola antti.arola@fmi.fi |

Trace Gases Products

Total column and slant column values of trace gases, NO2, BrO, HCHO, OClO, and SO2 will be produced using absorption spectrum fitting techniques. Highlights of these products are provided in Table 6. The OMI algorithm will detect volcanic ash and sulfur dioxide produced in volcanic eruptions with up to 40 times more sensitivity than TOMS and GOME.18

Table 6.  OMI Trace Gases Products (Level-2)

| Trace Gases (NO2, BrO, HCHO, OClO, and SO2) |  |  |
| --- | --- | --- |
| Product | Parameters | Lead Scientist |
| OMNO2 | NO2 total & tropospheric column, slant column density, one sigma fitting uncertainties for the NO2 and the other species varied in the fitting window, correlation of other fitted species to the NO2, fitting root mean square (rms), surface reflectivity, cloud top height, geolocation , and solar and satellite viewing angles. | J. Gleason James.F.Gleason@nasa.gov P. Veefkind veefkind@knmi.nl |
| OMBRO | BrO total column and slant column abundance, one sigma fitting uncertainties for the BrO and the other species varied in the fitting window, correlation of other fitted species to the BrO, fitting rms, surface reflectivity, cloud top height, quality flags, geolocation , and solar and satellite viewing angles. | K. Chance kchance@cfa.harvard.edu |
| OMHCHO | HCHO total column and slant column abundance, one sigma fitting uncertainties for the HCHO and the other species varied in the fitting window, correlation of other fitted species to the HCHO, fitting rms, surface reflectivity, cloud top height, quality flags, geolocation , and solar and satellite viewing angles. | K. Chance kchance@cfa.harvard.edu |
| OMOCLO | OClO slant column and one sigma fitting uncertainties for the OClO and the other species varied in the fitting window, correlation of other fitted species to the OClO, fitting rms, surface reflectivity, cloud top height, quality flags, geolocation, and solar and satellite viewing angles. -This product is not global (retrieved for polar regions only). | K. Chance kchance@cfa.harvard.edu |
| OMSO2 | Total column SO2, prescribed SO2 layer height, fitting rms, convergence flags, total ozone, surface reflectivity, cloud top height, quality flags, geolocation, and solar and satellite viewing angles. | N.Krotkov Nickolay.A.Krotkov@nasa.gov |

4.3 OMI Global Gridded Atmospheric Products (Level-3)

- OMI Level-3 product will be produced for almost all Level-2 standard parameters. Each Level-2 product file contains data from a single orbit. For each Level-2 product there will be 14 files per day.  OMI Level-3 daily global product will be produced by averaging data over small equal angle grids (0.25 deg x 0.25 deg), (0.5 deg x 0.5 deg) or (1 deg x 1 deg) covering the whole globe. Each grid will also contain the corresponding statistical parameters (number of pixels, minimum, maximum, and sigma).

- 4.4 Other OMI Data Processing & Data Products

In addition to the standard data processing at the OSIPS (NASA GSFC) and at the ODPS (KNMI, Netherlands) together with “standard data products” archival at NASA GES DISC, there will be two other types of product generation.

- Near Real Time Products (NRT)

KNMI will produce a daily global Near Real Time (NRT) ozone product (Ozone total column). This product will be retrieved from “rate buffered data” (Level-0 data, not time ordered and without duplicity removed) and will be available within three hours of data capture. These NRT products will be used for several operational applications. For example assimilating these measurements in numerical weather prediction models will provide improved wind-field information for regions where wind-field measurements are scarce. OMI NRT products2 can also contribute to the operational forecasting of the UV-index.

- Very Fast Delivery Products (VFD)

FMI will produce “regional” Very Fast Delivery (VFD) products (Ozone total column, Surface UV-B flux, and UV spectra) using Direct Broadcast Data from the satellite. Finland will access Direct Broadcast (DB) data once a day as the spacecraft flies overhead. A ground station is built in Sodankylä to receive OMI data. These products will be retrieved from Direct Broadcast raw telemetry data and will have limited geographical coverage (2600 km swath from the Arctic Ocean to the Alps, covering most of the northeastern Europe).

4.5 Additional Total Column Ozone Products

The “Enhanced TOMS V8 algorithm” will be used to process the OMI's data. It has gone through three decades of refinements. This algorithm will also be used to reprocess all the BUV, SBUV and TOMS data taken since April 1970 for the continuity of more than 25-year long-time data record. TOMS has produced total ozone with r.m.s. of 2% for most conditions. OMI data will add more than 5 years (or longer) to this data record. These data will be reprocessed at OSIPS and will be archived at GES DISC. In the first phase, data will be processed for the TOMS measurements (Table 7). These TOMS products contain total ozone, SO2, aerosol indices, and surface reflectivities, in addition to the quality flags and geolocation information.

Table 7. TOMS Total Ozone Products (Level-2)

| Product | Instrument | Data Period | Spectral Bands | Lead Scientist |
| --- | --- | --- | --- | --- |
| TOMSN7L2 | TOMS Nimbus7 | 1978 - 1993 | 312, 317, 332, 340, 360, 380 nm | P. K. Bhartia Pawan.K.Bhartia@nasa.gov |
| TOMSM3L2 | TOMS Meteor-3 | 1991 - 1994 | 312, 317, 332, 340, 360, 380 nm |  |
| TOMSA1L2 | TOMS ADEOS-1 | 1996 - 1997 | 309, 313, 318, 322, 331, 360 nm |  |
| TOMSEPL2 | TOMS Earth Probe | 1996 - Present | 309, 313, 318, 322, 331, 360 nm |  |

5.  Data Validation & Field Campaigns

For algorithm development, testing, and evaluation, synthetic test data bases have been generated using radiative and chemical transport models and GOME data.  OMI validation activities have been at high priority. The Aura Validation Working Group19 has been developing a more “specific strategy” that will first focus on validation under quiet atmospheric conditions then it will be followed by comparison campaigns under more “challenging” conditions.

An OMI validation plan is well underway as part of the overall Aura validation activity. The creation of coincident satellite overpasses (i.e. TOMS, GOME, SCIAMACHY) for specific data validation sites are already planned19. Many field campaigns (involving high and low Level aircrafts, satellite overpasses, and in-situ balloons and sondes, and other surface measurements) are designed for testing OMI algorithms, for algorithm enhancement and data validation.

6. OMI DATA PRODUCT SUPPORT AVAILABLE FROM THE NASA GES DISC

Long-term Atmospheric Chemistry and Dynamics Data Records at the GES DISC

The data from the Aura instruments (OMI, MLS, HIRDLS), as well as the data from the other future missions such as, NPOESS Preparatory Project (NPP), will be archived at the NASA GES DISC.  The atmospheric data products from several past and present atmospheric data missions (Table 8) , including Nimbus-4 IRIS and BUV, Nimbus-7 SBUV and LIMS,  a series of TOMS instruments, SME, UARS instruments (CLAES, HALOES, ISAMS, MLS, HRDI, WINDII), ERS-2 GOME (courtesy of DFD/DLR ), Terra and Aqua instruments (MODIS, AIRS, AMSU, HSB), TOVS pathfinder, SSBUV, ATMOS, and DAO, are also archived at the NASA GES DISC.

Oct 1991- Present

Data Levels: 3AT& 3TP

Version: 4 & 3

Profiles: electron, proton, x-rays energy deposition rate

Spectrometers: High &Medium Energy Particle & X-Ray Spectrometers (HEPS & MEPS, AXIS),                       Vector Magnetometer (VMAG)

PEM

Particle Environment Monitor

J. David  Winningham, Southwest Research Institute

Data Availability    from GES DAAC

Parameters

Characteristics

Instruments

Chemical Measurements

Particle Deposition Measurements

Wind & Temperature Measurements

Solar Irradiance Measurements

Sept 1991- Aug 2001

Data Levels: 2 & 3AT

Total Solar Irradiance

Active Cavity Radiometer

ACRIM II

Active Cavity Radiometer Irradiance Monitor

R .C. Wilson, NASA/JPL

Nov 1991- Present

Data Levels: 3AL & 3AT

Version: 11

Temperature, Stratospheric & Mesospheric Wind Fields, Volume Emission Rate

Fabry Perot Interferometer earth limb views; visible and near-infrared spectral region (0.557 - 0 .775 micron)

HRDI

High Resolution Doppler Imager

Paul B. Hays, University of Michigan

Nov 1991- Present

Data Levels: 3AL & 3AT

Version: 11

Temperature, Stratospheric & Mesospheric Wind Fields, Volume Emission Rate

Michelson Interferometer; earth limb views in visible and near-infrared spectral region (0.557 - 0 .9 micron)

WINDII

Wind  Imaging Interferometer

Gordon Shepherd, York University, Canada

Oct 1991- Present

Data Levels: 3BS

Version: 20

Solar Ultraviolet Spectral Irradiance

Spectrometer:   UV region 120 - 400 nm with spectral resolution of 0.1 nm.

SUSIM

Solar Ultraviolet Spectral Irradiance Monitor

Dianne K. Prinz, Naval Research Laboratory

Oct 1991- Present

Data Levels: 3BS

Version: 9

Solar Ultraviolet Spectral  Irradiance

Grating spectrometer  to provide spectral irradiance in 115 to 430 nm spectral region.

SOLSTICE

Solar/Stellar Irradiance Comparison Experiment

Gary J. Rottman , High Altitude Observatory/NCAR

Sept 1991- July 1999*

Data Levels:3AL, 3AT, 3B Version: 4 & 3

* in standby mode

Profiles: Ozone, CH3CN, ClO, H2O, HNO3, SO2, Geopotential Height, Humidity, Temperature

Thermal microwave emissions from selected molecular spectral lines (63,183, 205 GHZ)

MLS

Microwave Limb Sounder

Joe W. Waters,  NASA/Jet Propulsion Laboratory

Oct  1991- Present

Data Levels: 2 & 3AT Version: 19

Profiles: Ozone, CH4, H2O, HCl, HF, NO, NO2 , Aerosol Extinction for 2.45,  3.4, 3.46 ,  5.26 , 2.8  microns, Temperature

Solar Occultation method; earth limb emission spectra in the spectral region 2.43 to 10.25 micron

HALOE

Halogen Occultation Experiment

James M. Russell III, NASA/Langley Research Center

Sept 1991- July 1992

Data Levels: 3AL, 3AT, 3B

Version: 10

Profiles: Ozone, CH4, CO, H2O, HNO3, NO, NO2, N2O, N2O5), Aerosol Extinction, Temperature

Infrared pressure modulated radiometry; 4.6 to 16. 6 micron; earth limb views

ISAMS

Improved Stratospheric and Mesospheric Sounder

Fred Taylor , Oxford University, England

Oct 1991- May 1993

Data Levels:3AL, 3AT, 3B

Version: 9

Profiles: Ozone, CF2Cl2, CFCl3, CH4, ClONO2, H2O, HCl, HNO3, NO, NO2, N2O, N2O5), Aerosol Extinction, Temperature

Earth limb emission spectra in the spectral region from 3.5 to 12.7 microns; earth limb views

CLAES

Cryogenic Limb Array Etalon Spectrometer

Aidan E. Roche,  Palo Alto Research Laboratory

Oct 1991- Present

Data Levels: 3AT& 3TP

Version: 4 & 3

Profiles: electron, proton, x-rays energy deposition rate

Spectrometers: High &Medium Energy Particle & X-Ray Spectrometers (HEPS & MEPS, AXIS),                       Vector Magnetometer (VMAG)

PEM

Particle Environment Monitor

J. David Winningham, Southwest Research Institute

Data Availability    from GES DAAC

Parameters

Characteristics

Instruments

Chemical Measurements

Particle Deposition Measurements

Wind & Temperature Measurements

Solar Irradiance Measurements

Sept 1991- Aug 2001

Data Levels: 2 & 3AT

Total Solar Irradiance

Active Cavity Radiometer

ACRIM II

Active Cavity Radiometer Irradiance Monitor

R .C. Wilson, NASA/JPL

Nov 1991- Present

Data Levels: 3AL & 3AT

Version: 11

Temperature, Stratospheric & Mesospheric Wind Fields, Volume Emission Rate

Fabry Perot Interferometer earth limb views; visible and near-infrared spectral region (0.557 - 0 .775 micron)

HRDI

High Resolution Doppler Imager

Paul B. Hays, University of Michigan

Nov 1991- Present

Data Levels: 3AL & 3AT

Version: 11

Temperature, Stratospheric & Mesospheric Wind Fields, Volume Emission Rate

Michelson Interferometer; earth limb views in visible and near-infrared spectral region (0.557 - 0 .9 micron)

WINDII

Wind Imaging Interferometer

Gordon Shepherd, York University, Canada

Oct 1991- Present

Data Levels: 3BS

Version: 20

Solar Ultraviolet Spectral Irradiance

Spectrometer:   UV region 120 - 400 nm with spectral resolution of 0.1 nm.

SUSIM

Solar Ultraviolet Spectral Irradiance Monitor

Dianne K. Prinz, Naval Research Laboratory

Oct 1991- Present

Data Levels: 3BS

Version: 9

Solar Ultraviolet Spectral  Irradiance

Grating spectrometer to provide spectral irradiance in 115 to 430 nm spectral region.

SOLSTICE

Solar/Stellar Irradiance Comparison Experiment

Gary J. Rottman, High Altitude Observatory/NCAR

Sept 1991- July 1999*

Data Levels: 3AL, 3AT, 3B Version: 4 & 3

* In standby mode

Profiles: Ozone, CH3CN, ClO, H2O, HNO3, SO2, Geopotential Height, Humidity, Temperature

Thermal microwave emissions from selected molecular spectral lines (63,183, 205 GHZ)

MLS

Microwave Limb Sounder

Joe W. Waters, NASA/Jet Propulsion Laboratory

Oct 1991- Present

Data Levels: 2 & 3AT Version: 19

Profiles: Ozone, CH4, H2O, HCl, HF, NO, NO2, Aerosol Extinction for 2.45, 3.4, 3.46, 5.26, 2.8 microns, Temperature

Solar Occultation method; earth limb emission spectra in the spectral region 2.43 to 10.25 micron

HALOE

Halogen Occultation Experiment

James M. Russell III, NASA/Langley Research Center

Sept 1991- July 1992

Data Levels: 3AL, 3AT, 3B

Version: 10

Profiles: Ozone, CH4, CO, H2O, HNO3, NO, NO2, N2O, N2O5), Aerosol Extinction, Temperature

Infrared pressure modulated radiometry; 4.6 to 16. 6 micron; earth limb views

ISAMS

Improved Stratospheric and Mesospheric Sounder

Fred Taylor, Oxford University, England

Oct 1991- May 1993

Data Levels: 3AL, 3AT, 3B

Version: 9

Profiles: Ozone, CF2Cl2, CFCl3, CH4, ClONO2, H2O, HCl, HNO3, NO, NO2, N2O, N2O5), Aerosol Extinction, Temperature

Earth limb emission spectra in the spectral region from 3.5 to 12.7 microns; earth limb views

CLAES

Cryogenic Limb Array Etalon Spectrometer

Aidan E. Roche, Palo Alto Research Laboratory

The continuity of the long-term atmospheric ozone, trace gases, clouds, aerosols, solar and surface UV spectral irradiances, and erythemal surface UV-B flux data, will be maintained with the launch of Aura spacecraft in early 2004. Some of the atmospheric chemistry and dynamics related climate products available from the NASA GES DISC are listed in Table 8.

Data Support

The NASA GES DISC has been engaged in the collection, archive, distribution, and support of satellite-based remote sensing data since its inception in 1993.  The GES DISC's key objective has been to facilitate data access, provide data in the format compatible with the user requirements, explore and promote the potential applications, and reach the broader satellite data user community.

The standard climate data sets, as well as special products such as regional subsets, applications products in GIS compatible format, related ancillary data sets, and data analysis tools are freely available to the public for the Earth System Science studies, environmental applications, and educational use. Detailed information pertaining to the availability of climate data sets is available from the GES DISC (http://disc.gsfc.nasa.gov/ ).

Table 8. Atmospheric Chemistry & Dynamics, and Other Climate related Data Sets, at the NASA GES DISC

| Sensor | Sensor Name | Data Period | Key Products |
| --- | --- | --- | --- |
| OMI | Aura Ozone Monitoring Instrument | 2004 - present | ozone, trace gases, aerosol, cloud, UV flux |
| HIRDLS | Aura High Resolution Limb Sounder | 2004 - present | total ozone & profile, trace gases, aerosols |
| MLS | Aura Microwave Limb Sounder | 2004 - present | ozone, trace gases, humidity, temperature |
| AIRS | Atmospheric Infrared Sounder | 2002 - present | ozone, atmospheric profiles, cloud, SST |
| AMSU-A | Advanced Microwave Sounding Unit | 2002 -present | temperature and humidity profiles |
| TOMS | Total Ozone Mapping Spectrometer | 1978 - 2006 | ozone, SO2 and aerosol indices, UV-B flux |
| SBUV | Solar Backscatter Ultraviolet Spectrometer | 1978 - 1990 | ozone profile, SO2 index, UV-B flux |
| BUV | Nimbus-4 Backscatter Ultraviolet Spectrometer | 1970 - 1977 | ozone, surface Lambertian reflectivity |
| IRIS | Infrared Interferometer Spectrometer | 1970 - 1971 | infrared radiances for ozone retrieval |
| LIMS | Limb Infrared Monitor of the Stratosphere | 1978 - 1985 | O3, NO2, water vapor, temperature |
| SME | Solar Mesospheric Explorer | 1981 - 1986 | total ozone and ozone profiles |
| CLAES | Cryogenic Limb Array Etalon Spectrometer | 1991 - 1993 | O3, trace gases, temperature, aerosols |
| ISAMS | Improved Stratospheric and Mesospheric Sounder | 1991 - 1992 | O3, trace gases, temperature, aerosols |
| MLS | UARS Microwave Limb Sounder | 1991 - 1999 | ozone, trace gases, humidity, temperature |
| HALOE | Halogen Occultation Experiment | 1991 - 2005 | O3, trace gases, temperature, aerosols |
| HRDI | High Resolution Doppler Imager | 1991 - 2005 | temperature and wind fields |
| WINDII | WIND Imaging Interferometer | 1991 - 2005 | temperature and wind fields |
| SSBUV | Shuttle Solar Backscatter Ultraviolet Spectrometer | 1989 - 1996 | O3 & trace gases profiles; solar irradiances |
| ATMOS | Atmospheric Trace Molecule Spectroscopy | 1985 - 1986 | profiles of ozone & trace gases |
| TOVS | TIROS Operational Vertical Sounder (Path Finder) | 1985 - 1995 | O3, atm-profiles, wind, SST, cloud, rainfall |
| DAO | Data Assimilation Office | 1980 - 1993 | O3, atm-profiles, wind, SST, cloud, rainfall |
| MSU | Microwave Sounding Unit- Limb93 | 1979 - 1994 | temperature and humidity profiles |
| TRMM | Tropical Rainfall Measuring Mission sensors | 1997 - present | surface rain fall and drop size distribution |
| SORCE | Solar Radiation and Climate Experiment Sensors | 2003 - present | solar total and spectral irradiance |
| SUSIM | Solar Ultraviolet Spectral Irradiance Monitor | 1991 - 2005 | solar UV spectral irradiance |
| SOLSTICE | Solar Stellar Irradiance Comparison Experiment | 1991 - 2005 | solar UV spectral and stellar irradiance |
| ACRIM | Active Cavity Radiometer Irradiance Monitor | 1980 - 2002 | total solar irradiance |
| PEM | PEM - Particle Environment Monitor | 1991 - 2005 | x-rays, high energy particles deposition |
| MERRA | Modern Era Retrospective-analysis for Research and Applications (MERRA) data products | 1979 –present | GEOS-5 based GMAO Assimilated atmospheric dynamics parameters |

Note: GES DISC also provides some selected ocean color, sediments, chlorophyll, and sea surface temperature data from SeaWiFS,  some aerosols and cloud parameters from MODIS, and a full suite of aerosol parameters from the GOCART model (see http://giovanni.gsfc.nasa.gov).

Data Read Software and Data Mining Tools for OMI Products

A majority of the data sets archived at the NASA GES DISC are in the Hierarchical Data Format-Earth Observing System (HDF-EOS) format.  This format has been adopted by NASA for standard data product distribution since it is able to handle multiple types of data objects and at the same time is independent of the platform or operating system the file has been created on. Two versions of HDF-EOS format, HDF-EOS 2.x (based on native HDF4 format) and HDF-EOS 5.x (based on native HDF5 format) are in use and are usually referred as HE4 and HE5 format, respectively. OMI Level 1B data files (radiances, irradiance, and calibration files) will be written in HE4 format and OMI Level-2 & 3 products (derived geophysical parameters) will be in HE5 format and the file names will have extension “.he4” and “.he5”, respectively.

Many software and tools have been developed by the NASA data centers and software developers to read HDF-EOS files. Following the Terra and Aqua EOS missions, NASA data centers (including GES DISC) have developed a number of tools for accessing and analyzing HE4 files. Some new tools are now being developed to handle data products in HE5 data format. These tools can examine and modify the HE4 and HE5 file contents, dump the files, extract the required data objects (images, orbital swath data, gridded data, and tables), can display the content, convert files from old HDF  format to new format and vice versa or  convert the file into familiar ASCII or binary format.

For example, HE5View and HDFView are file viewing tools developed for viewing the contents of HE5 data files. The HE5View application allows the user to convert HDF-EOS data files to various formats including binary and ASCII. Another popular software is HECONVERT. It is a command-line conversion utility used to convert HE4 files to HE5.  Complete list of the old and new software and tools for HDF-EOS file are available from the GES DISC web site http://disc.gsfc.nasa.gov/Aura/tools.shtml

The Atmospheric Composition Data Support Team at GES DISC provides assistance to the user in the areas of accessing data products, documentation, and data browse and data analysis software. For users convenience “Read Software” and data analysis tools have been developed by GES DISC Data Support Team in the C, Fortran, and IDL languages. These programs are capable of data extraction, subsetting, analysis, reprojection, and format conversion (binary, GIS, or ASCII file). Some (e.g. atmos, hdf_reader) are simple command-line based data browse and data extraction programs and consist of only a few lines which users can easily insert in their data analysis code. Some tools with sophisticated visualization and analysis capabilities though developed for some other sensors are modified to handle OMI Level-1B products created in HE4 format.

For example, a new multifunctional data processing and visualization tool (an enhanced version of HDFLook), has been developed for the data products in HE4 format. The capabilities of this tool include: automatic detection of  the fill values and masks, re-mapping of swath data to world map, geo-projection conversion, interactive and batch mode processing capabilities, subsetting features (availability of parameter, band, and spatial subsetting), multi-file processing (mosaic and stitch capabilities), capability of producing enhanced false and true color (RGB) images with option of map and grid overlays, data conversion from scaled quantities to physical units, format conversion from HDF-EOS to ASCII, Binary, JPEG, or GeoTIFF.  This tool and other data analysis software are available free from the NASA GES DISC (http://disc.gsfc.nasa.gov/ ). Similar tools are being developed at the GES DISC for OMI Level-2 & 3, and other Aura products created in HE5 format.

Web based Interactive Data Mining Capabilities Applicable to OMI Data Products

To promote wider distribution and use of the NASA remote sensing data archived at the GES DISC, web based servers (WebGIS, OpenGIS, WMT-DODS, OPenDap) have been made available to provide on-line, interactive capabilities for data searching, visualization, mapping, analysis, and retrieval of data in user requested formats. For example, WebGIS  is developed to provide on-the fly format conversion of data from large data sets (including the HDF format). For different type of data requested, WebGIS searches the DISC online data holdings and creates maps of spatially subsetted data in Geographic Information System (GIS) formats. The users can create images overlaid on top of each other (with desired transparency). The GES DISC also provides a Near-line Archive Data Mining (NADM) system (http://disc.gsfc.nasa.gov/services/). The NADM enables registered users to test and execute their own data mining algorithms against the data pool of Earth sciences data for long-term studies, including seasonal and annual variability. In addition to that, another interactive web based tool called Giovanni (http://giovanni.gsfc.nasa.gov) is available that provides the capability of on-line multi-satellite data analysis and visualization.

- Data Access

One of the main goals of the NASA GES DISC is making the data easily accessible to the wider user community. All of the data sets archived at GES DISC are freely made available to the public and science user communities. These data sets may be accessed via several mechanisms including:

- GES DISC Search and Order System

For accessing Aura (http://disc.gsfc.nasa.gov/Aura/ ) and other earth-atmospheric satellite and model data products, GES DISC provides a user friendly Web-based data ordering system called Mirador (http://mirador.gsfc.nasa.gov ). It provides spatial & temporal search, data format conversions, parameter extraction and spatial subsetting. Mirador also provides Google-like keywords (Ozone, Aerosol, Trace Gases, OML1B, etc ) or event based  ( hurricane, volcano etc.)  data search  capabilities.

- EOS Warehouse Inventory Search Tool (WIST)

This interface provides access to satellite and ground based data that are available from all NASA data archive centers and NASA affiliated centers (http://wist.echo.nasa.gov/ ).

7. SUMMARY

OMI is a new instrument, with a heritage from the European satellite instruments GOME and SCIAMACHY, and US satellite instruments SBUV and TOMS. OMI’s unique capabilities for measuring important trace gases with a small footprint and daily global coverage, in conjunction with the other Aura instruments, will make a major contribution to our understanding of stratospheric and tropospheric chemistry and climate change. OMI will provide data continuity to more than 30-year of ozone data record from TOMS and other BUV instruments. In addition to the total ozone column, OMI will provide several key parameters for stratospheric and tropospheric chemistry and climate research, including the ozone profile, columns of NO2, SO2, OClO, BrO, HCHO, surface UV Erythemal irradiance, and aerosol and cloud properties.  Extensive data validation activities are planned to evaluate and improve algorithm by comparing products derived from in-situ observations.  OMI products will be used by the scientific community to monitor ozone, air quality and detect subtle signatures of climate change. OMI atmospheric products, including calibrated and geolocated radiance products, will be archived at the NASA Goddard Earth Sciences (GES) Distributed Active Archive Center (DISC) and will be freely available (http://disc.gsfc.nasa. Gov/ ) to the scientific and other data user communities.

The Atmospheric Composition Data Support Team at the NASA GES DISC has been providing science and data support to assist users in accessing and using the atmospheric data products. A number of tools for data access, subsetting, reprojection and mapping of orbital (level-1B & 2) products, visualization of global gridded (Level-3) products, and data analysis have been developed at the NASA GES DISC and are provided free to the data user communities (http://disc.gsfc.nasa.gov ).

Acknowledgements

The authors would like to acknowledge the OMI Science Team Members and OSIPS Team Members for their help in providing the information. This work is supported by the NASA GSFC Earth Science Enterprise.

- REFERENCES

1.	P. F. Levelt, B. van den Oord, E. Hilsenrath, G. W. Leppelmeier, P. K. Bhartia, A. Malkki, H. Kelder, R. J. van der A, E. J. Brinksma, R. van Oss, P. Veefkind, M. van Weele, and R. Noordhoek, “Science Objectives of EOS-Aura's Ozone Monitoring Instrument (OMI),” Proc. Quad. Ozone  Symposium, Sapporo, Japan,  pp. 127-128,  2000.

2.	P. F. Levelt, et al., “Scientific Requirements Document for OMI-EOS,” RS-OMIE-KNMI-001, version 2, ISBN 90-369-2187-2, KNMI publication: 193, 2000.

3.	P. F. Levelt, J. P. Veefkind, R. H. M. Voors, and J. de Vries, “Instrument Description,”   Algorithm Theoretical Baseline Document: OMI Instrument, Level 0 - 1B processor, Calibration & Operations,  P. F. Levelt (ed.), vol. I, ATBD-OMI-01, version 2, Aug. 2002. (http://eospso.gsfc.nasa.gov/eos_homepage/for_scientists/atbd/docs/OMI/ ATBD-OMI-01.pdf)

4.	P. Stammes, P. F. Levelt, J. de Vries, H. Visser, B. Kruizinga, C. Smorenburg, G. Leppelmeier, and E. Hilsenrath, "Scientific requirements and optical design of the Ozone Monitoring Instrument on EOS-Aura," Proceedings of SPIE conference on Earth Observing Systems IV, Denver, Colorado, SPIE vol. 3750,  pp. 221-232, 1999.

5.	R. H. M. Voors, M. R. Dobber, and R. J. Dirksen, “ OMI Calibration and Characterisation,” Algorithm Theoretical Baseline Document: OMI Instrument, Level 0 - 1B processor, Calibration & Operations,  P. F. Levelt (ed.), vol. I, ATBD-OMI-01, version 2, Aug. 2002.

6.	G. H. J. van den Oord, R. H. M. Voors, and J. de Vries, “The Level 0 to Level 1B processor for OMI radiance, irradiance and calibration data,”  Algorithm Theoretical Baseline Document: OMI Instrument, Level 0-1B processor, Calibration & Operations,  P. F. Levelt (ed.), vol. I, ATBD-OMI-01, version 2, Aug. 2002.

7.	Dutch Space, “Output products and Metadata,” GDPS Input/Ouput Data Specification (IODS), vol. 2,

SD-OMIE-7200-DS-467, 9 April 2003.

8.	P. K. Bhartia and C. W. Wellemeyer, “OMI TOMS-V8 Total O3 Algorithm,”   Algorithm Theoretical Baseline Document: OMI Ozone Products, P. K. Bhartia (ed.), vol. II, ATBD-OMI-02, version 2.0, Aug. 2002. (http://eospso.gsfc.nasa.gov/eos_homepage/for_scientists/atbd/docs/OMI/ATBD-OMI-02.pdf)

9.	J. P. Veefkind and J. F. de Haan, “DOAS Total O3 Algorithm,” Algorithm Theoretical Baseline Document: OMI Ozone Products, P.  K. Bhartia (ed.), vol. II, ATBD-OMI-02, version 2.0, Aug. 2002.

10.	R. F. van Oss, R. H. M. Voors, and R.  D. J. Spurr, “Ozone Profile Algorithm,” Algorithm Theoretical Baseline Document: OMI Ozone Products, P.  K. Bhartia (ed.), vol. II, ATBD-OMI-02, version 2.0, Aug. 2002.

11.	J. Fishman, S. Chandra, and J. Ziemke, “Tropospheric O3 Residual,”  Algorithm Theoretical Baseline Document: OMI Ozone Products, P.  K. Bhartia (ed.), vol. II, ATBD-OMI-02, version 2.0, Aug. 2002.

12.	J. R. Acarreta and J. F. de Haan, “Cloud pressure algorithm based on O2-O2 absorption,” Algorithm Theoretical Baseline Document: Clouds, Aerosols, & Surface UV Irradiance, P. Stammes (ed.), vol. III, ATBD-OMI-03, version 2.0, Aug.2002. (http://eospso.gsfc.nasa.gov/eos_homepage/for_scientists/atbd/docs/OMI/ATBD-OMI-03.pdf ).

13.	J. Joiner, A. Vasilkov, D. Flittner, E. Buscela, and J. Gleason, “Retrieval of Cloud Pressure from Rotational Raman Scattering,”  in  Algorithm Theoretical Baseline Document: Clouds, Aerosols, and Surface UV Irradiance,  P.  Stammes (ed.), vol. III, ATBD-OMI-03, ver. 2.0, Aug. 2002.

14.	O. Torres, R. Decae, P. Veefkind, and G. de Leeuw, “OMI Aerosol Retrieval Algorithm,” Algorithm Theoretical Baseline Document: Clouds, Aerosols, and Surface UV Irradiance, P. Stammes (ed.), vol. III, ATBD-OMI-03, version 2.0, Aug. 2002.

15.	N. A. Krotkov, J. Herman, P. K.Bhartia, Colin Seftor, A. Arola, J. Kaurola, P. Taalas, A. Vasilkov, “OMI Surface UV Irradiance Algorithm,”Algorithm Theoretical Baseline Document: Clouds, Aerosols, and Surface UV Irradiance ,  P. Stammes (ed.), vol. III, ATBD-OMI-03, version 2.0, Aug. 2002.

16.	K. Chance, T. P. Kurosu, and L. S. Rothman, “HCHO”, “OClO” , “BrO”, Algorithm Theoretical Baseline Document: OMI Trace Gas Algorithms, K. Chance (ed.), vol. IV, ATBD-OMI-04, version 2.0, Aug. 2002.

(http://eospso.gsfc.nasa.gov/eos_homepage/for_scientists/atbd/docs/OMI/ATBD-OMI-04.pdf)

17.	F. Boersma, E. Bucsela, E. Brinksma, and J. F. Gleason, “NO2“, Algorithm Theoretical Baseline Document: OMI Trace Gas Algorithms, K. Chance (ed.), vol. IV, ATBD-OMI-04, version 2.0,  Aug. 2002.

18.	A. J. Krueger, N. A. Krotkov, S. Datta, D. Flittner, and O. Dubovik, “SO2”,  in  Algorithm Theoretical Baseline Document: OMI Trace Gas Algorithms, K. Chance (ed.), vol. IV, ATBD-OMI-04, version 2.0,  Aug. 2002.

19.	L. Froidevaux and A. Douglass, “Earth Observing System (EOS) Aura Science Data Validation Plan”, July 2001.

(http://aura.gsfc.nasa.gov/project/validation.html).

Suraiya.Ahmad-1@ nasa.gov; phone 301-614-5284; fax 301-614-5268; http://disc.gsfc.nasa.gov/