TITLE:	Common Upper Layer Requirements (CULR) for the Directory

SOURCE:	AOW/DIR SIG

DATE:	30-MAY-1996

STATUS:	Working Draft Version 1.0 of PDISP xxxxx-X: 1993(E)

This document is a draft for the specification of the Directory how the OSI Remote Operations Service Element, the Association Control Service Element, the Presentation Layer, and the Session Layer standards shall be used to provide the upper layer functions.

Contents

Contents	ii

Foreword	iii

Introduction	v

1 Scope	1

2 Normative References	1

2.1 Paired ITU-T Recommendations | International Standards equivalent in technical content	1

2.2 Normative Amendments and Technical Corrigenda	1

2.3 International Standardized Profiles	2

2.4 Additional References	2

3 Definitions	2

4 Abbreviations	2

5 Conformance	3

5.1 ROSE	3

5.2 ACSE	3

5.3 Presentation	4

5.4 Session	4

Annex A(normative) Profile Requirements List for ROSE, ACSE, Presentation and Session	5

A.1 General	5

A.2 References	5

A.3 Classification of requirements	5

A.4 ROSE PRL	7

A.5 ACSE PRL	9

A.6 Presentation PRL	10

A.7 Session PRL	10

Annex B(normative) Amendments and corrigenda	11

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standard through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work.

In the field of information technology, ISO and IEC have established a joint technical committee, ISO/IEC JTC1. In addition to developing International Standards, ISO/IEC JTC1 has created a Special Group on Functional Standardization for the elaboration of International Standardized Profiles (ISPs).

An International Standardized Profile is an internationally agreed, harmonized document which identifies a standard or group of standards, together with options and parameters, necessary to accomplish a function or a set of functions.

Draft International Standardized Profiles are circulated to national bodies for voting. Publication as an International Standardized Profile requires approval by at least 75 % of the national bodies casting a vote.

This ISP, ISO/IEC ISP xxxxx was prepared with the collaboration of the following organizations:

•	Asia-Oceania Workshop (AOW)

•	European Workshop for Open Systems (EWOS)

•	OSE Implementors Workshop (OIW)

ISO/IEC ISP xxxxx consists of the following parts, under the general title Information technology - International Standardized Profiles ADYnn - The Directory

•	Part 0 : Common Upper Layer Requirements for the Directory

•	Part 1 - ADY11: DUA support of Directory Access Protocol

•	Part 2 - ADY12: DUA support of Distributed Operations

•	Part 3 - ADY21: DSA support of Directory Access

•	Part 4 - ADY22: DSA support of Distributed Operations

•	Part 5 - ADY41: DUA Authentication as DAP initiator

•	Part 6 - ADY42: DSA Authentication as DAP responder

•	Part 7 - ADY43: DSA Authentication as DSP

•	Part 8 - ADY44: DSA Simple Access Control

•	Part 9 - ADY45: DSA Basic Access Control

•	Part 10 - ADY51: Shadowing using ROSE

•	Part 11 - ADY52: Shadowing using RTSE

•	Part 12 - ADY53: Shadowing subset

•	Part 13 - ADY61: Administrative areas

•	Part 14 - ADY62: Establishment and utilization of shadowing agreements

•	Part 15 - ADY63: Schema administration and publication

•	Part 16 - ADY71: Shadowing Operational Binding

•	Part 17 - ADY72: Hierarchical Operational Binding

•	Part 18 - ADY73: Non-specific Hierarchical Operational Binding

The present document contains two normative annexes:

•	Annexe A: Profile Requirements List for ROSE, ACSE, Presentation and Session (normative)

•	Annexe B: Amendments and corrigenda (normative)

Introduction

The concept and structure of International Standardized Profiles for Information Systems are laid down in the Technical Report ISO/IEC TR 10000.  The purpose of an International Standardized Profile is to recommend when and how certain fnformation technology standards shall be used.  This International Standardized Profile ISO/IEC ISP xxxxx-x specifies application profile ADYxx as defined in the Technical Report ISO/IEC TR 10000-2.

ISO/IEC ISP xxxxx-x is one of a set of International Standardized Profiles relating to the Directory (see TR 10000-2) for the ‘93 standards.

ISO/IEC ISP xxxxx-x profiles the Directory how the OSI Remote Operations Service Element, the Association Control Service Element, the Presentation Layer, and the Session Layer standards shall be used to provide the upper layer functions.

Information Technology-International Standardized

Profiles ADInn-OSI Directory

Part x: Common Upper Layer requirements for the Directory

1	Scope

This part of this ISP specifies the status for the support of the Session, presentation, ACSE and ROSE protocols for the profiles identified in ISO/IEC ISP xxxxx.  It documents Directory-specific upper layer requirements over and above the common upper layer requirements specified in ISO/IEC 11188 -1, -2 and -3.

This part of ISO/IEC xxxxx does not specify a full A-profile, and therefore is included by other parts of ISO/IEC xxxxx.

RTSE is a future study.

2	Normative References

The following ITU-T Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this International Standardized Profile.  This International Standardized Profile profiles the 1993 edition of the Directory Specifications.

Amendments and corrigenda to the base standards references : See annex B for a complete list of these documents which are used in this ISP.

2.1	Paired ITU-T Recommendations | International Standards equivalent in technical content

•	ITU-T Recommendation X.518: 1993(E), Data Communications - Open Systems Interconnection (OSI) - The Directory - Procedures for Distributed Operation.

ISO/IEC 9594-4, Information Technology - Open Systems Interconnection (OSI) - The Directory - Procedures for Distributed Operation.

•	ITU-T Recommendation X.519: 1993(E), Data Communications - Open Systems Interconnection (OSI) - The Directory - Protocol Specifications.

ISO/IEC 9594-5, Information Technology - Open Systems Interconnection (OSI) - The Directory - Protocol Specifications.

•	CCITT Recommendation X.249(1992), Remote Operations Service Element - Protocol Implementation Conformance Statement Proforma.

ISO/IEC DIS 14720, Information Technology - Open Systems Interconnection - Remote Operations Service Element Protocol Specification - Part 4: Protocol Implementation Conformance Statement(PICS) Proforma.

2.2	Normative Amendments and Technical Corrigenda

In accordance with TR10000-1 clause 6.3.2 c), attention is drawn to normative Amendments and Technical Corrigenda affecting the Directory Standards documents ISO/IEC 9594 and the ITU-T X.500 recommendations.

Annex B defines the references to the agreed amendments and corrigenda within the Directory Implementor’s Guide.  Compliance with these amendments and corrigenda is necessary to achieve the interoperability requirements for this document.  It also identifies those which appear to be relevant to this ISP.

•	Draft Technical Corrigendum 1 to Recommendation X.518 (1993) | ISO/IEC 9594-4: 1993 (addressing DRs 9594/094, 106, 108, 109, 111, 113, 114, 115)

•	Draft Technical Corrigendum 1 to Recommendation X.519 (1993) | ISO/IEC 9594-5: 1993 (addressing DRs 9594/075, 124)

•	Draft Technical Corrigendum 2 to Recommendation X.519 | ISO/IEC 9594-5 (addressing DR  9594/074)

•	Draft Technical Corrigendum 2 to Recommendation X.519 | ISO/IEC 9594-5 (addressing DR  9594/075)

•	Draft Technical Corrigendum 2 to Recommendation X.519 | ISO/IEC 9594-5 (addressing Defect Report 9594/074)

•	Draft Technical Corrigendum 4 to Recommendation X.518 | ISO/IEC 9594-4 (addressing DRs 9594/071, 072)

2.3	International Standardized Profiles

•	ISO/IEC ISP11188-1: 1995, Information technology - International Standardized Profile - Common upper layer requirements - Part 1: Basic connection oriented requirements

•	ISO/IEC DISP11188-2, Information technology - International Standardized Profile - Common upper layer requirements - Part 2: Basic connection oriented requirements for ROSE-based Profiles

•	ISO/IEC DISP11188-3, Information technology - International Standardized Profile - Common upper layer requirements - Part 3: Minimal OSI upper layer facilities

2.4	Additional References

•	ISO/IEC TR 10000-1: 1995, Information technology - Fremework and Taxonomy of International Standardized Profiles - Part 1: General principles & documentation framework

•	ISO/IEC TR 10000-2: 1995, Information technology - Framework and Taxonomy of International Standardized Profiles - Part 2: Principles & Taxonomy for OSI Profiles

•	ISO/IEC TR 10000-3: 1995, Information technology - Framework and Taxonomy of International Standardized Profiles - Part 3: Principles & Taxonomy for OSE Profiles

3	Definitions

The definitions in ISO/IEC ISP 11 188-2 are also valid for this part of ISO/IEC xxxxx.

(Editor's Note: Is a Directory specific definition of support level necessary?)

4	Abbreviations

The following abbreviations are used as defined in [ISO/IEC 9594 | CCITT Rec. X.500 series] or in ISO/IEC TR-10000-1:

AARE	A-Associate-Response (APDU)

AARQ	A-Associate-Request (APDU)

AB		Abort (SPDU)

ABRT	A-Abort (APDU)

AC		Accept (SPDU)

ACSE	Association Control Service Element

AE		Application Entity

AP		Application Process

APDU	Application Protocol Data Unit

ARP	Abnormal Release Provider (PPDU)

ASN.1	Abstract Syntax Notation One

CCITT	International Telegraph and Telephone Consultative Committee

CN		Connect (SPDU)

CP		Connect Presentation (PPDU)

CPA	Connect Presentation Accept (PPDU)

CPR	Connect Presentation Reject (PPDU)

CULR	Common Upper Layer Requirements

DAP	Directory Access Protocol

DSA	Directory System Agent

DSP	Directory System Protocol

DT		Data Transfer (SPDU)

DUA	Directory User Agent

FN		Finish (SPDU)

ID		Identifier

IEC	International Electrotechnical Commission

ISO	International Organaization for Standardization

ISP		International Standardized Profile

JTC1	Joint Technical Committee One

PICS	Protocol Implementation Conformance Statement

PPDU	Presentation Protocol Data Unit

PRL	Profile Requirements List

RF		Refuse (SPDU)

RLRE	A-Release-Response (APDU)

RLRQ	A-Release-Request (APDU)

RO		Remote Operations

ROIV	RO-INVOKE Application Protocol Data Unit

ROSE	Remote Operations Service Element

SPDU	Session Protocol Data Unit

TD		Presentation Data (PPDU)

5	Conformance

This part of ISO/IEC ISP xxxxx states requirements upon implementations for the purpose of achieving interworking. A claim of conformance to this part of ISO/IEC xxxxx is a claim that all requirements in the relevant base standards are satisfied, and that all requirements in the following clauses and in Annex A of this part of ISO/IEC ISP xxxxx are satisfied.

To conform to this part of ISO/IEC ISP xxxxx, implementations shall conform to all the requirements of clause 9 in ISO/IEC IS 9594-5, including the requirements directly and indirectly referenced that clause.

5.1	ROSE

To conform to the ROSE protocol used in this part of ISO/IEC ISP xxxxx implementations shall implement all supported features identified in A.4.

5.2	ACSE

To conform to the ACSE protocol used in this part of ISO/IEC ISP xxxxx implementations shall implement all supported features identified in A.5.

5.3	Presentation

To conform to the Presentation protocol used in this part of ISO/IEC ISP xxxxx implementations shall implement all supported features identified in A.6.

DSAs shall support ASN.1 errors in the following way:

(1) Errors in the ASN.1 TLV (type/length/value) structure of incoming Directory protocol (DAP or DSP) may be detected by the Presentation Layer and then may cause Presentation Provider Aborts. Similarly, syntactical errors in the use of ASN.1 by the Presentation Layer, ROSE, or ACSE may also be detected by the Presentation Layer, and may then cause Presentation Provider Aborts.

(2) Syntactical errors in relation to the DAP or DSP protocols, excluding syntactical errors associated with ASN.1 ANY types, shall not give rise to Presentation Layer errors, but shall give rise to a DAP Reject or A-ABORT, or be handled by errors defined within the DAP or DSP protocols, as appropriate. The detection of such syntactical errors shall take into account ASN.1 Extensibility requirements.

(3) DSAs shall not be obliged to detect syntactical errors within instances of ASN.1 types which are to be relayed and not acted upon locally.

(4) Syntactical errors associated with ASN.1 ANY types as used in the Attribute Value ASN.1 type shall be handled by errors or other indications as defined by the Directory standards and ISPs.

(5) The handling of syntactical errors in other ANY constructs (e.g. info in DomainInfo) shall be a local option.

5.4	Session

To conform to the Session protocol used in this part of ISO/IEC ISP xxxxx implementations shall implement all supported features identified in A.7

Annex A(normative)

Profile Requirements List for ROSE, ACSE, Presentation and Session

A.1	General

This annex describes the ROSE, ACSE, Presentation and Session requirements for the Directory. The requirements in this annex are formulated with tables that reference the base standards PICS proformas.

A.2 References

The PICS proforma reference column from A.4 to A.7 tables are refered the base standard PICS proformas. The first letter identifies the specific PICS proforma:

R: ROSE - ISO/IEC DIS 14720

A: ACSE - ISO/IEC 8650-2

P: Presentation - ISO/IEC 8823-2

S: Session - ISO/IEC 8327-2

The character from the second character to the solidus('/') form a reference to the specific subclause in annex A of that PICS proforma which contains the table in question. The number after the solidus references the row number in the table.

A.3	Classification of requirements

The Profile status indicators used in this Profile are presented in table 1.

Column 2 of table 1 is the meaning of "status value of CULR-3" column that is defined by CULR-3. Each entries of A.5, A.6 and A.7 are chosen from this list. A.4: ROSE PRL is not included in CULR-3, corresponding column of A.4 is defined by referencing CULR-2 and ISO/IEC DIS 14720.

Column 3 of table 1 is the meaning of "status value of this profile" column which reflects the requirements of this document. Each entries of A.4, A.5, A.6 and A.7 are chosen from this list.

Table 1 - Profile status identifiers

|  | Identifier | Meaning of "Status Value of CULR-3" column | Meaning of "Status Value of this Profile" column |
| --- | --- | --- | --- |

| 1 | m | mandatory - The feature shall be required for support. The status of the feature shall remain mandatory in a referencing specification. | mandatory - The implementation shall support the feature, i.e. its syntax and procedures shall be implemented as specified in the base standard or in ISO/IEC ISP 11188-1. However, it is not a requirement that the feature be used in all instances of communication unless mandated by the base standard or stated otherwise in ISO/IEC ISP 11188-1. The feature shall be the subject of an ISP conformance test. |
| --- | --- | --- | --- |

| 2 | o | optional - The choice of whether this feature is supported or it is not supported is made by the implementation. The status of the feature shall remain optional in a referencing specification. | optional - The implementation may decide either to support or to not support the feature. _ESupporting the feature means that the feature shall be handled as if it was mandatory. _ENot supporting a feature depends on receiving or sending. For sending, the feature's capability is not used. For receiving an optional parameter, the syntax shall be implemented and the parameter may be ignored. For receiving a valid PDU that is optional, the implementation may ignore the PDU or it may abort the association. |
| --- | --- | --- | --- |

| 3 | * | open - The status of this feature shall be decided by the referencing specification. The referencing specification shall indicate that the status of the feature is mandatory, optional, or out of scope. Alternately, the referencing specification may keep the feature open. | open - same as optional |
| --- | --- | --- | --- |

| 4 | x | excluded - The feature shall not be used in a referencing specification. The status of the feature shall remain excluded in a referencing specification. | excluded - The implementation shall not support the feature. When completing the associated PICS proforma table, the answer for the support column shall be that the feature has not been implemented. The implementation shall abort if the feature is received and should be the subject of an ISP conformance test. |
| --- | --- | --- | --- |

| 5 | i | out of scope - The requirement for the support of this feature is not covered by this Profile. The status of the feature shall remain out of scope in a referencing specification. | out of scope - Support for a feature shall follow the guidelines outlined for optional above with the exception that this feature shall not be the subject of an ISP conformance test. |
| --- | --- | --- | --- |

| 6 | - | not applicable - The feature is not relevant where mentioned in a table. Support for the feature is either meaningless, logically impossible, or physically impossible, after some conditions are evaluated. The status of the feature shall remain not applicable in a referencing specification. | not applicable - The feature is not defined by the base standard in the context where it is mentioned in a table. A support answer is not required from the implementor. |
| --- | --- | --- | --- |

| 7 | c[n] | conditionally supported - Support for the feature is further defined in this Profile by condition("n") annexed to the table. The value evaluated from the condition evaluates to one of the following values defined in this table: "m"; "o";"i", or "-". | conditionally supported - Support for the feature is further defined by a condition ("n") which is annexed to the table. Depending on the condition, when completing the associated PICS Proforma table, the answer for the support column shall either be: the feature has been implemented: the feature has not been implemented: or not applicable. |
| --- | --- | --- | --- |

The definitions of conditional items may be found in the respective PICS proformas.

Where the status and the profile entry contains two classifications separated by a comma, these reference the sending and receiving capabilities respectively.

Editor's note:

Column 2 of table 1 is the same definition that CULR-3 define as "meaning when referencing by a specification" in table 1. But this definition seems too strict. At least, status of "o" and "i" may change in a referencing specification.

A.4	ROSE PRL

| PICS Proforma Reference | Name of Item | Normative Reference | Status Value of CULR-2 | Status Value of this_@Profile |
| --- | --- | --- | --- | --- |

| R.A.6.1/1 | Is Operation Class 1 supported ? (application entity requirements) |  | o.1 | c.1 |
| --- | --- | --- | --- | --- |

| R.A.6.1/2 | Is Operation Class 2 supported ? (application entity requirements) |  | o.1 | c.2 |
| --- | --- | --- | --- | --- |

| R.A.6.1/3 | Is Operation Class 3 supported ? (application entity requirements) |  | o.1 | x |
| --- | --- | --- | --- | --- |

| R.A.6.1/4 | Is Operation Class 4 supported ? (application entity requirements) |  | o.1 | x |
| --- | --- | --- | --- | --- |

| R.A.6.1/5 | Is Operation Class 5 supported ? (application entity requirements) |  | o.1 | x |
| --- | --- | --- | --- | --- |

| R.A.6.1/8 | Is the ROSE a component of a application entity that supports the origination of linked operations ? |  | o | x |
| --- | --- | --- | --- | --- |

| R.A.6.1/9 | Is the ROSE a component of a application entity that supports the reception of linked operations ? |  | o | x |
| --- | --- | --- | --- | --- |

| R.A.6.4/2 | linked-ID (ROIV origination) |  | c.7 | x |
| --- | --- | --- | --- | --- |

| R.A.6.5/2 | linked-ID (ROIV reception) |  | c.8 | x |
| --- | --- | --- | --- | --- |

|  | Bind/Unbind Invoker |  |  | c.3 |
| --- | --- | --- | --- | --- |

|  | Bind/Unbind performer |  |  | c.4 |
| --- | --- | --- | --- | --- |

|  | Is Association Class 1 supported ? |  |  | c.5 |
| --- | --- | --- | --- | --- |

|  | Is Association Class 2 supported ? |  |  | x |
| --- | --- | --- | --- | --- |

|  | Is Association Class 3 supported ? |  |  | c.6 |
| --- | --- | --- | --- | --- |

o.1: optional. but at least, one of them shall be required for support.

c.1: if [DUA supporting DAP] or [DSA supporting DAP] then o.1 else n/a.

c.2: if [DUA supporting DAP] or [DSA supporting DAP] then o.1 else m.

c.3: if [DUA supporting DAP] or [Initiator role of DSA supporting DSP] then m else if [Responder role of DSA supporting DSP] then o else n/a.

c.4: if [DSA supporting DAP] or [Initiator role of DSA supporting DSP] or [Responder role of DSA supporting DSP] then m else n/a.

c.5: if [DUA supporting DAP] or [DSA supporting DAP] then m else n/a.

c.6: if [DSA supporting DSP] m else n/a.

c.7: if [ROSE of application entity supporting Initiator role of link operation] m else n/a.

c.8: if [ROSE of application entity supporting Responder role of link operation] m else n/a.

A.5	ACSE PRL

| PICS Proforma Reference | Name of Item | Normative Reference | Status Value of CULR-3 | Status Value of this Profile |
| --- | --- | --- | --- | --- |

| A.A.8/2 | Authentication (function units) |  | c | i |
| --- | --- | --- | --- | --- |

| A.A.10.1/3 | Calling AP Title (AARQ APDU) | ISP11188, ISO9594-4 _˜13.1.2.1 | *,* | o,o(Only Form 1,based on Directory Name shall be generated and accepted) |
| --- | --- | --- | --- | --- |

| A.A.10.1/4 | Calling AE Qualifier (AARQ APDU) | ISP11188, ISO9594-4 _˜13.1.2.1 | *,* | o,o(Only Form 1,based on Directory Name shall be generated and accepted) |
| --- | --- | --- | --- | --- |

| A.A.10.1/5 | Calling AP Invocation-identifier (AARQ APDU) |  | *,* | o,o |
| --- | --- | --- | --- | --- |

| A.A.10.1/6 | Calling AE Invocation-identifier (AARQ APDU) |  | *,* | o,o |
| --- | --- | --- | --- | --- |

| A.A.10.1/7 | Called AP Title (AARQ APDU) | ISP11188, ISO9594-4 _˜13.1.2.1 | *,* | o,o(Only Form 1,based on Directory Name shall be generated and accepted) |
| --- | --- | --- | --- | --- |

| A.A.10.1/8 | Called AE Qualifier (AARQ APDU) | ISP11188, ISO9594-4 _˜13.1.2.1 | *,* | o,o(Only Form 1,based on Directory Name shall be generated and accepted) |
| --- | --- | --- | --- | --- |

| A.A.10.1/9 | Called AP Invocation-idenrifier (AARQ APDU) |  | *,* | o,o |
| --- | --- | --- | --- | --- |

| A.A.10.1/10 | Called AE Invocation-identifier (AARQ APDU) |  | *,* | o,o |
| --- | --- | --- | --- | --- |

| A.A.10.1/15 | User Information (AARQ APDU) |  | *,* | m,m |
| --- | --- | --- | --- | --- |

| A.A.10.2/3 | Responding AP Title (AARE APDU) | ISP11188, ISO9594-4 _˜13.1.2.1 | *,* | o,o(Only Form 1,based on Directory Name shall be generated and accepted) |
| --- | --- | --- | --- | --- |

| A.A.10.2/4 | Responding AE Qualifier (AARE APDU) | ISP11188, ISO9594-4 _˜13.1.2.1 | *,* | o,o(Only Form 1,based on Directory Name shall be generated and accepted) |
| --- | --- | --- | --- | --- |

| A.A.10.2/5 | Responding AP Invocation-identifier (AARE APDU) |  | *,* | o,o |
| --- | --- | --- | --- | --- |

| A.A.10.2/6 | Responding AE Invocation-identifier (AARE APDU) |  | *,* | o,o |
| --- | --- | --- | --- | --- |

| A.A.10.2/13 | User Information (AARE APDU) |  | *,* | m,m |
| --- | --- | --- | --- | --- |

| A.A.10.3/1 | Reason (RLRQ APDU) |  | *,* | m,m |
| --- | --- | --- | --- | --- |

| A.A.10.3/2 | User Information (RLRQ APDU) |  | *,* | o,o |
| --- | --- | --- | --- | --- |

| A.A.10.4/1 | Reason (RLRE APDU) |  | *,* | m,m |
| --- | --- | --- | --- | --- |

| A.A.10.4/2 | User_@Information (RLRE APDU) |  | *,* | o,o |
| --- | --- | --- | --- | --- |

| A.A.10.5/3 | User_@Information (ABRT APDU) |  | *,* | o,o |
| --- | --- | --- | --- | --- |

A.6	Presentation PRL

| PICS Proforma Reference | Name of Item | Normative Reference | Status Value of CULR-3 | Status Value of this Profile |
| --- | --- | --- | --- | --- |

| P.A.8.1/5 | Default context name (CP PPDU) |  | *,* | x,x |
| --- | --- | --- | --- | --- |

| P.A.8.1/7 | Presentation requirements (CP PPDU) |  | i,m | o,m |
| --- | --- | --- | --- | --- |

| P.A.8.2/5 | Presentation requirements (CPA PPDU) |  | i,i | o,m |
| --- | --- | --- | --- | --- |

| P.A.8.3/4 | Default context result (CPR PPDU) |  | *,* | x,x |
| --- | --- | --- | --- | --- |

| P.A.8.5/2 | Event identifier (ARP PPDU) |  | *,* | m,m |
| --- | --- | --- | --- | --- |

| P.A.8.8/1 | User data (TD PPDU) |  | c,c | m,m |
| --- | --- | --- | --- | --- |

A.7	Session PRL

| PICS Proforma Reference | Name of Item | Normative Reference | Status Value of CULR-3 | Status Value of this Profile |
| --- | --- | --- | --- | --- |

| S.A.8.1.1/1 | Calling ss-user reference (CN SPDU) |  | i,i | o,o |
| --- | --- | --- | --- | --- |

| S.A.8.1.1/2 | Common reference (CN SPDU) |  | i,i | o,o |
| --- | --- | --- | --- | --- |

| S.A.8.1.1/3 | Additional reference information (CN SPDU) |  | i,i | o,o |
| --- | --- | --- | --- | --- |

| S.A.8.1.2.1/2 | Tsdu maximum size (CN SPDU) |  | i,m | o,m |
| --- | --- | --- | --- | --- |

| S.A.8.1.2.1/4 | Initial serial number (CN SPDU) |  | i,i | x,x |
| --- | --- | --- | --- | --- |

| S.A.8.1.2.1/5 | Token setting item (CN SPDU) |  | i,i | x,x |
| --- | --- | --- | --- | --- |

| S.A.8.4.1/1 | Calling ss-user reference (AC SPDU) |  | i,i | o,o |
| --- | --- | --- | --- | --- |

| S.A.8.4.1/2 | Common reference (AC SPDU) |  | i,i | o,o |
| --- | --- | --- | --- | --- |

| S.A.8.4.1/3 | Additional reference information (AC SPDU) |  | i,i | o,o |
| --- | --- | --- | --- | --- |

| S.A.8.4.2.1/2 | Tsdu maximum size (AC SPDU) |  | i,m | o,m |
| --- | --- | --- | --- | --- |

| S.A.8.4.2.1/4 | Initial serial number (AC SPDU) |  | i,i | x,x |
| --- | --- | --- | --- | --- |

| S.A.8.4.2.1/5 | Token setting item (AC SPDU) |  | i,i | x,x |
| --- | --- | --- | --- | --- |

| S.A.8.4.3/1 | Token item (AC SPDU) |  | i,i | x,x |
| --- | --- | --- | --- | --- |

| S.A.8.5.1/1 | Calling ss-user reference (RF SPDU) |  | i,i | o,o |
| --- | --- | --- | --- | --- |

| S.A.8.5.1/2 | Common reference (RF SPDU) |  | i,i | o,o |
| --- | --- | --- | --- | --- |

| S.A.8.5.1/3 | Additional reference information (RF SPDU) |  | i,i | o,o |
| --- | --- | --- | --- | --- |

| S.A.8.5.2/1 | Transport disconnect (RF SPDU) |  | i,i | m,m |
| --- | --- | --- | --- | --- |

| S.A.8.6/1 | Transport disconnect (FN SPDU) |  | i,i | m,m |
| --- | --- | --- | --- | --- |

| S.A.8.9/3 | Reflect Parameter Values (AB SPDU) |  | m,m | i,i |
| --- | --- | --- | --- | --- |

| S.A.8.11/1 | Enclosure Item (DT SPDU) |  | i,i | c,c |
| --- | --- | --- | --- | --- |

Annex B(normative)

Amendments and corrigenda

International standards are subject to constant review and revision by ISO/IEC Technical Committee concerned and by ITU-T. The following amendments and corrigenda are approved by ISO/IEC JTC1 and by ITU-T, but at the date of publication of this ISP they are not yet incorporated in the text of the corresponding base standards as referenced in this ISP. The amendments and corrigenda as listed below are considered as normative references in this ISP.