Title:	Information Technology - International Standardized Profile ADY11 -The Directory Part 1 - DUA Support of Directory Access Protocol

Source:	OIW DS-SIG

(Editors: Iman ELBakry, LOGICON

Bill Curtin Defense Information System Agency (DISA) )

Status:	This is the seventh working draft of this document.  Any comments or suggestions are welcomed.

Date:

Note:		This draft incorporates all the changes resulting from comments, to the previous version, supplied by EWOS.

Contents

Foreword

Introduction

1 Scope

1.1 General

1.2 Position within the taxonomy

1.3 Scenario

2 Normative References

2.1 Paired ITU-T Recommendations | International Standards equivalent in technical content

2.2 Normative Amendments and Technical Corrigenda

2.3 Additional normative references

3 Definitions

3.1 Directory definitions

3.2 Conformance definitions

3.3 Basic directory conformance definitions

4 Abbreviations

5 Conformance

5.1 Conformance Statement

5.2 Static Conformance Requirements

5.3 Dynamic Requirements

5.3.1 Accept all Conformant Responses

5.3.2 Generating Supported Protocol Elements

5.3.3 Restrictions on APDU Size

5.3.4 Non-support of Abandon

Annex A Profile Requirements List

A.1 Introduction

A.2 Identification of the Implementation

A.2.1 Identification of the PICS

A.3 Instructions

A.3.1 Purpose and structure of the PRL

A.3.2 Symbols, terms, and abbreviations

A.3.2.1 Introduction

A.3.2.2 Prerequisite notation

A.3.2.3 Item reference numbers s

A.3.2.4  Protocol Status column

A.3.2.5  Profile Status column

A.3.2.6 Predicate column

A3.2.7 Predicate Name

A.3.2.8 Note column

A.4 PRL Tables

A.4.1 Roles

A.4.2 General Capabilities and Global statement of conformance

A.4.2.1 General Capabilities

A.4.2.2 Supported Security Levels

A.4.2.3 Supported Access Control Schemes

A.4.3 Capabilities and options

A.4.3.1 Supported application context

A.4.3.2 Operations and extensions

A.4.3.2.1 Operations

A.4.3.2.2 Extensions

A.4.3.3  Protocol elements

A.4.3.3.1  DirectoryBind Elements

A.4.3.3.1.1 Directory Bind Arguments

A.4.3.3.1.2 Directory Bind Result

A.4.3.3.1.3 Directory Bind Error

A.4.3.3.2 Directory Unbind Elements

A.4.3.3.3 Read Elements

A.4.3.3.4 Compare Elements

A.4.3.3.5 Abandon Elements

A.4.3.3.6 List Elements

A.4.3.3.7 Search Elements

A.4.3.3.8 Add Entry Elements

A.4.3.3.9 Remove Entry Elements

A.4.3.3.10 Modify Entry Elements

A.4.3.3.11 ModifyDN Elements

A.4.3.3.12 Errors and Parameters

A.4.3.3.13 Common Arguments Elements

A.4.3.3.14 Common Results Elements

A.4.3.3.15 Service Controls

A.4.3.3.16 Entry Information Selection

A.4.3.3.17 Entry Information

A.4.3.3.18 Filter Elements

A.4.3.3.19 Filter Item Elements

A.4.3.3.20 Paged Results

A.4.3.3.21 Continuation Reference

A.4.3.3.22 Security Parameters

A.4.3.3.23 CertificationPath

A.4.3.3.24 Access Control

A.4.3.3.25 Supported References

A.4.3.3.26 Access Point

A.4.4 Directory Schema and Directory System Schema

A.4.5 Supported ISO/IEC 10646-1 Character Sets

Annex B Profile Specific Implementation Conformance Statement

B.1 Identification of Profile Corrigenda

Figures

2  Access to the Directory

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for world-wide standardization.  National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organizations to deal with particular fields of technical activity.  ISO and IEC technical committees collaborate in fields of mutual interest.  Other international organizations, governmental or non-governmental, in liaison with ISO and IEC, also take part in the work.

In the field of information technology, ISO and IEC have established a joint technical committee, ISO/IEC JTC1.  In addition to developing International Standards, ISO/IEC JTC1 has created a Special Group on Functional Standardization for the elaboration of International Standardized Profiles (ISPs).

An International Standardized Profile is an internationally agreed, harmonized document which identifies a standard or group of standards, together with options and parameters, necessary to accomplish a function or set of functions.

Draft International Standardized Profiles are circulated to national bodies for voting.  Publication as an International Standardized Profile requires approval by at least 75% of the national bodies casting a vote.

This ISP, ISO/IEC ISP 15125 was prepared with the collaboration of the following organizations:

- Asia-Oceania Workshop (AOW)

- European Telecommunications Standards Institute (ETSI)

- European Workshop for Open Systems (EWOS)

- Open Systems Environment Implementor’s Workshop (OIW)

- ISO/IEC ISP 15125 consists of the following parts under the general title Information Technology - International Standardized Profiles ISO/IEC ISP 15125 - The Directory

- Introduction

- Supporting Layers

- Part 0 - Common Upper Layer Requirements for the Directory

- Part 1-ADY11:	DUA support of Directory Access Protocol

- Part 2-ADY12:	DUA support of Distributed Operations

- Part 3-ADY21:	DSA support of Directory Access Protocol

- Part 4-ADY22:	DSA support of Distributed Operations

- Part 5-ADY41:	DUA Authentication as DAP initiator

- Part 6-ADY42:	DSA Authentication as DAP responder

- Part 7-ADY43:	DSA Authentication for DSP

- Part 9-ADY45:	DSA Access Control

- Part 10-ADY51:	Shadowing using ROSE

- Part 11-ADY52:	Shadowing using RTSE

- Part 12-ADY53:	Shadowing subset

- Part 13-ADY61:	Administrative areas

- Part 14-ADY62:	Establishment and utilization of shadowing agreements

- Part 15-ADY63:	Schema administration and publication

- Part 16-ADY71:	Shadowing Operational Binding

- Part 17-ADY72:	Hierarchical Operational Binding

- Part 18-ADY73:	Non-specific Hierarchical Operational Binding

- ISO/IEC ISP 15126 consists of the following parts under the general title Information Technology-International Standardized Profiles ISO/IEC ISP 15126 -The Directory

- Part 1-FDY11:	Common Directory Use

- Part 2-FDY12:	Directory System Schema

- The present document contains two normative annexes:

- Annex A: Profile Requirements List of ISO/IEC ISP 15125-1 - DUA support of Directory Access Protocol

- Annex B: Profile Specific Implementation Conformance Statement (ICS)

- Introduction

- The concept and structure of International Standardized Profiles for Information Systems are laid down in the Technical Report ISO/IEC TR 10000.  The purpose of an International Standardized Profile is to recommend when and how certain information technology standards shall be used.  This International Standardized Profile ISO/IEC ISP 15125-1, Part 1, specifies application profile ADY11 as defined in the Technical Report ISO/IEC TR 10000-2.

- ISO/IEC ISP 15125 is one of a set of International Standardized Profiles relating to the Directory (see TR10000-2) for the ‘93 standards.

- ISO/IEC ISP 15125-1 profiles the use of the Directory Access Protocol (DAP) by a Directory User Agent (DUA).

- Information Technology - International Standardized Profile

- 15125 - The Directory

- Part 1:  DUA Support of Directory Access

- 1	Scope

- ISO/IEC ISP 15125-1 (ADY 11) covers the primary DUA conformance requirements for Directory Access. It addresses non-distributed environments, with unsigned operations, and security levels of none, simple without password, and simple with password.

- This part of ISO/IEC ISP 15125 only addresses DUA Directory Access in a non-distributed environment. If DUA Directory Access in a distributed environment is required, the DUA shall conform to ISO/IEC ISP 15125-2 (ADY 12), DUA support of Distributed Operations, in addition to this part of the ISP.

- The security requirements in this part of the ISP apply only for the cases where the security levels is either none, simple without password, or simple with unprotected password.  If simple authentication with protected password or strong authentication is required, the DUA shall conform to ISO/IEC ISP 15125-5 (ADY 41), DUA Authentication as DAP Initiator, in addition to this part of the ISP.

- This part of the ISP only addresses DUA performing unsigned operations. If digitally signed operations are required, the DUA shall conform to ISO/IEC ISP 15125-5 (ADY 41), DUA Authentication as DAP Initiator, in addition to this part of the ISP.

- 1.1	General

- ISO/IEC ISP 15125-1 covers the use of the Directory Access Protocol as defined in 1993 ITU-T Recommendations X.500 and ISO/IEC 9594:1995 series of standards, by a DUA, which invokes an operation on a DSA and receives a response of a result or an error.

- The objective of this part is to specify constraints on the use of DSA by DUAs to interwork with the Directory services specified in the other parts of the Directory ISPs.

- 1.2	Position within the taxonomy

- This document, part 1 of ISO/IEC ISP 15125, is one part of a multi part ISP identified in ISO/IEC TR 10000-2 as "ADY11, DUA Support of Directory Access."

- 1.3	Scenario

- The DUA interacts with the Directory by communicating with one or more DSA.  A DUA need not be bound to any particular DSA.  It may interact directly with various DSAs to make requests.  For some administrative reasons, it may not always be possible to interact directly with the DSA which needs to carry out the request.  It is also possible that the DUA can access the Directory through a single DSA.  For this purpose, DSAs will need to interact with each other.  Specifications of this part of ISO/IEC ISP 15125 apply to the initiator role of DAP within a DUA as shown in Figure 1.

- Figure 1 - Access to the Directory2

- 2	Normative References

- The following ITU-T Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this International Standardized Profile.  This International Standardized Profile profiles the 1993 edition of the Directory Specifications.

- 2.1	Paired ITU-T Recommendations | International Standards equivalent in technical content

- ITU-T Recommendation X.500 (1993) | ISO/IEC 9594-1:1995, Information technology -- Open Systems Interconnection -- The Directory: Overview of concepts, models and services.

- ITU-T Recommendation X.501 (1993) | ISO/IEC 9594-2:1995, Information technology -- Open Systems Interconnection -- The Directory: Models.

- ITU-T Recommendation X.511 (1993) | ISO/IEC 9594-3:1995, Information technology -- Open Systems Interconnection -- The Directory: Abstract service definition.

- ITU-T Recommendation X.518 (1993) | ISO/IEC 9594-4:1995, Information technology -- Open Systems Interconnection -- The Directory: Procedures for distributed operations.

- ITU-T Recommendation X.519 (1993) | ISO/IEC 9594-5:1995, Information technology -- Open Systems Interconnection -- The Directory: Protocol specifications.

- ITU-T Recommendation X.520 (1993) | ISO/IEC 9594-6:1995, Information technology -- Open Systems Interconnection -- The Directory: Selected attribute types.

- ITU-T Recommendation X.521 (1993) | ISO/IEC 9594-7:1995, Information technology -- Open Systems Interconnection -- The Directory: Selected object classes.

- ITU-T Recommendation X.509 (1993) | ISO/IEC 9594-8:1995, Information technology -- Open Systems Interconnection -- The Directory: Authentication framework.

- ITU-T Recommendation X.525 (1993) | ISO/IEC 9594-9:1995, Information technology -- Open Systems Interconnection -- The Directory: Replication.

- ITU-T Recommendation  X.583 (1997) | ISO/IEC 13248-1: 1997, Information technology -- Open Systems Interconnection -Directory Access Protocol: Protocol Implementation Conformance Statement (PICS) Proforma.

- CCITT Recommendation X.680: 1994 | ISO/IEC 8824-1:1994, Information technology - Open Systems Interconnection (OSI) - Abstract Syntax Notation One (ASN.1): Specification of basic notation.

- CCITT Recommendation X.681: 1994 | ISO/IEC 8824-2:1994, Information technology - Open Systems Interconnection (OSI) - Abstract Syntax Notation One (ASN.1): Information Object Specification.

- CCITT Recommendation X.682: 1994 | ISO/IEC 8824-3:1994, Information technology - Open Systems Interconnection (OSI) - Abstract Syntax Notation One (ASN.1): Constraints Specification.

- CCITT Recommendation X.683: 1994 | ISO/IEC 8824-4:1994, Information technology - Open Systems Interconnection (OSI) - Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 specification.

- CCITT Recommendation X.684: 1994 | ISO/IEC 8825-1:1994, Information technology - Open Systems Interconnection (OSI) - Specification of Abstract Syntax Notation One (ASN.1) Encoding Rules: Basic, Canonical and Distinguished Encoding Rules.

- CCITT Recommendation X.880: 1994 | ISO/IEC 13712-1:1994, Information technology - Open Systems Interconnection (OSI) - Remote Operations: Concepts models and notation.

- CCITT Recommendation X.881: 1994 | ISO/IEC 13712-2:1994, Information technology - Open Systems Interconnection (OSI) - OSI Realizations - Remote Operations Service Element (ROSE) service definition.

- CCITT Recommendation X.882: 1994 | ISO/IEC 13712-3:1994, Information technology - Open Systems Interconnection (OSI) - OSI Realizations - Remote Operations Service Element (ROSE) protocol specification.

- 2.2	Normative Amendments and Technical Corrigenda

- Technical Corrigendum 1 to Recommendation X.501 (1993) | ISO/IEC 9594-2:1995 (covering resolutions to defect reports 088, 089, 090, 091, 102, 104, 125)

- Draft Technical Corrigendum 2 to Recommendation X.501 (1993) | ISO/IEC 9594-2:1995 (covering resolutions to defect reports 134, 136, 140, 143, 144, 145, 147, 149, 171, 172, 174)

- Technical Corrigendum 1 to Recommendation X.509 (1993) | ISO/IEC 9594-8:1995 (covering resolutions to defect report 128)

- Technical Corrigendum 2 to Recommendation X.509 (1993) | ISO/IEC 9594-8:1995 (covering resolutions to defect report 077, 078, 083, 084)

- Draft Technical Corrigendum 3 to Recommendation X.509 (1993) | ISO/IEC 9594-8:1995 (covering resolutions to defect report 80, 92, 100, 177, 183, 194, 196)

- Technical Corrigendum 1 to Recommendation X.511 (1993) | ISO/IEC 9594-3:1995 (covering resolutions to defect report 085)

- Draft Technical Corrigendum 2 to Recommendation X.511 (1993) | ISO/IEC 9594-3:1995 (covering resolutions to defect reports 104, 119, 133, 137, 138, 148, 150, 175)

- Technical Corrigendum 1 to Recommendation X.518 (1993) | ISO/IEC 9594-4:1995 (covering resolutions to defect reports 094, 106, 108, 109, 111, 112, 113, 114, 115)

- Draft Technical Corrigendum 2 to Recommendation X.518 (1993) | ISO/IEC 9594-4:1995 (covering resolutions to defect reports 116, 117, 118, 119, 120, 121, 130, 152, 153, 154, 155, 156, 158, 160, 161, 165, 167)

- Technical Corrigendum 1 to Recommendation X.519 (1993) | ISO/IEC 9594-5:1995 (covering resolutions to defect reports 075, 124)

- Draft Technical Corrigendum 2 to Recommendation X.519 (1993) | ISO/IEC 9594-5:1995 (covering resolutions to defect reports 127, 139)

- Technical Corrigendum 1 to Recommendation X.520 (1993) | ISO/IEC 9594-6:1995 (covering resolutions to defect reports 076, 122, 127)

- Draft Technical Corrigendum 2 to Recommendation X.520 (1993) | ISO/IEC 9594-6:1995 (covering resolutions to defect reports 135, 146)

- Technical Corrigendum 1 to Recommendation X.525 (1993) | ISO/IEC 9594-9:1995 (covering resolutions to defect reports 097, 099, 123)

- Draft Technical Corrigendum 2 to  Recommendation X.525 | ISO/IEC 9594-9:1995 (covering resolutions to defect reports 132, 141, 142)

- 2.3	Additional normative references

- CCITT Recommendation X.200 (1988) Reference Model of Open Systems Interconnection for CCITT Applications.

- ISO 7498:1984, Information Processing Systems - Open Systems Interconnection - Basic Reference Model.

- ISO/IEC 9646-1:1994, Information technology -- Open Systems Interconnection -- Conformance testing methodology and framework - Part 1: General concepts.

- ISO/IEC 9646-7:1995, Information technology -- Open Systems Interconnection -- Conformance testing methodology and framework - Part 7: Implementation conformance statements

- ISO/IEC TR 10000-1:1995, Information Technology - Framework and taxonomy of International Standardized Profiles - Part 1:  Framework.

- ISO/IEC TR 10000-2:1995, Information Technology - Framework and taxonomy of International Standardized Profiles - Part 2:  Taxonomy of Profiles.

3	Definitions

For the purposes of this part of ISO/IEC ISP 15125-1, the following definitions apply.

3.1	Directory definitions

This part of ISO/IEC ISP 15125-1 uses terms defined in ITU-T Recommendations X.500 (1993) | International Standards ISO/IEC 9594:1995.

3.2	Conformance definitions

The following terms are defined in ISO/IEC 9646:

a)	Protocol Implementation Conformance Statement (PICS);

b)	PICS Proforma;

c)	Profile Requirements List (PRL)

d)	conformance;

e)	mandatory requirement;

f)	optional requirement;

g)	conditional requirement.

3.3	Basic directory conformance definitions

The following terms are defined in this part of the ISP:

APDU size (for sending/receiving):  the size of the sent/received transfer encoding, including the ROSE header.

4	Abbreviations

For the purposes of this part of ISO/IEC ISP 15125-1, the following abbreviations apply.

ACSE	Association Control Service Element

APDU	Application Protocol Data Unit

ASN.1	Abstract Syntax Notation One

AVA	Attribute Value Assertion

CCITT	International Telegraph and Telephone Consultative Committee (now ITU)

DAP	Directory Access Protocol

DIB	Directory Information Base

DIT	Directory Information Tree

DN	Distinguished Name

DSA	Directory System Agent

DSP	Directory System Protocol

DUA	Directory User Agent

IEC	International Electrotechnical Commission

IPRL	ISPICS Requirements List

ISO	International Organization for Standardization

ISP	International Standardized Profile

ISPICS	ISP Implementation Conformance Statement

ITU	International Telecommunication Union

ITU-T	ITU Telecommunication Standardization Sector

OSI	Open Systems Interconnection

PDU	Protocol Data Unit

PICS	Protocol Implementation Conformance Statement

PRL	Profile Requirements List

RDN	Relative Distinguished Name

ROSE	Remote Operations Service Element

SPDU	Session Protocol Data Unit

SSDU	Session Service Data Unit

5	Conformance

This ISP states requirements on implementations to achieve interworking.  A claim of conformance to this ISP is a claim that all requirements in the relevant base standards are satisfied, and that all requirements in the following clauses and in Annex A of this ISP are satisfied.  Annex A states the relationship between these requirements and those of the base standards.

To conform to this part of ISO/IEC ISP 15125-1, implementations shall conform to all requirements of ISO/IEC 9594-5 clause 9.1 for a DUA implementing the directoryAccessAC application context, including the requirements directly and indirectly referenced by that clause.  A DUA claiming conformance to this part of ISO/IEC ISP 15125 shall also satisfy the requirements specified in 5.1 through 5.3 following.

For an implementation to conform to this part of ISO/IEC ISP 15125, the behavior of each operation and protocol element, whether supported or not, shall conform to the provisions of this part of ISO/IEC ISP 15125.

5.1	Conformance Statement

For each implementation claiming conformance to this part of ISO/IEC ISP 15125-1, an appropriate set of PICS, ITU-T Recommendation X.583 (1997) | ISO/IEC 13248-1: 1997,  shall be produced stating the support or non-support of each option identified in this part of ISO/IEC ISP 15125-1.

The PICS shall satisfy all requirements defined in ISO/IEC 9545-5, clause 9.1.1, in addition to Annex A  the PRL  and Annex B the Profile Specific ICS of this part of ISO/IEC ISP 15125.

ITU-T Recommendation  X.583 (1997) | ISO/IEC 13248-1: 1997 covers all aspects of the DAP protocol. For the purpose of conformance for this part of the ISP the implementation need only demonstrate support appropriate to this part of the ISP (i.e. DUA column and unsigned operations).

5.2	Static Conformance Requirements

The Directory specification defines Directory services in terms of the interaction between a DUA and a DSA, not in terms of services provided to the Directory user.  The specification does not require that any specific operations be provided to the Directory user.  However, this part of ISO/IEC ISP 15125 requires the support for several services and operations as specified in Annex A of this document .

Note:  	The user of the DUA can be a human user, or an application acting on behalf of a human user, or an application providing a service without human intervention, among other possibilities.  The capabilities of the DUA could be very general (e.g. access to all Directory operations), or very restricted (e.g. bind/unbind with access to one or more operations, possibly with restricted variants).  Another form of DUA could be an API which could be used by a variety of applications.  The profile applies to DUAs which have a significant number of options, accessible, by configuration rather than by on-line intervention.

Each feature (operation or protocol element) listed in Annex A is designated as having a support requirement of mandatory, optional, conditional, excluded, out-of-scope, or not applicable.  Some features have a conditional designation specifying whether the feature is mandatory or optional based on a predicate.  The conditions and predicates are stated in the PRL.  Implementations shall support all features designated mandatory in the PRL except those which are part of an unsupported protocol element.  Where protocol elements are nested, the requirement to support the nested element is relevant only when the immediately containing element is supported.

The set of PICS specified in 5.1 above tells which operations and protocol elements are supported.  For each supported operation and protocol element, the features and syntax shall be implemented as specified in the referenced base standards and in this ISP.  For each received protocol element that is supported, the DUA shall be capable of producing an effect which is made available to the Directory user or which results in the DUA taking further action on the behalf of the user.  For each generated protocol element that is supported, the DUA shall be capable of generating each value of the protocol element permitted by the abstract syntax at the request of the user or on behalf of the user, except as noted in the PICS.  This ISP is intended to be consistent with the PICS; however where inconsistencies arise, the ISP shall take precedence.

5.3	Dynamic Requirements

To conform to this part of ISO/IEC ISP 15125, implementations shall conform to all dynamic requirements of ISO/IEC 9594-5, ITU-T Recommendation X.519, clauses 9.1.3 and 7.5.1, for a DUA implementing directoryAccessAC application context.  Implementations shall conform to all procedures specified in the referenced base standards as amended by the corrigenda and defect reports.  Implementations shall support all procedures and capabilities in the base standards as they relate to operations and protocol elements for which support is claimed in the PICS.  Protocol elements for which support is not claimed on the PICS shall not be generated.

5.3.1	Accept all Conformant Responses

A DUA shall accept any protocol conformant response to each operation it supports, including responses containing protocol elements which are unknown or not supported, in accordance with Clause 7.5.1 of [ITU-T X.519 (1993) | ISO/IEC 9594-5].  For this purpose, to accept a response means to receive the response without taking the actions associated by the standards with protocol errors, including Presentation, ROSE, and DAP protocol errors.

5.3.2	Generating Supported Protocol Elements.

A protocol element is not required to be generated in any particular instance of communication, except as specified in the base standard or in this ISP, or as otherwise necessary to accomplish the purpose of  the communication.

5.3.3	Restrictions on APDU Size

A DUA may send an APDU of any size, but since the DSA is not required to process APDUs over I Mb in length, a DAP error may result for APDUs greater than that size.

A DUA shall be able to accept, but not necessarily process, a received APDU of any size, unless the PDU is erroneous.  If it receives a PDU larger than the locally defined PDU size limit, it shall be able to abort the association

Note:	It is recommended that a DUA be configurable to limit the size of response APDUs to the size that the DUA can process.

5.3.4	Non-support of Abandon

A DUA which does not support Abandon shall be capable of generating a non-zero timeLimit for all operation requests.

Annex A

(Normative)

Profile Requirements List

In the event of a discrepancy becoming apparent in the body of this part of ISO/IEC ISP 15125 and the tables in this annex, this annex is to take precedence.

A.1	Introduction

This annex specifies the constraints and characteristics on what shall or may appear in an implementor’s PICS for an implementation conformant to this part of ISO/IEC ISP 15125. This annex identifies restrictions placed by this part of the ISP on the corresponding base specification requirements.

A.2	Identification of the Implementation

A.2.1	Identification of the PICS

This part of the ISP is based on ITU-T Recommendation  X.583 (1997) | ISO/IEC 13248-1: 1997, Information technology -- Open Systems Interconnection -Directory Access Protocol: Protocol Implementation Conformance Statement (PICS) Proforma. PRL tables, while not following the same numbering as the PICS, carry the same title as the PICS. Implementations compliant with this part of the ISP should show support, based on this part of the ISP, in the PICS. The PICS covers all aspects of the DAP protocol, which is a broader scope than covered in this part of the ISP. Implementations showing support for this part of the ISP should address the following ICS tables:

- All general ICS tables (i.e. those tables which are not associated with a DUA or DSA). These are tables which do not delineate support for DUAs and DSAs. (PICS clause A.3, A.4, A.5, and subclause A.6.1)

- ICS tables which are included in a clause associated with a DUA. (PICS subclause A.6.2.2)

- The DUA support column for ICS tables which delineate between DUAs and DSAs support. Where the DUA support column is further delineated between digitally signed and unsigned columns, the unsigned column should be used (PICS subclause A.6.3).

A.3	Instructions

A.3.1	Purpose and structure of the PRL

The purpose of this PRL is to provide suppliers of  implementations of the ITU-T Recommendations X.500 (1993) | International Standards ISO/IEC 9594:1995 with a consistent  expression of restrictions to the corresponding PICS proforma based on this part of the ISP.

The PRL is in the form of a set of items. An item is provided for each capability for which an implementation choice is allowed. Items are also provided for mandatory capabilities for which no implementation choice is allowed. Each item includes an item number, item description, a protocol status value (which reflects the base specification requirements as expressed in the PICS), a profile status value (which reflects restrictions to the PICS imposed by this part of the ISP), a predicate column (which predicates set or employed by the profile), and profile note column.

This clause provides general information and instructions for completion of the proforma.

Subclause A.4 contains the PRL tables.

Subclause B.1 is for the identification of the technical corrigenda to this part of the ISP.

A.3.2	Symbols, terms, and abbreviations

A.3.2.1	Introduction

Notations have been introduced in order to reduce the size of the tables in the PRL. These have allowed the use of multi-column layout where the columns are headed ‘Protocol Status’ and ‘Profile Status’. Definitions of each are given below. Additionally, the following definitions apply:

protocol status (value):	An allowed entry in the status column for an item in a PICS proforma table;

profile status (value):	An allowed entry in the status column for an item in a PRL  table which reflects restrictions imposed by this part of the ISP;

A.3.2.2	Prerequisite notation

If a predicate applies to a whole table, a prerequisite line may be specified in front of the table to which it applies. A prerequisite line takes the form:

Prerequisite: <Predicate>

The meaning of such a line is that if <predicate> is True, then the table applies, else it is not applicable.

A.3.2.3	Item reference numbers

Each line within the PRL is numbered at the left-hand edge of the line. This numbering is included as a means of uniquely  identifying all possible implementation details within the PRL.  This referencing should be consistent with the corresponding PICS item reference.

The means of referencing individual responses is done by the following sequence:

-	a reference to the smallest enclosing the relevant item;

-	a solidus character, '/';

-	the reference number of the row in which the response appears;

-	if, and only if, more than one response occurs in the row identified by the reference number, then each possible entry is implicitly labeled a, b, c, etc. from left to right, and this letter is appended to the sequence.

An example of the use of this notation would be A.6.3.3.1.1/1, which refers to the support for credentials in a DirectoryBind protocol data unit.

A.3.2.4	 Protocol Status column

This column indicates the level of support required for conformance to this ITU-T | ISO/IEC standard, as expressed in the PICS.

The values are as follows:

m	the capability is required to be implemented, in conformance with the related specification;

o	the capability may be implemented, and if it is implemented it is required to conform to the related specification

c	the requirement on the capability depends on the selection of other optional or conditional items;

i	the capability is outside the scope of this part of the ISP, and hence irrelevant and not subject to conformance testing;

-	in the given context it is impossible to use this capability.

Nested conditionals are denoted by nested numbering (e.g. 1, 1.1, 1.1.1, etc.) of the item descriptions in the tables. A table may have zero, one or more levels of nesting.  The status of a leading  item is specified by its status entry, as defined above.  The status of a subordinate (that is nested) item is specified as follows: If the superior item is supported, the status of the subordinate item is determined by its status column entry and applicable predicate, if any.  If the superior item is not supported, the subordinate item is not applicable, independent of its status column entry.

Note:  It is possible to label conditional and optional status items (usually by use of a number). For the purpose of the PRL, in order to avoid confusion between the Protocol Status and Profile Status numbering, conditional and optional numbering as expressed in the PICS, and associated Protocol Status column, is dropped. The notation is instead replaced with ‘cn’ and ‘o.n’ to reflect that the PICS conditional and optional items have associated numbers.

A.3.2.5	 Profile Status column

This column indicates the level of support required for conformance to this part of the ISP.

The values are as follows:

m	the capability is required to be implemented, in conformance with the related specification;

o	the capability may be implemented, and if it is implemented it is required to conform to the related specification

c	the requirement on the capability depends on the selection of other optional or conditional items;

i	the capability is outside the scope of the this part of the ISP, and hence irrelevant and not subject to conformance testing;

-	in the given context it is impossible to use this capability.

Nested conditionals are denoted by nested numbering (e.g. 1, 1.1, 1.1.1, etc.) of the item descriptions in the tables. A table may have zero, one or more levels of nesting.  The status of a leading  item is specified by its status entry, as defined above.  The status of a subordinate (that is nested) item is specified as follows: If the superior item is supported, the status of the subordinate item is determined by its status column entry and applicable predicate, if any.  If the superior item is not supported, the subordinate item is not applicable, independent of its status column entry.

A.3.2.6	Predicate column

The item number contained in the predicate column, if any, means that the status in the "Profile Status" column applies only when the PRL states that one or more features identified by the item is supported.

A3.2.7	Predicate Name

The predicate name indicates that name upon which the predicate is based.  A predicate name flagged with an asterisk preceding the predicate name indicates the condition by which the predicate is being set.  A predicate name not flagged with an asterisk indicates the predicate on which the conditional support is based.

A.3.2.8	Note column

This column  indicates the following:

notexx	-	refers to Note xx;

See xx	-	refers to Table xx.

See Part xx	-	refers to another part of the ISO/IEC ISP 15125 associated with the element.

Information entered into this column applies to the PRL and the Profile Status column. These notes may also have meaning to the Protocol Status column, however, notes specific to the Protocol Status column are found in the PICS.

A.4	PRL Tables

These tables reflect restrictions to the base specification imposed by this part of the ISP. Not all of the PICS tables are listed in this PRL, but instead just those deemed necessary to reflect the PRL.

Cross references to the corresponding PICS tables are supplied in the table title. For instance  “Ref. A.6.3.3.1.1” would refer to table A.6.3.3.1.1 of Appendix A in the PICS.

Implementers should submit completed PICS based on guidance expressed in A.2.1

A.4.1	Roles		Ref. A.6.1

| Item No. | Role | Protocol Status | Profile Status | Predicate | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Centralized DSA | o | i |  |  |
| 2 | Cooperating DSA | o | i |  |  |
| 3 | First-Level DSA | o | i |  |  |
| 4 | DUA for connection to centralized DSA | o | m |  |  |
| 5 | DUA for connection to cooperating DSA | o | i |  | See Part 2 |

A.4.2	General Capabilities and Global statement of conformance

Answering "No" to A.6.2.2.1/1 in the PICS to element A.4.2.1 of this PRL indicates non-conformance to the protocol specification.   Non-supported mandatory capabilities are to be identified in the PICS, with an explanation of why the implementation is non-conformant.   Such information shall be provided in subclause  A.6.6 "Other Information" of the PICS.

A.4.2.1	General Capabilities		Ref. A.6.2.2.1

| Item No. | Question |  | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Are all mandatory general capabilities for the DUA implemented? |  | m | m |  |  |
| 2 | Is the directoryAccessAC application-context supported? |  | m | m |  |  |
| 3 | Is asynchronous (ROSE class 2) mode of operation supported? |  | o | c1 | *Async-DUA |  |
| 4 | Does the DUA follow the rules of extensibility as defined in section 7.5 of ITU-T X.519 ¦ ISO/IEC 9594-5? |  | m | m |  |  |
| 5 | Does the DUA support signed DAP operations and results? |  | o | i |  | See Part 5 |

c1	If Abandon then m else o.

A.4.2.2	Supported Security Levels		Ref. A.6.2.2.2

| Item No. | Question |  | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | none | o.n | o.1 |  |  |
| 2 | simple | o.n | o.1 |  |  |
| 2.1 | unprotected | - | c:c2 | *Simple-DUA |  |
| 2.2 | protected | - | i |  | See Part 5 |
| 3 | strong | o.n | i |  | See Part 5 |
| 4 | external | i | i |  |  |

c2:	If simple protected authentication is supported in ISO/IEC ISP 15125-5 (A.4.2.2/2.2)  then o else m.

o.1	The DUA must support at least one security level, unless the external mechanism is supported or this part of the ISP is combined with ADY 41 and strong authentication is supported.

A.4.2.3	Supported Access Control Schemes		Ref. A.6.2.2.3

| Item No. | Access Control Scheme |  | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Simplified Access Control | o | i |  | See Part 5 |
| 2 | Basic Access Control | o | i |  | See Part 5 |
| 3 | Other | i | i |  |  |

A.4.3	Capabilities and options.

This part of the ISP identifies the supported application context, the PDUs and operations.

The operation arguments and PDU parameters are identified.

A.4.3.1	Supported application context.

The only application context supported is Directory Access application context.

A.4.3.2	Operations and extensions

A.4.3.2.1	Operations	 	Ref. A.6.3.2.1

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | DirectoryBind | m | m |  |  |
| 2 | DirectoryUnbind | m | m |  |  |
| 3 | Read | o | o | *Read |  |
| 4 | Compare | o | o | *Compare |  |
| 5 | Abandon | cn | c3 | *Abandon | Note 1 |
| 6 | List | o | o | *List |  |
| 7 | Search | o | o | *Search |  |
| 8 | AddEntry | o | o | *AddEntry |  |
| 9 | RemoveEntry | o | o | *RemoveEntry |  |
| 10 | ModifyEntry | o | o | *ModifyEntry |  |
| 11 | ModifyDN | o | o | *ModifyDN |  |

c3: 	If [[Async-DUA]  then  support of  this feature is o

Note 1: 	The Abandon operation can only be supported if the asynchronous mode (ROSE class 2) of operation is supported in A.4.2.1/3.

A.4.3.2.2	Extensions		Ref. A.6.3.2.2

| Item No. | Extension | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | subentries | o | o |  |  |
| 2 | copyShallDo | o | o |  |  |
| 3 | attributesizelimit | o | o | *attrsizelimit |  |
| 4 | extraAttributes | o | o |  |  |
| 5 | modifyRightsRequest | o | o | *modrightsreq |  |
| 6 | pagedResultsRequest | o | o | *pageresreq |  |
| 7 | matchedValuesOnly | o | o | *matchvalonly |  |
| 8 | extendedFilter | o | o | *extfilter |  |
| 9 | targetSystem | o | o | *targetsystem |  |
| 10 | useAliasOnUpdate | o | o |  |  |
| 11 | newSuperior | o | o | *newsuperior |  |

A.4.3.3	Protocol elements

Some protocol elements may be digitally signed. In those cases where an operation can be signed, suppliers of implementations claiming conformance to signed protocol elements, should see ADY 41.

A.4.3.3.1	DirectoryBind Elements

A.4.3.3.1.1	Directory Bind Arguments		Ref. A.6.3.3.1.1

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | credentials | cn | c4 |  |  |
| 1.1 | simple | c:cn | c:c4 |  |  |
| 1.1.1 | name | c:m | c:m |  |  |
| 1.1.2 | validity | c:o | c:o |  |  |
| 1.1.2.1 | time1 | c:o | c:m |  |  |
| 1.1.2.2 | time2 | c:o | c:o |  |  |
| 1.1.2.3 | random1 | c:o | c:m |  |  |
| 1.1.2.4 | random2 | c:o | c:o |  |  |
| 1.1.3 | password | c:o | c:o |  |  |
| 1.1.3.1 | unprotected | c:o.n | c:m |  |  |
| 1.1.3.2 | protected | c:o.n | i |  | See Part 5 |
| 1.1.3.2.1 | algorithmIdentifier | c:m | - |  |  |
| 1.1.3.2.2 | encrypted | c:m | - |  |  |
| 1.2 | strong | c:cn | i |  | See Part 5 |
| 1.2.1 | certification-path | c:o | - |  |  |
| 1.2.2 | bind-token | c:m | - |  |  |
| 1.2.2.1 | toBeSigned | c:m | - |  |  |
| 1.2.2.1.1 | algorithm | c:m | - |  |  |
| 1.2.2.1.2 | name | c:m | - |  |  |
| 1.2.2.1.3 | time | c:m | - |  |  |
| 1.2.2.1.4 | random | c:m | - |  |  |
| 1.2.2.2 | algorithmIdentifier | c:m | - |  |  |
| 1.2.2.3 | encrypted | c:m | - |  |  |
| 1.2.3 | name | c:o | - |  |  |
| 1.3 | externalProcedure | i | i |  |  |
| 2 | versions | m | m |  |  |
| 2.1 | v1 | m | m |  |  |

c4:	If  [Simple-DUA] then support of this feature is m else o.

A.4.3.3.1.2	Directory Bind Result	 	Ref. A.6.3.3.1.2

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | credentials | cn | c4 |  |  |
| 1.1 | simple | c:cn | c:c4 |  |  |
| 1.1.1 | name | c:m | c:m |  | Note 2 |
| 1.1.2 | validity | c:o | c:o |  |  |
| 1.1.2.1 | time1 | c:o | c:m |  |  |
| 1.1.2.2 | time2 | c:o | c:o |  |  |
| 1.1.2.3 | random1 | c;o | c:m |  |  |
| 1.1.2.4 | random2 | c:o | c:o |  |  |
| 1.1.3 | password | c:o | c:o |  |  |
| 1.1.3.1 | unprotected | c:o.n | c:m |  | Note 3 |
| 1.1.3.2 | protected | c;o.n | i |  | See Part 5 |
| 1.1.3.2.1 | algorithmIdentifier | c:m | - |  |  |
| 1.1.3.2.2 | encrypted | c:m | - |  |  |
| 1.2 | strong | c:cn | i |  | See Part 5 |
| 1.2.1 | certification-path | c:o | - |  |  |
| 1.2.2 | bind-token | c:m | - |  |  |
| 1.2.2.1 | toBeSigned | c:m | - |  |  |
| 1.2.2.1.1 | algorithm | c:m | - |  |  |
| 1.2.2.1.2 | name | c:m | - |  |  |
| 1.2.2.1.3 | time | c:m | - |  |  |
| 1.2.2.1.4 | random | c:m | - |  |  |
| 1.2.2.2 | algorithmIdentifier | c:m | - |  |  |
| 1.2.2.3 | encrypted | c:m | - |  |  |
| 1.2.3 | name | c:o | - |  |  |
| 1.3 | externalProcedure | i | i |  |  |
| 2 | versions | m | m |  |  |
| 2.1 | v1 | m | m |  |  |

c4:	If  [Simple-DUA] then support of this feature is m else o.

Note 2:	DUAs should be capable of returning the name element to the user.

Note 3:	DUAs shall not display the clear text DSA password

A.4.3.3.1.3	Directory Bind Error	 	Ref. A.6.3.3.1.3

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | versions | m | m |  |  |
| 1.1 | v1 | m | m |  |  |
| 2 | error | m | m |  |  |
| 2.1 | ServiceError | m | m |  |  |
| 2.2 | SecurityError | m | m |  |  |

A.4.3.3.2	Directory Unbind Elements

DirectoryUnbind has no arguments (see Section 8.2 of ITU-T X.511 | ISO/IEC 9594-3).

A.4.3.3.3	Read Elements		Ref. A.6.3.3.3

Prerequisite: [Read]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | ReadArgument | m | m |  |  |
| 1.1 | toBeSigned | - | - |  |  |
| 1.2 | object | m | m |  |  |
| 1.3 | selection | m | m |  |  |
| 1.4 | modifyRightsRequest | cn | c5 |  |  |
| 1.5 | CommonArguments | m | m | *Comm-Arg | See A.4.3.13 |
| 1.6 | algorithmIdentifier | - | - |  |  |
| 1.7 | encrypted | - | - |  |  |
| 2 | ReadResult | m | m |  |  |
| 2.1 | toBeSigned | - | - |  |  |
| 2.2 | entry | m | m |  |  |
| 2.3 | modifyRights | o | c5 |  |  |
| 2.3.1 | item | c:m | c:m |  |  |
| 2.3.1.1 | entry | c:o | c:m |  |  |
| 2.3.1.2 | attribute | c:o | c:m |  |  |
| 2.3.1.3 | value | c:o | c:m |  |  |
| 2.3.2 | permission | c:o | c:m |  |  |
| 2.4 | CommonResults | m | m | *Comm-Res | See A.4.3.14 |
| 2.5 | algorithmIdentifier | - | - |  |  |
| 2.6 | encrypted | - | - |  |  |
| 3 | Errors | m | m |  |  |

c5:	If [modrightsreq] then support of this feature is m else o.

A.4.3.3.4	Compare Elements		Ref. A.6.3.3.4

Prerequisite: [Compare]

| IItem No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | CompareArgument | m | m |  |  |
| 1.1 | toBeSigned | - | - |  |  |
| 1.2 | object | m | m |  |  |
| 1.3 | purported | m | m |  |  |
| 1.4 | CommonArguments | m | m | *Comm-Arg | See A.4.3.13 |
| 1.5 | algorithmIdentifier | - | - |  |  |
| 1.6 | encrypted | - | - |  |  |
| 2 | CompareResult | m | m |  |  |
| 2.1 | toBeSigned | - | - |  |  |
| 2.2 | name | o | m |  |  |
| 2.3 | matched | m | m |  |  |
| 2.4 | fromEntry | o | m |  |  |
| 2.5 | matchedSubtype | o | m |  |  |
| 2.6 | CommonResults | m | m | *Comm-Res | See A.4.3.14 |
| 2.7 | algorithmIdentifier | - | - |  |  |
| 2.8 | encrypted | - | - |  |  |
| 3 | Errors | m | m |  |  |

A.4.3.3.5	Abandon Elements	 	Ref. A.6.3.3.5

Prerequisite: [Abandon]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | AbandonArgument | m | m |  |  |
| 1.1 | nvokeID | m | m |  |  |
| 2 | AbandonResult | m | m |  |  |
| 3 | Errors | m | m |  |  |

A.4.3.3.6	List Elements	 	Ref. A.6.3.3.6

Prerequisite:  [List]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | ListArgument | m | m |  |  |
| 1.1 | toBeSigned | - | - |  |  |
| 1.2 | object | m | m |  |  |
| 1.3 | pagedResults | cn | c6 |  | See A.4.3.20 |
| 1.4 | CommonArguments | m | m | *Comm-Arg | See A.4.3.13 |
| 1.5 | algorithmIdentifier | - | - |  |  |
| 1.6 | encrypted | - | - |  |  |
| 2 | ListResult | m | m |  |  |
| 2.1 | toBeSigned | - | - |  |  |
| 2.2 | listInfo | m | m |  |  |
| 2.2.1 | name | o | m |  |  |
| 2.2.2 | subordinates | m | m |  |  |
| 2.2.2.1 | rdn | m | m |  |  |
| 2.2.2.2 | aliasEntry | o | m |  |  |
| 2.2.2.3 | fromEntry | o | m |  |  |
| 2.2.3 | partialOutcomeQualifier | o | o |  |  |
| 2.2.3.1 | limitProblem | c:o | c:o |  |  |
| 2.2.3.1.1 | timeLimitExceeded | c:m | c:m |  |  |
| 2.2.3.1.2 | sizeLimitExceeded | c:m | c:m |  |  |
| 2.2.3.1.3 | administrativeLimitExceeded | c:m | c:m |  |  |
| 2.2.3.2 | unexplored | c:o | c:m |  |  |
| 2.2.3.3 | unavailableCriticalExtensions | c:m | c:m |  |  |
| 2.2.3.4 | unknownErrors | c:o | c:o |  |  |
| 2.2.3.5 | queryReference | c:cn | c:c6 |  |  |
| 2.2.4 | CommonResults | m | m | *Comm-Res | See A.4.3.14 |
| 2.3 | uncorrelatedListInfo | o | m |  |  |
| 2.4 | algorithmIdentifier | - | - |  |  |
| 2.5 | encrypted | - | - |  |  |
| 3 | Errors | m | m |  |  |

c6:	If [pageresreq] then support of this feature is m else o.

A.4.3.3.7	Search Elements		Ref. Ref. A.6.3.3.7

Prerequisite: [Search]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | SearchArgument | m | m |  |  |
| 1.1 | toBeSigned | - | - |  |  |
| 1.2 | baseObject | m | m |  |  |
| 1.3 | subset | o | m |  |  |
| 1.4 | filter | o | o | *Filter | See A.4.3.18 |
| 1.5 | searchAliases | o | m |  |  |
| 1.6 | selection | o | o | *Info-Sel | See A.4.3.16 |
| 1.7 | pagedResults | cn | c6 |  |  |
| 1.8 | matchedValuesOnly | o | o |  |  |
| 1.9 | extendedFilter | cn | c7 |  |  |
| 1.10 | CommonArguments | m | m | *Comm-Arg | See A.4.3.13 |
| 1.11 | algorithmIdentifier | - | - |  |  |
| 1.12 | encrypted | - | - |  |  |
| 2 | SearchResult | m | m |  |  |
| 2.1 | toBeSigned | - | - |  |  |
| 2.2 | searchInfo | m | m |  |  |
| 2.2.1 | name | o | m |  |  |
| 2.2.2 | entries | m | m | *Entry-Info | See A.4.3.17 |
| 2.2.3 | partialOutcomeQualifier | o | o |  |  |
| 2.2.3.1 | limitProblem | c:o | c:o |  |  |
| 2.2.3.1.1 | timeLimitExceeded | c:m | c:m |  |  |
| 2.2.3.1.2 | sizeLimitExceeded | c:m | c:m |  |  |
| 2.2.3.1.3 | administrativeLimitExceeded | c:m | c:m |  |  |
| 2.2.3.2 | unexplored | c:o | c:m |  |  |
| 2.2.3.3 | unavailableCriticalExtensions | c:o | c:m |  |  |
| 2.2.3.4 | unknownErrors | c:o | c:m |  |  |
| 2.2.3.5 | queryReference | c:cn | c:c6 |  |  |
| 2.2.4 | CommonResults | m | m | *Comm-Res | See A.4.3.14 |
| 2.3 | uncorrelatedSearchInfo | o | m |  |  |
| 2.4 | algorithmIdentifier | - | - |  |  |
| 2.5 | encrypted | - | - |  |  |
| 3 | Errors | m | m |  | See A.4.3.12 |

c6:	If [pageresreq] then support of this feature is m else o.

c7:	If [extfilter] then support of this feature is m else o.

A.4.3.3.8	Add Entry Elements	 	Ref. A.6.3.3.8

Prerequisite: [AddEntry]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | AddEntryArgument | m | m |  |  |
| 1.1 | toBeSigned | - | - |  |  |
| 1.2 | object | m | m |  |  |
| 1.3 | entry | m | m |  |  |
| 1.4 | targetSystem | cn | c8 |  | See A.4.3.26 |
| 1.5 | CommonArguments | m | m | *Comm-Arg | See A.4.3.13 |
| 1.6 | algorithmIdentifier | - | - |  |  |
| 1.7 | encrypted | - | - |  |  |
| 2 | AddEntryResult | m | m |  |  |
| 3 | Errors | m | m |  | See A.4.3.12 |

c8:	If[ targetsystem] then support of this feature is m else o.

A.4.3.3.9	Remove Entry Elements	 	Ref. A.6.3.3.9

Prerequisite: [RemoveEntry]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | RemoveEntryArgument | m | m |  |  |
| 1.1 | toBeSigned | - | - |  |  |
| 1.2 | object | m | m |  |  |
| 1.3 | CommonArguments | m | m | *Comm-Arg | See A.4.3.13 |
| 1.4 | algorithmIdentifier | - | - |  |  |
| 1.5 | encrypted | - | - |  |  |
| 2 | RemoveEntryResult | m | m |  |  |
| 3 | Errors | m | m |  | See A.4.3.12 |

A.4.3.3.10	Modify Entry Elements	 	Ref. A.6.3.3.10

Prerequisite: [ModifyEntry]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | ModifyEntryArgument | m | m |  |  |
| 1.1 | toBeSigned | - | - |  |  |
| 1.2 | object | m | m |  |  |
| 1.3 | changes | m | m |  |  |
| 1.3.1 | addAttribute | m | m |  |  |
| 1.3.2 | removeAttribute | m | m |  |  |
| 1.3.3 | addValues | m | m |  |  |
| 1.3.4 | removeValues | m | m |  |  |
| 1.4 | CommonArguments | m | m | *Comm-Arg | See A.4.3.13 |
| 1.5 | algorithmIdentifier | - | - |  |  |
| 1.6 | encrypted | - | - |  |  |
| 2 | ModifyEntryResult | m | m |  |  |
| 3 | Errors | m | m |  | See A.4.3.12 |

A.4.3.3.11	ModifyDN Elements	 	Ref. A.6.3.3.11

Prerequisite: [ModifyDN]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | ModifyDNArgument | m | m |  |  |
| 1.1 | toBeSigned | - | - |  |  |
| 1.2 | object | m | m |  |  |
| 1.3 | newRDN | m | m |  |  |
| 1.4 | deleteOldRDN | o | m |  |  |
| 1.5 | newSuperior | cn | c9 |  |  |
| 1.6 | CommonArguments | m | m | *Comm-Arg | See A.4.3.13 |
| 1.7 | algorithmIdentifier | - | - |  |  |
| 1.8 | encrypted | - | - |  |  |
| 2 | ModifyDNResult | m | m |  |  |
| 3 | Errors | m | m |  | See A.4.3.12 |

c9:	If [newsuperior] then support of this feature is m else o.

A.4.3.3.12	Errors and Parameters	 	Ref. A.6.3.3.12

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Abandoned | cn | c10 |  |  |
| 2 | AbandonFailed | cn | c10 |  |  |
| 2.1 | problem | c:m | c:m |  |  |
| 2.1.1 | noSuchOperation | c:m | c:m |  |  |
| 2.1.2 | tooLate | c:m | c:m |  |  |
| 2.1.3 | cannotAbandon | c:m | c:m |  |  |
| 2.2 | operation | c:m | c:m |  |  |
| 3 | AttributeError | cn | c11 |  |  |
| 3.1 | object | c:m | c:m |  |  |
| 3.2 | problems | c:m | c:m |  |  |
| 3.2.1 | problem | c:m | c:m |  |  |
| 3.2.1.1 | noSuchAttributeOrValue | c:m | c:m |  |  |
| 3.2.1.2 | invalidAttributeSyntax | c:m | c:m |  |  |
| 3.2.1.3 | undefinedAttributeType | c:m | c:m |  |  |
| 3.2.1.4 | inappropriateMatching | c:m | c:m |  |  |
| 3.2.1.5 | constraintViolation | c:m | c:m |  |  |
| 3.2.1.6 | attributeOrValueAlreadyExists | c:m | c:m |  |  |
| 3.3 | type | c:m | c:m |  |  |
| 3.4 | value | c:o | c:m |  |  |
| 4 | NameError | cn | c12 |  |  |
| 4.1 | problem | c:m | c:m |  |  |
| 4.1.1 | noSuchObject | c:m | c:m |  |  |
| 4.1.2 | aliasProblem | c:m | c:m |  |  |
| 4.1.3 | invalidAttributeSyntax | c:m | c:m |  |  |
| 4.1.4 | aliasDereferencingProblem | c:m | c:m |  |  |
| 4.2 | matched | c:m | c:m |  |  |
| 5 | Referral | cn | i |  | See Part 2 |
| 5.1 | candidate | c:m | - |  |  |
| 6 | SecurityError | cn | c12 |  |  |
| 6.1 | problem | c:m | c:m |  |  |
| 6.1.1 | InappropriateAuthentication | c:m | c:m |  |  |
| 6.1.2 | invalidCredentials | c:m | c:m |  |  |
| 6.1.3 | insufficientAccessRights | c:m | c:m |  |  |
| 6.1.4 | invalidSignature | c:cn | i |  | See Part 5 |
| 6.1.5 | protectionRequired | c: cn | i |  | See Part 5 |
| 6.1.6 | noInformation | c:m | c:m |  |  |
| 7 | ServiceError | cn | c12 |  |  |
| 7.1 | problem | c:m | c:m |  |  |
| 7.1.1 | busy | c:m | c:m |  |  |
| 7.1.2 | unavailable | c:m | c:m |  |  |
| 7.1.3 | unwillingToPerform | c:m | c:m |  |  |
| 7.1.4 | chainingRequired | c:m | c:m |  |  |
| 7.1.5 | unableToProceed | c:m | c:m |  |  |
| 7.1.6 | invalidReference | c:m | c:m |  |  |
| 7.1.7 | timeLimitExceeded | c:m | c:m |  |  |
| 7.1.8 | administrativeLimitExceeded | c:m | c:m |  |  |
| 7.1.9 | loopDetected | c:m | c:m |  |  |
| 7.1.10 | unavailableCriticalExtension | c:m | c:m |  |  |
| 7.1.11 | outOfScope | c:m | c:m |  |  |
| 7.1.12 | ditError | c:m | c:m |  |  |
| 7.1.13 | invalidQueryReference | c:cn | c:c13 |  |  |
| 8 | UpdateError | cn | c14 |  |  |
| 8.1 | problem | c:m | c:m |  |  |
| 8.1.1 | namingViolation | c:m | c:m |  |  |
| 8.1.2 | objectClassViolation | c:m | c:m |  |  |
| 8.1.3 | notAllowedOnNonLeaf | c:m | c:m |  |  |
| 8.1.4 | notAllowed OnRDN | c:m | c:m |  |  |
| 8.1.5 | entryAlreadyExists | c:m | c:m |  |  |
| 8.1.6 | affectsMultipleDSAs | c:m | c:m |  |  |
| 8.1.7 | objectClassModificationProhibited | c:m | c:m |  |  |

c10:	If [Abandon] then support of this feature is m else o.

c11:	If [Read or Compare or Search or AddEntry or ModifyEntry] then support of  this feature is m else o.

c12:	If [Read or Compare or List or Search or AddEntry or RemoveEntry or ModifyEntry or ModifyDN] then support of  this feature is m else o.

c13:	If [pageresreq] then support of this feature is m else -.

c14:	If [AddEntry or RemoveEntry or ModifyEntry or ModifyDN] then support of  this feature is m else o.

A.4.3.3.13	Common Arguments Elements	 	Ref. A.6.3.3.13

Prerequisite: [Comm-Arg]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | serviceControls | o | o | *Serv-Ctrls | SeeA.4.3.3.15 |
| 2 | securityParameters | cn | i |  | See Part 5 |
| 3 | requestor | o | o |  |  |
| 4 | operationProgress | o | i |  | See Part 2 |
| 4.1 | nameResolutionPhase | c:o | - |  |  |
| 4.1.1 | notStarted | c:m | - |  |  |
| 4.1.2 | proceeding | c:m | - |  |  |
| 4.1.3 | completed | c:m | - |  |  |
| 4.2 | nextRDNToBeResolved | c:o | - |  |  |
| 5 | aliasedRDNs | o | m |  |  |
| 6 | criticalExtensions | o | o |  |  |
| 7 | referenceType | o | i |  | See Part 2 |
| 8 | entryOnly | o | i |  | See Part 2 |
| 9 | exclusions | o | i |  | See Part 2 |
| 10 | nameResolveOnMaster | o | i |  | See Part 2 |

A.4.3.3.14	Common Results Elements	 	Ref. A.6.3.3.14

Prerequisite: [Comm-Res]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | securityParameters | cn | i |  | See Part 5 |
| 2 | performer | o | o |  |  |
| 3 | aliasDereferenced | o | m |  |  |

A.4.3.3.15	Service Controls		Ref. A.6.3.3.15

Prerequisite: [Serv-Ctrls]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | options | o | o |  |  |
| 1.1 | preferChaining | c:m | c:m |  |  |
| 1.2 | chainingProhibited | c:m | c:m |  |  |
| 1.3 | localScope | c:m | c:m |  |  |
| 1.4 | dontUseCopy | c:m | c:m |  |  |
| 1.5 | dontDereferenceAliases | c:m | c:m |  |  |
| 1.6 | subentries | c:m | c:m |  |  |
| 1.7 | copyShallDo | c:m | c:m |  |  |
| 2 | priority | o | o |  |  |
| 3 | timeLimit | o | o |  |  |
| 4 | sizeLimit | o | o |  |  |
| 5 | scopeOfReferral | o | o |  |  |
| 6 | attributeSizeLimit | cn | c15 |  |  |

c15:	If [attrsizelimit] then support of this feature is m else o.

A.4.3.3.16	Entry Information Selection	 	Ref. A.6.3.3.16

Prerequisite:[Info-Sel]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | attributes | o | m |  |  |
| 1.1 | allUserAttributes | c:m | m |  |  |
| 1.2 | select | c:m | m |  |  |
| 2 | infoTypes | o | o |  |  |
| 2.1 | attributeTypesOnly | c:m | c:m |  |  |
| 2.2 | attributeTypesAndValues | c:m | c:m |  |  |
| 3 | extraAttributes | o | o |  |  |
| 3.1 | allOperationalAttributes | c:m | c:m |  |  |
| 3.2 | select | c:m | c:m |  |  |

A.4.3.3.17	Entry Information	 	Ref. A.6.3.3.17

Prerequisite: [Entry-Info]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | name | m | m |  |  |
| 2 | fromEntry | o | m |  |  |
| 3 | information | o | m |  |  |
| 3.1 | attributeType | c:m | m |  |  |
| 3.2 | attribute | c:m | m |  |  |
| 4 | incompleteEntry | o | m |  |  |

A.4.3.3.18	Filter Elements	 	Ref. A.6.3.3.18

Prerequisite: [Filter or extfilter]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | item | o | m |  |  |
| 2 | and | o | o |  |  |
| 3 | or | o | o |  |  |
| 4 | not | o | o |  |  |

A.4.3.3.19	Filter Item Elements		Ref. A.6.3.3.319

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | equality | o | m |  |  |
| 2 | substrings | o | o |  |  |
| 2.1 | type | c:o | c:o |  |  |
| 2.2 | strings | c:o | c:o |  |  |
| 2.2.1 | initial | c:o | c:o |  |  |
| 2.2.2 | any | c:o | c:o |  |  |
| 2.2.3 | final | c:o | c:o |  |  |
| 3 | greaterOrEqual | o | o |  |  |
| 4 | lessOrEqual | o | o |  |  |
| 5 | present | o | o |  |  |
| 6 | approximateMatch | o | o |  |  |
| 7 | extensibleMatch | cn | c7 |  |  |
| 7.1 | matchingRule | c:o | c:o |  |  |
| 7.2 | type | c:o | c:o |  |  |
| 7.3 | matchValue | c:o | c:o |  |  |
| 7.4 | dnAttributes | c:o | c:o |  |  |

c7:	If [extfilter] then support of this feature is m else o.

A.4.3.3.20	Paged Results	 	Ref. A.6.3.3.20

Prerequisite: [pageresreq]

| Item No. | Protocol Element | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | PagedResultsRequest | m | m |  |  |
| 1.1 | newRequest | o.n | o.2 |  |  |
| 1.1.1 | pageSize | c:m | c:m |  |  |
| 1.1.2 | sortkeys | c:o | c:o |  |  |
| 1.1.2.1 | type | c:m | c:m |  |  |
| 1.1.2.2 | orderingRule | c:o | c:o |  |  |
| 1.1.3 | reverse | c:m | c:m |  |  |
| 1.1.4 | unmerged | c:m | c:m |  |  |
| 1.2 | queryReference | o.n | o.2 |  |  |

o.2:	At least one of newRequest or queryReference must be present for PagedResultsRequest.

A.4.3.3.21	Continuation Reference

Out of scope. See ISO/IEC ISP 15125-2.

A.4.3.3.22	Security Parameters

Out of scope. See ISO/IEC ISP 15125-5.

A.4.3.3.23	CertificationPath

Out of scope. See ISO/IEC ISP 15125-5.

A.4.3.3.24	Access Control

Out of scope. See ISO/IEC ISP 15125-9.

A.4.3.3.25	Supported References		Ref. A.6.3.3.25

| Item No. | Reference | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Self Reference | o | o |  |  |
| 2 | Superior Reference | o | i |  |  |
| 3 | Immediate Superior Reference | o | i |  |  |
| 4 | Subordinate Reference | o | i |  |  |
| 5 | Non-Specific Subordinate Reference | o | i |  |  |
| 6 | Cross Reference | o | i |  |  |
| 7 | Supplier Reference | i | i |  |  |
| 8 | Consumer Reference | i | i |  |  |

A.4.3.3.26	Access Point		Ref. A.6.3.3.26

| Item No. | Protocol Elements | Protocol Status | Profile Status | Predicate Name | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | AccessPoint | m | m |  |  |
| 1.1 | ae-title | m | m |  |  |
| 1.2 | address | m | m |  |  |
| 1.2.1 | pSelector | o | o |  |  |
| 1.2.2 | sSelector | o | o |  |  |
| 1.2.3 | tSelector | o | o |  |  |
| 1.2.4 | nAddresses | m | m |  |  |
| 1.3 | protocolInformation | o | o |  |  |

A.4.4	Directory Schema and Directory System Schema

Out of scope. See ISO/IEC ISP 15126-1 for Directory Schema and ISO/IEC ISP 15126-2 for Directory System Schema.

A.4.5	Supported ISO/IEC 10646-1 Character Sets

Out of scope. See ISO/IEC ISP 15126-1.

Annex B	)

(Normative)

Profile Specific Implementation Conformance Statement

B.1	Identification of the Profile Corrigenda

The supplier of the PICS proforma shall identify any corrigenda (i.e. Technical Corrigenda or equivalent) to the published ISP that have been applied. Suppliers of the proforma should modify the proforma, or attach relevant additional pages in order to apply the corrigenda, and then record the application of the profile corrigenda in the PICS tables.

| Identification of corrigenda applied to this part of the ISP | ISO/IEC 15125-1 Corr: Corr: Corr: Corr: |
| --- | --- |

Part 8 (ADY 44) DSA Simple Access Control, in the taxonomy, was combined with part 9 (ADY 45) to form a single profile on Access Control. Part 8 therefore does not exist.

Copyright release for ICS proformas: Users of this Recommendation/International Standard may freely reproduce this ICS proforma so that it can be used for its intended purpose and may further publish the completed ICS.