December 2004

PROGRAM GUIDE PG-18-14

ROOM FINISHES, DOOR, AND

HARDWARE SCHEDULE

Department of Veterans Affairs

Office of Facilities Management

Facilities Quality Service (181A)

810 Vermont Avenue, N.W.

Washington, D.C. 20420

CLICK HERE TO

OPEN	DOCUMENT

Department of

Veterans Affairs

http://www.va.gov/facmgt/standard/rms_idx.asp			         PG-18-14

Blank page left intentionally

FOREWORD

This document provides guidelines for interior finishes, doors, and hardware for new or renovated facilities constructed within the Department of Veterans Affairs; and the essential criteria for selection of these materials are their appropriateness for function and space; cost; durability; and ease of maintenance.

Lloyd H. Siegel

Associate CFMO for Strategic Management

Blank page left intentionally

SECTION I General

| Requirements on the use of this Program Guide |  |
| --- | --- |

SECTION II Doors and Hardware

| Type, size, glazing, hardware, and special features of wood and metal doors |  |
| --- | --- |

SECTION III Room Finishes, Door, and Hardware Schedule

Chapters of VHA Handbook 7610-Space Planning Criteria are listed below:

| CHAPTER | SERVICE AREA |
| --- | --- |
| General | Door Sizes |
| General | Telecom, Mechanical Equipment and Elevator Machine Rooms |
| 100 | M. S. and N. Nursing Units |
|  | General Medical Nursing Units General Surgical Nursing Units Orthopedic Nursing Units Neurological Nursing Units Extended Hospital Care (Intermediate) Nursing Units Rehabilitation Medicine Nursing Units Respiratory Care Nursing Units |
| 102 | Intensive Care Nursing Units |
| 104 | Spinal Cord Injury Center (SCI) |
| 106 | Nursing Home Care Units |
| 110 | Mental Health and Behavioral Patient Care Units |
| 202 | Substance Abuse Clinic |
| 204 | Audiology and Speech Pathology Service |
| 206 | Canteen Service |
| 208 | Chaplain Service |
| 210 | Cardiovascular Laboratories |
| 212 | Pulmonary Medicine |
| 214 | Clinical Services Administration |
| 218 | Veterans Assistance Unit |
| 220 | Credit Union |
| 222 | Dental Service |
| 224 | Nutrition and Food Service |
| 226 | Electroencephalography Laboratory |
| 230 | Engineering Service |
| 232 | Ear, Nose and Throat Clinic |
| 233 | Eye Clinic |
| 234 | Fiscal Service |
| 238 | Medical Center Director’s Suite |
| 240 | Pathology and Laboratory Medicine Service |
| 244 | Lobby |
| 246 | Medical Administration Service |
| 248 | Medical Media Service |
| 252 | Nuclear Medicine Service |
| 254 | Nursing Service Administration |
| 260 | Mental Health Clinic |
| 261 | Day Treatment Center |
| 262 | Ambulatory Care(Hospital Based) |
| 266 | Human Resources Management |
| 268 | Pharmacy Service |
| 269 | Recreation Service |
| 270 | Physical Medicine and Rehabilitation Service |
| 272 | Psychology Service |
| 274 | Quarters-On Call |
| 276 | Radiology Service |
| 277 | Radiation Therapy Unit |
| 278 | Medical Research and Development Research Animal Research Radioisotope Suite Health Service Research and Development Rehabilitation Engineering and Development |
| 279 | Police and Security Service |
| 280 | Service Organizations |
| 282 | Social Work Service |
| 284 | Acquisition and Material Management Service Administration |
| 285 | Supply Processing and Distribution |
| 286 | Surgical Service |
| 290 | Voluntary Service |
| 291 | Warehouse |
| 300 | Day Hospital |
| 308 | Prosthetics and Sensory Service |
| 312 | Domiciliary |
| 316 | Dialysis Center |
| 400 | Library Service |
| 402 | Educational Facilities |
| 406 | Environmental Management Service Administration |
| 408 | Environmental Management Service Laundry and Linen Operation |
| 410 | Environmental Management Service Lockers, Lounges, Toilets, Showers |
| 701 | Cemetery Service |
| 703 | Cemetery Service (Maintenance) |
| 800 | Veterans Benefits Administration (Regional Office) Office of Directors |
| 801 | Building Support Facilities |
| 802 | District Counsel |
| 821 | Adjudication Division |
| 823 | Administration Division |
| 824 | Finance Division |
| 825 | Personnel Division |
| 826 | Loan Guaranty Division |
| 827 | Veterans Service Division |
| 828 | Vocational Rehabilitation & Counseling |
| 834 | Support Service |
| 862 | Support Facilities |
| 870 | Service Organizations |

SECTION IV Interior Color and Finish Schedule Forms

| Interior Color and Finish Schedule (New Construction) |  |
| --- | --- |
| Interior Color and Finish Schedule (Renovation) |  |

SECTION I - GENERAL

A.	USE OF PROGRAM GUIDE:

1.	The requirements in this program guide apply to new construction projects and to renovation projects where the existing finishes and door openings are not suitable for the intended use of the space involved. Refer to Standard Details PG-18-4 for additional requirements.

2.	Doors and hardware requirements are indicated by their respective door symbols and hardware set numbers in Section III of this program guide. Additional requirements are shown in Section II, "Doors and Hardware". For door frame requirements refer to Standard Details PG-18-4.

3.	Finishes are scheduled in Section III for the floor, base, wainscot, wall, and ceiling surfaces for each room or space.  See list of finish abbreviations in paragraph (b) below.

a.	Where a combination of materials are separated by the symbol "/" i.e. GWB/CMU, AT/(GWB), etc., the designer shall select the appropriate materials from those listed on the basis of project requirements and cost and shall detail and specify only the selected materials in the construction  documents. The "/" indicate the option of materials listed. Where a combination of materials separated by the symbol "-" i.e. GWB-W, etc., indicates an additional material finish or modification.

b.	The following are abbreviations used for materials and codes throughout the room finishes, door, and hardware schedule. The A/E shall coordinate any additional abbreviations, per project requirements, with the construction documents.

ADO		Automatic Door Operator

AF		Access Flooring

AT		Acoustical Ceiling (Tile)

AT (SP)	Acoustical Ceiling (with Sprayed Plastic Finish)

AWF		Acoustical Wall Fabric (Tackable)

AWP		Acoustical Wall Panel

BP		Brick Pavers (Unit Pavers)

BR		Brick (Unit Masonry)

C		Concrete

CBB		Concrete Backer Board

CC		Color Code

CEF		Conductive Elastomeric (Liquid) Flooring

CMU		Concrete Masonry Units (Unit Masonry)

CP		Carpet (without Cushion Broadloom)

CPT		Carpet Tile

CT		Ceramic Tile (Floor, Base, and Wall)

EFTR		Existing Finish to Remain

EPY 		Epoxy (Coatings)

ERF		Epoxy Resinous Flooring

EX		Existing

EXP		Exposed

GL		Glass (Glazing)

GWB		Gypsum Wallboard Systems

HW		Hardware Set (Finish or Builders Hardware)

LM		Latex Mastic Flooring

MAT		Material

MC		Multi-Color Coating

NF		Natural Finish

NO		Number

P		Paint (Exterior, Interior, Transparent Finishes)

PC		Precast (Architectural Precast Concrete Pavers)

PCP		Portland Cement Plaster

PFW		Polypropylene Fabric Wallcovering

PL		Plaster

PUT		Polyurethane

QT		Quarry Tile

RAF		Resilient Athletic Flooring

RB		Resilient Base (Rubber, Vinyl)

RF		Raised Rubber Flooring

SC		High Build Glazed Coating (Special Coating)

SP		Special Faced

SPEC		Special (Architect's Choice)

ST		Stone (Cast)

RSF		Resilient Sheet Flooring

TT		Terrazzo Tile (Plastic Matrix)

VCT		Resilient Tile Flooring (Vinyl Composition Tile)

VP		Veneer Plaster

W		Wallcovering (Vinyl Coated Fabric)

WB		Wall Border

WD		Wood

WSF		Welded Seam Sheet Flooring

B.	Other Requirements

1.	Safety Glass: Refer to Architectural Design Manual PG-18-10 and Standard Details PG-18-4.

a.	There are three (3) types of safety glass required in Interior Spaces, and they are:

(1) Wire Glass ("W") in steel frames shall be used in the following areas:

a)	Observation Windows in fire rated partitions and smoke partitions.

b)	Vision panels in fire rated doors and smoke doors.

(2) Laminated Glass ("A") shall be used as vision panels in doors and observations windows in the following areas:

a)	Mental Health Unit

b)	Alcohol and Drug Nursing Units

c)	Isolation Rooms in M. S. and N. Nursing Units

(3) Tempered Glass ("T") shall be used in all other Observations Windows and Doors with vision panels, including side lights.

b.	In non-sprinklered buildings requiring fire rated corridors, all glazing between rooms and corridors shall be wire glass in steel frames.

2.	Partition Requirements

a.	Room partitions and corridor partitions shall generally have a single layer of 5/8 inch fire rated gypsum wallboard (GWB) attached to a metal stud system with additional finishes as shown in this Program Guide.  Other partition construction shall be provided where more stringent fire, safety and security requirements dictate the use of other materials. Where fire rated doors are required, partitions shall have a similar fire rating.

b.	For Security Requirements for various partitions, see Design and Construction Procedures PG-18-3, Topic 14, "Physical Security Requirements and Options".

c.	For partitions requiring sound attenuation, see Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

3.	Corner Guards: For corner protective guard requirements see Architectural Design Manual PG-18-10, Article 4.5, "Handrails, Wall Guards and Corner Guards".

4.	Stairways: Use molded rubber treads on Stairs and Resilient Tiles (VCT) on floor landings and rubber tile on intermediate landings except for stairs exposed to the weather or those in strictly utilitarian areas such as shops, boiler plants, etc.

5.	Wall Protection at Drinking Fountains and Lavatories: Provide vinyl wall covering behind Drinking Fountain Alcoves where wallboard or plaster finish occur and behind lavatories and end walls in patient bedrooms, examination rooms, treatment rooms, etc. at a minimum 1250 mm (50 inches) wide (or terminating at the nearest inside corner. The vinyl wall covering shall go from base to ceiling.

SECTION II - DOORS AND HARDWARE

A. DOORS

1.	For basic types and sizes of doors, refer to Architectural Standard Detail PG-18-4 for door and frame details, except where uncommon conditions occur.

2.	The Door Symbols identify the type, size and special features of doors for use in a room or space.  The number and letters used for the Door Symbol have the following meaning; the first number (or numbers) indicate door type and door material (odd nos. wood and even nos. steel), i.e. "1/2, 4, etc." Where combinations appear such as "1/2, 9/10, etc." either type may be used. The first letter (or letters) indicate door size i.e. "M, MM, V, etc." The letters after a dash i.e. “A” indicate a modification to a door. Use these symbols and designations at door openings on floor plan drawings.

3.	Refrigerator, Elevator, Chute and Access Doors are specific doors covered under the relevant specification sections.

4.	For additional door requirements see Barrier Free Design Handbook PG-18-13 and Architectural Design Manual PG-18-10, Article 3.2, "Exterior Entrances and Platforms".

5.	Requirements for exterior entrance doors are outlined in Architectural Design Manual, Article 3.4, "Automatic Doors" and Article 3.3, "Exit and Egress Door Locks", and specification Section 08721, "Automatic Door Operators".

6.	Wood doors, in lieu of steel doors, shall be specified in humid areas or where it is the local practice to use wood doors. Do not use wood doors for fire doors and where high impact conditions are anticipated. Select a suitable door type for these conditions.

7.	Door Swings:

a.	Doors to Housekeeping Aids Closets (HAC), Mental Health Security Bedrooms and Isolation Rooms in M. S. and N. Nursing Units shall open out. HAC doors shall swing out 180 degrees where possible.

b.	Doors to toilets or baths, opening off Patient Bedrooms shall open out or shall have break-away features.

B. FINISH HARDWARE

1.	HARDWARE RESPONSIBILITY: The selection of hardware and the preparation of Builders (Finish) Hardware Specification must be done by a Hardware Consultant or Specialist.

2. HARDWARE SETS:

a.	General: Hardware for door openings from corridors to rooms and spaces is listed under column headings "HARDWARE - H.W. NO." in this program guide, except as otherwise shown below. These hardware sets correspond to sets of hardware specified in specification Section 08710, "Builders Hardware".

b.	Exceptions: Hardware sets for the following conditions and door openings must be properly selected by a Hardware Consultant from the appropriate hardware sets listed in specification Section 08710, "Builders Hardware" or new non-proprietary sets must be developed to suit the project conditions and door openings.

(1) Communicating Doors: Generally, for doors in partitions that separate offices, specify hardware set HW 7 where no security is necessary, or a set with a lock if security is needed.

(2) Doors in Fire Partitions: Generally, the hardware sets listed under the column heading "HARDWARE - H.W. NO." are for non-fire resistance rated partitions. Where fire resistance rated partitions occur, substitute a hardware set which corresponds to the set listed in this handbook for the space, but is self-closing and self-latching.

In cases where it is important for a fire door to remain open under normal conditions, select a hardware set that has a combination closer-holder instead of a closer only. Closer-holders must be connected to the fire alarm system.

(3) Lead Lined Doors: Coordinate the hardware sets with the lead lined door and frame details shown on the Standard Detail drawings and the project drawings. Refer to Standard Details PG-18-4, Architectural.

(4) Chapel Doors: For corridor entrance doors to NAVE, make up a special chapel hardware set which reflects a nondenominational theme.

(5) Existing Doors or Frames to Remain in Place: Select and coordinate the hardware requirements for these conditions to insure that suitable/compatible hardware is provided for the intended use of the space.

(6) Corridor Doors: Cross corridor doors to Dementia Units and other Nursing Units may require magnetic locks to control the free movement of patients. Such locks must meet the requirements of NFPA-101 as well as allow staff movement in and out of the Nursing Unit. The hardware consultant may use hardware set HW 111 or select other non-proprietary hardware to accomplish the desired results.

SECTION III - ROOM FINISHES, DOOR AND HARDWARE SCHEDULE

DOOR SIZES

| MARK | WIDTH | HEIGHT | MARK | WIDTH | HEIGHT |
| --- | --- | --- | --- | --- | --- |
| A | 1'-6" | 6'-8" | N | 2'-0" | 7'-0" |
| B | 2'0" | 6'-8" | O | 2'-2" | 7'-0" |
| C | 2'-2" | 6'-8" | P | 2'-4" | 7'-0" |
| D | 2'-4" | 6'-8" | Q | 2'-6" | 7'-0" |
| E | 2'-6" | 6'-8" | R | 2'-8" | 7'-0" |
| F | 2'-8" | 6'-8" | S | 3'-0" | 7'-0" |
| G | 2'-10" | 6'-8" | T | 3'-4" | 7'-0" |
| H | 3'-0" | 6'-8" | U | 3'-6" | 7'-0" |
| I | 3'-2" | 6'-8" | V | 3'-8" | 7'-0" |
| J | 3'-4" | 6'-8" | W | 3'-10" | 7'-0" |
| K | 3'-8" | 6'-8" | X | 4'-0" | 7'-0" |
| L | 3'-10" | 6'-8" | Y | 3'-0" | 8'-0" |
| M | 1'-6" | 7'-0" | Z | UNASSIGNED |  |

DOOR MODIFICATIONS

A	LAMINATED GL

D	DOUBLE GLAZED TEMPERED GLASS

H	ELECTRIC HOLD

L	LOUVER

LP	LIGHT PROOF LOUVER

M	MECHANICAL SEAL FOR LIGHT PROOF & SOUND RETARDING DOOR

T	TEMPERED GLASS

U	UNDERCUT 1"

V	LEAD GLASS

W	WIRE GLASS

DOOR SYMBOLS                      DOOR TYPE

FRAME TYPE       1  2

(JAMB DETAIL)      S   T

DOOR MODIFICATION

DOOR

SIZE

(DOUBLE LETTERS

EQUAL SIZE OF

LEAF FOR PAIR OF DOORS)

LEGEND (SEE DOOR ELEVATIONS)

--

|  |	GL OR L REQUIRED BY MODIFICATION

--

__

|__|	GL OR L REQUIRED BY ELEVATION TYPE

NOTES:

1.	ALL DOORS SHALL BE FLUSH AND 1-3/4" THICK UNLESS NOTED OTHERWISE.

2.	TOP AND SIDE RAILS SHALL BE 9" MIN. BOTTOM RAILS 12" MIN.

3.	FOR SCHEDULING DOOR TYPES SEE SECTION III.

4.	ON THE DOOR TYPE SCHEDULE, EVEN DOOR NUMBERS ARE FOR METAL DOORS, AND ODD NUMBERS ARE FOR WOOD DOORS.

5.	SEE STANDARD DETAILS PG-18-4 FOR DOOR TYPES.

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Large Telephone Equipment Room | VCT | RB | GWB |  | AT | 1/2 X | 43 |
| Electric, Telephone & Signal Closets | C-EPY | RB | GWB |  | EXP | 1/2 T or as required | 13 |
| Mechanical Equipment Room | C-EPY | RB | GWB |  | EXP | 11/12 VV-W | 43 |
| Elevator Machine Room | C-EPY | RB | GWB1 |  | AT | 1/2 V | 43 |
| Passenger Elevator Lobbies | VCT | RB | GWB or CMU W/ Special |  | AT | 1/2 VV-T2 | 6 |
| Service Elevator Lobbies | VCT | RB | GWB or CMU | CT 4'0" | AT | 1/2 VV-T2 | 6 |
| Stairs & Stairwells | VCT2 | RB | GWB or CMU |  | EXP | Int. 9/10 S Ext. 2 S 10 S Interstitial | 38 83 43 |

| ROOM OR SPACE | FINISHES |  |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR |  | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| M & S Nursing Units include the following: GENERAL MEDICAL GENERAL SURGICAL ORTHOPEDIC NEUROLOGICAL INTERMEDIATE CARE REHABILITATION MEDICINE RESPIRATORY CARE See SERVICE AREAS for spaces not shown below. Note Nursing Units such as Intensive Care, Spinal Cord Injury, Nursing Home Care and Mental Health and Behavioral Sciences Nursing Units are published as separate chapters. |  |  |  |  |  |  |  |  |
| Nursing Unit: |  |  |  |  |  |  |  |  |
| Bedrooms |  | VCT | RB | GWB |  | AT | 1/2 W | 7 |
| Isolation Room* |  | VCT | RB | GWB (SC) |  | GWB | 3/4 W-A | 12 |
| Anteroom* |  | VCT | RB | GWB (SC) |  | GWB | 1/2 W-A | 7 |
| Isolation Bathroom/Toilet* |  | CT | CT | GWB2 | CT 4'0" | GWB | 1/2 S | 7 |
| Bathroom/Toilet |  | CT | CT | GWB2 | CT 4'0" | AT | 1/2 S | 7 |
| Congregate Bath/Toilet and Vestibule |  | CT | CT | CT |  | SC | 1/2 W | 2 |
| Communication Center (Nurses Station) |  | VCT/ CP | RB | GWB-W |  | AT | Open |  |
| Doctors Charting/Dictation Area |  | VCT/ CP | RB | GWB |  | AT | Open |  |
| Office Ward/Clerk |  | VCT/ CP | RB | GWB |  | AT | Open |  |
| Medication Room |  | VCT | RB | GWB |  | AT | 1/2 S | 13 |
| Toilet, Staff |  | CT | CT | GWB | CT 4'-0" | AT AT | 1/2 S 1/2 S | 1 |
| Nourishment Station |  | VCT | RB | GWB |  | AT | 1/2 S | 23 |
| Office, Head Nurse |  | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Physician/ Consultant |  | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Exam/ Treatment Room |  | RSF | RSF | GWB (SC) |  | AT (SP) 1 | 1/2 S | 69 |
| Cast Room |  | RSF | RB | GWB |  | AT (SP) | 1/2 W | 67 |
| Storage |  | VCT | RB | GWB |  | AT | 11/12 V-W | 69 |
| Office - Physicians Assistant/ Nurse Clinician |  | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Nursing Supervisor |  | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Social Worker |  | CP | RB | GWB-W1 |  | AT | 1/2 S-T | 23 |
| Clean Utility Room |  | VCT | RB | GWB(SC) |  | AT (SP) | 11/12 V-W | 45 |
| Soiled Utility Room |  | RSF | RB | GWB(SC) |  | AT (SP) | 11/12 V | 45 |
| Disposal Chute Rooms (Linen & Trash) |  | VCT | RB | GWB (SC) |  | AT | 11/12/ V-W | 45 |
| Linen Rooms: |  |  |  |  |  |  |  |  |
| Clean Linen Room | VCT |  | RB | GWB(SC) |  | GWB | 11/12 SS-W | 150 |
| Soiled Linen Room | VCT |  | RB | GWB(SC) |  | GWB | 11/12 SS-W | 150 |
| Day-Room Recreation | VCT/ CP |  | RB | GWB-W |  | AT | 3/4 V-T | 40 |
| Storage - Wheelchair & Stretcher Alcove | VCT |  | RB | GWB-W |  | AT | Open |  |
| Storage - Equipment | VCT |  | RB | GWB-W |  | AT | 11/12 V-W Open | 69 |
| Housekeeping Aids Closet | CT |  | CT | GWB (SC) | CT 4'-0" | AT (SP) | 1/2 T | 13 |
| Support Facilities: |  |  |  |  |  |  |  |  |
| Storage - Mobile X-Ray Machine Alcove | VCT |  | RB | GWB-W |  | AT | Open |  |
| Storage, Building | VCT |  | RB | GWB |  | AT | 11/12 V-W | 69 |
| Disposal Chute | VCT |  | RB | GWB(SC) | AT | 11/12 V-W | 153 |  |
| Visitors Facilities: |  |  |  |  |  |  |  |  |
| Lounge | CP/VCT |  | RB | GWB-W |  | AT |  |  |
| Toilets - M & F | CT |  | CT | GWB | CT 4'-0" | AT | 1/2 S | 1 |
| Material Handling (Clean) | VCT |  | RB | GWB(SC) |  | GWB | 11/12 SS-W | 150 |
| Material Handling (Soiled) | VCT |  | RB | GWB(SC) |  | GWB | 11/12 SS-W | 150 |
| Quarters On-Call |  |  | See Chapter 274 |  |  |  |  |  |
| Staff Lockers, Lounges, Toilets, and Showers |  |  | See Chapter 410 |  |  |  |  |  |
| Education: |  |  |  |  |  |  |  |  |
| Office Work Room - House Staff & Medical Student | CP |  | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Nursing Instructor and Student Work Room | CP |  | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Demonstration/Classroom | CP |  | RB | GWB-W |  | AT | 1/2 U-T | 40 |
| Storage Area | VCT |  | RB | GWB |  | AT | 1/2 S | 23 |
| Corridors | VCT |  | RB | GWB |  | AT | 1/2 VV-T | 6,1 111 |

See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Isolation Bed Room* | VCT | RB | GWB (SC) |  | GWB | 3/4 X-T | 12 |
| Ante Room* | VCT | RB | GWB (SC) |  | GWB | 1/2 S-T | 12 |
| One Bed Cubicle/Room | VCT | RB | GWB |  | AT (SP) | Open/ 3/4 X-T | 12 |
| Control Center | VCT | RB | GWB |  | AT (SP) | Open |  |
| Linen Closet | VCT | RB | GWB |  | AT | 11/12 U-W | 45 |
| Clean Utility Room | VCT | RB | GWB (SC) |  | AT (SP) | 11/12 V-W | 45 |
| Soiled Utility Room | RSF | RB | GWB (SC) |  | AT (SP) | 11/12 V-W | 45 |
| Patient Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 14 |
| Lockers, & Toilets |  |  |  |  |  |  |  |
| Lounge | CP/VCT | RB | GWB-W |  | AT | 1/2 S | 1 |
| Lockers | VCT | RB | GWB |  | AT | 1/2 S-T | 68 |
| Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Family Waiting Area | CP | RB | GWB-W |  | AT | 1/2 S-T | 39 |
| Public Toilet - Male & Female | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 19 |
| Consultation/ Meditation Room | CP | RB | GWB-W |  | AT2 | 1/2 V-T | 52 |
| Head Nurse Office | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Equipment Storage | VCT | RB | GWB |  | AT | Open | 69 |
| Stretcher Storage | VCT | RB | GWB |  | AT | 11/12 V-W | 69 |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'-0" | AT (SP) | 1/2 T | 13 |
| On-Call Room | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Bath, On-Call | CT | CT | GWB | CT 4'-0" | AT | 1/2 S | 18 or 17 |
| Corridor (Suite Entrance) | VCT | RB | GWB |  | AT | 1/2 SS-T/ 1/2 X-T ADO | 128 129 |

NOTE:  Spaces and areas shown below are based on the Spinal Cord Injury (SCI) Design Guide.

| ROOM OR SPACE | FINISHES |  |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE |  | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Bedrooms | VCT | RB |  | GWB |  | AT | 1/2 X-T | 7 |
| Isolation Room* | VCT | RB |  | GWB (SC) |  | GWB | 3/4 X-T | 12 |
| Ante Room* | VCT | RB |  | GWB (SC) |  | GWB | 1/2 S-T | 12 |
| Private Litter Bath | CT | CT |  | CT |  | PCP(SC) | 1/2 X ADO Sliding Door | 127 |
| Nurse Station | VCT/ CP | RB |  | GWB-W |  | AT |  |  |
| Medication Room | VCT | RB |  | GWB |  | AT | 1/2 S | 13 |
| Staff Toilet | CT | CT |  | GWB | CT 4'0" | AT | 1/2 S | 19 |
| Office Head Nurse | CP | RB |  | GWB-W |  | AT | 1/2 U | 23 |
| Nourishment Kitchen | VCT | RB |  | GWB |  | AT | 1/2 S-T | 23 |
| Clean Utility Room | VCT | RB |  | GWB |  | AT (SP) | 11/12 V-W | 153 |
| Soiled Utility Room | RSF | RB |  | GWB |  | AT (SP) | 11/12 V-W | 153 |
| Linen Rooms: |  |  |  |  |  |  |  |  |
| Clean Linen Room | VCT | RB |  | GWB |  | AT (SP) | 11/12 V-W | 153 |
| Soiled Linen Room | VCT | RB |  | GWB |  | AT (SP) | 11/12 V-W | 153 |
| Hopper/Shower Room | CT | CT |  | CT |  | PCP | 1/2 X ADO 1 | 127 |
| Tub Room | CT | CT |  | CT |  | PCP | 1/2 T | 4 |
| Storage Equipment: |  |  |  |  |  |  |  |  |
| Litter Storage | VCT | RB |  | GWB |  | AT | 11/12 X-W | 153 |
| Wheelchair Storage | VCT | RB |  | GWB |  | AT | 1/2 X-W | 34 |
| Medical Equipment Storage | VCT | RB |  | GWB |  | AT | 1/2 X-W | 13 |
| Education: |  |  |  |  |  |  |  |  |
| Conference/ Classroom | VCT | RB | GWB-W, |  |  | AT | 1/2 X-T | 52 |
| Quiet Room | VCT | RB | GWB-W |  |  | AT | 1/2 U-T | 52 |
| Housekeeping Aids Closet | CT | CT | GWB (SC) |  | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Patient Activities Areas: |  |  |  |  |  |  |  |  |
| Day Room | VCT | RB |  | GWB-W |  | AT | 3/4 SS-T/ ADO | 34 |
| Laundry Room | RSF | RB |  | GWB |  | AT | 1/2 U-T | 4 |
| Visitor's Lounge | VCT | RB |  | GWB-W |  | AT |  |  |
| Toilets, Visitor's | CT | CT |  | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Dining Room | VCT | RB |  | GWB-W |  | AT | 3/4 SS-T ADO3 | 120 |
| Vending Room | VCT | RB |  | GWB |  | AT |  |  |
| Serving Lines | QT | QT |  | CT |  | AT (SP) |  |  |
| Dishwashing | QT | QT |  | CT |  | AT (SP) |  |  |
| Non-Food Storage | VCT | RB |  | GWB |  | AT | 11/12 V-W | 76 |
| Recreational Therapy | VCT | RB |  | GWB-W |  | AT | 3/4 SS-T/ ADO3 | 80 |
| Self Directed Rec. Therapy | VCT | RB |  | GWB-W |  | AT | 3/4 U-T | 131/ 126/69 |
| Greenhouse | C |  |  | Spec. |  | Spec. | 3/4 U-T | 53 |
| Building Management Storage | LM | LM |  | GWB |  | AT (SC) | 11/12 V-W | 43 |
| Patient Clothing & Luggage Storage | VCT | RB |  | GWB |  | AT | 11/12 V-W | 76 |
| Soiled Linen Holding | CT | CT |  | GWB | CT 4'0" | AT (SP) | 11/12 U-T | 153 |
| Unisex Patient Toilet | CT | CT |  | GWB | CT 4'0" | AT | 1/2 U | 14 |
| On Call Bedroom | CP | RB |  | GWB-W |  | AT | 1/2 S | 19 |
| On Call Toilet/ Shower | CT | CT |  | GWB | CT 4'0" | AT1 | 1/2 S | 18/14 |
| Physicians Office | CP | RB |  | GWB-W |  | AT | 1/2 S | 23 |
| Exam/ Treatment | VCT | RB |  | GWB2 |  | AT/GWB | 1/2 X | 52 |
| Cystoscopy Room/ Control Area and Light Room | WSF | WSF |  | GWB (SC)2 |  | GWB | 15 X | 106 |
| Cystoscopy Scrub | WSF | WSF |  | GWB (SC) |  | GWB |  |  |
| Lithotripsy Room w/ Control & Scrub | WSF | WSF |  | GWB (SC) |  | GWB | 15 X | 106 |
| Lithotripsy Scrub | WSF | WSF |  | GWB (SC)3 |  | GWB |  |  |
| Recovery Room | VCT | RB |  | GWB |  | AT | 1/2 X-T | 72 |
| Film Process/ Dark Room | WSF | WSF |  | GWB |  | GWB | 1/2 S-M | 67 |
| G.U. Dressing Room | VCT | RB |  | GWB |  | AT | 1/2 X | 14 |
| Outpatient Clinic: |  |  |  |  |  |  |  |  |
| Outpatient Clinic Nurse Station | VCT | RB |  | GWB-W |  | AT | Open |  |
| Outpatient Clinic Clean Utility | VCT | RB |  | GWB (SC) |  | GWB | 11/12 V-W | 153 |
| SCI Home Care Interview Room | VCT | RB |  | GWB/ GWB-W |  | AT | 1/2 V | 23 |
| SCI Outpatient Pharmacy | CP | RB |  | GWB1 |  | AT1 | 36 V | 157 |
| Medication Preparation | CP | RB |  | GWB1 |  | AT1 | 36 V | 157 |
| Home Environment Clinic: |  |  |  |  |  |  |  |  |
| Clinic/ Living - Dining Room | CP | RB |  | GWB-W |  | AT | 1 S | 52 |
| Kitchen | VCT | RB |  | GWB |  | AT |  |  |
| Bedroom | CP | RB |  | GWB |  | AT | 1 S | 7 |
| Bathroom | CT | CT |  | GWB | CT 4'0" | GWB | 1 S | 18 |
| Hydrotherapy | CT | CT |  | CT | CT Full Height | PCP | 1/2 X | 7 |
| Combined Physical & Kinesiotherapy: |  |  |  |  |  |  |  |  |
| Treatment Clinic | VCT | RB |  | GWB2 |  | AT | 3/4 SS-T/ ADO3 | 80 126 |
| Physical Therapy: |  |  |  |  |  |  |  |  |
| Treatment Clinic | SF | RB |  | GWB3 |  | AT | 3/4 SS-T/ ADO3 | 80 126 |
| Kinesiotherapy: |  |  |  |  |  |  |  |  |
| Treatment Clinic | SF | RB |  | GWB1 |  | AT | 3/4 SS-T/ ADO3 | 80 126 |
| Occupational Therapy: |  |  |  |  |  |  |  |  |
| Treatment Clinic | VCT | RB |  | GWB |  | AT | 3/4 SS-T/ ADO3 | 80 126 |
| Activities of Daily Living: |  |  |  |  |  |  |  |  |
| Kitchen | VCT | RB |  | GWB |  | AT |  |  |
| Bedroom | CP | RB |  | GWB-W |  | AT | 1/2 S | 7 |
| Bathroom | CT | CT |  | GWB | CT 4'0" | GWB | 1/2 S | 18 |
| Therapeutic Pool | CT | CT |  | CT Full Height |  | PCP | 3/4 X-T/ ADO | 69 120 |
| Shower/Toilet | CT | CT |  | CT2 | CT 4'-0" | PCP |  |  |
| Suits/Towels | CT | CT |  | GWB |  | AT (SP) | 1/2 U-T | 69 |
| Pool Vestibule | CT | CT |  | GWB |  | AT (SP) | 1/2 U/ ADO3 | 69 120 |
| SCI Chief's Office | VCT/ CP | RB |  | GWB |  | AT | 1/2 U | 52 |
| Main Entrance | VCT | RB |  | GWB |  | AT | 2-3'x 7' /1-4'x 7' Slide Glass Doors ADO3 | 120 126 |
| Corridor | VCT | RB |  | GWB |  | AT | 1/2 VV-T/ ADO2 | 6 |

NOTE:  All Door heights in NHCUs not built integrally with a hospital building shall be 6'-8" unless otherwise noted.

| ROOM OR SPACE | FINISHES |  |  |  |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE |  | WALLS |  | WAINS |  | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |  |  |  |
| Office, Nursing Supervisor | CP | RB | GWB-W |  |  |  |  | AT | 1/2 H | 23 |
| Office, Secretary and Waiting | CP | RB | GWB-W |  |  |  |  | AT | 1/2 H | 23 |
| Office, Multi- Disciplinary | CP | RB | GWB-W |  |  |  |  | AT | 1/2 H | 23 |
| Office, Dietitian | CP | RB | GWB-W |  |  |  |  | AT | 1/2 H | 23 |
| Office, Physician | CP | RB | GWB-W |  |  |  |  | AT | 1/2 H | 23 |
| Office, Speech Pathology | CP | RB | GWB-W |  |  |  |  | AT | 1/2 H | 23 |
| Pharmacy1 | CP | RB | GWB |  |  |  |  | AT | 36 K | 157 |
| Conference/ Classroom | VCT | RB | GWB2,3 |  |  |  |  | AT | 1/2 K-T | 40 |
| Lobby Facilities: |  |  |  |  |  |  |  |  |  |  |
| Lobby | Finish to be selected on the basis of requirements and cost. |  |  |  |  |  |  |  |  |  |
| Toilets | CT | CT | GWB |  | CT |  |  | AT | 1/2 H | 14/1 |
| Employees Lockers, Lounge, Toilets | See Chapter 410 |  |  |  |  |  |  |  |  |  |
| Housekeeping Aids Closet | CT | CT | GWB (SC) |  | CT 4'-0" |  |  | AT (SP) | 1/2 J | 13 |
| Nursing Units: |  |  |  |  |  |  |  |  |  |  |
| Bedrooms | VCT | RB |  | GWB |  |  |  | AT | 1/2 L | 7 |
| Wheelchair Toilets | CT | CT |  | GWB |  |  | CT 4'-0" | AT | 1/2 H | 14 |
| Full Bathrooms | CT | CT |  | GWB |  |  | CT1 4'-0" | GWB(SC) | 1/2 H | 14 |
| Cong. Bath with Toilet and Hopper | CT | CT |  | CT |  |  |  | PCP | 1/2 H | 4 |
| Nurses Station Area: |  |  |  |  |  |  |  |  |  |  |
| Nurses Station and Ward Clerk | VCT/ CP | RB |  | GWB-W |  |  |  | AT | Open |  |
| Medication Room | VCT | RB |  | GWB |  |  |  | AT | 1/2 H | 13 |
| Office, Head Nurse | CP | RB |  | GWB |  |  |  | AT | 1/2 H | 23 |
| Office, Social Worker | CP | RB |  | GWB |  |  |  | AT | 1/2 H | 23 |
| Exam and Treatment Room | VCT | RB |  | GWB2 |  |  |  | AT/GWB2 Lay-in Panels in grid system | 1/2 L | 23 |
| Patient's Laundry | VCT | RB |  | GWB |  |  |  | AT | 1/2 H-T | 4 |
| Clean Utility Room | VCT | RB |  | GWB (SC) |  |  |  | AT (SP) | 11/12 K-W | 45 |
| Nourishment Kitchen | VCT | RB |  | GWB |  |  |  | AT | 1/2 H | 23 |
| Ward Supply Room | VCT | RB |  | GWB |  |  |  | AT | 1/2 K-W | 43 |
| Soiled Utility Room | RSF | RB |  | GWB (SC) |  |  |  | AT (SP) | 11/12 K-W | 45 |
| Equipment Storage | VCT | RB |  | GWB |  |  |  | AT | 11/12 K-W | 69 |
| Housekeeping Aids Closet | CT | CT |  | GWB (SC) |  | CT 4'-0" |  | AT (SP) | 1/2 J | 13 |
| Patients Lounge and TV Room | VCT | RB |  | GWB-W |  |  |  | AT | 1/2 K-T | 52 |
| Storage (off lounge) | VCT | RB |  | GWB |  |  |  | AT | 1/2 K | 13 |
| Support Facility: |  |  |  |  |  |  |  |  |  |  |
| Patients Quiet Room | CP | RB |  | GWB-W |  |  |  | AT | 1/2 K-T | 52 |
| Dining Room/ Recreation Area | VCT | RB |  | GWB-W |  |  |  | AT | 1/2 GG-T ADO | 126 |
| Dietetics: |  |  |  |  |  |  |  |  |  |  |
| Housekeeping Aids Closet | CT | CT |  | GWB (SC) |  | CT 4'-0" |  | AT (SP) | 1/2 J | 13/7 |
| Tray Cart Storage | CT | CT |  | CT |  |  |  | AT (SP) | 1/2 K-T | 69 |
| Serving Unit | CT | CT |  | CT |  |  |  | AT (SP) | 1/2 HH-T | 69 |
| Non-Food Storage | VCT | RB |  | GWB |  |  |  | AT | 11/12 K-W | 43 |
| Therapists Office | CP | RB |  | GWB |  |  |  | AT | 1/2 H | 23 |
| Physical Activity Area: |  |  |  |  |  |  |  |  |  |  |
| Clinic Space | RSF | RB |  | GWB |  |  |  | AT | 1/2 L-T | 69 |
| Storage Space | RSF | RB |  | GWB |  |  |  | AT | 1/2 J | 34 |
| Hydrotherapy | CT | CT |  | CT |  |  |  | AT (SP) | Open |  |
| Occupational Activity: |  |  |  |  |  |  |  |  |  |  |
| Clinic Space | VCT | RB |  | GWB |  |  |  | AT | 1/2 L-T | 69 |
| Storage Space | VCT | RB |  | GWB |  |  |  | AT | 1/2 K-T/ Open | 43 |
| General Storage | VCT | RB |  | GWB |  |  |  | AT | 11/12 K-W | 43 |
| Residents Clothing and Luggage Storage Room | VCT | RB |  | GWB |  |  |  | AT | 11/12 K-W | 43 |
| Building Management: |  |  |  |  |  |  |  |  |  |  |
| Supply and Equipment Storage Room | LM | LM |  | GWB (SC)/ CMU |  |  |  | AT | 11/12 K-W | 43 |
| Soiled Linen Holding Area | RSF | RB |  | GWB (SC)/ CMU |  |  |  | AT (SP) | 11/12 K-W | 153 |
| Trash Collection | C | RB |  | GWB (SC)/ CMU |  |  |  | AT (SP) | 11/12 K-W | 153 |
| Oxygen Storage | C |  |  |  |  |  |  |  |  |  |
| Corridor | VCT | RB |  | GWB |  |  |  | AT | 1/2 KK-T | 61 |

1 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |  |
| Bed Rooms | VCT | RB | GWB |  | GWB | 1/2 W | 7 |  |
| Seclusion Room | VCT | RB | GWB OR CMU |  | GWB | 1/2 W-A | 9 |  |
| Ante Room | VCT | RB | GWB1, OR CMU2 |  | GWB2 | 1/2 W-A | 54 |  |
| Bath Room, Patient and Wheelchair Patient | CT | CT | CT |  | GWB | 1/2 S | 14 10 for Seclusion |  |
| Toilets, Patient and Wheelchair Patient | CT | CT | GWB | CT 4'0" | GWB | 1/2 S | 14 10 for Seclusion |  |
| Congregate Facilities: Patient Toilet, Wheelchair Patient, Entrance Vestibule, Stretcher Shower, Tub Bath | CT | CT | CT |  | GWB(SC) | 1/2 W | 2 4 for Accessibility |  |
| Supporting Facilities: |  |  |  |  |  |  |  |  |
| Communication Center Nurses Station | VCT | RB | GWB |  | AT | Open |  |  |
| Doctor's Charting Dictation Area | VCT | RB | GWB |  | AT | Open |  |  |
| Ward Clerk | VCT | RB | GWB |  | AT | Open |  |  |
| Medication Room | VCT | RB | GWB |  | AT | 1/2 S |  | 13 |
| Staff Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 S |  | 13 |
| Nourishment Station | VCT | RB | GWB |  | AT | 1/2 S |  | 13 |
| Head Nurse's Office | CP | RB | GWB |  | AT | 1/2 S |  | 23 |
| Physician's Office and Exam/Treatment Room | VCT | RB | GWB2 |  | AT2 | 1/2 V |  | 23 |
| Physician Asst./ Nurse Clinicians Office | VCT/ CP | RB | GWB2 |  | AT2 | 1/2 |  |  |
| Psychologist’s Office | VCT/ CP | RB | GWB1 |  | AT1 | 1/2 S |  | 23 |
| Multipurpose Office | VCT/ CP | RB | GWB1 |  | AT | 1/2 S |  | 23 |
| Social Worker's Office | VCT/ CP | RB | GWB1 |  | AT1 | 1/2 S |  | 23 |
| Dietician's Office | VCT/ CP | RB | GWB |  | AT | 1/2 S |  | 23 |
| Nursing Supervisor's Office | VCT/ CP | RB | GWB1 |  | AT | 1/2 S |  | 23 |
| Clean/Soiled Utility Room | RSF | RB | GWB |  | AT (SP) | 11/12 V-W |  | 43 |
| Clean/Soiled Linen Room | VCT | RB | GWB |  | AT (SP) | 11/12 V-W |  | 43 |
| Disposal Chutes - Vestibule (Linen and/or Trash) | VCT | RB | GWB |  | AT (SP) | 11/12 V-W |  | 43 |
| Storage Room (Equipment) | VCT | RB | GWB |  | AT | 11/12 V-W |  | 43 |
| Day Room - Recreation | VCT | RB | GWB1 |  | GWB | 1/2 V-A |  | 52 |
| Quiet Room | VCT | RB | GWN1 |  | GWB | 1/2 S-A |  | 52 |
| Stretchers Wheel-chair, Alcove | VCT | RB | GWB |  | GWB | Open |  |  |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T |  | 43 |
| Building Management Equipment Room | VCT | RB | GWB |  | AT | 11/12 V-W |  | 43 |
| Visitors Facilities: |  |  |  |  |  |  |  |  |
| Toilets (Male and Female) | CT | CT | GWB | CT 4'0" | GWB | 1/2 S |  | 1 or 14 |
| Bulk Food Carts | CT | CT | CT |  | AT (SP) | 1/2 U-A |  | 69 |
| Serving Line | CT | CT | CT |  | AT (SP) | 1/2 U-A |  | 69 |
| Non-Food Storage | VCT | RB | GWB |  | AT | 1/2 U-A |  | 34 |
| Dining Space | VCT | RB | GWB-W |  | AT | 1/2 U-A |  | 67 |
| Interview/ Consultation Room | VCT/ CP | RB | GWB1 |  | AT1 | 1/2 V-A |  | 52 |
| Offices: Rehabilitation Medicine, Therapists and Vocational Rehabilitation Specialist | CP | RB | GWB1 |  | AT1 | 1/2 S |  | 23 |
| Rehabilitation Technician (includes interview space) | CP | RB | GWB1 |  | AT | 1/2 S |  | 23 |
| Education: |  |  |  |  |  |  |  |  |
| Office, House Staff | CP | RB | GWB |  | AT | 1/2 S |  | 23 |
| Office, Nursing Instructor and Student Work Room | CP | RB | GWB |  | AT | 1/2 U-A |  | 52 |
| Demonstration/ Class Room (and Storage Space) | VCT | RB | GWB1 |  | AT1 | 1/2 SS-A |  | 58 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-A |  | 822 |

1 Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Section II, "Doors and Hardware".

Where Treatment Clinics are not located at a VAMC, all doors shall be 6'-8" in lieu of 7'-0".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration Space: |  |  |  |  |  |  |  |
| Office, Nurse/ Pharmacist | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Physician | CP | RB | GWB-W1 |  | AT/GWB1 | 1/2 S | 23 |
| Office, Psychologist /Chaplain | CP | RB | GWB-W1 |  | AT/GWB1 | 1/2 S | 23 |
| Office, Rehabilitation Counselor | CP | RB | GWB-W1 |  | AT/GWB1 | 1/2 S | 23 |
| Office, Social Worker | CP | RB | GWB-W1 |  | AT/GWB1 | 1/2 S | 23 |
| Office Clerk | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Clinic Space: |  |  |  |  |  |  |  |
| Reception and Control Area | VCT/CP | RB | GWB-W |  | AT | 1/2 S-T/Open | 23 |
| Waiting Area | VCT/CP | RB | GWB-W |  | AT | 3/4 S-T/ Open | 67 |
| Group Therapy/ Conference Room | VCT/CP | RB | GWB-W1,2 |  | AT1 | 1/2 V-T | 52 |
| Specimen Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 14 |
| Utility & Storage | VCT | RB | GWB |  | AT | 1/2 V-T 9/10 V-W | 76 42 |
| Lockers, Lounges, Toilets and Showers (LLTS) | See Chapter 410 |  |  |  |  |  |  |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T | 813 |

1 Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 Provide acoustical/ tackable wall surface on one wall.

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Waiting Room | CP | RB | GWB |  | AT | Open |  |
| Reception/Control Clerk | CP | RB | GWB |  | AT | 1/2 S-T /Open | 13 |
| Office, Chief of Program | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Secretary | CP | RB | GWB |  | AT | 1/2 S-T /Open | 23 |
| Office, Staff, Examination, Interview, Audiologist and Speech Pathologist | CP | RB | GWB |  | AT | 19/20 V-T | 52 |
| Trainee Study Cubicle | CP | RB | Prefabricated |  | AT | Open |  |
| Sound Suite - Control Room, Examination Room | Prefabricated |  |  |  |  |  |  |
| Speech Pathology Group Therapy | CP | RB | GWB1 |  | AT | 19/20 V-T M | 52 |
| Instrument Calibration and Storage | VCT | RB | GWB1 |  | AT | 19/20 V-D M | 52 |
| Acoustic Impedance Room | CP | RB | GWB1 |  | AT | 19/20 V M | 52 |
| Electronystagmography | VCT | RB | GWB1 |  | AT | 19/20 V | 52 |
| Earmold Fabrication Room | VCT | RB | GWB1 |  | AT | 19/20 V | 52 |
| Aural Rehabilitation Room | CP | RB | GWB1 |  | AT | 19/20 V | 52 |
| Corridor | CP | RB | GWB1 |  | AT | 19/20 V-D | 52 |

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Administrative Service: |  |  |  |  |  |  |  |
| Office, Chief | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office Assistant Chief, Food Section | VCT | RB | GWB |  | AT | 1/2 S | 23 |
| Secured Money Handling Room1 | VCT | RB | GWB2 |  | AT | 36 S | 157 |
| Administrative Office | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Customer Service and Related Storage: |  |  |  |  |  |  |  |
| Retail Store (Sales Rooms)1 | CP | RB | GWB |  | AT | 1/2 SS-T ADO3 | 124 |
| Retail Storage and Receiving Room1 | VCT | RB | GWB |  | AT | 2 SS-T | 82 |
| Dining Area | VCT | RB | GWB-W |  | AT | 1/2 SS-T | 82 |
| Kitchen | QT | QT | CT2 |  | AT (SP) | 1/2 SS-T | 82 |
| Food Service Area | QT | QT | CT2 |  | AT (SP) | 1/2 V-T | 69 |
| Food Storage Room | VCT | RB | GWB |  | AT (SP) | 1/2 S-T | 69 |
| Vending Machine Area | VCT | RB | GWB |  | AT | 1/2 V-T | 1 |
| Barber Shop | VCT | RB | GWB-W |  | AT | 1/2 S-T | 67 |
| Beauty Shop | VCT | RB | GWB-W |  | AT | 1/2 S-T | 67 |
| Ward Cart Storage | VCT | RB | GWB |  | AT | 11/12 V-W | 69 |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Lockers, Lounges, Toilets and Showers (LLTS) | See Chapter 410 |  |  |  |  |  |  |
| Corridor | VCT | RB | GWB |  | AT | 1/2 QQ-T ADO1 | 62 126 |

See Architectural Design Manual PG-18-10, "Automatic Doors".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Nave | CP | RB | Special1 |  |  | 1/2 SS-T | 86/90 |
| Chancel including Choir and Altar | CP | RB | Special1 |  |  | ----- | ----- |
| Eucharistic Room | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Devotional Room | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Sacristy and Choir Robing | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Offices: |  |  |  |  |  |  |  |
| Office, Chaplains | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Secretary and Waiting | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Clerk | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Toilet Facilities Residents & Visitors: |  |  |  |  |  |  |  |
| Male or Female | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Staff Lounge and Toilets | See Chapter 410 |  |  |  |  |  |  |
| Housekeeping Aids Closets | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Corridors | CP | RB | GWB-W |  | AT | 1/2 VV-T | 62 |

Finishes are to be selected on the basis of project requirements.

2    See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W. NO. |
| Administration: |  |  |  |  |  |  |  |
| Office, Chief, Cardiology | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Chief, Cardiac Catheterization | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Chief, Non-Invasive Cardiology | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Physician | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, Secretary/ Reception | CP | RB | GWB |  | AT | 1/2 S/Open | 23 |
| Waiting | CP | RB | GWB |  | AT | Open | 23 |
| Office, Nurse/ Technician | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Consultation, Viewing, Teaching Room | CP | RB | GWB |  | AT/GWB1 Lay-in Panels in Grid | 1/2 V-T | 23 |
| Educational Facilities: |  |  |  |  |  |  |  |
| Office, Residents/ Fellows | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Student Study Cubicle | CP | RB | GWB |  | AT1 | 1/2 U-T | 17 |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Non-Invasive Cardiovascular Laboratories: |  |  |  |  |  |  |  |
| Electrocardiography (ECG) Computer Service: |  |  |  |  |  |  |  |
| Computer Room & Storage | VCT | RB | GWB |  | AT | 1/2 S | 45 |
| Reading Room | VCT | RB | GWB1 |  | AT | 1/2 S-T | 52 |
| Electrocardiographic Patient Testing: |  |  |  |  |  |  |  |
| ECG Cubicle & Dressing Area | VCT | RB | GWB |  | AT | 1/2 S | 52 |
| Waiting Space | VCT | RB | GWB |  | AT | Open |  |
| Storage Alcove | Match Corridor Finishes |  |  |  |  |  |  |
| Pacemaker/Holter Monitor Technology Room | VCT | RB | GWB |  | AT | 1/2 S-T | 67 |
| Echocardiography Area: |  |  |  |  |  |  |  |
| Exam Room | VCT | RB | GWB1 |  | AT | 1/2 V | 67 |
| Waiting Space | CP | RB | GWB |  | AT | 1/2 V-T | 52 |
| Cardiac Nuclear Imaging | VCT | RB | GWB2 |  | AT2 | 1/2 SS-T | 72 |
| Exercise Room | VCT | RB | GWB |  | AT | 1/2 V | 67 |
| Invasive Cardiovascular Labs and Electrophysiology Study: |  |  |  |  |  |  |  |
| Waiting Space | VCT | RB | GWB |  | AT | 1/2 V-T | 52 |
| Entrance and Patient Holding | VCT | RB | GWB |  | AT | 1/2 V-T | 69 |
| Scrub Room | WSF | WSF | GWB (SC)3 |  | GWB | 15 S-V | 106 |
| Cardiac Catheterization Laboratory: |  |  |  |  |  |  |  |
| Procedures Room | WSF | WSF | GWB (SC)3 |  | GWB | 15 SS-V | 109 |
| Data Gathering Room | WSF | WSF | GWB (SC)3 |  | GWB | 15 S-V | 108 |
| Blood Gas Analysis Laboratory | WSF | WSF | GWB (SC) |  | AT (SP) | 1/2 S-T | 67 |
| Cine Film Viewing and Storage Room | WSF | WSF | GWB |  | AT (SP) | 1/2 S-T | 67 |
| Darkroom & Unexposed Film Storage | WSF | WSF | GWB (SC) |  | GWB | Revolving Dark Room Door |  |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Clean Utility Room | VCT | RB | GWB (SC) |  | AT (SP) | 11/12 V | 45 |
| Soiled Utility Room | RSF | RB | GWB (SC) |  | AT (SP) | 11/12 V | 45 |
| Staff Facilities LLTS | CT | CT | GWB | CT 4'0" | GWB | 1/2 S | 67 |
| Cardiac Catheterization Computer Room | VCT | RB | GWB (SC) |  | AT (SP) | 1/2 S-T | 45 |
| Office, Chief Nurse/ Technician | VCT | RB | GWB (SC) |  | AT (SP) | 1/2 S | 23 |
| Electrophysiology Study: |  |  |  |  |  |  |  |
| Control & Observation Room | WSF | WSF | GWB (SC) |  | GWB | 15 S-T | 108 |
| Procedures Room1 | WSF | WSF | GWB (SC) |  | GWB | 15 SS-T | 72 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T | 62 |

See Standard Details PG-18-4 for shielding.

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |
| Office, Chief | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Office, Secretary/ Waiting | CP | RB | GWB/W |  | AT | 1/2 S-T/ Open | 23 |
| Waiting, Public | CP | RB | GWB/W |  | AT | Open |  |
| Storage, Active Patient's Files | CP | RB | GWB |  | AT | Open |  |
| Viewing Room/Library Audiovisuals | CP | RB | GWB/W |  | AT | 1/2 S-T | 52 |
| Office, Pulmonary Staff Physician | CP | RB | GWB/W |  | AT | 1/2 S | 23 |
| Office, Pulmonary Rehabilitation Nurse/RT (Patient Counseling--Privacy Essential) | CP | RB | GWB/W1 |  | AT1 | 1/2 S | 52 |
| Pulmonary Function Laboratory: |  |  |  |  |  |  |  |
| Ventilatory Test Room - Spirometry | VCT | RB | GWB-W |  | AT | 1/2 V-T | 69 |
| Exercise Room | VCT | RB | GWB |  | AT | 1/2 V-T | 69 |
| Blood Gas Analysis Room | VCT | RB | GWB |  | AT | 1/2 S-T | 69 |
| Storage, Gas Cylinders | VCT | RB | GWB |  | AT | 11/12 S-W | 42 |
| Supply Room, Clean Medical/Surgical | VCT | RB | GWB |  | AT | 1/2 S-T | 42 |
| Supply Room Soiled (Used Medical/ Surgical Supplies) | VCT | RB | GWB |  | AT | 1/2 S-T | 42 |
| Special Procedures or Bronchoscopy Room | RSF | RB | GWB-W |  | AT | 1/2 V-T | 69 |
| Sleep Study Laboratory or Sleep Diagnosis Clinic: |  |  |  |  |  |  |  |
| Patient Bedroom (Single) | VCT | RB | GWB |  | AT | 1/2 S-T | 7 |
| Bathroom, Wheelchair | CT | CT | GWB | CT 4'0" | AT (SP) | 1/2 S | 14 |
| Diagnostic Room – Polysomnography | VCT | RB | GWB |  | AT | 1/2 S-T | 69 |
| Office, Physician - Polysomnographer | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Storage Room | VCT | RB | GWB |  | AT | 1/2 S | 13 |
| Sleep Diagnosis Clinic: |  |  |  |  |  |  |  |
| Waiting, Public | CP | RB | GWB-W |  | AT | Open |  |
| Office, Secretary/ Receptionist | CP | RB | GWB-W |  | AT | 1/2 S-T/Open | 23 |
| Storage, Records (Study Printouts) | VCT | RB | GWB |  | AT | 11/12 V-W | 42 |
| Bedroom, Patient-Four | VCT | RB | GWB |  | AT | 1/2 W | 7 |
| Bathroom, Shared Wheelchair - Two | CT | CT | GWB | CT 4'0" | AT (SP) | 1/2 S | 1 |
| Staff Workroom | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Office, Physician - Polysomnographer | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Patient Preparation Room - with Oxygen and Medical Vacuum | VCT | RB | GWB |  | AT | 1/2 W | 69 |
| Toilet, Staff | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 or 18 |
| Computer Room | CP/ VCT | RB | GWB |  | AT | 1/2 S-T | 45 |
| Respiratory Therapy Unit: |  |  |  |  |  |  |  |
| Office, Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Secretary/ Receptionist | VCT | RB | GWB-W |  | AT | 1/2 S-T/ Open | 23 |
| Office, Chief Therapist | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Storage, Equipment | VCT | RB | GWB |  | AT | 11/12 V-W | 42 |
| Storage, Supplies & Spare Parts | VCT | RB | GWB |  | AT | 11/12 V-W | 42 |
| Clean-up/ Repair/ Testing Room | VCT | RB | GWB |  | AT | 1/2 V-T | 69 |
| Combined Respiratory Therapy Report Room and Pulmonary Function Technician Data Calculation Room | VCT | RB | GWB |  | AT | 1/2 S-T | 52 |
| Staff Lockers - Toilets: | See Chapter 410 |  |  |  |  |  |  |
| Education: |  |  |  |  |  |  |  |
| House Staff | VCT/ CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Study Cubicles | VCT/ CP | RB | GWB-W |  | AT | 1/2 S-T/ Open | 52 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T | 61 |

See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Administrative Space: |  |  |  |  |  |  |  |
| Chief of Service | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Assistant Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Secretary | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Additional Clerk | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Staff Physician | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Interview/Exam Room | VCT | RB | GWB1 |  | AT1 | 1/2 V-T | 23 |
| Chief Resident's Office | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 QQ-T | 62 |

Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Administrative Office: |  |  |  |  |  |  |  |
| Veterans Benefit Counselor | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Clerical & Waiting | VCT/CP | RB | GWB-W |  | AT | 1/2 S-T/Open | 23 |
| Corridor | VCT/CP | RB | GWB |  | AT | 9/10 VV-T | 81 |

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Itinerant Offices | CP | RB | GWB1,2 |  | AT | 1/2 S-T | 52 |
| Full Office | CP | RB | GWB1,2 |  | AT | 1/2 S-T | 157 |
| Corridor | VCT | RB | GWB1 |  | AT | 1/2 QQ-T | 63 |

Full height partition to underside of structure above. See Design and Construction Procedures PG-18-3, Topic 14."Physical Security Requirements and Options".

2 Pass through window wall; CMU. See Design and Construction Procedures PG-18-3, Topic 14."Physical Security Requirements and Options".

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| General Treatment Operatories | RSF | RSF | GWB (SC) |  | AT (SP) |  |  |
| Preventive Dentistry Suite: |  |  |  |  |  |  |  |
| Dental Hygiene Operatories | RSF | RSF | GWB (SC) |  | AT (SP) | 1 |  |
| Preventive Dentistry Room | RSF | RSF | GWB-W |  | AT | 1/2 S-T | 52 |
| Oral Diagnosis Suite: |  |  |  |  |  |  |  |
| Examination Operatories | RSF | RSF | GWB (SC) |  | AT (SP) | 1 |  |
| X-Ray Area | RSF | RSF | GWB (SC) 2 |  | AT | 15 W | 106 |
| Darkroom | RSF | RSF | GWB |  | GWB | 1/2 S-M/ LP Revolving Dark Room Door | 162 |
| Oral Surgery Suite: |  |  |  |  |  |  |  |
| Oral Surgery Room | RSF | RSF | GWB (SC) |  | AT (SP) | 3, 4 | 112 |
| Recovery Room | RSF | RSF | GWB-W |  | AT | 1/2 S-T | 67 |
| Maxillo Facial Suite: |  |  |  |  |  |  |  |
| Maxillo Facial Treatment Rooms | RSF | RSF | GWB (SC) |  | AT (SP) | 1/2 S-T | 67 |
| Maxillo Facial Laboratory | RSF | RSF | GWB (SC) |  | AT | 1/2 S-T | 69 |
| Oral Pathology Lab | RSF | RSF | GWB (SC) |  | AT | 1/2 S-T | 69 |
| Dental Prosthetic Laboratory | RSF | RSF | GWB (SC) |  | AT | 1/2 S-T | 69 |
| Dental Residents & Trainees Study/ Classroom | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Waiting Area | CP | RB | GWB-W |  | AT | 1/2 V-T/ 3/4 V-T | 52 |
| Instrument Prep. & Sterilization Area | RSF | RSF | GWB (SC) |  | AT | 1/2 U-T | 69 |
| Storage Room | VCT | RB | GWB |  | AT | 11/12 SS-W 11/12 V-W | 69 |
| Linen Room & Clean SPD Cart | VCT | RB | GWB (SC) | CT 4'0" | AT | 11/12 V-W | 45 |
| Soiled SPD Cart Holding | RSF | RSF | GWB (SC) | CT 4'0" | AT | 11/12 V-W | 45 |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Patients, Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Staff Lockers, Lounge & Toilets | See Chapter 410 |  |  |  |  |  |  |
| Administrative Offices: |  |  |  |  |  |  |  |
| Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Assistant Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Secretary, Files, & Additional Clerical | VCT | RB | GWB |  | AT | 1/2 S | 23 |
| Patient Interview Room | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Director, Dental Residency Program | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Coordinator, Dental Auxiliary Training | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T | 61 |

See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Administrative Space: |  |  |  |  |  |  |  |
| Office, Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Secretary & Waiting | CP | RB | GWB-W |  | AT | OPEN |  |
| Section Chief, Nutrition Therapy, Education & Research | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Clinical Dietitians | VCT | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Section Chief | VCT | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Chief, Food Production and Service | VCT | RB | GWB |  | AT | 3/4 S-T | 23 |
| Supervisors, Technicians and Clerks | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Food Service Coordination Center | VCT | RB | GWB |  | AT | 3/4 V-T | 52 |
| Serving, Cafeteria Line Unit | QT | QT | CT |  | AT (SP) | 3/4/ V-T | 4 |
| Patient Central Tray Service: |  |  |  |  |  |  |  |
| Tray Make-Up | QT | QT | CT |  | AT (SP) | 3/4 V-T | 4 |
| Food Cart Storage | QT | QT | CT |  | AT (SP) | 3/4 V-T | 4 |
| Dishwashing, Collection and Storage: |  |  |  |  |  |  |  |
| Dishwashing | QT | QT | CT |  | AT (SP) | 4 SS-T | 4 |
| Soiled Dish Collection | QT | QT | CT |  | AT (SP) |  |  |
| Clean Dish Storage | QT | QT | CT |  | AT (SP) |  |  |
| Pot Washing Center | QT | QT | CT |  | AT (SP) |  |  |
| Cart Wash Center | QT | QT | CT |  | PCP | 3/4 V-T | 4 |
| Can Washer and Can Crusher | QT | QT | CT |  | PCP | 1/2 V-T | 4 |
| Refrigerated Garbage Storage and Trash | QT | QT | Special |  |  |  |  |
| Cleaning Equipment Facilities | QT | QT | CT |  | AT (SP) | 2 SS-T | 82 |
| Food Processing and Preparation | QT | QT | CT |  | AT (SP) |  |  |
| Nourishment Preparation | QT | QT | CT |  | AT (SP) | 3/4 V-T or Open | 4 |
| Ingredient Control Area | QT | QT | CT |  | AT (SP) | 3/4 V-T | 4 |
| Housekeeping Aids Closet | QT | QT | CT |  | AT (SP) | 1/2 T | 13 |
| Food Storage Area | QT | QT | GWB |  | AT | 1/2 V-T | 69 |
| Receiving Area | VCT | RB | GWB or CMU (SC) |  | AT | Automatic Roll-up 9'0" x 10'0" |  |
| Nonfood Storage | VCT | RB | GWB or CMU (SC) |  | AT | 1/2 V-T | 69 |
| Sanitation Supply Storage | QT | QT | GWB or CMU (SC) |  | AT | 1/2 V-T | 69 |
| Refrigerated and Frozen Storage, (Preprocessing) | SpecialError: Reference source not found |  |  |  |  |  |  |
| Nutrition Clinic | CP | RB | GWB |  | AT | 1/2 V-T | 52 |
| Corridor | VCT | RB | GWB | CT 5'0" | AT | 1/2 VV-T | 82 |

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Examination, (Patient) Rooms | VCT | RB | GWB1 |  | AT/GWB Panels1 | 1/2 V-M | 52 |
| E.E.G. Instrument and Workroom | VCT | RB | GWB |  | AT | 1/2 U-T | 67 |
| Records Storage | VCT | RB | GWB |  | AT | 1/2 U-T | 13 |
| Physicians Reading Room | CP | RB | GWB |  | AT | 1/2 S-T | 67 |
| Waiting Alcove | VCT | RB | GWB |  | AT | Open |  |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T | 62 |

Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Administrative Space: |  |  |  |  |  |  |  |
| Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Assistant Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Secretary & Waiting | CP | RB | GWB-W |  | AT | 1/2 S-T/Open | 13 |
| Fire & Safety Specialist/ Officer | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Industrial Hygienist | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Other Clerical, Technical or Administrative Employees | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Conference Room | CP | RB | GWB-W |  | AT | 1/2 U-T | 40 |
| Project Engineer | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Hospital Plans & Drafting Room | VCT | RB | GWB |  | AT | 1/2 U-T | 23 |
| Driver Dispatch & Ready Room | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Staff Lockers & Toilets | See Chapter 410 |  |  |  |  |  |  |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T | 6 |
| Biomedical Engineering Repair Shop | VCT | RB | GWB |  | AT | 1/2 SS-T | 82 |
| Shop Areas: |  |  |  |  |  |  |  |
| Carpenter Shop | C |  | CMU |  | AT | 1/2 SS-W | 82 |
| Air Conditioning | C3 |  | CMU |  | AT | 1/2 SS-T | 82 |
| Plumbing Shop | C3 |  | CMU |  | AT | 1/2 SS-T | 82 |
| Electrical Shop | C3 |  | CMU |  | AT | 1/2 SS-T | 82 |
| Paint Shop | C3 |  | CMU |  | AT | 9/10 V-W | 42 |
| Machine Shop | C1 |  | CMU |  | AT | 1/2 SS-T | 82 |
| Mason Shop | C1 |  | CMU |  | AT | 1/2 SS-T | 82 |
| Grounds Maintenance Center | C |  | CMU |  | EXP | Int. 11/12 U-W, Ext. 1/2 SS-T | 62 82 |
| Locksmith Shop | C1 |  | CMU |  | AT | 1/2 V-T | 52 |
| Housekeeping Aids Closet | CT | CT | GWB or CMU (SC) | CT 4'0" | AT(SP) | 1/2 T | 13 |
| Engineering Control Center | VCT | RB | GWB2 |  | AT | 1/2 U-T | 45 |
| Corridor | C1 | RB | CMU or GWB |  | AT | 1/2 VV-T | 63 |

These floors shall be finished with a colored hardener (non-slip).

2 Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Examination Treatment Rooms | VCT | RB | GWB1 |  | AT/GWB Lay-in Panels | 1/2 V | 52 |
| Reception/Clerk/ Files | CP | RB | GWB-W |  | AT | 1/2 S-T/ Open | 23 |
| Patients Waiting | CP | RB | GWB-W |  | AT | Open |  |
| Physician's Office | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Resident's Office | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Audiometry Room and Test Alcove | Prefabricated2 |  |  |  |  |  |  |
| Test Alcove | VCT | RB | GWB-W |  | AT |  |  |
| Sterilization/ Preparation and Storage Room | VCT | RB | GWB |  | AT | 1/2 U/T |  |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T | 63 |

Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Design and Construction Procedures PG-18-3, Topic 6, "Floor Slab Depressions".

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Examination/ Treatment Room | VCT | RB | GWB/W1 |  | AT | 1/2 V | 52 |
| Reception, Clerk, | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Files |  |  |  |  |  |  |  |
| Patients Waiting Area | CP | RB | GWB-W |  | AT | 1/2 S-T/ Open | 23 |
| Visual Fields Room | VCT | RB | GWB |  | AT | 1/2 V | 52 |
| Darkroom Adaptation Room | VCT | RB | GWB |  | AT | 1/2 S-M | 52 |
| Photo Darkroom | VCT | RB | GWB |  | AT | 1/2 S-M | 52 |
| Chief, Ophthalmologist Office/ Consultation Room | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Ophthalmologist Office | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Resident's Office | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T | 62 |

1 Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Office, Chief | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Office, Assistant | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Chief |  |  |  |  |  |  |  |
| Office, Secretary/ Receptionist/ Waiting | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Additional Secretary | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, Budget Analyst | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, Auditor | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Work Area, Finance and Accounting: |  |  |  |  |  |  |  |
| Office, Accounting, Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Work Area, Clerical/ Technical Employee | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Support Area | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Keypunch Operations: |  |  |  |  |  |  |  |
| Keypunch Machine | VCT | RB | GWB |  | AT | 1/2 S-T | 52 |
| Storage Space | VCT | RB | GWB |  | AT | 1/2 S | 13 |
| Office-Travel Unit | CP | RB | GWB |  | AT | 1/2 S-T | 52 |
| Office, Patients Funds | CP | RB | GWB |  | AT | 1/2 S-T | 52 |
| Office, Agent Cashier1 | CP | RB | GWB |  | AT | 36 S | 157 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 QQ-T | 62 |

1 See Design and Construction Procedures PG-18-3, Topic 14, "Physical Security Requirements and Options".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Office, Director | CP | RB | GWB-W |  | AT1 | 1/2 S | 52 |
| Toilet | RSF | RSF | GWB-W |  | AT1 | 1/2 S | 18 or 1 |
| Office, Assistant Director | CP | RB | GWB-W1 |  | AT1 | 1/2 S | 23 |
| Office, Secretary Waiting | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Chief of Staff | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Office, Management Analyst/Special Assistant | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Administrative Assistant | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Trainee | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Conference Space, Administrative | CP | RB | GWB-W1,2 |  | AT | 1/2 U-T | 40 |
| Coatroom | CP | RB | GWB-W |  | AT | 1/2 U-T | 23 |
| Corridor | CP/VCT | RB | GWB-W |  | AT | 1/2 QQ-T/ Storefront | 137 |

1 See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission   Control".

2 Provide tackable surface on wall.

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administrative: |  |  |  |  |  |  |  |
| Chief Lab Service Office and Microscope Area | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Staff Pathologist Office and Microscope Area | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Secretary to Chief | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Clerk, Steno/ Receptionist | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Admin. Assistant | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Chief Medical Technologist | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Clinical Laboratory Scientist | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Education Area: |  |  |  |  |  |  |  |
| Pathology Resident, Office and Microscope Area | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Medical Tech. Program: |  |  |  |  |  |  |  |
| Office, Education Coordinator | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Student Lab/Class Room | VCT | RB | GWB |  | AT | 1/2 V-T | 23 |
| Student Study Cubicle | CP | RB | Prefab |  | AT/GWB Lay-in Panels in Grid |  |  |
| Conference Room | CP | RB | GWB1,2 |  | AT | 1/2 V-T | 40 |
| Laboratory Work Areas: |  |  |  |  |  |  |  |
| Specimen Accessioning, Processing, and Distribution Room | VCT | RB | GWB |  | AT (SP) | 1/2 V-T | 67 |
| Immediate Response (STAT) Lab | VCT | RB | GWB (SC) |  | AT (SP) | 1/2 V-T | 67 |
| ADP Control Area | VCT/CP | RB | GWB |  | AT | 1/2 V-T | 52 |
| Preprocessor Area | VCT/CP | RB | GWB |  | AT | 1/2 S | 52 |
| CRTs, and Printers | VCT/CP | RB | GWB |  | AT | Open |  |
| Clinical Pathology: |  |  |  |  |  |  |  |
| Clinical Chemistry | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Special Chemistry | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Radioimmunoassay | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Urine and Feces Lab | RSF | RB | GWB |  | AT | 1/2 V-T | 67 |
| Hematology: |  |  |  |  |  |  |  |
| Routine Lab | RSF | RB | GWB |  | AT | 1/2 V-T | 67 |
| Coagulation Lab | RSF | RB | GWB |  | AT | 1/2 V-T | 67 |
| Blood Bank: |  |  |  |  |  |  |  |
| Blood Product Storage, Testing and Issuance | RSF | RB | GWB |  | AT | 1/2 V-T | 76 |
| Blood Product Prep and Modification | RSF | RB | GWB |  | AT | 1/2 V-T | 67 |
| Donor Hemotherapeutics Room | RSF | RB | GWB |  | AT | 1/2 V-T | 52 |
| Microbiology: |  |  |  |  |  |  |  |
| Clinical Microbiology | VCT | RB | GWB |  | AT | 3/4 V-T | 67 |
| Biosafety Lab | VCT | RB | GWB |  | AT | 3/4 V-T | 67 |
| Mycology Lab | VCT | RB | GWB |  | AT | 3/4 V-T | 67 |
| Mycobacteriology Lab | VCT | RB | GWB |  | AT | 3/4 V-T | 67 |
| Media Preparation Room | CT | CT | CT |  | AT (SP) | 3/4 V-T | 67 |
| Immunopathology | VCT | RB | GWB |  | AT | 3/4 V-T | 67 |
| Fluorescent Microscopy Room | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Anatomic Pathology: |  |  |  |  |  |  |  |
| Histology | VCT | RB | GWB(SC) |  | AT (SP) | 3/4 V-T | 67 |
| Frozen and Gross Section Lab | VCT | RB | GWB(SC) |  | AT (SP) | 3/4 V-T | 67 |
| Cytology | VCT | RB | GWB(SC) |  | AT (SP) | 1/2 V-T | 67 |
| Slide, Block and Record Files | VCT | RB | GWB |  | AT | 1/2 V-T | 52 |
| Electron Microscope Suite: |  |  |  |  |  |  |  |
| Scope Room | VCT | RB | GWB |  | GWB | 1/2 V-M | 67 |
| Dark Room | LM | LM | GWB |  | GWB | 1/2 S-M/ Revolving Door | 67 |
| Preparation Room | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Cutting Room | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Developing, Printing, and Enlarging Dark Room | LM | LM | GWB |  | GWB | 1/2 S-M / Revolving Door | 67 |
| Finishing Room | VCT | RB | GWB |  | AT | 1/2 S | 67 |
| Autopsy Suite: |  |  |  |  |  |  |  |
| Autopsy Room | LM | LM | GWB(SC) |  | GWB (SC) | 1/2 QQ-T | 82 |
| Isolation/Teaching Room | LM | LM | GWB(SC) |  | GWB (SC) | 1/2 V-T | 69 |
| Mortuary Refrigerator (Cold Room) | QT | QT | Prefabricated, |  |  |  |  |
| Toilets, Lockers, Showers | See Chapter 410 |  |  |  |  |  |  |
| Gross Specimen Storage | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Housekeeping Aids Closet | CT | CT | GWB(SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Patient Specimen Collection Area: |  |  |  |  |  |  |  |
| Patient Waiting Area | RSF | RSF | GWB-W |  | AT | 1/2 V-T / 3/4 V-T | 52 |
| Blood Specimen Collection Room | VCT | RB | GWB |  | AT | 1/2 V-T | 52 |
| Urine Specimen Collection Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 V-T | 14 |
| LLTS: |  |  |  |  |  |  |  |
| Employee Biosafety Room (Lounge) | CP/VCT | RB | GWB |  | AT | 3/4 T | 1 |
| Lockers and Toilets | See Chapter 410 |  |  |  |  |  |  |
| Sterilization and Solution Prep. Room | CT | CT | CT |  | AT (SP) | 1/2 V-T | 69 |
| Bulk Storage | VCT | RB | GWB |  | AT | 11/12 SS-W/ 11/12 V-W | 60 42 |
| Refrigerated Storage (Prefab)1 | Prefabricated |  |  |  |  |  |  |
| Flammable and Toxic Substance Storage | VCT | RB | GWB2 |  | GWB | 11/12 V-T | 42 |
| Housekeeping Aids Closet | CT | CT | GWB(SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Corridors | VCT | RB | GWB |  | AT | 1/2 VV-T | 63 |

Prefabricated controlled temperature room.

2 Provide 1-hour rated enclosure and 6" perimeter concrete curb cast integrally with   floor in accordance with NFPA.

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Lobby Area | Finish to be selected on the basis of requirements and cost. |  |  |  |  |  |  |
| Public Toilets | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 14 for single use - 1 for multi-use |

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAINS | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration Offices: |  |  |  |  |  |  |  |
| Chief, MAS | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Assistant Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Secretary and | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Waiting | CP | RB | GWB-W |  | AT | Open |  |
| Decedent Affairs | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Processing Unit: |  |  |  |  |  |  |  |
| Clerk | CP | RB | GWB |  | AT | 1/2 S-T | 52 |
| Fee Service Section: |  |  |  |  |  |  |  |
| Supervisors Office | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| General Office Space | CP | RB | GWB |  | AT | 3/4 S-T | 52 |
| Medical Information Section: |  |  |  |  |  |  |  |
| Medical Records Administrator’s Office | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Transcription Unit: |  |  |  |  |  |  |  |
| Supervisor's Office | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Transcriptionists | CP | RB | GWB |  | AT | 3/4 S-T | 52 |
| General Office Space | CP | RB | GWB |  | AT | 3/4 S-T | 52 |
| Physicians Workroom (FTE Serving In-Patients) | CP | RB | GWB |  | AT | 1/2 S-T | 52 |
| Record Review Room | CP | RB | GWB |  | AT | 3/4 S-T | 52 |
| Ward Administration Section: |  |  |  |  |  |  |  |
| Supervisor's Office | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Ward or Unit Manager's Office | CP | RB | GWB |  | AT | 1/2 S-T | 52 |
| Ward Clerk | See Nursing Station for Individual Areas |  |  |  |  |  |  |
| Office Operation Section: |  |  |  |  |  |  |  |
| Supervisors Office | CP | RB | GWB-W |  | AT | 3/4 S-T | 52 |
| Teletypist Office | CP | RB | GWB |  | AT | 3/4 S-T | 52 |
| Reproduction Room | CP | RB | GWB |  | AT | 3/4 U-T | 52 |
| Mail Room and Distribution Center | CP | RB | GWB |  | AT | 3/4 U-T | 69 |
| Information Counter Area | CP | RB | GWB |  | AT | Open |  |
| Telephone Switchboard: |  |  |  |  |  |  |  |
| Switchboard | CP | RB | GWB |  | AT | 1/2 S | 43 |
| Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Medical Records Storage and Workroom: |  |  |  |  |  |  |  |
| General/Psychiatric Hospitals | VCT | RB | GWB |  | AT | 11/12 SS-W | 62 |
| Ambulatory Care/ Satellite Out-Patient Facility | VCT | RB | GWB |  | AT | 11/12 SS-W | 62 |
| Working Space | VCT | RB | GWB |  | AT | 3/4 U-T | 52 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 QQ-T | 61 |

See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |
| Office, Chief | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Reception & Waiting | CP | RB | GWB |  | AT | Open |  |
| Audiovisual (AV) Equipment Storage/ Check-Out Room | VCT | RB | GWB |  | AT | 11/12 V-W | 42 |
| Photography: |  |  |  |  |  |  |  |
| Photo Finishing Area | VCT | RB | GWB |  | GWB | 1/2 S-M | 67 |
| Darkroom Area | VCT | RB | GWB |  | GWB | 1/S M Liteproof |  |
| Darkroom, Black and White/Color Processing | VCT | RB | GWB |  | GWB | 1/S M Liteproof |  |
| Darkroom Printing and Enlarging | VCT | RB | GWB |  | GWB | 1/S M Liteproof |  |
| Camera Copy Room | VCT | RB | GWB |  | GWB | 1/2 X-M | 67 |
| Photomicrography/ Photomacrography Room | VCT | RB | GWB |  | GWB | 1/2 S-M Liteproof | 67 |
| Still Photography and Video Recording Studio | VCT | RB | GWB |  | GWB/EXP | 1/2 VV-M | 82 |
| Toilet/Changing Room | CT | CT | GWB |  | AT | 1/2 S | 18 |
| Storage | VCT | RB | GWB |  | AT | 11/12 V-W | 42 |
| Expanded Core Requirements for MMS: |  |  |  |  |  |  |  |
| Work Station, Secretary/Clerk | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Illustration Services: |  |  |  |  |  |  |  |
| Workroom; Illustrator | VCT | RB | GWB |  | AT | 1/2 S-T | 67 |
| Illustration Preparation Room: |  |  |  |  |  |  |  |
| Illustrator | VCT | RB | GWB |  | AT | 1/2 S-T | 67 |
| Work Station, Illustration Desktop Publishing | VCT | RB | GWB |  | AT | 1/2 S-T | 67 |
| Stat Camera Room | VCT | RB | GWB |  | AT | 1/2 S-M | 67 |
| Audiovisual (AV) Services: |  |  |  |  |  |  |  |
| Video Editing/(CCTV) | VCT | RB | GWB |  | AT | 1/2 V-M | 67 |
| Control Suite: |  |  |  |  |  |  |  |
| Storage, Props | VCT | RB | GWB |  | AT | 11/12 V-W | 67 |
| Client Review Room | VCT | RB | GWB |  | AT | 1/2 V-W | 42 |
| Computer Imaging System Network (CISN) Room | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Learning Resources Production Service (LRPS) Administration: |  |  |  |  |  |  |  |
| Office, Chief Media Service | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Chief of Learning Resources | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Reception Area | CP | RB | GWB-W |  | AT | Open |  |
| Workstation, Clerk/ Secretary | CP | RB | GWB |  | AT | Open |  |
| Audiovisual (AV) Equipment Storage/ Check-Out Room | VCT | RB | GWB |  | AT | 11/12 W-T | 43 |
| Photography: |  |  |  |  |  |  |  |
| Photo Finishing Area | VCT | RB | GWB |  | GWB | 1/2 S-M | 67 |
| Darkroom Area | VCT | RB | GWB |  | GWB | 1/ 2 S-M Liteproof |  |
| Camera Copy Room | VCT | RB | GWB |  | GWB | 1/2 W-M | 67 |
| Photomicrography/ Photomacrography Room | VCT | RB | GWB |  | GWB | 1/2 S-M Liteproof | 67 |
| Photo Studio | VCT | RB | GWB |  | GWB | 1/2 S-M | 67 |
| Toilet/Changing Room | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 67 |
| Storage | VCT | RB | GWB |  | AT | 11/12 V-W | 45 |
| Illustration Service: |  |  |  |  |  |  |  |
| Workroom, Illustrators | VCT | RB | GWB |  | AT | 1/2 S-T | 67 |
| Illustration Preparation | VCT | RB | GWB |  | AT | 1/2 S-T | 67 |
| Workstation Illustration Desk-Top Publishing | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Stat Camera Room | VCT | RB | GWB |  | AT | 1/2 S-T | 67 |
| Audiovisual (AV) Services: |  |  |  |  |  |  |  |
| Recording Studio | CP | RB | GWB1,2 |  | AT/EXP | 19/20 VV-M | 82 |
| Actor Dressing & Locker Room | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Toilet, Wheelchair | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Recording & Control Room | CP | RB | GWB2 |  | AT | 19/20 S-M | 67 |
| Off-Line Editing Room | CP | RB | GWB1 |  | AT | 1/2 W | 67 |
| Audio Recording and Narration Sound Room | CP | RB | GWB2 |  | AT | 19/20 W-M | 67 |
| Audiovisual (AV) Production Maintenance Room | VCT | RB | GWB |  | AT | 1/2 V-T | 43 |
| Visual Production Graphics Room | VCT | RB | GWB |  | AT | 1/2 V-T | 43 |
| AV Storage Room | VCT | RB | GWB |  | AT | 11/12 V-W | 43 |
| Office, Director of Production | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Video Production Staff | VCT | RB | GWB |  | AT | 1/2 S | 23 |
| Client Review Room | VCT | RB | GWB |  | AT | 1/2 S | 23 |
| Computer Imaging System Network (CISN) Room | VCT | RB | GWB |  | AT | 1/2 S-T | 67 |
| Corridors | VCT | RB | GWB |  | AT | 1/2 VV-T | 61 |

See Section II, "Doors and Hardware"

2 Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |
| Patient Treatment Files | VCT | RB | GWB |  | AT | 11/12 U-W | 45 |
| Office, Radiation Safety/ Health Physics | WSF | WSF | GWB |  | AT | 1/ 2 U-W | 45 |
| Education: |  |  |  |  |  |  |  |
| Nuclear Medicine Technology: |  |  |  |  |  |  |  |
| Student Laboratory/ Classroom | VCT | RB | GWB |  | AT | 1/2 U-T | 52 |
| Work Areas: |  |  |  |  |  |  |  |
| Imaging Unit: |  |  |  |  |  |  |  |
| Imaging Room | WSF | WSF | GWB |  | AT | 1/2 W | 69 |
| Computer (Image Processing Area) | VCT | RB | GWB |  | AT | 1/2 U-T | 52 |
| Data Processing Equipment | VCT | RB | GWB |  | AT | 1/2 V-T | 52 |
| Darkroom | LM | LM | GWB |  | GWB | 1/2 S-M or Revolving Darkroom Door | 162 |
| Lightroom | LM | LM | GWB |  | AT | 1/2 S | 69 |
| Reading and Consultation Room | CP | RB | GWB-W |  | AT1 | 1/2 S-T | 52 |
| Patient Examination | VCT | RB | GWB1 |  | AT/GWB1 Lay-in Panels | 1/2 U | 69 |
| Patient Dose Administration | WSF | WSF | GWB |  | AT | 1/2 U | 69 |
| Radiopharmacy | WSF | WSF | GWB |  | AT | 11/12 U-W | 45 |
| Non-Imaging Procedure Room | WSF | WSF | GWB |  | AT | 1/2 W | 69 |
| Ultrasound | VCT | RB | GWB |  | AT | 1/2 W | 69 |
| Patient Toilet (Contiguous w/ Ultrasound Room) | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 14 |
| Radioimmunoassay Unit: |  |  |  |  |  |  |  |
| Receiving, Preparation, Data Reduction and Reporting | WSF | WSF | GWB |  | AT | 1/2 W-T | 69 |
| Biohazard Prep. | WSF | WSF | GWB |  | AT | 1/2 W-T | 69 |
| Assay Preparation | WSF | WSF | GWB |  | AT | 1/2 W-T | 69 |
| Sample Counting, Measurement | WSF | WSF | GWB |  | AT | 1/2 W-T | 69 |
| Support Areas: |  |  |  |  |  |  |  |
| Equipment Storage Mobile | VCT | RB | GWB |  | AT | 1/2 U-T | 69 |
| Storage, Refrigerated | Prefabricated1 |  |  |  |  |  |  |
| Patient Film Records | VCT | RB | GWB |  | AT | 11/12 U-W | 45 |
| Storage, Radionuclide Waste | LM | LM | CMU (SC) |  | AT | 1/2 U | 34 |
| Equipment | VCT | RB | GWB |  | AT | 1/2 U-T | 34 |
| Staff Facilities: |  |  |  |  |  |  |  |
| Employee Bio Safety Room (Lounge) | VCT | RB | GWB (SC) |  | AT | 1/2 S-T | 52 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T | 62 |

See HVAC Design Manual PG-18-10, Article 4.11, "Walk-in Refrigerator".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |
| Secretary & Waiting | CP | RB | GWB-W |  | AT | 1/2 S/ Open | 23 |
| Clerical | CP | RB | GWB-W |  | AT | 1/2 S-T/ Open | 23 |
| Assistant Chief | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Supervisor, Clinical Service | VCT | RB | GWB-W |  | AT | 1/2 S | 23 |
| Administrative Officer | VCT | RB | GWB-W |  | AT | 1/2 S | 23 |
| Administrative Conference Room | CP | RB | GWB, |  | AT1 | 1/2 U-T | 40 |
| Education & Training: |  |  |  |  |  |  |  |
| Assoc. Chief Nurse for Education | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Secretary & Files | CP | RB | GWB/W |  | AT | 1/2 S/ Open | 23 |
| Instructor | CP | RB | GWB/W |  | AT | 1/2 S | 23 |
| Storage Room | VCT | RB | GWB |  | AT | 11/12 SS-W/ 11/12 V-W | 69 |
| Trainees: |  |  |  |  |  |  |  |
| Chief Nurse Trainee | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Resident Intern | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Study Cubicle, Grad Nurse | CP | RB | GWB/W |  | AT | 1/2 U-T | 17 |
| Classrooms | CP | RB | GWB |  | AT | 1/2 U-T | 40 |
| Lockers, Lounges, & Toilets | See Chapter 410 |  |  |  |  |  |  |
| Corridor | VCT | RB | GWB |  | AT | 1/2 QQ-T | 61 |

See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |
| Office, Chief | CP | RB | GWB |  | AT | 1/2 S | 52 |
| Office, Secretary | CP | RB | GWB |  | AT | 1/2 S-A | 52 |
| Office, Clerk | CP | RB | GWB |  | AT | 1/2 S-A Open | 52 |
| Office, Counseling Staff | CP | RB | GWB1 |  | AT1 | 1/2 S | 52 |
| Waiting Space | CP | RB | GWB-W |  | AT | Open |  |
| Examination and Treatment Room | VCT | RB | GWB1 |  | AT1 | 1/2 U | 52 |
| Biofeedback Lab: |  |  |  |  |  |  |  |
| Patient Treatment Room | VCT | RB | GWB1 |  | AT1 | 1/2 U | 52 |
| Control Room/Office | VCT | RB | GWB1 |  | AT1 | 1/2 U | 52 |
| Group Therapy Space | VCT | RB | GWB1 |  | AT1 | 1/2 SS-A | 62 |
| Toilet (Wheelchair Accessible) Male and Female use. | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 2 34 for single |
| Corridor | VCT | RB | GWB |  | AT | 1/2 QQ-T | 62 |

Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |
| Office, Chief | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Office, Secretary/ Reception | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Clerk | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Counseling Staff | CP | RB | GWB |  | AT | 1/2 S-T | 52 |
| Waiting Space | CP | RB | GWB-W |  | AT | Open |  |
| Examination Treatment Room | VCT | RB | GWB1 |  | AT/GWB1 Lay-in Panels in Grid | 1/2 S | 52 |
| Biofeedback Lab.: |  |  |  |  |  |  |  |
| Patient Treatment Room | VCT | RB | GWB1 |  | AT1 | 1/2 S | 52 |
| Control Room/Office | VCT | RB | GWB1 |  | AT1 | 1/2 S | 45 |
| Group Therapy Room | CP | RB | GWB-W1 |  | AT1 | 1/2 U-M | 52 |
| Social Activities/ Dining/Multipurpose | VCT | RB | GWB |  | AT | 1/2 SS-T | 81 |
| Storage | VCT | RB | GWB |  | AT | 11/12 S-W | 43 |
| Kitchen | VCT | RB | GWB-W |  | AT (SP) | 1/2 V-T | 69 |
| Education Therapy | VCT | RB | GWB |  | AT | 1/2 U-T | 52 |
| Compensated Work Therapy | VCT | RB | GWB |  | AT | 1/2 SS-T | 81 |
| Occupational Therapy: |  |  |  |  |  |  |  |
| Basic Clinic | VCT | RB | GWB |  | AT | 1/2 SS-T | 52 |
| Storage | VCT | RB | GWB |  | AT | 11/12 S-W | 43 |
| Toilet Facilities (Wheelchair Access) | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 2 or 14 |
| Corridor | CP | RB | GWB |  | AT | 1/2 QQ-T | 62 |

Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Reception Area: |  |  |  |  |  |  |  |
| Waiting Area | CP | RB | GWB-W |  | AT | 1/2 S-T/ 3/4 V-T | 52 |
| Toilet, Public (V&P) | CT | CT | GWB | CT | AT | 1/2 S | 1/13 |
| Storage, Wheelchair and Stretcher | VCT | RB | GWB |  | AT | 11/12 S-W | 69 |
| Escort/Messenger Service Room | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Vital Sign Station | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| MAS Reception Control/Processing Unit: |  |  |  |  |  |  |  |
| Reception Counter | CP | RB | GWB-W |  | AT | Open |  |
| Clerical Work Station/Interview Booths | CP | RB | GWB |  | AT | Open |  |
| Office, MAS Supervisor | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Bed Control | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Health Benefits Advisor | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Emergency Care (ER): |  |  |  |  |  |  |  |
| Emergency Waiting | VCT | RB | GWB |  | AT | Open |  |
| Toilet, Public (Visitor & Patient) | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Storage, Wheelchair and Stretcher | VCT | RB | GWB |  | AT | 11/12 S-W | 69 |
| Security Police Alcove | VCT | RB | GWB |  | AT | Open | --- |
| Nurse Communication Station | CP | RB | GWB |  | AT | Open | --- |
| Medication Room | VCT | RB | GWB |  | AT | 1/2 S | 43 |
| Clean Utility/ Medical Equipment Storage Room | VCT | RB | GWB (SC) |  | AT (SP) | 11/12 V-W | 43 |
| Soiled Utility Room | RSF | RB | GWB (SC) |  | AT (SP) | 11/12 V-W | 43 |
| Office, Head Nurse | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Chief of ER/ Physician | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Secretary | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Life Support Unit | RSF | RB | GWB (SC) |  | GWB (SC) | 1/2 QQ-T /X (ADO) | 79 |
| Observation and Treatment Room | VCT | RB | GWB (SC) |  | AT (SP) | 1/2 QQ-T /X (ADO)1 | 120 |
| Multi-Purpose Exam | RSF | RSF | GWB (SC)2 |  | GWB | 1/2 S | 69 |
| Toilet/Bath, Wheelchair | CT | CT | GWB3 | CT 4'0" | AT | 1/2 S | 18 |
| Security Exam Room | VCT | RB | CMU or PCP4 |  | PCP | 1/2 S-M-D | 42 |
| Toilet Room | CT | CT | CMU or PCP4 | CT 4'0" | PCP | 1/2 S | 42 |
| On-Call Room | See Chapter 274 |  |  |  |  |  |  |
| Staff Lockers/ Lounges | VCT | RB | GWB |  | AT | 1/2 S | 7 |
| Toilet Staff | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Examination/Treatment Modules: |  |  |  |  |  |  |  |
| Multi-Purpose Exam Room | VCT | RB | GWB2 |  | GWB Lay-in Panels in Grid2 | 1/2 S | 69 |
| Special Purpose Exam Room | VCT | RB | GWB |  | GWB Lay-in Panels in Grid2 | 1/2 S | 69 |
| MAS Reception/ Control Unit | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Waiting Area | VCT/CP | RB | GWB-W |  | AT | Open |  |
| Toilet, Public (Visitor & Patient) | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Consultation Room | CP | RB | GWB |  | GWB Lay-in Panels in Grid1 | 1/2 S-T | 23 |
| Staff/Patient Multi-Purpose Conference/ Class Room | VCT | RB | GWB1 |  | AT | 1/2 V-T | 23 |
| Nurse Station Area: |  |  |  |  |  |  |  |
| Communication Center | VCT/CP | RB | GWB-W |  | AT | Open |  |
| Medication Room | VCT | RB | GWB |  | AT | 1/2 S | 43 |
| Staff Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Intake/Exit Interview Room | VCT/CP | RB | GWB1 |  | GWB Lay-in Panels in Grid1 | 1/2 S-T | 23 |
| Clean Utility/Linen, Stretcher, and Medical Equip. Storage | VCT | RB | GWB (SC) |  | AT | 11/12 V-W | 45 |
| Soiled Utility Room | RSF | RB | GWB (SC) |  | AT | 11/12 V-W | 45 |
| Multi-Purpose Procedure Room | VCT | RB | GWB1 |  | AT1 | 1/2 V | 69 |
| Toilet, Wheelchair | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Exam/Treatment Module Support Space MAS Clerical Support: |  |  |  |  |  |  |  |
| Agent Orange/POW Clerk | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office/MAS Supervisory | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Health Benefits Advisor | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Scheduling | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Compensation and Pension Clerk(s) | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, Exam Room, Physician | CP | RB | GWB-W1 |  | GWB Lay-in Panels in Grid1 | 1/2 S | 23 |
| Office/Exam Room, Physician Extender | CP | RB | GWB1 |  | GWB Lay-in Panels in Grid1 | 1/2 S | 69 |
| Prescription Receiving Room | CP/VCT | RB | GWB |  | AT |  |  |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Exam/Treatment Module Specialty Clinic Space: |  |  |  |  |  |  |  |
| Gynecology Clinic: |  |  |  |  |  |  |  |
| Dressing/Toilet Area | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Genito-Urinary (GU)/Urology Clinic: |  |  |  |  |  |  |  |
| Toilet, Urine Specimen Collection | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Urine Testing Alcove | VCT | RB | GWB |  | AT | Open | --- |
| Gastroenterology Clinic: |  |  |  |  |  |  |  |
| Screening Proctoscopy/ Sigmoidoscopy | RSF | RSF | GWB |  | GWB Lay-in Panels in Grid1 | 1/2 S | 69 |
| Toilet, Wheelchair | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Gastric (Esophageal) Motility Procedure Room | RSF | RSF | GWB |  | AT | 1/2 S | 18 |
| Toilet, Wheelchair | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Dermatology Clinic: |  |  |  |  |  |  |  |
| Dermatology Procedures Treatment Room | VCT | RB | GWB |  | AT1 | 1/2 S | 23 |
| Dermatology Laboratory | VCT | RB | GWB |  | AT | 1/2 S | 23 |
| Dermatology Medication Preparation Storage Room | VCT | RB | GWB |  | AT | 1/2 S | 69 |
| Phototherapy Treatment Room | RSF | RSF | GWB |  | AT | 1/2 S | 69 |
| Phototherapy Shower Room | CT | CT | CT2 |  | PCP | 1/2 S | 18 |
| MOH's Chemosurgery (Micro Controlled Surgery): |  |  |  |  |  |  |  |
| Reception & Waiting | CP | RB | GWB |  | AT | Open |  |
| Exam/Consult Room | VCT | RB | GWB2 |  | GWB Lay-in Panels in Grid | 1/2 S | 69 |
| Procedure Room | RSF | RSF | GWB (SC) |  | AT (SP) | 1/2 S | 29 |
| Laboratory | VCT | RB | GWB |  | AT | 1/2 S-T | 29 |
| Orthopedic Clinic: |  |  |  |  |  |  |  |
| Cast Room | RSF | RSF | GWB |  | AT | 1/2 V | 29 |
| Oncology Clinic: |  |  |  |  |  |  |  |
| Chemotherapy Treatment Room | RSF | RSF | GWB |  | AT (SP) | 1/2 V | 29 |
| Toilet, Wheelchair | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Chemotherapy Agent Preparation Room | RSF | RSF | GWB |  | AT (SP) | 1/2 S-T | 23 |
| Office, Data Coordinator Tumor Registry | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Employee Health Unit Employee Health Office: |  |  |  |  |  |  |  |
| Employee Health Nurse | VCT/CP | RB | GWB |  | AT | 1/2 S | 23 |
| Employee Health Clerk | VCT/CP | RB | GWB |  | AT | 1/2 S | 23 |
| Storage, Employees Health Records | VCT/CP | RB | GWB |  | AT | 1/2 S | 23 |
| Waiting Area | VCT/CP | RB | GWB-W |  | AT | Open |  |
| Exam/Treatment Room | VCT | RB | GWB |  | GWB Lay-in Panels in Grid1 | 1/2 S | 69 |
| Medication Storage/ Preparation | VCT | RB | GWB |  | AT | 1/2 S | 69 |
| Disposition Area: |  |  |  |  |  |  |  |
| Waiting Area | VCT/CP | RB | GWB-W |  | AT | Open |  |
| Toilet, Public (V&P) | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Medical Administration: |  |  |  |  |  |  |  |
| Office, MAS Supervisor, Disposition | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Coding Clerk | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Patients Funds Clerk | VCT/CP | RB | GWB2 |  | AT | 1/2 S-T | 23 |
| Office, Prosthetic Clerk | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Clothing Room Clerk | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Billing Clerk | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Community Nursing Home Clerk | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Details Clerk | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Grieving Room | CP | RB | GWB1 |  | AT | 1/2 S-T | 23 |
| Toilet, Wheelchair | CT | CT | GWB-W | CT 4'0" | AT | 1/2 S | 14 |
| Office, Health Benefits Advisor | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Travel Clerk | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Agent Cashier | VCT | RB | GWB |  | AT1 | 36 S | 157 |
| Outpatient Pharmacy | See Chapter 268 |  |  |  |  |  |  |
| Office, Dietitian/ Nutrition Clinic | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Hospital Based Home Care (HBHC): |  |  |  |  |  |  |  |
| Office, HBHC Coordinator | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, Secretary & Records | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Private Interview Room | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, HBHC Staff | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, Social Worker | CP | RB | GWB2 |  | AT | 1/2 S | 23 |
| Office, Public Health Nurse | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Ambulatory Care Support: |  |  |  |  |  |  |  |
| Satellite Radiology Suite | See Chapter 276 |  |  |  |  |  |  |
| Specimen Collection Area | See Chapter 240 |  |  |  |  |  |  |
| Ambulatory Surgery | See Chapter 286 |  |  |  |  |  |  |
| Diagnosis Testing Functions: |  |  |  |  |  |  |  |
| ECG Testing | See Chapter 210 |  |  |  |  |  |  |
| Ventilator Testing | See Chapter 212 |  |  |  |  |  |  |
| Hearing/Audiometry Testing | See Chapter 204 |  |  |  |  |  |  |
| EEG Testing | See Chapter 226 |  |  |  |  |  |  |
| EMG Testing | See Chapter 270 |  |  |  |  |  |  |
| Vending Machine Area | See Project Requirements |  |  |  |  |  |  |
| Staff Lockers, Lounges & Toilets | See Chapter 410 |  |  |  |  |  |  |
| Staff Offices: |  |  |  |  |  |  |  |
| Administration: |  |  |  |  |  |  |  |
| Office-ACOS Ambulatory Care | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Secretary/ Clerk | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Admin. Asst. to ACOS | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Ambulatory Care Coordinator | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Nursing Support for Ambulatory Care | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Chief MAS for Ambulatory Care | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Professional: |  |  |  |  |  |  |  |
| Office, Chief of Clinic Section | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Physician | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Education: |  |  |  |  |  |  |  |
| Office, Psychology/ Social Work Trainees | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Other Resident Physicians | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Study Cubicles for Student/Trainees | CP | RB | GWB1 |  | AT | 1/2 U-T | 17 |
| Multi-Purpose Conference/ Classroom | VCT | RB | GWB1 |  | AT/GWB1 Lay-in Panels in Grid | 1/2 U-T | 40 |
| Corridors | VCT | RB | GWB |  | AT | 1/2 QQ-T | 62 |

Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Office, Chief | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Office, Secretary | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Waiting Space | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Office, Asst. Chief | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Personnel Management Specialist | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Open Office Space | CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Combination Conf. Room/Placement and Exam Room | CP | RB | GWB1,2 |  | AT | 1/2 U-T | 52 |
| Toilet Facilities | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Storage Area | CP | RB | GWB |  | AT | 1/2 S | 13 |
| Active Personnel Records File Area | CP | RB | GWB |  | AT | 1/2 S-T | 13 |
| Corridor | CP | RB | GWB |  | AT | 1/2 QQ-T | 63 |

Provide acoustical/tackable wall fabric on one wall.

2 Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |
| Office - Chief | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Office - Secretary/ Waiting | CP | RB | GWB/W |  | AT | 1/2 S | 23 |
| Office - Assistant/ Associate Chief | CP | RB | GWB/W |  | AT | 1/2 S | 23 |
| Office - Supervisor Pharmacist | CP | RB | GWB/W |  | AT | 1/2 S | 23 |
| Office - Clerical | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Inpatient Pharmacy |  |  |  |  |  | 36 S | 157 |
| Dispensing Station | CP | RB | GWB |  | AT | --- |  |
| Unit Dose | CP | RB | GWB |  | AT | --- |  |
| Ward Stock or Automatic Replenisher | CP | RB | GWB |  | AT | --- |  |
| Medication Assignment Area | CP | RB | GWB |  | AT | --- |  |
| Controlled Substance Vault and Secured Dispensing Area |  |  |  |  |  | 30 U1 | 157 |
| Vault1 | CP | RB | GWB1 |  | AT1 | --- |  |
| Secured Dispensing1 | CP | RB | GWB |  | AT1 | --- |  |
| Storage, Flammable | VCT | RB | GWB |  | AT | 11/12 S | 157 |
| Receiving Area |  |  |  |  |  | 36 V1 | 157 |
| Breakdown and Verification Area | VCT | RB | GWB |  | AT | --- |  |
| Inventory Control/ Stock Management Clerk | VCT | RB | GWB |  | AT | --- |  |
| Extemporaneous Repackaging | VCT/CP | RB | GWB |  | AT | --- |  |
| Extemporaneous Compounding | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 67 |
| Stat Counter | VCT/CP | RB | GWB |  | AT | --- |  |
| Intravenous Admixture and Aseptic Transfer Room | CT | CT | GWB |  | AT (SP) | 1/2 V-T | 67 |
| Freezer Space | Prefabricated |  |  |  |  |  |  |
| Storage Sterile Fluids and Admixture Sets | VCT | RB | GWB |  | AT | 11/12 V-W | 45 |
| Refrigeration/ Freezer Area | Prefabricated |  |  |  |  |  |  |
| Storage, Active | VCT | RB | GWB |  | AT | --- |  |
| Storage, Cart | VCT | RB | GWB |  | AT | --- |  |
| Storage, Poison Control | VCT | RB | GWB |  | AT | --- |  |
| Outpatient Pharmacy: |  |  |  |  |  |  |  |
| Prescription Receiving | CP | RB | GWB1 |  | AT | 36 V | 157 |
| Filing and Assembly | CP | RB | GWB |  | AT | Open |  |
| Dispensing | CP | RB | GWB1 |  | AT | Open |  |
| Active Storage | CP | RB | GWB |  | AT | Open |  |
| Prepackaging | CP | RB | GWB |  | AT |  |  |
| Controlled Substances Storage and Secured Dispensing Area1 | VCT | RB | GWB1 |  | GWB1 | 30 U | 157 |
| Storage, Flammable | VCT | RB | GWB |  | AT | 11/12 S | 157 |
| Extemporaneous Compounding | VCT/CP | RB | GWB |  | AT | 36 S | 157 |
| Mail Out | VCT/CP | RB | GWB |  | AT | 36 U | 157 |
| Refrigeration/ Freezer Area | Prefabricated Unit |  |  |  |  |  |  |
| Drug Receiving: |  |  |  |  |  |  |  |
| Breakdown and Verification Area | VCT/CP | RB | GWB1 |  | AT1 | 36 V | 157 |
| Inventory Control/ Stock Management Clerk | VCT/CP | RB | GWB |  | AT |  |  |
| Storage, Prosthetic and Medical Supplies | VCT | RB | GWB |  | AT | 11/12 V-W | 157 |
| Consultation Room | CP | RB | GWB |  | AT1Lay-in Panels in Grid1 | 1/2 S | 45 |
| Waiting Area | CP | RB | GWB |  | AT | Open | --- |
| Specialized Programs Education: |  |  |  |  |  |  |  |
| Office, Clinical Pharmacy Teaching Coordinator | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Pharmaceutical Experimentation Laboratory | CP | RB | GWB |  | AT | 1/2 V-T | 13 |
| Conference/Classroom | CP | RB | GWB1, |  | AT1 | 1/2 U-T | 52 |
| Drug Information Service | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 52 |
| Study Cubicle | Prefabricated |  |  |  |  |  |  |
| Radiopharmacy | See Chapter 252 |  |  |  |  |  |  |
| Drug Dependency Treatment Clinic: |  |  |  |  |  |  |  |
| Prepackaging | VCT/CP | RB | GWB |  | AT | --- |  |
| Receiving and Storage | VCT/CP | RB | GWB |  | AT | 36 V | 157 |
| Record Control | VCT/CP | RB | GWB |  | AT | --- |  |
| Consultation Room | CP | RB | GWB1 |  | AT/GWB Lay-in Panels in Grid1 | 1/2 S | 45 |
| Investigational Drugs: |  |  |  |  |  |  |  |
| Administrative Work Area | CP | RB | GWB |  | AT | 1/2 S-T/ Open | 52 |
| Storage, Investigational Drugs | VCT | RB | GWB |  | AT | 11/12 V-W | 43 |
| Oncology Drugs | VCT | RB | GWB |  | AT | 1/2 S-T | 13 |
| Preparation Area | VCT | RB | GWB |  | AT | --- |  |
| Reference Area | VCT | RB | GWB |  | AT | --- |  |
| Storage and Clean/ Decontamination Area | VCT | RB | GWB |  | AT | 1/2 S | 13 |
| Medication Preparation Areas: |  |  |  |  |  |  |  |
| Unit Dose Dispensing | VCT | RB | GWB |  | AT | 1/2 S-T |  |
| Stat Counter | VCT | RB | GWB |  | AT | --- |  |
| Support Area | VCT | RB | GWB |  | AT | --- |  |
| Drug Utilization Review Area | VCT | RB | GWB |  | AT | --- |  |
| Data Processing Area: |  |  |  |  |  |  |  |
| Work Station | VCT/CP | RB | GWB |  | AT | --- |  |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Staff Lockers, Lounge & Toilets | See Chapter 410 |  |  |  |  |  |  |
| Spinal Cord Injury Nursing Unit Outpatient Pharmacy: | See Chapter 410 |  |  |  |  |  |  |
| Corridor | VCT | RB | GWB1 |  | AT | 9/102 VV-T | 62 |

Perimeter corridor full height walls.

2 See Section II, "Doors and Hardware" for corridor door and hardware information.

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Staff Offices: |  |  |  |  |  |  |  |
| Chief of Service | CP | RB | GWB |  | AT | 1/2 S | 52 |
| Secretary & Waiting | CP | RB | GWB |  | AT | 1/2 S-T/ Open | 23 |
| Clinic Admin. Space | CP | RB | GWB |  | AT | 1/2 S-T/ Open | 23 |
| Education Space: |  |  |  |  |  |  |  |
| Study Cubicles | CP | RB | GWB |  | AT1 | 1/2 U-T | 25 |
| Recreation Program Modalities: |  |  |  |  |  |  |  |
| Multi-Purpose Recreation Room | VCT | RB | GWB/W |  | AT | 1/2 SS-T | 82 |
| Storage Multi-Purpose and Recreation Room | VCT | RB | GWB |  | AT | 11/12 V-W | 42 |
| Serving Kitchen | RSF | RB | GWB |  | AT | 1/2 S-T | 69 |
| Game and Activity Room | VCT | RB | GWB/W |  | AT | 1/2 U-T | 67 |
| Game and Activity Room Storage Area | VCT | RB | GWB |  | AT | 1/2 S-T | 13 |
| Billiard Room | VCT | RB | CMU or PCP1, |  | AT | 1/2 S-T | 67 |
| Arts, Crafts, and Hobby Room | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Storage Room for Activities | VCT | RB | GWB |  | AT | 11/12 V-W | 42 |
| Music Room | CP | RB | GWB1,2 |  | AT | 1/2 S-T | 45 |
| Bowling Lanes | WD | WD | CMU or PCP1,2 |  | AT | 1/2 S-T | 67 |
| Theater (House) | VCT/CP | RB | GWB1 |  | AT | 1/2 VV-T | 84 |
| Stage | WD | WD | GWB |  | AT |  | 86 |
| Storage for Stage Materials | VCT | RB | GWB |  | AT | 11/12 V-W | 43 |
| Wheelchair/Litter Area | VCT | RB | GWB |  | AT |  |  |
| Projection Booth | VCT | RB | GWB |  | AT | 1/2 S-T | 43 |
| Therapeutic Pool | See Chapter 270 |  |  |  |  |  |  |
| Toilet Facilities for Activity Rooms | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 or 14 |
| Toilet Facility for Theater | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 or 14 |
| Vestibule | CT | CT | GWB |  | AT | 1/2 S | 4 |
| Toilet Facilities for Multi-Purpose Recreation, Billiards, and Bowling Rooms | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 or 14 |
| Corridor | VCT | RB | CMU or GWB |  | AT | 1/2 SS-T | 61 |

See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |
| Chief of Service | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Staff Physician | CP | RB | GWB/W |  | AT | 1/2 S | 23 |
| Secretary & Waiting | CP | RB | GWB/W |  | AT | 1/2 S-T | 23 |
| Assistant Chief Coordinator | CP | RB | GWB/W |  | AT | 1/2 S | 23 |
| Staff Lockers, Lounges, Toilets, and Showers | See Chapter 410 |  |  |  |  |  |  |
| Escort Service | VCT/CP | RB | GWB/W |  | AT | 1/2 S-T | 23 |
| Education: |  |  |  |  |  |  |  |
| Office Resident Physician | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Study Cubicles | CP | RB | GWB |  | AT | Open |  |
| Conference/Classroom for Residents and Students | CP | RB | GWB/W , |  | AT1 | 1/2 U-T | 40 |
| Clinical: |  |  |  |  |  |  |  |
| Exam Room | VCT | RB | GWB1 |  | AT | 1/2 V3 | 52 |
| Electromyography (EMG) Room | VCT | RB | GWB4 |  | AT | 1/2 V-T3 | 53 |
| Physical Therapy: |  |  |  |  |  |  |  |
| Clinic Office | VCT/CP | RB | GWB/W |  | AT | 3/4 S-T | 23 |
| Hubbard Tanks | CT | CT | CT |  | PCP | 1/2 W3 | 69 |
| Arm and Leg Whirlpool | CT | CT | CT |  | PCP | 1/2 V3 | 69 |
| Wheelchair Toilet | CT | CT | CT |  | GWB | 1/2 S | 14 |
| Treatment Clinic | VCT | RB | GWB |  | AT | 1/2 V-T | 53 |
| Equipment Storage | VCT | RB | GWB |  | AT | 11/12 V-W | 42 |
| Utility Area | VCT | RB | GWB |  | AT | 1/2 S-T | 69 |
| Spec. Treatment Room | VCT | RB | GWB |  | AT | 1/2 V1 | 69 |
| Dressing Rooms | VCT | RB | GWB |  | AT | 1/2 S | 1 or 14 |
| Waiting Area | VCT | RB | GWB |  | AT | 1/2 U-T/Open | 53 |
| Basic Clinic: |  |  |  |  |  |  |  |
| Treatment Table (Private Room) | VCT | RB | GWB |  | AT | 1/2 V1 | 69 |
| Treatment Tables | VCT | RB | GWB |  | AT | 1/2 V1 | 69 |
| Treatment Stations | VCT | RB | GWB |  | AT | 1/2 V1 | 69 |
| Equipment Storage | VCT | RB | GWB |  | AT | 1/2 S | 34 |
| Utility Area | VCT | RB | GWB |  | AT | 1/2 S | 69 |
| Kinesiotherapy: |  |  |  |  |  |  |  |
| Clinic Office | VCT/CP | RB | GWB/W |  | AT | 3/4 S-T | 23 |
| Treatment Clinic | VCT | RB | GWB |  | AT | 1/2 V1 | 53 |
| Psychiatric Exercise Clinic | VCT | RB | GWB |  | AT | 1/2 V1 | 53 |
| Equipment Storage | VCT | RB | GWB |  | AT | 1/2 S | 54 |
| Basic Clinic: |  |  |  |  |  |  |  |
| Treatment Area | VCT | RB | GWB |  | AT | 1/2 V1 | 53 |
| Storage | VCT | RB | GWB |  | AT | 1/2 S | 13 |
| Occupational Therapy: |  |  |  |  |  |  |  |
| Clinic Office | VCT/CP | RB | GWB |  | AT | 3/4 S-T | 23 |
| Treatment Clinic | VCT | RB | GWB |  | AT | 1/2 V-T | 53 |
| Equipment Storage | VCT | RB | GWB |  | AT | 1/2 V-T | 34 |
| Orthotic and Splinting Area | VCT | RB | GWB |  | AT | 1/2 V-T | 69 |
| Patient Project Storage | See Chapter 100 |  |  |  |  |  |  |
| Basic Clinic: |  |  |  |  |  |  |  |
| Treatment Area | VCT | RB | GWB |  | AT | 1/2 V-T1 | 53 |
| Storage | VCT | RB | GWB |  | AT | 1/2 U-W or 11/ 12 | 13 |
| Activities of Daily Living Evaluation and Training Center: |  |  |  |  |  |  |  |
| Kitchen | VCT | RB | GWB |  | AT |  |  |
| Bedroom | VCT/CP | RB | GWB/W |  | AT | 1/2 S | 18 |
| Bathroom | CT | CT | GWB | CT 4'0" | GWB | 1/2 S | 18 |
| Vocational Rehab. Therapy: |  |  |  |  |  |  |  |
| Clinic Office | VCT/CP | RB | GWB |  | AT | 3/4 S-T | 23 |
| Treatment Clinic | VCT | RB | GWB |  | AT | 1/2 V-T1 | 53 |
| Equipment Storage | VCT | RB | GWB |  | AT | 1/2 S | 34 |
| Educational Therapy: |  |  |  |  |  |  |  |
| Clinic Office | VCT/CP | RB | GWB |  | AT | 3/4 S-T | 23 |
| Treatment Clinic | VCT | RB | GWB |  | AT | 1/2 V-T1 | 53 |
| Equipment Storage | VCT | RB | GWB |  | AT | 1/2 S | 34 |
| Driver Training: |  |  |  |  |  |  |  |
| Clinic Office | VCT/CP | RB | GWB |  | AT | 3/4 S-T | 23 |
| Instruction, Orientation, and Simulator Area | VCT/CP | RB | GWB |  | AT | 1/2 V-T | 52 |
| Compensated Work Therapy: |  |  |  |  |  |  |  |
| Clinic Office | VCT/CP | RB | GWB |  | AT | 3/4 S-T | 23 |
| Clinic Work Area | VCT | RB | GWB |  | AT | 1/2 V-T | 52 |
| Storage Area | VCT | RB | GWB |  | AT | 11/12 U-W | 13 |
| Male Wheelchair Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 14 |
| Female Wheelchair Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 14 |
| Evaluation Area | VCT | RB | GWB |  | AT | 1/2 U-T | 53 |
| Patient Rest Area | VCT | RB | GWB |  | AT | 1/2 U-T | 53 |
| Incentive Therapy: |  |  |  |  |  |  |  |
| Clinic Office | VCT/CP | RB | GWB |  | AT | 3/4 S-T | 23 |
| Assignment Area | VCT | RB | GWB |  | AT | 1/2 V-T | 52 |
| Tool Area | VCT | RB | GWB |  | AT | 13/14 S | 26 |
| Showers, Lavatory, and Watercloset | CT | CT | GWB1 | CT 4'0" | AT (SP) | 1/2 S | 1 |
| Cardiopulmonary Rehab. Center Work Evaluation Program: |  |  |  |  |  |  |  |
| Administration Offices | CP | RB | GWB |  | AT | 3/4 S-T | 23 |
| Evaluation Clinic | VCT | RB | GWB |  | AT | 1/2 V-T | 53 |
| Equipment Storage | VCT | RB | GWB |  | AT | 1/2 S | 34 |
| Basic Clinic | VCT | RB | GWB |  | AT | 1/2 V-T | 53 |
| Therapeutic Pool | CT | CT | CT |  | PCP | 1/2 V-T2 | 53 |
| Dressing, Showers, and Toilet Area | CT | CT | CT |  | PCP | 1/2 S | 1 |
| Clean/Soiled Towels and Suits | CT | CT | CT |  | PCP | 13/14 T | 31 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T ADO | 1263 |

Full height CT at tub/shower on concrete backboard with PL ceiling.

2  Provide X door if VAMC includes SCI unit.

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |
| Office, Chief Psychology Service | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Secretary | CP | RB | GWB-W |  | AT | 1/2 S/Open | 23 |
| Office, Clerical | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, Psychologist | CP | RB | GWB-W1 |  | AT1 | 1/2 S | 23 |
| Office, Vocational Rehabilitation Spec. | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Psychology Interns | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Psychology Technician | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Waiting Space | CP | RB | GWB |  | AT | Open |  |
| Storage | VCT | RB | GWB |  | AT | 11/12 V-W | 43 |
| Multi-Purpose Rooms: |  |  |  |  |  |  |  |
| Multipurpose Room | CP/VCT | RB | GWB |  | AT | 1/2 U-T | 52 |
| Control Room/Office | CP | RB | GWB |  | AT | 1/2 U | 23 |
| Automated Group Testing Rooms: |  |  |  |  |  |  |  |
| MS&N Hospitals | VCT | RB | GWB |  | AT | 1/2 U-T | 52 |
| Mental Health and Behavioral Hospitals | VCT | RB | GWB |  | AT | 1/2 U-T | 52 |
| Specialized Assessment Treatment Programs: |  |  |  |  |  |  |  |
| Biofeedback Laboratory: |  |  |  |  |  |  |  |
| Patient Treatment | VCT | RB | GWB |  | AT | 1/2 U | 52 |
| Control Room/ Tech. Offices | VCT | RB | GWB |  | AT | 1/2 U | 23 |
| Neuropsychology Laboratory: |  |  |  |  |  |  |  |
| Laboratory | VCT/CP | RB | GWB1 |  | AT1 | 1/2 U-M | 67 |
| Viewing Room | VCT/CP | RB | GWB1 |  | AT1 | 1/2 S-M | 67 |
| Vocational Rehabilitation Unit | CP/VCT | RB | GWB |  | AT | 1/2 U-T | 52 |
| Health Psychology Clinics | CP/VCT | RB | GWB1 |  | AT1 | 1/2 U | 52 |
| Lockers, Lounges & Toilets | See Chapter 410 |  |  |  |  |  |  |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T | 62 |

Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Bedroom | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Bath | CT | CT | GWB1 | CT 4'0" | AT | 1/2 S | 18 or 172 |
| Closet | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Lounge | CP | RB | GWB |  | AT | 1/2 S-T | 52 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 QQ-T | 633 |

CT full height at tub on concrete backer board with PL ceiling.

2 If bathroom serves two bedrooms.

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS |  | WAIN |  | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |  |  |
| Office, Chief Radiology | CP | RB | GWB |  |  |  | AT | 1/2 S | 52 |
| Office, Secretary and Waiting | CP | RB | GWB |  |  |  | AT | 1/2 S-T | 23 |
| Office, Administration Assistant | CP | RB | GWB |  |  |  | AT | 1/2 S-T | 23 |
| Office, Reception | CP | RB | GWB |  |  |  | AT | 1/2 S-T | 23 |
| Office, Staff Physician | CP | RB | GWB |  |  |  | AT | 1/2 S-T | 23 |
| Office, Chief Technician | VCT | RB | GWB |  |  |  | AT | 1/2 S-T | 23 |
| Education Facilities: |  |  |  |  |  |  |  |  |  |
| Office, Resident Intern | CP | RB | GWB |  |  |  | AT | 1/2 S-T | 23 |
| Study Cubicle | CP | RB | GWB,2 |  |  |  | AT1 | 1/2 U-T |  |
| Conference/Classroom | CP | RB | GWB-W1,2 |  |  |  | AT1 | 1/2 U-T | 40 |
| Staff Lockers, Lounges, Toilets, and Showers | See Chapter 410 |  |  |  |  |  |  |  |  |
| Main Radiology Suite: |  |  |  |  |  |  |  |  |  |
| Radiographic Fluoroscopic Room and Control Area | VCT | RB | GWB3 |  |  | AT1,4 |  | 15 W/15 X5 | 106 |
| Dedicated Chest Room and Control Area | VCT | RB | GWB3 |  |  | AT1 |  | 15 W/15 X1 | 106 |
| Automated Room (Rapido) and Control Area | VCT | RB | GWB3 |  |  | AT4 |  | 15 W/15 X5 | 106 |
| All Other (G.P. Rooms and Control Areas) | VCT | RB | GWB3 |  |  | AT4 |  | 15 W/15 X5 | 106 |
| Special Procedures: |  |  |  |  |  |  |  |  |  |
| Affil. Facilities Appr. Radiology Teaching Programs | WSF | WSF | GWB |  |  |  | AT | 15 W/15 X | 72 |
| Special Procedure Support Area: |  |  |  |  |  |  |  |  |  |
| Anesthesiology Prep. and Recovery Room | WSF | WSF | GWB |  |  | AT |  | 1/2 W-T | 72 |
| Scrub Room | WSF | WSF | GWB |  |  | AT |  | 1/2S-T/15S | 12 |
| Viewing Room | VCT | RB | GWB |  |  | AT |  | 1/2 S-T | 68 |
| Film Processing Area | VCT | RB | GWB |  |  | GWB |  | 1/2 S-M | 69 |
| Common Viewing Room | VCT/ CP | RB | GWB |  |  | AT |  | 1/2 S-T | 67 |
| Multi-Viewing Area | VCT/ CP | RB | GWB |  |  | AT |  | 1/2 S-T | 67 |
| Film Library | VCT | RB | GWB |  |  | AT |  | 1/2 U-T | 69 |
| Barium Prep. | VCT | RB | GWB |  |  | AT |  | 1/2 S-T | 69 |
| Waiting Space | VCT/ CP | RB | GWB |  |  | AT |  | Open | --- |
| Wheelchair and Stretcher Waiting Space | VCT | RB | GWB |  |  | AT |  | Open | --- |
| Dressing Room | VCT | RB | GWB |  |  | AT |  | 1/2 S | 14 |
| Toilet Room, Wheelchair | CT | CT | GWB | CT 4'0" |  | AT |  | 1/2 S | 14 |
| Toilet Room, Standard | CT | CT | GWB | CT 4'0" |  | AT |  | 1/2 S | 14 |
| Central Silver Coll. | LM | LM | GWB |  |  | GWB |  | 11/12 S-W | 42 |
| Unexposed Film and Chemical Storage | LM | LM | GWB |  |  | GWB |  | 1/2 S-T | 76 |
| Clean Utility Room | VCT | RB | GWB |  |  | AT (SP) |  | 1/2 V-W | 42 |
| Soiled Utility Room | VCT | RB | GWB |  |  | AT (SP) |  | 11/12 V-W | 42 |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" |  | AT (SP) |  | 1/2 T | 13 |
| Equipment Storage | VCT | RB | GWB |  |  | AT |  | 1/2 SS-W/ 1/2 V-W | 64 42 |
| Corridors | VCT | RB | GWB |  |  | AT |  | 1/2 VV-T | 821 |

See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration: |  |  |  |  |  |  |  |
| Office, Chief Radiation Therapy | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Office, Staff Physician | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Prof. Non-Physician | CP | RB | GWB/W |  | AT | 1/2 S | 23 |
| Office, Secretary Waiting | CP | RB | GWB/W |  | AT | 1/2 S-T/ Open | 23 |
| Clerical Employee | CP | RB | GWB |  | AT | 1/2 S-T/ Open | --- |
| Reception & Control Counter | CP | RB | GWB/W |  | AT | Open | --- |
| Office, Nurse | CP | RB | GWB/W |  | AT | 1/2 S | 23 |
| Office, Technologist Supervisor | CP | RB | GWB/W |  | AT | 1/2 S | 23 |
| Office, Resident Physicians | CP | RB | GWB/W |  | AT | 1/2 S-T | 23 |
| Patients Records Filing | VCT | RB | GWB |  | AT | 11/12 S-W | 42 |
| Viewing & Consultation Room | CP | RB | GWB |  | AT1 | 1/2 S-T | 52 |
| Staff Lockers & Lounges | See Chapter 410 |  |  |  |  |  |  |
| Storage Room | VCT | RB | GWB |  | AT | 11/12 S-W | 42 |
| Toilet Room, Staff | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 18 |
| Exam Area: |  |  |  |  |  |  |  |
| Exam Room | VCT | RB | GWB1 |  | AT1 | 1/2 V | 69 |
| Dressing Room | VCT | RB | GWB |  | AT | 1/2 S | 14 |
| General Waiting Space | VCT/ CP | RB | GWB/W |  | AT | Open |  |
| Stretcher Waiting | VCT | RB | GWB |  | AT | Open |  |
| Toilet Room, Wheelchair | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 14 |
| Treatment Planning Unit: |  |  |  |  |  |  |  |
| Simulator Unit | VCT | RB | GWB |  | AT | 17 W | 106 |
| Control Area | VCT | RB | GWB1 |  | AT | Open |  |
| Film Processing | VCT | RB | GWB |  | GWB | 1/2 S-M or Revolving Dark Room Door | 69 162 |
| Mold Room | VCT | RB | GWB |  | AT | 11/12 V-T | 42 |
| Planning Ultrasound Unit | VCT | RB | GWB |  | AT | 1/2 W-T | 69 |
| Treatment Planning Computer Room | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 67 |
| Treatment Area: |  |  |  |  |  |  |  |
| Superficial Therapy Unit & Controls | VCT | RB | GWB |  | AT | 15 W | 106 |
| Orthovoltage Therapy Unit1 | VCT | RB | GWB1 |  | AT | 17 W | 106 |
| Control Area | VCT | RB | GWB |  | AT | Open |  |
| Supervoltage Unit (Cobalt 60)1 | VCT | RB | GWB1 |  | AT | 17X Motor Operated | 119 |
| Megavoltage Unit1 Linear Accelerator | VCT | RB | GWB1 |  | AT | 17 X | 119 |
| Patient Pre-treatment Room 1 | VCT | RB | GWB1 |  | AT | 1/2 W | 69 |
| Radium Sealed Source Room | LM | LM | GWB2 |  | AT2 | 1/2 W/ 15W2 | 69 106 |
| Physics Laboratory | VCT | RB | GWB |  | AT | 1/2 U-T | 69 |
| Stretcher/ Wheelchair Storage | VCT | RB | GWB3 |  | AT | 1/2 U | 69 |
| Clean Utility/ Supplies Room | VCT | RB | GWB (SC) |  | AT (SP) | 1/2 U | 69 |
| Soiled Utility/ Supplies Room | RSF | RB | GWB (SC) |  | AT (SP) | 11/12 U | 42 |
| Housekeeping Aids Closet (HAC) | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T ADO1 | 1262 |

See Architectural Design Manual PG-18-10, "Automatic Doors".

2 See Section II, "Doors and Hardware".

3 Provide wall guards and bumper guards in this room.

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Biomedical Research: |  |  |  |  |  |  |  |
| Administration Area: |  |  |  |  |  |  |  |
| Office, Associate Chief of Staff | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Secretary | CP | RB | GWB |  | AT | 1/2 S/Open | 23 |
| Office, Clerical | CP | RB | GWB |  | AT | 1/2 S/Open | 23 |
| Office, Administrative Officer | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Conference Room | CP | RB | GWB/ GWB-W, |  | AT | 1/2 U-T | 40 |
| Reproduction and Copy Room | CP | RB | GWB |  | AT | 1/2 S-T | 52 |
| ADP Workstation | CP | RB | GWB |  | AT | 1/2 S-T/Open | 23 |
| ADP Printer Area | CP | RB | GWB |  | AT | 1/2 S-T/Open | 23/52 |
| Office, Special Staff Position | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Storage, Files | CP | RB | GWB |  | AT | 11/12 V-W | 13 |
| Storage, Office Supplies | CP | RB | GWB |  | AT | 11/12 V-W | 13 |
| Word Processing/ Computer Room | CP | RB | GWB/ GWB-W |  | AT | 1/2 S-T | 23 |
| Medical Research: |  |  |  |  |  |  |  |
| Laboratory Area: |  |  |  |  |  |  |  |
| Office Investigator | CP | RB | GWB |  | AT | 1/2S/ 1/2S-M | 23 |
| Laboratory Units (General, Chemistry, Media Preparation, Radioisotope) | ERF | ERF | GWB |  | AT | 1/2 V-T | 67 |
| Electron Microscope Suite: |  |  |  |  |  |  |  |
| Scope Room | VCT | RB | GWB |  | GWB | 1/2 V | 67 |
| Dark Room | VCT | RB | GWB |  | GWB | 1/2 S-M or Revolving Dark Room Door | 14 162 |
| Preparation Room | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Microtome/Cutting Room | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Controlled Temperature Rooms: |  |  |  |  |  |  |  |
| Walk-in Refrigerators/ incubators | Pre-Fabricated |  |  |  |  |  | 1 |
| Walk-in Freezers1 | Pre-Fabricated |  |  |  |  |  | 1 |
| Storage Facilities: |  |  |  |  |  |  |  |
| Gas Cylinder Storage2 | C | RB | CMU/ GWB |  | GWB | 26 V / 11/12 V | 51 |
| Flammable Storage3 | C | RB | CMU/ GWB |  | GWB | 26 V / 11/12 V | 43 |
| Acid Storage | VCT | RB | CMU |  | GWB | 26 V / 11/12 V | 43 |
| General Storage | VCT | RB | CMU/ GWB |  | AT | 11/12 V-W | 43 |
| Waste Storage Facilities: |  |  |  |  |  |  |  |
| Radioactive Waste Storage Room | VCT | RB | GWB |  | GWB | 38 V | 51 |
| Biohazardous Waste Storage Room | VCT | RB | GWB |  | GWB | 38 V | 51 |
| Laboratory Support Space: |  |  |  |  |  |  |  |
| Common Instrument Room | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Glassware Washing and Sterilization Room | ERF | ERF | CMU (SC) |  | PCP | 1/2 V-T | 69 |
| Ultralow Freezer Room | VCT | RB | GWB |  | GWB | 1/2 W | 69 |
| Corridor | VCT | RB | CMU/ GWB |  | AT | 1/2 VV-T | 82 |
| Special Instrument Spaces: |  |  |  |  |  |  |  |
| Confocal Microscope | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Fluorescent Activated Cell Sorter (FACS) | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Cell Irradiator | VCT | RB | GWB |  | AT | 1/2 V-T | 67 |
| Nuclear Magnetic Resonance (NMR) Spectrometer | VCT | RB | GWB |  | AT | 1 QQ-T | 82 |
| Mass Spectrometer | VCT | RB | GWB |  | AT | 1 V-T | 67 |
| Polymerase Chain Reaction (PCR): |  |  |  |  |  |  |  |
| Airlock (Corridor) | VCT | RB | GWB |  | AT | 1/2 S-T | 71 |
| Preparation Lab | VCT | RB | GWB |  | AT | 1/2 V-T | 68 |
| PCR Room | VCT | RB | GWB |  | AT | 1/2 S | 68 |
| Electrophoresis Room | VCT | RB | GWB |  | AT | 1/2 S | 68 |
| Veterinary Medical Unit (VMU): |  |  |  |  |  |  |  |
| Animal Receiving and Examination Room | ERF | ERF | CMU (SC) |  | PCP | 20 V-D | 69 |
| Diagnostic Laboratory | ERF | ERF | CMU (SC) |  | PCP | 20 V-D | 69 |
| Animal Treatment Room | ERF | ERF | CMU (SC) |  | PCP | 20 V-D | 69 |
| Quarantine Room | ERF | ERF | CMU (SC) |  | PCP | 20 V-D | 69 |
| Animal Rooms Cubicle Housing | ERF | ERF | CMU (SC) |  | PCP | Special | 2 |
| Animal Rooms Conventional Housing | ERF | ERF | CMU (SC) |  | PCP | 20 V-D-M | 69 |
| Procedural Laboratory | ERF | ERF | CMU (SC) |  | PCP | 20 V-T | 69 |
| Surgical Suite: |  |  |  |  |  |  |  |
| Operating Rooms (OR) Survival (Aseptic) Surgery | ERF | ERF | CMU (SC) |  | GWB | 2 V-T | 69 |
| Terminal (Non-Survival) Surgery | ERF | ERF | CMU (SC) |  | GWB | 2 V-T | 69 |
| Surgical Preparation Room | ERF | ERF | CMU (SC) |  | PCP | 2 V-T | 69 |
| Scrub and Gown Room | ERF | ERF | CMU (SC) |  | PCP | 2 S-T | 12 |
| Surgical Work and Supply Room | ERF | ERF | CMU (SC) |  | PCP | 2 V-T | 69 |
| Post Operative Intensive Care | ERF | ERF | CMU (SC) |  | PCP | 2 V-T | 69 |
| Storage, Dry Feed and Bedding | ERF | ERF | CMU (SC) |  | PCP | 12 V | 51 |
| Necropsy | ERF | ERF | CMU (SC) |  | PCP | 2 V-T | 69 |
| Carcass and Refrigerator Waste Storage | ERF | ERF | CMU (SC) |  | PCP | 2 V-T | 69 |
| Walk-in Refrigerator (Cold Room) | Pre-Fabricated |  |  |  |  |  | Padlock |
| Cage Servicing and Sanitation Area: |  |  |  |  |  |  |  |
| Cage Wash Room (Tunnel Washer and/or Cage and Rack Washer) | ERF | ERF | CMU (SC) |  | PCP | 2 SS-T | 4 |
| Clean Cage Storage Room | ERF | ERF | CMU (SC) |  | 80 | 2 SS-T | 80 |
| Equipment Storage | ERF | ERF | CMU |  | 80 | 12 SS-W | 80 |
| Administrative Offices: |  |  |  |  |  |  |  |
| Office, Chief Veterinarian | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Supervisor of VMU | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Secretary | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Lounges, Lockers, Toilets & Showers (LLTS) | See Chapter 410 |  |  |  |  |  |  |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Radiographic Suite: |  |  |  |  |  |  |  |
| Radiographic Room | ERF | ERF | CMU (SC) |  | GWB | 15 V | 106 |
| Control Booth | ERF | ERF | CMU (SC) |  | GWB | Open |  |
| Dark Room | ERF | ERF | CMU (SC) |  | GWB | 2 S-M | 14 |
| Diet Kitchen | ERF | ERF | CMU (SC) |  | GWB | 2 S-T | 69 |
| Incinerator Room | ERF | ERF | CMU (SC) |  | GWB | 10 SS-W | 64 |
| Environmental Lab | ERF | ERF | CMU (SC) |  | GWB | 2 V-M | 69 |
| Laboratory Biohazard Area: |  |  |  |  |  |  |  |
| Chemical and/or Radioisotope Suite: |  |  |  |  |  |  |  |
| Animal Room (Cubicle Concept) | ERF | ERF | CMU (SC) |  | GWB | 2 V-T/ Special |  |
| Procedural Laboratory | ERF | ERF | CMU (SC) |  | PCP | 2 V-T | 69 |
| Hazardous Waste Disposal Room | ERF | ERF | CMU (SC) |  | PCP | 2 V-T | 51 |
| Infectious Disease Suite: |  |  |  |  |  |  |  |
| Air Lock (Ante Room) | ERF | ERF | CMU (SC) |  | PCP | 2 V-M-T, | 51 |
| Animal Room (Cubicle Concept) | ERF | ERF | CMU (SC) |  | PCP | Special | 1 |
| Procedural Laboratory | ERF | ERF | CMU (SC) |  | PCP | 2 V-T | 72 |
| Clothing Change Room | ERF | ERF | CMU (SC) |  | PCP | 2 S | 1 |
| Shower Room | CT | CT | CT |  | PCP | 2 S | 1 |
| Toilet Room | CT | CT | SC | CT 4.0 | PCP | 2 S | 12 |
| Barrier Suite: |  |  |  |  |  |  |  |
| Clothing Change Room | ERF | ERF | CMU (SC) |  | PCP | 2 S | 1 |
| Shower Room | CT | CT | CT |  | PCP | 2 S | 1 |
| Toilet Room | CT | CT | SC | CT 4.0 | PCP | 2 S | 12 |
| Air Lock (Ante Room) | ERF | ERF | CMU (SC) |  | PCP | 2 V-M-T | 51 |
| Animal Room (Cubicle Concept) | ERF | ERF | CMU (SC) |  | PCP | 2 V-T/ Special2 | 3 |
| Procedural Laboratory | ERF | ERF | CMU (SC) |  | PCP | 2 V-T | 72 |
| Corridor | ERF | ERF | CMU (SC) |  | PCP | 2 QQ-T/ 20 QQ-M4 | 82 |
| Health Services Research and Development: |  |  |  |  |  |  |  |
| Rehabilitation Research and Development | (To be determined by approved space requirements) |  |  |  |  |  |  |
| Office, Investigator | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, Secretary | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Office, Management Analyst | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Corridors | VCT | RB | GWB |  | AT | 1/2 QQ-T | 61 |

See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Staff Offices: |  |  |  |  |  |  |  |
| Chief of Service | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Assistant Chief or Detective | VCT/CP | RB | GWB |  | AT | 1/2 S | 23 |
| Clerk | VCT/CP | RB | GWB |  | AT | 1/2 S-T/ Open | 23 |
| Duty Area: |  |  |  |  |  |  |  |
| Operations Room | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 52 |
| Holding Room | VCT | RB | CMU or PCPPL1 |  | GWBPL | 22 S-A | 10 |
| Equipment Storage | VCT | RB | GWB |  | AT | 1/2 S | 13 |
| Staff Lockers | See Chapter 410 |  |  |  |  |  |  |
| Corridor | VCT | RB | GWB |  | AT | 1/2 QQ-T | 62 |

Plaster on high rib lath.

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administrative Offices | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Corridors | VCT | RB | GWB |  | AT | 1/2 QQ-T | 61 |

See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Office, Chief | CP | RB | GWB-W1 |  | AT | 1/2 S | 52 |
| Social Worker | VCT/CP | RB | GWB1 |  | AT | 1/2 S | 52 |
| Associate Social Worker | VCT/CP | RB | GWB1 |  | AT | 1/2 S | 52 |
| Secretary | VCT/CP | RB | GWB |  | AT | 1/2 S-T/ Open | 23 |
| Clerk-Typist and/or Stenographer | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 52 |
| Multipurpose Room | CP/VCT | RB | GWB |  | AT | 1/2 U-T | 52 |
| Waiting Area | CP | RB | GWB-W |  | AT | Open |  |
| Corridors | VCT | RB | GWB |  | AT | 1/2 QQ-T | 62 |

Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Office, Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Secretary and Waiting | VCT/CP | RB | GWB-W |  | AT | 1/2 S-T/ Open | 23 |
| Office, Assistant Chief | VCT/CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, Supervisor, Procurement Section | VCT/CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, Supervisor, Supply Management Section | VCT/CP | RB | GWB |  | AT | 1/2 S | 23 |
| Office, (Technical, Clerical, and Administrative) | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Bid Opening Room | VCT/CP | RB | GWB-W |  | AT | 1/2 S-T | 52 |
| Corridors | VCT | RB | GWB |  | AT | 1/2 QQ-T | 61 |

1 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Soiled Receiving and Decontamination Area | QT | QT | GWB (SC) |  | GWB (SC) | 2 UU-T/ 12 UU-W | 82 161 |
| Manual Equipment Area | QT | QT | CT |  | PCP | 2 W-T or as req'd | 69 |
| Automatic Cart Wash | QT | QT | CT1 |  | PCP | 2 W-T or as req'd | 4 |
| Housekeeping Aides Closet | QT | QT | CT |  | GWB (SC) | 1/2 T | 13 |
| Lockers, Lounge, Toilet, and Shower Facilities | See Chapter 410 |  |  |  |  |  |  |
| Clean Side: |  |  |  |  |  |  |  |
| Prep. Assembly and Sterilization | QT2 | QT | GWB (SC) |  | AT (SP) | 2 UU-T/ 12 UU-W | 82 161 |
| ETO Gas Cylinder Storage | QT | QT | GWB (SC) |  | GWB (SC) | 12 W | 43 |
| Sterile/Non-Sterile Storage Area: |  |  |  |  |  |  |  |
| Sterile Areas | VCT | RB | GWB (SC) |  | AT (SP) | 12 UU-W | 161 |
| Non-Sterile Areas | VCT | RB | GWB (SC) |  | AT (SP) | 12 UU-W | 161 |
| Bulk Storage Area | VCT | RB | GWB (SC) |  | AT (SP) | 12 UU-W | 161 |
| Sterilizer Mechanical | C | Prefab Panel |  |  | PCP | Prefab |  |
| Clean Receiving and Breakout Room | VCT | RB | GWB (SC) |  | AT (SP) | 2 UU-T/ 12 UU-W | 82 161 |
| Dispatch Area: |  |  |  |  |  |  |  |
| Control Station/Office | VCT | RB | GWB (SC) |  | AT | 1/2 S-T/ 11/12 S-W | 23 45 |
| Work Station | VCT | RB | GWB (SC) |  | AT | 1/2 S-T | 52 |
| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Inspection and Preparation of Surgical Linen and Packs | QT | QT | GWB (SC) |  | GWB (SC) | 2 UU-T/ 12 UU-W | 82 161 |
| Equipment Storage and Testing Room | VCT | RB | GWB (SC) |  | AT (SP) | 2 SS-T/ Open | 82 |
| Housekeeping Aids Closet | QT | QT | CT |  | GWB (SC) | 1/2 T | 13 |
| Administrative Offices: |  |  |  |  |  |  |  |
| Chief, SPD | CP | RB | GWB/W |  | AT | 1/2 S | 52 |
| Assistant Chief, SPD | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Clerical Staff | VCT/CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Volunteers and Training Room | VCT | RB | GWB |  | AT | 1/2 U-T | 52 |
| SPD Terminal (Clean) | VCT | RB | GWB (SC) |  | AT (SP) | 2 U-W | 152 |
| SPD Terminal (Soiled) | QT | QT | CT (SC) |  | GWB | 2 U-W | 152 |
| Lockers, Lounge, Toilet, and Shower | See Chapter 410 |  |  |  |  |  |  |
| Corridors | VCT | RB | GWB |  | AT | 1/2 VV-T | 61 |

1 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| NOTES: (a) Rooms or Spaces are based on Space Criteria and the Surgical Design Guide. (b) Doors from corridor to operating rooms to be a minimum of 4'-0". Doors from clean core to operating rooms to be a minimum of 3'-8". All other doors into operating rooms 3'-0". |  |  |  |  |  |  |  |
| Administrative/Staff Offices: |  |  |  |  |  |  |  |
| Office, Chief, Surgical Service | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Office, Chief, Anesthesiology | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Head of Surgical Section | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Office, Asst. Chief, Surgery | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Asst. Chief, Anesthesiology | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Secretary/Waiting | CP | RB | GWB-W |  | AT | 1/2 S-T/ Open | 23 |
| Clerical | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Staff Surgeon, Anesthesiologist, Resident, and Nurse Anesthetist | CP | RB | GWB-W |  | AT | 1/2 S | 52 |
| Office, Surgical/ Anesthesiology Residents | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Operating Room Coordinator | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Quarters, On-Call | See Chapter 274 |  |  |  |  |  |  |
| Operating Rooms (OR) (Aseptic): |  |  |  |  |  |  |  |
| General OR (Ambulatory, Eye, ENT, etc.) | WSF | WSF | GWB (SC) |  | GWB | 1/2 V-T 1/2 QQ-T | 12 79 |
| Special OR (Orthopedic, Vascular) | WSF | WSF | GWB (SC) |  | GWB | 15 V-V 15 QQ-V | 104 109 |
| Cardiac/ Neurosurgery OR | WSF | WSF | GWB (SC)1,3 |  | GWB | 15 V-V2 15 QQ-V | 104 109 |
| Operating Room Support:3 |  |  |  |  |  |  |  |
| Clean Core (Staff Workroom) | WSF | WSF | GWB (SC) |  | GWB | 1/2 VV-T5 | 12 |
| Scrub-up Area (Aseptic OR)3 | WSF | WSF | GWB (SC) |  | GWB | 1/2 S-T or 15 S-V and Open | 108 |
| Sub-sterile Room (Cabinet-enclosed Equipment)3 | WSF | WSF | GWB (SC) |  | GWB | 15 S-V | 12 |
| Sub-sterile Room (Recessed Equipment)3 | WSF | WSF | GWB (SC) |  | GWB | 15 S-V | 12 |
| Radiographic Film Processing Room | WSF | WSF | GWB (SC) |  | GWB | 1/2 S or 1/2 S-M | 69 or 14 |
| Frozen Section Laboratory | WSF | WSF | GWB (SC) |  | GWB | 1/2 S | 67 |
| Receiving Room/ Dispatch Room (Cart Lifts To/From SPD) | WSF | WSF | GWB (SC) |  | GWB | Open |  |
| Equipment and Apparatus Storage | WSF | WSF | GWB (SC) |  | GWB | 1/2 V-T | 69 |
| Clean and Sterile Supplies Storage | WSF | WSF | GWB (SC) |  | GWB | 11/12 V-T | 42 |
| Soiled Holding/ Disposal Room | WSF | WSF | GWB (SC) |  | GWB | 11/12 V-W | 42 |
| External Radiographic Control Room4 | WSF | WSF | GWB (SC) |  | GWB | 15 S-V | 67 |
| Heart/Lung Machine Preparation Room | WSF | WSF | GWB (SC) |  | GWB | 15 S | 69 |
| Plaster and Splint Storage Room | WSF | WSF | GWB (SC) |  | GWB | 15 S | 69 |
| Special Recording Equipment Room1 | WSF | WSF | GWB (SC) |  | GWB | 15 S | 69 |
| Anesthesia Workroom and Equipment Storage | WSF | WSF | GWB (SC) |  | GWB | 1/2 V-T | 69 |
| Office, Head Nurse | WSF | WSF | GWB (SC) |  | GWB | 1/2 S-T | 23 |
| Control and Communications Center | WSF | WSF | GWB (SC) |  | GWB | Open |  |
| Patient Holding Area | WSF | WSF | GWB (SC) |  | GWB | Open |  |
| Family Waiting Room | WSF | WSF | GWB (SC)/ GWB-W |  | GWB | 1/2 S-T | 40 |
| Gas Cylinder Storage Room | WSF | WSF | GWB (SC) |  | GWB | 11/12 S | 42 |
| Gurney Storage Area | WSF | WSF | GWB (SC) |  | GWB | 1/2 V | 4 |
| Nerve Block Induction Room | WSF | WSF | GWB (SC) |  | GWB | 1/2 W-T | 72 |
| Housekeeping Aids Closet (Clean Core 60 NSF) | WSF | WSF | GWB (SC) | CT 4'0" | GWB | 1/2 T | 25 |
| Housekeeping Aids Closet (Peripheral Corridor 100 NSF) | WSF | WSF | GWB (SC) | CT 4'0" | GWB | 1/2 T | 13 |
| Dedicated Electrical Closet (Clean Core OR's) | WSF | WSF | GWB (SC) |  | GWB | 1/2 V | 13 |
| Automatic Uniform Dispensing Room | VCT | RB | GWB |  | AT | 1/2 S | 67 |
| Locker, Lounge, Toilet, and Shower Facilities (LLTS): |  |  |  |  |  |  |  |
| Lounge, Entire Surgical Suite Staff | CP | RB | GWB/W |  | AT | 1/2 S-T | 1 |
| Lounge, Surgeons, Residents, and Students | CP | RB | GWB-W |  | AT | 1/2 S-T | 1 |
| Lounge, Nurses & Other OR Staff Members | CP | RB | GWB-W |  | AT | 1/2 S-T | 1 |
| Nourishment Kitchen (Reserved) | VCT | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Dictation Cubicle | CP | RB | GWB-W |  | AT | Open |  |
| Locker Rooms (Male/Female) | VCT | RB | GWB |  | AT | 1/2 S | 1 |
| Shower Area Male/Female | CT | CT | CT |  | PCP | 1/2 S/ Open | 1 |
| Ambulatory Surgery Support Space: |  |  |  |  |  |  |  |
| Changing Cubicles & Patient Lockers | WSF | WSF | GWB (SC) |  | GWB | 1/2 S | 1 |
| Wheelchair Toilet (Male/Female) | CT | CT | GWB (SC) | CT 4'0" | GWB | 1/2 S | 14 |
| Post Operative Recovery Lounge | WSF | WSF | GWB (SC) |  | GWB | 1/2 W-T | 67 |
| Cystoscopy Suite and Cystoscopy Suite Support (Septic Surgery)1: |  |  |  |  |  |  |  |
| Cystoscopy Room | WSF | WSF | GWB (SC)2 |  | GWB | 15 V-V 15 QQ-V | 104 109 |
| Cystoscopy Room with Shockwave Lithotripsy | WSF | WSF | GWB (SC)2 |  | GWB | 15 V-V 15 QQ-V | 104 109 |
| Instrument Preparation and Storage Room | WSF | WSF | GWB (SC)2 |  | GWB | 1/2 S-T | 69 |
| Radiographic Film Processing Room | WSF | WSF | GWB (SC) |  | GWB | 1/2 S or 1/2 S-M | 69 or 14 |
| External Radiographic Control Room | WSF | WSF | GWB (SC) |  | GWB | Open |  |
| Scrub-up Area | WSF | WSF | GWB (SC)2 |  | GWB | 15 S-V3 1/2 S-T | 108 11 |
| Housekeeping Aids Closet | WSF | WSF | GWB (SC) | CT 4'0" | GWB | 1/2 T | 13 |
| Post Anesthesia Recovery Unit (PACU): |  |  |  |  |  |  |  |
| Entry | WSF | WSF | GWB (SC) |  | GWB | 1/2 SS-T (ADO)1 | 126 |
| Recovery Beds (Cubicles) | WSF | WSF | GWB (SC) |  | AT (SP) | Open |  |
| Nurses Station | WSF | WSF | GWB (SC) |  | AT (SP) | Open |  |
| Staff Toilet | WSF | WSF | GWB (SC) | CT 4'0" | GWB | 1/2 S | 1 or 18 |
| Clean Utility | WSF | WSF | GWB (SC) |  | GWB | 11/12 V-W | 42 |
| Soiled Utility | WSF | WSF | GWB (SC) |  | GWB | 11/12 V-W | 42 |
| Storage | WSF | WSF | GWB (SC) |  | GWB | 1/2 V | 42 |
| Medication Preparation Room | WSF | WSF | GWB (SC) |  | AT (SP) | 1/2 S | 13 |
| Isolation Room | WSF | WSF | GWB (SC) |  | GWB | 3/4 X-T | 67 |
| Ante Room To PAR Isolation Room | WSF | WSF | GWB (SC) |  | GWB | 1/2 S-T | 12 |
| Corridors, Peripheral | WSF | WSF | GWB (SC) |  | AT (SP) | 1/2 VV-T/ ADO | 1262 |

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administrative Office: |  |  |  |  |  |  |  |
| Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Assistant Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Secretary and Receptionist | CP | RB | GWB-W |  | AT | 1/2 S-T Open | 23 |
| Clerical Staff | CP | RB | GWB-W |  | AT | 1/2 S-T/Open | 23 |
| Volunteer Sign-in | CP | RB | GWB-W |  | AT | Open |  |
| VA VS Organizational Work Station | CP | RB | GWB-W |  | AT | 1/2 V-T/ 1/2 SS-T | 52 |
| Volunteer Multipurpose Workroom | CP | RB | GWB-W |  | AT | 1/2 V-T/ 1/2 SS-T | 23 28 |
| Storage | VCT/CP | RB | GWB |  | AT | 11/12 V-W/ 11/12 SS-W | 43 60 |
| Corridor | VCT/CP | RB | GWB-W |  | AT | 1/2 QQ-T1 | 6 |

1 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Medical and General Storage | C-PUT | CMU/ RB | CMU/ GWB1 |  | EXP | 11/12 UU-W | 43 |
| Medical Research Storage | C-PUT | CMU/ RB | CMU/ GWB1 |  | EXP | 11/12 UU-W | 43 |
| Receiving and Issue Area | C-PUT | CMU/ RB | CMU/ GWB1 |  | EXP | 10'0" x 10'0" Power Operated Roll-Up Doors |  |
| Unposted Stock Holding Area | C-PUT | CMU/ RB | CMU/ GWB1 |  | EXP | 11/12 UU-W | 77 |
| Flammable Storage | C | CMU | CMU2 |  | EXP | 26 U | 43 |
| Medical Gas Storage | C-PUT | CMU | CMU1 |  | EXP | 26U | 43 |
| Equipment Storage | C-PUT | CMU/ RB | CMU/ GWB1 |  | EXP | 11/12 V-W | 43 |
| Forms Storage/ Processed Stores (Office Supplies) | C-PUT | CMU/ RB | CMU/ GWB1 |  | EXP | 11/12 V-W | 43 |
| Subsistence Storage | C-PUT/VCT | CMU/ RB | CMU/ RB1 |  | EXP | 11/12 UU-W | 43 |
| Excess Property Storage | C-PUT | CMU/ RB | CMU/ GWB1 |  | EXP | 11/12 V-W | 43 |
| Dialysis Storage | C-PUT | CMU/ RB | CMU/ GWB1 |  | EXP | 11/12 V-W | 43 |
| Office, Storekeeper | VCT | RB | CMU/ GWB1 |  | AT | 1/2 S-T | 23 |
| Staff Toilets, Lockers and Lounge | See Chapter 410 |  |  |  |  |  |  |
| Corridor | C-PUT | CMU/ RB | CMU/ GWB1 |  | EXP | 1/2 VV-T | 823 |

Full height partitions to underside of structural floor.

2 Provide one-hour rated enclosure and 6” perimeter concrete curb cast integrally with floor in accordance with NFPA requirements.

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Staff Offices: |  |  |  |  |  |  |  |
| Office, Psychiatrist | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Psychologist | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Social Worker | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Resident/ Trainee | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Rehab. Therapist | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Nurse | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Secretary & Waiting | VCT/ CP | RB | GWB-W |  | AT | Open |  |
| Treatment Areas: |  |  |  |  |  |  |  |
| Examination Room | VCT | RB | GWB1 |  | GWB Lay in Panels in Grid System1 | 1/2 S | 52 |
| Group Therapy Room | CP | RB | GWB-W1 |  | AT1 | 1/2 U-M | 47 |
| Activities Room and Dining Space | VCT | RB | GWB |  | AT | 1/2 SS-T | 81 |
| Quiet Room | CP | RB | GWB-W1 |  | AT1 | 1/2 U-M | 52 |
| Patient Lockers, Toilets & Showers: |  |  |  |  |  |  |  |
| Locker Rooms | VCT | RB | GWB |  | AT | 1/2 S | 1 |
| Toilets & Showers | CT | CT | GWB2 | CT 4'0" | AT | 1/2 S | 1 |
| Corridors | VCT | RB | GWB |  | AT | 1/2 QQ-T | 63 |

1 Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 Full Height ceramic tiles on concrete backer board at tub/ shower with plaster   ceiling.

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administrative Offices: |  |  |  |  |  |  |  |
| Chief of Services | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Assistant Chief of Services/Prosthetic Representative | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Secretary & Waiting | CP | RB | GWB-W |  | AT | Open |  |
| Dispensing Prosthetics Clerks | VCT | RB | GWB |  | AT | 1/2 S-T | 23 |
| Reception and Waiting | VCT | RB | GWB |  | AT | 3/4 U-T | 23 |
| Prosthetic Appliance Storage Room: |  |  |  |  |  |  |  |
| Storage Area | VCT | RB | GWB |  | AT | 1/2 V-W | 34 |
| Mailing Area | VCT | RB | GWB |  | AT | 1/2 S-T | 53 |
| Orthopedic Evaluation: |  |  |  |  |  |  |  |
| Team Evaluation/ Multi-Purpose Room | VCT | RB | GWB |  | AT1 | 1/2 V-T | 52 |
| Fitting/Exam Room | VCT | RB | GWB |  | AT | 1/2 V | 69 |
| Dressing Room | VCT | RB | GWB |  | AT | 1/2 S | 14 |
| Patient Wheelchair Toilets | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Orthotic Laboratory: |  |  |  |  |  |  |  |
| Orthotist Office | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Orthotic Laboratory | VCT | RB | GWB |  | AT | 11/12 U-W | 45 |
| Shoe Modification Materials Storage | VCT | RB | GWB |  | AT | 11/12 U-W | 45 |
| Wheelchair Repair Area | VCT | RB | GWB1 |  | AT | 1/2 U-T | 69 |
| Wheelchair Workspace | VCT | RB | GWB |  | AT | 1/2 V-T | 52 |
| Wheelchair-Active Storage | VCT | RB | GWB |  | AT | 1/2 S-T | 52 |
| Wheelchair Parts Storage | VCT | RB | GWB |  | AT | 1/2 S-T | 52 |
| Lockers, Lounge | See Chapter 410 |  |  |  |  |  |  |
| Staff Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Restoration Clinic: |  |  |  |  |  |  |  |
| Eye Fitting Studio | VCT | RB | GWB |  | AT | 1/2 U | 67 |
| Facial/ Body Fitting Studio | VCT | RB | GWB |  | AT | 1/2 W | 67 |
| Restoration Laboratory (with materials and cast storage) | VCT | RB | GWB |  | AT | 11/12 U-W | 45 |
| Corridors | VCT | RB | GWB |  | AT | 1/2 VV-T/ | 61 |

1 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Patient Bed Area: |  |  |  |  |  |  |  |
| Patient Bedrooms | VCT | RB | GWB |  | AT | 1/2 H | 7 |
| Private & Shared Toilets | CT | CT | GWB1 | CT 4'0" | AT1 | 1/2 H | 14 |
| Full Baths | CT | CT | GWB | CT 4'0" | AT (SP) | 1/2 H | 2 |
| Congregate Baths with Toilets | CT | CT | CT1 |  | PCP | 1/2 L | 13 |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 I | 13 |
| Patient Open TV Lounge | CP | RB | GWB-W |  | AT | Open |  |
| Dom Patient Multi-Purpose Room Kitchenette | VCT | RB | GWB |  | AT | 1/2 H | 40 |
| Quiet Room | CO | RB | GWB |  | AT | 1/2 H2 | 52 |
| Office Domiciliary Assistant | CP | RB | GWB |  | AT | 1/2 H | 52 |
| Office Section Leader | CP | RB | GWB |  | AT | 1/2 H | 52 |
| Multiple Living | CP | RB | GWB-W |  | AT | 1/2 L | 14 |
| Support Facility Area-Administration: |  |  |  |  |  |  |  |
| Office, Domiciliary Chief | CP | RB | GWB-W |  | AT | 1/2 H | 23 |
| Office, Assistant Chief | CP | RB | GWB-W |  | AT | 1/2 H | 23 |
| Office, Secretary & Waiting | CP |  | GWB |  | AT | 1/2 H-T | 23 |
| Clerical Space | VCT | RB | GWB |  | AT | 1/2 H-T | 23 |
| Office, Staff | CP | RB | GWB |  | AT | 1/2 H-T | 23 |
| Lobby | SPECIAL |  |  |  |  |  |  |
| Conference/Dom. Patient Education Room | CP | RB | GWB-W2 |  | AT1 | 1/2 K-T | 40 |
| Male or Female Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 H | 1 |
| Patient Laundry | RSF | RSF | GWB |  | AT | 1/2 H-T | 1 |
| Nurse Treatment Area: |  |  |  |  |  |  |  |
| Medication Room | VCT | RB | GWB |  | AT | 1/2 H | 43 |
| Examination Room | VCT | RB | GWB |  | AT/GWB1 Lay-In Panels in Grid | 1/2 H | 52 |
| Office | CP | RB | GWB |  | AT | 1/2 H | 23 |
| Waiting | VCT | RB | GWB |  | AT | Open |  |
| Vending Area | VCT | RB | GWB |  | AT | Open |  |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 J | 13 |
| Clean Linen Room | VCT | RB | GWB (SC) |  | GWB | 11/12 I-W | 45 |
| Soiled Linen Collection Area | VCT | RB | GWB (SC) |  | GWB | 11/12 I-W | 45 |
| Staff Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 I | 23 |
| Trash Collection | RSF | RSF | GWB (SC) |  | AT (SP) | 11/12 K-W | 42 |
| General Storage | VCT | RB | GWB |  | AT | 11/12 HH-W/ 11/12 I-W | 69 |
| Central Dom. Patient Clothing & Luggage Storage | VCT | RB | GWB |  | AT | 11/12 I-W | 43 |
| Employees Lockers, Lounges, Toilets & Showers | See Chapter 410 |  |  |  |  |  |  |
| Building Management Storage | VCT | RB | GWB |  | AT | 11/12 I-W | 69 |
| Dietetics: |  |  |  |  |  |  |  |
| Office-Dietitian | CP | RB | GWB |  | AT | 1/2 H | 23 |
| Dining Area | VCT | RB | GWB-W |  | AT | 1/2 HH-T | 126 |
| Serving Unit | QT | QT | CT |  | AT (SP) | 1/2 HH-T | 82 |
| Bulk Food Cart | VCT | RB | GWB (SC) |  | AT | 1/2 K-T | 13 |
| Non-Food Storage | VCT | RB | GWB |  | AT | 11/12 K-T | 82 |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 J | 13/7 |
| Recreation/Multi- Purpose Area | VCT | RB | GWB-W1 |  | AT | 1/2 H-T | 52 |
| Storage | VCT | RB | GWB |  | AT | 1/2 K-T | 13 |
| Exercise Room | VCT | RB | GWB |  | AT | 1/2 L-T | 52 |
| Storage | VCT | RB | GWB |  | AT | 1/2 K-T | 13 |
| Arts & Crafts Area | VCT | RB | GWB |  | AT | 1/2 K-T | 52 |
| Storage | VCT | RB | GWB |  | AT | 1/2 H | 13 |
| Domiciliary Patients Toilets Male or Female | CT | CT | GWB | CT 4'0" | AT | 1/2 H | 1 |
| Corridors | CP | RB | GWB |  | AT | 8 II-T | 62 |

1 Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Dialysis Bed/Chair Area: |  |  |  |  |  |  |  |
| One-Bed Room | RSF | RSF | GWB-W |  | AT | 1/2 W-T | 7/5 |
| Bedrooms and Multi-Bed Area | RSF | RSF | GWB-W |  | AT | 1/2 W-T | 7/5 |
| Dialysis Training Bedroom | RSF | RSF | GWB-W |  | AT | 1/2 W | 23 |
| Toilet Room: |  |  |  |  |  |  |  |
| Wheelchair Water Closet and Lavatory | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Lockers | VCT | RB | GWB |  | AT | 1/2 S | 1 |
| Staff Toilet | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Staff Showers & Locker Facilities | See Chapter 410 |  |  |  |  |  |  |
| Nurse Station | CP | RB | GWB-W |  | AT | Open |  |
| Clean Utility Room | VCT | RB | GWB (SC) |  | AT (SP) | 1/2 V-W | 45 |
| Soiled Utility Room | RSF | RB | GWB (SC) |  | AT (SP) | 11/12 V-W | 45 |
| Storage Alcove, Wheelchair and Stretcher | VCT | RB | GWB |  | AT | Open |  |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Treatment Equipment Service Room: |  |  |  |  |  |  |  |
| Soiled Equipment Service Area | RSF | RSF | GWB (SC) |  | AT (SP) | 11/12 X-T | 69 |
| Clean Equipment Service Area | RSF | RSF | GWB (SC) |  | AT (SP) | 1/2 X-T | 69 |
| Storage Area | VCT | RB | GWB |  | AT | 11/12 V-W | 69 |
| Dialysate Preparation Room | RSF | RSF | GWB (SC) |  | AT (SP) | 1/2 V-T | 69 |
| Water Treatment Room | RSF | RSF | GWB (SC) |  | AT (SP) | 1/2 V-T | 69 |
| Examination and Treatment Room | RSF | RSF | GWB1 |  | AT (SP) /GWB Lay-in Panels in Grid System | 1/2 X | 69 |
| Laboratory, Biochemistry | RSF | RSF | GWB |  | AT | 1/2 V-T | 69 |
| Administration: |  |  |  |  |  |  |  |
| Office, Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Asst. Chief | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Secretary & Waiting | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Clerical | CP | RB | GWB-W |  | AT | 1/2 S-T | 23 |
| Office, Staff Physician | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Social Worker | CP | RB | GWB-W1 |  | AT1 | 1/2 S | 23 |
| Office, Chief Chemist | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Chief Nurse | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Office, Self-Dialysis Support Personnel | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Storage Area | VCT | RB | GWB |  | AT | 11/12 V-W | 69 |
| Education: |  |  |  |  |  |  |  |
| Office, House Staff | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Study Cubicle | CP | RB | GWB1 |  | AT1 | 1/2 U-T | 17 |
| Conference/Classroom | CP | RB | GWB-W1,2 |  | AT1 | 1/2 U-T | 40 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 VV-T | 63 |

1 Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 Provide acoustical/ tackable wall surface on one wall.

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN |  | CEILING | DOOR SYMBOL | H.W.NO. |
| Health Science Library and Patient Library: |  |  |  |  |  |  |  |  |
| Circulation Desk Area | CP | RB | GWB |  |  | AT | Open |  |
| Card Catalog | CP | RB | GWB |  |  | AT | Open |  |
| Shelving Collections (Periodicals, Microfilm, Monograph, Basic Reference, Abstract and Index) | CP | RB | GWB |  |  | AT | Open |  |
| Photocopy Area | CP | RB | GWB |  |  | AT | Open |  |
| Microfilm Area | CP | RB | GWB |  |  | AT | Open |  |
| Audiovisual Room | CP | RB | GWB-W1 |  |  | AT1 | 3/4 U-T | 52 |
| Seating Space (Reader Stations) | CP | RB | GWB1 |  |  | AT1 | Open |  |
| Reference Desk Area | CP | RB | GWB |  |  | AT | Open |  |
| Media Conference Room | CP | RB | GWB-W1,2 |  |  | AT1 | 1/2 U-M | 52 |
| Workroom/Technical Services | CP | RB | GWB1 |  |  | AT1 | 1/2 U-T | 52 |
| Office, Chief, Library Service | CP | RB | GWB-W |  |  | AT | 1/2 S | 23 |
| Clerical Space | CP | RB | GWB |  |  | AT | Open |  |
| Staff Librarian's Area | CP | RB | GWB |  |  | AT | Open |  |
| Computer Terminal's Word Processors/ Printers and Storage Cabinets | CP | RB | GWB |  |  | AT | Open |  |
| Audiovisual Storage | VCT | RB | GWB |  |  | AT | 11/12 V-W | 43 |
| Corridor | VCT | RB | GWB |  |  | AT | 1/2 QQ-T | 63 |

1 Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 Provide acoustical/ tackable wall surface on one wall.

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN |  | CEILING | DOOR SYMBOL | H.W.NO. |
| Professional Conference Rooms | CP | RB | GWB-W, |  |  | AT1 | 1/2 U-T | 52 |
| Chief Resident | CP | RB | GWB-W |  |  | AT | 1/2 S | 23 |
| Medical Service, Laboratory, House Staff and Students | VCT/CP | RB | GWB-W |  |  | AT | 1/2 V-T | 67 |
| Nursing Unit Educational Service | VCT/CP | RB | GWB-W |  |  | AT | 1/2 V-T | 67 |
| Office, Work Room, Two House Staff and Medical Students | CP | RB | GWB-W |  |  | AT | 1/2 S-T | 52 |
| Office Nursing Instructor and Student Workroom | CP | RB | GWB-W |  |  | AT | 1/2 S-T | 52 |
| Demonstration/ Classroom | CP | RB | GWB1 |  |  | AT1 | 1/2 V-T | 52 |
| Administrative Space: |  |  |  |  |  |  |  |  |
| Secretary and Waiting | CP | RB | GWB-W |  | AT |  | 1/2 S-T | 23 |
| Additional Clerk | CP | RB | GWB |  | AT |  | 1/2 S-T | 23 |
| Coatroom | CP | RB | GWB |  | AT |  | 1/2 S | 7 |
| Nursing Education and Training; In Service Training: |  |  |  |  |  |  |  |  |
| Secretary/Waiting | CP | RB | GWB-W |  | AT |  | 1/2 S-T |  |
| Classroom | CP | RB | GWB |  | AT |  | 1/2 S-T | 52 |
| Medical Center Director's Training Program | CP | RB | GWB-W |  | AT |  | 1/2 S-T | 52 |
| Other Medical Center Activities: |  |  |  |  |  |  |  |  |
| Laboratory | VCT | RB | GWB |  | AT |  | 1/2 V-T | 67 |
| Educational Auditorium: |  |  |  |  |  |  |  |  |
| Auditorium | Special finishes to be selected on basis of requirements. |  |  |  |  |  |  | 86/90 |
| Stage or Instruction Area | Special finishes to be selected on basis of requirements. |  |  |  |  |  |  | 40 |
| Projection Booth | VCT | RB | GWB |  |  | AT | 1/2 S-T | 40 |
| Rear Projection | VCT | RB | GWB |  |  | AT | 1/2 S-T | 40 |
| Patient Preparation | VCT | RB | GWB |  |  | AT | 1/2 V-T | 52 |
| Corridor | VCT | RB | GWB |  |  | AT | 1/2 QQ-T | 61 |

1 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administrative Space: |  |  |  |  |  |  |  |
| Chief, Building Management Service | CP | RB | GWB-W |  | AT | 1/2 S | 23 |
| Secretary/Waiting | CP | RB | GWB |  | AT | Open | 1 |
| Assistant Chief | CP | RB | GWB |  | AT | 1/2 S-T | 23 |
| Supervisors & Clerical Technical Administrative Employee | CP | RB | GWB |  | AT | 1/2 S | 23 |
| Storage-Building Management Supplies and Equipment | VCT | RB | GWB |  | AT | 11/12 V-W | 69 |
| Housekeeping Aids Closet | CT | CT | GWB (SC) | CT 4'0" | AT (SP) | 1/2 T | 13 |
| Central Storage | LM | LM | GWB (SC) |  | AT (SP) | 11/12 QQ-W/ 11/12 V-W | 80 69 |
| Soiled Linen Sorting and Storage | VCT | RB | GWB (SC) |  | GWB | 11/12 SS-W | 150 |
| Clean Linen Sorting and Storage | VCT | RB | GWB (SC) |  | GWB | 11/12 SS-W | 150 |
| Trash Collection (Lowest Level) | C-EPY | CT/ EPY | CT/ CMU (SC) |  | EXP | 10 X | 43 |
| Trash Collection (Upper Levels) | VCT | RB | GWB (SC) |  | AT (SP) | 11/12 V-W | 43 |
| Seamstress Space | VCT | RB | GWB |  | AT | 1/2 V-T | 52 |
| Patient Clothing and Luggage Storage | RSF | RSF | CT |  | AT | 11/12 V-W | 43 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 QQ-T | 61 |

1 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARD-WARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W. NO. |
| Administrative Offices: |  |  |  |  |  |  |  |
| Office, Laundry Plant Manager | VCT/CP | RB | GWB-W |  | AT | 1/2 H | 23 |
| General Administration Area (Open) | VCT/CP | RB | GWB-W |  | AT | Open |  |
| Lunch Room/Training Room/Vending Machine/ Kitchen | VCT | RB | GWB-W |  | AT | 1/2 H-T | 52 |
| Toilets, Showers & Lockers | See Chapter 410 |  |  |  |  |  |  |
| Housekeeping Aids Closet (HAC) | CT | CT | CT |  | AT (SP) | 1/2 J | 13 |
| Storage | VCT | RB | GWB |  | AT | 11/12 K-W | 69 |
| Receiving Area | RF | RB | CMU/ GWB |  | EXP |  |  |
| Sorting and Washing Area | RF | RB | CMU/ GWB |  | EXP | 1 |  |
| Linen Storage Room | RF | RB | CMU/ GWB |  | EXP | 11/12 HH(W) | 43 |
| Central Liquid Supply System Room | C/RF | RB | CMU (SC) |  | EXP | 11/12 K-W | 43 |
| Cart Washer Area | RF | RB | CMU (SC)/ GWB (SC) |  | EXP | 1 |  |
| Clean Linen Processing Area | RF | RB | CMU (SC)/ GWB (SC) |  | EXP | 1 |  |
| Clean Linen Holding (Flow Rack) and Assembly Area | RF | RB | CMU (SC)/ GWB (SC) |  | EXP | 1 |  |
| Distribution Area (Cart Storage, Dispatch Area, Dock Area) | RF | RB | CMU (SC)/ GWB (SC) |  | EXP | 1 |  |
| Linen Pack Preparation Room | RF | RB | CMU (SC)/ GWB (SC) |  | AT (SP) | 1/2 K-T | 69 |
| Linen Repair Area | RF | RB | GWB |  | AT | 1/2 K-T | 69 |
| Entrance Vestibules | RF | RB | CMU (SC)/ GWB (SC) |  | AT | 1/2 K-T | 69 |
| Medical Center Linen Support Area: |  |  |  |  |  |  |  |
| Linen Collection Room | VCT | RB | GWB |  | AT | 1/2 V-T | 69 |
| Soiled Linen Cart Storage Room | VCT | RB | GWB (SC) | CT 5'0" | AT (SP) | 11/12 SS-W | 43 |
| Clean Linen Cart Storage Room | VCT | RB | GWB (SC) | CT 5'0" | AT (SP) | 11/12 SS-W | 43 |
| Uniform Exchange Room | VCT | RB | GWB (SC) |  | AT (SP) | 1/2 V-T or Open | 69 |
| Corridor | VCT | RB | CMU (SC)/ GWB (SC) |  | EXP | 1/2 V | 691 |

1 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Locker Rooms | VCT | RB | GWB |  | AT | 1/2 S | 1 |
| Lounges (Employee) | CP/VCT | RB | GWB |  | AT | 1/2 S-T | 38 |
| Staff Shower Rooms | CT | CT | CT1 |  | PCP | 1/2 S | 1 |
| Toilets, Patient | CT | CT | GWB | CT 4'0" | AT | 1/2 S | 1 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 QQ-T | 62 |

1 CT on concrete backer board.

2 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Administration Building: |  |  |  |  |  |  |  |
| Offices | CP | RB/WD | GWB-W |  | AT/GWB | 1/2 H | 23 |
| Operational Center | CP | RB/WD | GWB-W |  | AT/GWB | 1/2 V-T | 52 |
| Lunch Room | VCT | RB | GWB-W |  | AT/GWB | 1/2 H-T | 52 |
| Storage | RSF | RSF | GWB |  | AT | 1/2 K-T | 13 |
| Housekeeping Aids Closet | RSF | RSF | GWB (SC) | CT 4'0" | AT (SP) /GWB | 1/2 J | 13 |
| Multipurpose Room (Conference Room) | CP | RB/WD | GWB-W |  | AT/GWB | 1/2 H | 52 |
| Lobby & Reception1 | Special |  |  |  |  | 1/2 JJ-T | 81 |
| Toilet | CT | CT | GWB (SC) | CT 4.0 | AT/GWB | 1/2 H | 1 |
| Public Toilet, Exterior | LM | LM | CMU (SC) |  | AT (SP) /EXP | 1/2 H | 142 |
| Corridor | VCT | RB | GWB |  | AT | 1/2 K-T | 43 |

Finishes are be selected on the basis of architectural requirements and cost considerations. Floor to be hard surface material such as: QT, STONE, BRICK, SLATE, TERRAZO, etc.

2 Use H.W. No. 14 for single use and 1 for multi-use.

3 See Section II, "Doors and Hardware".

| ROOM OR SPACE | FINISHES |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS | WAIN | CEILING | DOOR SYMBOL | H.W.NO. |
| Service Building: |  |  |  |  |  |  |  |
| Office Foreman | VCT | RB | GWB/CMU |  | AT/GWB | 1/2 H-T | 23 |
| Lunch Room | VCT | RB | GWB/CMU |  | AT/GWB | 1/2 H-T | 23 |
| Housekeeping Aids Closet | C/CT | RB/CT | GWB/CMU | CMU/CT 4'0" | EXP/AT/GWB | 1/2 J | 13 |
| Locker | VCT | RB | GWB |  | AT | 1/2 H | 1 |
| Raincoat Holding Area and Boot & Towel Storage | C/CT | RB/CT | CMU (SC) |  | AT (SP)/ EXP | 1/2 H-T | 13 |
| Vehicle, Equipment, and Maintenance & Storage | C | CMU | CMU |  | EXP | 12'x12' Overhead Roll-up Door | --- |

| ROOM OR SPACE | FINISHES |  |  |  |  |  |  |  |  | DOORS | HARDWARE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLOOR | BASE | WALLS |  | WAIN |  |  | CEILING |  | DOOR SYMBOL | H.W.NO. |
| All areas in Chapters 800 through 870 that utilize the open office planning concept will have the following finishes: |  |  |  |  |  |  |  |  |  |  |  |
|  | CP | RB | GWB |  |  |  | AT |  |  |  |  |
| The following spaces in Chapters 800 through 870 will be private (i.e. enclosed): |  |  |  |  |  |  |  |  |  |  |  |
| CHAPTER 800 - OFFICE OF DIRECTOR |  |  |  |  |  |  |  |  |  |  |  |
| Office, Director | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Office, Assistant Director | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Administrative Conference Room | CP | RB | GWB-W, |  |  |  |  | AT |  | 1/2 J-T | 40 |
| Director's Toilet | CT | CT | GWB-W |  | CT 4’-0” |  |  | AT |  | 1/2 H | 1 |
| CHAPTER 801 - BUILDING SUPPORT FACILITIES |  |  |  |  |  |  |  |  |  |  |  |
| Toilet Facilities | CT | CT | GWB(SC) |  | CT 4'0" |  |  | AT |  | 1/2 H | 1 |
| Storage Room | VCT | RB | GWB |  |  |  |  | AT |  | 11/12 HH-W 11/12 K-W | 69 13 |
| Housekeeping Aids Closet | CT | CT | GWB(SC) |  | CT 4'0" |  |  | AT |  | 1/2 K | 13 |
| CHAPTER 802 - DISTRICT COUNSEL |  |  |  |  |  |  |  |  |  |  |  |
| Office, District Counsel | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Office, Assistant District Counsel | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Office, General Attorneys | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Library | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 J-T | 40 |
| CHAPTER 821 - ADJUDICATION DIVISION |  |  |  |  |  |  |  |  |  |  |  |
| Office, Division Chief | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Office, Assistant Division Chief | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Hearing Rooms | CP | RB | GWB-W1 |  |  |  |  | AT1 |  | 1/2 H | 40 |
| CHAPTER 823 - ADMINISTRATIVE DIVISION |  |  |  |  |  |  |  |  |  |  |  |
| Office, Division Chief | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Office, Assistant Division Chief | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Central Transcription Unit | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 K-T | 23 |
| Teletype and Data Terminal Unit | CP | RB | GWB-W |  |  |  |  | AT1 |  | 1/2 H-T | 23 |
| Mail Unit | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H-T | 23 |
| Central Reproduction Area | CP | RB | GWB-W1 |  |  |  |  | AT1 |  | 1/2 K-T | 23 |
| CHAPTER 824 - FINANCE DIVISION |  |  |  |  |  |  |  |  |  |  |  |
| Office, Division Chief | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Office, Assistant Division Chief | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Agent Cashier | CP | RB | GWB |  |  |  |  | AT |  | 36 H | 157 |
| CHAPTER 825 - PERSONNEL DIVISION |  |  |  |  |  |  |  |  |  |  |  |
| Office, Division Chief | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Office, Assistant Division Chief | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Audiovisual Storage | VCT | RB | GWB |  |  |  |  | AT |  | 11/12 K-W | 69 |
| Training Room | CP | RB | GWB1 |  |  |  |  | AT |  | 1/2 J-T | 40 |
| Testing Room | CP | RB | GWB1 |  |  |  |  | AT |  | 1/2 J-T | 40 |
| CHAPTER 826 - LOAN GUARANTY DIVISION |  |  |  |  |  |  |  |  |  |  |  |
| Office, Division Chief | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Office, Assistant Division Chief | CP | RB | GWB-W |  |  |  |  | AT |  | 1/2 H | 23 |
| Security Files Room | VCT | RB | GWB2 |  |  |  |  | AT |  | 36 H | 157 |
| Interview Rooms | CP | RB | GWB1 |  |  |  |  | AT1 |  | 1/2 H | 23 |
| CHAPTER 827 - VETERANS SERVICE DIVISION |  |  |  |  |  |  |  |  |  |  |  |
| Office, Division Chief | CP | RB | GWB-W |  |  |  |  |  | AT | 1/2 H | 23 |
| Office, Assistant Division Chief | CP | RB | GWW |  |  |  |  |  | AT | 1/2 H | 23 |
| Telephone Unit | CP | RB | GWB |  |  |  |  |  | AT | 1/2 H | 23 |
| CHAPTER 828 - VOCATIONAL REHABILITATION & COUNSELING |  |  |  |  |  |  |  |  |  |  |  |
| Office, Division Chief | CP | RB | GWB-W |  |  |  |  |  | AT | 1/2 H | 23 |
| Office, Assistant Division Chief | CP | RB | GWB-W |  |  |  |  |  | AT | 1/2 H | 23 |
| Office, Counselor | CP | RB | GWB |  |  |  |  |  | AT1 | 1/2 H-M | 23 |
| Test Room | CP | RB | GWB1 |  |  |  |  |  | AT1 | 1/2 H | 23 |
| Career Development Center | CP | RB | GWB-W |  |  |  |  |  | AT | 1/2 H | 23 |
| CHAPTER 834 - SUPPORT SERVICE |  |  |  |  |  |  |  |  |  |  |  |
| Office, Division Chief | CP | RB | GWB-W |  |  |  |  |  | AT | 1/2 H | 23 |
| Office, Assistant Division Chief | CP | RB | GWB-W |  |  |  |  |  | AT | 1/2 H | 23 |
| Central Reproduction Activity | VCT | RB | GWB1 |  |  |  |  |  | AT1 | 1/2 K | 69 |
| Centralized Transcription Activity | CP | RB | GWB |  |  |  |  |  | AT | 1/2 K | 69 |
| Agent Cashier | CP | RB | GWB2 |  |  |  |  |  | AT2 | 36 H | 157 |
| CHAPTER 862 - SUPPORT FACILITIES |  |  |  |  |  |  |  |  |  |  |  |
| Special Projects Room | CP | RB | GWB1 |  |  |  |  |  | AT1 | 1/2 J-T | 69 |
| Office, Credit Union | CP | RB | GWB |  |  |  |  |  | AT2 | 1/2 H-T | 23 |
| Lounge, Employee | CP | RB | GWB-W |  |  |  |  |  | AT | 1/2 K-T | 69 |
| Lobby | See Chapter 244 |  |  |  |  |  |  |  |  |  |  |
| Office, Union Official | CP | RB | GWB-W |  |  |  |  |  | AT | 1/2 H | 23 |
| CHAPTER 870 - SERVICE ORGANIZATIONS |  |  |  |  |  |  |  |  |  |  |  |
| Computer (Target) Video Display and Printer | CP | RB | GWB |  |  |  | AT |  |  | 1/2 H-T | 69 |

| INTERIOR COLOR AND ROOM FINISH SCHEDULE (NEW CONSTRUCTION) |  |  |  |  |  |  |  |  |  |  |  | DATE: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROOM NO. | ROOM NAME | FLOOR |  |  | BASE |  | WALL |  | WAINSCOT |  |  | CEILING | REMARKS |
|  |  | MATERIAL | CC |  | MATERIAL | CC | MATERIAL | CC | MATERIAL | CC | MATERIAL | CC |  |
|  |  |  |  | N |  |  |  |  |  |  |  |  |  |
|  |  |  |  | E |  |  |  |  |  |  |  |  |  |
|  |  |  |  | S |  |  |  |  |  |  |  |  |  |
|  |  |  |  | W |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N |  |  |  |  |  |  |  |  |  |
|  |  |  |  | E |  |  |  |  |  |  |  |  |  |
|  |  |  |  | S |  |  |  |  |  |  |  |  |  |
|  |  |  |  | W |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N |  |  |  |  |  |  |  |  |  |
|  |  |  |  | E |  |  |  |  |  |  |  |  |  |
|  |  |  |  | S |  |  |  |  |  |  |  |  |  |
|  |  |  |  | W |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N |  |  |  |  |  |  |  |  |  |
|  |  |  |  | E |  |  |  |  |  |  |  |  |  |
|  |  |  |  | S |  |  |  |  |  |  |  |  |  |
|  |  |  |  | W |  |  |  |  |  |  |  |  |  |

| INTERIOR COLOR AND ROOM FINISH SCHEDULE (RENOVATION) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROOM NO. | ROOM NAME | FLOOR |  |  |  | BASE |  |  | WALL |  |  | WAINSCOT |  |  | CEILING |  |  | REMARKS |
|  |  | MATERIAL |  | CC |  | MATERIAL |  | CC | MATERIAL |  | CC | MATERIAL |  | CC | MATERIAL |  | CC |  |
|  |  | EX | NEW |  |  | EX | NEW |  | EX | NEW |  | EX | NEW |  | EX | NEW |  |  |
|  |  |  |  |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | S |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | W |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | S |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | W |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | S |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | W |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | S |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | W |  |  |  |  |  |  |  |  |  |  |  |  |  |

See VHA Program Guide PG-18-3, Topic 11, "Noise Transmission Control".

2 Intermediate landing to be rubber flooring, stair treads to be molded rubber.

Door from isolation room shall swing into anteroom.

2  Full height CT at Tub/ Shower on concrete backer board with PL ceiling.

* Extend enclosed walls to underside of floor slab above and fully seal them to maintain proper pressure relationship and direction of air flow.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

Sliding glass doors with flush bottom rail, or swinging door(s).

2 See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

* Extend enclosed walls to underside of floor slab above and fully seal them to maintain proper pressure relationship and direction of air flow.

CT full height at tub on concrete backer.

See Architectural Design Manual PG-18-10, Article 3.4, "Automatic Doors".

See Architectural Design Manual PG-18-10, Article 3.4 "Automatic Doors".

* Extend enclosed walls to underside of floor slab above and fully seal them to maintain proper pressure relationship and direction of air flow.

Provide acoustical/ tackable wall fabric on one wall.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Architectural Design Manual PG-18-10, "Automatic Doors".

CT full height in shower on concrete backer board.

2 See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

3 See Standard Details PG-18-4 for radiation shielding.

1 See Design and Construction Procedures PG-18-3, Topic 14,"Physical Security Requirements and Options".

2 Veneer Plywood Surface.

3 See Architectural Design Manual PG-18-10, "Automatic Doors".

1 See Design and Construction Procedures PG-18-3, Topic 14,"Physical Security Requirements and Options".

2 CT full height in shower on concrete backer board.

3 See Architectural Design Manual PG-18-10, "Automatic Doors".

See Design and Construction Procedures PG-18-3, Topic 14, "Physical Security Requirements and Options".

2 See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

3 Provide acoustical/ tackable wall covering on one wall.

21 CT full height tub/ shower on concrete backer board.

2 See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Architectural Design Manual PG-18-10, "Automatic Doors".

One wall of 3/4" hardwood veneer plywood.

Double Layer GWB on walls.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

1 See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

1

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Design and Construction Procedures PG-18-3, Topic 6, "Floor Slab Depressions".

See Design and Construction Procedures PG-18-3, Topic 14, "Physical Security Requirements and Options".

2 Full height partition to underside of structure above.

3 See Architectural Design Manual PG-18-10, "Automatic Doors".

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Architectural Design Manual PG-18-10, Chapter 5, "X-Ray Radiation Shielding and Special Control Room Requirements".

23  See Standard Details PG-18-4 for shielding.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

Double-acting doors with dual hold-open features at entrances to oral surgery.

2 See Standard Details PG-18-4 for shielding requirements.

3 Provide 4 ft. wide metal cased opening for all intra-service Operatories.

4 Provide a 24” sliding door (manual) between Oral Surgery Rooms.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

Prefabricated, walk-in refrigerator with finished floor flush with adjoining floor. See Design and Construction Procedures PG-18-3, Topic 6, "Floor Slab Depressions".

See Standard Details PG-18-4.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Section II, "Doors and Hardware".

These floors shall be finished with a colored hardener (non-slip).

Provide acoustical/tackable surface on one wall.

2   See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See HVAC Design Manual PG-18-10, Article 4.11, "Walk-in Refrigerators and Freezers".

See Design and Construction Procedures PG-18-3, Topic 6, "Floor Slab Depressions".

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

Provide tackable surface on one wall.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Architectural Design Manual PG-18-10, "Automatic Doors".

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

3 Full height CT at tub/ shower on concrete backer board with PL ceiling.

4 Plaster on high rib metal lath.

2

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

1

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Design and Construction Procedures PG-18-3, Topic 11,"Noise Transmission Control".

2 Full height CT on concrete backer board.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2	 See Design and Construction Procedures PG-18-3, Topic 14, "Physical Security Requirements and Options".

1 See Design and Construction Procedures PG-18-3, Topic 14, "Physical Security Requirements and Options".

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Design and Construction Procedures PG-18-3, Topic 14, "Physical Security Requirements and Options".

Provide 1-Hour rated enclosure and 6” perimeter concrete curb cast integrally with floor, in accordance with NFPA.

See Design and Construction Procedures PG-18-3, Topic 14, "Physical Security Requirements and Options".

1

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

Provide acoustical/ tackable wall fabric on one wall.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

Apply fabric type acoustical treatment to two walls.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

Provide acoustical/tackable wall fabric on one wall.

3 Use X door where VAMC includes SCI unit.

4 See Architectural Design Manual PG-18-10, Chapter 5, "Electromagnetic Shielding".

Provide X door if VAMC includes SCI Unit.

Provide X door if VAMC includes SCI Unit.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 Provide acoustical/ tackable wall fabric on one wall.

3 See Architectural Design Manual PG-18-10, Chapter 5, "X-Ray Radiation Shielding and Special Control Room".

4 2’x2’ acoustic tiles to be used in these areas.

5 X width door to be used where VAMC includes an SCI unit.

See Architectural Design Manual PG-18-10, Chapter 5 "X-Ray Radiation Shielding and Special Control Room Requirements".

2’x2’ acoustic tiles shall be used in these areas.

X width door to be used where VAMC includes an SCI unit.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Architectural Design Manual PG-18-10, Chapter 5 for radiation shielding requirements.

2 See Architectural Design Manual PG-18-10, Chapter 5 for electromagnetic shielding.

Provide acoustical/tackable fabric on one wall.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See HVAC Design Manual PG-18-10, Article 4.11, "Walk-In Refrigerators".

2 Provide one-hour rated in this area.

3 Provide one-hour rated enclosure and 6" perimeter concrete curb cast integrally with floor, in accordance with NFPA requirements.

See Section II, "Doors and Hardware".

To be determined by project requirements.

1 See HVAC Design Manual PG-18-10, Article 4.11, "Walk-in Refrigerators".

To be determined by project requirements.

1 Provide door interlocks at Air Lock.

2 Use sound rated doors in Veterinary Medical Units where necessary.

3 To be determined by project requirements.

4 See Section II, "Doors and Hardware".

Mount CT on concrete backer board.

2 Extend QT 10’ beyond sterilizers.

Use lead lined doors if room contains fixed x-ray units.

Doors from clean core into operating rooms.

See Architectural Design Manual PG-18-10, paragraph 4.3.3.2, for lead lining".

Doors from clean core into operating rooms.

3 See Architectural Deign Manual PG-18-10, Chapter 5, for electromagnetic shielding.

4 Doors from support spaces into operating rooms may need to be lead lined.

5 See Section II, "Doors and Hardware".

Doors from support spaces into operating rooms may need to be lead lined.

See Architectural Design Manual PG-18-10, Chapter 5, for electromagnetic shielding.

CT on concrete backer board.

2 See Architectural Design Manual PG-18-10, Chapter 5, for lead lining in X-Ray Rooms.

1 See Architectural Design Manual PG-18-10, Chapter 3, "Automatic Doors".

2 See Architectural Design Manual PG-18-10, Chapter 5, for lead lining in X-Ray Rooms.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

Full height ceramic tile on concrete backer board at tub/shower with plaster ceiling.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

Provide acoustical/ tackable wall surface on one wall.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

Provide acoustical/tackable surface on one wall.

As required.

Barrier wall furnished by equipment contractor.

Full height partition.

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

One wall to receive acoustical/ tackable wall surface.

1 See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

2 See Design and Construction Procedures PG-18-3, Topic 14, "Physical Security Requirements and Options".

See Design and Construction Procedures PG-18-3, Topic 11, "Noise Transmission Control".

See Design and Construction Procedures PG-18-3, Topic 14, "Physical Security Requirements and Options".