A/E REVIEW CHECKLIST

ARCHITECTURAL

Reviewers should - Use Checklists when reviewing any type of VA construction project for the following disciplines: 
  Site and Landscape, 
  Architectural, 
  Structural, 
  Plumbing, Fire Protection, and 
        Sanitary, 
  Heating, Ventilating, and Air 
        Conditioning (HVAC), 
  Steam Generation, 
  Steam Distribution,
  Incineration/Solid Waste, and
  Electrical.

Reviewers should - Insure that A/E Submission Instructions (PG-18-15) for Schematic, Design Development, and Construction Documents are followed for various types of VA construction projects.

Reviewers should - Insure that every VA construction project is in compliance with all life safety issues.

Reviewers should - Be aware that these checklists are not all-inclusive but only provide minimum review items.

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| GENERAL INFORMATION FOR REVIEWERS |  |
| --- | --- |
| ARCHITECTURAL REVIEW |  |
| The reviewer should be thoroughly familiar with the following VA standards before conducting a design review. These are available on Internet/Intranet: http://www.va.gov/facmgt/standard/va_gov.htm http://vaww.va.gov/facmgt/standard/va_gov.htm |  |
| 1. | DESIGN MANUALS (PG-18-10) |
| 2. | # MASTER CONSTRUCTION SPECIFICATIONS (PG-18-1) |
| 3. | STANDARD DETAILS (PG-18-4) (Available in AutoCAD 2000 format) |
| 4. | DESIGN AND CONSTRUCTION PROCEDURES (formerly Construction Standards) (H-18-3) (Policies defining the minimum level of excellence in the design of VA facilities) |
| 5. | # DESIGN GUIDES (PG-18-12) (Graphic information on specific programs in the design development of VA facilities) |
| 6. | # DESIGN ALERTS (These alerts are issued on a regular basis for design and construction related issues) |
| 7. | # A/E QUALITY ALERTS (These alerts are issued to guard against common and repeat design errors) |
| 8. | A/E SUBMISSION INSTRUCTIONS, PROGRAM GUIDE, PG-18-15 |
| 9. | TECHNICAL SUMMARIES (The summaries of HVAC design requirements for special and critical areas) |
| 10. | VA NCS APPLICATION GUIDE |
| 11. | NCS (NATIONAL CAD STANDARD) |

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| LIFE SAFETY ISSUES |  |  |
| --- | --- | --- |
| ARCHITECTURAL REVIEW |  |  |
| NO. | ITEM | COMMENTS/ YES/NO/NA |
| 1 | The design of all fire safety features in new construction and renovation at VA facilities shall comply with the latest edition (the edition in effect at the bid date) of the National Fire codes (a compilation of National Fire Protection Association Codes, Standards, Recommended Practices, and Manuals). Fire protection features not included in the National Fire Codes shall be designed in accordance with the requirements of the latest edition of the Uniform Building Code (International Conference of Building Officials). |  |
| 2 | For existing buildings, the Fire Protection Engineering consultant may propose equivalent fire safety protection. The consultant shall document equivalency requests with sufficient rationale in the fire protection report for the project. VA’s Authority Having Jurisdiction shall approve all equivalency requests prior to the equivalent method being incorporated into the design. |  |
| 3 | Every project should have a comprehensive fire protection design plan which includes protection throughout the building by an approved automatic sprinkler system. A fire protection report, including fire protection drawings that are developed during the design development stage of the project, shall support the fire protection plan. The fire protection report shall include related design considerations and criteria that have been coordinated among all the affected disciplines, and shall serve as the basis for the design, construction, and future operation of the building. |  |

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| SCHEMATIC 1 |  |  |
| --- | --- | --- |
| ARCHITECTURAL REVIEW |  |  |
| NO. | ITEM | COMMENTS/ YES/NO/NA |
| 1 | Compliance with A/E Submission Instructions (PG-18-15) |  |
| 2 | Architectural narrative - Explain alternative designs and their proposed exterior materials |  |
| 3 | Schematic block plans showing: |  |
|  | a. All floors (new and renovated) |  |
|  | b. Penthouse |  |
|  | c. Roof plan |  |
|  | d. Pipe basement |  |
|  | e. Key plan, title block, north arrow, and scale on each floor plan |  |
|  | f. Temporary index of drawings |  |
|  | Exterior dimensions/total building gross area: |  |
|  | a. Circulation |  |
|  | b. Mechanical areas |  |
|  | c. Electrical Areas |  |
|  | d. Exterior walls |  |
|  | Size and shape of all departmental functions and services: |  |
|  | a. Label each service or activity |  |
|  | b. Indicate boundaries |  |
|  | c. Show activity code number |  |
| 4 | Location of: |  |
|  | a. Entrances to buildings |  |
|  | b. Main entry point to each functional block |  |
|  | c. Mechanical spaces |  |
|  | d. Expansion joints |  |
|  | e. Structural grid |  |
|  | f. Special seismic structural features |  |
| 5 | Finish floor elevations |  |
| 6 | Finish grades at corners, entrances, exits, platforms, and ramps |  |
| 7 | Schematic section of building |  |
| 8 | Lead abatement |  |
| 9 | Compliance with VA NCS Application Guide and applicable NCS modules. |  |

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| SCHEMATIC 2 |  |  |
| --- | --- | --- |
| ARCHITECTURAL REVIEW |  |  |
| NO. | ITEM | COMMENTS/ YES/NO/NA |
| 1 | Compliance with A/E Submission Instructions (PG-18-15) |  |
| 2 | Narrative for building construction, fire and smoke separation, fire sprinkler/standpipe systems, pump sizes, water supply available/maximum demand, water flow testing results, fire alarm systems, and kitchen extinguishing systems. |  |
| 3 | Floor Plans/Drawings; |  |
|  | a. All floors (new and renovated) |  |
|  | b. Penthouse |  |
|  | c. Roof plan |  |
|  | d. Pipe basement |  |
| 4 | Location of: |  |
|  | a. Rooms with room names and numbers |  |
|  | b. Doors |  |
|  | c. Corridors |  |
|  | d. Basic column/grid sizes |  |
|  | e. Expansion and seismic joints |  |
|  | f. Electrical closets |  |
|  | g. Equipment rooms |  |
|  | h. Signal and telephone closets |  |
|  | i. Mechanical shafts and space |  |
|  | j. Stairs |  |
|  | k. Ramps |  |
|  | l. Elevators |  |
|  | m. Automatic conveyances |  |
|  | n. Trash and linen chutes |  |
|  | o. Fire and smoke rated partitions |  |
| 5 | Program net area/designated net area |  |
| 6 | Exterior dimensions/total building gross area |  |
| 7 | Each integral or fragmented service outlined with name and code |  |

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| SCHEMATIC 2 |  |  |
| --- | --- | --- |
| ARCHITECTURAL REVIEW |  |  |
| NO. | ITEM | COMMENTS/ YES/NO/NA |
| 8 | Layout of critical areas (1:50 - 1/4 in.) showing: |  |
|  | a. Minimum clearances around patient beds |  |
|  | b. Correct bed sizes |  |
|  | c. Dimensions of patient wardrobes in mental health units |  |
|  | d. Accessible toilet and bedrooms |  |
| 9 | Exterior building elevations: |  |
|  | a. Significant materials including colors |  |
|  | b. Massing |  |
|  | c. Fenestrations |  |
|  | d. Relationship to adjacent structures |  |
|  | e. Finish grades |  |
|  | f. Floor-to-floor heights |  |
|  | g. Floor elevations |  |
|  | h. Exterior materials |  |
|  | i. Canopies at entrances and loading docks (minimum clearances) |  |
| 10 | Building section |  |
| 11 | Coordinate all drawings with other technical disciplines |  |
| 12 | Compliance with VA NCS Application Guide and applicable NCS modules. |  |

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| Design Development 1 (DD1) |  |  |
| --- | --- | --- |
| ARCHITECTURAL REVIEW |  |  |
| NO. | ITEM | COMMENTS/ YES/NO/NA |
| 1 | Compliance with A/E Submission Instructions (PG-18-15) |  |
| 2 | Incorporate all of the revisions required from S2 Review |  |
| 3 | Floor Plans/Drawings; |  |
|  | a. All floors (new and renovated) |  |
|  | b. Penthouse |  |
|  | c. Roof plan (mechanical equipment and architectural screens) |  |
|  | d. Pipe basement (emergency exit, ventilation, and stairs) |  |
|  | e. Pipe tunnel |  |
|  | f. Equipment floor plans 1:50 (1/4 inch) scale |  |
|  | g. Fire Protection plan |  |
|  | (1) Room names and numbers |  |
|  | (2) Door locations and swings |  |
|  | (3) Smoke and fire rated partitions |  |
| 4 | Location of: |  |
|  | a. Rooms with room names and numbers |  |
|  | b. Doors and door swings |  |
|  | c. Corridors |  |
|  | d. Basic column/grid sizes |  |
|  | e. Expansion and seismic joints |  |
|  | f. Electrical closets |  |
|  | g. Signal and telephone closets |  |
|  | h. Equipment rooms |  |
|  | i. Mechanical shafts and space |  |
|  | j. Plumbing fixtures |  |
|  | k. Stairs |  |
|  | l. Ramps |  |
|  | m. Elevators |  |
|  | n. Automatic conveyances |  |
|  | o. Trash and linen chutes |  |
|  | p. Fire and smoke rated partitions |  |
|  | q. Fire extinguisher cabinets |  |

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| Design Development 1 (DD1) |  |  |
| --- | --- | --- |
| ARCHITECTURAL REVIEW |  |  |
| NO. | ITEM | COMMENTS/ YES/NO/NA |
| 5 | Wall thickness and chase walls |  |
| 6 | Handrail locations and dimensions (where required) |  |
| 7 | Program net area/designated net area |  |
| 8 | Exterior building elevations: |  |
|  | a. Significant materials including colors |  |
|  | b. Massing |  |
|  | c. Fenestrations |  |
|  | d. Relationship to adjacent structures |  |
|  | e. Finish grades |  |
|  | f. Floor-to-floor heights |  |
|  | g. Floor elevations |  |
|  | h. Canopies at entrances and loading docks (minimum clearances) |  |
|  | i. Future expansion |  |
| 9 | Building sections (same scale as floor plans): |  |
|  | a. Finish floor elevations |  |
|  | b. Relationship of finish ground floor to finish grade at major entrances |  |
|  | c. Floor elevation alignment if building abuts existing structure |  |
| 10 | Wall sections showing construction, building materials and systems, and proposed sill and head heights of openings |  |
| 11 | Finish schedule |  |
| 12 | Specification Section 09050 for materials and products for interior design |  |
| 13 | Coordinate all drawings with other technical disciplines |  |
| 14 | Compliance with VA NCS Application Guide and applicable NCS modules. |  |

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| Design Development 2 (DD2) |  |  |
| --- | --- | --- |
| ARCHITECTURAL REVIEW |  |  |
| NO. | ITEM | COMMENTS/ YES/NO/NA |
| 1 | Compliance with A/E Submission Instructions (PG-18-15) |  |
| 2 | Incorporate all of the revisions required from DD1 Review |  |
| 3 | Floor Plans/Drawings; |  |
|  | a. All floors (new and renovated) |  |
|  | b. Penthouse |  |
|  | c. Roof plan (mechanical equipment and architectural screens) |  |
|  | d. Pipe basement (emergency exit, ventilation, and stairs) |  |
|  | e. Pipe tunnel |  |
|  | f. Ceiling plan |  |
|  | (1) Ceiling mounted equipment |  |
|  | (2) Lighting fixtures |  |
|  | (3) Air diffusers |  |
|  | (4) Registers |  |
|  | (5) Tracks |  |
|  | (6) Room names and numbers |  |
|  | g. Fire Protection plan add: |  |
|  | (1) main fire alarm panel |  |
|  | (2) fire extinguisher cabinets |  |
|  | (3) Areas not protected by automatic sprinklers |  |
|  | h. Equipment floor plans 1:50 (1/4 inch) scale coordinated with VA Equipment Guide List (PG 7610) |  |
| 4 | Location of: |  |
|  | a. Rooms with room names and numbers |  |
|  | b. Doors and door swings |  |
|  | c. Corridors with dimensions |  |
|  | d. Column grid lines/grid sizes |  |
|  | e. Expansion and seismic joints |  |
|  | f. Electrical closets |  |
|  | g. Signal and telephone closets |  |
|  | h. Equipment rooms |  |
|  | i. Mechanical shafts and space |  |

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| Design Development 2 (DD2) |  |  |
| --- | --- | --- |
| ARCHITECTURAL REVIEW |  |  |
| NO. | ITEM | COMMENTS/ YES/NO/NA |
| 4 (cont.) | j. Plumbing fixtures |  |
|  | k. Fixed equipment |  |
|  | l. Stairs with dimensions |  |
|  | m. Ramps |  |
|  | n. Elevators |  |
|  | o. Automatic conveyances |  |
|  | p. Trash and linen chutes |  |
|  | q. Fire and smoke rated partitions |  |
|  | r. Lead-lined and radio-frequency shielded partitions |  |
|  | s. Fire extinguisher cabinets |  |
|  | t. Water coolers |  |
|  | u. Floor drains and pitch |  |
|  | v. wheelchair accessible facilities |  |
| 5 | Wall thickness and chase walls |  |
| 6 | Handrail locations and dimensions (where required) |  |
| 7 | Program net area/designated net area |  |
| 8 | Exterior building elevations: |  |
|  | a. Significant materials including colors |  |
|  | b. Massing |  |
|  | c. Fenestrations |  |
|  | d. Relationship to adjacent structures |  |
|  | e. Finish grades |  |
|  | f. Floor-to-floor heights |  |
|  | g. Floor elevations |  |
|  | h. Expansion and control joints |  |
|  | i. Roof top equipment and architectural screens |  |
|  | j. Areaways and below grade openings |  |
|  | k. Downspouts and gutters |  |
|  | h. Canopies at entrances and loading docks (minimum clearances) |  |
|  | i. Future expansion |  |

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| Design Development 2 (DD2) |  |  |
| --- | --- | --- |
| ARCHITECTURAL REVIEW |  |  |
| NO. | ITEM | COMMENTS/ YES/NO/NA |
| 9 | Building sections (same scale as floor plans): |  |
|  | a. Finish floor elevations |  |
|  | b. Relationship of finish ground floor to finish grade at major entrances |  |
|  | c. Floor elevation alignment if building abuts existing structure |  |
|  | d. Floor-to-floor heights |  |
|  | e. Pipe basements and roof mechanical equipment |  |
|  | d. Section through elevator shaft |  |
|  | e. Section through stairways |  |
| 10 | Wall sections showing construction, building materials and systems, and proposed sill and head heights of openings |  |
| 11 | Finish schedule with Specification Section 09050 edited and complete |  |
| 12 | Specification Section 09050 for materials and products for interior design |  |
| 13 | Sample specifications sections edited in pencil |  |
| 14 | Equipment Activation List |  |
| 15 | Coordinate all drawings with other technical disciplines |  |
| 16 | Compliance with VA NCS Application Guide and applicable NCS modules. |  |

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| Construction Documents (CD1) |  |  |
| --- | --- | --- |
| ARCHITECTURAL REVIEW |  |  |
| NO. | ITEM | COMMENTS/ YES/NO/NA |
| 1 | Compliance with A/E Submission Instructions (PG-18-15) |  |
| 2 | Incorporate all of the revisions required from DD2 Review |  |
| 3 | Floor Plans/Drawings; |  |
|  | a. Room names and numbers |  |
|  | b. Interior dimensions |  |
|  | c. Exterior dimensions and match lines |  |
|  | d. Column grid lines |  |
|  | e. Expansion joints between new and existing |  |
|  | f. Floor drains and pitch |  |
|  | g. Corner guards, handrails, bumperguards, and grab bars |  |
|  | h. Water coolers (near waiting rooms and along corridors) |  |
|  | i. Air-lock vestibules at main entrance |  |
|  | j. Elevator lobbies (min 12" deep) |  |
|  | k. Ducts and utility shafts that pass through floors |  |
|  | l. Transformer rooms with curbs and no overhead pipes or ducts |  |
|  | m. Roof slope is minimum 1/4" per foot |  |
|  | n. Roof-top equipment with screens |  |
|  | o. Access doors to roof |  |
|  | p. Walkways on roof |  |
|  | q. Lightning protection |  |
|  | r. Door symbols |  |
|  | s. Lead-lines partitions and "universal floor T's" |  |
|  | t. Location and details of Dedication Plaque |  |
|  | u. Floor slab depressions identified and coordinated with structural drawings |  |
|  | v. Areaways with rails or gratings |  |
| 4 | Demolition Plans with existing finish schedule and notes |  |
| 5 | Reflected Ceiling Plan |  |
| 6 | Equipment Plans with elevations and details |  |

A/E CHECKLIST

TITLE________________________________PROJECT NO.

LOCATION___________________________________DATE

REVIEWED BY

ORGANIZATION

| Construction Documents (CD1) |  |  |
| --- | --- | --- |
| ARCHITECTURAL REVIEW |  |  |
| NO. | ITEM | COMMENTS/ YES/NO/NA |
| 7 | Building sections, wall sections, and exterior elevations showing: |  |
|  | a. Finish floor elevations |  |
|  | b. All building systems |  |
|  | c. Materials |  |
|  | d. Future expansion |  |
| 8 | Fire Protection Plan: |  |
|  | a. Details of stairwell sign |  |
|  | b. Door and window schedule with fire rating and fire rated glazing |  |
|  | c. Location of main annunciator panel |  |
|  | d. Height and configuration of storage racks and shelving |  |
|  | e. Exit calculations |  |
|  | Graphics and Signage Plan |  |
|  | Interior details, elevations, and sections |  |
|  | Finish schedule |  |
|  | Architectural specifications edited and marked in pencil |  |
|  | Drafting symbols, abbreviations, general notes, and schedules |  |
| 9 | Coordinate all drawings with other technical disciplines |  |
| 10 | Compliance with VA NCS Application Guide and applicable NCS modules. |  |