| Table 5-12 Summary of TA-50 Radionuclide, Nitrate, and Fluoride Discharges 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ______________2001______________ |  |  | ______________2002______________ |  |  | ______________2003______________ |  |  |
| Radionuclide | Total annual activity (mCi) | Mean activity (pCi/L) | Ratio of activity to DCG2 | Total annual activity (mCi) | Mean activity (pCi/L) | Ratio of activity to DCG2 | Total annual activity (mCi) | Mean activity (pCi/L) | Ratio of activity to DCG2 |
| 3H | 126 | 9,297 | 0.0046 | 78.98 | 7,174 | 0.0036 | 117.1 | 10,400 | 0.005 |
| 241Am | 0.056 | 4.11 | 0.1370 | 0.114 | 10.368 | 0.3456 | 0.055 | 4.9 | 0.163 |
| 137Cs | 0.213 | 15.7 | 0.0052 | 0.582 | 52.827 | 0.0176 | 0.614 | 54.6 | 0.018 |
| 238Pu | 0.074 | 5.46 | 0.1365 | 0.073 | 6.669 | 0.1667 | 0.109 | 9.7 | 0.243 |
| 239,240Pu | 0.024 | 1.79 | 0.0597 | 0.024 | 2.187 | 0.0729 | 0.043 | 3.8 | 0.127 |
| 89Sr | 0.039 | 2.91 | 0.0001 | 0.004 | 0.3498 | 0.0000 | 0.000 | 0.0 | 0.000 |
| 90Sr | 0.029 | 2.14 | 0.0021 | 0.003 | 0.2997 | 0.0003 | 0.000 | 0.0 | 0.000 |
| 234U | 0.027 | 2.03 | 0.0041 | 0.006 | 0.5738 | 0.0011 | 0.000 | 0.0 | 0.000 |
| 235U | 0.0016 | 0.12 | 0.0002 | ND3 | ND | ND | 0.000 | 0.0 | 0.000 |
| Constituent | Total annual mass (kg) | Mean concentration (mg/L) | Ratio of concentration to MCL4 | Total annual mass (kg) | Mean concentration (mg/L) | Ratio of concentration to MCL | Total annual mass (kg) | Mean concentration (mg/L) | Ratio of concentration to MCL |
| NO3+NO2-N | 52.5 | 3.86 | 0.39 | 17.31 | 1.54 | 0.16 | 12.8 | 1.13 | 0.11 |
| F | 9.96 | 0.73 | 0.46 | 5.07 | 0.46 | 0.29 | 4.3 | 0.38 | 0.24 |
| ClO4 | 2.29 | 0.169 | No standard | 0.175 | 0.016 | No standard | 0.0 | 0.0 | No standard |
| Total annual effluent volume (107 liters) | 1.36 |  |  | 1.10 |  |  | 1.12 |  |  |
| 1Compiled from Radioactive Liquid Waste Group (FWO-WFM) Annual Reports. 2Public dose limit 3ND- nondetect 4New Mexico Groundwater Limit |  |  |  |  |  |  |  |  |  |