| Company Name: | Equipment/Job Identification: PLANT MAINTENANCE CONVEYOR BELT CHANGE-OUT Type of Equipment: CIRCUIT 1 BELT 2 Make: JEFFREY Model: 24-INCH BELT CONVEYOR Year: 1956 Use: RAW STONE |
| --- | --- |
| Mine Name: |  |
| Date of Analysis |  |

# Pre-Assessment

### List pre-requisites here

Personal Protective Equipment

Proper Equipment/Tools

General Safety Training

Company Policies

Back Safety and Proper Lifting Procedures

HazCom

New Miner/Experienced Miner Training

Pre-Operational Inspection

Location of Safety Equipment and First Aid Supplies

Fire Extinguishers

Name of First Responders

Names of the Safety Committee

# Duty 1: Start of Shift (Pre-plan) Activities

Learner will explain the importance of communicating and exchanging information as part of the start of shift (pre-plan activities). Learner will also explain the job duties, why they are conducted, any associated risk, and how to implement company policies and required regulations. A thorough start of shift activities includes the following job steps:

| Job Steps | Importance Narrative (Consider Safety, Production, Maintenance) | Importance Ranking 1=Important 2=Very Important 3=Critical | Satisfactory or Needs Work | Procedures/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Check in (Time Clock) |  |  |  |  |
| Put on work clothes |  |  |  |  |
| - Steel tip shoes | To avoid injuries and company policy |  |  | Must be minimum 6” boot |
| Report to lunch room |  |  |  |  |
| Receive job assignment from supervisor |  |  |  |  |
| - Obtain work order |  |  |  | Work order number for time card and for charge card Equipment number Work order description Who originated the work order |
| - Discuss special instructions as needed | Efficiency of the job and the safety of the task |  |  |  |
| - Discuss safety issues of job | Efficiency of the job and the safety of the task and the people involved |  |  |  |
| - Inquire if job area has been cleaned |  |  |  |  |
| Walk to shop |  |  |  |  |
| - Watch for traffic |  |  |  |  |
| - Watch for wet/icy surfaces | Safety concern/could cause personal injury |  |  |  |
| Obtain PPE located in shop | Personal protection/company policy |  |  |  |
| - Get hard hat | Personal protection from serious injury |  |  |  |
| - Get hearing protection |  |  |  |  |
| - Ear plugs |  |  |  |  |
| - Muffs |  |  |  |  |
| - Get gloves |  |  |  |  |
| - Get respiratory protection |  |  |  |  |
| - Get safety glasses | Personal protection from serious injury/company policy |  |  |  |
| - Get safety locks | Could lead to death/serious injury and equipment damage/company policy |  |  |  |
| Receive further job instructions from lead person (Clip process/Vulcanizing process) |  |  |  |  |
| - Assign mobile equipment |  |  |  | Pre-shift inspections Show Pre-Operational power point presentations |
| - Fork lift/gradall |  |  |  |  |
| - Repair truck |  |  |  |  |
| - Obtain materials and parts | Can’t perform job |  |  | Complete charge out slips Turn charge slips into store house |
| - Use fork lift to remove conveyor belt (24”) from storage area |  |  |  | Prefer using fork lift |
| - Get belt clips (190 type clips) |  |  |  | Explain why this size/type of clip is used |
| - Get belt tape |  |  |  |  |
| Obtain tools | Without proper tools you can not do the job |  |  | Radios – a minimum of three plant radios required, Two belt clamps/blocks, 24-inch, 190 template, Framing square, four come-a-longs, one chain wrench (3-ton), two 5/8-inch x 4 ft. slings, two chains (3/4-inch), two ½-inch drive electric impact wrenches, #1 belt punch, #1 nut driver, two stud breaker bars, one 4-1/2 inch electric grinder, face shield, one claw hammer, Two utility knives with a spare blade Combination wrench set, socket set (1/2-inch drive), Light cord with gfci Shaft and bearing assembly Two clevises Two 5/16-inch chains ½-inch cable (minimum 25 ft.) Six ½-inch cable clamps Danger tape, Torch and burning goggles Belt clevises, two 2” x 12” x 36” boards/planks, one spool of 3/4” hemp rope, punch quick connect for impact wrench, and tool bag – each person is responsible for their own tool bag |
| Call Central Control | Critical to the safety of the work crew/company policy |  |  | Review company policy |
| - Report names |  |  |  |  |
| - Report clock numbers |  |  |  |  |
| - Report equipment I. D. being worked on | Central Control has to be informed |  |  |  |
| - Report what job description |  |  |  |  |
| - Have control room place CRT. #1, belt #1 and #2 in the “jog” mode |  |  |  |  |
| Perform mobile equipment pre-shift inspection | Prevention of serious injury, damage to equipment |  |  |  |
| - Fill out pre-shift inspection check list |  |  |  |  |
| - Report or correct any problems | Serious injury, damage to equipment |  |  |  |
| - Refuel equipment as needed |  |  |  | Discuss HazCom regulations |
| Load materials and tools onto truck |  |  |  |  |
| Load belt onto forklift or gradall | Hard to transport to the job |  |  | Discuss qualifications to operate equipment |
| Proceed to job site |  |  |  |  |
| - Observe speed limit |  |  |  |  |
| - Watch for foot traffic |  |  |  |  |
| - Watch for equipment right-of-way | Prevent personal injury |  |  | Stay 40 feet from mobile equipment while walking |
| - Beware of restricted areas |  |  |  |  |

# Duty 2:   Perform the Job Activities

Learner will demonstrate how to safely perform the job and explain the importance of determining the work to be performed to the trainer.  Learner will also explain the job duties, why they are conducted, any associated risk and the results of committing unsafe acts on the job.  A thorough performing the job activities includes the following job steps:

| Job Steps | Importance Narrative (Consider Safety, Production, Maintenance) | Importance Ranking 1=Important 2=Very Important 3=Critical | Satisfactory or Needs Work | Procedures/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Arrive at job site |  |  |  |  |
| Observe work area upon approach |  |  |  |  |
| Park truck in safe area |  |  |  |  |
| - Chock wheels |  |  |  |  |
| Perform work place examination |  |  |  |  |
| Correct or report safety defects | When not corrected or reported could lead to personal injury or damage to equipment |  |  |  |
| Verify that crew is at correct belt by visual observation |  |  |  |  |
| Lock equipment out | Could result in loss of life/ Machinery start-up |  |  | Each individual applies his/her own locks Review company lock out policy |
| Apply locks on CRT. #1 Belt #1 |  |  |  |  |
| Try to jog belt #1 to make sure belt will not start |  |  |  |  |
| Apply locks on CRT. #1 Belt #2 | May lead to injury or loss of life |  |  |  |
| Try to jog belt #2 to make sure belt will not start |  |  |  |  |
| Unload tools at tail pulley at Belt #2 | Will delay or hamper job |  |  |  |
| - All tools except 3-ton chain wrench and ¾” chain |  |  |  |  |
| - Bring adjustable wrenches |  |  |  |  |
| Remove wheel chock and drive to head pulley area |  |  |  |  |
| Park truck and chock wheels |  |  |  |  |
| Unload tools at gravity take-up |  |  |  |  |
| Remove the gravity take-up guards | Can’t do the job without this step |  |  |  |
| Install the chain on beam above the center line of the gravity take-up weight |  |  |  |  |
| Connect 3-ton chain wrench to the chain | Can’t do the job without this step |  |  | Verify safety latches on hooks Weight limitations in effect |
| Connect the chain wrench lifting hook into the gravity weight take-up lug |  |  |  | Verify safety latches on hooks |
| Lift the gravity take-up into the half way position using the 3-ton chain wrench | Position of the take-up is important to the job |  |  | Explain this procedure |
| Close the gravity take-up guards |  |  |  |  |
| Back off tension on belt scrapers |  |  |  |  |
| Remove chocks and drive back to the tail pulley |  |  |  |  |
| Park truck and chock wheels |  |  |  |  |
| Use fork lift/gradall to put belt in position at tail pulley area for installation |  |  |  |  |
| Install shaft /bearings assembly into the core of the new belt |  |  |  | Verify belt is not hung backwards |
| Install two 5/16” chains to overhead beams |  |  |  | Minimum 5/16” Watch your loading capacity |
| Hook one come-a-long to each chain |  |  |  | Close safety latches on hooks |
| Install one clevis to each bearing |  |  |  |  |
| Connect each come-a-long lifting hook to each clevises |  |  |  | Close safety latches on hooks |
| Use come-a-long to lift new belt about 6” off floor |  |  |  |  |
| Remove guards from tail pulley area |  |  |  |  |
| Remove skirting from tail chute |  |  |  |  |
| Install a belt clamp at Grimmies bridge |  |  |  | Explain that hemp rope can be used here Do not use rope on incline belts |
| Remove belt covers if necessary |  |  |  |  |
| Connect two come-a-longs to the belt clamp |  |  |  | Close safety latches on hooks |
| Install two cables around the next set of idlers |  |  |  |  |
| Connect come-a-longs into the two cables |  |  |  | Close safety latches on hooks Explain when necessary why chains are used instead of cables |
| Tension the come-a-longs |  |  |  |  |
| Install the plank underneath the bottom of the top belt |  |  |  |  |
| Set the framing square on the belt top cover and square with the belt above the plank |  |  |  |  |
| Use the utility knife to cut the belt using the framing square as your guide cutting away from your body |  |  |  |  |
| Pull the non-clamped belt end through the tail pulley |  |  |  |  |
| Cut bands on new belt |  |  |  | Stay clear – explain danger of release energy |
| Pull and feed the new belt end over the tail pulley and through the wall to the old belt end |  |  |  |  |
| Square the new belt end up to the old belt end |  |  |  |  |
| Install planks under both belt ends |  |  |  |  |
| Using the top part of belt clip, mark six sets of holes evenly spaced across belt ends |  |  |  |  |
| Use the electric impact wrench with a #1 punch and quick connector to punch holes into the belt ends |  |  |  |  |
| Install clips through bottom side of belt ends through the punch holes |  |  |  |  |
| Install top of belt clips and nuts |  |  |  |  |
| Tighten each nut using the impact wrench and nut driver |  |  |  | Make sure clip ends are countersunk in top cover of belt ends |
| Use breaker bars to break studs from all six clips |  |  |  |  |
| Remove come-a-longs, belt clamp, planks and both cables |  |  |  |  |
| Install belt clevis on old return end of belt using ½” bolts |  |  |  | Holes must be punched prior to attaching the clevis |
| Tighten bolts on belt clevis |  |  |  |  |
| Install one end of the 25’ cable through the belt clevis cable |  |  |  |  |
| Use three ½” cable clamps to fasten a loop around the belt clevis cable |  |  |  | Explain properly installed clamps |
| Danger tape all affected areas |  |  |  |  |
| Contact pay loader operator using plant radio |  |  |  |  |
| Make a loop in the cable fastening with three ½” cable clamps |  |  |  | Reinforce cable clamps proper installation/ configuration |
| Connect the loop of the cable to pay loader’s hitch pin |  |  |  |  |
| All personnel must remove locks from Belt #2 |  |  |  |  |
| Verify that radios are working properly |  |  |  |  |
| Spot personnel to the proper location all with radio using Channel 3 |  |  |  | Still need to have people (possibly more people) to communicate without radios |
| - Assign one to tail pulley |  |  |  |  |
| - Assign one head pulley |  |  |  |  |
| - Assign one with splice |  |  |  |  |
| - Assign one outside with pay loader |  |  |  |  |
| Verify communication with each person | If you don’t communicate, serious injury could occur |  |  |  |
| Put the disconnect switch into “On” position |  |  |  | Stand to the side of the disconnect switch throwing switch while facing away from it |
| Verify that safety lanyard is operating by using “jog” |  |  |  | Explain safety switch procedure |
| Reset safety lanyard |  |  |  |  |
| Call to make sure that all personnel are clear and ready to pull belt | If you don’t communicate, serious injury could occur |  |  | Explain the procedure of removing old belt |
| Begin “pulling” belt using radio communication between “jogging” and “pulling” with a pay loader |  |  |  |  |
| Continue the “pulling” process until the new belt is all the way around |  |  |  |  |
| Lock #2 belt out | Prevent injuries Inadvertent starting |  |  | All personnel install their locks |
| Disconnect the cable from the pay loader’s hitch pin |  |  |  |  |
| Release pay loader operator |  |  |  |  |
| Disconnect cable and belt clevis from belt |  |  |  |  |
| Place cable and belt clevis on truck |  |  |  |  |
| Cut the old belt from new belt |  |  |  |  |
| Feed new belt through tail pulley to the other end of new belt |  |  |  |  |
| Install one belt clamp on each end of belt |  |  |  | Explain this procedure on distances between belt clamps |
| Install come-a-longs to belt clamps |  |  |  | Close safety latches on hooks |
| Use come-a-longs to remove belt slack |  |  |  |  |
| Place plank under the top belt |  |  |  |  |
| Square belt ends |  |  |  | Explain this procedure |
| Nail one end of belt to the plank |  |  |  |  |
| Install splicing template |  |  |  | Explain this procedure |
| Install holes in belt through template |  |  |  | Explain this procedure |
| Remove template from belt |  |  |  |  |
| Put clips into one end of belt |  |  |  | Follow manufacturer’s procedures |
| Turn template upside down, slide onto bolts of clips to line up with holes |  |  |  |  |
| Take other end of belt and install clips through holes |  |  |  |  |
| Install top half of clips with nuts (hand tight) |  |  |  |  |
| Install belt tape to seal the joint |  |  |  |  |
| Ensure splice is square |  |  |  |  |
| Tighten all nuts on clips with impact wrench |  |  |  | Explain this procedure |
| Use breaker bars to break off all clip bolts |  |  |  |  |
| Grind the clips with a 4 ½” disc grinder |  |  |  | Must wear full face shield Grind bolts flush with the nuts |
| Remove nail from belt |  |  |  |  |
| Remove planks from belt |  |  |  |  |
| Remove the belt blocks and come-a-longs |  |  |  |  |
| Install skirting and guards on tail pulley | Job not done until all of the guards are on, could cause serious injury |  |  |  |
| Reinstall all belt covers |  |  |  |  |
| Go to drive end of belt and lower gravity take-up |  |  |  |  |
| Remove chains and chain wrench |  |  |  |  |
| Install guards at gravity take-up | Job not done until all of the guards are on, could cause serious injury |  |  |  |
| Re-tension belt scrapers |  |  |  | Explain this procedure |
| Remove all locks |  |  |  | Belt #1 Belt #2 |
| Put disconnect switch to the “On” position |  |  |  | Stand to the side of the disconnect switch throwing switch while facing away from it |
| “Jog” and “train” belt |  |  |  |  |
| Check and replace all bad idlers, as needed |  |  |  | Lock belt out before any idler replacement is started |
| Release system to Central Control |  |  |  |  |

# Duty 3:   Clean-Up Activities

Learner will demonstrate how to conduct a safe and thorough clean-up procedure of the job. Learner will also explain the job duties, why they are conducted, any associated risk, and how to implement appropriate controls.  A thorough clean-up activity procedure includes the following job steps:

| Job Steps | Importance Narrative (Consider Safety, Production, Maintenance) | Importance Ranking 1=Important 2=Very Important 3=Critical | Satisfactory or Needs Work | Procedures/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Gather all tools up and load them on the truck |  |  |  |  |
| - If torches were used, gases must be bled | Eliminate fire and burn hazards |  |  | Show video |
| - Before moving truck, ensure chocks are removed |  |  |  |  |
| Pick up all debris |  |  |  |  |
| Clean up old belt |  |  |  |  |
| - Roll up and move with gradall/forklift |  |  |  |  |
| Clean up, remove new belt on roll and return to belt storage location |  |  |  |  |
| Remove danger tape |  |  |  |  |
| Perform workplace inspection |  |  |  |  |
| - Correct any deficiencies | Could lead to injuries or loss of production |  |  |  |
| Ensure all locks are off |  |  |  |  |
| - Ensure all disconnects/ lanyards have been place in the “on” position |  |  |  |  |
| Call Central Control |  |  |  |  |
| - Report names |  |  |  |  |
| - Report numbers |  |  |  |  |
| - Report equipment I.D. |  |  |  |  |
| - Verify “ready” light |  |  |  |  |
| Return to shop |  |  |  |  |
| Park truck in front of shop |  |  |  |  |
| Chock truck wheels |  |  |  |  |
| Remove debris from truck and place into proper containers |  |  |  |  |
| Unload tools/equipment from truck |  |  |  |  |
| Visual inspection of your tools |  |  |  |  |
| - Replace/repair damaged tools |  |  |  |  |
| Store tools in proper location |  |  |  |  |
| Remove chocks from wheels |  |  |  |  |
| Drive truck to wash rack |  |  |  |  |
| Clean truck |  |  |  |  |
| - Blow off/high pressure wash |  |  |  |  |
| Drive truck back to shop |  |  |  |  |
| Chock wheels |  |  |  |  |
| Ensure personal and company tool boxes are locked |  |  |  |  |

# Duty 4:   End of Shift Activities

Learner will explain importance of communicating and exchanging information as part of the work to be accomplished in completing the end of shift activities. Learner will also explain the job duties, why they are conducted, any associated risk, and how to implement appropriate controls.  A thorough end of shift activities includes the following job steps:

| Job Steps | Importance Narrative (Consider Safety, Production, Maintenance) | Importance Ranking 1=Important 2=Very Important 3=Critical | Satisfactory or Needs Work | Procedures/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Ensure that all parts have been charged out and slips turned in |  |  |  |  |
| Fill out work order and explain the work performed |  |  |  |  |
| Fill out time card |  |  |  |  |
| Place all paperwork into box in front of shop |  |  |  |  |
| Notify supervisor of job completion |  |  |  |  |
| Root cause analysis meeting |  |  |  |  |
| - Hold job critique meeting |  |  |  |  |
| - Improve job efficiency |  |  |  |  |
| - Improve job safety |  |  |  |  |
| - Abnormal conditions dictate discussion |  |  |  |  |
| Shower up |  |  |  |  |
| Check out (time clock) |  |  |  |  |