| Company Name: | Equipment/Job Identification: Front-end Loader Operator Type of Equipment: Front-end Loader Make: Cat Model: 988B Year: Use: Loading/Tramming/stockpiling |
| --- | --- |
| Mine Name: |  |
| Date of Analysis: |  |

# Pre-Assessment:

# Pre-requisites:

The materials that could be used to conduct Front-end Loader Job Training are as follows:

(These are to be shown and discussed prior to field training on the loader)

- Videos

  - Front-end Loader Safety (MSHA)

  - Stockpiling Safety  ( MSHA)

  - Berms and Haul roads (MSHA)

  - 988B End-Loader (Caterpillar)

  - Silica Control in Equipment Cab (NIOSH)

- Printed Materials

  - Company Policy

  - MSHA Regulations and Policy

  - Cat Operator’s Manual for 988B

  - Fatal Reports (as applicable)

# Job:  Pre-Operational Inspection

## The instructor will describe the machine, its purpose, the manufacturer, the model; the machines work capacities and capabilities. He will also discuss the importance of a daily self-assessment for work on each shift.  Learner will describe the machine and its capabilities and capacities to the instructor. He will also describe how to do a self-assessment for work fitness.

## Shift Preparation

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Employee assessment |  |  |  |  |
| Communication between shifts |  |  |  |  |
| Equipment Status (In-Service/Out-of-Service) |  |  |  |  |
| Personal Protective Equipment |  |  |  |  |

## Walkaround

The instructor will explain and demonstrate a walk around inspection of the end loader. This will include what and how to make checks, and the most common type of problems encountered.

The learner will conduct a walk around inspection of the loader in the presence of the instructor, point out and explain components to be checked and any necessary information about the check.

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Begin at right-rear corner - Turn master switch “OFF" - Check under machine for leaks |  |  |  |  |
| Continue to left-rear corner - Check radiator (cleanliness/cracks) - Check left-rear tire/wheel/lugs/lock ring |  |  |  |  |
| Continue to left center of machine - Visual check of hydraulic lines/steering cylinders - Check ladders/grab irons - Visual check of fuel tank |  |  |  |  |
| Continue to left-front of machine - Check under machine for leaks - Check left-front tire/wheel/lugs/lockring - Check bucket pins/linkage/hydraulic cylinders and lines - Check bucket wear items |  |  |  |  |
| Continue to right-front of machine - Check right-front tire/wheel/lugs/lockring - Check bucket pins/linkage/hydraulic cylinders - Check bucket wear items |  |  |  |  |
| Continue to right-center of machine - Check hydraulic oil level (sight gauge) - Check hydraulic lines/steering cylinders - Check ladders/grab irons - Check hydraulic fluid fill cap - Must climb ladder to check |  |  |  |  |
| Continue to right-rear of machine - Check right-rear tire/wheel/lugs/locking ring - Turn master switch “ON” |  |  |  |  |

## Outside Cab Inspection

The instructor will explain and demonstrate the method of mounting and dismounting the loader.  He will also demonstrate and explain body positioning and the use of seat belts in the cab of the loader.

The learner will demonstrate mounting/dismounting.

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Mount machine |  |  |  |  |
| Check fire extinguisher |  |  |  |  |
| Check handrails |  |  |  |  |
| Check/Clean Glass/Mirrors |  |  |  |  |

## Inside Cab Inspection

The instructor will demonstrate and explain body positioning and the use of seat belts in the cab of the loader. The instructor will explain and demonstrate all controls of the loader. He will also describe the function of all gauges, lights, and warnings necessary on the loader. The learner will demonstrate to the instructor with the loader not running how all controls work. He will also explain all gauges and lights in the machine.

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |

# Job: On-Shift Operation

The instructor will demonstrate and explain the various activities of the loader, starting with moving the machine around on a flat area making starts and stops to ensure the trainee can get the machine stopped.  As the learner becomes familiar with the loaders basic operating functions, the more complex functions can be demonstrated and added as this outline suggests.

## Control Familiarization

Learner will demonstrate how to operate controls on the front-end loader. The learner will explain the technical aspects of the job duties as they are demonstrated.

The learner will demonstrate each activity to the instructor after a period of practice.

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| - Hydraulic Controls - Lift/Tilt Bucket Controls |  |  |  |  |
| - Transmission Operations |  |  |  |  |
| - Foot Pedal Controls |  |  |  |  |
| - All Hand Controls |  |  |  |  |

## Maintain Communications

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| - Eye contact |  |  |  |  |
| - Signals |  |  |  |  |
| - Hazard Reporting |  |  |  |  |

## Demonstrate Blind Spots

The instructor will demonstrate blind spots and explain the importance of communication with other persons on the site.

The learner will describe blind spots and explain the importance of communication at the site

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Directly Behind Machine |  |  |  |  |
| Right-Rear Corner |  |  |  |  |
| Left-Rear Corner |  |  |  |  |
| - Right-front of Machine |  |  |  |  |

## Demonstrate Machine Movement by Instructor

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Articulation - Release Parking Brake - Maintain Bucket Contact with Ground - Explain Pinch-points and Steeringe |  |  |  |  |
| Forward and reverse movement - Explain the manual operation of the machine |  |  |  |  |
| - Trainee’s Practice - Articulate Loader - Move forward and reverse and articulate - Raise/lower bucket - Check all indicator gauges - Proper shut-down procedures - Provide feedback/demo |  |  |  |  |
| - Bucket loading techniques |  |  |  |  |
| - Pick and carry techniques |  |  |  |  |
| - Hopper Loading Techniques |  |  |  |  |
| - Truck Loading Techniques |  |  |  |  |
| - Towing techniques |  |  |  |  |
| - Stone bay road techniques |  |  |  |  |
| - Trainee’s Practice - Bucket Loading Techniques - Pick-and-carry Techniques - Truck Loading Techniques - Hopper Loading Techniques - Stone bay road techniques |  |  |  |  |

## Field Practice

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| - Check gauges/indicators |  |  |  |  |
| - Maintain Safe Speed for Ground Conditions - Grade - Weather - Road - Visibility - Traffic |  |  |  |  |
| - Check for Road Hazards |  |  |  |  |
| - Check for Highwall /Bank Hazards/ground conditions - Check berms - Check overhanging/loose material - Check for work overhead |  |  |  |  |
| - Practice Defensive Driving - Awareness for Other Vehicles - Scan Ahead - Know/Remember Blindspots |  |  |  |  |
| - Traveling on a Grade - Yield to Loaded Truck |  |  |  |  |
| - Monitor Gauges |  |  |  |  |
| - Clean Windshield as necessary |  |  |  |  |
| - Use Signals |  |  |  |  |
| - Count Loads/Scale |  |  |  |  |
| - Accelerate Smoothly |  |  |  |  |
| - Obey Traffic Signs |  |  |  |  |
| - Fueling Equipment |  |  |  |  |
| - Wear Seatbelts |  |  |  |  |

# Job: Post-Operational Inspection

The instructor will demonstrate end of shift activities, and reporting procedures if problems have developed with the loader or the site during the shift.  He will also introduce the learner to the on-coming operator and encourage a few minutes of conversation at shift change.

The learner will demonstrate and explain post operation activities, including completion of any record required.

## Park Equipment

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| - Set park brake |  |  |  |  |
| - Lower bucket to ground |  |  |  |  |
| - Place controls in neutral condition and lock-out |  |  |  |  |
| - Idle Down/Cool Down Engine |  |  |  |  |
| - Observe gauges/Indicators |  |  |  |  |
| - Stop Engine/Turn Off Lights |  |  |  |  |

## Dismount Machine

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| - Clean Out Cab - Wet Wipe or vacuum cab |  |  |  |  |
| - Observe Ground Conditions |  |  |  |  |
| - Check Ladder/Grab Irons |  |  |  |  |
| - Face Ladder |  |  |  |  |
| - Maintain 3-point contact |  |  |  |  |
| - Final walk-around/Turn off master switch |  |  |  |  |

## Communications Changes

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| - End-Loader |  |  |  |  |
| - Mine |  |  |  |  |

# Job: Emergency Procedures/Abnormal Occurrences/Non-Routine Procedures

Objective:  The learner will explain, and when possible, safely demonstrate the emergency procedures needed to respond to an emergency situation (after the learner feels comfortable with necessary operating skills).

## Fire

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Stop Machine/Exit Immediately |  |  |  |  |
| Turn key “OFF”/Master Switch, if possible |  |  |  |  |
| Notify supervision |  |  |  |  |

## Medical

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Injury/Illness - Report to Supervisor |  |  |  |  |

## Critical Manfunctions

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Engine Stall/Power Loss |  |  |  |  |
| React to Panel Lights |  |  |  |  |
| Tire Failure |  |  |  |  |
| - Brake Failure |  |  |  |  |
| - Steering Failure |  |  |  |  |
| - Hydraulic Failure |  |  |  |  |
| - Electrical Failure |  |  |  |  |

## Sudden Loss of Visibility

| Loss of visibility |  |  |  |  |
| --- | --- | --- | --- | --- |

## Loss of Control

| Job Steps | Importance Narrative (Consider Safety/Production/Maintenance) | Importance Ranking 1-Important 2-Very important 3-Critical | Satisfactory or Needs Work | Procedure/Risk Resolution/ Notes/Comments |
| --- | --- | --- | --- | --- |
| Skidding |  |  |  |  |
| Submersion |  |  |  |  |
| Roll Over |  |  |  |  |
| - Ground Failure |  |  |  |  |

## Contact with Power LInes

| Overhead |  |  |  |  |
| --- | --- | --- | --- | --- |
| Underground |  |  |  |  |

## Other Vehicle out of Control

| Other Vehicle out of Control |  |  |  |  |
| --- | --- | --- | --- | --- |