| Company Name: | Type of Equipment: Haul truck Make: HaulPack Model: 930E Year: 1997 – 1999 Use: In the pit hauling ore and waste |
| --- | --- |
| Mine Name: |  |
| Date of Analysis: |  |

# Pre-Assessment:

Pre-requisites:

# Duty 1: Pre-Operational Inspection

Learner will be able to demonstrate how to conduct a safe and thorough pre-operational inspection. Learner will also be able to explain job steps, why they are conducted, any associated risk, and how to implement appropriate controls.  A thorough pre-operational inspection includes the following steps:

## Conduct Ground/Walk-Around Inspection

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| - Perform self assessment for readiness to work— communicate w/shifter as necessary - Attend crew line-out - Shifter talks to previous shift/maintenance and passes information to crew--quick exchange of info between operators as necessary - Check weather conditions/forecast - Get duty assignment from shifter - Check for PPE before leaving line-out |  |  |  |  |
| Check for red tags, operator/maintenance chains |  |  |  |  |
| Check for slip/trip hazards |  |  |  |  |
| Check truck is secured against movement – wheels chocked or in ditch |  |  |  |  |
| Check gen. condition of truck (cosmetic appearance) - Look for mud/rocks under bed and on bed rails - Body pins - Body/frame for cracks - Broken or cracked welds - Roll over protection structure (rops) - Suspension cylinder & pins |  |  |  |  |
| Check for leaks |  |  |  |  |
| Check suspension |  |  |  |  |
| Check pump drive shaft |  |  |  |  |
| Check steering linkage - Cylinder - Tie rod ends - Drag links |  |  |  |  |
| Check tires/wheels - Cuts - Damage - Inflation appearance - Rocks between wheels - Missing/damaged mounting hardware - Cracked wheels - Locking ring - Heat & odor |  |  |  |  |
| Check physical condition/cleanliness of lights |  |  |  |  |
| Check fire suppression system actuator |  |  |  |  |
| Check radiator for debris/damage |  |  |  |  |
| Check for unusual odors/noises/excess heat |  |  |  |  |
| Check cooling air duct work Rear axle housing & mounts |  |  |  |  |
| Check hydraulic shut-off valves |  |  |  |  |
| Check heaters |  |  |  |  |
| Check fluid levels/inspect engine compartment - Hydraulic - Coolant - Fuel - Loose wires |  |  |  |  |
| Check hoses/lines/tubes/fittings for damage - Routing (not rubbing) - Excessive wear - Missing/loose clamps |  |  |  |  |
| Check ground level emergency shutdown switch |  |  |  |  |
| Check drive belts/guards |  |  |  |  |
| Check and clean rear camera |  |  |  |  |

## On-Machine Inspection

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Mount machine - Maintain 3-point contact - Face ladder |  |  |  |  |
| Check for safe access to cab - Hand holds - Railings - Steps - Decks - Ladders - Doors/latches - Chains |  |  |  |  |
| Check site glass for coolant |  |  |  |  |
| Check fire extinguisher |  |  |  |  |
| Check electrical system |  |  |  |  |
| Check and clean mirrors/windows |  |  |  |  |
| Check and clean cameras |  |  |  |  |

## In Cab Inspection

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Check shift selector position |  |  |  |  |
| Check wipers |  |  |  |  |
| Check horn |  |  |  |  |
| Check seat belt |  |  |  |  |
| Check lights/turn signals |  |  |  |  |
| Check two-way radio/computer |  |  |  |  |
| Check camera monitor |  |  |  |  |
| Check backup devices |  |  |  |  |
| Adjust seat/steering wheel |  |  |  |  |
| Check warning lights with test switch |  |  |  |  |
| Check cab/health & safety - Door seals/latches and acoustical materials - Housekeeping |  |  |  |  |
| Check fire suppression actuator |  |  |  |  |
| Check heat/air conditioning/defroster |  |  |  |  |
| Fasten seat belt |  |  |  |  |
| Verify gauge operation - Engine oil pressure - Engine water temperature - AC drive system temperature - Hydraulic/brake oil temperature - Fuel - Volt meter - Tachometer - Speedometer - Hour meter |  |  |  |  |

## Complete Defect/Status Report

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Complete pre-shift inspection sheet |  |  |  |  |

# Duty 2:  Operation

Learner will be able to demonstrate how to safely and productively operate the haul truck.  Learner will also be able to explain job steps, why they are conducted, any associated risk, and how to implement appropriate controls.  Operational steps include:

## Monitor Lights & Gauges

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Monitor lights & gauges |  |  |  |  |

## Maintain Communication

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Maintain communication - Radio - Computers - Eye contact - Horn signals - Hand/lights/turn signals - Spotters - Report hazards/malfunctions/road conditions |  |  |  |  |

## Traveling Unloaded

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Release parking brake |  |  |  |  |
| Sound horn/check for clearance |  |  |  |  |
| Engage selector switch/signal before moving |  |  |  |  |
| Perform initial movement checks - Check emergency steering/brake - Rolling brake test/dynamic retarder function - Service/Secondary brake test - Steering - Front end play - Proper right/left steering |  |  |  |  |
| Recognize abnormal performance - Vibration - Unusual odors/noises - Gauges/indicators |  |  |  |  |
| Maintain safe speed for conditions - Weather - Road - Visibility - Traffic - Do not exceed speed limits |  |  |  |  |
| Know your blindspots |  |  |  |  |
| Check for road hazards |  |  |  |  |
| Check for highwall hazards and berm height |  |  |  |  |
| Practice defensive driving - Awareness for other vehicles - Vehicle spacing - Scan ahead - Seatbelt |  |  |  |  |
| Traveling on a Grade - Use dynamic retarder - Yield to loaded truck |  |  |  |  |
| Maintain clean windshield |  |  |  |  |
| Use signals |  |  |  |  |
| Count loads |  |  |  |  |
| Accelerate/decelerate smoothly |  |  |  |  |
| Obey traffic signs |  |  |  |  |
| Monitor camera screens/mirrors |  |  |  |  |

## Loading

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Slow down and be aware in congested areas |  |  |  |  |
| Follow instructions of loading unit operator |  |  |  |  |
| Check mirrors/camera when backing |  |  |  |  |
| Put selector switch in neutral |  |  |  |  |
| Set wheel lock brake switch |  |  |  |  |
| Stay in cab/wear seatbelt |  |  |  |  |
| Look both ways prior to proceeding |  |  |  |  |
| Be aware of surroundings before proceeding |  |  |  |  |
| Release wheel lock brake switch |  |  |  |  |
| Check handling after loading |  |  |  |  |

## Traveling While Loaded

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Adjust operation for load and conditions |  |  |  |  |
| Use dynamic retarder |  |  |  |  |
| Recognize abnormal performance - Driveline noise and vibration - Unusual odors/noises - Gauges/indicators - Tires/handling |  |  |  |  |
| Maintain safe speed for conditions - Weather - Road - Visibility - Traffic - Do not exceed speed limit |  |  |  |  |
| Know your blind spots |  |  |  |  |
| Check for road hazards - Berm heights - Highwall/stockpile hazards |  |  |  |  |
| Practice defensive driving - Maintain awareness for other vehicles - Vehicle spacing - Scan ahead - Seatbelt |  |  |  |  |
| Traveling on a grade - Use retarder - Know what RSC is - Yield to loaded truck |  |  |  |  |
| - Perform parking brake test – first opportunity |  |  |  |  |
| Use signals |  |  |  |  |
| Accelerate/decelerate smoothly |  |  |  |  |
| Obey traffic signs |  |  |  |  |
| Monitor mirrors and camera screens |  |  |  |  |

## Dumping

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Enter dump area - Ensure adequate area lighting - Enter area in clockwise direction - Maintain awareness for other vehicles - Slow down in congested areas - View dump crest - Inspect the ground surface for cracks - Check for mid -axle high berm along roadway and as dump site restraint - Select a reasonably level area to dump - Avoid softspots and depressions - Do not back up if visibility is impaired - If stockpile isn’t attended, dump short |  |  |  |  |
| Prepare to Dump - Back slowly - Do not impact berm – back square to berm - Keep safe distance between other dumping trucks and dozer (1 truck length) - Set wheel brake lock - Put selector switch in neutral |  |  |  |  |
| Stay in cab |  |  |  |  |
| Dump when approved - Increase rpm to 1500 or what is adequate to clear the load - Raise bed/dump load - Check bed movement - Lower bed completely - Dump short as required - Use caution; watch for spillage - Signal before moving |  |  |  |  |
| Be aware of abnormal conditions - Load hang-up - Off-center loads - Hydraulic hoist failures - Soft spots/cracks/signs of dump movement - Missing berms - Vertical displacement |  |  |  |  |

## Emergency Procedures/ Abnormal Occurrences/ Non-Routine Procedures

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Fire |  |  |  |  |
| Medical |  |  |  |  |
| React to panel lights |  |  |  |  |
| Engine stall/power loss - On grade - On level |  |  |  |  |
| Tire failure |  |  |  |  |
| Brake failure |  |  |  |  |
| Dynamic retarder failure |  |  |  |  |
| Steering failure |  |  |  |  |
| Strut failure |  |  |  |  |
| Hydraulic lift failure |  |  |  |  |
| Electrical failure |  |  |  |  |
| Contact with powerlines - Signal others - Stay in cab or drive away, if possible |  |  |  |  |
| Sudden loss of visibility – communicate situation - Stop - Proceed when visibility allows it |  |  |  |  |
| Loss of control - Skidding - Roll over - Ground failure |  |  |  |  |
| Other vehicle out of control |  |  |  |  |

## Field Maintenance

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| - Fueling – during breaks or shift change. Fueling times indicated by sticker on dash. |  |  |  |  |

# Duty 3:  Post-Operational Inspection Procedures

Learner will be able to demonstrate how to conduct a safe and thorough post-operational inspection.  Learner will also be able to explain job steps, why they are conducted, any associated risk, and how to implement appropriate controls.  A thorough post-operational inspection includes the following steps:

## Park Truck

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Enter parking trench from left to right |  |  |  |  |
| Set parking brake |  |  |  |  |
| Place selector in neutral position |  |  |  |  |
| Turn on all lights |  |  |  |  |
| Complete pre-shift form as necessary |  |  |  |  |
| Log off modular mining |  |  |  |  |
| Dismount machine - Clean out cab - Observe gauges/indicators for abnormalities - Check ladder - Observe ground condition - Maintain 3-point contact – don’t jump - Face ladder |  |  |  |  |

## Communicate Changes in Operating Conditions

| Job Steps | Importance Narrative (Consider Safety, Production, and Maintenance) | Importance: 1=important 2=very important 3=critical | Satisfactory or Needs Work | Procedures/Risk Resolutions/Notes/Comments |
| --- | --- | --- | --- | --- |
| Communicate changes in operating conditions - Truck - Mine |  |  |  |  |