Naval Safety Center

1

### High-Risk Training

### Inspection Checklist

### For

Rappel Towers

## 1/27/04									RapelIns2.doc

RAPPEL   TOWER   INSPECTION

| Tower Inspection: | # Yes | # No | # Remarks |
| --- | --- | --- | --- |
| 1. Is the tower structurally sound? Ref: MCRP 3-11.4A, App E |  |  |  |
| 2. Are all open areas (above 4’) guarded with guard rails? Ref: 29CFR1910.23(c)(1) |  |  |  |
| 3. Are all guard rails a minimum of 42” high and capable of withstanding a side force of 200 lbs? Ref: 29CFR1910.23 (e)(1) and (e)(3)(iv) |  |  |  |
| 4. Are toe boards installed in all areas where personnel could pass underneath? Ref: 29CFR1910.23(c)(1) |  |  |  |
| 5. Do all stairs/fixed ladders comply with OSHA standards? Ref: 29CFR1910.24 and .27 |  |  |  |
| 6. Do all tower rope stations have three anchor points each? Ref: MCRP 3-11.4A, pg 3-1 |  |  |  |
| 7. Have all rappel rope anchor points been load tested and certified to 5,000 pounds each? Ref: ANSI Z359.1 (1992), 29CFR1910.66 para (I)(c)(10) of AppC |  |  |  |
| 8. Have all fast rope anchor points been tested to 5,000 pounds each, per person on the rope(e.g. 2 ropers on the rope at the same time requires testing to 10,000 lbs. each [2 x 5,000 lbs., etc.]). Ref: ANSI Z359.1 (1992), 29CFR1910.66 para (I)(c)(10) of App C |  |  |  |

1

|  | # Yes | # No | # Remarks |
| --- | --- | --- | --- |
| 9. Are all anchor points in good condition and free of corrosion, sharp edges, burrs, etc? Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 10. Is the tower deck free of trip/slip hazards (water, protruding nails/bolts, splinters, etc)? Ref: 29CFR1910.141(a)(3)(ii) |  |  |  |
| 11. Is the rappel wall face and the fastrope area free of protrusions, broken or loose boards? Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 12. Does the tower deck and the stairway treads have a non-skid surface? Ref: 29CFR1910.24(f) |  |  |  |
| 13. Is there padding on all edges that ropes and/or personnel cross? Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 14. Is the edge padding in good condition and securely fastened? Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 15. Are there any signs of insect infestation? Ref: 29CFR1910.141(a)(5) |  |  |  |
| 16. Is the landing area free of obstructions and hazards? Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 17. Do the landing areas consist of a cushioning material (recommend 24” non-compressed depth of wood chips or mulch)? Ref: MCRP 3-11.4A, Appendix E |  |  |  |

2

|  | # Yes | # No | # Remarks |
| --- | --- | --- | --- |
| 18. Has the landing area been loosened up prior to use and, if large number of students are rappelling, loosened up again during training? Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 19. Are all structural areas of the tower, that a rappeller might contact during rappel/fast rope operations, properly padded? Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 20. Is all structural padding in good condition and securely fastened? Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 21. Are all areas, that pose a trip hazard or head hazard, marked in yellow? Ref: 29CFR1910.144(a)(3) |  |  |  |
| 22. Is the tower platform and all rappel rope stations accessible without having to climb over any obstacles (i.e: guard rails, support cables, etc.)? Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 23. Is the tower on a regularly scheduled inspection/maintenance program? Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 24. Is the tower inspected periodically by a qualified engineer? Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 25. Is a copy of the engineer’s certification kept on file? (The certification should cover the load test rating of the existing rope anchor points.) Ref: MCRP 3-11.4A, Appendix E |  |  |  |
| 26. Is tower access restricted to authorized personnel only (secured stairs/ladders)? |  |  |  |

3