TEXAS DEPARTMENT OF LICENSING AND REGULATION

COMPLIANCE DIVISION – INDUSTRIALIZED HOUSING AND BUILDINGS

- P.O. Box 12157  Austin, Texas 78711    (512) 463-7353   (800) 803-9202    FAX (512) 475-4364

- industrialized.buildings@license.state.tx.us     www.license.state.tx.us

Residential Site Inspection Checklist for Industrialized Housing

General, Foundation, and Structural

This checklist is a guide to items involving code compliance of the foundation and structural connections that need to be checked during a residential site inspection but does not necessarily constitute a complete list of items to be checked.  The checklist provides minimum check points for work performed at the installation site; however, not all check points may be applicable.  The inspector shall also check to assure that construction performed on site does not result in non-compliance of construction completed in the manufacturing facility.  Note:  NA = Not applicable

| Industrialized builder’s name & reg #: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Texas Decal or Insignia Number(s): |  |  |  |  |
| Manufacturer’s Serial or ID Number(s): |  |  |  |  |
| Owner’s Name and Address: |  |  |  |  |
| Installation Address: |  |  |  |  |

IRC = International Residential Code (2003 edition); IECC = International Energy Conservation Code (2003 edition); Rule = Chapter 70 rules governing Industrialized Housing and Buildings; ACI = ACI 318-02, Building Code Requirements for Structural Concrete (2002 edition)

| NA | Meets |  | Item # | Check Point |
| --- | --- | --- | --- | --- |
|  | YES | NO |  |  |
|  |  |  | 1 | IRC R301.1 – Wind speed on data plate is correct for installation site, foundation is designed for correct wind speed (note: per figure R301.2(4) of the IRC, the minimum basic wind speed in Texas is 90 mph and maximum basic wind speed in Texas is 140 mph, depending on location) |
|  |  |  | 2 | IRC N1101. – House is designed for climate zone where installed. Climate zone for site per code is _____________________________________________________________________________________ |
|  |  |  | 3 | Rules 70.70(d) and (e) and 70.73 – Foundation plan available – plan is either approved foundation plan from manufacturer or unique “engineered” foundation plan. If engineered, provide architect’s or engineer’s name and license number: _______________________________________________________________ |
|  |  |  | 4 | Rules 70.70(d) and (e) and 70.73 – Plans and specifications for completion of house at installation site provided – either approved details and typicals from manufacturer or unique “engineered” details |
|  |  |  | 5 | IRC R301.1 – Soil bearing pressure complies with foundation plan: Soil bearing pressure = __________ psf Builder shall provide documentation to substantiate that site complies |
|  |  |  | 6 | IRC R401.4 – Expansive soil site: Yes No Builder shall provide documentation to substantiate that site complies. Soil test report # & name of responsible party: ________________________________________________ |
|  |  |  | 7 | IRC R403.1 – Footings are supported on undisturbed natural soils or engineered fill |
|  |  |  | 8 | Foundation plan does not call for use of ground, or soil, anchors [reference decision of Texas Industrialized Building Code Council of July 27, 1989] |
|  |  |  | 9 | IRC R408.4 – Under-floor grade is cleaned of all debris, vegetation, and organic material |
|  |  |  | 10 | IRC R403.1.4.1 – Foundation walls, piers and other permanent supports extend below the frost line |
|  |  |  | 11 | IRC R403.1.4 – Exterior footings are at least 12” below undisturbed ground |
|  |  |  | 12 | IRC R403.1 and IRC R404.1.5.1 – Load bearing walls placed on continuous concrete footings |
|  |  |  | 13 | IRC R403.1.1 – Minimum footing depth (thickness of footing) not less than 6” or as required on plans, whichever is greater |
|  |  |  | 14 | IRC R403.1.1, IRC Table R403.1, and IRC Figure R403.1(1) – Minimum footing width not less than foundation design or minimum requirements of code, whichever is greater |
|  |  |  | 15 | IRC R403.1.1 and Figure R403.1(1) – Footing projection is minimum 2” but not more than depth (thickness) of footing |
|  |  |  | 16 | IRC R403.1.5 – Top surface of footings level, slope of bottom surface of footings does not exceed 1 unit vertical in 10 units horizontal (10% slope) |
|  |  |  | 17 | IRC R402.2 and IRC Table R402.2 – Minimum concrete compressive strength per Table R402.2 of the IRC or as required by foundation plan, whichever is greater |

| Installation address: |  |
| --- | --- |

IRC = International Residential Code (2003 edition); IECC = International Energy Conservation Code (2003 edition); Rule = Chapter 70 rules governing Industrialized Housing and Buildings; ACI = ACI 318-02, Building Code Requirements for Structural Concrete (2002 edition)

| NA | Meets |  | Item # | Check Point |
| --- | --- | --- | --- | --- |
|  | YES | NO |  |  |
|  |  |  | 18 | Rule 70.73 – Reinforcement grade and placement as required on foundation plan |
|  |  |  | 19 | IRC R402.2 and ACI 5.6 – Evaluation and acceptance of concrete Less than 50 yd3, evidence of satisfactory strength provided Greater than 50 yd3, specimens taken in accordance with ACI 5.6 for strength tests |
|  |  |  | 20 | IRC R402.2 and ACI 7.7 – Concrete coverage of reinforcement as required on foundation plan, but not less than 3” |
|  |  |  | 21 | IRC R404.1.5.1 and IRC R606.5 – Hollow concrete masonry units filled solidly with concrete or Type M or S mortar. Unfilled hollow piers may be used if unsupported height is not more than 4 times their least dimension |
|  |  |  | 22 | IRC R404.1.5.1 and IRC R606.5.1 – Hollow piers capped with 4 inches of solid masonry or concrete; or cavities of top course filled with concrete or grout or other approved materials |
|  |  |  | 23 | IRC R403.1.6 – Anchor bolts extend a minimum of 7” into concrete |
|  |  |  | 24 | IRC R403.1.6 – Minimum ½” anchor bolts spaced a maximum of 6 feet o.c. |
|  |  |  | 25 | IRC R403.1.6, Exception – Foundation anchor straps spaced as required to provide equivalent anchorage to ½” diameter anchor bolts |
|  |  |  | 26 | IRC R403.1 and IRC R407.2 – Foundation anchor straps are corrosion resistant |
|  |  |  | 27 | IRC R319 – Materials used to enclose crawl space protected from decay or corrosion |
|  |  |  | 28 | IRC R408.3 – Minimum crawl space access of 16” x 24” |
|  |  |  | 29 | IRC R408.2 – Minimum net area of openings for crawl space ventilation |
|  |  |  | 30 | IRC R408.2 – One crawl space ventilation opening within 3 feet of each corner of building |
|  |  |  | 31 | IRC R408.2 – Materials for ventilation openings |
|  |  |  | 32 | IRC R401.3 and R405 – Minimum 6” in 10’ slope for drainage or drains provided |
|  |  |  | 33 | IRC R406.1 or IRC R406.3 – Foundation walls damp proofed |
|  |  |  | 34 | IRC R406.2 – Foundation walls water proofed |
|  |  |  | 35 | IRC R408.5 – Finished grade of under-floor space |
|  |  |  | 36 | Rule 70.73 – Connections between units in accordance with manufacturer’s approved details |
|  |  |  | 37 | Rules 70.73 and 70.70(d) and IRC R703 – Exterior siding completed in accordance with home manufacturer’s approved details, siding manufacturer’s installation instructions, and code |
|  |  |  | 38 | Rules 70.73 and 70.70(d), IRC R903, and IRC R905 – Roofing completed in accordance with home manufacturer’s approved details, installation instructions for roofing materials, and code |
|  |  |  | 39 | Rule 70.73 and 70.70(d) and IRC R702 – Interior wall coverings completed in accordance with manufacturer’s approved details, installation instructions for wall coverings, and code |
|  |  |  | 40 | IRC N1102.1.11 – Air leakage: Sealing of joints, seams, and penetrations |

Notes, special conditions, unique construction requirements: