| DIRECTIVE NUMBER: CPL 03-00-007 | EFECTIVE DATE: October 7, 2010 |
| --- | --- |

SUBJECT: Regional Emphasis Program - Silica

REGIONAL IDENTIFIER: Atlanta Regional Office (Region IV)

ABSTRACT

Purpose:	This instruction implements in Region IV the National Emphasis Program (NEP) to reduce occupational exposures to Crystalline Silica.

Scope:	This REP applies to all area offices in Region IV.

References:	OSHA Instruction CPL 02-00-148, Field Operations Manual (FOM)

OSHA Instruction CPL 02-00-025, Scheduling Systems for Programmed Inspections

OSHA Instruction CPL 04-00-001, Procedures for Approval of Local Emphasis Programs (LEP’s) and Experimental Programs

OSHA Instruction CPL 03-00-007, National Emphasis Program – Crystalline Silica

Cancellations:	None

State Impact:	None

Action Offices:	Area Offices

Originating Office: 	Atlanta Regional Office

Contact:  	Benjamin Ross, Assistant Regional Administrator for Enforcement Programs

By and Under the Authority of

CINDY A. COE

Regional Administrator

Executive Summary

This instruction establishes a Regional Emphasis Program to reduce occupational exposures to crystalline silica.  The REP also ensures effective targeting, enforcement and outreach for crystalline silica hazards in response to the National Emphasis Program for Crystalline Silica.

Significant Changes

None

TABLE OF CONTENTS

I.	Purpose	3

II.	Scope	3

III.	References	3

IV.	Expiration	3

V.	Action	3

VI.	Background	3

VII.	Program Procedures	4

VIII.	Scheduling of Inspections	4

IX.	Inspection Procedures	5

X.	Outreach	5

XI.	Recording in IMIS	5

XII.	Evaluation	5

Appendix A: Industries with Documented Silica Overexposures in Region IV	7

Appendix B (NEP): Industries with Potential Overexposure to Crystalline Silica	9

Appendix C: Links to Reference Documents	11

Appendix D: CSHO Checklist for Conducting Silica-Related Inspections	12

I.   	PURPOSE.  Regional Emphasis Program for the implementation of the National Emphasis Program for Crystalline Silica.

II.  	SCOPE.  This REP applies to all area offices in Region IV.

III. 	REFERENCES.

OSHA Instruction CPL 03-00-007, NEP for Crystalline Silica

OSHA Instruction CPL 02-00-148, Field Operations Manual (FOM)

OSHA Instruction CPL 04-00-001, Procedures for Approval of Local Emphasis Program and Experimental Programs

OSHA Instruction CPL 02-00-025, Scheduling System for Programmed Inspections

Other references in Directive Number: CPL 03-00-007

IV. 	EXPIRATION.  This instruction expires October 7, 2012, unless extended.

V.	ACTION.  OSHA compliance personnel shall follow the procedures contained in this instruction when conducting outreach and enforcement activities.

VI.	BACKGROUND.  Crystalline silica is an important industrial material, and occupational exposure occurs in a variety of workplace settings, including mining, manufacturing, construction, maritime, and agriculture.  Processes associated historically with high rates of silicosis include sandblasting, sand-casting foundry operations, mining, tunneling, cement cutting and demolition, masonry work, and granite cutting.

Reducing and ultimately eliminating the workplace incidence of silicosis has been a primary goal of the Agency since its inception.  In 1972, OSHA issued guidelines for conducting inspections in workplaces with significant crystalline silica exposure. In the early 1980s, the Agency placed a special emphasis on the prevention of silicosis in foundry personnel, and in 1996 OSHA implemented a Special Emphasis Program (SEP) to reduce the workplace incidence of silicosis.

The 1994 Government Performance and Results Act (GPRA) mandates that federal agencies improve performance and devise a system for measuring results. To comply with the provisions of GPRA, OSHA developed a Strategic Plan for improving the safety and health of all employees.  In 1998 and again in 2003, under the Strategic Plan, OSHA identified crystalline silica as one of the focused hazards.

This REP is being implemented to direct OSHA's Region IV's field staff inspection efforts to address elevated silica exposures in the workplace, including General Industry (1910) and Construction (1926).  These efforts meet the Strategic Plan goal of reducing silicosis set forth by the Agency.

VII.	PROGRAM PROCEDURES.

- As required by the Crystalline Silica NEP, all inspections (SST, complaint, etc.) conducted in the industries listed in Appendix B, where the compliance officer determines that there is a potential employee overexposure to silica, the inspection shall be counted under the NEP, and coded as a strategic plan silica inspection.

- All inspections covered by this REP shall be conducted according to instructions contained in the NEP.

- Other inspections, where the potential for exposures above the permissible exposure limit to Crystalline Silica exist in the industries listed in Appendix A, shall be conducted in accordance with the Paragraph XI.A.4 – XI.A.7 of the NEP.

- VIII.	SCHEDULING OF INSPECTIONS.

- A.	The Area Director shall compile a master list of establishments utilizing local information, as described in the NEP, paragraph XI.A.4.a. – XI.A.4.d; historical inspection data; and the list of industries in Appendix A (Industries where overexposures to crystalline silica have been documented in Region IV).  Adjustments to the list for additions and deletions will follow guidelines in CPL 02-00-025, paragraph B.1.b.(1)(b)(6) and CPL 03-00-007, paragraph XI.A.2.c..

- Each establishment on the corrected list will be assigned a sequential number, starting at the top of the list with number one.  A random number table will then be applied to create the first inspection cycle of five or more establishments.  Subsequent cycles will be created in the same way until the expiration of the REP or until all establishments on the list have been assigned to a cycle.  Cycles may be created all at once or as needed, and need not be of the same size.  Whenever an office becomes aware of a previously unknown establishment in one of the identified SICs/NAICSs, that establishment shall be added to the master list for inclusion in the next inspection cycle.

- Establishments may be selected from the specified cycle for inspection in any order that makes efficient use of available resources.  Each inspection cycle must be completed before another cycle is started except as allowed by CPL 02-00-025, paragraph B.1.b.(1)(d).  It is anticipated that safety hazards will be identified during the course of these inspections and referrals will be made as appropriate to address the safety hazards noted.

- IX.	INSPECTION PROCEDURES.

- A.	All inspections shall be conducted in accordance with Paragraph XI.B. of the NEP.  If exposure monitoring for crystalline silica is not conducted, the rationale must be included in the case file.  Appendix D (Appendix H in NEP) contains a checklist that summarizes the information to be documented during a silica-related inspection.  The checklist may be used by the CSHO to ensure proper coverage of the essential elements of a silica-related inspection.  A detailed account of all findings shall be documented in the case file.

- Where citations are issued for overexposure to crystalline silica, follow-up site visits must be conducted to determine whether the company is eliminating silica exposures or reducing exposures below the PEL.

- For those employers where follow-ups cannot be done, (i.e., construction sites or temporary abrasive blasting operations) the Area Office should request that the employer provide written updates documenting the progress of their abatement efforts per 1903.19.

- Once the necessary abatement verification is received, a copy of the relevant information in the case file (OSHA 1; OSHA 1A; OSHA 1B/IH for overexposures to silica; and all Engineering Controls used to control silica exposure) will be sent to the National Office - Directorate of Enforcement Programs.  Information in the case file will be distributed to the Office of Statistics for use as the Goal Indicator for silica.

- OUTREACH.  Each Area Director is encouraged to develop outreach programs that will support their enforcement efforts.  Suggested outreach materials and targeted audiences are referenced in Paragraph XI.C. of the NEP.

- RECORDING IN IMIS.  Instructions contained in the NEP shall be followed in coding NEP silica inspections.  In addition, the OSHA-1 forms for all inspections conducted in establishments that are covered by this REP shall be coded as local emphasis program inspections in block 25c and marked “SILICA.”

XII.	EVALUATION:

- Area Director’s Responsibility:  The area director affected by this regional emphasis program shall submit via the Performance Planning System by October 31, of each year an evaluation report that shall include the following elements:

- The goal of the program

- The area director’s opinion of how effective the program was in meeting its goal.

- Data and information to support the conclusions stated in (2) above including:

  - enforcement statistics;

  - serious hazards related to crystalline silica exposure eliminated;

  - number of times engineering controls cited; and

  - number of violations cited relating to overexposures to crystalline silica.

- Statement of whether the program should be continued, accompanied by a brief rationale.

- Description of any legal issues that arose which would necessitate a review by the Solicitor before renewal of the program could be approved.

- Any other comments or recommendations, including findings that might have an impact on how OSHA or the industry does business.

- Regional Office Responsibility:  Based on the area director's evaluation and any other pertinent information, Enforcement Programs will prepare an evaluation of this program for the Regional Administrator's submission to the national office as required by current instructions.

DISTRIBUTION:  	All Area Offices

Enforcement Programs

Regional Solicitor

Cooperative and State Programs

Administrative Programs

APPENDIX A

Industries with Documented Overexposures to Crystalline Silica in Region IV

| This appendix contains a list of industries in which employees may be exposed to elevated levels of crystalline silica. The list is based on a review of the Region IV Sampling Database for the last five fiscal years. This table is intended to show the range of industries in which overexposures to crystalline silica have been documented in Region IV, but should not be considered to be an exhaustive listing. |  |  |
| --- | --- | --- |
| Industries in Region IV with Crystalline Silica Overexposures |  |  |
| 1987 SIC Code1 | 1987 SIC Industry Title | 2002 NAICS Code2 |
| 1522 | General Contractors-Residential Buildings Other Than Single-Family | 236115, 236118 |
| 1541 | General Contractors-Industrial Buildings and Warehouses | 236210, 236220 |
| 1542 | General Contractor-Nonresidential Buildings, Other than Industrial Buildings and Warehouses | 236220 |
| 1611 | Highway and Street Construction, Except Elevated Highways | 237310 |
| 1629 | Heavy Construction, n.e.c. | 236210, 237110, 237120, 237130, 237990 |
| 1711 | Plumbing, Heating and Air-Conditioning | 238210, 238220, 238910 |
| 1721 | Painting and Paper Hanging* | 237310, 238320 |
| 1741 | Masonry, Stone Setting, and Other Stone Work | 238140 |
| 1743 | Terrazzo, Tile, Marble and Mosaic Work | 238310, 238340 |
| 1752 | Floor Laying and Other Floor Work, NEC | 238330 |
| 1771 | Concrete Work | 238110, 238140, 238990 |
| 1794 | Excavation Work | 238910 |
| 1799 | Special Trade Contractors, n.e.c. | 236220, 237990, 238150, 238190, 238290 |
| 2434 | Wood Kitchen Cabinets | 337110 |
| 3088 | Plastics Plumbing Fixtures | 326191 |
| 3211 | Flat Glass | 327211 |
| 3269 | Pottery Products, n.e.c. | 327112 |
| 3271 | Concrete Block and Brick | 327331 |
| 3272 | Concrete Products, Except Block and Bricks | 327332, 327390, 32799 |
| 3273 | Ready-Mixed Concrete | 327320 |
| 3281 | Cut Stone and Stone Products | 327991 |
| 3291 | Abrasive Products | 327910, 332999 |
| 3296 | Mineral Wool | 327993 |
| 3321 | Gray and Ductile Iron Foundries | 331511 |
| 3325 | Steel Foundries, n.e.c. | 331513 |
| 3365 | Aluminum Foundries | 331524 |
| 3369 | Nonferrous Foundries, Except Aluminum and Copper | 331528 |
| 3443 | Fabricated Plate Work (Boiler Shops)* | 332313, 332410, 332420 |
| 3471 | Electroplating, Polishing, Anodizing, and Coloring* | 332813 |
| 3496 | Coating, Engraving, and Allied Services, n.e.c.* | 332812, 339911, 339912, 339914 |
| 3537 | Industrial Trucks, Tractors, Trailers, and Stackers | 332439, 332999, 333924 |
| 3599 | Industrial and Commercial Machinery and Equipment* | 332710, 332813, 332999, 333319, 333999, 334519, 336399 |
| 3711 | Motor Vehicles and Passenger Car Bodies | 336111, 336112, 336120, 336211, 336992 |
| 3728 | Aircraft Parts and Auxiliary Equipment, NEC | 332912, 336411, 336413 |
| 3731 | Ship Building and Repairing | 336611 |
| 3999 | Manufacturing Industries, NEC3 | 316110, 321999, 325998, 326199, 332211, 332212, 332812, 332999, 333319, 335121, 335211, 337127, 339932, 339999 |
| 5032 | Brick, Stone, and Related Construction Materials3 | 423320, 425110, 425120, 444190 |
| 5211 | Lumber and Other Building Materials Dealers | 444110, 444190 |
| 5713 | Floor Coverings Store3 | 442210 |
| *Crystalline silica exposure primarily from abrasive blasting operations 1 Standard Industrial Classification Manual, 1987. Executive Office of the President, Office of Management and Budget. 2 North American Industry Classification System, United States, 2002. Executive Office of the President, Office of Management and Budget. 3 This industry may be subject to OSHA Instruction CPL 02-00-051 - Enforcement Exemptions and Limitations under the Appropriations Act (or a subsequent version). Source: Region IV Sampling Database. |  |  |

Appendix B (NEP)

Industries with Potential Overexposure to Crystalline Silica

This appendix contains a list of industries in which employees may be exposed to elevated levels of crystalline silica.  The list is based on a review of inspection data from OSHA's Integrated Management Information System (IMIS) for crystalline silica (quartz), for the period January 1996 through March 2007.  This table is intended to show the range of industries in which crystalline silica exposure may occur, but should not be considered to be an exhaustive listing.  Employee exposure to crystalline silica may occur in industries not listed here.  Likewise, crystalline silica exposure does not occur in all establishments encompassed within these North American Industry Classification System (NAICS) or Standard Industrial Classification (SIC) codes.

| Industries with Crystalline Silica Exposure, 1996-2007 |  |  |
| --- | --- | --- |
| 1987 SIC Code1 | 1987 SIC Industry Title | 2002 NAICS Code2 |
| 1521 | General Contractors-Single Family Houses | 236115, 236118 |
| 1522 | General Contractors-Residential Buildings Other Than Single-Family | 236115, 236118 |
| 1541 | General Contractors-Industrial Buildings and Warehouses | 236210, 236220 |
| 1611 | Highway and Street Construction, Except Elevated Highways | 237310 |
| 1622 | Bridge, Tunnel, and Elevated Highway Construction | 237310, 237990 |
| 1623 | Water, Sewer, Pipeline, and Communications and Power Line Construction | 237110, 237120, 237130 |
| 1629 | Heavy Construction, n.e.c. | 236210, 237110, 237120, 237130, 237990 |
| 1721 | Painting and Paper Hanging* | 237310, 238320 |
| 1741 | Masonry, Stone Setting, and Other Stone Work | 238140 |
| 1742 | Plastering, Drywall, Acoustical, and Insulation Work | 238310 |
| 1761 | Roofing, Siding, and Sheet Metal Work | 238160, 238170, 238390 |
| 1771 | Concrete Work | 238110, 238140, 238990 |
| 1794 | Excavation Work | 238910 |
| 1795 | Wrecking and Demolition Work | 238910 |
| 1799 | Special Trade Contractors, n.e.c. | 236220, 237990, 238150, 238190, 238290 |
| 3251 | Brick and Structural Clay Tile | 327121, 327331 |
| 3253 | Ceramic Wall and Floor Tile | 327122 |
| 3255 | Clay Refractories | 327124 |
| 3261 | Vitreous China Plumbing Fixtures and China and Earthenware Fittings and Bathroom Accessories | 327111 |
| 3262 | Vitreous China Table and Kitchen Articles | 327112 |
| 3264 | Porcelain Electrical Supplies | 327113 |
| 3269 | Pottery Products, n.e.c. | 327112 |
| 3271 | Concrete Block and Brick | 327331 |
| 3272 | Concrete Products, Except Block and Bricks | 327332, 327390, 32799 |
| 3273 | Ready-Mixed Concrete | 327320 |
| 3281 | Cut Stone and Stone Products | 327991 |
| 3291 | Abrasive Products | 327910, 332999 |
| 3299 | Nonmetallic Mineral Products, n.e.c. | 327112, 327420, 327999 |
| 3312 | Steel Works, Blast Furnaces (Including Coke Ovens), and Rolling Mills | 324199, 331111, 331221 |
| 3321 | Gray and Ductile Iron Foundries | 331511 |
| 3322 | Malleable Iron Foundries | 331511 |
| 3325 | Steel Foundries, n.e.c. | 331513 |
| 3334 | Primary Production of Aluminum | 331312 |
| 3365 | Aluminum Foundries | 331524 |
| 3366 | Copper Foundries | 331525 |
| 3369 | Nonferrous Foundries, Except Aluminum and Copper | 331528 |
| 3431 | Enameled Iron and Metal Sanitary Ware | 332998 |
| 3441 | Fabricated and Structural Metal* | 332312 |
| 3443 | Fabricated Plate Work (Boiler Shops)* | 332313, 332410, 332420 |
| 3444 | Sheet Metal Work* | 332321, 332322, 332439, 333415 |
| 3471 | Electroplating, Polishing, Anodizing, and Coloring* | 332813 |
| 3479 | Coating, Engraving, and Allied Services, n.e.c.* | 332812, 339911, 339912, 339914 |
| 3531 | Construction Machinery and Equipment* | 333120, 333923, 336510 |
| 3599 | Industrial and Commercial Machinery and Equipment* | 332710, 332813, 332999, 333319, 333999, 334519, 336399 |
| 3715 | Truck Trailers* | 336212 |
| 5032 | Brick, Stone, and Related Construction Materials3 | 423320, 425110, 425120, 444190 |
| 7532 | Top, Body, and Upholstery Repair Shops and Paint Shops* | 811121 |
| *Crystalline silica exposure primarily from abrasive blasting operations 1 Standard Industrial Classification Manual, 1987. Executive Office of the President, Office of Management and Budget. 2 North American Industry Classification System, United States, 2002. Executive Office of the President, Office of Management and Budget. 3 This industry may be subject to OSHA Instruction CPL 02-00-051 - Enforcement Exemptions and Limitations under the Appropriations Act (or a subsequent version). Source: Federal OSHA Inspection Data for Silica (Code 9010-Quartz) compiled in the OSHA Integrated Management Information System (IMIS), from 01/01/1996 through 03/31/2007. |  |  |

APPENDIX C

Links to Reference Documents

CPL 02-00-148

OSHA Instruction: Field Operations Manual (FOM)

https://www.osha.gov/OshDoc/Directive_pdf/CPL_02-00-148.pdf

CPL 03-00-007

OSHA Instruction: National Emphasis Program-Crystalline Silica

http://www.osha.gov/pls/oshaweb/owadisp.show_document?p_table=DIRECTIVES&p_id=3790

TED 01-00-015

OSHA Technical Manual (OTM)

https://www.osha.gov/dts/osta/otm/otm_toc.html

APPENDIX D

CSHO Checklist for Conducting Silica-Related Inspections

This non-mandatory checklist is intended as a quick reference tool for Compliance Safety and Health Officers (CSHOs) conducting silica-related inspections.  The CSHO may wish to review the checklist before completing the inspection to make sure that none of the essential elements have been overlooked.  The checklist addresses all of the topics discussed in Section XI(B), Inspection Procedures, of the NEP.

|  | Employee Exposure Monitoring |  |  | Medical Surveillance |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Sample for Respirable Dust/Silica |  |  | Employer Aware of Silicosis Risk |
|  |  | Leak Test Filters/Cyclones |  |  | Employer Identifying Possible Cases |
|  |  | Bulk Samples of Settled Dust |  |  | Employer Referring Cases to Physician |
|  |  | Employer's Monitoring Records |  |  | Other |
|  |  | Other |  |  |  |
|  |  |  |  | Housekeeping and Hygiene Practices |  |
|  | Engineering and Work Practice Controls |  |  |  | Facility Cleanliness |
|  |  | Location of Employees |  |  | Clean-up Methods (Compressed Air, Dry Sweeping?) |
|  |  | Ventilation |  |  |  |
|  |  | Wet Methods |  |  | Change Rooms/PPE Storage |
|  |  | Other |  |  | Separate Break Areas |
|  |  |  |  |  | Other |
|  | Respiratory Protection |  |  |  |  |
|  |  | Written Program |  | Employee Exposure and Medical Records |  |
|  |  | Cartridge Selection and Change-out Schedule |  |  | Employer Monitoring and Medical Records |
|  |  | Medical and Fit Test Records |  |  | Employee Access and Confidentiality |
|  |  | Breathing Air Quality and Use |  |  | Other |
|  |  | Other |  |  |  |
|  |  |  |  | Abrasive Blasting (on-site or off-site) |  |
|  | Hazard Communication |  |  |  | Sample for Silica and Metals |
|  |  | Written Program |  |  | (including Bystanders) |
|  |  | MSDSs |  |  | Sample for Noise |
|  |  | Training |  |  | Ventilation and Dust Containment |
|  |  | Bulk Samples of Products |  |  | PPE and Respirators |
|  |  | Other |  |  | Carbon Monoxide Alarm on Respirator |
|  |  |  |  |  | Manual Control of Blast Nozzle |
|  | Symptoms of Silicosis in Workplace |  |  |  | Operating Valve |
|  |  | Survey/Interview Employees |  |  | Electrical Grounding |
|  |  | Employees Obtaining Medical Evaluations |  |  | Pressure Range (90-120 psi) |
|  |  | Other |  |  | Heat Stress |
|  |  |  |  |  | Other |